| NPRR Number | 524 | NPRR Title | Resource Limits in Providing Ancillary Service |
| --- | --- | --- | --- |
| Date |  | June 4, 2013 |  |
| Submitter’s Information |  |  |  |
| Name |  | Robert Helton |  |
| E-mail Address |  | Robert.helton@gdfsuezna.com |  |
| Company |  | GDF Suez Energy Marketing NA, Inc. |  |
| Phone Number |  | 713-636-1814 |  |
| Cell Number |  | 832-435-7815 |  |
| Market Segment |  | Independent Generator |  |

Comments

During the joint QMWG/PDCWG meeting on May 29, 2013 there was some confusion on how the language would be implemented.  GDF Suez NA offers the following comments as clarification as well as two examples.

A 100 MW unit offers in there full 24% (24MW) into RRS and was awarded the full amount.

24MW x .83 = 20MW (must be Hz Responsive)

24MW – 20MW = 4MW (can be power augmentation)

A 100MW unit offers in the 24% into RRS but is awarded 18MW

18MW x .83 = 15MW (must be Hz responsive)

18MW – 15MW = 3MW (can be Hz responsive)

Revised Cover Page Language

| Revision Description | This Nodal Protocol Revision Request (NPRR) will clarify and will allow Generating Resources with supplemental capacity that is not always frequency responsive to provide the supplemental capacity to the market and also continue to participate in the Responsive Reserve (RRS) Service market. |
| --- | --- |
| Reason for Revision | Many Generation Resources in the ERCOT Region have identified opportunities to produce additional energy above their traditional High Sustained Limit (HSL) by using various types of supplemental MW enhancements. While the Generating Resource continues to provide frequency response when operating between the Low Sustained Limit (LSL) and the traditional HSL, the new supplemental capacity above the traditional HSL may or may not be frequency responsive at all times. |

Revised Proposed Protocol Language

3.18	Resource Limits in Providing Ancillary Service

(1)	For both Generation Resources and Load Resources the High Sustained Limit (HSL) must be greater than or equal to the Low Sustained Limit (LSL) and the sum of the Resource-specific designation of capacity to provide Responsive Reserve (RRS), Regulation Up (Reg-Up), Regulation Down (Reg-Down), and Non-Spinning Reserve (Non-Spin).

(2)	For Non-Spin, the amount of Non-Spin provided must be less than or equal to the HSL for Off-Line Generation Resources.

(3)	For RRS Service:

(a)	The full amount of RRS provided from a Generation Resource must be less than or equal to 24% of thermal unit HSL for an Ancillary Service Offer, and must be less than or equal to ten times the Emergency Ramp Rate.  The first 83% of the awarded RRS capacity per Generation Resource must be immediately and fully frequency responsive.  Additional capacity above 17% may come from supplemental capacity through power augmentation technology;

(b)	Hydro Generation Resources operating in the synchronous condenser fast-response mode may provide RRS up to the hydro Generation Resource’s proven 20-second response capability (which may be 100% of the HSL).  The initiation setting of the automatic under-frequency relay setting shall not be lower than 59.80 Hz;

(c)	For any hydro Generation Resource with a 5% droop setting operating as a generator, the amount of RRS provided may never be more than 24% of the HSL; and

(d)	The amount of RRS provided from a Load Resource must be less than or equal to the HSL minus the sum of the LSL, Reg-Up Resource Responsibility, Reg-Down Resource Responsibility, and Non-Spin Resource Responsibility.