| NPRR Number | 541 | NPRR Title | SPS and RAP Modeling Clarification |
| --- | --- | --- | --- |
| Date |  | July 22, 2013 |  |
| Submitter’s Information |  |  |  |
| Name |  | Frank Owens on behalf of the Operations Working Group (OWG) |  |
| E-mail Address |  | fowens@crosstexas.com |  |
| Company |  | Cross Texas Transmission LLC |  |
| Phone Number |  | 512-276-6122 |  |
| Cell Number |  | 512-348-3294 |  |
| Market Segment |  | Not Applicable |  |

Comments

At its July 17, 2013 meeting, OWG reviewed Nodal Protocol Revision Request (NPRR) 541 including the 7/10/13 ERCOT comments.  After discussion, OWG was in consensus to endorse NPRR541 as amended by the 7/10/13 ERCOT comments and as revised by OWG.

Revised Proposed Protocol Language

3.10.7.4	Special Protection Systems and Remedial Action Plans

(1)	All approved Special Protection Systems (SPSs) and Remedial Action Plans (RAPs) must be defined in the Network Operations Model.

(2)	Proposed new SPSs and RAPs and proposed changes to SPSs and RAPs must be submitted to ERCOT for review and approval.  ERCOT shall seek input from TSPs and Resource Entities that own Transmission Facilities included in the SPS or RAP, and shall approve proposed new SPSs and RAPs and proposed changes to SPSs and RAPs in accordance with the process outlined in the Operating Guides.  This shall include verification of the Network Operations Model.  ERCOT shall provide notification to the market and post all SPSs and RAPs under consideration on the MIS Secure Area within five Business Days of receipt.

(3)	ERCOT shall model approved SPSs and RAPs using a NOMCR and include the SPS or RAP in the security analysis.  The NOMCR shall include a detailed description of the system conditions required to implement the SPS or RAP.  Execution of an SPS or RAP shall be included or assumed in the calculation of LMPs as well as the Network Operations Model.  ERCOT shall provide notification to the market and post on the MIS Secure Area all approved SPSs and RAPs at least two Business Days before implementation, identifying the date of implementation.  For RAPs developed in Real-Time, ERCOT shall provide notification to the market as soon as practicable.