Requirements for 
Aggregate Load Resource Participation 
in the ERCOT Markets

Version 1.0

EFFECTIVE DATE: ___________

Revision History

| Date of TAC Approval | Version | Description | Author |
| --- | --- | --- | --- |
| _____ | 1.0 | Adapted from White Paper “Functional Description of Aggregated Load Resources” v. 0.1.4 | ERCOT Staff (Market Design & Development, Demand Integration) |

PROTOCOL DISCLAIMER

This Business Practice describes ERCOT Systems and the response of these systems to Market Participant submissions incidental to the conduct of operations in the ERCOT Texas Nodal Market implementation and is not intended to be a substitute for the ERCOT Nodal Protocols (available at http://www.ercot.com/mktrules/nprotocols/current), as amended from time to time. If any conflict exists between this document and the ERCOT Nodal Protocols, the ERCOT Nodal Protocols shall control in all respects.

Table of Contents

1	Background and Introduction	5

2	Change Control Process	6

3	Telemetry Validation	7

4	Management of Changes to ALR Populations	11

5	Network Modeling	12

6	Measurement & Verification	14

# Background and Introduction

Qualification as a Load Resource is a pre-requisite for the provision of demand response in the Ancillary Services markets and Real-Time Energy Market. Since the ERCOT market opened in 2002, the fleet of Load Resources has been limited to single-site Loads.  Due to limitations in various ERCOT systems and to technological barriers to entry, aggregations of Loads have been unable to qualify as Load Resources.

Enabling access to the ERCOT markets for Aggregate Load Resources (ALRs) offers the prospect of broader customer participation in electricity supply and demand, and could increase the pool of participants in ERCOT’s real-time energy and day-ahead Ancillary Services markets. Demand response from ALRs may come from such sources as controls of residential HVAC, water heaters, and pool pumps; commercial lighting and HVAC; and variable-speed motor loads.

This Other Binding Document (Version 1.0) sets forth the detailed requirements for aggregations of loads (more than one single load site) to qualify as ALRs and maintain such qualification, thus becoming eligible to provide Ancillary Services.  This initial version of the document is limited to ALR qualification for the provision of Non-Spin.

# Change Control Process

Any changes to this document shall be approved by TAC and may be subject to subcommittee review at TAC’s discretion.

# Telemetry Validation

A Qualified Scheduling Entity (QSE) representing a Load Resource is required to send real-time telemetered data to ERCOT every 2 seconds consistent with the requirements in Section 6 of the ERCOT Protocols, Section 7 of the ERCOT Operating Guides, and the ERCOT ICCP Handbook.  Telemetered data points are specific to the service being provided, and are listed in detail in ERCOT Protocols Section 6.5.5.2.

As noted previously, this version (1.0) of this document applies solely to the provision of Non-Spin by ALRs, with telemetry validation provided by interval metering at the premise-level.

For an ALR, requiring full-time physical telemetry from each device or premise under control is neither reasonable nor necessary. Such a requirement would potentially cause the number of telemetry points coming into the ISO to reach an unreasonably burdensome level, while the relative value of the individual points – potentially numbering in the hundreds of thousands for a single ALR -- would be insignificant.

The objective of ALR telemetry validation is to create an acceptable standard that provides ERCOT operations with assurance that the telemetered values from the QSE provide an accurate representation of the physical load characteristics of the aggregation.

This section describes the processes ERCOT will use to conduct qualification testing and validation for QSE telemetry.  Telemetry validation, as a step toward qualification to provide one more ERCOT services, will insure the integrity of an ALR’s telemetered data points as a representation of aggregated distributed field data that meets reasonableness criteria consistent with good utility practice.

ERCOT shall validate telemetry data by comparing physical premise-level data (“resource-level data”) to the comparable representative QSE telemetry signal (“QSE telemetry”) over specified test periods, using the procedures described below.  To enable this comparison, the following will be required:

- Each premise in the ALR is required to have 15-minute interval metering, whether an ESI ID within the competitive choice areas of ERCOT, or billing meter data within a NOIE.  ERCOT will use this premise-level interval meter data as a part of the telemetry validation process.

- The QSE telemetry signal to ERCOT must meet all existing requirements and specifications as defined in the Protocols, Section 7 of the ERCOT Operating Guides, and the ERCOT ICCP Handbook.

QSE telemetry data for an LR whose Ancillary Service qualification is limited to Non-Spin shall be validated using 15-minute interval data at the ALR member Load sites.  Such 15-minute data must be time-stamped within appropriate standards in correlation with ERCOT 15-minute settlement clock intervals, and shall be provided to ERCOT for all metered sites within the ALR through one of the following methods:

- Interval Data Recorder (IDR) or Advanced Metering Infrastructure (AMI) Electric Service Identifier (ESI ID) metering maintained and read by a Transmission & Distribution Service Provider (TDSP) and submitted regularly to ERCOT via the Texas Standard Electronic Transaction (TX SET) process (for IDR metering) or via the approved file format defined in Retail Market Guides Appendix G (for AMI metering);

- IDR, AMI, or equivalent metering, in the form of a non-settlement ESI ID or a designated unique meter identifier, maintained and read by the TDSP within a Non-Opt In Entity (NOIE), and submitted to ERCOT by the NOIE TDSP either via TX SET or on a schedule and in a format defined by ERCOT.

Ongoing telemetry validation for non-ESI ID meter data is dependent upon timely and accurate meter data submissions, meeting a schedule approved by ERCOT.  Unless otherwise specified by ERCOT, non-ESI ID meter data shall be submitted to ERCOT on a monthly basis, no later than 35 days after the last Operating Day in the data set.  Failure to meet the requirements may result in suspension of the ALR’s qualification to provide Non-Spin.  An ALR that has been suspended for this reason may be reinstated only upon successful reinstatement of accurate and timely meter data submissions.

and will be subject to SCED)Market (a.k.a., SCED).enablement of Load Resource participatoin cation section of this white paper

ERCOT will aggregate the resource-level 15-minute interval meter data to the ALR level.  The aggregated resource-level data sets will be compared to the QSE telemetry values for Net Real Power Consumption, averaged over each 15-minute settlement interval, to verify that the telemetry values are an acceptable representation of the ALR.  ERCOT will conduct this comparison across all 15-minute settlement intervals on a monthly basis.

The telemetry must validate to the following criteria:  for each month of participation, 90% of the 15-minute aggregated Net Real Power Consumption values must be within 10% of the resource-level interval meter data.

ERCOT will conduct a telemetry validation test as part of the ALR’s qualification test to provide Non-Spin as follows: for the duration of the qualification test, 80% of the 15-minute aggregated SPC+2 values must be within 10% of the NPF values for the corresponding interval.  In addition, in order to verify on an ongoing basis that the QSE telemetry to ERCOT is a valid representation of an ALR that provides Non-Spin, ERCOT will validate the telemetry as follows: on a monthly basis, 80% of the 15-minute aggregated SPC+2 values must be within 10% of the SPC values for the corresponding interval.

During all Non-Spin deployment events ERCOT will compare the SPC and SPC+2 values for each interval of the event to the ERCOT baseline for the interval.  If the difference between the ERCOT baseline and both the SPC and SPC+2 values is less than or equal to 10%, the telemetry will be deemed valid for that event.

Failure to meet the telemetry validations may result in suspension of the ALR’s qualification to provide Non-Spin.  An ALR that has been suspended for failure of the annual unannounced telemetry validation test may be reinstated only upon successfully completing a new telemetry test as prescribed herein.

No later than April 1 of each year, ERCOT shall submit a report to TAC containing the results of telemetry validation testing for the prior calendar year.  The report shall contain, at a minimum:

- The total number of qualified ALRs in the ERCOT region;

- The number of monthly telemetry validation tests conducted;

- The number of monthly telemetry validation test failures.

# Management of Changes to ALR Populations

Changing ALR parameters will be managed by the Resource Entity and the QSE using a market interface dedicated to ALR population maintenance.

- ALR parameters will be established in the Network Model by the ALR’s Resource Entity using the Resource Asset Registration Form (RARF).  ALRs that are subject to dynamically changing populations should set their RARF parameters at levels that will accommodate several months of potential growth so as to reduce the need for frequent RARF submissions.

- ALR population changes will be managed via the market interface accessible by participating REs and QSEs:

  - In the competitive choice areas, QSEs will manage the ALR population by ESI ID, which ERCOT will then cross-reference to its internal systems.

  - In the NOIE territories, QSEs will provide a unique meter identifier for each member of the aggregation (“Site”).  NOIE QSEs and TDSPs will be required to affirm the accuracy and integrity of ALR membership and meter data.

# Network Modeling

Opening the ERCOT markets to participation by aggregations of distribution-connected small commercial and residential loads will require development of alternative Network Modeling provisions.  This section of the requirements document sets forth the criteria for the initial rollout of those provisions.

The location of a Load Resource in the Network Model is identified in the Resource Asset Code.  During the initial phase of ALR participation in the ERCOT markets, membership in an ALR shall be limited to metered load sites within the same ERCOT Load Zone.  Consistent with current practice for distribution-level single-site LRs, the TDSP in collaboration with the Resource Entity and ERCOT will assign each ALR to a single Load point in the ERCOT Common Information Model.  The total demand response capability of all ALRs assigned to any single Load point shall be capped at 100% of the rating of the Load point.  The rating of a Load point is defined as the value estimated by the ERCOT State Estimator for that Load point at the time of the ERCOT historic coincident peak demand.

In the long-term, ALR participation in the markets may require an ALR to associate with multiple Loads in the ERCOT CIM while preserving the ability of the ERCOT ISO to dispatch resources for congestion management based on their location.  ERCOT will engage with stakeholders during the phase 1 of ALR participation to identify workable options for this phase 2 approach.  Because phase 2 will require changes to market rules and potentially Substantive Rules, and is certain to require significant ERCOT system upgrades, ERCOT hereby establishes a set of caps on initial ALR participation.  These caps are implemented in order to avoid system degradation (which could occur if large numbers of ALRs begin are participating) and potential challenges to effective congestion management and grid reliability (due to dispersion of participating loads with insufficient locational specificity).  The caps shall be lifted upon development and implementation of phase 2 of the ALR network modeling approach.

- System-wide ALR participation shall be capped at 250 ALRs.

- The combined demand response capability of all ALRs within any single ERCOT Load Zone shall be capped at 5% of the Load Zone’s highest historic summer peak demand.

If ERCOT or a TDSP determines that any of the caps described in this section are insufficient to prevent an operational challenge, ERCOT commits to working with stakeholders to determine appropriate changes and seek expedited TAC approval of an amended version of this document.

# Measurement & Verification

As part of the qualification process for an ALR to provide Non-spin, ERCOT will assign the ALR to its appropriate performance evaluation methodology based on an analysis of the ALR’s historical meter data. This process will be similar to the baseline assignment process used by ERCOT in the administration of Emergency Response Service.  In order to qualify to provide Non-Spin, an ALR must be deemed by ERCOT to be eligible for measurement and verification via either the Meter Before/Meter After or Baseline performance evaluation methodologies.

Performance evaluation methodology assignments will depend on the following factors:

- The predictability of the load as determined through analysis of historical meter data.

- The amount of historical interval meter data available.

- Whether the ALR’s membership is dynamic (subject to migration in either direction) or static.

  - If the ALR membership is dynamic, the following provisions are in effect:

    - Any ALR consisting entirely of residential sites will be considered eligible for assignment to a Baseline methodology, and will retain that designation so long as any sites added to the ALR are residential.

    - An ALR consisting of commercial and industrial (C&I) sites and also subject to migration will be subject to baseline review by ERCOT any time a site is added.  This review provision may be waived by ERCOT if ERCOT, in consultation with the QSE, determines that the added sites meet a uniformity test consistent with the existing sites in the ALR.  To avoid ongoing baseline reviews, the ALR should be composed of loads with similar load shapes and, depending on the size of the aggregation, load magnitude.  Uniformity (a.k.a. homogeneity) enables scalable growth, statistical sampling consistent with industry standard load research practices, and acceptable migration management.  ERCOT may revoke an ALR’s Non-spin qualification if ERCOT determines that the composition of the ALR fails to meet a uniformity standard consistent with good utility practice.

  - If the ALR membership is static (not subject to migration), the ALR will retain the performance evaluation methodology assigned at the time of registration and qualification.  ERCOT may annually review a static ALR’s load characteristics to ensure the performance evaluation methodology assignment continues to apply.

ERCOT shall deny Non-spin qualification for an ALR if it fails to qualify for either the Meter Before/Meter After or Baseline methodologies.  For the latter, ERCOT may evaluate the ALR against any of the four baseline types described in document entitled “Emergency Response Service Default Baseline Methodologies.

As described in Protocols Section 8.1.1.4.3, the data used for primary measurement and verification of Load Resource performance in a Non-spin event are the telemetry values for net real power consumption (net power flow), scheduled power consumption, and scheduled power consumption plus two.  As a secondary validation step, ERCOT may use interval meter data from the ALR to verify an ALR’s performance in a Non-spin deployment event.   If the interval meter data evaluation indicates that the ALR met its performance obligations in the Non-spin event, the ALR will be considered in compliance for that event irrespective of the telemetry values.  If the interval meter data evaluation indicates that the ALR failed to meet its performance obligations in the Non-spin event, the ALR will be deemed to have failed to meet its responsibility for that event irrespective of the telemetry values.  ERCOT may revoke the ALR’s qualification to provide Non-spin if the ALR demonstrates a continuing pattern of failure to perform.

ERCOT may use statistical sampling for event performance validation using 15-minute interval meter data if the ALR consists of a large numbers of member Sites and is deemed by ERCOT to be capable of statistical sampling consistent with industry best practices.    ERCOT shall determine the sample size and makeup for any evaluation of this type.  Statistical sampling does not relieve the ALR of the responsibility for universal 15-minute interval metering on all member Sites, nor does it change the requirement for timely submission of meter data to ERCOT for all sites in an ALR.

Available at http://www.ercot.com/services/mdt/userguides/.

NOIE advanced meter data submission must meet formatting requirements in place for Emergency Response Service.  See document entitled “Interval Data File Format Descriptions at http://www.ercot.com/services/programs/load/eils/documents.

The market interface at the outset will be spreadsheets, formatted according to ERCOT specifications, submitted to a specified email inbox.  Future enhancements may include a user interface software platform dedicated to demand response asset maintenance.

See Protocols Sections Sections 8.1.1.2.1.3 and 8.1.1.4.3, as revised by NPRR 532.

Posted at: http://www.ercot.com/services/programs/load/eils/documents

See Protocols Sections Section 8.1.1.4.3 as revised by NPRR 532.

An example of industry best practice is the lagged-dynamic sampling methodology detailed in the former ERCOT Zonal Protocols, Section 18.7.2, Load Profiling of ESI IDs under Direct Load Control and Load Profiling Guide Section 16.2, Direct Load Control.