| NPRR Number | 524 | NPRR Title | Resource Limits in Providing Ancillary Service |  |
| --- | --- | --- | --- | --- |
| Timeline | Urgent | Action |  | Recommended Approval |
| Date of Decision |  | August 22, 2013 |  |  |
| Proposed Effective Date |  | Upon implementation of manual workaround. |  |  |
| Priority and Rank Assigned |  | Priority – 2013; Rank – 890 |  |  |
| Nodal Protocol Sections Requiring Revision |  | 3.18, Resource Limits in Providing Ancillary Service 6.5.5.2, Operational Data Requirements |  |  |
| Market Guide Section(s) Requiring Revision |  | None. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) will clarify and allow Generation Resources with supplemental capacity that is not always frequency responsive to provide the supplemental capacity to the market and also continue to participate in the Responsive Reserve (RRS) Service market. |  |  |
| Reason for Revision |  | Many Generation Resources in the ERCOT Region have identified opportunities to produce additional energy above their traditional High Sustained Limit (HSL) by using various types of supplemental MW enhancements. While the Generation Resource continues to provide frequency response when operating between the Low Sustained Limit (LSL) and the traditional HSL, the new supplemental capacity above the traditional HSL may or may not be frequency responsive at all times. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR524 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 3/5/13, NPRR524 was posted. - On 3/8/13, Calpine comments were posted. - On 3/11/13, GDF Suez Energy Marketing NA comments were posted. - On 3/20/13, ERCOT comments were posted. - On 3/21/13, PRS considered NPRR524. - On 4/15/13, ROS comments were posted. - On 4/15/13, WMS comments were posted. - On 5/13/13, a second set of GDF Suez Energy Marketing NA comments were posted. - On 5/15/13, a third set of GDF Suez Energy Marketing NA comments were posted. - On 6/4/13, a fourth set of GDF Suez Energy Marketing NA comments were posted. - On 6/11/13, a second set of ERCOT comments were posted. - On 6/14/13, a second set of WMS comments were posted. - On 6/17/13, a second set of ROS comments were posted. - On 6/18/13, TIEC comments were posted. - On 6/20/13, PRS again considered NPRR524. - On 7/3/13, Performance, Disturbance, Compliance Working Group (PDCWG) comments were posted. - On 7/15/13, a third set of ROS comments were posted. - On 7/18/13, PRS again considered NPRR524. - On 7/30/13, Luminant comments were posted. - On 8/5/13, a second set of PDCWG comments were posted. - On 8/19/13, an Impact Analysis was posted. - On 8/22/13, PRS considered the 7/18/13 PRS Report and Impact Analysis for NPRR524. |  |  |
| PRS Decision |  | On 3/21/13, PRS unanimously voted to table NPRR524 and to refer the issue to ROS and WMS. All Market Segments were present for the vote. On 6/20/13, PRS voted via roll call vote to table NPRR524 and refer the issue to ROS. There was one opposing vote from the Independent Generator Market Segment and one abstention from the Independent Power Marketer (IPM) Market Segment. All Market Segments were present for the vote. On 7/18/13, PRS voted to grant NPRR524 Urgent status, recommend approval of NPRR524 as amended by the 6/11/13 ERCOT comments, and direct that NPRR524 return to PRS with an Impact Analysis. There were two opposing votes from the Consumer Market Segment and six abstentions from the Independent Generator, IPM (2), Independent Retail Electric Provider (IREP), Investor Owned Utility (IOU) and Municipal Market Segments. All Market Segments were present for the vote. On 8/22/13, PRS voted to endorse and forward the 7/18/13 PRS Report as amended by the 7/30/13 Luminant comments and Impact Analysis for NPRR524 to TAC with a recommended priority of 2013 and rank of 890. There were six abstentions from the Consumer (2), IPM, IREP, IOU and Municipal Market Segments. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 3/21/13, some participants requested that ROS and WMS review NPRR524 in conjunction with NPRR527, Required Combined Cycle Telemetry for Operational Awareness and PDCWG Analysis, to provide an opportunity to better understand the implications of both NPRRs. It was noted that discussions are expected to occur at the PDCWG and the Qualified Scheduling Entity (QSE) Managers Working Group (QMWG). ERCOT Staff noted that both NPRRs are related to compliance with Texas Reliability Entity (Texas RE) Regional Reliability Standard BAL-001-TRE-01, Primary Frequency Response in the ERCOT Region. On 6/20/13, PRS reviewed the 6/18/13 TIEC comments. Some participants recommended that PRS refer discussion of NPRR524 back to ROS so that it could consider the 6/18/13 TIEC comments. The sponsor of NPRR524 proposed that PRS vote on the merits of NPRR524, noting that the concepts included in the 6/18/13 TIEC comments would require system changes and could be proposed as a separate NPRR. Participants also discussed whether there should be a comprehensive review of the treatment of Primary Frequency Response, including whether it should be a paid service. On 7/18/13, participants reviewed the 7/15/13 ROS comments and discussed the corrections PDCWG intends to make in its analysis. Participants also discussed whether ROS had sufficiently addressed the concern raised in the 6/18/13 TIEC comments regarding the expansion of NPRR524 to Load Resources. On 8/22/13, it was noted that the 8/5/13 PDCWG comments, which state that there was a PDCWG consensus that NPRR524 does not impact reliability, were presented at the 8/15/13 ROS meeting. |  |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | - This NPRR will allow otherwise stranded capacity to be brought to the energy and Ancillary Service markets. |
|  | 2 | - ERCOT will be able to monitor supplemental capacity that may not be responsive to frequency at all times, by mapping Inter-Control Center Communications Protocol (ICCP) data to existing ERCOT Real-Time operations displays. |

| Sponsor |  |
| --- | --- |
| Name | Robert Helton |
| E-mail Address | Robert.helton@gdfsuezna.com |
| Company | GDF Suez Energy Marketing NA, Inc. |
| Phone Number | 713-636-1814 |
| Cell Number | 832-435-7815 |
| Market Segment | Independent Generator |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| Calpine 030813 | Questioned the use of HRL as a boundary value. Recommended additional discussion regarding the provision of RRS and Fast-Responding Regulation Service (FRRS) without the potential stranding of power augmentation capability and the disparate treatment of Primary Frequency Response providers. |
| GDF Suez Energy Marketing NA 031113 | Revised NPRR to replace HRL with High Sustained Limit (HSL) as the threshold for requiring frequency responsiveness. |
| ERCOT 032013 | Expressed concern with NPRR as originally submitted. Proposed revisions to define “Frequency Responsive,” clarify that all RRS capacity must be Frequency Responsive, and require that the portion of Resource capacity that is available but is not Frequency Responsive be sent to ERCOT via ICCP telemetry. |
| ROS 041513 | Recommended that NPRR524 be tabled for one month. |
| WMS 041513 | Recommended that NPRR524 be tabled for an additional month. |
| GDF Suez Energy Marketing NA 051313 | Proposed revisions to allow 4% of RRS capacity to be provided by power augmentation and recommended that NPRR524 be granted Urgent status to allow implementation prior to summer 2013. |
| GDF Suez Energy Marketing NA 051513 | Proposed revisions clarify that it is the first 20% of RRS capacity per Generation Resource that must be immediately and fully frequency responsive. |
| GDF Suez Energy Marketing NA 060413 | Proposed revisions to clarify that the first 83% of awarded RRS capacity per Generation Resource must be immediately and fully frequency responsive and provided examples. |
| ERCOT 061113 | Proposed revisions to clarify the changes proposed in the 6/4/13 GDF Suez Energy Marketing NA comments, clarify that the full amount of RRS capacity must be deliverable within ten minutes, and require Generation Resources to telemeter the portion of RRS capacity that is not frequency responsive. |
| WMS 061413 | Endorsed NPRR524 as amended by the 6/11/13 ERCOT comments. |
| ROS 061713 | Endorsed NPRR524 as amended by the 6/11/13 ERCOT comments. |
| TIEC 061813 | Proposed revisions to expand applicability of NPRR524 to Load Resources and to provide that if the total RRS procurement is reduced below 2,800 MW, the percentage of RRS capacity that must be immediately and fully frequency responsive will be automatically adjusted so that the amount of frequency-responsive RRS capacity remains at least 2,300 MW. |
| PDCWG 070313 | Identified a reliability concern and recommended rejection of NPRR524. |
| ROS 071513 | Endorsed NPRR524 as amended by the 6/11/13 ERCOT comments. |
| Luminant 073013 | Proposed revisions to clarify language in paragraph (3)(a) of Section 3.18. |
| PDCWG 080513 | Stated that the PDCWG was not able to identify a negative impact to reliability as a result of NPRR524 and that the PDCWG did not reach consensus to endorse the NPRR. |

Comments

Please note that the following NPRRs also propose revisions to Section 6.5.5.2:

- NPRR190, Clarification of Resource Definitions and Resource Registration of Self-Serve Generators for Reliability Purposes

- NPRR527, Required Combined Cycle Telemetry for Operational Awareness and PDCWG Analysis

- NPRR532, Performance Measurement and Verification and Telemetry Requirements for Load Resources Providing Non-Spin

- NPRR555, Load Resource Participation in Security-Constrained Economic Dispatch

Proposed Protocol Language Revision

3.18	Resource Limits in Providing Ancillary Service

(1)	For both Generation Resources and Load Resources the High Sustained Limit (HSL) must be greater than or equal to the Low Sustained Limit (LSL) and the sum of the Resource-specific designation of capacity to provide Responsive Reserve (RRS), Regulation Up (Reg-Up), Regulation Down (Reg-Down), and Non-Spinning Reserve (Non-Spin).

(2)	For Non-Spin, the amount of Non-Spin provided must be less than or equal to the HSL for Off-Line Generation Resources.

(3)	For RRS Service:

(a)	The full amount of RRS provided from a Generation Resource must be less than or equal to 24% of thermal unit HSL for an Ancillary Service Offer, and must be less than or equal to ten times the Emergency Ramp Rate.  The first 83% of the RRS capacity per Generation Resource must be immediately and fully frequency responsive.  The remaining 17% may come from supplemental capacity through power augmentation technology that is not required to be immediately frequency responsive but must be able to meet all other RRS requirements as described in Section 8.1.1.2.1.2, Responsive Reserve Service Qualification;

(b)	Hydro Generation Resources operating in the synchronous condenser fast-response mode may provide RRS up to the hydro Generation Resource’s proven 20-second response capability (which may be 100% of the HSL).  The initiation setting of the automatic under-frequency relay setting shall not be lower than 59.80 Hz;

(c)	For any hydro Generation Resource with a 5% droop setting operating as a generator, the amount of RRS provided may never be more than 24% of the HSL; and

(d)	The amount of RRS provided from a Load Resource must be less than or equal to the HSL minus the sum of the LSL, Reg-Up Resource Responsibility, Reg-Down Resource Responsibility, and Non-Spin Resource Responsibility.

6.5.5.2	Operational Data Requirements

(1)	ERCOT shall use Operating Period data to monitor and control the reliability of the ERCOT Transmission Grid and shall use it in network analysis software to predict the short-term reliability of the ERCOT Transmission Grid.  Each TSP, at its own expense, may obtain that Operating Period data from ERCOT or directly from QSEs.

(2)	A QSE representing a Generation Resource connected to Transmission Facilities or distribution facilities shall provide the following Real-Time telemetry data to ERCOT for each Generation Resource.  ERCOT shall make that data available, in accordance with ERCOT Protocols, NERC standards and policies, and Governmental Authority requirements, to requesting TSPs and DSPs operating within ERCOT.  Such data must be provided to the requesting TSP or DSP at the requesting TSP’s or DSP’s expense, including:

(a)	Net real power (in MW) as measured by installed power metering or as calculated in accordance with the Operating Guides based on metered gross real power and conversion constants determined by the Resource Entity and provided to ERCOT as a result of Section 3.7, Resource Parameters.  Net real power represents the actual generation of a Resource for all real power dispatch purposes, including use in Security-Constrained Economic Dispatch (SCED), determination of the High Ancillary Service Limit (HASL), High Dispatch Limit (HDL), Low Dispatch Limit (LDL) and Low Ancillary Service Limit (LASL), and is consistent with telemetered HSL and LSL;

(b)	Gross real power (in MW) as measured by installed power metering or as calculated in accordance with the Operating Guides based on metered real power, which may include Supervisory Control and Data Acquisition (SCADA) metering, and conversions constants determined by the Resource Entity and provided to ERCOT as a result of Section 3.7;

(c)	Gross Reactive Power (in Megavolt-Amperes reactive (MVAr));

(d)	Net Reactive Power (in MVAr);

(e)	Power to standby transformers serving plant auxiliary Load;

(f)	Status of switching devices in the plant switchyard not monitored by the TSP or DSP affecting flows on the ERCOT Transmission Grid;

(g)	Any data mutually agreed to by ERCOT and the QSE to adequately manage system reliability;

(h)	Generation Resource breaker and switch status;

(i)	HSL;

(j)	High Emergency Limit (HEL), under Section 6.5.9.2, Failure of the SCED Process;

(k)	Low Emergency Limit (LEL), under Section 6.5.9.2;

(l)	LSL;

(m)	Configuration identification for Combined Cycle Generation Resources;

(n)	Ancillary Service Schedule for each quantity of RRS and Non-Spin which is equal to the Ancillary Service Resource Responsibility minus the amount of Ancillary Service deployment;

(o)	Ancillary Service Resource Responsibility for each quantity of Regulation Up (Reg-Up), Regulation Down (Reg-Down), RRS and Non-Spin.  The sum of Ancillary Service Resource Responsibility for all Resources in a QSE is equal to the Ancillary Service Supply Responsibility for that QSE.  For a Generation Resource that is providing RRS using supplemental capacity through power augmentation technology that is not frequency responsive as described in paragraph (3)(a) of Section 3.18, Resource Limits in Providing Ancillary Service, the QSE shall separately telemeter the portion of RRS capacity in MW that is not frequency responsive; and

(p)	Reg-Up and Reg-Down Services participation factors represent how a QSE is planning to deploy the Ancillary Service energy on a percentage basis to specific qualified Resource(s).

(q)	The designated Master QSE of a Generation Resource that has been split to function as two or more Split Generation Resources shall provide Real-Time telemetry for items (a), (b), (c), (d), (e), (g), and (h) above, PSS and AVR status for the total Generation Resource in addition to the Split Generation Resource the Master QSE represents.

(3)	For each Wind-powered Generation Resource (WGR), the QSE shall set the HSL equal to the current net output capability of the facility.  The net output capability should consider the net real power of the WGR, turbine availability, weather conditions, and whether the WGR net output is being affected by compliance with a SCED Dispatch Instruction.

(4)	For each Aggregate Generation Resource (AGR), the QSE shall telemeter the number of its generators online.

(5)	A QSE representing a Load Resource connected to Transmission Facilities or distribution facilities shall provide the following Real-Time data to ERCOT for each Load Resource and ERCOT shall make the data available, in accordance with ERCOT Protocols, NERC standards and policies, and Governmental Authority requirements, to the Load Resource’s host TSP or DSP at the TSP’s or DSP’s expense.  The Load Resource’s net real power consumption, Low Power Consumption (LPC) and Maximum Power Consumption (MPC) shall be telemetered to ERCOT using a positive (+) sign convention:

(a)	Load Resource net real power consumption (in MW);

(b)	Any data mutually agreed to by ERCOT and the QSE to adequately manage system reliability;

(c)	Load Resource breaker status;

(d)	LPC (in MW);

(e)	MPC (in MW);

(f)	Ancillary Service Schedule (in MW) for each quantity of RRS and Non-Spin;

(g)	Ancillary Service Resource Responsibility (in MW) for each quantity of Reg-Up and Reg-Down for Controllable Load Resources, and RRS and Non-Spin for all Load Resources;

(h)	The status of the high-set under-frequency relay, if required for qualification;

(i)	For a Controllable Load Resource, the Scheduled Power Consumption that represents zero Ancillary Service deployments;

(j)	For a Controllable Load Resource, net Reactive Power (in MVAr);

(k)	Resource Status (Resource Status shall be ONRL if high-set under-frequency relay is active); and

(l)	Reg-Up and Reg-Down services participation factor, which represents how a QSE is planning to deploy the Ancillary Service energy on a percentage basis to specific qualified Resource.

(6)	A QSE with Resources used in SCED shall provide communications equipment to receive ERCOT-telemetered control deployments.

(7)	A QSE providing any Regulation Service shall provide telemetry indicating the appropriate status of Resources providing Reg-Up or Reg-Down, including status indicating whether the Resource is temporarily blocked from receiving Reg-Up and/or Reg-Down deployments from the QSE.  This temporary blocking will be indicated by the enabling of the Raise Block Status and/or Lower Block Status telemetry points.

(a)	Raise Block Status and Lower Block Status are telemetry points used in transient unit conditions to communicate to ERCOT that a Resource’s ability to adjust its output has been unexpectedly impaired.

(b)	When one or both of the telemetry points are enabled for a Resource, ERCOT will cease using the regulation capacity assigned to that Resource for Ancillary Service deployment.

(c)	This hiatus of deployment will not excuse the Resource’s obligation to provide the Ancillary Services for which it has been committed.

(d)	These telemetry points shall only be utilized during unforeseen transient unit conditions such as plant equipment failures.  Raise Block Status and Lower Block Status shall only be enabled until the Resource operator has time to update the Resource limits and Ancillary Service telemetry to reflect the problem.

(e)	The Resource limits and Ancillary Service telemetry shall be updated as soon as practicable.  Raise Block Status and Lower Block Status will then be disabled.

(8)	Real-Time data for reliability purposes must be accurate to within three percent.  This telemetry may be provided from relaying accuracy instrumentation transformers.

(9)	Each QSE shall report the current configuration of combined-cycle Resources that it represents to ERCOT.  The telemetered Resource Status for a Combined Cycle Generation Resource may only be assigned a Resource Status of OFFNS if no generation units within that Combined Cycle Generation Resource are On-Line.

(10)	A QSE representing combined-cycle Resources shall provide ERCOT with the possible operating configurations for each power block with accompanying limits.  Combined Cycle Train power augmentation methods may only be included as part of one or more of the registered Combined Cycle Generation Resource configurations.  The Energy Offer Curve in the Three-Part Supply Offer that includes the offered power augmentation method may reflect the price of the added capability.  Such power augmentation methods may include:

(a)	Combustion turbine inlet air cooling (CTIAC) methods;

(b)	Duct firing;

(c)	Other ways of temporarily increasing the output of combined-cycle Resources; and

(d)	For Qualifying Facilities (QFs), an LSL that represents the minimum energy available, in MW, from the Combined Cycle Generation Resource for economic dispatch based on the minimum stable steam delivery to the thermal host plus a justifiable reliability margin that accounts for changes in ambient conditions.