| NPRR Number | 550 | NPRR Title | Modifications to Planning Reserve Margin Inputs |  |
| --- | --- | --- | --- | --- |
| Timeline | Urgent | Action |  | Tabled |
| Date of Decision |  | October 3, 2013 |  |  |
| Proposed Effective Date |  | To be determined. |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Nodal Protocol Sections Requiring Revision |  | 3.2.6.1, Minimum ERCOT Planning Reserve Margin Criterion 3.2.6.2.1, Peak Load Estimate 3.2.6.2.2, Total Capacity Estimate 21.1, Introduction |  |  |
| Market Guide Section(s) Requiring Revision |  | None. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) proposes modifications to the Planning Reserve Margin (PRM) based on the results of the 2012 ERCOT Loss of Load Study and subsequent discussions of the Generation Adequacy Task Force (GATF). This NPRR also removes language in Section 21.1 that allows Section 3.2.6.1 to be revised via the Administrative NPRR process. |  |  |
| Reason for Revision |  | To update the PRM calculations based on the 2012 Loss of Load Study. WMS endorsed the language contained in NPRR550 as submitted at its 6/12/13 meeting. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR550 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 6/27/13, NPRR550 was posted. - On 7/1/13, ERCOT comments were posted. - On 7/18/13, PRS considered NPRR550. - On 8/22/13, PRS again considered NPRR550. - On 9/9/13, ERCOT comments were posted. - On 9/12/13, an Impact Analysis was posted. - On 9/13/13, WMS comments were posted. - On 9/19/13, PRS considered the 8/22/13 PRS Report and Impact Analysis for NPRR550. - On 9/24/13, a revised Impact Analysis was posted. - On 10/1/13, TIEC comments were posted. - On 10/3/13, TAC considered NPRR550. |  |  |
| PRS Decision |  | On 7/18/13, PRS voted to table NPRR550 via roll call vote. There were seven opposing votes from the Independent Generator (2), Independent Power Marketer (IPM) (3), Consumer and Municipal Market Segments and one abstention from the Consumer Market Segment. All Market Segments were present for the vote. On 8/22/13, PRS voted to recommend approval of NPRR550 as amended by the 7/1/13 ERCOT comments and as revised by PRS. There was one abstention from the Consumer Market Segment. All Market Segments were present for the vote. On 9/19/13, PRS voted to endorse and forward the 8/22/13 PRS Report as amended by the 9/9/13 ERCOT comments and the Impact Analysis for NPRR550 to TAC. There was one opposing vote from the Consumer Market Segment and one abstention from the Independent Retail Electric Provider (IREP) Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 7/18/13, some participants suggested that NPRR550 be tabled until the ERCOT Board considers pending revisions to the PRM criterion. Others suggested that proposed calculation inputs for the Report on Capacity, Demand and Reserves in the ERCOT Region (CDR) should be advanced to allow for inclusion in the December CDR. On 8/22/13, there were concerns expressed with the proposed calculation inputs related to Mothballed Generation Resources. It was suggested that NPRR550 move forward without the PRM percentage to allow for the revised calculation inputs to be used in the December CDR; and that the approved PRM percentage should not reside in the Protocols. On 9/19/13, it was clarified that the Planning Guide and the Protocols should have two different criteria including the need for a federal Greenhouse Gas permit. |  |  |
| TAC Decision |  | On 10/3/13, TAC voted to grant NPRR550 Urgent status and to table NPRR550 for one month. There were three abstentions from the Independent Generator Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of TAC Decision |  | On 10/3/13, the 10/1/13 TIEC comments were discussed. It was requested that ERCOT provide information on how the changes proposed in NPRR550 would impact CDR capacity. It was commented that NPRR550 did not propose significant changes but rather improvements to the CDR and had been thoroughly vetted through the stakeholder process. |  |  |
| ERCOT Opinion |  | ERCOT supports approval of NPRR550 as it revises the PRM calculation inputs based on analysis of the 2012 Loss of Load Study, recommendations from the GATF, and comments from the Texas Reliability Entity (Texas RE). |  |  |

| Business Case |  |
| --- | --- |
| 1 | Updates the PRM calculation inputs based on the 2012 Loss of Load Study. |

| Sponsor |  |
| --- | --- |
| Name | Adrian Pieniazek on behalf of the GATF |
| E-mail Address | adrian.pieniazek@nrgenergy.com |
| Company | NRG Texas LLC |
| Phone Number | (512) 473-8895 |
| Cell Number | (512) 844-9888 |
| Market Segment | Not applicable. |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Ann Boren |
| E-Mail Address | aboren@ercot.com |
| Phone Number | (512) 275-7411 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 070113 | Proposed revisions to allow for the PRM to be updated on the first day of the month following ERCOT Board approval and proposed removing language that allows the PRM to be revised via an Administrative NPRR. |
| ERCOT 090913 | Proposed changes to the PRM calculation related to the forecast of Emergency Response Service (ERS) capacity and a requirement for ERCOT to post the approved minimum ERCOT PRM on the ERCOT website. |
| WMS 091313 | Endorsed NPRR550 as amended by the 9/9/13 ERCOT comments. |
| TIEC 100113 | Expressed concerns regarding changing the CDR methodology while Public Utility Commission of Texas (PUCT) projects that examine the CDR inputs are underway and while the CDR is being used to evaluate the impact of recent market design changes. |

Comments

Please note that NPRR567, Stakeholder Review of Resource Registration Data Requirements, also proposes revisions to Section 3.2.6.2.2.

Revised Proposed Protocol Language

- 3.2.6.1	Minimum ERCOT Planning Reserve Margin Criterion

The minimum ERCOT PRM criterion is approved by the ERCOT Board.  ERCOT shall periodically review and recommend to the ERCOT Board any changes to the minimum ERCOT PRM to help ensure adequate reliability of the ERCOT System.  ERCOT shall update the minimum PRM on the first day of the month following ERCOT Board approval unless otherwise directed by the ERCOT Board.  ERCOT shall post the revised minimum PRM to the ERCOT website prior to implementation.

- 3.2.6.2.1	Peak Load Estimate

ERCOT shall prepare, at least annually, a forecast of the total peak Load for both summer and winter Peak Load Seasons for the current year and a minimum of ten future years using an econometric forecast, taking into account econometric inputs, weather conditions, demographic data and other variables as deemed appropriate by ERCOT.  The firm Peak Load Season estimate shall be determined by the following equation:

FIRMPKLD s, i	=	TOTPKLD s, i – LRRRS s, i – LRNSRS s, i – ERS s, i – CLR s, i – ENERGYEFF s, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| FIRMPKLD s, i | MW | Firm Peak Load Estimate—The Firm Peak Load Estimate for the Peak Load Season s for the year i. |
| TOTPKLD s, i | MW | Total Peak Load Estimate—The Total Peak Load Estimate for the Peak Load Season s for the year i. |
| LRRRS s, i | MW | Load Resource providing RRS—The amount of RRS a Load Resource is providing for the Peak Load Season s for the year i. |
| LRNSRS s, i | MW | Load Resource providing Non-Spinning Reserve (Non-Spin)—The estimated amount of Non-Spin that Load Resources are providing for the Peak Load Season s for the year i. |
| ERS s, i | MW | Emergency Response Service (ERS)—The estimated amount of ERS for the Peak Load Season s for the year i, calculated as follows: Year (i); Winter Peak Load; Summer Peak Load. Current Year (i=1); The simple average of the amount of ERS procured by ERCOT for the current year Standard Contract Term of October 1 to January 31 for the ERS Time Periods covering all or any part of Hour Ending 0600 and Hour Ending 1800.; The amount of ERS procured by ERCOT for the current year Standard Contract Term of June 1 through September 30 for an ERS Time Period covering all or any part of Hour Ending 1800.. Second Year (i = 2); The current year Winter Peak Load ERS amount escalated by the compound annual growth rate of the three Winter Peak Load ERS amounts preceding the current year.; The current year Summer Peak Load ERS amount escalated by the compound annual growth rate of the three Summer Peak Load ERS amounts preceding the current period.. Third Year (i = 3); The second year Winter Peak Load ERS amount escalated by the compound annual growth rate of the three Winter Peak Load ERS amounts preceding the current year.; The second year Summer Peak Load ERS amount escalated by the compound annual growth rate of the three Summer Peak Load ERS amounts preceding the current year.. Years after Third Year (i > 3); Equal to third year amount.; Equal to third year amount.. |
| CLR s, i | MW | Amount of Controllable Load Resource—Estimated amount of Controllable Load Resource that is available for Dispatch by ERCOT during the current year i for the Peak Load Season s not already included in LRRRS or LRNSRS. This value does not include Wholesale Storage Load (WSL). |
| ENERGYEFF s, i | MW | Amount of Energy Efficiency Programs Procured—Estimated amount of energy efficiency programs procured by Transmission and/or Distribution Service Providers (TDSPs) pursuant to P.U.C. Subst. R. 25.181, Energy Efficiency Goal, for the Peak Load Season s for the year i. ERCOT may also consider any energy efficiency and/or demand response initiatives reported by Non-Opt-In Entities (NOIEs). |
| i | None | Year. |
| s | None | Peak Load Season. |

- 3.2.6.2.2	Total Capacity Estimate

The total capacity estimate shall be determined based on the following equation:

TOTCAP s ,i	=	INSTCAP s, i + PUNCAP s, i + WINDCAP s, i  + HYDROCAP s, i + SOLARCAPs, i + RMRCAP s, i + DCTIECAP s, i + SWITCHCAP s, i + MOTHCAP s, i + PLANNON s, i + PLANIRR s, i – UNSWITCH s, i – RETCAP s, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| TOTCAP s, i | MW | Total Capacity—Estimated total capacity available during the Peak Load Season s for the year i. |
| INSTCAP s, i | MW | Seasonal Net Max Sustainable Rating—The Seasonal net max sustainable rating for the Peak Load Season s as reported in the approved Resource registration process for each operating Generation Resource for the year i excluding WGRs, hydro Generation Resource capacity, solar unit capacity, Resources operating under RMR Agreements, and Generation Resources capable of “switching” from the ERCOT Region to a non-ERCOT Region. |
| PUNCAP s, i | MW | Private Use Network Capacity—The Private Use Network capacities as provided to ERCOT pursuant to Section 3.10.7.3, Modeling of Private Use Networks. |
| WINDCAP s, i | MW | Effective Load Carrying Capability of WGRs—The effective Load carrying capability of all existing WGRs as determined by ERCOT for the Peak Load Season s for the year i. |
| HYDROCAPs, i | MW | Hydro Unit Capacity – The average hydro Generation Resource capacity available, as determined from the Current Operating Plan (COP), during the highest 20 peak Load hours for each preceding three year period for Peak Load Season s and year i. |
| SOLARCAPs, i | MW | Solar Unit Capacity – 100% of the nameplate capacity for operational solar units until a threshold value of 200 MWs of registered wholesale installed solar capacity is reached for Peak Load Season s and year i. Once the 200 MW threshold value is reached, the average solar unit capacity available, as determined from the COP, during the highest 20 peak Load hours for each preceding three year period for Peak Load Season s and year i. |
| RMRCAP s, i | MW | Seasonal Net Max Sustainable Rating for Generation Resource providing RMR Service—The Seasonal net max sustainable rating for the Peak Load Season s as reported in the approved Resource registration process for each Generation Resource providing RMR Service for the year i until the approved exit strategy for the RMR Resource is expected to be completed. |
| DCTIECAP s, i | MW | Seasonal Net Max Sustainable Rating for Direct Current Tie (DC Tie) Resource—The average DC Tie capacity imported into the ERCOT Region during the highest 20 peak Load hours for each preceding three year period for Peak Load Season s and year i. |
| SWITCHCAP s, i | MW | Seasonal Net Max Sustainable Rating for Switchable Generation Resource—The Seasonal net max sustainable rating for the Peak Load Season s as reported in the approved Resource asset registration process for each Generation Resource for the year i that can electrically connect (i.e., “switch”) from the ERCOT Region to another power region. |
| MOTHCAP s, i | MW | Seasonal Net Max Sustainable Rating for Mothballed Generation Resource—The Seasonal net max sustainable rating for the Peak Load Season s as reported in the approved Resource registration process for each Mothballed Generation Resource for the year i based on the lead time and probability information furnished by the owners of Mothballed Generation Resources pursuant to Section 3.14.1.9, Generation Resource Return to Service Updates. If the value furnished by the owner of a Mothballed Generation Resource pursuant to Section 3.14.1.9 is greater than or equal to 50%, then use the Seasonal net max sustainable rating for the Peak Load Season s as reported in the approved Resource registration process for the Mothballed Generation Resource for the year i. If the value furnished by the owner of a Mothballed Generation Resource pursuant to Section 3.14.1.9 is less than 50%, then exclude that Resource from the Total Capacity Estimate. |
| PLANNON s, i | MW | - New, non-Wind Generating Capacity—The amount of new, non-wind generating capacity for the Peak Load Season s and year i that: (a) has a Texas Commission on Environmental Quality (TCEQ)-approved air permit, (b) has a federal Greenhouse Gas permit, if required, (c) has obtained water rights sufficient to operate the Resource, and (d) has a signed Standard Generation Interconnect Agreement (SGIA), or a public, financially-binding agreement between the Resource owner and TSP under which generation interconnection facilities would be constructed; or for a Municipally Owned Utility (MOU) or Electric Cooperative (EC), a public commitment letter to construct a new Resource. Exclude new, non-wind generating capacity that has met the requirements of (a), (b), (c) and (d) above in which ERCOT has received written Notification from the developer that the new capacity will not proceed with construction. |
| PLANIRR s, i | MW | Effective Load Carrying Capability of New IRR Capacity—The effective Load carrying capability of new IRR capacity as determined by ERCOT for the Peak Load Season s and year i that has an SGIA or other public, financially-binding agreement between the Resource owner and TSP under which generation interconnection facilities would be constructed or, for a MOU or EC, a public commitment letter to construct a new IRR. |
| UNSWITCH s, i | MW | Capacity of Unavailable Switchable Generation Resource—The amount of capacity reported by the owners of a switchable Generation Resource that will be unavailable to ERCOT during the Peak Load Season s and year i pursuant to paragraph (2) of Section 16.5.4, Maintaining and Updating Resource Entity Information. |
| RETCAP s, i | MW | Capacity Pending Retirement—The amount of capacity in Peak Load Season s of year i that is pending retirement based on information submitted on a Notification of Suspension of Operations form (Section 22, Attachment E, Notification of Suspension of Operations) pursuant to Section 3.14.1.11, Budgeting Eligible Costs, but is under review by ERCOT pursuant to Section 3.14.1.2, ERCOT Evaluation, that has not otherwise been considered in any of the above defined categories. |
| i | None | Year. |
| s | None | Peak Load Season. |

- 21.1	Introduction

(1)	A request to make additions, edits, deletions, revisions, or clarifications to these Protocols, including any attachments and exhibits to these Protocols, is called a Nodal Protocol Revision Request (NPRR).  Except as specifically provided otherwise in the following sentence or in other sections of these Protocols, Sections 21.2, Submission of a Nodal Protocol Revision Request or System Change Request, through 21.8, Review of Guide Changes, apply to all NPRRs.  ERCOT Members, Market Participants, Public Utility Commission of Texas (PUCT) Staff, Texas Reliability Entity (Texas RE) Staff, ERCOT, and any other Entities are required to utilize the process described herein prior to requesting, through the PUCT or other Governmental Authority, that ERCOT make a change to these Protocols, except for good cause shown to the PUCT or other Governmental Authority.

(2)	A request that ERCOT change its computer systems that does not require a revision to the Protocols is called a System Change Request (SCR).  Except as specifically provided in other sections of these Protocols, Sections 21.2 through 21.7, Review of Project Prioritization and Annual Budget Process, apply to all SCRs.

(3)	The “next regularly scheduled meeting” of the Protocol Revision Subcommittee (PRS), the Technical Advisory Committee (TAC), an Assigned TAC Subcommittee (as defined below), or the ERCOT Board shall mean the next regularly scheduled meeting for which required notice can be timely given regarding the item(s) to be addressed, as specified in the appropriate ERCOT Board or committee procedures.

(4)	ERCOT may make non-substantive corrections at any time during the processing of a particular NPRR.  Under certain circumstances, however, the Nodal Protocols can also be revised by ERCOT rather than using the NPRR process outlined in Section 21.4, Nodal Protocol Revision and System Change Procedure.

(a)	This type of revision is referred to as an “Administrative NPRR” or “Administrative Changes” and shall consist of non-substantive corrections, such as typos (excluding grammatical changes), internal references (including table of contents), improper use of acronyms, and references to ERCOT Protocols, PUCT Substantive Rules, the Public Utility Regulatory Act (PURA), North American Electric Reliability Corporation (NERC) regulations, Federal Energy Regulatory Commission (FERC) rules, etc.

(b)	ERCOT shall post such Administrative NPRRs to the ERCOT website and distribute the NPRR to PRS at least ten Business Days before implementation.  If no Entity submits comments to the Administrative NPRR in accordance with paragraph (1) of Section 21.4.4, Protocol Revision Subcommittee Review and Action, ERCOT shall implement it according to paragraph (4) of Section 21.6, Nodal Protocol Revision Implementation.  If any ERCOT Member, Market Participant, PUCT Staff, Texas RE Staff or ERCOT submits comments to the Administrative NPRR, then it shall be processed in accordance with the NPRR process outlined in Section 21.4.