MEETING NOTES:  LOADS IN SCED v2 Subgroup

Jan. 29, 2014

- Group decided to move meetings to Mondays going forward, with next meeting tentatively scheduled for Feb. 24th.  Colin to work with ERCOT staff to set up dates through 2014.

- Definition of block load: A load that is incapable of moving incrementally, and therefore incapable of following base points in SCED as currently designed.  Typically, a block load is either ‘on’ or ‘off’.

  - ERS 10 & 30 serve as a useful proxy for most block loads

    - Important to recognize that in most cases, backup generation cannot be used for non-emergency DR

- Specifics of temporal constraints discussed (primarily, “recovery” or “return to service” periods)

  - Many loads can return to service within 1 hour generally but depending on the nature of the load the timeframe may be difficult to predict with 100% accuracy

    - Recovery for some manufacturing processes may depend on material qualities that are unknown prior to recovery

  - Some loads have return to service times that extend beyond 1-2 hours

- Options considered to modify SCED to accommodate block loads and loads with temporal constraints:

  - Multi-interval SCED

    - 3-6 interval SCED runs (i.e., 15- to 30-minute “look ahead”) discussed

      - Only the first interval would have a binding LMP

      - Commitment instructions (e.g., for block deployment of DR) issued for future 3-6 intervals in the optimization horizon would be binding

      - This creates the need for a make-whole payment if the forecast error for a future interval results in DR dispatch and the expected prices do not materialize.

      - One possible solution would be to require a separate pricing run (i.e., use some concepts from NPRR 444 approach).

    - More discussion needed on make-whole payment

    - Concern was expressed that loads deployed by SCED with a minimum run time longer than the multi-interval optimization horizon (e.g., 3-6 SCED intervals) could cause price reversal

    - Price reversal could also occur if the load could not return to service timely after SCED recall

    - Discussed possibility of not considering Loads in multi-interval optimization if temporal constraints exceed specified thresholds

    - It was noted that incorporating loads into SCED would result in at least some contribution to price formation; currently when loads curtail “passively” they simply reduce the price

    - Some parties opposed to multi-interval approach:  Concern that multi-interval SCED would cause ERCOT to operate more like a pool

  - “Portfolio approach” proposed by EFH

    - Allowing block loads to participate in SCED provided they are part of a portfolio that can be managed in such a way as to meet current SCED dispatch

      - Would potentially require fewer system changes

      - Method needed to communicate real-time portfolio capabilities to ERCOT

      - Concern was expressed that many larger users may not want to be managed within a portfolio

      - Participation by DR QSEs would be necessary to manage portfolios across multiple LSEs (this is a separate major topic a.k.a. “bids vs. offers.”

  - Relaxed SCED requirements similar to treatment of quick start generators

    - Could be combined with either portfolio approach or multi-interval SCED to simplify necessary changes for either approach

    - It was noted by ERCOT staff that the 10-minute “waiver” from base point deviation charges applies only to the first two intervals after dispatch; in subsequent intervals quick-starts are required to follow base points

      - Would this proposal contemplate a continuous waiver?

    - As DR increases and/or becomes more sophisticated rules would need to be revisited

    - Similar to the way ERCOT incorporated wind

- Next steps (for Feb. 24 meeting)

  - ERCOT to invite PJM to present at next meeting on incorporating block loads into their real-time commitment & dispatch process

  - ERCOT to present a report on the 2013 performance of the following inputs used in the real-time indicative pricing tool:  a) load forecast, b) net load forecast for 15 minutes (3 intervals), and c) same for 30 minutes (6 intervals)

  - EFH to present on portfolio approach + possibility for some relaxed standards