| NPRR Number | 590 | NPRR Title | Inclusion of WSL in the Default Uplift Invoice Allocation |
| --- | --- | --- | --- |
| Date |  | February 12, 2014 |  |
| Submitter’s Information |  |  |  |
| Name |  | Matthew Tozer |  |
| E-mail Address |  | mtozer@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 512-248-6332 |  |
| Cell Number |  |  |  |
| Market Segment |  | Not applicable. |  |

Comments

At its November 13, 2013 meeting, WMS requested that ERCOT report back to WMS the Maximum MWh Activity Total (MMATOT) for the month of November in order to help ERCOT Counter-Parties to better understand their exposure to a theoretical default of a Market Participant.

Currently Counter-Parties have no way of knowing the MMATOT until Default Uplift Invoices are posted.  ERCOT recognizes the value in posting a monthly MMATOT report to help Counter-Parties determine their default exposure and proposes language to provide for the posting of the MMATOT to the Market Information System (MIS) Public Area on a monthly basis as reflected in the new Section 9.19.3, Maximum MWh Total Activity Posting.

Additionally, ERCOT also suggests adding Uplift Metered Energy for Wholesale Storage Load at bus per Market Participant (UWSLTOT) to Uplift Real-Time Adjusted Metered Load per Market Participant (URTAML) in the calculation of MMA.

Revised Cover Page Language

| Nodal Protocol Section(s) Requiring Revision | 6.6.3.1, Real-Time Energy Imbalance Payment or Charge at a Resource Node 9.19.1, Default Uplift Invoices 9.19.3, Maximum MWh Total Activity Posting (new) |
| --- | --- |
| Other Binding Documents Requiring Revision or Related Revision Requests | None. |
| Revision Description | This Nodal Protocol Revision Request (NPRR) updates the Default Uplift Invoice allocation process to incorporate Wholesale Storage Load (WSL) volumes in order to equitably distribute costs to the market in case of a market shortfall; provides for monthly posting of the Maximum MWh Activity Total (MMATOT and corrects the definition for the variable MEBL. |
| Reason for Revision | Addresses current operational issues Meets strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board) Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) |

Revised Proposed Protocol Language

6.6.3.1	Real-Time Energy Imbalance Payment or Charge at a Resource Node

(1)	The payment or charge to each QSE for Energy Imbalance Service is calculated based on the Real-Time Settlement Point Price for the following amounts at a particular Resource Node Settlement Point:

(a)	The energy produced by all its Generation Resources or consumed as WSL at the Settlement Point; plus

(b)	The amount of its Self-Schedules with sink specified at the Settlement Point; plus

(c)	The amount of its Day-Ahead Market (DAM) Energy Bids cleared in the DAM at the Settlement Point; plus

(d)	The amount of its Energy Trades at the Settlement Point where the QSE is the buyer; minus

(e)	The amount of its Self-Schedules with source specified at the Settlement Point; minus

(f)	The amount of its energy offers cleared in the DAM at the Settlement Point; minus

(g)	The amount of its Energy Trades at the Settlement Point where the QSE is the seller.

(2)	The payment or charge to each QSE for Energy Imbalance Service at a Resource Node Settlement Point for a given 15-minute Settlement Interval is calculated as follows:

RTEIAMT q, p	=	(-1) * {Σgsc(Σr(GSPLITPER q, r, gsc, p * NMSAMTTOT gsc) + (ΣrWSLAMTTOT q, r, p) + RTSPP p * [(SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼)]}

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for Real-Time Energy Imbalance Service at Settlement Point p, for the 15-minute Settlement Interval. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| SSSK q, p | MW | Self-Schedule with Sink at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with sink at Settlement Point p, for the 15-minute Settlement Interval. |
| DAEP q, p | MW | Day-Ahead Energy Purchase per QSE per Settlement Point—The QSE q’s DAM Energy Bids at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. |
| RTQQEP q, p | MW | Real-Time QSE-to-QSE Energy Purchase per QSE per Settlement PointThe amount of MW bought by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| SSSR q, p | MW | Self-Schedule with Source at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with source at Settlement Point p, for the 15-minute Settlement Interval. |
| DAES q, p | MW | Day-Ahead Energy Sale per QSE per Settlement Point—The QSE q’s energy offers at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. |
| RTQQES q, p | MW | Real-Time QSE-to-QSE Energy Sale per QSE per Settlement PointThe amount of MW sold by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| NMSAMTTOT gsc | $ | Net Metering Settlement—The total payment or charge to a generation site with a net metering arrangement. |
| WSLAMTTOT q, r, p | $ | Wholesale Storage Load Settlement—The total payment or charge to QSE q, Resource r, at Settlement Point p, for WSL for each 15-minute Settlement Interval. |
| GSPLITPER q, r, gsc, p | none | Generation Resource SCADA Splitting Percentage—The generation allocation percentage for Resource r that is part of a net metering arrangement. GSPLITPER is calculated by taking the Supervisory Control and Data Acquisition (SCADA) values (GSSPLITSCA) for a particular Generation Resource r that is part of a net metering configuration and dividing by the sum of all SCADA values for all Resources that are included in the net metering configuration for each interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | A Generation Resource or an energy storage Load Resource that is located at the Facility with net metering. |
| gsc | none | A generation site code. |

[NPRR419: Replace paragraph (2) above with the following upon system implementation:] (2) The payment or charge to each QSE for Energy Imbalance Service at a Resource Node Settlement Point for a given 15-minute Settlement Interval is calculated as follows: RTEIAMT q, p = (-1) * {Σgsc(Σr(RESREV q, r, gsc, p)) + (ΣrWSLAMTTOT q, r, p) + RTSPP p * [(SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼)]} Where: RESREV q, r, gsc, p = GSPLITPER q, r, gsc, p * NMSAMTTOT gsc RESMEB q, r, gsc, p = GSPLITPER q, r, gsc, p * NMRTETOT gsc WSLTOT q, p = Σr (Σb MEBL q,r,b) RNIMBAL q, p = Σgsc(ΣrRESMEB q, r, gsc, p) + WSLTOT q, p + (SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼) The above variables are defined as follows: Variable; Unit; Description. RTEIAMT q, p; $; Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for Real-Time Energy Imbalance Service at Settlement Point p, for the 15-minute Settlement Interval.. RNIMBAL q, p; MWh; Resource Node Energy Imbalance per QSE per Settlement Point—The Resource Node volumetric imbalance for QSE q for Real-Time Energy Imbalance Service at Settlement Point p, for the 15-minute Settlement Interval.. RTSPP p; $/MWh; Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval.. SSSK q, p; MW; Self-Schedule with Sink at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with sink at Settlement Point p, for the 15-minute Settlement Interval.. DAEP q, p; MW; Day-Ahead Energy Purchase per QSE per Settlement Point—The QSE q’s DAM Energy Bids at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval.. RTQQEP q, p; MW; Real-Time QSE-to-QSE Energy Purchase per QSE per Settlement PointThe amount of MW bought by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval.. SSSR q, p; MW; Self-Schedule with Source at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with source at Settlement Point p, for the 15-minute Settlement Interval.. DAES q, p; MW; Day-Ahead Energy Sale per QSE per Settlement Point—The QSE q’s energy offers at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval.. RTQQES q, p; MW; Real-Time QSE-to-QSE Energy Sale per QSE per Settlement PointThe amount of MW sold by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval.. RESREV q, r, gsc, p; $; Resource Share Revenue Settlement Payment—The Resource share of the total payment to the entire Facility with a net metering arrangement attributed to Resource r that is part of a generation site code gsc for the QSE q at Settlement Point p.. RESMEB q, r, gsc, p; MWh; Resource Share Net Meter Real-Time Energy Total—The Resource share of the net sum for all Settlement Meters attributed to Resource r that is part of a generation site code gsc for the QSE q at Settlement Point p.. WSLTOT q, p; MWh; WSL Total—The total WSL energy metered by the Settlement Meters which measure WSL for the QSE q at Settlement Point p.. MEBL q,r,b; MWh; Metered Energy for Wholesale Storage Load at busThe WSL energy metered by the Settlement Meter which measures WSL for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b.. NMSAMTTOT gsc; $; Net Metering Settlement—The total payment or charge to a generation site with a net metering arrangement.. WSLAMTTOT q, r, p; $; Wholesale Storage Load Settlement—The total payment or charge to QSE q, Resource r, at Settlement Point p, for WSL for each 15-minute Settlement Interval.. NMRTETOT gsc; MWh; Net Meter Real-Time Energy Total—The net sum for all Settlement Meters included in generation site code gsc. A positive value indicates an injection of power to the ERCOT System.. GSPLITPER q, r, gsc, p; none; Generation Resource SCADA Splitting Percentage—The generation allocation percentage for Resource r that is part of a net metering arrangement. GSPLITPER is calculated by taking the Supervisory Control and Data Acquisition (SCADA) values (GSSPLITSCA) for a particular Generation Resource r that is part of a net metering configuration and dividing by the sum of all SCADA values for all Resources that are included in the net metering configuration for each interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. q; none; A QSE.. p; none; A Resource Node Settlement Point.. r; none; A Generation Resource or an energy storage Load Resource that is located at the Facility with net metering.. gsc; none; A generation site code.. b; none; An Electrical Bus..

(3)	For a facility with Settlement Meters that measure WSL, the total payment or charge for WSL is calculated for a QSE, energy storage Load Resource, and Settlement Point for each 15-minute Settlement Interval.

The WSL is settled as follows:

WSLAMTTOT q, r, p	= Σb (RTRMPRWSL b * MEBL q, r, b)

Where the price for Settlement Meter is determined as follows:

RTRMPRWSL b 	= (RNWFL b, y * RTLMP b, y)

Where the weighting factor for the Electrical Bus associated with the meter is:

RNWFL b, y 		= [Max (0.001,  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  TL r, y)) * TLMP y] /

[Max (0.001,  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET  TL r, y)) * TLMP y]

For every SCED LMP calculated at each Settlement Meter, there shall be an administrative price floor of -$251/MWh.

The summation is over all WSL r associated to the individual meter.  The determination of which Resources are associated to an individual meter is static and based on the normal system configuration of the generation site code, gsc.

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTLMP b, y | $/MWh | Real-Time Locational Marginal Price at bus per intervalThe Real-Time LMP for the meter at Electrical Bus b, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per intervalThe duration of the SCED interval y. |
| MEBL q,r,b | MWh | Metered Energy for Wholesale Storage Load at busThe WSL energy metered by the Settlement Meter which measures WSL for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b. |
| WSLAMTTOT q, r, p | $ | Wholesale Storage Load Settlement—The total payment or charge to QSE q, Resource r, at Settlement Point p, for WSL for each 15-minute Settlement Interval. |
| RNWFL b, y | none | Net meter Weighting Factor per interval for the Energy Metered as Wholesale Storage LoadThe weight factor used in net meter price calculation for meters in Electrical Bus b, for the SCED interval y, for the WSL associated with an energy storage Load Resource. The weighting factor used in the net meter price calculation shall not be recalculated after the fact due to revisions in the association of Resources to Settlement Meters. |
| RTRMPRWSL b | $/MWh | Real-Time Price for the Energy Metered as Wholesale Storage Load at busThe Real-Time price for the Settlement Meter which measures WSL at Electrical Bus b, for the 15-minute Settlement Interval. |
| TL r, y | MW | Telemetered WSL charging per intervalThe telemetered Load associated with the energy storage Load Resource r for the SCED interval y. |
| gsc | none | A generation site code. |
| r | none | An energy storage Load Resource. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An Electrical Bus. |

[NPRR568: Replace paragraph (3) above with the following upon system implementation:] (3) For a facility with Settlement Meters that measure WSL, the total payment or charge for WSL is calculated for a QSE, energy storage Load Resource, and Settlement Point for each 15-minute Settlement Interval. The WSL is settled as follows: WSLAMTTOT q, r, p = Σb (RTRMPRWSL b * MEBL q, r, b) Where the price for Settlement Meter is determined as follows: RTRMPRWSL b = Max [-$251, ((RNWFL b, y * RTLMP b, y) + RTRSVPOR)] Where the weighting factor for the Electrical Bus associated with the meter is: RNWFL b, y = [Max (0.001, INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET TL r, y)) * TLMP y] / [Max (0.001, INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET INCLUDEPICTURE "cid:image001.png@01CDB6B1.84F77950" \* MERGEFORMATINET TL r, y)) * TLMP y] Where: RTRSVPOR = INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET (RNWF y * RTORPA y) RNWF y= TLMP y / ΣyTLMP y The summation is over all WSL r associated to the individual meter. The determination of which Resources are associated to an individual meter is static and based on the normal system configuration of the generation site code, gsc. The above variables are defined as follows: Variable; Unit; Description. RTLMP b, y; $/MWh; Real-Time Locational Marginal Price at bus per intervalThe Real-Time LMP for the meter at Electrical Bus b, for the SCED interval y.. TLMP y; second; Duration of SCED interval per intervalThe duration of the SCED interval y.. RTRSVPOR; $/MWh; Real-Time Reserve Price for On-Line ReservesThe Real-Time Reserve Price for On-Line Reserves for the 15-minute Settlement Interval.. RTORPA y; $/MWh; Real-Time On-Line Reserve Price Adder per intervalThe Real-Time On-Line Reserve Price Adder for the SCED interval y.. RNWF y; none; Resource Node Weighting Factor per intervalThe weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval.. MEBL q,r,b; MWh; Metered Energy for Wholesale Storage Load at busThe WSL energy metered by the Settlement Meter which measures WSL for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b.. WSLAMTTOT q, r, p; $; Wholesale Storage Load Settlement—The total payment or charge to QSE q, Resource r, at Settlement Point p, for WSL for each 15-minute Settlement Interval.. RNWFL b, y; none; Net meter Weighting Factor per interval for the Energy Metered as Wholesale Storage LoadThe weight factor used in net meter price calculation for meters in Electrical Bus b, for the SCED interval y, for the WSL associated with an energy storage Load Resource. The weighting factor used in the net meter price calculation shall not be recalculated after the fact due to revisions in the association of Resources to Settlement Meters.. RTRMPRWSL b; $/MWh; Real-Time Price for the Energy Metered as Wholesale Storage Load at busThe Real-Time price for the Settlement Meter which measures WSL at Electrical Bus b, for the 15-minute Settlement Interval.. TL r, y; MW; Telemetered WSL charging per intervalThe telemetered Load associated with the energy storage Load Resource r for the SCED interval y.. gsc; none; A generation site code.. r; none; An energy storage Load Resource.. y; none; A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval.. b; none; An Electrical Bus..

(4)	The total payment or charge to a Facility with a net metering arrangement for each 15-minute Settlement Interval shall be calculated as follows:

NMRTETOT gsc 	= 	Max (0, (Σb (MEB gsc, b + MEBC gsc, b)))

If NMRTETOT gsc = 0 for a 15-minute Settlement Interval, then

The Load that is not WSL is included in the Real-Time AML per QSE and is included in the Real-Time energy imbalance payment or charge at a Load Zone.

Otherwise, when NMRTETOT gsc > 0 for a 15-minute Settlement Interval, then

NMSAMTTOT gsc	=	Σb [(RTRMPR b * MEB gsc, b) + (RTRMPR b * MEBC gsc, b)]

Where the price for Settlement Meter is determined as follows:

RTRMPR b 		=	 INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET (RNWF b, y * RTLMP b, y)

Where the weighting factor for the Electrical Bus associated with the meter is:

RNWF b, y 		= [Max (0.001, ΣrBP r, y) * TLMP y] /

[ INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET  INCLUDEPICTURE  "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET Max (0.001, ΣrBP r, y) * TLMP y]

For every SCED LMP calculated at each Settlement Meter, there shall be an administrative price floor of -$251/MWh.

The summation is over all Resources r associated to the individual meter.  The determination of which Resources are associated to an individual meter is static and based on the normal system configuration of the generation site code, gsc.

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| NMRTETOT gsc | MWh | Net Meter Real-Time Energy Total—The net sum for all Settlement Meters included in generation site code gsc. A positive value indicates an injection of power to the ERCOT System. |
| NMSAMTTOT gsc | $ | Net Metering Settlement—The total payment or charge to a generation site with a net metering arrangement. |
| RTRMPR b | $/MWh | Real-Time Price for the Energy Metered for each Resource meter at busThe Real-Time price for the Settlement Meter at Electrical Bus b, for the 15-minute Settlement Interval. |
| MEB gsc, b | MWh | Metered Energy at busThe metered energy by the Settlement Meter which is not upstream from another Settlement Meter which measures WSL for the 15-minute Settlement Interval. A positive value represents energy produced, and a negative value represents energy consumed. |
| RTLMP b, y | $/MWh | Real-Time Locational Marginal Price at bus per intervalThe Real-Time LMP for the meter at Electrical Bus b, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per intervalThe duration of the SCED interval y. |
| RNWF b, y | none | Net meter Weighting Factor per intervalThe weight factor used in net meter price calculation for meters in Electrical Bus b, for the SCED interval y. The weighting factor used in the net meter price calculation shall not be recalculated after the fact due to revisions in the association of Resources to Settlement Meters. |
| BP r, y | MW | Base Point per Resource per intervalThe Base Point of Resource r, for the SCED interval y. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MEBC gsc, b | MWh | Metered Energy at bus (Calculated)The calculated energy for the 15-minute Settlement Interval for a Settlement Meter which is upstream from another Settlement Meter which measures WSL. A positive value represents energy produced, and a negative value represents energy consumed. |
| gsc | none | A generation site code. |
| r | none | A Generation Resource that is located at the Facility with net metering. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An Electrical Bus. |

[NPRR568: Replace paragraph (4) above with the following upon system implementation:] (4) The total payment or charge to a Facility with a net metering arrangement for each 15-minute Settlement Interval shall be calculated as follows: NMRTETOT gsc = Max (0, (Σb (MEB gsc, b + MEBC gsc, b))) If NMRTETOT gsc = 0 for a 15-minute Settlement Interval, then The Load that is not WSL is included in the Real-Time AML per QSE and is included in the Real-Time energy imbalance payment or charge at a Load Zone. Otherwise, when NMRTETOT gsc > 0 for a 15-minute Settlement Interval, then NMSAMTTOT gsc = Σb [(RTRMPR b * MEB gsc, b) + (RTRMPR b * MEBC gsc, b)] Where the price for Settlement Meter is determined as follows: RTRMPR b = Max [-$251, ( INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET (RNWF b, y * RTLMP b, y) + RTRSVPOR)] Where the weighting factor for the Electrical Bus associated with the meter is: RNWF b, y = [Max (0.001, ΣrBP r, y) * TLMP y] / [ INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET Max (0.001, ΣrBP r, y) * TLMP y] Where: RTRSVPOR = INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET INCLUDEPICTURE "cid:image010.gif@01C7362C.D715A790" \* MERGEFORMATINET (RNWF y * RTORPA y) RNWF y = TLMP y / ΣyTLMP y The summation is over all Resources r associated to the individual meter. The determination of which Resources are associated to an individual meter is static and based on the normal system configuration of the generation site code, gsc. The above variables are defined as follows: Variable; Unit; Description. NMRTETOT gsc; MWh; Net Meter Real-Time Energy Total—The net sum for all Settlement Meters included in generation site code gsc. A positive value indicates an injection of power to the ERCOT System.. NMSAMTTOT gsc; $; Net Metering Settlement—The total payment or charge to a generation site with a net metering arrangement.. RTRMPR b; $/MWh; Real-Time Price for the Energy Metered for each Resource meter at busThe Real-Time price for the Settlement Meter at Electrical Bus b, for the 15-minute Settlement Interval.. MEB gsc, b; MWh; Metered Energy at busThe metered energy by the Settlement Meter which is not upstream from another Settlement Meter which measures WSL for the 15-minute Settlement Interval. A positive value represents energy produced, and a negative value represents energy consumed.. RTRSVPOR; $/MWh; Real-Time Reserve Price for On-Line ReservesThe Real-Time Reserve Price for On-Line Reserves for the 15-minute Settlement Interval.. RTORPA y; $/MWh; Real-Time On-Line Reserve Price Adder per intervalThe Real-Time On-Line Reserve Price Adder for the SCED interval y.. RNWF y; none; Resource Node Weighting Factor per intervalThe weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval.. RTLMP b, y; $/MWh; Real-Time Locational Marginal Price at bus per intervalThe Real-Time LMP for the meter at Electrical Bus b, for the SCED interval y.. TLMP y; second; Duration of SCED interval per intervalThe duration of the SCED interval y.. RNWF b, y; none; Net meter Weighting Factor per intervalThe weight factor used in net meter price calculation for meters in Electrical Bus b, for the SCED interval y. The weighting factor used in the net meter price calculation shall not be recalculated after the fact due to revisions in the association of Resources to Settlement Meters.. BP r, y; MW; Base Point per Resource per intervalThe Base Point of Resource r, for the SCED interval y. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train.. MEBC gsc, b; MWh; Metered Energy at bus (Calculated)The calculated energy for the 15-minute Settlement Interval for a Settlement Meter which is upstream from another Settlement Meter which measures WSL. A positive value represents energy produced, and a negative value represents energy consumed.. gsc; none; A generation site code.. r; none; A Generation Resource that is located at the Facility with net metering.. y; none; A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval.. b; none; An Electrical Bus..

(5)	The Generation Resource SCADA Splitting Percentage for each Resource within a net metering arrangement for the 15-minute Settlement Interval is calculated as follows:

GSPLITPER q,  r, gsc, p 	= GSSPLITSCA r / ΣrGSSPLITSCA r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| GSPLITPER q, r, gsc, p | none | Generation Resource SCADA Splitting Percentage—The generation allocation percentage for Resource r that is part of a generation site code gsc for the QSE q at Settlement Point p. GSPLITPER is calculated by taking the SCADA values (GSSPLITSCA) for a particular Generation Resource r that is part of a net metering configuration and dividing by the sum of all SCADA values for all Resources that are included in the net metering configuration for each interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| GSSPLITSCA r | MWh | Generation Resource SCADA Net Real Power provided via Telemetry—The net real power provided via telemetry per Resource within the net metering arrangement, integrated for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| gsc | none | A generation site code. |
| r | none | A Generation Resource that is located at the Facility with net metering. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |

(6)	The total net payments and charges to each QSE for Energy Imbalance Service at all Resource Node Settlement Points for the 15-minute Settlement Interval is calculated as follows:

RTEIAMTQSETOT q	=	Σp RTEIAMT q, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTEIAMTQSETOT q | $ | Real-Time Energy Imbalance Amount QSE Total per QSEThe total net payments and charges to QSE q for Real-Time Energy Imbalance Service at all Resource Node Settlement Points for the 15-minute Settlement Interval. |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for Real-Time Energy Imbalance Service at Settlement Point p, for the 15-minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |

9.19.1	Default Uplift Invoices

(1)	ERCOT shall collect the total short-pay amount for all Settlement Invoices for a month, less the total payments expected from a payment plan, from Qualified Scheduling Entities (QSEs) and CRR Account Holders.  ERCOT must pay the funds it collects from payments on Default Uplift Invoices to the Entities previously short-paid.  ERCOT shall notify those Entities of the details of the payment.

(2)	Each Counter-Party’s share of the uplift is calculated using True-Up Settlement data for each Operating Day in the month prior to the month in which the default occurred, and is calculated as follows:

DURSCPcp = TSPA * MMARScp

Where:

MMARS cp = MMA cp / MMATOT

MMA cp = Max { ∑mp (URTMG mp + URTDCIMP mp),

∑mp (URTAML mp + UWSLTOT mp),

∑mp URTQQES mp,

∑mp URTQQEP mp,

∑mp UDAES mp,

∑mp UDAEP mp,

∑mp (URTOBL mp + URTOBLLO mp),

∑mp (UDAOPT mp + UDAOBL mp + UOPTS mp + UOBLS mp),

∑mp (UOPTP mp + UOBLP mp)}

MMATOT = ∑cp (MMAcp)

Where:

URTMG mp = ∑p, r, i (RTMG mp, p, r, i), excluding RTMG for RMR Resources and RTMG in Reliability Unit Commitment (RUC)-Committed Intervals for RUC-committed Resources

URTDCIMP mp = ∑p, i (RTDCIMP mp, p, i) / 4

URTAML mp = ∑p, i (RTAML mp, p, i)

URTQQES mp = ∑p, i (RTQQES mp, p, i) / 4

URTQQEP mp = ∑p, i (RTQQEP mp, p, i) / 4

UDAES mp = ∑p, h (DAES mp, p, h)

UDAEP mp = ∑p, h (DAEP mp, p, h)

URTOBL mp = ∑(j, k), h (RTOBL mp, (j, k), h)

URTOBLLO mp = ∑(j, k), h (RTOBLLO mp, (j, k), h)

UDAOPT mp = ∑(j, k), h (DAOPT mp, (j, k), h)

UDAOBL mp = ∑(j, k), h (DAOBL mp, (j, k), h)

UOPTS mp = ∑(j, k), h (OPTS mp, (j, k), h)

UOBLS mp = ∑(j, k), h (OBLS mp, (j, k), h)

UOPTP mp = ∑(j, k), h (OPTP mp, j, h)

UOBLP mp = ∑(j, k), h (OBLP mp, (j, k), h)

UWSLTOT mp = (-1) * ∑,r,b (MEBLmp,r,b)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DURSCPcp | $ | Default Uplift Ratio Share per Counter-Party—The Counter-Party’s pro rata portion of the total short-pay amount for all Day-Ahead Market (DAM) and Real-Time Market (RTM) Invoices for a month. |
| TSPA | $ | Total Short Pay Amount—The total short-pay amount calculated by ERCOT to be collected through the Default Uplift Invoice process. |
| MMARScp | None | Maximum MWh Activity Ratio Share—The Counter-Party’s pro rata share of Maximum MWh Activity. |
| MMAcp | MWh | Maximum MWh Activity—The maximum MWh activity of all Market Participants represented by the Counter-Party in the DAM, RTM and CRR Auction for a month. |
| MMATOT | MWh | Maximum MWh Activity Total—The sum of all Counter-Party’s Maximum MWh Activity. |
| RTMG mp, p, r, i | MWh | Real-Time Metered Generation per Market Participant per Settlement Point per Resource—The Real-Time energy produced by the Generation Resource r represented by Market Participant mp, at Resource Node p, for the 15-minute Settlement Interval i, where the Market Participant is a QSE. |
| URTMG mp | MWh | Uplift Real-Time Metered Generation per Market Participant—The monthly sum of Real-Time energy produced by Generation Resources represented by Market Participant mp, excluding generation for RMR Resources and generation in RUC-Committed Intervals, where the Market Participant is a QSE assigned to the registered Counter-Party. |
| RTDCIMP mp, p, i | MW | Real-Time DC Import per QSE per Settlement Point—The aggregated Direct Current Tie (DC Tie) Schedule submitted by Market Participant mp, as an importer into the ERCOT System through DC Tie p, for the 15-minute Settlement Interval i, where the Market Participant is a QSE. |
| URTDCIMP mp | MW | Uplift Real-Time DC Import per Market Participant—The monthly sum of the aggregated DC Tie Schedule submitted by Market Participant mp, as an importer into the ERCOT System where the Market Participant is a QSE assigned to a registered Counter-Party. |
| RTAML mp, p, i | MWh | Real-Time Adjusted Metered Load per Market Participant per Settlement Point—The sum of the Adjusted Metered Load (AML) at the Electrical Buses that are included in Settlement Point p represented by Market Participant mp for the 15-minute Settlement Interval i, where the Market Participant is a QSE. |
| URTAML mp | MWh | Uplift Real-Time Adjusted Metered Load per Market Participant—The monthly sum of the AML represented by Market Participant mp, where the Market Participant is a QSE assigned to the registered Counter-Party. |
| RTQQES mp, p, i | MW | QSE-to-QSE Energy Sale per Market Participant per Settlement Point—The amount of MW sold by Market Participant mp through Energy Trades at Settlement Point p for the 15-minute Settlement Interval i, where the Market Participant is a QSE. |
| URTQQES mp | MWh | Uplift QSE-to-QSE Energy Sale per Market Participant—The monthly sum of MW sold by Market Participant mp through Energy Trades, where the Market Participant is a QSE assigned to the registered Counter-Party. |
| RTQQEP mp, p, i | MW | QSE-to-QSE Energy Purchase per Market Participant per Settlement Point—The amount of MW bought by Market Participant mp through Energy Trades at Settlement Point p for the 15-minute Settlement Interval i, where the Market Participant is a QSE. |
| URTQQEP mp | MWh | Uplift QSE-to-QSE Energy Purchase per Market Participant—The monthly sum of MW bought by Market Participant mp through Energy Trades, where the Market Participant is a QSE assigned to the registered Counter-Party. |
| DAES mp, p, h | MW | Day-Ahead Energy Sale per Market Participant per Settlement Point per hour—The total amount of energy represented by Market Participant mp’s cleared Three-Part Supply Offers in the DAM and cleared DAM Energy-Only Offers at Settlement Point p, excluding the offers submitted for RMR Units at the same Settlement Point, for the hour h, where the Market Participant is a QSE. |
| UDAES mp | MWh | Uplift Day-Ahead Energy Sale per Market Participant—The monthly total of energy represented by Market Participant mp’s cleared Three-Part Supply Offers in the DAM and cleared DAM Energy-Only Offer Curves, where the Market Participant is a QSE assigned to the registered Counter-Party. |
| DAEP mp, p, h | MW | Day-Ahead Energy Purchase per Market Participant per Settlement Point per hour—The total amount of energy represented by Market Participant mp’s cleared DAM Energy Bids at Settlement Point p for the hour h, where the Market Participant is a QSE. |
| UDAEP mp | MWh | Uplift Day-Ahead Energy Purchase per Market Participant—The monthly total of energy represented by Market Participant mp’s cleared DAM Energy Bids, where the Market Participant is a QSE assigned to the registered Counter-Party. |
| RTOBL mp, (j, k), h | MW | Real-Time Obligation per Market Participant per source and sink pair per hour—The number of Market Participant mp’s Point-to-Point (PTP) Obligations with the source j and the sink k settled in Real-Time for the hour h, and where the Market Participant is a QSE. |
| URTOBL mp | MWh | Uplift Real-Time Obligation per Market Participant—The monthly total of Market Participant mp’s PTP Obligations settled in Real-Time, counting the quantity only once per source and sink pair, and where the Market Participant is a QSE assigned to the registered Counter-Party. |
| RTOBLLO q, (j, k) | MW | Real-Time Obligation with Links to an Option per QSE per pair of source and sinkThe total MW of the QSE’s PTP Obligation with Links to an Option Bids cleared in the DAM and settled in Real-Time for the source j and the sink k for the hour. |
| URTOBLLO q, (j, k) | MW | Uplift Real-Time Obligation with Links to an Option per QSE per pair of source and sinkThe monthly total of Market Participant mp’s MW of PTP Obligation with Links to Options Bids cleared in the DAM and settled in Real-Time for the source j and the sink k for the hour, where the Market Participant is a QSE assigned to the registered Counter-Party. |
| DAOPT mp, (j, k), h | MW | Day-Ahead Option per Market Participant per source and sink pair per hourThe number of Market Participant mp’s PTP Options with the source j and the sink k owned in the DAM for the hour h, and where the Market Participant is a CRR Account Holder. |
| UDAOPT mp | MWh | Uplift Day-Ahead Option per Market ParticipantThe monthly total of Market Participant mp’s PTP Options owned in the DAM, counting the ownership quantity only once per source and sink pair, and where the Market Participant is a CRR Account Holder assigned to the registered Counter-Party. |
| DAOBL mp, (j, k), h | MW | Day-Ahead Obligation per Market Participant per source and sink pair per hour—The number of Market Participant mp’s PTP Obligations with the source j and the sink k owned in the DAM for the hour h, and where the Market Participant is a CRR Account Holder. |
| UDAOBL mp | MWh | Uplift Day-Ahead Obligation per Market ParticipantThe monthly total of Market Participant mp’s PTP Obligations owned in the DAM, counting the ownership quantity only once per source and sink pair, where the Market Participant is a CRR Account Holder assigned to the registered Counter-Party. |
| OPTS mp, (j, k), a, h | MW | PTP Option Sale per Market Participant per source and sink pair per CRR Auction per hour—The MW quantity that represents the total of Market Participant mp’s PTP Option offers with the source j and the sink k awarded in CRR Auction a, for the hour h, where the Market Participant is a CRR Account Holder. |
| UOPTS mp | MWh | Uplift PTP Option Sale per Market Participant—The MW quantity that represents the monthly total of Market Participant mp’s PTP Option offers awarded in CRR Auctions, counting the awarded quantity only once per source and sink pair, where the Market Participant is a CRR Account Holder assigned to the registered Counter-Party. |
| OBLS mp, (j, k), a, h | MW | PTP Obligation Sale per Market Participant per source and sink pair per CRR Auction per hour—The MW quantity that represents the total of Market Participant mp’s PTP Obligation offers with the source j and the sink k awarded in CRR Auction a, for the hour h, where the Market Participant is a CRR Account Holder. |
| UOBLS mp | MWh | Uplift PTP Obligation Sale per Market Participant—The MW quantity that represents the monthly total of Market Participant mp’s PTP Obligation offers awarded in CRR Auctions, counting the quantity only once per source and sink pair, where the Market Participant is a CRR Account Holder assigned to the registered Counter-Party. |
| OPTP mp, (j, k), a, h | MW | PTP Option Purchase per Market Participant per source and sink pair per CRR Auction per hour—The MW quantity that represents the total of Market Participant mp’s PTP Option bids with the source j and the sink k awarded in CRR Auction a, for the hour h, where the Market Participant is a CRR Account Holder. |
| UOPTP mp | MWh | Uplift PTP Option Purchase per Market Participant—The MW quantity that represents the monthly total of Market Participant mp’s PTP Option bids awarded in CRR Auctions, counting the quantity only once per source and sink pair, where the Market Participant is a CRR Account Holder assigned to the registered Counter-Party. |
| OBLP mp, (j, k), a, h | MW | PTP Obligation Purchase per Market Participant per source and sink pair per CRR Auction per hour—The MW quantity that represents the total of Market Participant mp’s PTP Obligation bids with the source j and the sink k awarded in CRR Auction a, for the hour h, where the Market Participant is a CRR Account Holder. |
| UOBLP mp | MWh | Uplift PTP Obligation Purchase per Market Participant—The MW quantity that represents the monthly total of Market Participant mp’s PTP Obligation bids awarded in CRR Auctions, counting the quantity only once per source and sink pair, where the Market Participant is a CRR Account Holder assigned to the registered Counter-Party. |
| UWSLTOT mp | MWh | Uplift Metered Energy for Wholesale Storage Load at bus per Market ParticipantThe monthly sum of Market Participant mp’s Wholesale Storage Load (WSL) energy metered by the Settlement Meter which measures WSL. |
| MEBL mp,r,b | MWh | Metered Energy for Wholesale Storage Load at busThe WSL energy metered by the Settlement Meter which measures WSL for the 15-minute Settlement Interval represented as a negative value, for the Market Participant mp, Resource r, at bus b. |
| cp | none | A registered Counter-Party. |
| mp | none | A Market Participant that is a non-defaulting QSE or CRR Account Holder. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| a | none | A CRR Auction. |
| p | none | A Settlement Point. |
| i | none | A 15-minute Settlement Interval. |
| h | none | The hour that includes the Settlement Interval i. |
| r | none | A Resource. |

(3)	The uplifted short-paid amount will be allocated to the Market Participants (QSEs or CRR Account Holders) assigned to a registered Counter-Party based on the pro-rata share of MWhs that the QSE or CRR Account Holder contributed to its Counter-Party’s maximum MWh activity ratio share.

(4)	Any uplifted short-paid amount greater than $2,500,000 must be scheduled so that no amount greater than $2,500,000 is charged on each set of Default Uplift Invoices until ERCOT uplifts the total short-paid amount.  ERCOT must issue Default Uplift Invoices at least 30 days apart from each other.

(5)	ERCOT shall issue Default Uplift Invoices no earlier than 180 days following a short-pay of a Settlement Invoice on the date specified in the Settlement Calendar.  The Invoice Recipient is responsible for accessing the Invoice on the MIS Certified Area once posted by ERCOT.

(6)	Each Default Uplift Invoice must contain:

(a)	The Invoice Recipient’s name;

(b)	The ERCOT identifier (Settlement identification number issued by ERCOT);

(c)	Net Amount Due or Payable – the aggregate summary of all charges owed by a Default Uplift Invoice Recipient;

(d)	Run Date – the date on which ERCOT created and published the Default Uplift Invoice;

(e)	Invoice Reference Number – a unique number generated by the ERCOT applications for payment tracking purposes;

(f)	Default Uplift Invoice Reference – an identification code used to reference the amount uplifted;

(g)	Payment Date and Time – the date and time that Default Uplift Invoice amounts must be paid;

(h)	Remittance Information Details – details including the account number, bank name, and electronic transfer instructions of the ERCOT account to which any amounts owed by the Invoice Recipient are to be paid or of the Invoice Recipient’s account from which ERCOT may draw payments due; and

(i)	Overdue Terms – the terms that would apply if the Market Participant makes a late payment.

(7)	Each Invoice Recipient shall pay any net debit shown on the Default Uplift Invoice on the payment due date whether or not there is any Settlement and billing dispute regarding the amount of the debit.

9.19.3	Maximum MWh Total Activity Posting

On the 15th day of each month, ERCOT shall post on the MIS Public Area, the Maximum MWh Activity Total (MMATOT) value calculated using data from the previous month.  If the 15th day of the month is a weekend or ERCOT holiday, ERCOT shall post the MMATOT on the first Business Day following the 15th day of the month.  The posting shall include:

(a)	The MMATOT as defined in paragraph (2) of Section 9.19.1, Default Uplift Invoices, with the exception that the calculation shall use Initial Settlement data for each Operating Day in the month prior to the month of the posting.

(b)	The month of the data used in the calculation of the MMATOT.