| NPRR Number | 560 | NPRR Title | Caps and Floors for Energy Storage Resources |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Tabled |
| Date of Decision |  | October 17, 2013 |  |  |
| Proposed Effective Date |  | To be determined. |  |  |
| Priority and Rank Assigned |  | To be determined. |  |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 4.4.9.2.3, Startup Offer and Minimum Energy Offer Generic Caps 4.4.9.3.3, Energy Offer Curve Caps for Make-Whole Calculation Purposes 4.4.9.4.1, Mitigated Offer Cap 4.4.9.4.2, Mitigated Offer Floor 5.6.1, Verifiable Costs 7.9.1.3, Minimum and Maximum Resource Prices |  |  |
| Market Guide Section(s) Requiring Revision |  | None. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) creates a definition of Energy Storage Resource and specifies values for the various caps and floors used in the mitigation and make-whole calculation processes for such Resources. |  |  |
| Reason for Revision |  | The Public Utility Commission of Texas (PUCT) rule to implement Senate Bill 943 (Act of the 82nd Texas Legislature), which specifies that Energy Storage Resources should be registered as Generation Resources (see Final Order, PUCT Project No. 39657, Rulemaking to Implement SB 943, Relating to Electric Energy Storage Equipment or Facilities) when offering into the competitive energy market. All Generation Resources are required to have these caps and floors specified for each Resource type before a Resource can offer into the ERCOT market. The Protocols do not currently specify these values for Energy Storage Resources. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR560 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 7/18/13, NPRR560 was posted. - On 8/22/13, PRS considered NPRR560. - On 9/12/13, WMS comments were posted. - On 9/19/13, PRS again considered NPRR560. - On 10/15/13, ERCOT comments were posted. - On 10/16/13, a second set of ERCOT comments were posted. - On 10/17/13, PRS considered the 9/19/13 PRS Report for NPRR560. |  |  |
| PRS Decision |  | On 8/22/13, PRS voted to table NPRR560 and refer the issue to WMS. There was one abstention from the Consumer Market Segment. All Market Segments were present for the vote. On 9/19/13, PRS voted to recommend approval of NPRR560 as revised by PRS. There was one opposing vote from the Consumer Market Segment. All Market Segments were present for the vote. On 10/17/13, PRS unanimously voted to table NPRR560. |  |  |
| Summary of PRS Discussion |  | On 8/22/13, it was noted that WMS voted to authorize the Emerging Technologies Working Group (ETWG) to submit NPRR560 but has not yet taken action on the merits of the NPRR. On 9/19/13, participants discussed whether Energy Storage Resources should be allowed to submit offers that exceed the applicable Resource Category Minimum-Energy Generic Cap, and whether there is a better alternative method to account for the fact that the primary fuel for Energy Storage Resources is electricity, which is subject to highly volatile prices, such as a dispute mechanism. Some participants recommended reviewing the offer cap exception in the future after the market has more practical experience with Energy Storage Resources. It was agreed that it was not necessary to state in the Protocols that the Independent Market Monitor (IMM) may review Energy Storage Resource offers that exceed the cap, since it is already in the IMM’s purview to do so. On 10/17/13, ERCOT Staff requested that NPRR560 be tabled to allow additional time for completion of an Impact Analysis. |  |  |

| Business Case |  |
| --- | --- |
| 1 | Energy Storage Resources are currently undefined in the Protocols. |
| 2 | Compressed air energy storage and battery Resources are currently completing the ERCOT Resource asset registration process. Defining the various caps and floors for each of these Resource technology types is a prerequisite for this process and their market participation. |
| 3 | Energy Storage Resources are different than conventional Load Resources and Generation Resources. Whether they withdraw or inject energy depends upon other factors in addition to price, including the unit’s state of charge, duration of stored energy, etc. The relevant caps and floors for these technology types should recognize their unique operating characteristics and opportunity costs, allowing them the flexibility to maximize their value to the system. |

| Sponsor |  |
| --- | --- |
| Name | Mark Bruce on behalf of ETWG |
| E-mail Address | mark.bruce@stratusenergy.com |
| Company | Stratus Energy Group |
| Phone Number |  |
| Cell Number | 512-810-1516 |
| Market Segment | Not applicable. |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jonathan.levine@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| WMS 091213 | Endorsed NPRR560 as submitted. |
| ERCOT 101513 | Proposed revisions to clarify that all hours of the most recent Day-Ahead Market (DAM) will be used to calculate the daily average price corresponding to the Wholesale Storage Load (WSL) Resource Node Settlement Point Price. |
| ERCOT 101613 | Proposed an alternative schedule for completion of an Impact Analysis. |

Comments

Please note that the baseline Protocol language in Section 5.6.1 has been updated to reflect the August 1, 2013 incorporation of the following NPRRs into the Protocols:

- NPRR513, Removal of Nodal Implementation Surcharge

- NPRR516, Change to Verifiable Cost Manual Revision Process

Proposed Protocol Language Revision

2.1	DEFINITIONS

Resource

The term is used to refer to both a Generation Resource and a Load Resource.  The term “Resource” used by itself in these Protocols does not include a Non-Modeled Generator or an ERS Resource.

...

Energy Storage Resource

A Resource that is capable of storing energy using Wholesale Storage Load (WSL) and releasing that energy at a later time to generate electric energy.

4.4.9.2.3	Startup Offer and Minimum-Energy Offer Generic Caps

(1)	The Resource Category Startup Offer Generic Cap, by applicable Resource category, is determined by the following Operations and Maintenance (O&M) costs by Resource category:

| Resource Category | O&M Costs ($) |
| --- | --- |
| Nuclear, coal, lignite and hydro | 7,200 |
| Combined Cycle Generation Resource with a combustion turbine ≥ 90 MW, as determined by the largest combustion turbine in the Combined Cycle Generation Resource and for each combustion turbine in the Combined Cycle Generation Resource | 6,810 |
| Combined Cycle Generation Resource with a combustion turbine < 90 MW, as determined by the largest combustion turbine in the Combined Cycle Generation Resource and for each combustion turbine in the Combined Cycle Generation Resource | 6,810 |
| Gas steam supercritical boiler | 4,800 |
| Gas steam reheat boiler | 3,000 |
| Gas steam non-reheat or boiler w/o air-preheater | 2,310 |
| Simple cycle greater than 90 MW | 5,000 |
| Simple cycle less than or equal to 90 MW | 2,300 |
| Reciprocating Engines | $58/MW * the average of the seasonal net max sustainable ratings |
| Compressed air Energy Storage Resource | 5,000 |
| RMR Resource | Not Applicable |
| Wind generation Resources | 0 |
| All other Energy Storage Resources | 0 |
| Any Resources not defined above | 0 |

(2)	The Resource Category Minimum-Energy Generic Cap is the cost per MWh of energy for a Resource to produce energy at the Resource’s LSL and is as follows:

(a)	Hydro = $10.00/MWh;

- (b)	Coal and lignite = $18.00/MWh;

- (c)	Combined-cycle greater than 90 MW = 8 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (d)	Combined-cycle less than or equal to 90 MW = 9 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (e)	Gas steam supercritical boiler = 14 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (f)	Gas steam reheat boiler = 14.5 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (g)	Gas steam non-reheat or boiler without air-preheater = 16.0 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (h)	Simple-cycle greater than 90 MW = 15.0 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (i)	Simple-cycle less than or equal to 90 MW = 14.0 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (j)	Reciprocating engines = 16.0 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Minimum-Energy Offer;

- (k)	Energy Storage Resource = a1 * (Previous DAM 24-hour average of corresponding Wholesale Storage Load (WSL) Resource Node Settlement Point Price) + b * FIP + c, where for:

- (i)	Natural gas driven compressed air Energy Storage Resources: a1 = 1, b = 6 MMBTU/MWh, c = $15/MWh;

- (ii)	For other compressed air Energy Storage Resources: a1 = 1.25, b = 0, c = $35/MWh; and

- (iii)	For all other Energy Storage Resources: a1 = 1.25, b = 0, c = $35/MWh;

- The Resource Category Minimum-Energy Generic Cap for an Energy Storage Resource represents a threshold limit.  An Energy Storage Resource may submit offers that exceed this limit;

- (l)	RMR Resource = RMR contract estimated fuel cost using its contract I/O curve at its LSL times FIP;

- (m)	Nuclear = Not Applicable;

- (n)	Wind generation Resources = $0; and

- (o)	Other Resources not defined above = $0.

- (3)	The FIP and FOP used to calculate the Resource Category Minimum-Energy Generic Cap shall be the FIP or FOP for the Operating Day.  In the event the Resource Category Minimum-Energy Generic Cap must be calculated before the FIP or FOP is available for the particular Operating Day, the FIP and FOP for the most recent preceding Operating Day shall be used.  Once the FIP and FOP are available for a particular Operating Day, those values shall be used in the calculations.  If the percentage fuel mix is not specified for Resource categories having the option to specify the fuel mix, then the minimum of FIP or FOP shall be used.

(4)	Items (2)(c) and (2)(d) above are determined by capacity of largest simple-cycle combustion turbine in the train.

4.4.9.3.3	Energy Offer Curve Caps for Make-Whole Calculation Purposes

(1)	The following Energy Offer Curve Caps must be used for the purpose of make-whole Settlements:

- (a)	Nuclear = $15.00/MWh;

- (b)	Coal and Lignite = $18.00/MWh;

- (c)	Combined Cycle greater than 90 MW = 9 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (d)	Combined Cycle less than or equal to 90 MW = 10 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (e)	Gas - Steam Supercritical Boiler = 10.5 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (f)	Gas Steam Reheat Boiler = 11.5 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (g)	Gas Steam Non-reheat or boiler without air-preheater = 14.5 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (h)	Simple Cycle greater than 90 MW = 14 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (i)	Simple Cycle less than or equal to 90 MW = 15 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (j)	Reciprocating Engines = 16 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (k)	Energy Storage Resource = Minimum-Energy Offer submitted by the Energy Storage Resource and, in the absence of any Minimum-Energy Offer, a2 * (Previous DAM 24-hour average of corresponding WSL Resource Node Settlement Point Price) + b * FIP + c, where for:

- (i)	Natural gas driven compressed air Energy Storage Resources: a2 = 1.5, b = 6 MMBTU/MWh, c = $15/MWh;

- (ii)	For other compressed air Energy Storage Resources: a2 = 1.75, b = 0, c = $35/MWh; and

- (iii)	For all other Energy Storage Resources: a2 = 1. 75, b = 0, c = $35/MWh;

- (l)	Hydro = $10.00/MWh;

- (m)	Other Renewable = $0/MWh; and

- (n)	RMR Resource = RMR contract price Energy Offer Curve.

(2)	Items in paragraphs (1)(c) and (d) above are determined by capacity of largest simple-cycle combustion turbine in the train selected.

(3)	The FIP and FOP used to calculate the Energy Offer Curve Cap for Make-Whole Payment calculation purposes shall be the FIP or FOP for the Operating Day.  In the event the Energy Offer Curve Cap for Make-Whole Payment calculation purposes must be calculated before the FIP or FOP is available for the particular Operating Day, the FIP and FOP for the most recent preceding Operating Day shall be used.  Once the FIP and FOP are available for a particular Operating Day, those values shall be used in the calculations.  If the percentage fuel mix is not specified or if no Energy Offer Curve exists, then the minimum of FIP or FOP shall be used.

4.4.9.4.1	Mitigated Offer Cap

Energy Offer Curves may be subject to mitigation in Real-Time operations under Section 6.5.7.3, Security Constrained Economic Dispatch, using a Mitigated Offer Cap.  The “Mitigated Offer Cap” is:

(a)	For a Resource contracted by ERCOT under paragraph (2) of Section 6.5.1.1, ERCOT Control Area Authority, ERCOT shall increase the O&M cost such that every point on the Mitigated Offer Cap curve (cap vs. output level) is greater than the SWCAP in $/MWh.

- (b)	For a Generation Resource that commences commercial operation after January 1, 2004, ERCOT shall construct an incremental Mitigated Offer Cap curve (Section 6.5.7.3) such that each point on the Mitigated Offer Cap curve (cap vs. output level) is the greater of:

(i)	14.5 MMBtu/MWh times the FIP; or

(ii)	The Resource’s verifiable incremental heat rate (MMBtu/MWh) plus consideration of a fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual for the output level multiplied by ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (e) below; or

(iii)	For an Energy Storage Resource, a3 * (Previous DAM 24-hour average of corresponding WSL Resource Node Settlement Point Price) + b * FIP + c, where for:

(A)	Natural gas driven compressed air Energy Storage Resources: a3 = 1.5, b = 6 MMBTU/MWh, c = $15/MWh;

(B)	For other compressed air Energy Storage Resources: a3 = 1.75, b = 0, c = $35/MWh; and

(C)	For all other Energy Storage Resources: a3 = 1.75, b = 0, c = $35/MWh.

- (c)	For all other Generation Resources, each point on the Mitigated Offer Cap curve (cap vs. output level) is the greater of:

(i)	10.5 MMBtu/MWh times the FIP; or

(ii)	The Resource’s verifiable incremental heat rate (MMBtu/MWh) plus consideration of a fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual for the output level multiplied by ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (e) below.

(d)	Notwithstanding paragraphs (b)(ii), (b)(iii), and (c)(ii) above, the Mitigated Offer Cap verifiable variable O&M cost ($/MWh) for Quick Start Generation Resources (QSGRs) shall incorporate the generic or verifiable O&M cost to start the Resource from first fire to LSL including the startup fuel, plus a minimum energy component to account for LSL commitment costs, and consideration of a fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual.

- (e)	The multipliers for paragraphs (b)(ii) and (c)(ii) above are as follows:

(i)	1.10 for Resources running at a ≥ 50% capacity factor for the previous 12 months;

(ii)	1.15 for Resources running at a ≥ 30 and < 50% capacity factor for the previous 12 months;

(iii)	1.20 for Resources running at a ≥ 20 and < 30% capacity factor for the previous 12 months;

(iv)	1.25 for Resources running at a ≥ 10 and < 20% capacity factor for the previous 12 months;

(v)	1.30 for Resources running at a ≥ 5 and < 10% capacity factor for the previous 12 months;

(vi)	1.40 for Resources running at a ≥ 1 and < 5% capacity factor for the previous 12 months; and

(vii)	1.50 for Resources running at a less than 1% capacity factor for the previous 12 months.

- (f)	The previous 12 months’ capacity factor must be updated by ERCOT by the 20th day of each month using the most recent data for use in the next month.  ERCOT shall post to the MIS Secure Area the capacity factor for each Resource before the start of the effective month.

- (g)	The process for developing the Mitigated Offer Cap in paragraphs (a), (b), (c), and (d) above must be described by ERCOT in a procedure approved by the appropriate Technical Advisory Committee (TAC) subcommittee, and posted to the MIS Secure Area within one Business Day after initial approval, and after each approved change.

4.4.9.4.2	Mitigated Offer Floor

Energy Offer Curves may be subject to mitigation in the Real-Time market under Section 6.5.7.3, Security Constrained Economic Dispatch, using a Mitigated Offer Floor.  The “Mitigated Offer Floor” is:

| Resource Category | Mitigated Offer Floor |
| --- | --- |
| Nuclear, Energy Storage Resource, and Hydro | -$250/MWh |
| Coal and Lignite | -$20/MWh |
| Combined Cycle | 1 MMBtu/MWh * FIP |
| Gas/Oil Steam and Combustion Turbine | 6 MMBtu/MWh * FIP or FOP, as specified in the Energy Offer Curve |
| Qualifying Facility (QF) | -$ 50/MWh |
| Wind | -$100/MWh |
| Other Renewables | -$ 50/MWh |

5.6.1	Verifiable Costs

(1)	The Qualified Scheduling Entity (QSE) is responsible for submitting verifiable costs unless both the QSE and Resource Entity agree that the Resource Entity will have this responsibility, in which case both the QSE and Resource Entity shall submit an affidavit to ERCOT stating this arrangement.  Notwithstanding the foregoing, QSEs that submit Power Purchase or Tolling Agreements (PPAs) do not have the option of allowing Resource Entities to file verifiable costs.

(2)	Make-Whole Payments for a Resource are based on the Startup Offers and Minimum-Energy Offers for the Resource, limited by caps.  Until ERCOT approves verifiable unit-specific costs for that Resource, the caps are the Resource Category Startup Generic Cap and the Resource Category Minimum-Energy Generic Cap.  When ERCOT approves verifiable unit-specific costs for that Resource the caps are those verifiable unit-specific costs.  A QSE or Resource Entity may file verifiable unit-specific costs for a Resource at any time, but it must file those costs no later than 30 days after five Reliability Unit Commitment (RUC) events for that Resource in a calendar year.  A RUC event begins when a Resource receives a RUC instruction to come or stay On-Line and ends the later of when the Resource shuts down or the end of the Operating Day.  The most recent ERCOT-approved verifiable costs must be used going forward.

(3)	These unit-specific verifiable costs may include and are limited to the following average incremental costs:

(a)	Allocation of maintenance requirements based on number of starts between maintenance events using, at the option of the QSE or Resource Entity, either:

(i)	Manufacturer-recommended maintenance schedule;

(ii)	Historical data for the unit and actual maintenance practices; or

(iii)	Another method approved in advance by ERCOT in writing;

(b)	Startup fuel calculations based on recorded actual measured flows when the data is available or based on averages of historical flows for similar starts (for example, hot, cold, intermediate) when actual data is not available.  Startup fuel will include filing separately the startup fuel required to reach breaker close and fuel after breaker close to Low Sustained Limit (LSL).  Any fuel required to shutdown a Resource will be submitted as the fuel from breaker open to shutdown;

(c)	Operation costs;

(d)	Chemical costs;

(e)	Water costs; and

(f)	Emission credits.

(4)	Standard Operations and Maintenance (O&M) costs pursuant to paragraph (6) below may be used in lieu of the incremental O&M costs set forth in items (3)(a), (c), (d) and (e) above.

(5)	These unit-specific verifiable costs may not include:

(a)	Fixed costs, which are any cost that is incurred regardless of whether the unit is deployed or not; and

(b)	Costs for which the QSE or Resource Entity cannot provide sufficient documentation for ERCOT to verify the costs.

(6)	At their election, QSEs or Resource Entities may receive standard O&M costs for both startup and minimum energy.  This election may be made by submitting an election form to ERCOT.  If a QSE or Resource has received final approval for actual verifiable O&M costs under the verifiable cost process, it may not elect to receive standard O&M costs.

(a)	Until December 31, 2011, standard O&M costs are defined as follows:

| Resource Category Start Year = 2009 | Cold Startup ($/start) | Intermediate Startup ($/start) | Hot Startup ($/start) | Variable O&M ($/MWh) |
| --- | --- | --- | --- | --- |
| Aeroderivative simple cycle commissioned after 1996 | 1,000.00 | 1,000.00 | 1,000.00 | 3.94 |
| Reciprocating Engine | $58/MW * the average of the Seasonal net max sustainable ratings | $58/MW * the average of the Seasonal net max sustainable ratings | $58/MW * the average of the Seasonal net max sustainable ratings | 5.09 |
| Simple cycle ≤ 90 MW | 2,300.00 | 2,300.00 | 2,300.00 | 3.94 |
| Simple cycle ≥ 90 MW | 5,000.00 | 5,000.00 | 5,000.00 | 3.94 |
| Combined cycle: for each Combined-Cycle Configuration, the Startup Cost for that configuration is the sum of the Startup Costs for each unit within that configuration as follows: |  |  |  | 3.19 |
| Combustion turbine < 90 MW | 2,300.00 | 2,300.00 | 2,300.00 |  |
| Combustion turbine ≥ 90 MW | 5,000.00 | 5,000.00 | 5,000.00 |  |
| Steam turbine | 3,000.00 | 2,250.00 | 1,250.00 |  |
| Gas-steam non-reheat boiler | 2,310.00 | 1,732.50 | 866.25 | 7.08 |
| Gas-steam reheat boiler | 3,000.00 | 2,250.00 | 1,125.00 | 7.08 |
| Gas-steam supercritical boiler | 4,800.00 | 3,600.00 | 1,800.00 | 7.08 |
| Nuclear, coal, lignite and hydro | 7,200.00 | 5,400.00 | 2,700.00 | 5.02 |
| Renewable | Not Applicable | Not Applicable | Not Applicable | 5.50 |

(b)	For the period beginning January 1, 2012 and ending December 31, 2012, standard O&M costs shall be reduced by 10% from the levels specified in the table in paragraph (a) above as follows:

| Resource Category | Cold Startup ($/start) | Intermediate Startup ($/start) | Hot Startup ($/start) | Variable O&M ($/MWh) |
| --- | --- | --- | --- | --- |
| Start Year = 2009 |  |  |  |  |
| Aeroderivative simple cycle commissioned after 1996 | 900.00 | 900.00 | 900.00 | 3.55 |
| Reciprocating Engine | $52.20/MW * the average of the Seasonal net max sustainable ratings | $52.20/MW * the average of the Seasonal net max sustainable ratings | $52.20/MW * the average of the Seasonal net max sustainable ratings | 4.58 |
| Simple cycle ≤ 90 MW | 2,070.00 | 2,070.00 | 2,070.00 | 3.55 |
| Simple cycle ≥ 90 MW | 4,500.00 | 4,500.00 | 4,500.00 | 3.55 |
| Combined cycle: for each Combined-Cycle Configuration, the Startup Cost for that configuration is the sum of the Startup Costs for each unit within that configuration as follows: |  |  |  | 2.87 |
| Combustion turbine < 90 MW | 2,070.00 | 2,070.00 | 2,070.00 |  |
| Combustion turbine ≥ 90 MW | 4,500.00 | 4,500.00 | 4,500.00 |  |
| Steam turbine | 2,700.00 | 2,025.00 | 1,125.00 |  |
| Gas-steam non-reheat boiler | 2,079.00 | 1,559.25 | 779.63 | 6.37 |
| Gas-steam reheat boiler | 2,700.00 | 2,025.00 | 1,012.50 | 6.37 |
| Gas-steam supercritical boiler | 4,320.00 | 3,240.00 | 1,620.00 | 6.37 |
| Nuclear, coal, lignite and hydro | 6,480.00 | 4,860.00 | 2,430.00 | 4.52 |
| Renewable | Not Applicable | Not Applicable | Not Applicable | 4.95 |

(c)	Beginning January 1, 2013 and going forward, standard O&M costs shall be reduced by 20% from the levels specified in the table in paragraph (a) above as follows:

| Resource Category | Cold Startup ($/start) | Intermediate Startup ($/start) | Hot Startup ($/start) | Variable O&M ($/MWh) |
| --- | --- | --- | --- | --- |
| Start Year = 2009 |  |  |  |  |
| Aeroderivative simple cycle commissioned after 1996 | 800.00 | 800.00 | 800.00 | 3.15 |
| Reciprocating Engine | $46.40/MW * the average of the Seasonal net max sustainable ratings | $46.40 /MW * the average of the Seasonal net max sustainable ratings | $46.40 /MW * the average of the Seasonal net max sustainable ratings | 4.07 |
| Simple cycle ≤ 90 MW | 1,840.00 | 1,840.00 | 1,840.00 | 3.15 |
| Simple cycle ≥ 90 MW | 4,000.00 | 4,000.00 | 4,000.00 | 3.15 |
| Combined cycle: for each Combined-Cycle Configuration, the Startup Cost for that configuration is the sum of the Startup Costs for each unit within that configuration as follows: |  |  |  | 2.55 |
| Combustion turbine < 90 MW | 1,840.00 | 1,840.00 | 1,840.00 |  |
| Combustion turbine ≥ 90 MW | 4,000.00 | 4,000.00 | 4,000.00 |  |
| Steam turbine | 2,400.00 | 1,800.00 | 1,000.00 |  |
| Gas-steam non-reheat boiler | 1,848.00 | 1,386.00 | 693.00 | 5.66 |
| Gas-steam reheat boiler | 2,400.00 | 1,800.00 | 900.00 | 5.66 |
| Gas-steam supercritical boiler | 3,840.00 | 2,880.00 | 1,440.00 | 5.66 |
| Nuclear, coal, lignite and hydro | 5,760.00 | 4,320.00 | 2,160.00 | 4.02 |
| Compressed air Energy Storage Resource | 5,000.00 | 5,000.00 | 5,000.00 | 3.15 |
| All other Energy Storage Resources | 0.00 | 0.00 | 0.00 | 0.00 |
| Renewable | Not Applicable | Not Applicable | Not Applicable | 4.40 |

(d)	If the QSE or Resource Entity chooses to utilize the standard O&M costs for O&M, standard O&M costs will be used by ERCOT going forward until either:

(i)	Verifiable variable O&M costs are filed; or

(ii)	ERCOT notifies the QSE or Resource Entity to update its verifiable costs as set forth in either paragraph (9) or (10) below.  If a Resource is receiving standard O&M costs, it may reelect standard O&M costs when resubmitting verifiable costs.

(7)	When submitting verifiable costs for combined cycle Resources, the QSE or Resource Entity must elect standard O&M costs for all Combined-Cycle Configurations or verifiable costs for all Combined-Cycle Configurations within the combined cycle train.

(8)	QSEs submitting PPAs as Resource-specific verifiable costs documentation are subject to the guidelines detailed below and in the Verifiable Cost Manual.

(a)	Only QSEs offering Three-Part Supply Offers for a specific Resource may submit a PPA as verifiable costs documentation.

(b)	A QSE submitting a PPA as verifiable costs documentation must represent 100% of the Resource’s capacity.

(c)	Only PPAs:

(i) 	Signed prior to July 16, 2008; and

(ii)	Not between Affiliates, subsidiaries or partners will be accepted as verifiable cost documentation.

(d)	Verifiable costs for PPAs shall be capped at the level of the highest comparable Resource (referred to as the reference Resource) specific verifiable costs approved by ERCOT without a PPA.  The ERCOT approved verifiable costs for a PPA shall be equal to the lesser of:

(i)	The cap as described in paragraph (d) above; and

(ii)	The costs from the PPA.

(e)	ERCOT shall use the Resource actual fuel costs submitted by the QSE for startup and operation at minimum-energy level (LSL), and shall use the Resource Category Startup Offer Generic Costs as the cap for the O&M portion of the Startup Costs until ERCOT receives and approves comparable Resource specific verifiable costs.

(f)	PPAs will no longer be accepted as verifiable cost documentation after the primary term of the contract expires.

(g)	ERCOT shall produce a report each April that provides the percentage of RUC Make-Whole Payments for Resources with PPAs during the 12 months of the previous calendar year.  The report shall be based on the final Settlements and include the total number of Resources that used a PPA for their most recent verifiable cost submission that was approved by ERCOT.  ERCOT shall present the results of this study to the appropriate Technical Advisory Committee (TAC) subcommittee.

(h)	Notwithstanding anything to the contrary in this Section 5.6.1, QSEs representing PPAs may, at any time, submit data from a Resource as verifiable costs documentation and such documentation will be accepted for consideration by ERCOT.  A QSE submitting verifiable costs documentation pursuant to this paragraph shall not be required to submit a PPA to ERCOT for consideration for verifiable cost recovery.

(9)	ERCOT shall notify a QSE to update verifiable cost data of a Resource when the Resource has received more than 50 RUC instructions meeting the criteria in Section 5.6.2, RUC Startup Cost Eligibility, in a year, but ERCOT may not request an update more frequently than annually.

(10)	ERCOT shall notify a QSE to update verifiable cost data of a Resource if at least five years have passed since ERCOT previously approved verifiable cost data for that Resource.

(11)	Within 30 days after receiving an update Notice from ERCOT under either paragraph (9) or (10) above, a QSE or Resource Entity must submit verifiable cost data for the Resource.  Despite the provisions in paragraph (2) above, if the QSE or Resource Entity does not submit verifiable cost data within 30 days after receiving an update Notice, then, for all Operating Days until updated verifiable costs are approved, ERCOT shall determine payment using the lower of:

(a)	Resource Category Startup Generic and Resource Category Minimum-Energy Generic Caps; and

(b)	Current ERCOT-approved verifiable startup and minimum-energy costs.

(12)	Resource Entities that represent Reliability Must-Run (RMR) Resources shall submit to ERCOT, Startup and variable O&M Cost estimates to be used by ERCOT as proxies for verifiable Startup Cost and minimum-energy verifiable cost and for Settlement.  The ERCOT-approved verifiable Startup Cost estimate will equal the startup fuel estimate times the sum of the appropriate Fuel Index Price (FIP) or Fuel Oil Price (FOP) and the fuel adder, plus the startup O&M.  The ERCOT-approved minimum-energy verifiable cost estimate will equal the heat rate from the RMR Agreement contract times the sum of the appropriate FIP or FOP and the fuel adder, plus the variable O&M.  The O&M cost estimates shall be revised monthly to be consistent with the latest actual costs for the RMR Unit submitted in accordance with Section 3.14.1.12, Reporting Actual Eligible Cost.  The O&M values will be effective until updated costs have been submitted to ERCOT.

7.9.1.3	Minimum and Maximum Resource Prices

(1)	For purposes of Section 7.9.1, Day-Ahead CRR Payments and Charges, Settlements data published to the MIS Secure Area shall include the association of the Resource Category for each Generation Resource.  The following prices specified in paragraphs (2) and (3) below are used in the CRR hedge value calculation for CRRs settled in the DAM.

(2)	Minimum Resource Prices of source Settlement Points are:

MINRESPR j	 =	Min ( MINRESRPR j, r ) r

Where:

Minimum Resource Prices for Resources located at source Settlement Points (MINRESRPR j, r) are:

(a)	Nuclear = -$20.00/MWh;

(b)	Hydro = -$20.00/MWh;

(c)	Energy Storage Resource = -$20.00/MWh;

(d)	Coal and Lignite = $0.00/MWh;

(e)	Combined Cycle greater than 90 MW = Fuel Index Price (FIP) * 5 MMBtu/MWh;

(f)	Combined Cycle less than or equal to 90 MW = FIP * 6 MMBtu/MWh;

(g)	Gas -Steam Supercritical Boiler = FIP * 6.5 MMBtu/MWh;

(h)	Gas Steam Reheat Boiler = FIP * 7.5 MMBtu/MWh;

(i)	Gas Steam Non-Reheat or Boiler without Air-Preheater = FIP * 10.5 MMBtu/MWh;

(j)	Simple Cycle greater than 90 MW = FIP * 10 MMBtu/MWh;

(k)	Simple Cycle less than or equal to 90 MW = FIP * 11 MMBtu/MWh;

(l)	Diesel = FIP * 12 MMBtu/MWh;

(m)	Wind = -$35/MWh;

(n)	Reliability Must-Run (RMR) Resource = RMR contract price Energy Offer Curve at Low Sustained Limit (LSL); and

(o)	Other Renewable = -$10.

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| MINRESPR j | $/MWh | Minimum Resource Price for source—The lowest Minimum Resource Price for the Resources located at the source Settlement Point j. |
| MINRESRPR j | $/MWh | Minimum Resource Price for Resource—The Minimum Resource Price for the Resources located at the source Settlement Point j. |
| r | none | A Generation Resource located at the source Settlement Point j. |
| j | none | A source Settlement Point. |

(3)	Maximum Resource Prices of sink Settlement Points are:

MAXRESPR k	 =	Max (MAXRESRPR k, r ) r

Where:

Maximum Resource Prices for Resources located at sink Settlement Points (MAXRESRPR k, r ) are:

(a)	Nuclear = $15.00/MWh;

(b)	Hydro = $10.00/MWh;

(c)	Coal and Lignite = $18.00/MWh;

(d)	Combined Cycle greater than 90 MW = FIP * 9 MMBtu/MWh;

(e)	Combined Cycle less than or equal to 90 MW = FIP * 10 MMBtu/MWh;

(f)	Gas -Steam Supercritical Boiler = FIP * 10.5 MMBtu/MWh;

(g)	Gas Steam Reheat Boiler = FIP * 11.5 MMBtu/MWh;

(h)	Gas Steam Non-Reheat or Boiler without Air-Preheater = FIP * 14.5 MMBtu/MWh;

(i)	Simple Cycle greater than 90 MW = FIP * 14 MMBtu/MWh;

(j)	Simple Cycle less than or equal to 90 MW = FIP * 15 MMBtu/MWh;

(k)	Diesel = FIP * 16 MMBtu/MWh;

(l)	Energy Storage Resource = a4 * (Previous DAM 24-hour average of corresponding Wholesale Storage Load (WSL) Resource Node price) + b * FIP + c, where for

(i)	Natural gas-driven compressed air Energy Storage Resources: a4 = 1.5, b = 6 MMBTU/MWh, c = $15/MWh;

(ii)	Other compressed air Energy Storage Resources: a4 = 1.75, b = 0, c = $35/MWh; and

(iii)	All other Energy Storage Resources: a4 = 1.75, b = 0, c = $35/MWh;

(m)	Wind = $0/MWh;

(n)	RMR Resource = RMR contract price Energy Offer Curve at HSL; and

(o)	Other Renewable = $0.

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| MAXRESPR k | $/MWh | Maximum Resource Price for source—The highest Maximum Resource Price for the Resources located at the sink Settlement Point k. |
| MAXRESRPR k | $/MWh | Maximum Resource Price for Resource—The Maximum Resource Price for the Resources located at the sink Settlement Point k. |
| r | none | A Generation Resource located at the sink Settlement Point k. |
| k | none | A sink Settlement Point. |