| NPRR Number | 585 | NPRR Title | Clarification of Administrative Pricing Requirements for Self-Committed Combined Cycle Combustion Turbines on a Resource Committed by RUC |
| --- | --- | --- | --- |
| Date Posted |  | December 10, 2013 |  |
| Requested Resolution |  | Normal |  |
| Nodal Protocol Section Requiring Revision |  | 6.4.3.1, Energy Offer Curve for RUC-Committed Resources |  |
| Market Guide Section(s) Requiring Revision |  | None. |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) clarifies that for Qualified Scheduling Entities (QSEs) managing Combined Cycle Trains, self-commitment of combustion turbines, in addition to a configuration committed by Reliability Unit Commitment (RUC), may be priced at values other than the System-Wide Offer Cap (SWCAP) and should precede RUC-committed MW in merit order on the Energy Offer Curve for the entire Combined Cycle Train. |  |
| Reason for Revision |  | The current language in Section 6.4.3.1 does not address the requirements of administrative pricing for self-committed combustion turbines in addition to a RUC-committed configuration. |  |
| Credit Implications |  | No. None anticipated. |  |

| Business Case |  |
| --- | --- |
| 1 | Qualitative benefits: - This NPRR will remove any ambiguity over the how an additional self-committed combustion turbine capability can be priced in addition to a combined cycle configuration that has been RUC-committed. |
| 2 | Quantifiable benefits: - The total quantifiable benefits are difficult to state without projecting RUC commitment of Combined Cycle Generation Resources and knowing in advance the corresponding behaviors of the Resource owner. |
| 3 | Impacts to Market Segments: - Potential positive impact to the Consumer Market Segment in the form of lower energy prices from reduced congestion costs. |

| Sponsor |  |
| --- | --- |
| Name | Randy Jones |
| E-mail Address | rajones@calpine.com |
| Company | Calpine |
| Phone Number | 713.830.8846 |
| Cell Number | 832.385.3322 |
| Market Segment | None |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | Jonathan.Levine@ercot.com |
| Phone Number | 512.248.6464 |

Proposed Protocol Language Revision

- 6.4.3.1	Energy Offer Curve for RUC-Committed Resources

(1)	Prior to the end of the Adjustment Period for an Operating Hour during which a Generation Resource has been committed by ERCOT as part of a Reliability Unit Commitment (RUC) process, the QSE shall ensure (i) that an amount of energy equal to the dispatchable capability from LSL to HSL of the Generation Resource is priced at the SWCAP in its Energy Offer Curve, and (ii) that the Energy Offer Curve has been submitted and accepted by ERCOT.  For the avoidance of doubt, additional capacity that may be available from the Generation Resource (e.g., higher configurations on a Combined Cycle Train) may be priced at values other than the SWCAP.  The Technical Advisory Committee (TAC) shall review the market impacts of the dollar value for energy for Generation Resources committed by the RUC process for RUC-Committed Intervals every January.

[NPRR568: Replace paragraph (1) above with the following upon system implementation:] (1) Prior to the end of the Adjustment Period for an Operating Hour during which a Generation Resource has been committed by ERCOT as part of a Reliability Unit Commitment (RUC) process, the QSE shall ensure (i) that an amount of energy equal to the dispatchable capability from LSL to HSL of the Generation Resource is priced at $1,000 per MWh in its Energy Offer Curve, and (ii) that the Energy Offer Curve has been submitted and accepted by ERCOT. For the avoidance of doubt, additional capacity that may be available from the Generation Resource (e.g., higher configurations on a Combined Cycle Train) may be priced at values other than the SWCAP. The Technical Advisory Committee (TAC) shall review the market impacts of the dollar value for energy for Generation Resources committed by the RUC process for RUC-Committed Intervals every January.

(2)	If the QSE receives a RUC Dispatch Instruction from ERCOT for its Generation Resource during the Operating Period that includes a RUC-Committed Hour, then the QSE shall be exempt from the submission timeline requirement specified in paragraph (1) above for the RUC-Committed Hours during the Operating Period.  The QSE shall submit the required Energy Offer Curve as soon as reasonably practicable after receipt of the RUC Dispatch Instruction for the RUC-Committed Hours in the Adjustment Period.

- (3)	The requirement in paragraph (1) above is not applicable for Weekly Reliability Unit Commitment (WRUC)-instructed hours during which the Resource was Day-Ahead Market (DAM)-committed or QSE self-committed.