ERCOT Supplemental Reserve (SR) Service

Working Document – Work Session (3-7-2014):

# Needs of SR Service

SR service is the reserve that is used to compensate for net load forecast error and/or net load ramps.  While ERCOT believes that SR Service will not ultimately be required in the long term, it may be needed during the transition.

# Discussion

- Capacity of a Resource with SR responsibility deployed by SCED once certain trigger conditions (alert/watch/PRC etc.) are met.

- Once the Resources carrying SR capacity is above LSL and released to SCED, SCED will dispatch Resources with SR Responsibility as any other Resource in SCED, based on economics, system needs and constraints.

- SR resource should be capable of ramping to its full SR responsibility within 30 minutes. SCED base point instructions will determine SR resource dispatch in real time.

- SR Resource should be capable of sustaining its full response for as long as it has responsibility to provide this service. SCED base point instructions will determine SR resource dispatch in real time.

- ERCOT to propose similar concept in the SR working document as in today’s protocol regarding load participation in SR

  - Minimum run time similar to what we have for NSRS today

  - Minimum restoration time

  - Allow aggregations for Load Resources providing the service and limit to a single Load Zone

- SR capacity can be recalled once frequency and PRC has reached stable threshold.

- SR recall will be similar to recall of today’s offline NSRS.

- Resources providing SR will be subject to disqualification in addition to SCED base point deviation charges for failure to follow deployment instructions.