Credit Formulas and Detail Whitepaper for the Day-Ahead Market

Version: 0.8

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 09/24/2010 | 0.1 | NPRR206 Implementation Supplement | ERCOT |
| 07/05/2011 | 0.2 | NPRR323 Implementation Details Market Notice for PtP Obligation Bids | ERCOT |
| 8/12/2011 | 0.3 | NPRR323 Update for DAM Energy Bids | ERCOT |
| 01/16/2012 | 0.4 | Addition of CRR Credit Calculations as part of NPRR357/430 | ERCOT |
| 02/2/12 | 0.5 | TAC approved adding this document to the OBD list. This document is effective upon implementation of NPRR430, Corrections Related to NPRR357, Revisions to Collateral Requirements Concerning CRR Auctions, and Clarification to Collateral Requirements. | ERCOT |
| 09/13/12 | 0.6 | Effective 9/13/12 due to implementation of NPRR430, Corrections Related to NPRR357, Revisions to Collateral Requirements Concerning CRR Auctions, and Clarification to Collateral Requirements. | ERCOT |
| 2/8/13 | 0.7 | Effective 2/13/13 for Operating Day 2/14/13 due to implementation of NPRR322, Real-Time PTP Option Modeling | ERCOT |
| 2/25/14 | 0.8 | With the implementation of the NPRR484, the CRR-specific sections of the document are no longer applicable or necessary. For this version of the document, section 8 of the document is removed and the document title is updated to reflect that it will now only cover the Day-Ahead Market. Changed 95% to 90% in section 2.2 to be consistent with protocol section 4.4.10(6)(b)(i)(B). Changed sections 4.1.1 and 5.1.1 to reflect the recent change of the PTP discount factor from 80% to 90%. | ERCOT |

Contents

1	Credit Exposure Calculation – DAM Energy Bids	6

1.1	DAM Energy Only Bid as single price and quantity.	7

1.2	DAM Energy Only Bid as curve	7

2	Credit Exposure Calculation – DAM Energy Offers	8

2.1	Checks	8

2.2	Credit Exposure for a Line Segment	8

3	Credit Exposure Calculation – Three-Part Offers	12

3.1	Checks	12

3.2	Credit Exposure Calculation	12

4	Credit Exposure Calculation – PTP Obligation Bids	16

4.1	PTP Obligation bid credit exposure rule	16

Supporting ERCOT Protocols, Section 4.4.10 , with regard to PTP Obligation bid exposure calculation only	16

4.1.1	PTP Obligation Bids: Validation rules for Insert/Update or running Phase 2 Validation	16

4.1.2	PTP Obligation Bids: Validation rules for Cancel regarding credit exposure	17

4.1.3	PTP Obligation Bids: Eligibility for Credit Exposure Reduction on a QSE basis	17

4.1.4	PTP Obligation Bids: Maintenance of CRR ownership at counterparty level	17

5	Credit Exposure Calculation – PTP Obligation Bids with Links to an Option	18

5.1	PTP Obligation bid credit exposure rule	18

Supporting ERCOT Protocols, Section 4.4.10 , with regard to PTP Obligation Bids with Links to an Option bid exposure calculation only	18

5.1.1	PTP Obligation Bids: Validation rules for Insert/Update or running Phase 2 Validation	18

5.1.2	PTP Obligation Bids: Validation rules for Cancel regarding credit exposure	19

5.1.3	PTP Obligation Bids: Maintenance of CRR ownership at counterparty level	19

6	Credit Exposure Calculation – AS Not self-arranged	19

7	Credit Parameter Values	19

- Credit Exposure Calculation – DAM Energy Bids

Please note that this section has been updated to incorporate the changes to the bid curve calculation contained in NPRR323.  NPRR323 adjusts the credit exposure of an Energy Bid curve-type to be at that price/quantity pair to produces the maximum possible credit exposure, rather than the exposure for the entire bid area. An illustration of this concept is provided below.

Bid Price ($/MW)

Quantity (MW)

NPRR323 implementation – maximum possible credit exposure.

## DAM Energy Only Bid as single price and quantity.

- If price  <=  0,  then,

Credit Exposure = 0

- If price  >  dth % DA_SPP,  then,

Credit Exposure = quantity*(dth %DA_SPP + e1*(price- dth %DA_SPP ))

- If 0 < price <= dth % DA_SPP,  then,

Credit Exposure = quantity* price

## DAM Energy Only Bid as curve

The submitted Bid curve is monotonically non-increasing, i.e. successive prices on the curve must be less than or equal to the previous price on the curve.

Credit exposure calculation is performed only for the portion of the curve where the price  >= 0 If price  <=  0.

- Determine the price to be used for credit exposure calculation for each point on the Energy Bid curve as per the following:

  - If price  >  dth % DA_SPP,   then,

CreditPrice = dth %DA_SPP + e1*(price- dth %DA_SPP )

  - If 0 < price  <=  dth % DA_SPP,  then,

CreditPrice = price

- The modified Energy Bid Curve (prices modified by above step and only for purpose of credit exposure calculation) is treated as a set of line segments (between point k and k+1) and the maximum credit exposure for each line segment is determined as per the following:

  - If the line segment is horizontal or vertical, then the credit exposure is:

Credit Exposure(k to k+1) = quantity(k+1)* CreditPrice(k+1)

  - Otherwise, based on the slope and intercept of this line segment (between k and k+1), calculate the quantity where the maximum credit exposure will occur as:

quantitymax = 0.5 *

(CreditPrice(k)* quantity(k+1) - CreditPrice(k+1)* quantity(k))  / (CreditPrice(k) - CreditPrice(k+1))

  - If quantity(k) <= quantitymax  <=  quantity(k+1),  then,

Credit Exposure(k to k+1) = 0.25 *

(CreditPrice(k)* quantity(k+1) - CreditPrice(k+1)* quantity(k)) *

(CreditPrice(k)* quantity(k+1) - CreditPrice(k+1)* quantity(k)) /

((CreditPrice(k) - CreditPrice(k+1))*( quantity(k+1) - quantity(k)))

  - Otherwise,

Credit Exposure(k to k+1) = CreditPrice(k+1)* quantity(k+1)

- The maximum credit exposure for the entire Energy Bid Curve is the maximum of the  credit exposure calculated for the individual line segments as:

- Maximum Credit Exposure for Energy Only Bid Curve =

MAX (Credit Exposure(1 to 2), … Credit Exposure(k to k+1), ….)

# Credit Exposure Calculation – DAM Energy Offers

## Checks

1.	Make the first (Q,P) pair as (q0,p0) = (0,p1)

where p1 = price of the first submitted price from (q1,p1)

- If abs(q1-q0) < 0.01, then ignore the first segment for credit check.

- For a line segment, if abs[q(i+1)-q(i)) < 0.01, then consider it as vertical line segment. No area under curve, no credit check required. Proceed to next line segment.

## Credit Exposure for a Line Segment

First pair 	= (q(i), p(i))

Second pair	= (q(i+1), p(i+1))

B = 	[q(i+1)-q(i)] * 90% [the Hrly diff of RT-SPP-DA-SPP)] * e3

IF		[p(i) <= ath%DA-SPP]	//Figures 3,4,5,6

- THEN

- q(a) = q(i+1)

IF		[p(i+1) > ath%DA-SPP]	//Figures 5,6

THEN		Calculate intercept q(a)

- IF 		[bth%DA-SPP > 0]

- THEN 		A = - ( (q(a)-q(i)) * bth%DA-SPP * e2)

- ELSE		A = -((q(a)-q(i)) * bth%DA-SPP)

- ELSE		A = 0				//Figures 7,8

Credit Exposure = A+B

Also see Figures 3-8 for visual interpretation of the Credit Exposure calculation.

Figure 3 – Credit Exposure Calculation (Energy Offers) – Positive ath Percentile above the Line Segment

Figure 4 – Credit Exposure Calculation (Energy Offers) – Negative ath Percentile above the Line Segment

Figure 5 – Credit Exposure Calculation (Energy Offers) – Positive ath Percentile intercepting the Line Segment

Figure 6 – Credit Exposure Calculation (Energy Offers) – Negative ath Percentile intercepting the Line Segment

Figure 7 – Credit Exposure Calculation (Energy Offers) – Positive ath Percentile below the Line Segment

Figure 8 – Credit Exposure Calculation (Energy Offers) – Negative ath Percentile below the Line Segment

# Credit Exposure Calculation – Three-Part Offers

## Checks

1.	Make the first (Q,P) pair as (q0,p0) = (0,p1)

where p1 = price of the first submitted price from (q1,p1)

- If abs(q1-q0) < 0.01, then ignore the first segment for credit check.

- For a line segment, if abs[q(i+1)-q(i)) < 0.01, then consider it as vertical line segment. No area under curve, no credit check required. Proceed to next line segment.

## Credit Exposure Calculation

The Credit Exposure calculation for a Three-Part offer is calculated by identifying the target line segment first.

The target line segment is one of the following line segments –

- The last segment of the Three-Part offer curve, if yth percentile is above or below the offer curve, or

- The segment of the Three-Part offer curve that is intercepting the yth percentile.

Target Segment first pair 		= (q(i), p(i))

Target Segment second pair		= (q(i+1), p(i+1))

IF		[p(i) <= yth%DA-SPP]	//Figures 9,10,11,12

- THEN

- q(y) = q(i+1)

IF		[p(i+1) > yth%DA-SPP]	//Figures 11,12

THEN		Calculate intercept q(y)

Credit Exposure = - [q(y) * (zth%DA-SPP)]

- ELSE		Credit Exposure = 0			//Figures 13,14

Also see Figures 9-14 for visual interpretation of the Credit Exposure calculation.

Figure 9 – Credit Exposure Calculation (Three-Part Offers) – Positive yth Percentile above the Line Segment

Figure 10 – Credit Exposure Calculation (Three-Part Offers) – Negative yth Percentile above the Line Segment

Figure 11 – Credit Exposure Calculation (Three-Part Offers) – Positive yth Percentile intercepting the Line Segment

Figure 12 – Credit Exposure Calculation (Three-Part Offers) – Negative yth Percentile intercepting the Line Segment

Figure 13 – Credit Exposure Calculation (Three-Part Offers) – Positive yth Percentile below the Line Segment

Figure 14 – Credit Exposure Calculation (Three-Part Offers) – Negative yth Percentile below the Line Segment

# Credit Exposure Calculation – PTP Obligation Bids

## PTP Obligation bid credit exposure rule

## Supporting ERCOT Protocols, Section 4.4.10, with regard to PTP Obligation bid exposure calculation only

### PTP Obligation Bids: Validation rules for Insert/Update or running Phase 2 Validation

| Validation Rule Description | Validation at time of submission | Validation in Phase 2 |
| --- | --- | --- |
| Cannot exceed the credit limit based on credit exposure calculation for each hour: If PTP_PRICE is negative then consider PTP_PRICE as zero for credit exposure calculations i.e PTP_PRICE=0 Default Credit Exposure= PTP_MW*(max(0,PTP_PRICE) + uth%) Where, PTP_MW= PTP Bid MW for given hour PTP_PRICE= PTP Bid Price ($/MWHr) for given hour uth%=uth percentile of the hourly positive(>0) price differences between the source Real-Time SPP minus the sink Real-Time SPP over the previous 30 days for the given hour. In addition if a QSE qualifies, the Default Credit Exposure is further reduces as described below: Credit Exposure Reduction by netting out CRRs with PTPs: If, at a counterparty, the total CRR is greater than or equal to total MW of all PTP for a given hour and the same source/sink combination, then the credit exposure reduction is: Credit Exposure Reduction (complete netting)= 0.9*PTP_MW*max(0,PTP_PRICE) However, if the total CRRs minus the total MW of PTP is greater than zero but is less than the submitted MW of the new submitted PTP, then the credit exposure reduction is: Credit Exposure Reduction (partial netting)= 0.9[or DerationFactor, if different]*REMAINING_CRR_MW*max(0,PTP_PRICE) Where, REMAINING_CRR_MW=Total CRR MW – Total PTP MW for given hour and same combination of source/sink The final credit exposure will be Final Credit Exposure = Default Credit Exposure – Credit Exposure Reduction(for qualified QSEs) | Enforce rule only in post phase2 | Yes |

### PTP Obligation Bids: Validation rules for Cancel regarding credit exposure

Validation Rule Description

On cancellation, the credit exposure previously subtracted from the counterparties credit limit must be undone upon successful cancellation. Cannot exceed the credit limit based on credit exposure calculation for each hour:

### PTP Obligation Bids: Eligibility for Credit Exposure Reduction on a QSE basis

A flag indicating eligibility for additional Credit Exposure Reduction on a QSE level will be maintained.

### PTP Obligation Bids: Maintenance of CRR ownership at counterparty level

Net CRR ownership from the CRR system on a MW basis will be maintained at an hourly and unique source/sink combination. For each PTP Bid submission/cancel, the PTP MW on an hourly basis and given source/sink combination will be compared with the Net CRR to see if full or partial reduction of credit exposure due to CRR netting can be applied.

# Credit Exposure Calculation – PTP Obligation Bids with Links to an Option

## PTP Obligation bid credit exposure rule

## Supporting ERCOT Protocols, Section 4.4.10, with regard to PTP Obligation Bids with Links to an Option bid exposure calculation only

### PTP Obligation Bids: Validation rules for Insert/Update or running Phase 2 Validation

| Validation Rule Description | Validation at time of submission | Validation in Phase 2 |
| --- | --- | --- |
| Cannot exceed the credit limit based on credit exposure calculation for each hour: If PTP_PRICE is negative then consider PTP_PRICE as zero for credit exposure calculations i.e PTP_PRICE=0 Default Credit Exposure= PTP_MW*(max(0,PTP_PRICE) Where, PTP_MW= PTP Bid MW for given hour PTP_PRICE= PTP Bid Price ($/MWHr) for given hour Credit Exposure Reduction by netting out CRRs with PTPs: If, at a counterparty, the total CRR is greater than or equal to total MW of all PTP for a given hour and the same source/sink combination, then the credit exposure reduction is: Credit Exposure Reduction (complete netting)= 0.9*PTP_MW*max(0,PTP_PRICE) However, if the total CRRs minus the total MW of PTP is greater than zero but is less than the submitted MW of the new submitted PTP, then the credit exposure reduction is: Credit Exposure Reduction (partial netting)= 0.9[or DerationFactor, if different]*REMAINING_CRR_MW*max(0,PTP_PRICE) Where, REMAINING_CRR_MW=Total CRR MW – Total PTP MW for given hour and same combination of source/sink The final credit exposure will be Final Credit Exposure = Default Credit Exposure – Credit Exposure Reduction(for qualified QSEs) | Enforce rule only in post phase2 | Yes |

### PTP Obligation Bids: Validation rules for Cancel regarding credit exposure

Validation Rule Description

On cancellation, the credit exposure previously subtracted from the counterparties credit limit must be undone upon successful cancellation. Cannot exceed the credit limit based on credit exposure calculation for each hour:

### PTP Obligation Bids: Maintenance of CRR ownership at counterparty level

Net CRR ownership from the CRR system on a MW basis will be maintained at an hourly and unique source/sink combination. For each PTP Bid submission/cancel, the PTP MW on an hourly basis and given source/sink combination will be compared with the Net CRR to see if full or partial reduction of credit exposure due to CRR netting can be applied.

# Credit Exposure Calculation – AS Not self-arranged

- Credit Exposure = 	[Q * tth%(DAM-AS-MCPC)]

# Credit Parameter Values

- Credit parameter values are posted on the ERCOT public website at the link:

- http://www.ercot.com/content/mktinfo/dam/kd/Procedures%20for%20Setting%20Nodal%20DAM%20Credit%20Requirement%20Paramete.doc

- The decisions to enforce or ignore the credit constraints in auction optimization process is listed below:

CRRAH1:

CRRAH2:

Revisions to the Credit Formulas and Detail Whitepaper for the Day-Ahead Market shall be approved by the Technical Advisory Committee (TAC).