Detailed Requirements:

PR 117-01 Requirements for Data Submission to Support Aggregate Load Resource Participation in the ERCOT Markets

Version .13

Document Revisions

| Date | Version | Description | Author(s) |
| --- | --- | --- | --- |
| 3/4/14 | .01 | First draft | Catherine Meiners |
| 3/4/14 | .02 | Added delimiter to table in section 1.1. | Catherine Meiners |
| 3/10/14 | .03 | Synched with internal document. | Catherine Meiners |
| 3/10/14 | .04 | Adjusted language | Catherine Meiners |
| 3/10/14 | .05 | Updated to use Aggregate instead of Aggregated | Catherine Meiners |
| 3/10/14 | .06 | Updated report name to 36 characters | Catherine Meiners |
| 3/11/14 | .07 | Updates from CPS-ERCOT meeting – changed title, corrected direction (ERCOT to QSE) on 1.4.2, added option of Blank Appliance for NonRes premise. | Catherine Meiners |
| 3/12/14 | .08 | Updated title. | Catherine Meiners |
| 3/12/14 | .09 | Additional updates based on meeting notes. | Paul Wattles, Carl Raish, Catherine Meiners |
| 3/20/14 | .10 | Added format for date fields in ALRISSiteData file. Added additional column for Interval Meter Start Date to ALRISSiteData file. Added field name column to section 1.2.3.3. for additional clarification. Corrected Invalid-Period-Start long description in 1.3.2.3. Added additional error description for ‘Invalid-Empty-Value;’ to 1.3.2.3. Clarified the description for ‘Invalid-Start-Time’ in 1.4.2.1. | Catherine Meiners |
| 3/26/14 | .11 | Corrected section 1.2.2.2, ER1 record. Changed ESIID to Resource Code. | Catherine Meiners |
| 3/26/14 | .12 | Corrected section 1.2.2.2, ER2 record and section 1.2.3.2 ER3 record. Changed ESIID to Resource Code on both. | Catherine Meiners |
| 4/3/14 | .13 | Changed filenaming convention for the ERCOTValidation files, adding an additional dot between the file type and the “ERCOT Validation” string. | Catherine Meiners |

Table of Contents

1.	Requirements for Data Submission to Support Aggregate Load Resource Participation in the ERCOT Markets	1

1.1.	Aggregate Load Resources in SCED (ALRIS) Site Data Files	1

1.1.1.	File 1 (ALRISSiteData) – Inbound Record Layout for ALRISSiteData file	2

1.1.1.1.	File 1 (ALRISSiteData) - Naming Convention	2

1.1.1.2.	File 1 (ALRISSiteData) – Record Layout	2

1.1.1.3.	File 1 (ALRISSiteData) – Valid Values	4

1.1.2.	File 2 (ALRISSiteDataERCOTResponse) – Outbound Record Layout for ALRISSiteDataERCOTResponse file	4

1.1.2.1.	File 2 (ALRISSiteDataERCOTResponse) - Naming Convention	4

1.1.2.2.	File 2 (ALRISSiteDataERCOTResponse)  – Record Layout	5

1.1.3.	File 3 (ALRISSiteData.ERCOTValidation) – Outbound Record Layout for ALRISSiteData.ERCOTValidation file	6

1.1.3.1.	File 3 (ALRISSiteData.ERCOTValidation) - Naming Convention	7

1.1.3.2.	File 3 (ALRISSiteData.ERCOTValidation)  – Record Layout	7

1.1.3.3.	File 3 (ALRISSiteData.ERCOTValidation)  – Error Descriptions and Long Descriptions	8

1.2.	Aggregate Load Resources in SCED (ALRIS) Event Data Files	8

1.2.1.	File 4 (ALRISEventData) – Inbound Record Layout for ALRISSEventData file	9

1.2.1.1.	File 4 (ALRISEventData) - Naming Convention	9

1.2.1.2.	File 4 (ALRISEventData)  – Record Layout	9

1.2.2.	File 5 (ALRISEventDataERCOTResponse) – Outbound Record Layout for ALRISEventDataERCOTResponse file	10

1.2.2.1.	File 5 (ALRISEventDataERCOTResponse) - Naming Convention	11

1.2.2.2.	File 5 (ALRISEventDataERCOTResponse)  – Record Layout	11

1.2.3.	File 6 – Outbound Record Layout for ALRISEventData.ERCOTValidation file	12

1.2.3.1.	File 6 (ALRISEventData.ERCOTValidation) - Naming Convention	13

1.2.3.2.	File 6 (ALRISEventData.ERCOTValidation)  – Record Layout	13

1.2.3.3.	File 6 (ALRISEventData.ERCOTValidation)  – Error Descriptions and Long Descriptions	14

1.3.	Aggregate Load Resources in SCED (ALRIS) NonIDR (NIDR) Files	14

1.3.1.	File 7 (ALRISNIDRData) - Record Layout for NIDR Data file	15

1.3.1.1.	File 7 (ALRISNIDR Data) - Naming Convention	15

1.3.1.2.	File 7 (ALRISNIDRData)  – Record Layout	15

1.3.2.	File 8 (ALRISNIDRData.ERCOTValidation) – Outbound Record Layout for ALRISNIDRData.ERCOTValidation file	17

1.3.2.1.	File 8 (ALRISNIDRData.ERCOTValidation) - Naming Convention	17

1.3.2.2.	File 8 (ALRISNIDRData.ERCOTValidation)  – Record Layout	17

1.3.2.3.	File 8 (ALRISNIDRData.ERCOTValidation)  – Error Descriptions and Long Descriptions	18

1.4.	Aggregate Load Resources in SCED (ALRIS) Advanced Metering Data (LSE) Files	19

1.4.1.	File 9 (ALRISIntervalData) - Record Layout for Interval Data file	19

1.4.1.1.	File 9 (ALRISIntervalData) - Naming Convention	19

1.4.1.2.	File 9 (ALRISIntervalData)  – Record Layout	19

1.4.2.	File 10 (ALRISIntervalData.ERCOTValidation) - Record Layout for Interval Data file	22

1.4.2.1.	File 10 (ALRISIntervalData.ERCOTValidation) - Naming Convention	22

1.4.2.2.	File 10 (ALRISIntervalData.ERCOTValidation) – Record Layout	23

1.4.2.1.	File 10 (ALRISIntervalData.ERCOTValidation) – Error Descriptions and Long Descriptions	24

# Requirements for Data Submission to Support Aggregate Load Resource Participation in the ERCOT Markets

Qualification as a Load Resource is a pre-requisite for the provision of demand response in the ERCOT Ancillary Services markets and Real-Time Energy Market (aka Security-Constrained Economic Dispatch, or SCED).  Since the ERCOT market opened in 2002, the fleet of Load Resources has been limited to single-site Loads.  Due to limitations in various ERCOT systems and to technological barriers to entry, aggregations of Loads have been unable to qualify as Load Resources.

Enabling access to the ERCOT markets for Aggregate Load Resources (ALRs) offers the prospect of broader customer participation in electricity supply and demand, and could increase the pool of participants in ERCOT’s real-time energy and day-ahead Ancillary Services markets.

An individual ALR potentially may consist of thousands, or tens of thousands, of end-use customers.  In order to support participation by such Resources in the ERCOT markets, a market interface must be established to permit QSEs to maintain the populations of their ALRs, and to enable ERCOT to receive the necessary supporting data to qualify the ALR for participation and measure and verify their performance.

This document sets forth the detailed requirements for data formatting in that market interface.

In order to gather the necessary data for Aggregate Load Resources in SCED from the QSEs, ERCOT is using 4 inbound file types and 6 outbound file types.  Two of the inbound file types were created specifically for this project.  The other two inbound file types are already being used at ERCOT and are being repurposed to satisfy the needs of the Aggregate Load Resources in SCED Project.

| Type of Data | File Extension | Delimiter | Inbound Report Name | Formatting Validation Response Report Name | Business Validation Response Report Name |
| --- | --- | --- | --- | --- | --- |
| Site Data | .csv | Pipe | ALRISSiteData | ALRISSiteDataERCOT Response | ALRISSiteData.ERCOT Validation |
| Event Data | .csv | Pipe | ALRISEventData | ALRISEventDataERCOT Response | ALRISEventData.ERCOT Validation |
| NIDR Data | .nidr | Comma | ALRISNIDRData | N/A | ALRISNIDRData.ERCOT Validation |
| Advanced Meter Data | .lse | Comma | ALRISIntervalData | N/A | ALRISIntervalData.ERCOT Validation |

## Aggregate Load Resources in SCED (ALRIS) Site Data Files

The section covers the requirements for the Aggregate Load Resources in SCED Site Data files and the response files.

All information must be sent in a pipe delimited flat file with .csv extension via North American Energy Standards Board (NAESB) and must contain all required fields.  All records sent in the file must be terminated by a Carriage Return Line Feed (CRLF).

ERCOT will validate that all mandatory data elements are present and meet formatting requirements.  ERCOT will send a response to the submitting QSE via NAESB.  ERCOT will inform the submitting QSE of any data fields that did not meet formatting requirements or of any required data fields that were not provided.  In instances where one or more fields fail formatting requirements or are missing required data, ERCOT will not process any of the file and the QSE should resubmit the entire file, including rows that passed validation in the original submission, plus corrected rows.  The corrected file should have a new unique Report ID in the header row.  If all rows submitted pass formatting requirements, file will be processed through business validations.

ERCOT’s validation and analysis processes will always treat the most recently submitted file as properly reporting all current and previous site participation.  Each monthly file submitted by the QSE should be a full list of all ESIIDs that are currently or have previously participated. If any fields on a row of data for an ESIIID were previously submitted with incorrect values, the fields should be corrected on the row in the file being submitted.  If an entire row was previously submitted and should not have been, the row should be omitted in the file being submitted.

Note:  If an ESIID has more than one period of participation in a resource, separate rows must be submitted for each period with non-overlapping start and stop dates.

### File 1 (ALRISSiteData) – Inbound Record Layout for ALRISSiteData file

This is the inbound file sent by the QSE containing Site Information.

Sent from QSE to ERCOT

### File 1 (ALRISSiteData) - Naming Convention

Convention-

[QSEDUNS][ALRISSiteData][YYYYMMDDHHMI][GlobalCounter].csv

Example -

111111111ALRISSiteData201303180930101.csv

### File 1 (ALRISSiteData) – Record Layout

Header record – The HDR record contains information on the type of file, the sender DUNS, and a unique identifier.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “HDR.” | Alphanumeric (3) |
| Report Name | Mandatory | Mutually defined report definition. Hard Code “ALRISSiteData”. | Alphanumeric (36) |
| Report ID | Mandatory | The unique report number designated by the Sender to be used in the ALRISSiteDataERCOTResponse file and the ALRISSiteData.ERCOTValidation file. | Alphanumeric (80) |
| QSE DUNS Number | Mandatory | Qualified Scheduling Entity DUNS. Associated with the ESIIDs sent in the file. | Numeric (9 or 13) |

Detail record - The DET record contains the ESIID level Site Data information.  This is an optional record and can repeat multiple times.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “DET.” | Alphanumeric (3) |
| Record Number | Mandatory | The unique sequential record number starting with “1.” | Numeric (8) |
| QSE DUNS Number | Mandatory | DUNS number of QSE sending site data. | Numeric (9 or 13) |
| Resource Code | Mandatory | Code that links site to a resource. | Alphanumeric (13) |
| ESIID Number | Mandatory | The identification for the customer active within the category. The ESIID is the basic identifier assigned to each Service Delivery Point (SDP) Note: If an ESIID has more than one period of participation in a resource, separate rows must be submitted for each period with non-overlapping start and stop dates. | Alphanumeric (36) |
| Premise Type | Mandatory | Premise type associated with ESIID. See list below with valid Premise values. | Alphanumeric (6) |
| House Type | Optional | House type associated with ESIID. See list below with valid House values. | Alphanumeric (3) |
| Appliance | Optional | Type of controllable appliance(s). Multiple values may be included using “-“ as a delimiter. See list below with valid appliance values. | Alphanumeric (80) |
| Substation Code | Conditional | Substation serving the ESIID. Mandatory for NOIEs, not used for Opt-In. | Alphanumeric (36) |
| Zip Code | Conditional | Zip Code for service address associated with the ESIID. Mandatory for NOIEs, not used for Opt-In. | Numeric (9) |
| Transmission Load Point | Conditional | Transmission Load Point serving the ESIID. Mandatory for NOIEs, not used for Opt-In. | Alphanumeric (36) |
| Start Date | Mandatory | The date the ESIID began current participation. YYYYMMDD | Numeric (8) |
| Stop Date | Conditional | The date the ESIID ended most recent participation. Leave blank if ESIID is currently participating. Required if ESIID is no longer participating at time of submission. YYYYMMDD | Numeric (8) |
| Interval Meter Start Date | Optional | Date that the collection of interval data for ESIID began. YYYYMMDD | Numeric (8) |

Summary record – The SUM row contains a count of detail records included in the file.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “SUM.” | Alphanumeric (3) |
| Total Number of DET Records | Mandatory | Total number of DET records in file. | Numeric (8) |

### File 1 (ALRISSiteData) – Valid Values

| Premise Type |  |  | Housing Type |  |  | Appliance |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Comment |  | Value | Comment |  | Value | Comment |
| NONRES | Non Residential |  | SFD | Single Family Detached |  | HVAC | HVAC |
| RES | Residential |  | MF | Multi Family |  | PP | Pool Pump |
|  |  |  | MH | Mobile Home |  | WH | Water Heater |
|  |  |  | Blank | Blank if NONRES |  | OTHER | Other Appliance(s) |
|  |  |  |  |  |  | Blank | Blank if NONRES |

### File 2 (ALRISSiteDataERCOTResponse) – Outbound Record Layout for ALRISSiteDataERCOTResponse file

This is the outbound file sent by ERCOT to the QSE containing information on the processing of the inbound file and details of any records failing for formatting or file validations.  This information will be sent in a pipe delimited flat file with .csv extension via North American Energy Standards Board (NAESB) and will contain all required fields.  All records sent in the file will be terminated by a Carriage Return Line Feed (CRLF).

Sent from ERCOT to QSE

If the QSE receives an ALRISSiteDataERCOTResponse file with one or more error rows, the entire original file should be resubmitted.  The new file will contain a new unique Report ID, all the original rows that passed validations, plus corrections to the rows that contained errors.

### File 2 (ALRISSiteDataERCOTResponse) - Naming Convention

Convention-

[QSEDUNS][ALRISSiteDataERCOTResponse][YYYYMMDDHHMI][GlobalCounter].csv

Example -

111111111ALRISSiteDataERCOTResponse201303180930101.csv

### File 2 (ALRISSiteDataERCOTResponse)  – Record Layout

Header record – The HDR record contains information on the type of file, the receiver DUNS, and the unique identifier provided in the inbound file.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “HDR.” | Alphanumeric (3) |
| Report Name | Mandatory | Mutually defined report definition. Hard Code “ALRISSiteDataERCOTResponse”. | Alphanumeric (36) |
| Report ID | Mandatory | Report ID as sent in the ‘ALRISSiteData’ file. | Alphanumeric (80) |
| QSE DUNS Number | Mandatory | Qualified Scheduling Entity DUNS number receiving this response report information based on the original file submission. | Numeric (9 or 13) |

ER1 record – Used when an inbound record contains an invalid value or format.  Contains a reference to the original record.  This is an optional record and can repeat multiple times.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “ER1.” | Alphanumeric (3) |
| Record Number | Mandatory | The unique sequential record number starting with “1”. Same as the sequential number used for ER2. | Numeric (8) |
| ESIID Number | Conditional | The basic identifier assigned to each SDP. | Alphanumeric (36) |
| Original Record Type | Mandatory | The type of record in error. Valid values are DET, HDR, and SUM. | Alphanumeric (3) |
| Original Record Number | Conditional | Original DET Record Number sent in the ALRISSiteData file that is in error. Required if Original Record Type is DET. | Numeric (8) |
| Field Name | Mandatory | Field name of record that is in error. | Alphanumeric (80) |
| Error Description | Mandatory | Description of error. | Alphanumeric (80) |

ER2 record – Used when an inbound record is missing a mandatory field.  Contains a reference to the original record.  This is an optional record and can repeat multiple times.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “ER2.” | Alphanumeric (3) |
| Record Number | Mandatory | The unique sequential record number starting with “1”. Same as the sequential number used for ER1. | Numeric (8) |
| ESIID Number | Conditional | The basic identifier assigned to each SDP. | Alphanumeric (36) |
| Original Record Type | Mandatory | The type of record in error. Valid values are DET, HDR, and SUM. | Alphanumeric (3) |
| Original Record Number | Conditional | Original DET Record Number sent in the ALRISSiteData file that is in error. Required if Original Record Type is DET. | Numeric (8) |
| Field Name | Mandatory | Field name of record that is in error. | Alphanumeric (80) |
| Error Description | Mandatory | Description of error. | Alphanumeric (80) |

Summary record – The SUM row contains a count of detail records included in the inbound file and the number of DET records in error.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “SUM.” | Alphanumeric (3) |
| Total Number of DET Records | Mandatory | Total number of DET records in the original ALRISSiteData file. | Numeric (8) |
| Total Number of processed DET Records | Mandatory | Total number of DET records that did not contain any errors in the original ALRISSiteData file. | Numeric (8) |
| Total Number of Error Records | Mandatory | Total number of ER1 and ER2 Records contained in response file. | Numeric (8) |

### File 3 (ALRISSiteData.ERCOTValidation) – Outbound Record Layout for ALRISSiteData.ERCOTValidation file

This is the outbound file sent by ERCOT to the QSE containing information on the processing of the inbound file and details of any records failing for business validations.  This information will be sent in a pipe delimited flat file with .csv extension via North American Energy Standards Board (NAESB) and will contain all required fields.  All records sent in the file will be terminated by a Carriage Return Line Feed (CRLF).

Sent from ERCOT to QSE

If the QSE receives an ALRISSiteData.ERCOTValidation file with one or more error rows, the entire original file should be resubmitted.  The new file will contain a new unique Report ID, all the original rows that passed validations, plus corrections to the rows that contain errors.

### File 3 (ALRISSiteData.ERCOTValidation) - Naming Convention

Convention-

[QSEDUNS][ALRISSiteData.ERCOTValidation][YYYYMMDDHHMI][GlobalCounter].csv

Example -

111111111ALRISSiteData.ERCOTValidation201303180930101.csv

### File 3 (ALRISSiteData.ERCOTValidation)  – Record Layout

Header record – The HDR record contains information on the type of file, the receiver DUNS, and the unique identifier provided in the inbound file.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “HDR.” | Alphanumeric (3) |
| Report Name | Mandatory | Mutually defined report definition. Hard Code “ALRISSiteData.ERCOTValidation”. | Alphanumeric (36) |
| Report ID | Mandatory | Report ID as sent in the ‘ALRISSiteData’ file. | Alphanumeric (80) |
| QSE DUNS Number | Mandatory | Qualified Scheduling Entity DUNS number receiving this response report information based on the original file submission. | Numeric (9 or 13) |

ER3 record – Used when an inbound record contains a value that fails business validation.  Contains a reference to the original record.  This is an optional record and can repeat multiple times.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “ER3.” | Alphanumeric (3) |
| Record Number | Mandatory | The unique sequential record number starting with “1”. | Numeric (8) |
| ESIID Number | Conditional | The basic identifier assigned to each SDP. | Alphanumeric (36) |
| Original Record Type | Mandatory | The type of record in error. Valid values are DET, HDR, and SUM. | Alphanumeric (3) |
| Original Record Number | Conditional | Original DET Record Number sent in the ALRISSiteData file that is in error. Required if Original Record Type is DET. | Numeric (8) |
| Field Name | Mandatory | Field name of record that is in error. | Alphanumeric (80) |
| Error Description | Mandatory | Description of error. See list of error descriptions and long descriptions below. | Alphanumeric (80) |

Summary record – The SUM row contains a count of detail records included in the inbound file and the number of DET records in error.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “SUM.” | Alphanumeric (3) |
| Total Number of DET Records | Mandatory | Total number of DET records in the original ALRISSiteData file. | Numeric (8) |
| Total Number of Processed DET Records | Mandatory | Total number of DET records that did not contain any errors in the original ALRISSiteData file. | Numeric (8) |
| Total Number of Error Records | Mandatory | Total number of ER3 Records contained in response file. | Numeric (8) |

### File 3 (ALRISSiteData.ERCOTValidation)  – Error Descriptions and Long Descriptions

| Field Name | Short Description | Long Description |
| --- | --- | --- |
| ResourceCode | Not-in-ERCOT-System | Resource code is not in ERCOT system |
| ESIID | In-Multiple-Resources | The same ESIID is reported for more than one resource |
| ESIID | Active-Multiple-Resources | The same ESIID is reported as active in more than one resource |
| ESIID | Not-in-ERCOT-System | ESIID in competitive area is not in ERCOT system |
| ESIID | Incorrect-DOE-Code | ESIID does not begin with DOE code for the submitting QSE |
| SubstationCode | Not-in-ERCOT-System | Substation code is not in ERCOT system |
| TranmissionLoadPoint | Not-in-ERCOT-System | Transmission Load Point is not in ERCOT system |
| ZipCode | Invalid-for-NOIE | ZIP Code is not valid for NOIE |
| StartDate | Before-Resource-Start-Date | Start date for ESIID is before resource start date |
| StopDate | Before-Start-Date | Stop date for ESIID is before start date |
| StartDate | Overlapping-period-of-participation-for-an-ESIID | Participation start and stop dates overlap one or more previous periods of participation |

## Aggregate Load Resources in SCED (ALRIS) Event Data Files

The section covers the requirements for the Aggregate Load Resources in SCED Deployment Event Data files and the response files.

All information must be sent in a pipe delimited flat file with .csv extension via North American Energy Standards Board (NAESB) and must contain all required fields.  All records sent in the file must be terminated by a Carriage Return Line Feed (CRLF).

ERCOT will validate that all mandatory data elements are present and meet formatting requirements.  ERCOT will send a response to the submitting QSE via NAESB.  ERCOT will inform the submitting QSE of any data fields that did not meet formatting requirements or of any required data fields that were not provided.  In instances where one or more fields fail formatting requirements or are missing required data, ERCOT will not process any of the file and the QSE should resubmit the entire file, including rows that passed validation in the original submission, plus corrected rows.  The corrected file should have a new unique Report ID in the header row.  If all rows submitted pass formatting requirements, file will be processed through business validations.

ERCOT’s validation and analysis processes will always treat the most recently submitted file as properly reporting a complete history of QSE-initiated deployment events.  If any fields on a row of data for a deployment event were previously submitted with incorrect values, the fields should be corrected on the row in the file being submitted.  If an entire row was previously submitted and should not have been, the row should be omitted in the file being submitted.

### File 4 (ALRISEventData) – Inbound Record Layout for ALRISSEventData file

This is the inbound file sent by the QSE containing Deployment Event Information.

Sent from QSE to ERCOT

### File 4 (ALRISEventData) - Naming Convention

Convention-

[QSEDUNS][ALRISEventData][YYYYMMDDHHMI][GlobalCounter].csv

Example -

111111111ALRISEventData201303180930101.csv

### File 4 (ALRISEventData)  – Record Layout

Header record – The HDR record contains information on the type of file, the sender DUNS, and a unique identifier.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “HDR.” | Alphanumeric (3) |
| Report Name | Mandatory | Mutually defined report definition. Hard Code “ALRISEventData”. | Alphanumeric (36) |
| Report ID | Mandatory | The unique report number designated by the Sender to be used in the ALRISEventDataERCOTResponse and ALRISEventData.ERCOTValidation file. | Alphanumeric (80) |
| QSE DUNS Number | Mandatory | Qualified Scheduling Entity DUNS. Associated with Deployment Events sent in the file. | Numeric (9 or 13) |

Detail record - The DET record contains the Deployment Event Data information.  This is an optional record and can repeat multiple times.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “DET.” | Alphanumeric (3) |
| Record Number | Mandatory | The unique sequential record number starting with “1.” | Numeric (8) |
| QSE DUNS Number | Mandatory | DUNS number of QSE sending deployment data. | Numeric (9 or 13) |
| Resource Code | Mandatory | Code that identifies the resource. | Alphanumeric (13) |
| Deployment Start Date and Time | Mandatory | Begin date and time of QSE-initiated non-SCED deployment. Must be in MM/DD/YYYY:HH:MM:SS (24-hour) format. | Alphanumeric (19) |
| Deployment Stop Date and time | Mandatory | End date and time of QSE-initiated non-SCED deployment. Must be in MM/DD/YYYY:HH:MM:SS (24-hour) format. | Alphanumeric (19) |

Summary record – The SUM row contains a count of detail records included in the file.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “SUM.” | Alphanumeric (3) |
| Total Number of DET Records | Mandatory | Total number of DET records in file. | Numeric (8) |

### File 5 (ALRISEventDataERCOTResponse) – Outbound Record Layout for ALRISEventDataERCOTResponse file

This is the outbound file sent by ERCOT to the QSE containing information on the processing of the inbound file and details of any records failing for formatting or file validations.  This information will be sent in a pipe delimited flat file with .csv extension via North American Energy Standards Board (NAESB) and will contain all required fields.  All records sent in the file will be terminated by a Carriage Return Line Feed (CRLF).

Sent from ERCOT to QSE

If the QSE receives an ALRISEventDataERCOTResponse file with one or more error rows, the entire original file should be resubmitted.  The new file will contain a new unique Report ID, all the original rows that passed validations, plus corrections to the rows containing errors.

### File 5 (ALRISEventDataERCOTResponse) - Naming Convention

Convention-

[QSEDUNS][ALRISEventDataERCOTResponse][YYYYMMDDHHMI][GlobalCounter].csv

Example -

111111111ALRISEventDataERCOTResponse201303180930101.csv

### File 5 (ALRISEventDataERCOTResponse)  – Record Layout

Header record – The HDR record contains information on the type of file, the receiver DUNS, and the unique identifier provided in the inbound file.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “HDR.” | Alphanumeric (3) |
| Report Name | Mandatory | Mutually defined report definition. Hard Code “ALRISEventDataERCOTResponse”. | Alphanumeric (36) |
| Report ID | Mandatory | Report ID as sent in the ‘ALRISEventData’ file. | Alphanumeric (80) |
| QSE DUNS Number | Mandatory | Qualified Scheduling Entity DUNS number receiving this response report information based on the original file submission. | Numeric (9 or 13) |

ER1 record – Used when an inbound record contains an invalid value or format.  Contains a reference to the original record.  This is an optional record and can repeat multiple times.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “ER1.” | Alphanumeric (3) |
| Record Number | Mandatory | The unique sequential record number starting with “1”. Same as the sequential number used for ER2. | Numeric (8) |
| Resource Code | Mandatory | Code that identifies the resource. | Alphanumeric (36) |
| Original Record Type | Mandatory | The type of record in error. Valid values are DET, HDR, and SUM. | Alphanumeric (3) |
| Original Record Number | Conditional | Original DET Record Number sent in the ALRISSiteData file that is in error. Required if Original Record Type is DET. | Numeric (8) |
| Field Name | Mandatory | Field name of record that is in error. | Alphanumeric (80) |
| Error Description | Mandatory | Description of error. | Alphanumeric (80) |

ER2 record – Used when an inbound record is missing a mandatory field.  Contains a reference to the original record.  This is an optional record and can repeat multiple times.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “ER2.” | Alphanumeric (3) |
| Record Number | Mandatory | The unique sequential record number starting with “1”. Same as the sequential number used for ER1. | Numeric (8) |
| Resource Code | Conditional | The basic identifier assigned to each SDP. | Alphanumeric (36) |
| Original Record Type | Mandatory | The type of record in error. Valid values are DET, HDR, and SUM. | Alphanumeric (3) |
| Original Record Number | Conditional | Original DET Record Number sent in the ALRISSiteData file that is in error. Required if Original Record Type is DET. | Numeric (8) |
| Field Name | Mandatory | Field name of record that is in error. | Alphanumeric (80) |
| Error Description | Mandatory | Description of error. | Alphanumeric (80) |

Summary record – The SUM row contains a count of detail records included in the inbound file and the number of DET records in error.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “SUM.” | Alphanumeric (3) |
| Total Number of DET Records | Mandatory | Total number of DET records in the original ALRISSiteData file. | Numeric (8) |
| Total Number of processed DET Records | Mandatory | Total number of DET records that did not contain any errors in the original ALRISSiteData file. | Numeric (8) |
| Total Number of Error Records | Mandatory | Total number of ER1 and ER2 Records contained in response file. | Numeric (8) |

### File 6 – Outbound Record Layout for ALRISEventData.ERCOTValidation file

This is the outbound file sent by ERCOT to the QSE containing information on the processing of the inbound file and details of any records failing for business validations.  This information will be sent in a pipe delimited flat file with .csv extension via North American Energy Standards Board (NAESB) and will contain all required fields.  All records sent in the file will be terminated by a Carriage Return Line Feed (CRLF).

Sent from ERCOT to QSE

If the QSE receives an ALRISEventData.ERCOTValidation file with one or more error rows, the entire original file should be resubmitted.  The new file will contain a new unique Report ID, all the original rows that passed validations, plus corrections to the rows that contain errors.

### File 6 (ALRISEventData.ERCOTValidation) - Naming Convention

Convention-

[QSEDUNS][ALRISEventData.ERCOTValidation][YYYYMMDDHHMI][GlobalCounter].csv

Example -

111111111ALRISEventData.ERCOTValidation201303180930101.csv

### File 6 (ALRISEventData.ERCOTValidation)  – Record Layout

Header record – The HDR record contains information on the type of file, the receiver DUNS, and the unique identifier provided in the inbound file.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “HDR.” | Alphanumeric (3) |
| Report Name | Mandatory | Mutually defined report definition. Hard Code “ALRISEventData.ERCOTValidation”. | Alphanumeric (36) |
| Report ID | Mandatory | Report ID as sent in the ‘ALRISEventData’ file. | Alphanumeric (80) |
| QSE DUNS Number | Mandatory | Qualified Scheduling Entity DUNS number receiving this response report information based on the original file submission. | Numeric (9 or 13) |

ER3 record – Used when an inbound record contains a value that fails business validation.  Contains a reference to the original record.  This is an optional record and can repeat multiple times.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “ER3.” | Alphanumeric (3) |
| Record Number | Mandatory | The unique sequential record number starting with “1”. | Numeric (8) |
| Resource Code | Conditional | The basic identifier assigned to each SDP. | Alphanumeric (36) |
| Original Record Type | Mandatory | The type of record in error. Valid values are DET, HDR, and SUM. | Alphanumeric (3) |
| Original Record Number | Conditional | Original DET Record Number sent in the ALRISEventData file that is in error. Required if Original Record Type is DET. | Numeric (8) |
| Field Name | Mandatory | Field name of record that is in error. | Alphanumeric (80) |
| Error Description | Mandatory | Description of error. See list of error descriptions and long descriptions below. | Alphanumeric (80) |

Summary record – The SUM row contains a count of detail records included in the inbound file and the number of DET records in error.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “SUM.” | Alphanumeric (3) |
| Total Number of DET Records | Mandatory | Total number of DET records in the original ALRISSiteData file. | Numeric (8) |
| Total Number of Processed DET Records | Mandatory | Total number of DET records that did not contain any errors in the original ALRISSiteData file. | Numeric (8) |
| Total Number of Error Records | Mandatory | Total number of ER3 rows contained in response file. | Numeric (8) |

### File 6 (ALRISEventData.ERCOTValidation)  – Error Descriptions and Long Descriptions

| Field Name | Error Description | Long Description |
| --- | --- | --- |
| ResourceCode | Invalid-Resource-Code | Resource code is not in ERCOT system |
| DeploymentStart | Invalid Deployment Start Date and Time | Deployment start date and time has incorrect values or occurs in the future |
| DeploymentStop | Invalid Deployment Stop Date and Time | Deployment stop date and time has incorrect values, occurs before start date and time or occurs in the future |
| DeploymentStart | Deployment-Overlap | Deployment start and stop dates and times overlap one or more previous deployments |

## Aggregate Load Resources in SCED (ALRIS) NonIDR (NIDR) Files

The section covers the requirements for the Aggregate Load Resources in SCED NIDR data files and response file.

All information must be sent in a comma delimited flat file format via North American Energy Standards Board (NAESB) and must contain all required fields.  All records sent in the file must be terminated by a Carriage Return Line Feed (CRLF).  The filename must contain the .nidr extension.

ERCOT will validate that all mandatory data elements are present and meet formatting requirements.  ERCOT will send a response to the submitting QSE via NAESB.  ERCOT will inform the submitting QSE of any data fields that did not meet formatting requirements or of any required data fields that were not provided.  In instances where one or more fields fail formatting requirements or are missing required data, ERCOT will not process any of the file and the QSE should resubmit the entire file, including rows that passed validation in the original submission, plus corrected rows.

Note:  The complete cumulative history of NIDR readings should be included with each submission.

Instead of developing a new file format for this project, ERCOT is repurposing an existing format that is currently in use at ERCOT.  In order to make that format met the needs of Aggregate Load Resources in SCED, a number of columns in the format will be left blank since they are not applicable to this project.

### File 7 (ALRISNIDRData) - Record Layout for NIDR Data file

This section contains information for the inbound NIDR file from the QSEs.

Sent from QSE to ERCOT

### File 7 (ALRISNIDR Data) - Naming Convention

Convention-

[QSEDUNS][ALRISNIDRData][YYYYMMDDHHMI][GlobalCounter].nidr

Example -

111111111ALRISNIDRData201303180930101.nidr

### File 7 (ALRISNIDRData)  – Record Layout

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| ESI ID | Mandatory | Unique identifier assigned by QSE | Alphanumeric (80) |
| Transaction Type | Mandatory | Mutually defined value. Hard Code “Original Readings”. | Alphanumeric (42) |
| MRE | Mandatory | Utility DUNS Number | Numeric (9 or 13) |
| BusinessProcessID | Mandatory | Unique row identification, start with ‘1’ and increase sequentially. | Numeric (5) |
| GlobalProcessId | Leave Blank | Leave Blank | Leave Blank |
| Sender | Mandatory | Sender DUNS Number | Numeric (9 or 13) |
| Receiver | Mandatory | Receiver DUNS number – 183529049 (ERCOT’s DUNS) | Numeric (9) |
| DeliveryMethod | Leave Blank | Leave Blank | Leave Blank |
| TransactionFormat | Leave Blank | Leave Blank | Leave Blank |
| ActivityName | Leave Blank | Leave Blank | Leave Blank |
| ActivityId | Leave Blank | Leave Blank | Leave Blank |
| BPMServer | Leave Blank | Leave Blank | Leave Blank |
| CALBPID | Leave Blank | Leave Blank | Leave Blank |
| CALBPMServer | Leave Blank | Leave Blank | Leave Blank |
| CALACTID | Leave Blank | Leave Blank | Leave Blank |
| UtilityName | Leave Blank | Leave Blank | Leave Blank |
| REPDUNS | Leave Blank | Leave Blank | Leave Blank |
| REPNAME | Leave Blank | Leave Blank | Leave Blank |
| ClearingHouseDUNS | Mandatory | 183529049 (ERCOT’s DUNS) | Numeric (9) |
| TransactionNumber | Leave Blank | Leave Blank | Leave Blank |
| PeriodStart | Mandatory | MM/DD/YYYY | Alphanumeric (10) |
| PeriodEnd | Mandatory | MM/DD/YYYY | Alphanumeric (10) |
| MeterType | Mandatory | Usually KHMON (for kWh values) or K1MON ( for kW values) | Alphanumeric (5) |
| ReadStatus | Mandatory | A=Actual, E=Estimated | Alphanumeric (1) |
| Total | Mandatory | Total for month | Numeric (20) |
| OnPeak | Leave Blank | Leave Blank | Leave Blank |
| OffPeak | Leave Blank | Leave Blank | Leave Blank |
| MidPeak | Leave Blank | Leave Blank | Leave Blank |
| SuperPeak | Leave Blank | Leave Blank | Leave Blank |

### File 8 (ALRISNIDRData.ERCOTValidation) – Outbound Record Layout for ALRISNIDRData.ERCOTValidation file

This section contains information on the outbound response file sent by ERCOT.  This information will be sent in a comma delimited flat file with .nidr extension via North American Energy Standards Board (NAESB) and will contain all required fields.  All records sent in the file will be terminated by a Carriage Return Line Feed (CRLF).

Sent from ERCOT to QSE

### File 8 (ALRISNIDRData.ERCOTValidation) - Naming Convention

Convention-

[QSEDUNS][ALRISNIDRDataERCOT.Validation][YYYYMMDDHHMI][GlobalCounter].nidr

Example -

111111111ALRISNIDRData.ERCOTValidation201303180930101.nidr

### File 8 (ALRISNIDRData.ERCOTValidation)  – Record Layout

Header record – The HDR record contains information on the type of file, receiver DUNS, and the date and time that the response file was created.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “HDR”. | Alphanumeric (3) |
| Report Name | Mandatory | Mutually defined report definition. Hard Code “ALRISNIDRData.ERCOTValidation”. | Alphanumeric (36) |
| QSE DUNS Number | Mandatory | Qualified Scheduling Entity DUNS number receiving this response report information based on the original file submission. | Numeric (9 or 13) |
| Date / Timestamp | Mandatory | Date and time that response file is generated | Date/time YYYYMMDDHHMI |

ER3 Record – Used when an inbound record contains a value that fails business validation.  Contains a reference to the original record.  This is an optional record and can repeat multiple times.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “ER3”. | Alphanumeric (3) |
| ESI ID | Mandatory | Unique identifier assigned by QSE | Alphanumeric (80) |
| Row ID | Mandatory | Unique row identifier from the inbound file for row with error. | Numeric (5) |
| Error Description | Mandatory | Description of error. See list of error descriptions and long descriptions below. | Alphanumeric (80) |

Summary record – The SUM row contains a count of detail records included in the inbound file and the number of DET records in error.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “SUM.” | Alphanumeric (3) |
| Total Number of Rows | Mandatory | Total number of rows in the original ALRISNIDRData file. | Numeric (8) |
| Total Number of Processed Rows | Mandatory | Total number of rows that did not contain any errors in the original ALRISNIDRData file. | Numeric (8) |
| Total Number of Error Records | Mandatory | Total number of ER3 records in response file. | Numeric (8) |

### File 8 (ALRISNIDRData.ERCOTValidation)  – Error Descriptions and Long Descriptions

| Error Description | Long Description |
| --- | --- |
| No-site-data | No site data is in ERCOT system for ESIID |
| Invalid-ESIID | ESIID does not begin with DOE code for the submitting QSE |
| Invalid-Transaction-Type | Transaction Type not equal to 'Original Readings' |
| Invalid-MRE-DUNS | MRE DUNS not equal to submitter DUNS |
| Invalid-Sender-DUNS | Sender DUNS not equal to submitter DUNS |
| Invalid-Receiver-DUNS | Receiver DUNS not equal to 183529049 |
| Invalid-ClearingHouse-DUNS | ClearingHouseDUNS not equal to 183529049 |
| Invalid-Period-Start | Period Start is not a valid date |
| Invalid-Period-Stop | Period Stop is not a valid date or is before Period Start |
| Invalid-Meter-Type | Meter Type not equal to KHMON or K1MON |
| Invalid-Read-Status | Read Status not equal to A or E |
| Invalid-Total | Total missing or not numeric |
| Invalid-Period-Overlap | Period Dates Overlap One or More Previous Dates |
| Invalid-Empty-Value | Empty Value not blank. |

## Aggregate Load Resources in SCED (ALRIS) Advanced Metering Data (LSE) Files

The section covers the requirements for the Aggregate Load Resources in SCED Advanced Meter data file and response file.

All information must be sent in a comma delimited flat file format via North American Energy Standards Board (NAESB) and must contain all required fields.  All records sent in the file must be terminated by a Carriage Return Line Feed (CRLF).  The filename must contain the .lse extension.

ERCOT will validate that all mandatory data elements are present and meet formatting requirements.  ERCOT will send a response to the submitting QSE via NAESB.  ERCOT will inform the submitting QSE of any data fields that did not meet formatting requirements or of any required data fields that were not provided.  In instances where one or more fields fail formatting requirements or are missing required data, ERCOT will not process any of the file and the QSE should resubmit the entire file, including rows that passed validation in the original submission, plus corrected rows.

Note:  Data for more than one ESIID may be provided in the same file.  If data is resubmitted for an ESIID, it will replace data for the same days previously submitted based on the timestamp.

Instead of developing a new file format for this project, ERCOT is repurposing an existing format that is currently in use at ERCOT.  In order to make that format met the needs of Aggregate Load Resources in SCED, a number of columns in the format will be left blank since they are not applicable to this project.

### File 9 (ALRISIntervalData) - Record Layout for Interval Data file

This section contains information for the inbound .lse file from the QSEs.  One set of five header rows plus the twenty two through twenty four detail rows comprises a data set and communicates the interval data collected for one ESIID for a 24 hour period.

Sent from QSE to ERCOT

### File 9 (ALRISIntervalData) - Naming Convention

Convention-

[QSEDUNS][ALRISIntervalData][YYYYMMDDHHMI][GlobalCounter].lse

Example -

111111111ALRISIntervalData201303180930101.lse

### File 9 (ALRISIntervalData)  – Record Layout

Header Row One – This header row contains the ESI ID plus start and stop time for the data in this data record.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Sort Code | Mandatory | Hard Code “00000001”. | Numeric (8) |
| ESI ID | Mandatory | Unique Identifier assigned by QSE. | Alphanumeric (64) |
| Channel | Mandatory | Hard Code “4”. | Numeric (1) |
| Start Time | Mandatory | YYYYMMDD000000 Must be valid date and time value must be 000000 | Numeric (14) |
| Stop Time | Mandatory | YYYYMMDD235900 Must be valid date and time value must be 235900 | Numeric (14) |
| DST Participation | Mandatory | Hard Code “Y”. | Alphanumeric (1) |
| Record Flag | Mandatory | Use N for new data. Use Y to delete existing data or to indicate no data exists for day. | Alphanumeric (1) |

Header Row Two – This header row contains a number of fields not being used by Aggregate Load Resources in SCED.  These should be left blank.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Sort Code | Mandatory | Hard Code “00000002”. | Numeric (8) |
| Meter Start Reading | Leave Blank | Leave Blank | Leave Blank |
| Meter Stop Reading | Leave Blank | Leave Blank | Leave Blank |
| Meter Multiplier | Leave Blank | Leave Blank | Leave Blank |
| Empty value | Leave Blank | Leave Blank | Leave Blank |
| Pulse multiplier | Leave Blank | Leave Blank | Leave Blank |
| Empty value | Leave Blank | Leave Blank | Leave Blank |
| Seconds per Interval | Mandatory | Hard Code “900”. | Numeric (3) |
| Unit of Measure Code | Mandatory | Hard Code “01”. | Numeric (2) |
| Basic Unit Code | Leave Blank | Leave Blank | Leave Blank |
| Time Zones West of GMT | Leave Blank | Leave Blank | Leave Blank |
| Population | Leave Blank | Leave Blank | Leave Blank |
| Weight | Leave Blank | Leave Blank | Leave Blank |
| Time Zone Standard Name | Leave Blank | Leave Blank | Leave Blank |

Header Row Three – This header row contains a number of fields not being used by Aggregate Load Resources in SCED.  These should be left blank.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Sort Code | Mandatory | Hard Code “00000003”. | Numeric (8) |
| Descriptor | Leave Blank | Leave Blank | Leave Blank |

Header Row Four – This header row contains the timestamp of the read.  This value will determine which read will ‘win’ for a day if there are multiple reads.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Sort Code | Mandatory | Hard Code “00000004”. | Numeric (8) |
| Timestamp | Mandatory | Timestamp of read. This value will determine which read will ‘win’ for a day if there are multiple reads. YYYYMMDDHHMMSSMM (24-hour) | Leave Blank |
| Origin | Mandatory | Hard Code “M”. | Alphanumeric (1) |

Header Row Thirty – This header row contains the DUNS numbers associated with the ESI ID.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Sort Code | Mandatory | Hard Code “00000030”. | Numeric (8) |
| Name Value Pairs | Mandatory | Hard Code “ATTRIBUTE_VALUE_PAIRS”. | Alphanumeric (16) |
| MRE DUNS Number | Mandatory | Must be MRE= followed by DUNS number. MRE=666666666 | Alphanumeric (16) |
| TDSP DUNS Number | Mandatory | Must be Sender= followed by DUNS number. Sender=666666666 | Alphanumeric (20) |
| ERCOT DUNS Number | Mandatory | Must be Receiver=183529049. Receiver=183529049 | Alphanumeric (18) |
| CR DUNS Number | Optional | Must be blank or REP= followed by DUNS Number REP=111111111 | Alphanumeric (17) |

Detailed Record Rows 10000000 through 10000024 – Detail rows contain interval data.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Sort Code | Mandatory | 10000000 through 10000024 Each row must contain four 15-minute interval sets. For a 92 interval day the data records will go through row 22 (10000022). For a 96 interval day the data records will go through row 23 (10000023). For a 100 interval day the data records will go through row 24 (10000024). | Numeric (8) |
| Interval Value | Mandatory | KWh for the interval, maximum of 3 digits to the right of the decimal and must be a positive value | Numeric (10) |
| Status Code | Mandatory | A or E A = Actual, E = Estimate | Alphanumeric (1) |
| Empty Value | Leave Blank | Leave Blank | Leave Blank |
| Interval Value | Mandatory | KWh for the interval, maximum of 3 digits to the right of the decimal and must be a positive value | Numeric (10) |
| Status Code | Mandatory | A or E A = Actual, E = Estimate | Alphanumeric (1) |
| Empty Value | Leave Blank | Leave Blank | Leave Blank |
| Interval Value | Mandatory | KWh for the interval, maximum of 3 digits to the right of the decimal and must be a positive value | Numeric (10) |
| Status Code | Mandatory | A or E A = Actual, E = Estimate | Alphanumeric (1) |
| Empty Value | Leave Blank | Leave Blank | Leave Blank |
| Interval Value | Mandatory | KWh for the interval, maximum of 3 digits to the right of the decimal and must be a positive value | Numeric (10) |
| Status Code | Mandatory | A or E A = Actual, E = Estimate | Alphanumeric (1) |
| Empty Value | Leave Blank | Leave Blank | Leave Blank |

### File 10 (ALRISIntervalData.ERCOTValidation) - Record Layout for Interval Data file

This section contains information for ERCOT’s response to the interval data sent by QSE.  All information will be sent in a comma delimited flat file format via North American Energy Standards Board (NAESB) and will contain all required fields.  All records sent in the file will be terminated by a Carriage Return Line Feed (CRLF).  The filename will contain the .lse extension.

Sent from ERCOT to QSE

### File 10 (ALRISIntervalData.ERCOTValidation) - Naming Convention

Convention-

[QSEDUNS][ALRISIntervalData.ERCOTValidation][YYYYMMDDHHMI][GlobalCounter].lse

Example -

111111111ALRISIntervalData.ERCOTValidation201303180930101.lse

### File 10 (ALRISIntervalData.ERCOTValidation) – Record Layout

Header record – The HDR record contains information on the type of file, receiver DUNS, and the date and time that the response file was created.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “HDR”. | Alphanumeric (3) |
| Report Name | Mandatory | Mutually defined report definition. Hard Code “ALRISIntervalData.ERCOTValidation”. | Alphanumeric (36) |
| QSE DUNS Number | Mandatory | Qualified Scheduling Entity DUNS number receiving this response report information based on the original file submission. | Numeric (9 or 13) |
| Date / Timestamp | Mandatory | Date and time that response file is generated | Date/time YYYYMMDDHHMI |

ER3 Record – Used when an inbound record contains a value that fails business validation.  Contains a reference to the original record.  This is an optional record and can repeat multiple times.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “ER3”. | Alphanumeric (3) |
| ESI ID | Mandatory | Unique identifier assigned by QSE | Alphanumeric (80) |
| Start Time | Mandatory | Start time provided in Header Row One for the data set containing the error. YYYYMMDD000000 | Numeric (14) |
| Sort Code | Conditional | Unique row identifier from the inbound file for row with error. Required if communicating an error on a row from the inbound file. Not used if warning that data set is not being used because a data set for the same ESIID / Day combination exists with a more recent timestamp. Not used if entire file rejected because record limit exceeded. | Numeric (5) |
| Error Description | Mandatory | Description of error. See list of error descriptions and long descriptions below. | Alphanumeric (80) |

Summary record – The SUM row contains a count of detail records included in the inbound file and the number of DET records in error.  This is a mandatory record and it does not repeat.

| Data Element | Mandatory / Optional | Comments | Format |
| --- | --- | --- | --- |
| Record Type | Mandatory | Record Tag “SUM.” | Alphanumeric (3) |
| Total Number of Data Sets Provided | Mandatory | Total number of data sets in the original ALRISIntervalData file. A data set is defined as the 5 header rows plus the 22 to 24 accompanying detail rows. | Numeric (8) |
| Total Number of Data Sets Processed | Mandatory | Total number of data sets that did not contain any errors in the original ALRISIntervalData file. A data set is defined as the 5 header rows plus the 22 to 24 accompanying detail rows. | Numeric (8) |
| Total Number of Error Records | Mandatory | Total number of ER3 Records contained in response file. | Numeric (8) |

### File 10 (ALRISIntervalData.ERCOTValidation) – Error Descriptions and Long Descriptions

| Error Description | Long Description |
| --- | --- |
| Record-Limit-Exceeded | Record count exceeds file limit of 50,000 records. |
| Invalid-Sort-Code | Sort code not allowed |
| Sort-Code-Out-Of-Order | Sort codes not in ascending order |
| Sort-Code-Missing | Record with required sort code missing |
| Invalid-Number-Of-Fields | Number of fields on record not consistent with number required for sort code |
| Invalid-ESIID | No site data for ESIID in ERCOT system |
| Invalid-Channel | Channel code not 4 |
| Invalid-Start-Time | Start date not in valid date format or occurs in future. And/or time not equal to 000000 |
| Invalid-Stop-Time | Stop Date not equal to Start Date or Time not equal to 235900 |
| Invalid-DST | DST Participation not Y |
| Invalid-Record-Flag | Invalid Record Flag not Y or N |
| Invalid-Period-Stop | Period Stop is not a valid date |
| Invalid-Read-Status | Read Status not equal to A or E |
| Invalid-Seconds-Per-Interval | Seconds per interval not equal to 900 |
| Invalid-UOM | Unit of measure code not equal to 01 |
| Invalid-Timestamp | Timestamp not in valid date/time format |
| Invalid-Origin | Origin code not M |
| Invalid-Name-Value-Pairs | Name Value Pairs not equal to ATTRIBUTE_VALUE_PAIRS |
| Invalid-MRE-DUNS | MRE DUNS not equal to MRE={submitter DUNS} |
| Invalid-TDSP-DUNS | TDSP DUNS not equal to Sender={submitter DUNS} |
| Invalid-ERCOT-DUNS | ERCOT DUNS number not equal to Receiver=183529049 |
| Invalid-Interval-Value | Interval value negative, not numeric, or too many decimal values |
| Invalid-Status-Code | Interval Status Code not equal to A or E |
| Invalid-Empty-Value | Empty Value not blank |
| Invalid-Day | Incorrect number of intervals for a day |
| Interval-data-ignored | Interval data with a more recent time stamp has been submitted |