RUC options

| Option Description | Description | Pros | Cons | Language |
| --- | --- | --- | --- | --- |
| Shifting mu by a factor of standard deviation (Augmented ORDC) | Shifts the ORDC curve by a standard deviation under set parameters | Increases price and incents generators to come on line and reduces the need for RUCing | Timing and sequence of when RUC decisions have to be made vs. when augmentation would become effective (possibly fixed with good advance notice) Enhance the differences in shape as cross over time periods Shift startup risk from generators to buyers Disruption (reduced liquidity) in the bilateral markets Divorcing price signals from adder from available reserves Incentives created relative to the buyback | Draft OBD available |
| Shifting mu by fixed MW amount |  | Increases price and incents generators to come on line and reduces the need for RUCing | Timing and sequence of when RUC decisions have to be made vs. when augmentation would become effective (possibly fixed with good advance notice) Enhance the differences in shape as cross over time periods Shift startup risk from generators to buyers Disruption (reduced liquidity) in the bilateral markets Divorcing price signals from adder from available reserves Incentives created relative to the buyback | Draft OBD available |
| Removing RUC HSL from Rs |  | Prevents out of merit resources from suppressing the adder for competitive resources Reduces RUC short charge allocation costs | Divorcing price signals from adder from available reserves Incentives created relative to the buyback | Developed as Luminant Comments on NPRR598 would need updating to OBD language |
| SCED pricing run | Relaxing LSL down to zero, figuring out what system lambda would have been | Directly adjusts system lambda for the impact of zero to LSL | Implementation time could be longer | Draft NPRR developed |
| Increasing AS procurement during risk periods |  | Would replace need for RUC commitment with market-based products | ERCOT discretion needed Decrease ORDC based on AS buy May increase the HASL issue | AS procurement methodology change would be needed |