1-PAGER:  PATH TO LOAD PARTICIPATION IN SCED:

- Identify the load site(s) that will comprise the Controllable Load Resource (CLR)

  - May be a single site Load or an aggregation (ALR)

  - If ALR, all sites must be located within same ERCOT Load Zone

- Create a Resource Entity – legal entity with ownership/control of the Resource

  - ERCOT Client Services can assist

- Establish a QSE relationship

  - Must be “Level 4” QSE with capability of representing a Resource

  - Level 4 functions may be performed in-house or subcontracted thru agency agreement

- Complete Resource Asset Registration Form (RARF) and submit to ERCOT

  - Process typically takes 90+ days

- If ALR, provide data needed by ERCOT to establish and maintain ALR population

  - See OBD on Aggregate Load Resource participation

  - Submit data using “NAESB Server” process

    - Both docs:  http://www.ercot.com/calendar/2014/04/20140409-WMS

  - If in NOIE, collect and submit necessary historical meter data

- Confirm QSE telemetry is in place and functional

  - Multiple CLR-specific data points sent to ERCOT every 2 seconds

    - Protocols 6.5.5.2 (5)

  - Includes:

    - Net Power Consumption (= real-time Resource Load)

    - CLR up- and down-ramp rate values

    - If CLR will provide Non-spin, SPC+2

- Confirm QSE can submit energy bids to ERCOT on behalf of CLR

- Confirm QSE can submit Non-spin offers to ERCOT on behalf of CLR (if providing Non-spin)

- Confirm QSE can receive CLR-specific SCED Base Point Instructions

- Confirm QSE can receive Non-spin deployment instructions (if providing Non-spin)

- Complete appropriate qualification tests with ERCOT:

  - SCED dispatch

  - Non-spin (if providing)

- Confirm CLR can provide full MW value of energy bid within 5 minutes, subject to ramp rate limitations

- Confirm CLR can move load incrementally if energy bid is marginal

  - In such case, SCED may instruct CLR to consume power at any point between Low Power Consumption (LPC) and Max Power Consumption (MPC), subject to ramp rates