| NPRR Number | 605 | NPRR Title | Ancillary Service Make-Whole Payments Under a Watch |
| --- | --- | --- | --- |
| Date |  | May 8, 2014 |  |
| Submitter’s Information |  |  |  |
| Name |  | Seth Cochran on behalf of WMS |  |
| E-mail Address |  | seth.cochran@dc-energy.com |  |
| Company |  | DC Energy |  |
| Phone Number |  | 512-971-8767 |  |
| Cell Number |  |  |  |
| Market Segment |  | Not applicable. |  |

Comments

At its May 7, 2014 meeting, WMS voted to endorse NPRR605 as amended by the 5/5/14 Reliant comments as revised by WMS and recommend the cost associated with the payment to Resources with Ancillary Services Assignments per 6.7.6 (2) shall be assigned per the Real Time Ancillary Services Settlements in Protocol Section 6.7.3.  There were five abstentions from the Consumer and Cooperative (4) Market Segments.  All Market Segments were present for the vote.

Proposed Protocol Language Revision

## DEFINITIONS

Ancillary Service Assignment

A process to ensure that there is adequate Ancillary Service capacity available in the proper locations to serve ERCOT forecasted Load by assigning Ancillary Service to an On-Line Resource

6.5.9.3.3	Watch

(1)	A Watch is the third of four levels of communication issued by ERCOT in anticipation of a possible Emergency Condition.

(2)	ERCOT shall issue a Watch when ERCOT determines that:

(a)	Conditions have developed such that additional Ancillary Services are needed in the current Operating Period;

(b)	There are insufficient Ancillary Services or Energy Offers in the DAM or in a Supplemental Ancillary Services Market (SASM);

(c)	Market-based congestion management techniques embedded in SCED as specified in these Protocols will not be adequate to resolve transmission security violations;

(d)	Forced Outages or other abnormal operating conditions have occurred, or may occur that require operations with active violations of security criteria as defined in the Operating Guides unless a CMP exists;

(e)	ERCOT varies from timing requirements or omits one or more Day-Ahead or Adjustment Period and Real-Time procedures;

(f)	ERCOT varies from timing requirements or omits one or more scheduling procedures in the Real-Time process; or

(g)	The SCED process fails to reach a solution, whether or not ERCOT is using one of the measures specified in paragraph (3) of Section 6.5.9.2, Failure of the SCED Process.

(3)	With the issuance of a Watch pursuant to paragraph (2)(a) above, ERCOT may exercise its authority to immediately procure the following services from existing offers:

(a)	Regulation Services;

(b)	RRS services; and

(c)	Non-Spin services.

(4)	If the actions in paragraph (3) above do not relieve the insufficiency described in paragraph (2)(a) above, then ERCOT may issue an Ancillary Service Assignment to Resources certified to provide the insufficient service, even though there is not an existing Ancillary Service Offer for that Resource.

(5)	If ERCOT issues a Watch because insufficient Ancillary Service Offers were received in the DAM, and if the Watch does not result in sufficient offer and the DAM is executed with insufficient offers, then ERCOT shall acquire the insufficient amount of Ancillary Services in the Day-Ahead Reliability Unit Commitment (DRUC) and shall issue Dispatch Instructions to the QSEs for Resources that were RUC-committed to provide Ancillary Services, informing them of the requirement that the Resources be prepared to provide those Ancillary Services.

[NPRR589: Replace paragraph (5) above with the following upon system implementation:] (5) If ERCOT issues a Watch because insufficient Ancillary Service Offers were received in the DAM, and if the Watch does not result in sufficient offer and the DAM is executed with insufficient offers, then ERCOT may acquire the insufficient amount of Ancillary Services in the Day-Ahead Reliability Unit Commitment (DRUC) and shall issue Dispatch Instructions to the QSEs for Resources that were RUC-committed to provide Ancillary Services, informing them of the requirement that the Resources be prepared to provide those Ancillary Services.

[Upon implementation of SWCAP of $9,000 re-insert 6.5.9.3.3(6) and renumber. (6) If ERCOT issues a Watch because insufficient Ancillary Service Offers were received in a SASM, and if the Watch does not result in sufficient offer and the SASM is executed with insufficient offers, then ERCOT shall acquire the insufficient amount of Ancillary Services in the next HRUC and shall issue Dispatch Instructions to the QSEs for Resources that were RUC-committed to provide Ancillary Services, informing them of the requirement that the Resources be prepared to provide those Ancillary Services.

[NPRR589: Delete paragraph (6) above and renumber accordingly upon system implementation.]

(7)	ERCOT shall post the Watch message electronically to the MIS Public Area and shall provide verbal notice to all TSPs and QSEs via the Hotline.  Corrective actions identified by ERCOT must be communicated through Dispatch Instructions to all TSPs, DSPs and QSEs required to implement the corrective action.  Each QSE shall immediately notify the Market Participants that it represents of the Watch.  To minimize the effects on the ERCOT System, each TSP or DSP shall identify and prepare to implement actions, including restoration of transmission lines as appropriate and preparing for Load shedding.  ERCOT may instruct TSPs or DSPs to reconfigure ERCOT System elements as necessary to improve the reliability of the ERCOT System.  On notice of a Watch, each QSE, TSP, and DSP shall prepare for an Emergency Condition in case conditions worsen.  ERCOT may require information from QSEs representing Resources regarding the Resources’ fuel capabilities.  Requests for this type of information shall be for a time period of no more than seven days from the date of the request.  The specific information that may be requested shall be defined in the Operating Guides.  QSEs representing Resources shall provide the requested information in a timely manner, as defined by ERCOT at the time of the request.

6.7.3	Adjustments to Cost Allocations for Ancillary Services Procurement

(1)	Each QSE, for which ERCOT purchases Ancillary Service capacity in the DAM and SASMs (if any), is charged for the QSE’s share of the net costs incurred for each service.  For each QSE, its share of the DAM costs has been calculated in Section 4.6.4, Settlement of Ancillary Services Procured in the DAM; its share of the net total costs incurred in both DAM and SASMs less its DAM charge is calculated in this section.

(2)	For Reg-Up, if applicable:

(a)	The net total costs for Reg-Up for a given Operating Hour is calculated as follows:

RUCOSTTOT	=	(-1) * (Σm(RTPCRUAMTTOT m) + PCRUAMTTOT  + RUFQAMTTOT+RUASATOT)

Where:

Total payment of SASM-procured capacity for Reg-Up by market

RTPCRUAMTTOT m	=	ΣqRTPCRUAMT q, m

Total payment of DAM-procured capacity for Reg-Up

PCRUAMTTOT  	= 	ΣqPCRUAMT q

Total charge of failure on Ancillary Service Supply Responsibility for Reg-Up

RUFQAMTTOT	=	ΣqRUFQAMT q

Total payment for undeployed Reg-Up Ancillary Service Assignments

RUASATOT   = (Σi=4(RTASMWAMTTOT * RUUNDRS))

Where:  RUUNDRS =  ΣqRTUNDASRU q,r,p  / ΣqRTUNDASq,r,p

And: RTUNDAS q, r, p =RTUNDASRU q,r,p  + RTUNDASRR q,r,p  +  RTUNDASRD q,r,p

Total payment of SASM procured capacity for Reg-Up by QSE

RTPCRUAMTQSETOT q = ΣmRTPCRUAMT q, m

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RUCOSTTOT | $ | Reg-Up Cost Total—The net total costs for Reg-Up for the hour. |
| RTPCRUAMTTOT m | $ | Procured Capacity for Reg-Up Amount Total by market—The total payments to all QSEs for the Ancillary Service Offers cleared in the market m for Reg-Up, for the hour. |
| RTPCRUAMT q, m | $ | Procured Capacity for Reg-Up Amount per QSE by market—The payment to QSE q for its Ancillary Service Offers cleared in the market m for Reg-Up, for the hour. |
| RUASATOT | $ | Reg-Up Ancillary Service Assignments Total – The total payments to all QSEs for Regulation Up Assignments, as described in 6.7.6 Make-Whole Payment for Undeployed Ancillary Service Assignment during Watch, for the hour |
| RUFQAMTTOT | $ | Reg-Up Failure Quantity Amount Total—The total charges to all QSEs for their capacity associated with failures and reconfiguration reductions on their Ancillary Service Supply Responsibilities for Reg-Up, for the hour. |
| RUFQAMT q | $ | Reg-Up Failure Quantity Amount per QSE—The charge to QSE q for its total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for Reg-Up, for the hour. |
| RTASMWAMTTOT | $ | Real-Time Resource Ancillary Service Make-Whole Amount—The total payments to all QSEs for Ancillary Service Assignments under the Real-Time Ancillary Service make-whole for each 15-minute Settlement Interval. |
| RTUNDASRU q,r,p | MW | Regulation Up Undeployed Assignment- The quantity of undeployed Regulation Up Ancillary Service assigned under a Watch to a QSE and Resource for the 15-minute Settlement Interval. |
| RTUNDASRR q,r,p | MW | Responsive Reserve Undeployed Assignment - The quantity of undeployed Responsive Reserve Ancillary Service assigned under a Watch to a QSE and Resource for 15-minute Settlement Interval. |
| RTUNDASRD q,r,p | MW | Regulation Down Undeployed Assignment - The quantity of undeployed Regulation Down Ancillary Service assigned under a Watch to a QSE and Resource for 15-minute Settlement Interval. |
| RTUNDAS q, r, p | MW | Real-Time Undeployed Ancillary Service Schedule per Resource - The Real-Time Undeployed Ancillary Service Schedule pursuant to the Ancillary Service awards for Reg-Up, Reg-Down, and RRS for the Resources due to Ancillary Service Assignment during a Watch as set forth in paragraph (4) of Section 6.5.9.3.3, for the QSE q, for the 15-minute Settlement Interval.. |
| RUUNDRS | none | Regulation Up Undeployed Ratio Share- The percentage of undeployed assigned Regulation-Up that contributed to the total assigned Ancillary Service that was undeployed for the 15-minute Settlement Interval. |
| RTPCRUAMTQSETOT q | $ | Procured Capacity for Reg-Up Amount Total per QSE—The total payments to a QSE in all SASM markets for the Ancillary Service Offers cleared for Reg-Up Service, for the hour. |
| PCRUAMT q | $ | Procured Capacity for Reg-Up Amount per QSE in DAM—The DAM Reg-Up Service payment for QSE q for the hour. |
| PCRUAMTTOT | $ | Procured Capacity for Reg-Up Amount Total in DAM—The total of the DAM Reg-Up payments for all QSEs for the hour. |
| q | none | A QSE. |
| m | none | A SASM for the given Operating Hour. |

(b)	Each QSE’s share of the net total costs for Reg-Up for the Operating Hour is calculated as follows:

RUCOST q	=	RUPR * RUQ q

Where:

RUPR	=	RUCOSTTOT / RUQTOT

RUQTOT	=	ΣqRUQ q

RUQ q	=	RUO q – SARUQ q

RUO q	=	(WARUTOT  + Σq(SARUQ q + Σm(RTPCRU q, m) + PCRU q – RURP q – RUFQ q)) * HLRS q + RURP q

SARUQ q	=	DASARUQ q + RTSARUQ q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RUCOST q | $ | Reg-Up Cost per QSE—QSE q’s share of the net total costs for Reg-Up, for the hour. |
| RUPR | $/MW per hour | Reg-Up Price—The price for Reg-Up calculated based on the net total costs for Reg-Up, for the hour. |
| RUCOSTTOT | $ | Reg-Up Cost Total—The net total costs for Reg-Up for the hour. See item (a) above. |
| RUQTOT | MW | Reg-Up Quantity Total—The sum of every QSE’s portion of its Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour. |
| RUQ q | MW | Reg-Up Quantity per QSE—The portion of QSE q’s Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour. |
| RUO q | MW | Reg-Up Obligation per QSE—The Ancillary Service Obligation of QSE q, for the hour. |
| DASARUQ q | MW | Day-Ahead Self-Arranged Reg-Up Quantity per QSE—The self-arranged Reg-Up quantity submitted by QSE q before 1000 in the Day-Ahead. |
| RTSARUQ q | MW | Self-Arranged Reg-Up Quantity per QSE for all SASMs—The sum of all self-arranged Reg-Up quantities submitted by QSE q for all SASMs. |
| RTPCRU q, m | MW | Procured Capacity for Reg-Up per QSE by market—The MW portion of QSE q’s Ancillary Service Offers cleared in the market m to provide Reg-Up, for the hour. |
| WARUTOT | MW | Watch Assigned Regulation Up Quantity- The total market wide quantity of Regulation Up Ancillary Service assigned under a Watch for the hour. |
| RUFQ q | MW | Reg-Up Failure Quantity per QSE—QSE q’s total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for Reg-Up, for the hour. |
| HLRS q | none | The Hourly Load Ratio Share calculated for QSE q for the hour. See Section 6.6.2.3, QSE Load Ratio Share for an Operating Hour. |
| RURP q | MW | Reg-Up Replacement per QSE—The total Reg-Up capacity that was a portion of the Ancillary Service Supply Responsibility of QSE q but is replaced in a SASM for the hour. |
| PCRU q | MW | Procured Capacity for Reg-Up per QSE in DAM—The total Reg-Up Service capacity quantity awarded to QSE q in the DAM for all the Resources represented by the QSE for the hour. |
| SARUQ q | MW | Total Self-Arranged Reg-Up Quantity per QSE for all markets—The sum of all self-arranged Reg-Up quantities submitted by QSE q for DAM and all SASMs. |
| q | none | A QSE. |
| m | none | A SASM for the given Operating Hour. |

(c)	The adjustment to each QSE’s DAM charge for the Reg-Up for the Operating Hour, due to changes during the Adjustment Period or Real-Time operations, is calculated as follows:

RTRUAMT q	=	RUCOST q – DARUAMT q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTRUAMT q | $ | Real-Time Reg-Up Amount per QSE—The adjustment to QSE q’s share of the costs for Reg-Up, for the hour. |
| RUCOST q | $ | Reg-Up Cost per QSE—QSE q’s share of the net total costs for Reg-Up, for the hour. |
| DARUAMT q | $ | Day-Ahead Reg-Up Amount per QSE—QSE q’s share of the DAM cost for Reg-Up, for the hour. |
| q | none | A QSE. |

(3)	For Reg-Down, if applicable:

(a)	The net total costs for Reg-Down for a given Operating Hour is calculated as follows:

RDCOSTTOT	=	(-1) * (Σm(RTPCRDAMTTOT m + PCRDAMTTOT + RDFQAMTTOT+ RDASATOT)

Where:

Total payment of SASM-procured capacity for Reg-Down by market

RTPCRDAMTTOT m	=	ΣqRTPCRDAMT q, m

Total payment of DAM-procured capacity for Reg-Down

PCRDAMTTOT	=	 ΣqPCRDAMT q

Total charge of failure on Ancillary Service Supply Responsibility for Reg-Down

RDFQAMTTOT	=	ΣqRDFQAMT q

Total payment for undeployed Reg-Down Ancillary Service Assignments

RDASATOT   = (Σi=4(RTASMWAMTTOT * RDUNDRS))

Where:  RDUNDRS =  ΣqRTUNDASRD q,r,p  / ΣqRTUNDASq,r,p

And: RTUNDAS q, r, p =RTUNDASRU q,r,p  + RTUNDASRR q,r,p  + RTUNDASRD q,r,p

Total payment of SASM procured capacity for Reg-Down by QSE

RTPCRDAMTQSETOT q = ΣmRTPCRDAMT q, m

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RDCOSTTOT | $ | Reg-Down Cost Total—The net total costs for Reg-Down for the hour. |
| RTPCRDAMTTOT m | $ | Procured Capacity for Reg-Down Amount Total by market—The total payments to all QSEs for the Ancillary Service Offers cleared in the market m for Reg-Down, for the hour. |
| RTPCRDAMT q, m | $ | Procured Capacity for Reg-Down Amount per QSE by market—The payment to QSE q for its Ancillary Service Offers cleared in the market m for Reg-Down, for the hour. |
| RDFQAMTTOT | $ | Reg-Down Failure Quantity Amount Total—The total charges to all QSEs for their capacity associated with failures on their Ancillary Service Supply Responsibilities for Reg-Down, for the hour. |
| RDFQAMT q | $ | Reg-Down Failure Quantity Amount per QSE—The charge to QSE q for its total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for Reg-Down, for the hour. |
| RTPCRDAMTQSETOT q | $ | Procured Capacity for Reg-Down Amount Total per QSE—The total payments to a QSE in all SASM markets for the Ancillary Service Offers cleared for Reg-Down Service, for the hour. |
| PCRDAMT q | $ | Procured Capacity for Regulation Down Amount per QSE for DAM—The DAM Reg-Down Service payment for QSE q for the hour. |
| PCRDAMTTOT | $ | Procured Capacity for Reg-Down Amount Total in DAM—The total of the DAM Reg-Down payments for all QSEs for the hour. |
| RTASMWAMTTOT | $ | Real-Time Resource Ancillary Service Make-Whole Amount—The total payments to all QSEs for Ancillary Service Assignments under the Real-Time Ancillary Service make-whole for each 15-minute Settlement Interval. |
| RDASATOT | $ | Reg-Down Ancillary Service Assignments Total – The total payments to all QSEs for Regulation Down Assignments, as described in 6.7.6 Make-Whole Payment for Undeployed Ancillary Service Assignment during Watch, for the hour |
| RTUNDASRU q,r,p | MW | Regulation Up Undeployed Assignment- The quantity of undeployed Regulation Up Ancillary Service assigned under a Watch to a QSE and Resource for the 15-minute Settlement Interval. |
| RTUNDASRR q,r,p | MW | Responsive Reserve Undeployed Assignment - The quantity of undeployed Responsive Reserve Ancillary Service assigned under a Watch to a QSE and Resource for 15-minute Settlement Interval. |
| RTUNDASRD q,r,p | MW | Regulation Down Undeployed Assignment - The quantity of undeployed Regulation Down Ancillary Service assigned under a Watch to a QSE and Resource for 15-minute Settlement Interval. |
| RTUNDAS q, r, p | MW | Real-Time Undeployed Ancillary Service Schedule per Resource - The Real-Time Undeployed Ancillary Service Schedule pursuant to the Ancillary Service awards for Reg-Up, Reg - Down and RRS for the Resources due to Ancillary Service Assignment during a Watch as set forth in paragraph (4) of Section 6.5.9.3.3, for the QSE q, for the 15-minute Settlement Interval.. |
| RDUNDRS | none | Regulation Down Undeployed Ratio Share- The percentage of undeployed assigned Regulation-Down that contributed to the total assigned Ancillary Service that was undeployed for the 15-minute Settlement Interval. |
| q | none | A QSE. |
| m | none | A SASM for the given Operating Hour. |

(b)	Each QSE’s share of the net total costs for Reg-Down for the Operating Hour is calculated as follows:

RDCOST q	=	RDPR * RDQ q

Where:

RDPR	=	RDCOSTTOT / RDQTOT

RDQTOT	=	ΣqRDQ q

RDQ q	=	RDO q – SARDQ q

RDO q	=	(WARDTOT  + Σq(SARDQ q + Σm(RTPCRD q, m) + PCRD q – RDRP q – RDFQ q)) * HLRS q + RDRP q

SARDQ q	=	DASARDQ q + RTSARDQ q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RDCOST q | $ | Reg-Down Cost per QSE—QSE q’s share of the net total costs for Reg-Down, for the hour. |
| RDPR | $/MW per hour | Reg-Down Price—The price for Reg-Down calculated based on the net total costs for Reg-Down, for the hour. |
| RDCOSTTOT | $ | Reg-Down Cost Total—The net total costs for Reg-Down for the hour. See item (a) above. |
| RDQTOT | MW | Reg-Down Quantity Total—The sum of every QSE’s portion of its Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour. |
| RDQ q | MW | Reg-Down Quantity per QSE—The portion of QSE q’s net Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour. |
| RDO q | MW | Reg-Down Obligation per QSE—The Ancillary Service Obligation of QSE q, for the hour. |
| DASARDQ q | MW | Self-Arranged Reg-Down Quantity per QSE for DAM—The self-arranged Reg-Down quantity submitted by QSE q before 1000 in the Day-Ahead. |
| RTSARDQ q | MW | Self-Arranged Reg-Down Quantity per QSE for all SASMs—The sum of all self-arranged Reg-Down quantities submitted by QSE q for all SASMs. |
| RTPCRD q, m | MW | Procured Capacity for Reg-Down per QSE by market—The MW portion of QSE q’s Ancillary Service Offers cleared in the market m to provide Reg-Down, for the hour. |
| RDFQ q | MW | Reg-Down Failure Quantity per QSE—QSE q’s total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for Reg-Down, for the hour. |
| HLRS q |  | The Hourly Load Ratio Share calculated for QSE q for the hour. See Section 6.6.2.3. |
| RDRP q | MW | Reg-Down Replacement per QSE per market—The total Reg-Down capacity that was a portion of the Ancillary Service Supply Responsibility of QSE q but is replaced in a SASM, for the hour. |
| PCRD q | MW | Procured Capacity for Reg-Down per QSE in DAM—The total Reg-Down Service capacity quantity awarded to QSE q in the DAM for all the Resources represented by the QSE for the hour. |
| SARDQ q | MW | Total Self-Arranged Reg-Down Quantity per QSE for all markets—The sum of all self-arranged Reg-Down quantities submitted by QSE q for DAM and all SASMs. |
| WARDTOT | MW | Watch Assigned Regulation Down Quantity- The total market wide quantity of Regulation Down Ancillary Service assigned under a Watch for the hour. |
| q | none | A QSE. |
| m | none | A SASM for the given Operating Hour. |

(c)	The adjustment to each QSE’s DAM charge for the Reg-Down for the Operating Hour, due to changes during the Adjustment Period or Real-Time operations, is calculated as follows:

RTRDAMT q	=	RDCOST q – DARDAMT q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTRDAMT q | $ | Real-Time Reg-Down Amount per QSE—The adjustment to QSE q’s share of the costs for Reg-Down, for the hour. |
| RDCOST q | $ | Reg-Down Cost per QSE—QSE q’s share of the net total costs for Reg-Down, for the hour. |
| DARDAMT q | $ | Day-Ahead Reg-Down Amount per QSE—QSE q’s share of the DAM cost for Reg-Down, for the hour. |
| q | none | A QSE. |

(4)	For RRS service, if applicable:

(a)	The net total costs for RRS for a given Operating Hour is calculated as follows:

RRCOSTTOT 	=	(-1) * (Σm(RTPCRRAMTTOT m) + PCRRAMTTOT  + RRFQAMTTOT + RRASATOT)

Where:

Total payment of SASM-procured capacity for RRS by market

RTPCRRAMTTOT m	=	ΣqRTPCRRAMT q, m

Total payment of DAM-procured capacity for RRS

PCRRAMTTOT	=	ΣqPCRRAMT q

Total charge of failure on Ancillary Service Supply Responsibility for RRS

RRFQAMTTOT	=	ΣqRRFQAMT q

Total payment for undeployed RRS Ancillary Service Assignments

RRASATOT   = (Σi=4(RTASMWAMTTOT * RRUNDRS))

Where:  RRUNDRS = ΣqRTUNDASRR q,r,p /  ΣqRTUNDASq,r,p

And: RTUNDAS q, r, p =RTUNDASRU q,r,p  + RTUNDASRR q,r,p + RTUNDASRD q,r,p

Total payment of SASM procured capacity RRS Service by QSE

RTPCRRAMTQSETOT q = ΣmRTPCRRAMT q, m

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RRCOSTTOT | $ | Responsive Reserve Cost Total—The net total costs for RRS for the hour. |
| RTPCRRAMTTOT m | $ | Procured Capacity for Responsive Reserve Amount Total by market—The total payments to all QSEs for the Ancillary Service Offers cleared in the market m for RRS, for the hour. |
| RTPCRRAMT q, m | $ | Procured Capacity for Responsive Reserve Amount per QSE by market—The payment to QSE q for its Ancillary Service Offers cleared in the market m for RRS, for the hour. |
| RRASATOT | $ | Responsive Reserve Ancillary Service Assignments Total – The total payments to all QSEs for Responsive Reserve Assignments, as described in 6.7.6 Make-Whole Payment for Undeployed Ancillary Service Assignment during Watch, for the hour. |
| RRFQAMTTOT | $ | Responsive Reserve Failure Quantity Amount Total—The total charges to all QSEs for their capacity associated with failures and reconfiguration reductions on their Ancillary Service Supply Responsibilities for RRS, for the hour. |
| RTASMWAMTTOT | $ | Real-Time Resource Ancillary Service Make-Whole Amount—The total payments to all QSEs for Ancillary Service Assignments under the Real-Time Ancillary Service make-whole for each 15-minute Settlement Interval. |
| RTUNDASRU q,r,p | MW | Regulation Up Undeployed Assignment- The quantity of undeployed Regulation Up Ancillary Service assigned under a Watch to a QSE and Resource for the 15-minute Settlement Interval. |
| RTUNDASRR q,r,p | MW | Responsive Reserve Undeployed Assignment - The quantity of undeployed Responsive Reserve Ancillary Service assigned under a Watch to a QSE and Resource for 15-minute Settlement Interval. |
| RTUNDASRD q,r,p | MW | Regulation Down Undeployed Assignment - The quantity of undeployed Regulation Down Ancillary Service assigned under a Watch to a QSE and Resource for 15-minute Settlement Interval. |
| RTUNDAS q, r, p | MW | Real-Time Undeployed Ancillary Service Schedule per Resource - The Real-Time Undeployed Ancillary Service Schedule pursuant to the Ancillary Service awards for Reg-Up and RRS for the Resources due to Ancillary Service Assignment during a Watch as set forth in paragraph (4) of Section 6.5.9.3.3, for the QSE q, for the 15-minute Settlement Interval. |
| RRUNDRS | none | Responsive Reserve Undeployed Ratio Share- The percentage of undeployed assigned Responsive Reserve that contributed to the total assigned Ancillary Service that was undeployed for the 15-minute Settlement Interval. |
| RRFQAMT q | $ | Responsive Reserve Failure Quantity Amount per QSE—The charge to QSE q for its total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for RRS, for the hour. |
| RTPCRRAMTQSETOT q | $ | Procured Capacity for Responsive Reserve Amount Total per QSE—The total payments to a QSE in all SASM markets for the Ancillary Service Offers cleared for RRS, for the hour. |
| PCRRAMT q | $ | Procured Capacity for Responsive Reserve Amount per QSE for DAM—The DAM RRS payment for QSE q, for the hour. |
| PCRRAMTTOT | $ | Procured Capacity for Responsive Reserve Amount Total in DAM—The total of the DAM RRS payments for all QSEs, for the hour. |
| q | none | A QSE. |
| m | none | A SASM for the given Operating Hour. |

(b)	Each QSE’s share of the net total costs for RRS for the Operating Hour is calculated as follows:

RRCOST q	=	RRPR * RRQ q

Where:

RRPR	=	RRCOSTTOT / RRQTOT

RRQTOT	=	ΣqRRQ q

RRQ q	=	RRO q – SARRQ q

RRO q	=	(WARRTOT + Σq(SARRQq + Σm(RTPCRR q, m) + PCRR q – RRRP q – RRFQ q)) * HLRS q + RRRP q

SARRQ q	=	DASARRQ q + RTSARRQ q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RRCOST q | $ | Responsive Reserve Cost per QSE—QSE q’s share of the net total costs for RRS, for the hour. |
| RRPR | $/MW per hour | Responsive Reserve Price—The price for RRS calculated based on the net total costs for RRS, for the hour. |
| RRCOSTTOT | $ | Responsive Reserve Cost Total—The net total costs for RRS for the hour. See item (a) above. |
| RRQTOT | MW | Responsive Reserve Quantity Total—The sum of every QSE’s portion of its Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour. |
| RRQ q | MW | Responsive Reserve Quantity per QSE—The portion of QSE q’s Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour. |
| RRO q | MW | Responsive Reserve Obligation per QSE—The Ancillary Service Obligation of QSE q, for the hour. |
| DASARRQ q | MW | Day-Ahead Self-Arranged Responsive Reserve Quantity per QSE—The self-arranged RRS quantity submitted by QSE q before 1000 in the Day-Ahead. |
| RTSARRQ q | MW | Self-Arranged Responsive Reserve Quantity per QSE for all SASMs—The sum of all self-arranged RRS quantities submitted by QSE q for all SASMs. |
| RTPCRR q, m | MW | Procured Capacity for Responsive Reserve per QSE by market—The MW portion of QSE q’s Ancillary Service Offers cleared in the market m to provide RRS, for the hour. |
| WARRTOT | MW | Watch Assigned Responsive Reserve Quantity- The total market wide quantity of Responsive Reserve Ancillary Service assigned under a Watch for the hour. |
| RRFQ q | MW | Responsive Reserve Failure Quantity per QSE—QSE q’s total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for RRS, for the hour. |
| HLRS q | none | The Hourly Load Ratio Share calculated for QSE q for the hour. See Section 6.6.2.3. |
| RRRP q | MW | Responsive Reserve Replacement per QSE per market—The total RRS capacity that was a portion of the Ancillary Service Supply Responsibility of QSE q but is replaced in a SASM for the hour. |
| PCRR q | MW | Procured Capacity for Responsive Reserve per QSE in DAM—The total RRS capacity quantity awarded to QSE q in the DAM for all the Resources represented by the QSE for the hour. |
| SARRQ q | MW | Total Self-Arranged Responsive Reserve Quantity per QSE for all markets—The sum of all self-arranged RRS quantities submitted by QSE q for DAM and all SASMs. |
| q | none | A QSE. |
| m | none | A SASM for the given Operating Hour. |

(c)	The adjustment to each QSE’s DAM charge for the RRS for the Operating Hour, due to changes during the Adjustment Period or Real-Time operations, is calculated as follows:

RTRRAMT q	=	RRCOST q – DARRAMT q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTRRAMT q | $ | Real-Time Responsive Reserve Amount per QSE—The adjustment to QSE q’s share of the costs for RRS, for the hour. |
| RRCOST q | $ | Responsive Reserve Cost per QSE—QSE q’s share of the net total costs for RRS, for the hour. |
| DARRAMT q | $ | Day-Ahead Responsive Reserve Amount per QSE—QSE q’s share of the DAM cost for RRS, for the hour. |
| q | none | A QSE. |

(5)	For Non-Spin, if applicable:

(a)	The net total costs for Non-Spin for a given Operating Hour is calculated as follows:

NSCOSTTOT	=	(-1) * (Σm(RTPCNSAMTTOT m) + 
PCNSAMTTOT + NSFQAMTTOT)

Where:

Total payment of SASM-procured capacity for Non-Spin by market

RTPCNSAMTTOT m	=	ΣqRTPCNSAMT q, m

Total payment of DAM-procured capacity for Non-Spin

PCNSAMTTOT	=	ΣqPCNSAMT q

Total charge of failure on Ancillary Service Supply Responsibility for Non-Spin

NSFQAMTTOT	=	ΣqNSFQAMT q

Total payment of SASM procured capacity for Non-Spin by QSE

RTPCNSAMTQSETOTq = ΣmRTPCNSAMT q, m

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| NSCOSTTOT | $ | Non-Spin Cost Total—The net total costs for Non-Spin for the hour. |
| RTPCNSAMTTOT m | $ | Procured Capacity for Non-Spin Amount Total by market—The total payments to all QSEs for the Ancillary Service Offers cleared in the market m for Non-Spin, for the hour. |
| RTPCNSAMT q, m | $ | Procured Capacity for Non-Spin Amount per QSE by market—The payment to QSE q for its Ancillary Service Offers cleared in the market m for Non-Spin, for the hour. |
| NSFQAMTTOT | $ | Non-Spin Failure Quantity Amount Total—The total charges to all QSEs for their capacity associated with failures and reconfiguration reductions on their Ancillary Service Supply Responsibilities for Non-Spin, for the hour. |
| NSFQAMT q | $ | Non-Spin Failure Quantity Amount per QSE—The charge to QSE q for its total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for Non-Spin, for the hour. |
| RTPCNSAMTQSETOT q | $ | Procured Capacity for Non-Spin Amount Total per QSE—The total payments to a QSE in all SASM markets for the Ancillary Service Offers cleared for Non-Spin, for the hour. |
| PCNSAMT q | $ | Procured Capacity for Non-Spin Amount per QSE in DAM—The DAM Non-Spin payment for QSE q for the hour. |
| PCNSAMTTOT | $ | Procured Capacity for Non-Spin Amount Total in DAM—The total of the DAM Non-Spin payments for all QSEs for the hour. |
| q | none | A QSE. |
| m | none | A SASM for the given Operating Hour. |

(b)	Each QSE’s share of the net total costs for Non-Spin for the Operating Hour is calculated as follows:

NSCOST q	=	NSPR * NSQ q

Where:

NSPR	=	NSCOSTTOT / NSQTOT

NSQTOT	=	ΣqNSQ q

NSQ q	=	NSO q – SANSQ q

NSO q	=	Σq(SANSQ q + Σm(RTPCNS q, m) + PCNS q – NSRP q – NSFQ q) * HLRS q + NSRP q

SANSQ q	=	DASANSQ q + RTSANSQ q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| NSCOST q | $ | Non-Spin Cost per QSE—QSE q’s share of the net total costs for Non-Spin, for the hour. |
| NSPR | $/MW per hour | Non-Spin Price—The price for Non-Spin calculated based on the net total costs for Non-Spin, for the hour. |
| NSCOSTTOT | $ | Non-Spin Cost Total—The net total costs for Non-Spin for the hour. See item (a) above. |
| NSQTOT | MW | Non-Spin Quantity Total—The sum of every QSE’s portion of its Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour. |
| NSQ q | MW | Non-Spin Quantity per QSE—The portion of QSE q’s Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour. |
| NSO q | MW | Non-Spin Obligation per QSE—The Ancillary Service Obligation of QSE q, for the hour. |
| DASANSQ q | MW | Day-Ahead Self-Arranged Non-Spin Quantity per QSE for DAM—The self-arranged Non-Spin quantity submitted by QSE q before 1000 in the Day-Ahead. |
| RTSANSQ q | MW | Self-Arranged Non-Spin Quantity per QSE for all SASMs—The sum of all self-arranged Non-Spin quantities submitted by QSE q for all SASMs. |
| RTPCNS q, m | MW | Procured Capacity for Non-Spin per QSE by market—The MW portion of QSE q’s Ancillary Service Offers cleared in the market m to provide Non-Spin, for the hour. |
| NSFQ q | MW | Non-Spin Failure Quantity per QSE—QSE q’s total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for Non-Spin, for the hour. |
| HLRS q | none | The Hourly Load Ratio Share calculated for QSE q for the hour. See Section 6.6.2.3. |
| NSRP q | MW | Non-Spin Replacement per QSE per market—The total Non-Spin capacity that was a portion of the Ancillary Service Supply Responsibility of QSE q but is replaced in a SASM for the hour. |
| PCNS q | MW | Procured Capacity for Non-Spin Service per QSE in DAM—The total Non-Spin capacity quantity awarded to QSE q in the DAM for all the Resources represented by the QSE for the hour. |
| SANSQ q | MW | Total Self-Arranged Non-Spin Supplied Quantity per QSE for all markets—The sum of all self-arranged Non-Spin quantities submitted by QSE q for DAM and all SASMs. |
| q | none | A QSE. |
| m | none | A SASM for the given Operating Hour. |

(c)	The adjustment to each QSE’s DAM charge for the Non-Spin for the Operating Hour, due to changes during the Adjustment Period or Real-Time operations, is calculated as follows:

RTNSAMT q	=	NSCOST q – DANSAMT q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTNSAMT q | $ | Real-Time Non-Spin Amount per QSE—The adjustment to QSE q’s share of the costs for Non-Spin, for the hour. |
| NSCOST q | $ | Non-Spin Cost per QSE—QSE q’s share of the net total costs for Non-Spin, for the hour. |
| DANSAMT q | $ | Day-Ahead Non-Spin Amount per QSE—QSE q’s share of the DAM cost for Non-Spin, for the hour. |
| q | none | A QSE. |

[Upon implementation of System Wide Offer Cap (SWCAP) of $9,000 replace Section 6.7.3 above with the following:] 6.7.3 Adjustments to Cost Allocations for Ancillary Services Procurement (1) Each QSE, for which ERCOT purchases Ancillary Service capacity in the DAM and SASMs (if any), is charged for the QSE’s share of the net costs incurred for each service. For each QSE, its share of the DAM costs has been calculated in Section 4.6.4, Settlement of Ancillary Services Procured in the DAM; its share of the net total costs incurred in both DAM and SASMs less its DAM charge is calculated in this section. (2) For Reg-Up, if applicable: (a) The net total costs for Reg-Up for a given Operating Hour is calculated as follows: RUCOSTTOT = (-1) * (Σm(RTPCRUAMTTOT m) + PCRUAMTTOT + RUFQAMTTOT) Where: Total payment of SASM-procured capacity for Reg-Up by market RTPCRUAMTTOT m = ΣqRTPCRUAMT q, m Total payment of DAM-procured capacity for Reg-Up PCRUAMTTOT = ΣqPCRUAMT q Total charge of failure on Ancillary Service Supply Responsibility for Reg-Up RUFQAMTTOT = ΣqRUFQAMT q Total payment of SASM procured capacity for Reg-Up by QSE RTPCRUAMTQSETOT q = ΣmRTPCRUAMT q, m The above variables are defined as follows: Variable; Unit; Description. RUCOSTTOT; $; Reg-Up Cost Total—The net total costs for Reg-Up for the hour.. RTPCRUAMTTOT m; $; Procured Capacity for Reg-Up Amount Total by market—The total payments to all QSEs for the Ancillary Service Offers cleared in the market m for Reg-Up, for the hour.. RTPCRUAMT q, m; $; Procured Capacity for Reg-Up Amount per QSE by market—The payment to QSE q for its Ancillary Service Offers cleared in the market m for Reg-Up, for the hour.. RUFQAMTTOT; $; Reg-Up Failure Quantity Amount Total—The total charges to all QSEs for their capacity associated with failures and reconfiguration reductions on their Ancillary Service Supply Responsibilities for Reg-Up, for the hour.. RUFQAMT q; $; Reg-Up Failure Quantity Amount per QSE—The charge to QSE q for its total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for Reg-Up, for the hour.. RTPCRUAMTQSETOT q; $; Procured Capacity for Reg-Up Amount Total per QSE—The total payments to a QSE in all SASM markets for the Ancillary Service Offers cleared for Reg-Up Service, for the hour.. PCRUAMT q; $; Procured Capacity for Reg-Up Amount per QSE in DAM—The DAM Reg-Up Service payment for QSE q for the hour.. PCRUAMTTOT; $; Procured Capacity for Reg-Up Amount Total in DAM—The total of the DAM Reg-Up payments for all QSEs for the hour.. q; none; A QSE.. m; none; A SASM for the given Operating Hour.. (b) Each QSE’s share of the net total costs for Reg-Up for the Operating Hour is calculated as follows: RUCOST q = RUPR * RUQ q Where: RUPR = RUCOSTTOT / RUQTOT RUQTOT = ΣqRUQ q RUQ q = RUO q – SARUQ q RUO q = Σq(SARUQ q + Σm(RTPCRU q, m) + PCRU q – RURP q – RUFQ q) * HLRS q + RURP q SARUQ q = DASARUQ q + RTSARUQ q The above variables are defined as follows: Variable; Unit; Description. RUCOST q; $; Reg-Up Cost per QSE—QSE q’s share of the net total costs for Reg-Up, for the hour.. RUPR; $/MW per hour; Reg-Up Price—The price for Reg-Up calculated based on the net total costs for Reg-Up, for the hour.. RUCOSTTOT; $; Reg-Up Cost Total—The net total costs for Reg-Up for the hour. See item (a) above.. RUQTOT; MW; Reg-Up Quantity Total—The sum of every QSE’s portion of its Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour.. RUQ q; MW; Reg-Up Quantity per QSE—The portion of QSE q’s Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour.. RUO q; MW; Reg-Up Obligation per QSE—The Ancillary Service Obligation of QSE q, for the hour.. DASARUQ q; MW; Day-Ahead Self-Arranged Reg-Up Quantity per QSE—The self-arranged Reg-Up quantity submitted by QSE q before 1000 in the Day-Ahead.. RTSARUQ q; MW; Self-Arranged Reg-Up Quantity per QSE for all SASMs—The sum of all self-arranged Reg-Up quantities submitted by QSE q for all SASMs.. RTPCRU q, m; MW; Procured Capacity for Reg-Up per QSE by market—The MW portion of QSE q’s Ancillary Service Offers cleared in the market m to provide Reg-Up, for the hour.. RUFQ q; MW; Reg-Up Failure Quantity per QSE—QSE q’s total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for Reg-Up, for the hour.. HLRS q; none; The Hourly Load Ratio Share calculated for QSE q for the hour. See Section 6.6.2.3, QSE Load Ratio Share for an Operating Hour.. RURP q; MW; Reg-Up Replacement per QSE—The total Reg-Up capacity that was a portion of the Ancillary Service Supply Responsibility of QSE q but is replaced in a SASM for the hour.. PCRU q; MW; Procured Capacity for Reg-Up per QSE in DAM—The total Reg-Up Service capacity quantity awarded to QSE q in the DAM for all the Resources represented by the QSE for the hour.. SARUQ q; MW; Total Self-Arranged Reg-Up Quantity per QSE for all markets—The sum of all self-arranged Reg-Up quantities submitted by QSE q for DAM and all SASMs.. q; none; A QSE.. m; none; A SASM for the given Operating Hour.. (c) The adjustment to each QSE’s DAM charge for the Reg-Up for the Operating Hour, due to changes during the Adjustment Period or Real-Time operations, is calculated as follows: RTRUAMT q = RUCOST q – DARUAMT q The above variables are defined as follows: Variable; Unit; Description. RTRUAMT q; $; Real-Time Reg-Up Amount per QSE—The adjustment to QSE q’s share of the costs for Reg-Up, for the hour.. RUCOST q; $; Reg-Up Cost per QSE—QSE q’s share of the net total costs for Reg-Up, for the hour.. DARUAMT q; $; Day-Ahead Reg-Up Amount per QSE—QSE q’s share of the DAM cost for Reg-Up, for the hour.. q; none; A QSE.. (3) For Reg-Down, if applicable: (a) The net total costs for Reg-Down for a given Operating Hour is calculated as follows: RDCOSTTOT = (-1) * (Σm(RTPCRDAMTTOT m + PCRDAMTTOT + RDFQAMTTOT) Where: Total payment of SASM-procured capacity for Reg-Down by market RTPCRDAMTTOT m = ΣqRTPCRDAMT q, m Total payment of DAM-procured capacity for Reg-Down PCRDAMTTOT = ΣqPCRDAMT q Total charge of failure on Ancillary Service Supply Responsibility for Reg-Down RDFQAMTTOT = ΣqRDFQAMT q Total payment of SASM procured capacity for Reg-Down by QSE RTPCRDAMTQSETOT q = ΣmRTPCRDAMT q, m The above variables are defined as follows: Variable; Unit; Description. RDCOSTTOT; $; Reg-Down Cost Total—The net total costs for Reg-Down for the hour.. RTPCRDAMTTOT m; $; Procured Capacity for Reg-Down Amount Total by market—The total payments to all QSEs for the Ancillary Service Offers cleared in the market m for Reg-Down, for the hour.. RTPCRDAMT q, m; $; Procured Capacity for Reg-Down Amount per QSE by market—The payment to QSE q for its Ancillary Service Offers cleared in the market m for Reg-Down, for the hour.. RDFQAMTTOT; $; Reg-Down Failure Quantity Amount Total—The total charges to all QSEs for their capacity associated with failures on their Ancillary Service Supply Responsibilities for Reg-Down, for the hour.. RDFQAMT q; $; Reg-Down Failure Quantity Amount per QSE—The charge to QSE q for its total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for Reg-Down, for the hour.. RTPCRDAMTQSETOT q; $; Procured Capacity for Reg-Down Amount Total per QSE—The total payments to a QSE in all SASM markets for the Ancillary Service Offers cleared for Reg-Down Service, for the hour.. PCRDAMT q; $; Procured Capacity for Regulation Down Amount per QSE for DAM—The DAM Reg-Down Service payment for QSE q for the hour.. PCRDAMTTOT; $; Procured Capacity for Reg-Down Amount Total in DAM—The total of the DAM Reg-Down payments for all QSEs for the hour.. q; none; A QSE.. m; none; A SASM for the given Operating Hour.. (b) Each QSE’s share of the net total costs for Reg-Down for the Operating Hour is calculated as follows: RDCOST q = RDPR * RDQ q Where: RDPR = RDCOSTTOT / RDQTOT RDQTOT = ΣqRDQ q RDQ q = RDO q – SARDQ q RDO q = Σq(SARDQ q + Σm(RTPCRD q, m) + PCRD q – RDRP q – RDFQ q) * HLRS q + RDRP q SARDQ q = DASARDQ q + RTSARDQ q The above variables are defined as follows: Variable; Unit; Description. RDCOST q; $; Reg-Down Cost per QSE—QSE q’s share of the net total costs for Reg-Down, for the hour.. RDPR; $/MW per hour; Reg-Down Price—The price for Reg-Down calculated based on the net total costs for Reg-Down, for the hour.. RDCOSTTOT; $; Reg-Down Cost Total—The net total costs for Reg-Down for the hour. See item (a) above.. RDQTOT; MW; Reg-Down Quantity Total—The sum of every QSE’s portion of its Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour.. RDQ q; MW; Reg-Down Quantity per QSE—The portion of QSE q’s net Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour.. RDO q; MW; Reg-Down Obligation per QSE—The Ancillary Service Obligation of QSE q, for the hour.. DASARDQ q; MW; Self-Arranged Reg-Down Quantity per QSE for DAM—The self-arranged Reg-Down quantity submitted by QSE q before 1000 in the Day-Ahead.. RTSARDQ q; MW; Self-Arranged Reg-Down Quantity per QSE for all SASMs—The sum of all self-arranged Reg-Down quantities submitted by QSE q for all SASMs.. RTPCRD q, m; MW; Procured Capacity for Reg-Down per QSE by market—The MW portion of QSE q’s Ancillary Service Offers cleared in the market m to provide Reg-Down, for the hour.. RDFQ q; MW; Reg-Down Failure Quantity per QSE—QSE q’s total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for Reg-Down, for the hour.. HLRS q; The Hourly Load Ratio Share calculated for QSE q for the hour. See Section 6.6.2.3.. RDRP q; MW; Reg-Down Replacement per QSE per market—The total Reg-Down capacity that was a portion of the Ancillary Service Supply Responsibility of QSE q but is replaced in a SASM, for the hour.. PCRD q; MW; Procured Capacity for Reg-Down per QSE in DAM—The total Reg-Down Service capacity quantity awarded to QSE q in the DAM for all the Resources represented by the QSE for the hour.. SARDQ q; MW; Total Self-Arranged Reg-Down Quantity per QSE for all markets—The sum of all self-arranged Reg-Down quantities submitted by QSE q for DAM and all SASMs.. q; none; A QSE.. m; none; A SASM for the given Operating Hour.. (c) The adjustment to each QSE’s DAM charge for the Reg-Down for the Operating Hour, due to changes during the Adjustment Period or Real-Time operations, is calculated as follows: RTRDAMT q = RDCOST q – DARDAMT q The above variables are defined as follows: Variable; Unit; Description. RTRDAMT q; $; Real-Time Reg-Down Amount per QSE—The adjustment to QSE q’s share of the costs for Reg-Down, for the hour.. RDCOST q; $; Reg-Down Cost per QSE—QSE q’s share of the net total costs for Reg-Down, for the hour.. DARDAMT q; $; Day-Ahead Reg-Down Amount per QSE—QSE q’s share of the DAM cost for Reg-Down, for the hour.. q; none; A QSE.. (4) For RRS service, if applicable: (a) The net total costs for RRS for a given Operating Hour is calculated as follows: RRCOSTTOT = (-1) * (Σm(RTPCRRAMTTOT m) + PCRRAMTTOT + RRFQAMTTOT) Where: Total payment of SASM-procured capacity for RRS by market RTPCRRAMTTOT m = ΣqRTPCRRAMT q, m Total payment of DAM-procured capacity for RRS PCRRAMTTOT = ΣqPCRRAMT q Total charge of failure on Ancillary Service Supply Responsibility for RRS RRFQAMTTOT = ΣqRRFQAMT q Total payment of SASM procured capacity RRS Service by QSE RTPCRRAMTQSETOT q = ΣmRTPCRRAMT q, m The above variables are defined as follows: Variable; Unit; Description. RRCOSTTOT; $; Responsive Reserve Cost Total—The net total costs for RRS for the hour.. RTPCRRAMTTOT m; $; Procured Capacity for Responsive Reserve Amount Total by market—The total payments to all QSEs for the Ancillary Service Offers cleared in the market m for RRS, for the hour.. RTPCRRAMT q, m; $; Procured Capacity for Responsive Reserve Amount per QSE by market—The payment to QSE q for its Ancillary Service Offers cleared in the market m for RRS, for the hour.. RRFQAMTTOT; $; Responsive Reserve Failure Quantity Amount Total—The total charges to all QSEs for their capacity associated with failures and reconfiguration reductions on their Ancillary Service Supply Responsibilities for RRS, for the hour.. RRFQAMT q; $; Responsive Reserve Failure Quantity Amount per QSE—The charge to QSE q for its total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for RRS, for the hour.. RTPCRRAMTQSETOT q; $; Procured Capacity for Responsive Reserve Amount Total per QSE—The total payments to a QSE in all SASM markets for the Ancillary Service Offers cleared for RRS, for the hour.. PCRRAMT q; $; Procured Capacity for Responsive Reserve Amount per QSE for DAM—The DAM RRS payment for QSE q, for the hour.. PCRRAMTTOT; $; Procured Capacity for Responsive Reserve Amount Total in DAM—The total of the DAM RRS payments for all QSEs, for the hour.. q; none; A QSE.. m; none; A SASM for the given Operating Hour.. (b) Each QSE’s share of the net total costs for RRS for the Operating Hour is calculated as follows: RRCOST q = RRPR * RRQ q Where: RRPR = RRCOSTTOT / RRQTOT RRQTOT = ΣqRRQ q RRQ q = RRO q – SARRQ q RRO q = Σq(SARRQq + Σm(RTPCRR q, m) + PCRR q – RRRP q – RRFQ q) * HLRS q + RRRP q SARRQ q = DASARRQ q + RTSARRQ q The above variables are defined as follows: Variable; Unit; Description. RRCOST q; $; Responsive Reserve Cost per QSE—QSE q’s share of the net total costs for RRS, for the hour.. RRPR; $/MW per hour; Responsive Reserve Price—The price for RRS calculated based on the net total costs for RRS, for the hour.. RRCOSTTOT; $; Responsive Reserve Cost Total—The net total costs for RRS for the hour. See item (a) above.. RRQTOT; MW; Responsive Reserve Quantity Total—The sum of every QSE’s portion of its Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour.. RRQ q; MW; Responsive Reserve Quantity per QSE—The portion of QSE q’s Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour.. RRO q; MW; Responsive Reserve Obligation per QSE—The Ancillary Service Obligation of QSE q, for the hour.. DASARRQ q; MW; Day-Ahead Self-Arranged Responsive Reserve Quantity per QSE—The self-arranged RRS quantity submitted by QSE q before 1000 in the Day-Ahead.. RTSARRQ q; MW; Self-Arranged Responsive Reserve Quantity per QSE for all SASMs—The sum of all self-arranged RRS quantities submitted by QSE q for all SASMs.. RTPCRR q, m; MW; Procured Capacity for Responsive Reserve per QSE by market—The MW portion of QSE q’s Ancillary Service Offers cleared in the market m to provide RRS, for the hour.. RRFQ q; MW; Responsive Reserve Failure Quantity per QSE—QSE q’s total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for RRS, for the hour.. HLRS q; none; The Hourly Load Ratio Share calculated for QSE q for the hour. See Section 6.6.2.3.. RRRP q; MW; Responsive Reserve Replacement per QSE per market—The total RRS capacity that was a portion of the Ancillary Service Supply Responsibility of QSE q but is replaced in a SASM for the hour.. PCRR q; MW; Procured Capacity for Responsive Reserve per QSE in DAM—The total RRS capacity quantity awarded to QSE q in the DAM for all the Resources represented by the QSE for the hour.. SARRQ q; MW; Total Self-Arranged Responsive Reserve Quantity per QSE for all markets—The sum of all self-arranged RRS quantities submitted by QSE q for DAM and all SASMs.. q; none; A QSE.. m; none; A SASM for the given Operating Hour.. (c) The adjustment to each QSE’s DAM charge for the RRS for the Operating Hour, due to changes during the Adjustment Period or Real-Time operations, is calculated as follows: RTRRAMT q = RRCOST q – DARRAMT q The above variables are defined as follows: Variable; Unit; Description. RTRRAMT q; $; Real-Time Responsive Reserve Amount per QSE—The adjustment to QSE q’s share of the costs for RRS, for the hour.. RRCOST q; $; Responsive Reserve Cost per QSE—QSE q’s share of the net total costs for RRS, for the hour.. DARRAMT q; $; Day-Ahead Responsive Reserve Amount per QSE—QSE q’s share of the DAM cost for RRS, for the hour.. q; none; A QSE.. (5) For Non-Spin, if applicable: (a) The net total costs for Non-Spin for a given Operating Hour is calculated as follows: NSCOSTTOT = (-1) * (Σm(RTPCNSAMTTOT m) + PCNSAMTTOT + NSFQAMTTOT) Where: Total payment of SASM-procured capacity for Non-Spin by market RTPCNSAMTTOT m = ΣqRTPCNSAMT q, m Total payment of DAM-procured capacity for Non-Spin PCNSAMTTOT = ΣqPCNSAMT q Total charge of failure on Ancillary Service Supply Responsibility for Non-Spin NSFQAMTTOT = ΣqNSFQAMT q Total payment of SASM procured capacity for Non-Spin by QSE RTPCNSAMTQSETOTq = ΣmRTPCNSAMT q, m The above variables are defined as follows: Variable; Unit; Description. NSCOSTTOT; $; Non-Spin Cost Total—The net total costs for Non-Spin for the hour.. RTPCNSAMTTOT m; $; Procured Capacity for Non-Spin Amount Total by market—The total payments to all QSEs for the Ancillary Service Offers cleared in the market m for Non-Spin, for the hour.. RTPCNSAMT q, m; $; Procured Capacity for Non-Spin Amount per QSE by market—The payment to QSE q for its Ancillary Service Offers cleared in the market m for Non-Spin, for the hour.. NSFQAMTTOT; $; Non-Spin Failure Quantity Amount Total—The total charges to all QSEs for their capacity associated with failures and reconfiguration reductions on their Ancillary Service Supply Responsibilities for Non-Spin, for the hour.. NSFQAMT q; $; Non-Spin Failure Quantity Amount per QSE—The charge to QSE q for its total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for Non-Spin, for the hour.. RTPCNSAMTQSETOT q; $; Procured Capacity for Non-Spin Amount Total per QSE—The total payments to a QSE in all SASM markets for the Ancillary Service Offers cleared for Non-Spin, for the hour.. PCNSAMT q; $; Procured Capacity for Non-Spin Amount per QSE in DAM—The DAM Non-Spin payment for QSE q for the hour.. PCNSAMTTOT; $; Procured Capacity for Non-Spin Amount Total in DAM—The total of the DAM Non-Spin payments for all QSEs for the hour.. q; none; A QSE.. m; none; A SASM for the given Operating Hour.. (b) Each QSE’s share of the net total costs for Non-Spin for the Operating Hour is calculated as follows: NSCOST q = NSPR * NSQ q Where: NSPR = NSCOSTTOT / NSQTOT NSQTOT = ΣqNSQ q NSQ q = NSO q – SANSQ q NSO q = Σq(SANSQ q + Σm(RTPCNS q, m) + PCNS q – NSRP q – NSFQ q) * HLRS q + NSRP q SANSQ q = DASANSQ q + RTSANSQ q The above variables are defined as follows: Variable; Unit; Description. NSCOST q; $; Non-Spin Cost per QSE—QSE q’s share of the net total costs for Non-Spin, for the hour.. NSPR; $/MW per hour; Non-Spin Price—The price for Non-Spin calculated based on the net total costs for Non-Spin, for the hour.. NSCOSTTOT; $; Non-Spin Cost Total—The net total costs for Non-Spin for the hour. See item (a) above.. NSQTOT; MW; Non-Spin Quantity Total—The sum of every QSE’s portion of its Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour.. NSQ q; MW; Non-Spin Quantity per QSE—The portion of QSE q’s Ancillary Service Obligation that is not self-arranged in either DAM or any SASM, for the hour.. NSO q; MW; Non-Spin Obligation per QSE—The Ancillary Service Obligation of QSE q, for the hour.. DASANSQ q; MW; Day-Ahead Self-Arranged Non-Spin Quantity per QSE for DAM—The self-arranged Non-Spin quantity submitted by QSE q before 1000 in the Day-Ahead.. RTSANSQ q; MW; Self-Arranged Non-Spin Quantity per QSE for all SASMs—The sum of all self-arranged Non-Spin quantities submitted by QSE q for all SASMs.. RTPCNS q, m; MW; Procured Capacity for Non-Spin per QSE by market—The MW portion of QSE q’s Ancillary Service Offers cleared in the market m to provide Non-Spin, for the hour.. NSFQ q; MW; Non-Spin Failure Quantity per QSE—QSE q’s total capacity associated with failures and reconfiguration reductions on its Ancillary Service Supply Responsibility for Non-Spin, for the hour.. HLRS q; none; The Hourly Load Ratio Share calculated for QSE q for the hour. See Section 6.6.2.3.. NSRP q; MW; Non-Spin Replacement per QSE per market—The total Non-Spin capacity that was a portion of the Ancillary Service Supply Responsibility of QSE q but is replaced in a SASM for the hour.. PCNS q; MW; Procured Capacity for Non-Spin Service per QSE in DAM—The total Non-Spin capacity quantity awarded to QSE q in the DAM for all the Resources represented by the QSE for the hour.. SANSQ q; MW; Total Self-Arranged Non-Spin Supplied Quantity per QSE for all markets—The sum of all self-arranged Non-Spin quantities submitted by QSE q for DAM and all SASMs.. q; none; A QSE.. m; none; A SASM for the given Operating Hour.. (c) The adjustment to each QSE’s DAM charge for the Non-Spin for the Operating Hour, due to changes during the Adjustment Period or Real-Time operations, is calculated as follows: RTNSAMT q = NSCOST q – DANSAMT q The above variables are defined as follows: Variable; Unit; Description. RTNSAMT q; $; Real-Time Non-Spin Amount per QSE—The adjustment to QSE q’s share of the costs for Non-Spin, for the hour.. NSCOST q; $; Non-Spin Cost per QSE—QSE q’s share of the net total costs for Non-Spin, for the hour.. DANSAMT q; $; Day-Ahead Non-Spin Amount per QSE—QSE q’s share of the DAM cost for Non-Spin, for the hour.. q; none; A QSE..

6.7.5	Real-Time Ancillary Service Imbalance Revenue Neutrality Allocation

The total cost for Ancillary Service Imbalance payments and charges is allocated to the QSEs representing Load based on Load Ratio Share (LRS).  The Real-Time Ancillary Service imbalance revenue neutrality allocation to each QSE for a given 15-minute Settlement Interval is calculated as follows:

LAASIRNAMT q	=	(-1) * [(RTASIAMTTOT + RTRUCRSVAMTTOT) * LRS q]

Where:

RTASIAMTTOT		=Σq RTASIAMT q

RTRUCRSVAMTTOT	=Σq RTRUCRSVAMT q

Σq

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LAASIRNAMT q | $ | Load-Allocated Ancillary Service Imbalance Revenue Neutrality Amount per QSE—The QSE q’s share of the total Real-Time Ancillary Service imbalance revenue neutrality amount for the 15-minute Settlement Interval. |
| RTASIAMTTOT | $ | Real-Time Ancillary Service Imbalance Market Total Amount—The total payment or charge to all QSEs for the Real-Time Ancillary Service imbalance for each 15-minute Settlement Interval. |
| RTASIAMT q | $ | Real-Time Ancillary Service Imbalance Amount—The total payment or charge to QSE q for the Real-Time Ancillary Service imbalance for each 15-minute Settlement Interval. |
| RTRUCRSVAMTTOT | $ | Real-Time RUC Ancillary Service Reserve Market Total Amount—The total payment to all QSEs for the Real-Time RUC Ancillary Service reserve payments for each 15-minute Settlement Interval. |
| RTRUCRSVAMT q | $ | Real-Time RUC Ancillary Service Reserve Amount—The total payment to QSE q for the Real-Time RUC Ancillary Service reserve payment for each 15-minute Settlement Interval. |
| LRS q | none | The LRS calculated for QSE q for the 15-minute Settlement Interval. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. |
| q | none | A QSE. |

6.7.6	Make-Whole Payment for Undeployed Ancillary Service Assignment During a Watch

- Resources that have received an Ancillary Service Assignment to provide Ancillary Services under a Watch as set forth in paragraph (4) of Section 6.5.9.3.3, Watch, may be paid the difference between the Real-Time Settlement Point Price at the Resource Node and the Real-Time On-Line price adder only for Settlement Intervals where the corresponding DAM RRS MCPC is greater than or equal to $7,000/MW per hour.

The Real-Time Make-Whole Payment for undeployed Ancillary Service to each QSE for the 15-minute Settlement Interval is calculated as follows:

RTRASMWAMT q, r, p 	=	-1 * (RTSPP p - RTRSVPOR) * RTUNDAS q, r, p * ¼

Where:

RTUNDAS q, r, p =RTUNDASRU q,r,p  + RTUNDASRR q,r,p  + RTUNDASRD q,r,p

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTRASMWAMT q, r, p | $ | Real-Time Resource Ancillary Service Make-Whole Amount—The payment to QSE q for Generation Resource r at Resource Node p, for the Real-Time Ancillary Service make-whole for each 15-minute Settlement Interval. |
| RTUNDASRU q,r,p | MW | Regulation Up Undeployed Assignment- The quantity of undeployed Regulation Up Ancillary Service assigned under a Watch to a QSE and Resource for the 15-minute Settlement Interval. |
| RTUNDASRR q,r,p | MW | Responsive Reserve Undeployed Assignment - The quantity of undeployed Responsive Reserve Ancillary Service assigned under a Watch to a QSE and Resource for 15-minute Settlement Interval. |
| RTUNDASRD q,r,p | MW | Regulation Down Undeployed Assignment - The quantity of undeployed Regulation Down Ancillary Service assigned under a Watch to a QSE and Resource for 15-minute Settlement Interval. |
| RTUNDAS q, r, p | MW | Real-Time Undeployed Ancillary Service Schedule per Resource - The Real-Time Undeployed Ancillary Service Schedule pursuant to the Ancillary Service awards for Reg-Up, Reg-Down, and RRS for the Resources due to Ancillary Service Assignment during a Watch as set forth in paragraph (4) of Section 6.5.9.3.3, for the QSE q, for the 15-minute Settlement Interval. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Resource Node p, for the 15-minute Settlement Interval |
| RTRSVPOR | $/MWh | Real-Time Reserve Price for On-Line ReservesThe Real-Time Reserve Price for On-Line Reserves for the 15-minute Settlement Interval. |
| q | none | A QSE. |
| r | none | A Generation Resource. |
| p | none | A Settlement Point. |

- Additionally, Resources that have received an Ancillary Service Assignment during a Watch as set forth in paragraph (4) of Pection 6.5.9.3.3, may receive a payment for the Ancillary Service reserves associated with each hour of the Ancillary Service Assignment and will be considered as a Real-Time RUC Ancillary Service Awards for purposes of  Paragraph (6) of Pection 6.7.4, Real-Time Ancillary Service Imbalance Payment or Charge, and the QSE may file a Settlement dispute to be considered for the Real-Time RUC Ancillary Service Reserve Amount payment.

- The total compensation to each QSE for Make-Whole Payment for Undeployed Ancillary Service of Generation Resources for the 15-minute Settlement Interval is calculated as follows:

RTASMWAMTQSETOT q =	Σr RTRASMWAMT q, r, p

- The total payments to the market to be considered for the Ancillary Service Cost Allocation for Make-Whole Payment for Undeployed Ancillary Service of Generation Resources for the 15-minute Settlement Interval is calculated as follows:

RTASMWAMTTOT =  Σq RTRASMWAMTQSETOT q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTASMWAMTQSETOT q | $ | Real-Time Ancillary Service Make-Whole Amount QSE Total per QSE - The total of the payments to QSE q for the Real-Time Ancillary Service make-whole of the Generation Resources represented by this QSE for the 15-minute Settlement Interval i. |
| RTRASMWAMT q, r, p | $ | Real-Time Resource Ancillary Service Make-Whole Amount—The payment to QSE q for Generation Resource r at Resource Node p, for the Real-Time Ancillary Service make-whole for each 15-minute Settlement Interval i. |
| RTASMWAMTTOT | $ | Real-Time Resource Ancillary Service Make-Whole Amount—The total payments to all QSEs for Ancillary Service Assignments under the Real-Time Ancillary Service Make-Whole for each 15-minute Settlement Interval. |
| q | none | A QSE. |
| r | none | A Generation Resource. |
| p | none | A Settlement Point. |

Upon implementation of System Wide Offer Cap (SWCAP) of $9,000 delete section 6.7.6