DEPLOYMENT SEQUENCE GUIDELINE

CAPACITY INSUFFICIENCY (Manual)

SR1

CR1

SR2

CR2

PFR (Manual release of HASLs to SCED)

ERS-30 (EEA must be in effect)

ERS-10 (EEA2 must be in effect)

FFR2 (Manual)

FFR1 (Manual)

Firm Load Shed

RECALL SEQUENCE

Firm Load Restoration

PFR

FFR1

FFR2

CR1

CR2

ERS-10

ERS-30

SR1

SR2

SLOW FREQUENCY RECOVERY (potential DCS Event with frequency at or below 59.7 Hz)

PFR (Automatic)

CR1 (Automatic release to SCED at 59.91 Hz)

FFR1 (Relay action at 59.80 Hz)

FFR2 (Relay action at 59.70 Hz)

CR2 (Manual)

SR1 (Release to SCED) and SR2 (Manual)

---At this point if frequency is not recovered, ERCOT will deploy any available capacity under the capacity insufficiency sequence for both deployment and recall.

RECALL SEQUENCE

PFR (restored automatically)

FFR1

FFR2

CR1

CR2

SR1

SR2