There are 3 interrelated issues currently under discussion in the stakeholder community:

- (NPRR 595)

  - Load Resource capacity behind UFR

  - Capacity with no RRS responsibility (not awarded due to proration) from a resource that is providing RRS from some portion of its capacity

- OFF10/OFF30 Load Resources

  - LR capacity not behind UFR and no AS responsibility

- OFF10/OFF30 Generation

  - No AS responsibility

OFF-10/OFF-30 GENERAL PRINCIPLES

- Provided by Load Resources with no A/S responsibility and with capacity NOT behind a UFR

  - LRARF on file

  - Full-time telemetry

- Load Resources that are qualified to provide RRS are automatically qualified to provide OFF10 or OFF30

- Load Resources that are not qualified to provide RRS needs to qualify the amount that can be used to provide in 10 minutes (OFF10) or 30 minutes (OFF30)

  - Similar to RRS testing

- Unannounced testing needs to be added to ensure that the telemetered OFF10 can be provided by NPF-LPC

- Load Resource need to submit COP OFF 10 needed for ORDC error analysis and performance monitoring

- Obligation for each operation hour is determined by COP submitted at end of adjustment period for that operating hour

  - Removed via Forced Outage submission subject to compliance review

  - Performance measurement – LR need to carry at least X% of the amount specified in OFF10cop more than Y% of time

- Load Resource need to submit telemetered OFF 10

- AS payment will be based on min( OFF10cop, OFF10telemetry, NPF-LPC)

- Deployment will be during or before EEA1 based on a specified PRC level

- Performance measurement – LR need to deploy the MW amount specified in OFF10cop more than X% of time

- Deployment could be in groups if the amount of OFF10 MW exceeds a threshold

- OFF10 MW after deployment should be added to GTBD in pricing run if NPRR626 is implemented

Difference for NPRR595

- AS payment will be based on min( OFF10cop, 150% RRS responsibility, NPF-LPC- AS responsibility ) if UFR is armed

- Cost/benefit analysis needed

- ERS issues in OFF10/30

- Could this serve as a reconfiguration of ERS?  (Hourly commitments vs. 4-months)