| Current Loads in SCED Technical Barriers |  |  |  |
| --- | --- | --- | --- |
| Category | Item | Description | Proposal(s) |
| Temporal Constraint | < 5 minute ramp time | Many loads need longer lead times to dispatch (10 - 30 minutes) | Multi-interval SCED; relaxing dispatch requirements on Loads |
| Temporal Constraint | Min ramp time | Many loads when dispatched need to run for a certain length of time before they can be recalled to their pre-dispatch levels | Multi-interval SCED; relaxing dispatch requirements on Loads |
| Temporal Constraint | Max ramp time | Many loads can only run for a certain length of time during any given dispatch | Multi-interval SCED; relaxing dispatch requirements on Loads |
| Temporal Constraint | Return to service time | Most loads take longer than 5 minutes to return to pre-dispatch levels (10-120 minutes) | Multi-interval SCED; relaxing dispatch requirements on Loads |
| Size constraint | Loads must be dispatchable in 100kW increments | Many C&I loads can only be dispatched in blocks larger than 100 kW | Multi-interval SCED; relaxing dispatch requirements on Loads |
| Ancillary Services | Tracking FAST developments | Loads in SCED needs to be compatible with future ancillary services changes & vice versa |  |
| DR QSE issues | DR QSE participation | DR QSEs need to be able to participate in SCED directly to allow DR to be fully competitive in the market | This requires other issues below be addressed |
| DR QSE issues | Estimating "-G" credit to LSEs | To maintain the TAC endorsed LMP-G structure it is necessary to estimate the amount of load drop resulting from DR deployment that needs to be credited back to the LSE | ERCOT approval of DR QSE estimates based on detailed premise or even device-level estimates. |
| DR QSE issues | Customer billing | The TAC endorsed LMP-G approach requires the LSE to bill customer for unused energy, which may result in unintended consequences | Proxy G or other alternative approaches? |
| DR QSE Issues | Managing customer transactions | Mechanism needed to register customers with DR QSEs and track customer-DR QSE-LSE relationships | TX SET modifications |