| Current Loads in SCED Technical Barriers |  |  |  |
| --- | --- | --- | --- |
| Category | Item | Description | Proposal(s) |
| Temporal Constraint | < 5 minute ramp time | Many loads need longer lead times to dispatch (10 - 30 minutes) | Multi-interval SCED; relaxing dispatch requirements on Loads |
| Temporal Constraint | Minimum response period | Many loads when dispatched must be curtailed for a minimum length of time before they can be recalled to their pre-dispatch levels | Multi-interval SCED; relaxing dispatch requirements on Loads |
| Temporal Constraint | Maximum deployment time | Many loads can only run for a certain length of time during any given dispatch | Multi-interval SCED; relaxing dispatch requirements on Loads |
| Temporal Constraint | Load restoration period | The time required for a curtailed load to return to pre-dispatch load level. Most loads take longer than 5 minutes to return to pre-dispatch levels (10-120 minutes) | Multi-interval SCED; relaxing dispatch requirements on Loads |
| Temporal Constraint | Post curtailment dispatch availability time | Time required for a load resource to be available again for dispatch following release from curtailment. Exclusive of Load restoration time. | Multi-interval SCED; relaxing dispatch requirements on Loads |
| Multi-interval SCED | Make-whole payment and charge allocation | Multi-interval SCED will require a make-whole process. Stakeholders will need to define the make-whole payment and cost allocation process. | Multi-interval SCED; relaxing dispatch requirements on Loads |
| Size constraint | Block bids/offers for load resources | Include the ability for loads to block bid/off in LRIS. Many C&I loads cannot be dispatched incrementally. | Multi-interval SCED; relaxing dispatch requirements on Loads |
| Curtailment validation | Accurate measurement of curtailment and ability to qualify and verify | Any additional implementation of LRIS must allow for accurate measurement of curtailment by ERCOT. ERCOT must be able to qualify and verify the resource. | LR/ALR must qualify for baseline methodology, process TBD |
| Ancillary Services | Tracking FAST developments | Loads in SCED needs to be compatible with future ancillary services changes & vice versa |  |
| DR QSE issues | DR QSE participation | DR QSEs need to be able to participate in SCED directly to allow DR to be fully competitive in the market | This requires other issues below be addressed |
| DR QSE issues | Estimating "-G" credit to LSEs | To maintain the TAC endorsed LMP-G structure it is necessary to estimate the amount of load drop resulting from DR deployment that needs to be credited back to the LSE | Depart from TAC endorsed LMP-G, develop process targeted at residential customers |
| DR QSE issues | Customer billing | The TAC endorsed LMP-G approach requires the LSE to bill customer for unused energy, which may result in unintended consequences | Implement LMP-Proxy $G |
| Rebound effect | Increased load after deployment | How to manage excess energy as a result of the deployment |  |
| DR QSE Issues | Proxy $G | Establish a process to define a proxy $G | TBD |