Examples for the DAM procurement and pricing of

RegUp/FRRS-Up, RegDn/FRRS-Dn, PFR, FFR1, FFR2, CR1, CR2, SR1 & SR2

9/15/2014

Revision Description:

5/2/2014:

- Removed reference to % contribution of FFR2 towards CR (missed cleanup item from 4/25 FAST meeting)

- Added Scenario 5 to include RegUp, RegDn, & SR

4/25/2014:

- Corrected the label for the ratio of FFR/PFR ratio to PFR/FFR i.e. this ratio defines how many MW of PFR is equivalent to 1 MW of FFR

- Included in Scenarios 1,2,3,4, a Generation Resource G5 offering FFR1 only (e.g. Battery)

- Changed the percentage contribution of FFR2 toward CR down to zero – based on answer to question 2 in list of consensus items

  - In Scenarios 1, 2, 3, & 4, LR3 Maximum Power consumption (MPC) changed from 500 MW to 600 MW to keep the resulting energy and AS prices the same as was presented on 4/18/2014 FAST Work Session Meeting

- Modified Scenarios 3, & 4 by increasing the Energy Bid amount from 40,000 to 40,001 to remove any ambiguity on power balance shadow price – this will show that the power balance shadow price is 9,000 $/MWh and is set by the Energy Bid. Does not change any of the resulting energy and AS prices

- Crossed out all MCPC price options except for Option 1 for FFR1, FFR2,  & CR2 – based on answer to question 1 in list of consensus items

- Added statement to indicate the input data shown in this document are for illustrative purpose only.

9/15/2014:

- Corrected typos in the tabulated shadow prices for the Maximum FFR procurement and Maximum FFR1 procurement constraints in Scenarios 1-4

- Added, where applicable,  Alternate Proposal 1 and Alternate Proposal 2 in the determination of MCPC  for FFR1, FFR2, CR2, SR2, FRRS-Up, FRRS-Dn

- Accepted changes to improve readability

Case Setup Scenarios 1-4:

- Five Generation Resources : G1, G2, G3, G4, & G5

- Three “Blocky” Load Resources (AS offer exclusivity modeled by market participant bidding): LR1, LR2, & LR3

- No congestion

- Only Dispatch (procurement) and pricing part of DAM considered (no commitment, i.e. all Resources ONLINE)

- All data shown in this document are for illustrative purpose ONLY. These numbers should NOT be taken as an indication of outcome of any study on the ERCOT system (e.g. the numbers in the AS plan have no relationship to the actual numbers that ERCOT will post – they are for illustrative purposes only).

- Scenarios 1-4 are examples illustrating the procurement and interaction between energy, PFR, FFR and CR. RegUp, RegDn, and SR requirements are assumed to be zero MW.

Case Setup Scenario 5:

- Five Generation Resources : G1, G2, G3, G4, & G5

- Four “Blocky” Load Resources (AS offer exclusivity modeled by market participant bidding): LR1, LR2, LR3 & LR4

- One Controllable Load Resource offering FRRS-Dn

- No congestion

- Only Dispatch (procurement) and pricing part of DAM considered (no commitment, i.e. all Resources ONLINE)

- All data shown in this document are for illustrative purpose ONLY. These numbers should NOT be taken as an indication of outcome of any study on the ERCOT system (e.g. the numbers in the AS plan have no relationship to the actual numbers that ERCOT will post – they are for illustrative purposes only).

AS Plan for Scenarios 1-4: Assume RegUp, RegDn, & SR requirements are zero MW

| PFR Minimum Requirement (MW) | FFR Target Requirement (MW) | FFR Max Requirement (MW) | FFR1 Max Requirement (MW) | PFR/FFR Ratio | CR Requirement (MW) | CR1 Min Requirement (MW) |
| --- | --- | --- | --- | --- | --- | --- |
| 1,400 | 800 | 800 | 100 | 2 | 700 | 200 |

Energy Bid for Scenarios 1-4:

| Demand Bid (MWh) | Bid Price ($/MWh) |
| --- | --- |
| 40,000 | 9,000 |

Generation Resource Limits, Energy Offer, AS Offer for Scenarios 1-4:

| Generation Resource | LSL (MW) | HSL (MW) | EOC ($/MWh) | PFR ($/MW) | CR ($/MW) | FFR1 ($/MW) |
| --- | --- | --- | --- | --- | --- | --- |
| G1 | 0 | 5,000 | 7,000 | 20 | 19 | - |
| G2 | 0 | 15,000 | 50 | 15 | 14 | - |
| G3 | 0 | 20,000 | 20 | 10 | 9 | - |
| G4 | 0 | 10,000 | 10 | - | - | - |
| G5 | 0 | 20 | - | - | - | 2 |

Load Resource Limits, AS Offer for Scenarios 1-4:

| Load Resource | Type | LPC (MW) | MPC (MW) | FFR ($/MW) | CR ($/MW) |
| --- | --- | --- | --- | --- | --- |
| LR1 | FFR1 | 0 | 200 | 3 | - |
| LR2 | FFR2 | 0 | 1,000 | 6 | - |
| LR3 | CR | 0 | 600 | - | 4 |

Scenarios 1-4: Assume RegUp, RegDn, & SR requirements are zero MW

- Base Case

  - Lots of capacity

  - No opportunity costs in clearing prices for energy and AS

- Reduce G2 capacity (from Base Case) to 11,500 MW to force procurement of CR from more expensive G1.

  - Opportunity cost of CR impacts prices

- Reduce G2 capacity (from Base Case) to 6,600 MW to force procurement of energy, PFR, CR from more expensive G1. Two scenarios. Scenario 3a is where scarcity pricing occurs and the expected value of the Ratio of PFR to FFR will be closer to 1. Scenario 3b is also where scarcity pricing occurs but with a UNLIKELY occurrence of the Ratio of PFR to FFR being higher than 1  e.g. 2

  - Opportunity prices of energy impacts prices

  - Demonstrate that Alternative Proposal 1 - MCPC for FFR1, FFR2 and CR2 will at least provide the same MCPC for Load Resources as what the Generation Resources get.  However, there may be an issue with replacing FFR MW in Real-Time when the ratio of PFR to FFR is greater than 1

  - Demonstrate that Alternative Proposal 2 – MCPC  for FFR1, FFR2 and CR2 will always be equal to the marginal respective AS Offer from the Load Resource. Opportunity costs are not given to MCPC for AS from Load Resources

- Reduce G2 capacity (from Base Case) to 6,600 MW to force procurement of energy, PFR, CR from more expensive G1. Also reduce Energy Offer price of G1 from 7000 to 100 (still the most expensive energy offer). Two scenarios. Scenario 4a is where scarcity pricing occurs and the expected value of the Ratio of PFR to FFR will be closer to 1. Scenario 4b is also where scarcity pricing occurs but with a UNLIKELY occurrence of the Ratio of PFR to FFR being higher than 1 e.g. 2

  - Opportunity prices of energy impacts price

  - Demonstrates MCPC (FAST Proposal) for FFR (depending on option) can exceed VOLL

  - Demonstrate that Alternative Proposal 1 - MCPC for FFR1, FFR2 and CR2 will at least provide the same MCPC for Load Resources as what the Generation Resources get.  However, there may be an issue with replacing FFR MW in Real-Time when the ratio of PFR to FFR is greater than 1

  - Demonstrate that Alternative Proposal 2 – MCPC  for FFR1, FFR2 and CR2 will always be equal to the marginal respective AS Offer from the Load Resource. Opportunity costs are not given to MCPC for AS from Load Resources

Observations - Scenarios 1-4:

- MCPC (FAST Proposal) for FFR1 and FFR2 needs to be limited to VOLL  as it can lead to excessive values of FFR1 and FFR2 MCPC . This occurs in some scenarios due to the multiplication of the shadow price SPPFR+FFR  by the equivalency ratio of FFR to PFR.

Scenario 5:

- Similar to Scenario 1 , with all Future AS considered

- Lots of capacity

- No opportunity costs in clearing prices for energy and AS

- Demonstrate that Alternative Proposal 1 - MCPC for FFR1, FFR2, CR2, SR2, FRRS-Up, and FRRS-Dn  will at least provide the same MCPC for Load Resources as what the Generation Resources get.  However, there may be an issue with replacing FFR MW in Real-Time when the ratio of PFR to FFR is greater than 1

- Demonstrate that Alternative Proposal 2 – MCPC  for FFR1, FFR2, CR2, SR2, FRRS-Up, and FRRS-Dn  will always be equal to the marginal respective AS Offer from the Load Resource. Opportunity costs are not given to MCPC for AS from Load Resources

Scenario 1: Base Case

Yellow highlights are results/outputs of DAM

AS Plan: Assume RegUp, RegDn, & SR requirements are zero MW

| PFR Minimum Requirement (MW) | FFR Target Requirement (MW) | FFR Max Requirement (MW) | FFR1 Max Requirement (MW) | PFR/FFR Ratio | CR Requirement (MW) | CR1 Min Requirement (MW) |
| --- | --- | --- | --- | --- | --- | --- |
| 1,400 | 800 | 800 | 100 | 2 | 700 | 200 |

Energy Bid:

| Demand Bid (MWh) | Bid Price ($/MWh) | Bid Award (MW) |
| --- | --- | --- |
| 40,000 | 9,000 | 40,000 |

Generation Resource Limits, Energy Offer, AS Offer:

| Generation Resource | LSL (MW) | HSL (MW) | EOC ($/MWh) | PFR ($/MW) | CR ($/MW) | FFR1 ($/MW) | Energy Award (MW) | PFR Award (MW) | CR Award (MW) | FFR1 Award (MW) | HSL Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 | 0 | 5,000 | 7,000 | 20 | 19 | - | 0 | 0 | 0 |  | 0 % |
| G2 | 0 | 15,000 | 50 | 15 | 14 | - | 10,000 | 1,400 | 200 |  | 77 % |
| G3 | 0 | 20,000 | 20 | 10 | 9 | - | 20,000 | 0 | 0 |  | 100 % |
| G4 | 0 | 10,000 | 10 | - | - | - | 10,000 | - |  |  | 100 % |
| G5 | 0 | 20 | - | - | - | 2 |  |  |  | 20 | 100% |

Load Resource Limits, AS Offer:

| Load Resource | Type | LPC (MW) | MPC (MW) | FFR ($/MW) | CR ($/MW) | FFR Award (MW) | CR Award (MW) | MPC Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR1 | FFR1 | 0 | 200 | 3 | - | 80 | - | 40 % |
| LR2 | FFR2 | 0 | 1,000 | 6 | - | 700 | - | 70 % |
| LR3 | CR | 0 | 600 | - | 4 | - | 500 | 83 % |

Constraints:

| Constraints | Equation | Shadow Price ($/MW) | Comments |
| --- | --- | --- | --- |
| Power Balance | EnergySupply - EnergyDemand = 0 | 50 | - G2 EOC is marginal and sets energy price - The Energy bid is fully awarded |
| Combined PFR/FFR Procurement | PFR Award + Ratio *( FFR1+FFR2) Award >= PFR Req. + Ratio * FFR Req. | 15 | - Ratio=2 - The right hand side of the equation is 1400 + 2*800 = 3000 - G2 PFR Offer is marginal - G2 capacity is not fully utilized. There is no opportunity cost for energy or other AS incorporated into this constraint’s shadow price |
| Max FFR Procurement | ( FFR1+FFR2) Award <= FFR Max Req. | -24 | - Shadow price is negative - The right hand side of the equation is 800 |
| Max FFR1 Procurement | FFR1 Award <= FFR1 Max Req. | -3 | - Shadow price is negative - The right hand side of the equation is 100 |
| CR Procurement | CR1 Award + CR2 Award >= CR Req. | 4 | - The right hand side of the equation is 700 - LR3 CR Offer is marginal |
| Min CR1 Procurement | CR1 Award >= CR1 Req. | 10 | - The right hand side of the equation is 200 - G2 CR Offer is marginal - G2 capacity is not fully utilized. There is no opportunity cost for energy incorporated into this constraint’s shadow price |

MCPCs:

| AS MCPC | Equation | $/MW |
| --- | --- | --- |
| PFR | SPPFR+FFR | 15 $/MW Note: This is the marginal Offer price for PFR from Gen |
| FFR1 | FAST Proposal: Min(R*SPPFR+FFR, VOLL) | Min(2*15, 9000) = 30 $/MW |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax+ SPFFR1Max) | Max(15, 2*15-24-3) = 15 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax+ SPFFR1Max | 2*15-24-3 = 3 $/MW Note: This is the marginal Offer price for FFR1 |
| FFR2 | FAST Proposal: Min(R*SPPFR+FFR, VOLL) | Min(2*15, 9000) = 30 $/MW |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax) | Max(15, 2*15-24) = 15 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax | 2*15-24 = 6 $/MW Note: This is the marginal Offer price for FFR2 from Load Resource |
| CR1 | SPCR+ SPCR1 | 4 + 10 = 14 $/MW Note: This is the marginal Offer price for CR from Gen |
| CR2 | FAST Proposal: SPCR+ SPCR1 | 4 + 10 = 14 $/MW |
|  | Alternative Proposal 2: SPCR | 4 $/MW Note: This is the marginal Offer price for CR from Load Resource |

Scenario 2:  Base Case changed - G2 HSL = 11,500 MW

Red highlights are changes to input from Base Case

Yellow highlights are results/outputs of DAM

AS Plan: Assume RegUp, RegDn, & SR requirements are zero MW

| PFR Minimum Requirement (MW) | FFR Target Requirement (MW) | FFR Max Requirement (MW) | FFR1 Max Requirement (MW) | PFR/FFR Ratio | CR Requirement (MW) | CR1 Min Requirement (MW) |
| --- | --- | --- | --- | --- | --- | --- |
| 1,400 | 800 | 800 | 100 | 2 | 700 | 200 |

Energy Bid:

| Demand Bid (MWh) | Bid Price ($/MWh) | Bid Award (MW) |
| --- | --- | --- |
| 40,000 | 9,000 | 40,000 |

Generation Resource Limits, Energy Offer, AS Offer:

| Generation Resource | LSL (MW) | HSL (MW) | EOC ($/MWh) | PFR ($/MW) | CR ($/MW) | FFR1 ($/MW) | Energy Award (MW) | PFR Award (MW) | CR Award (MW) | FFR1 Award (MW) | HSL Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 | 0 | 5,000 | 7,000 | 20 | 19 | - | 0 | 0 | 100 |  | 2 % |
| G2 | 0 | 11,500 | 50 | 15 | 14 | - | 10,000 | 1,400 | 100 |  | 100 % |
| G3 | 0 | 20,000 | 20 | 10 | 9 | - | 20,000 | 0 | 0 |  | 100 % |
| G4 | 0 | 10,000 | 10 | - | - | - | 10,000 | - |  |  | 100 % |
| G5 | 0 | 20 | - | - | - | 2 |  |  |  | 20 | 100% |

Load Resource Limits, AS Offer:

| Load Resource | Type | LPC (MW) | MPC (MW) | FFR ($/MW) | CR ($/MW) | FFR Award (MW) | CR Award (MW) | MPC Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR1 | FFR1 | 0 | 200 | 3 | - | 80 | - | 40 % |
| LR2 | FFR2 | 0 | 1,000 | 6 | - | 700 | - | 70 % |
| LR3 | CR | 0 | 600 | - | 4 | - | 500 | 83 % |

Constraints:

| Constraints | Equation | Shadow Price ($/MW) | Comments |
| --- | --- | --- | --- |
| Power Balance | EnergySupply - EnergyDemand = 0 | 55 | - G2 EOC is marginal for energy - Energy price set by combination of energy offer price of G2 and opportunity cost of CR (between G1 & G2) - The Energy bid is fully awarded |
| Combined PFR/FFR Procurement | PFR Award + Ratio *( FFR1+FFR2) Award >= PFR Req. + Ratio * FFR Req. | 20 | - Ratio=2 - The right hand side of the equation is 1400 + 2*800 = 3000 - G2 PFR Offer is marginal - G2 capacity is fully utilized. There is opportunity cost for CR (between G1 & G2) incorporated into this constraint’s shadow price |
| Max FFR Procurement | ( FFR1+FFR2) Award <= FFR Max Req. | -34 | - Shadow price is negative - The right hand side of the equation is 800 |
| Max FFR1 Procurement | FFR1 Award <= FFR1 Max Req. | -3 | - Shadow price is negative - The right hand side of the equation is 100 |
| CR Procurement | CR1 Award + CR2 Award >= CR Req. | 4 | - The right hand side of the equation is 700 - LR3 CR Offer is marginal |
| Min CR1 Procurement | CR1 Award >= CR1 Req. | 15 | - The right hand side of the equation is 200 - G1 CR Offer is marginal - G1 capacity is not fully utilized. There is no opportunity cost for energy incorporated into this constraint’s shadow price |

MCPCs:

| AS MCPC | Equation | $/MW |
| --- | --- | --- |
| PFR | SPPFR+FFR | 20 $/MW Note: This is the marginal Offer price for PFR from Gen plus opportunity cost of CR |
| FFR1 | FAST proposal: Min(R*SPPFR+FFR, VOLL) | Min(2*20, 9000) = 40 $/MW |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax+ SPFFR1Max) | Max(20, 2*20-34-3) = 20 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax+ SPFFR1Max | 2*20-34-3 = 3 $/MW Note: This is the marginal Offer price for FFR1 |
| FFR2 | FAST proposal: Min(R*SPPFR+FFR, VOLL) | Min(2*20, 9000) = 40 $/MW |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax) | Max(20, 2*20-34) = 20 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax | 2*20-34 = 6 $/MW Note: This is the marginal Offer price for FFR2 from Load Resource |
| CR1 | SPCR+ SPCR1 | 4 + 15 = 19 $/MW Note: This is the Offer price for CR from Gen |
| CR2 | FAST proposal: SPCR+ SPCR1 | 4 + 15 = 19 $/MW |
|  | Alternative Proposal 2: SPCR | 4 $/MW Note: This is the marginal Offer price for CR from Load Resource |

Scenario 3a:  Base Case changed – Demand Bid MW = 40,001 MW, G2 HSL = 6,600 MW, PFR/FFR Ratio = 1

Red highlights are changes to input from Base Case

Yellow highlights are results/outputs of DAM

AS Plan: Assume RegUp, RegDn, & SR requirements are zero MW

| PFR Minimum Requirement (MW) | FFR Target Requirement (MW) | FFR Max Requirement (MW) | FFR1 Max Requirement (MW) | PFR/FFR Ratio | CR Requirement (MW) | CR1 Min Requirement (MW) |
| --- | --- | --- | --- | --- | --- | --- |
| 1,400 | 800 | 800 | 100 | 1 | 700 | 200 |

Energy Bid:

| Demand Bid (MWh) | Bid Price ($/MWh) | Bid Award (MW) |
| --- | --- | --- |
| 40,001 | 9,000 | 40,000 |

Generation Resource Limits, Energy Offer, AS Offer:

| Generation Resource | LSL (MW) | HSL (MW) | EOC ($/MWh) | PFR ($/MW) | CR ($/MW) | FFR1 ($/MW) | Energy Award (MW) | PFR Award (MW) | CR Award (MW) | FFR1 Award (MW) | HSL Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 | 0 | 5,000 | 7,000 | 20 | 19 | - | 3,400 | 1,400 | 200 |  | 100 % |
| G2 | 0 | 6,600 | 50 | 15 | 14 | - | 6,600 | 0 | 0 |  | 100 % |
| G3 | 0 | 20,000 | 20 | 10 | 9 | - | 20,000 | 0 | 0 |  | 100 % |
| G4 | 0 | 10,000 | 10 | - | - | - | 10,000 | - |  |  | 100 % |
| G5 | 0 | 20 | - | - | - | 2 |  |  |  | 20 | 100% |

Load Resource Limits, AS Offer:

| Load Resource | Type | LPC (MW) | MPC (MW) | FFR ($/MW) | CR ($/MW) | FFR Award (MW) | CR Award (MW) | MPC Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR1 | FFR1 | 0 | 200 | 3 | - | 80 | - | 40 % |
| LR2 | FFR2 | 0 | 1,000 | 6 | - | 700 | - | 70 % |
| LR3 | CR | 0 | 600 | - | 4 | - | 500 | 83 % |

Constraints:

| Constraints | Equation | Shadow Price ($/MW) | Comments |
| --- | --- | --- | --- |
| Power Balance | EnergySupply - EnergyDemand = 0 | 9,000 | - Energy Bid is marginal and sets energy price - Energy Bid partially awarded (40,000 MWh) |
| Combined PFR/FFR Procurement | PFR Award + Ratio *( FFR1+FFR2) Award >= PFR Req. + Ratio * FFR Req. | 2,020 | - Ratio=1 - The right hand side of the equation is 1400 + 1*800 = 2200 - G1 PFR Offer is marginal - G1 capacity is fully utilized. Opportunity cost for energy incorporated into this constraint’s shadow price |
| Max FFR Procurement | ( FFR1+FFR2) Award <= FFR Max Req. | -2,014 | - Shadow price is negative - The right hand side of the equation is 800 |
| Max FFR1 Procurement | FFR1 Award <= FFR1 Max Req. | -3 | - Shadow price is negative - The right hand side of the equation is 100 |
| CR Procurement | CR1 Award + CR2 Award >= CR Req. | 4 | - The right hand side of the equation is 700 - LR3 CR Offer is marginal |
| Min CR1 Procurement | CR1 Award >= CR1 Req. | 2,015 | - The right hand side of the equation is 200 - G1 CR Offer is marginal - G1 capacity is fully utilized. Opportunity cost for energy incorporated into this constraint’s shadow price |

MCPCs:

| AS MCPC | Equation | $/MW |
| --- | --- | --- |
| PFR | SPPFR+FFR | 2,020 $/MW Note: This is the marginal Offer price for PFR from Gen plus opportunity cost of energy |
| FFR1 | FAST Proposal: Min(R*SPPFR+FFR,VOLL) | Min(1*2020, 9000) = 2,020 $/MW |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax+ SPFFR1Max) | Max(2020, 1*2020-2014-3) = 2020 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax+ SPFFR1Max | 1*2020-2014-3 = 3 $/MW Note: This is the marginal Offer price for FFR1 |
| FFR2 | FAST Proposal: Min(R*SPPFR+FFR,VOLL) | Min(1*2020, 9000) = 2,020 $/MW |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax) | Max(2020, 1*2020-2014) = 2020 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax | 1*2020-2014 = 6 $/MW Note: This is the marginal Offer price for FFR2 from Load Resource |
| CR1 | SPCR+ SPCR1 | 4 + 2015 = 2,019 $/MW Note: This is the marginal Offer price for CR from Gen plus opportunity cost of energy |
| CR2 | FAST Proposal: SPCR+ SPCR1 | 4 + 2015 = 2,019 $/MW |
|  | Alternative Proposal 2: SPCR | 4 $/MW Note: This is the marginal Offer price for CR from Load Resource |

Scenario 3b:  Base Case changed - Demand Bid MW = 40,001 MW, G2 HSL = 6,600 MW

Red highlights are changes to input from Base Case

Yellow highlights are results/outputs of DAM

AS Plan: Assume RegUp, RegDn, & SR requirements are zero MW

| PFR Minimum Requirement (MW) | FFR Target Requirement (MW) | FFR Max Requirement (MW) | FFR1 Max Requirement (MW) | PFR/FFR Ratio | CR Requirement (MW) | CR1 Min Requirement (MW) |
| --- | --- | --- | --- | --- | --- | --- |
| 1,400 | 800 | 800 | 100 | 2 | 700 | 200 |

Energy Bid:

| Demand Bid (MWh) | Bid Price ($/MWh) | Bid Award (MW) |
| --- | --- | --- |
| 40,001 | 9,000 | 40,000 |

Generation Resource Limits, Energy Offer, AS Offer:

| Generation Resource | LSL (MW) | HSL (MW) | EOC ($/MWh) | PFR ($/MW) | CR ($/MW) | FFR1 ($/MW) | Energy Award (MW) | PFR Award (MW) | CR Award (MW) | FFR1 Award (MW) | HSL Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 | 0 | 5,000 | 7,000 | 20 | 19 | - | 3,400 | 1,400 | 200 |  | 100 % |
| G2 | 0 | 6,600 | 50 | 15 | 14 | - | 6,600 | 0 | 0 |  | 100 % |
| G3 | 0 | 20,000 | 20 | 10 | 9 | - | 20,000 | 0 | 0 |  | 100 % |
| G4 | 0 | 10,000 | 10 | - | - | - | 10,000 | - |  |  | 100 % |
| G5 | 0 | 20 | - | - | - | 2 |  |  |  | 20 | 100% |

Load Resource Limits, AS Offer:

| Load Resource | Type | LPC (MW) | MPC (MW) | FFR ($/MW) | CR ($/MW) | FFR Award (MW) | CR Award (MW) | MPC Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR1 | FFR1 | 0 | 200 | 3 | - | 80 | - | 40 % |
| LR2 | FFR2 | 0 | 1,000 | 6 | - | 700 | - | 70 % |
| LR3 | CR | 0 | 600 | - | 4 | - | 500 | 83 % |

Constraints:

| Constraints | Equation | Shadow Price ($/MW) | Comments |
| --- | --- | --- | --- |
| Power Balance | EnergySupply - EnergyDemand = 0 | 9,000 | - Energy Bid is marginal and sets energy price - Energy Bid partially awarded (40,000 MWh) |
| Combined PFR/FFR Procurement | PFR Award + Ratio *( FFR1+FFR2) Award >= PFR Req. + Ratio * FFR Req. | 2,020 | - Ratio=2 - The right hand side of the equation is 1400 + 2*800 = 3000 - G1 PFR Offer is marginal - G1 capacity is fully utilized. Opportunity cost for energy incorporated into this constraint’s shadow price |
| Max FFR Procurement | ( FFR1+FFR2) Award <= FFR Max Req. | -4,034 | - Shadow price is negative - The right hand side of the equation is 800 |
| Max FFR1 Procurement | FFR1 Award <= FFR1 Max Req. | -3 | - Shadow price is negative - The right hand side of the equation is 100 |
| CR Procurement | CR1 Award + CR2 Award >= CR Req. | 4 | - The right hand side of the equation is 700 - LR3 CR Offer is marginal |
| Min CR1 Procurement | CR1 Award >= CR1 Req. | 2,015 | - The right hand side of the equation is 200 - G1 CR Offer is marginal - G1 capacity is fully utilized. Opportunity cost for energy incorporated into this constraint’s shadow price |

MCPCs:

| AS MCPC | Equation | $/MW |
| --- | --- | --- |
| PFR | SPPFR+FFR | 2,020 $/MW Note: This is the marginal Offer price for PFR from Gen plus opportunity cost of energy |
| FFR1 | FAST Proposal: Min(R*SPPFR+FFR,VOLL) | Min(2*2020, 9000) = 4,040 $/MW |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax+ SPFFR1Max) | Max(2020, 2*2020-4034-3) = 2020 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax+ SPFFR1Max | 2*2020-4034-3 = 3 $/MW Note: This is the marginal Offer price for FFR1 |
| FFR2 | FAST Proposal: Min(R*SPPFR+FFR,VOLL) | Min(2*2020, 9000) = 4,040 $/MW |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax) | Max(2020, 2*2020-4034) = 2020 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax | 2*2020-4034 = 6 $/MW Note: This is the marginal Offer price for FFR2 from Load Resource |
| CR1 | SPCR+ SPCR1 | 4 + 2015 = 2,019 $/MW Note: This is the marginal Offer price for CR from Gen plus opportunity cost of energy |
| CR2 | FAST Proposal: SPCR+ SPCR1 | 4 + 2015 = 2,019 $/MW |
|  | Alternative Proposal 2: SPCR | 4 $/MW Note: This is the marginal Offer price for CR from Load Resource |

Scenario 4a:  Base Case changed - G2 HSL = 6,600 MW, G1 Energy Offer = 100 $/MWh, PFR/FFR Ratio = 1

Red highlights are changes to input from Base Case

Yellow highlights are results/outputs of DAM

AS Plan: Assume RegUp, RegDn, & SR requirements are zero MW

| PFR Minimum Requirement (MW) | FFR Target Requirement (MW) | FFR Max Requirement (MW) | FFR1 Max Requirement (MW) | PFR/FFR Ratio | CR Requirement (MW) | CR1 Min Requirement (MW) |
| --- | --- | --- | --- | --- | --- | --- |
| 1,400 | 800 | 800 | 100 | 1 | 700 | 200 |

Energy Bid:

| Demand Bid (MWh) | Bid Price ($/MWh) | Bid Award (MW) |
| --- | --- | --- |
| 40,001 | 9,000 | 40,000 |

Generation Resource Limits, Energy Offer, AS Offer:

| Generation Resource | LSL (MW) | HSL (MW) | EOC ($/MWh) | PFR ($/MW) | CR ($/MW) | FFR1 ($/MW) | Energy Award (MW) | PFR Award (MW) | CR Award (MW) | FFR1 Award (MW) | HSL Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 | 0 | 5,000 | 100 | 20 | 19 | - | 3,400 | 1,400 | 200 |  | 100 % |
| G2 | 0 | 6,600 | 50 | 15 | 14 | - | 6,600 | 0 | 0 |  | 100 % |
| G3 | 0 | 20,000 | 20 | 10 | 9 | - | 20,000 | 0 | 0 |  | 100 % |
| G4 | 0 | 10,000 | 10 | - | - | - | 10,000 | - |  |  | 100 % |
| G5 | 0 | 20 | - | - | - | 2 |  |  |  | 20 | 100% |

Load Resource Limits, AS Offer:

| Load Resource | Type | LPC (MW) | MPC (MW) | FFR ($/MW) | CR ($/MW) | FFR Award (MW) | CR Award (MW) | MPC Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR1 | FFR1 | 0 | 200 | 3 | - | 80 | - | 40 % |
| LR2 | FFR2 | 0 | 1,000 | 6 | - | 700 | - | 70 % |
| LR3 | CR | 0 | 600 | - | 4 | - | 500 | 83 % |

Constraints:

| Constraints | Equation | Shadow Price ($/MW) | Comments |
| --- | --- | --- | --- |
| Power Balance | EnergySupply - EnergyDemand = 0 | 9,000 | - Energy Bid is marginal and sets energy price - Energy Bid partially awarded (40,000 MWh) |
| Combined PFR/FFR Procurement | PFR Award + Ratio *( FFR1+FFR2) Award >= PFR Req. + Ratio * FFR Req. | 8,920 | - Ratio=1 - The right hand side of the equation is 1400 + 1*800 = 2200 - G1 PFR Offer is marginal - G1 capacity is fully utilized. Opportunity cost for energy incorporated into this constraint’s shadow price |
| Max FFR Procurement | ( FFR1+FFR2) Award <= FFR Max Req. | -8914 | - Shadow price is negative - The right hand side of the equation is 800 |
| Max FFR1 Procurement | FFR1 Award <= FFR1 Max Req. | -3 | - Shadow price is negative - The right hand side of the equation is 100 |
| CR Procurement | CR1 Award + CR2 Award >= CR Req. | 4 | - The right hand side of the equation is 700 - LR3 CR Offer is marginal |
| Min CR1 Procurement | CR1 Award >= CR1 Req. | 8,915 | - The right hand side of the equation is 200 - G1 CR Offer is marginal - G1 capacity is fully utilized. Opportunity cost for energy incorporated into this constraint’s shadow price |

MCPCs:

| AS MCPC | Equation | $/MW |
| --- | --- | --- |
| PFR | SPPFR+FFR | 8,920 $/MW Note: This is the marginal Offer price for PFR from Gen plus opportunity cost of energy |
| FFR1 | FAST Proposal: Min(R*SPPFR+FFR, VOLL) | Min(1*8920, 9000) = 8,920 $/MW |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax+ SPFFR1Max) | Max(8920, 1*8920-8914-3) = 8920 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax+ SPFFR1Max | 1*8920-8914-3 = 3 $/MW Note: This is the marginal Offer price for FFR1 |
| FFR2 | FAST Proposal: Min(R*SPPFR+FFR, VOLL) | Min(1*8920, 9000) = 8,920 $/MW |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax) | Max(8920, 1*8920-8914) = 8920 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax | 1*8920-8914 = 6 $/MW Note: This is the marginal Offer price for FFR2 from Load Resource |
| CR1 | SPCR+ SPCR1 | 4 + 8915 = 8919 $/MW Note: This is the marginal Offer price for CR from Gen plus opportunity cost of energy |
| CR2 | FAST Proposal: SPCR+ SPCR1 | 4 + 8915 = 8919 $/MW |
|  | Alternative Proposal 2: SPCR | 4 $/MW Note: This is the marginal Offer price for CR from Load Resource |

Scenario 4b:  Base Case changed - G2 HSL = 6,600 MW, G1 Energy Offer = 100 $/MWh

Red highlights are changes to input from Base Case

Yellow highlights are results/outputs of DAM

AS Plan: Assume RegUp, RegDn, & SR requirements are zero MW

| PFR Minimum Requirement (MW) | FFR Target Requirement (MW) | FFR Max Requirement (MW) | FFR1 Max Requirement (MW) | PFR/FFR Ratio | CR Requirement (MW) | CR1 Min Requirement (MW) |
| --- | --- | --- | --- | --- | --- | --- |
| 1,400 | 800 | 800 | 100 | 2 | 700 | 200 |

Energy Bid:

| Demand Bid (MWh) | Bid Price ($/MWh) | Bid Award (MW) |
| --- | --- | --- |
| 40,001 | 9,000 | 40,000 |

Generation Resource Limits, Energy Offer, AS Offer:

| Generation Resource | LSL (MW) | HSL (MW) | EOC ($/MWh) | PFR ($/MW) | CR ($/MW) | FFR1 ($/MW) | Energy Award (MW) | PFR Award (MW) | CR Award (MW) | FFR1 Award (MW) | HSL Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 | 0 | 5,000 | 100 | 20 | 19 | - | 3,400 | 1,400 | 200 |  | 100 % |
| G2 | 0 | 6,600 | 50 | 15 | 14 | - | 6,600 | 0 | 0 |  | 100 % |
| G3 | 0 | 20,000 | 20 | 10 | 9 | - | 20,000 | 0 | 0 |  | 100 % |
| G4 | 0 | 10,000 | 10 | - | - | - | 10,000 | - |  |  | 100 % |
| G5 | 0 | 20 | - | - | - | 2 |  |  |  | 20 | 100% |

Load Resource Limits, AS Offer:

| Load Resource | Type | LPC (MW) | MPC (MW) | FFR ($/MW) | CR ($/MW) | FFR Award (MW) | CR Award (MW) | MPC Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LR1 | FFR1 | 0 | 200 | 3 | - | 80 | - | 40 % |
| LR2 | FFR2 | 0 | 1,000 | 6 | - | 700 | - | 70 % |
| LR3 | CR | 0 | 600 | - | 4 | - | 500 | 83 % |

Constraints:

| Constraints | Equation | Shadow Price ($/MW) | Comments |
| --- | --- | --- | --- |
| Power Balance | EnergySupply - EnergyDemand = 0 | 9,000 | - Energy Bid is marginal and sets energy price - Energy Bid partially awarded (40,000 MWh) |
| Combined PFR/FFR Procurement | PFR Award + Ratio *( FFR1+FFR2) Award >= PFR Req. + Ratio * FFR Req. | 8,920 | - Ratio=2 - The right hand side of the equation is 1400 + 2*800 = 3000 - G1 PFR Offer is marginal - G1 capacity is fully utilized. Opportunity cost for energy incorporated into this constraint’s shadow price |
| Max FFR Procurement | ( FFR1+FFR2) Award <= FFR Max Req. | -17,834 | - Shadow price is negative - The right hand side of the equation is 800 |
| Max FFR1 Procurement | FFR1 Award <= FFR1 Max Req. | -3 | - Shadow price is negative - The right hand side of the equation is 100 |
| CR Procurement | CR1 Award + CR2 Award >= CR Req. | 4 | - The right hand side of the equation is 700 - LR3 CR Offer is marginal |
| Min CR1 Procurement | CR1 Award >= CR1 Req. | 8,915 | - The right hand side of the equation is 200 - G1 CR Offer is marginal - G1 capacity is fully utilized. Opportunity cost for energy incorporated into this constraint’s shadow price |

MCPCs:

| AS MCPC | Equation | $/MW |
| --- | --- | --- |
| PFR | SPPFR+FFR | 8,920 $/MW Note: This is the marginal Offer price for PFR from Gen plus opportunity cost of energy |
| FFR1 | FAST Proposal: Min(R*SPPFR+FFR, VOLL) | Min(2*8920, 9000) = 9,000 $/MW Note: R*SPPFR+FFR = 17,840 Exceeds VOLL |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax+ SPFFR1Max) | Max(8920, 2*8920-17834-3) = 8920 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax+ SPFFR1Max | 2*8920-17834-3 = 3 $/MW Note: This is the marginal Offer price for FFR1 |
| FFR2 | FAST Proposal: Min(R*SPPFR+FFR, VOLL) | Min(2*8920, 9000) = 9,000 $/MW Note: : R*SPPFR+FFR = 17,840 Exceeds VOLL |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax) | Max(8920, 2*8920-17834) = 8920 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax | 2*8920-17834 = 6 $/MW Note: This is the marginal Offer price for FFR2 from Load Resource |
| CR1 | SPCR+ SPCR1 | 4 + 8915 = 8919 $/MW Note: This is the marginal Offer price for CR from Gen plus opportunity cost of energy |
| CR2 | FAST Proposal: SPCR+ SPCR1 | 4 + 8915 = 8919 $/MW |
|  | Alternative Proposal 2: SPCR | 4 $/MW Note: This is the marginal Offer price for CR from Load Resource |

Scenario 5: Example with RegUp/FRRS-Up,  RegDn/FRRS-Dn, PFR, FR1, FFR2, CR1, CR2, SR1, SR2

Yellow highlights are results/outputs of DAM

AS Plan:

| RegUp Requirement (MW) | FRRS-Up Max MW | RegDn Requirement (MW) | FRRS-Dn Max MW | SR Requirement (MW) | SR1 Min Requirement (MW) |  |
| --- | --- | --- | --- | --- | --- | --- |
| 200 | 50 | 100 | 30 | 100 | 50 |  |
| PFR Minimum Requirement (MW) | FFR Target Requirement (MW) | FFR Max Requirement (MW) | FFR1 Max Requirement (MW) | PFR/FFR Ratio | CR Requirement (MW) | CR1 Min Requirement (MW) |
| 1,400 | 800 | 800 | 100 | 2 | 700 | 200 |

Energy Bid:

| Demand Bid (MWh) | Bid Price ($/MWh) | Bid Award (MW) |
| --- | --- | --- |
| 40,000 | 9,000 | 40,000 |

Generation Resource Limits, Energy Offer, AS Offer:

| Generation Resource | LSL (MW) | HSL (MW) | EOC ($/MWh) | RegUp ($/MW) | PFR ($/MW) | CR ($/MW) | SR ($/MW) | RegDn ($/MW) | FFR1 ($/MW) | FRRS-Up ($/MW) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 | 0 | 5,000 | 7,000 | 22 | 20 | 19 | 16 |  | - | - |
| G2 | 0 | 15,000 | 50 | 15 | 15 | 13 | 10 | 12 | - | - |
| G3 | 0 | 25,000 | 20 | 12 | 10 | 7 | 4 | 8 | - | - |
| G4 | 0 | 10,000 | 15 |  | - | - |  |  | - | - |
| G5 | 0 | 20 | - |  | - | - |  |  | 8 | 8 |

Generation Resource Energy and Ancillary Service Awards::

| Generation Resource | Energy Award (MW) | RegUp Award (MW) | PFR Award (MW) | CR Award (MW) | SR Award (MW) | RegDn Award (MW) | FFR1 Award (MW) | FRRS-Up Award (MW) | HSL Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 | 0 | 0 | 0 | 0 | 0 |  |  |  | 0 % |
| G2 | 5,000 | 180 | 1,400 | 200 | 50 | 0 |  |  | 46 % |
| G3 | 25,000 | 0 | 0 | 0 | 0 | 80 |  |  | 100 % |
| G4 | 10,000 |  |  |  |  |  |  |  | 100 % |
| G5 |  |  |  |  |  |  | 0 | 20 | 100% |

Load Resource Limits, AS Offer:

| Load Resource | Offer-Type | LPC (MW) | MPC (MW) | FFR ($/MW) | CR ($/MW) | SR ($/MW) | FRRS-Dn ($/MW) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LR1 | FFR1 | 0 | 200 | 3 | - |  |  |
| LR2 | FFR2 | 0 | 1,000 | 6 | - |  |  |
| LR3 | CR2 | 0 | 600 | - | 4 |  |  |
| LR4 | SR2 | 0 | 700 |  |  | 2 |  |
| CLR-1 | FRRS-Dn | 0 | 20 |  |  |  | 8 |

Load Resource Ancillary Service Awards:

| Load Resource | FFR1 Award (MW) | FFR2 Award (MW) | CR2 Award (MW) | SR2 Award (MW) | FRRS-Dn Award (MW) | MPC Capacity Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- |
| LR1 | 100 |  |  |  |  | 50 % |
| LR2 |  | 700 |  |  |  | 70 % |
| LR3 |  |  | 500 |  |  | 83 % |
| LR4 |  |  |  | 50 |  | 7 % |
| CLR-1 |  |  |  |  | 20 | 100 % |

Constraints:

| Constraints | Equation | Shadow Price ($/MW) | Comments |
| --- | --- | --- | --- |
| Power Balance | EnergySupply - EnergyDemand = 0 | 50 | - G2 EOC is marginal and sets energy price - The Energy bid is fully awarded |
| Combined PFR/FFR Procurement | PFR Award + Ratio *( FFR1+FFR2) Award >= PFR Req. + Ratio * FFR Req. | 15 | - Ratio=2 - The right hand side of the equation is 1400 + 2*800 = 3000 - G2 PFR Offer is marginal - G2 capacity is not fully utilized. There is no opportunity cost for energy or other AS incorporated into this constraint’s shadow price |
| Max FFR Procurement | ( FFR1+FFR2) Award <= FFR Max Req. | -24 | - Shadow price is negative - The right hand side of the equation is 800 |
| Max FFR1 Procurement | FFR1 Award <= FFR1 Max Req. | -3 | - Shadow price is negative - The right hand side of the equation is 100 |
| CR Procurement | CR1 Award + CR2 Award >= CR Req. | 4 | - The right hand side of the equation is 700 - LR3 CR Offer is marginal |
| Min CR1 Procurement | CR1 Award >= CR1 Req. | 9 | - The right hand side of the equation is 200 - G2 CR Offer is marginal - G2 capacity is not fully utilized. There is no opportunity cost for energy incorporated into this constraint’s shadow price |
| RegUp Procurement | RegUp Award + FRRS-Up Award >= RegUp Req. | 15 | - The right hand side of the equation is 200 - G2 RegUp Offer is marginal - G2 capacity is not fully utilized. There is no opportunity cost for energy incorporated into this constraint’s shadow price |
| Max FRRS-Up Procurement | FRRS-Up Award <= FRRS-Up Max Req. | 0 | - The right hand side of the equation is 50 - Shadow Price is zero as the constraint is not binding. G5 cannot sell more than 20 MW and the limit is 50 MW |
| RegDn Procurement | RegDn Award+FRRS-Dn Award >= RegDn Req. | 8 | - The right hand side of the equation is 100 - G3 RegDn Offer is marginal - G3 capacity is utilized. However, there is no opportunity cost for energy incorporated into this constraint’s shadow price. |
| Max FRRS-Dn Procurement | FRRS-Dn Award <= FRRS-Dn Max Req. | 0 | - The right hand side of the equation is 30 - Shadow Price is zero as the constraint is not binding. CLR-1 cannot sell more than 20 MW and the limit is 30 MW |
| SR Procurement | SR1 Award + SR2 Award >= SR Req. | 2 | - The right hand side of the equation is 100 - LR4 SR2 Offer is marginal - LR4 capacity is not fully utilized. |
| Min SR1 procurement | SR1 Award >= SR1 Req. | 8 | - The right hand side of the equation is 50 - G2 CR Offer is marginal - G2 capacity is not fully utilized. There is no opportunity cost for energy incorporated into this constraint’s shadow price |

MCPCs:

| AS MCPC | Equation | $/MW |
| --- | --- | --- |
| PFR | SPPFR+FFR | 15 $/MW Note: This is the marginal Offer price for PFR from Gen |
| FFR1 | FAST Proposal: Min(R*SPPFR+FFR, VOLL) | Min(2*15, 9000) = 30 $/MW |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax+ SPFFR1Max) | Max(15, 2*15-24-3) = 15 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax+ SPFFR1Max | 2*15-24-3 = 3 $/MW Note: This is the marginal Offer price for FFR1 |
| FFR2 | FAST Proposal: Min(R*SPPFR+FFR, VOLL) | Min(2*15, 9000) = 30 $/MW |
|  | Alternative Proposal 1: Max(SPPFR+FFR,R*SPPFR+FFR + SPFFRMax) | Max(15, 2*15-24) = 15 $/MW |
|  | Alternative Proposal 2: R*SPPFR+FFR + SPFFRMax | 2*15-24 = 6 $/MW Note: This is the marginal Offer price for FFR2 from Load Resource |
| CR1 | SPCR+ SPCR1 | 4 + 9 = 13 $/MW Note: This is the marginal Offer price for CR from Gen |
| CR2 | FAST Proposal: SPCR+ SPCR1 | 4 + 9 = 13 $/MW |
|  | Alternative Proposal 2: SPCR | 4 $/MW Note: This is the marginal Offer price for CR from Load Resource |
| RegUp | SPRegUp | 15 $/MW Note: This is the marginal Offer price for RegUp from Gen |
| FRRS-Up | FAST Proposal: SPRegUp | 15 $/MW |
|  | Alternative Proposal 1: Max(SPRegUp,SPRegUp + SPFRRS-UpMax) | Max(15,15+0) = 15 $/MW |
|  | Alternative Proposal 2: SPRegUp + SPFRRS-UpMax | 15+0 = 15 $/MW Note: If the FRRS-Up Maximum constraint was binding, then the MCPC for FRRS-Up would be less than MCPC for conventional RegUp |
| RegDn | SPRegDn | 8 $/MW Note: This is the marginal Offer price for RegDn from Gen |
| FRRS-Dn | FAST Proposal: SPRegDn | 8 $/MW |
|  | Alternative Proposal 1: Max(SPRegDn,SPRegDn + SPFRRS-DnMax) | Max(8,8+0) = 8 $/MW |
|  | Alternative Proposal 2: SPRegDn + SPFRRS-DnMax | 8+0 = 8 $/MW Note: If the FRRS-Dn Maximum constraint was binding, then the MCPC for FRRS-Dn would be less than MCPC for conventional RegDn |
| SR1 | SPSR+ SPSR1 | 2+8 = 10 $/MW Note: This is the marginal Offer price for SR from Gen |
| SR2 | FAST Proposal: SPSR+ SPSR1 | 2+8 = 10 $/MW |
|  | Alternative Proposal 2: SPSR | 2 $/MW Note: This is the marginal Offer price for SR from Load Resource |