| NPRR Number | 640 | NPRR Title | As-Built Clarification of Load Resource Participation in SCED and Non-Spin Qualification |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Recommended Approval |
| Date of Decision |  | October 9, 2014 |  |  |
| Proposed Effective Date |  | January 1, 2015 |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Nodal Protocol Sections Requiring Revision |  | 3.6.1, Load Resource Participation 8.1.1.2.1.3, Non-Spinning Reserve Qualification |  |  |
| Other Binding Documents Requiring Revision or Related Revision Requests |  | None. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) clarifies requirements for Load Resource participation in Security Constrained Economic Dispatch (SCED), and also aligns Section 8.1.1.2.1.3 with the current requirement that Load Resources may only qualify to provide Non-Spinning Reserve (Non-Spin) if they are a Controllable Load Resource qualified to participate in SCED. |  |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |  |
| Credit Work Group Review |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR640 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 7/10/14, NPRR640 and an Impact Analysis were posted. - On 8/14/14, PRS considered NPRR640. - On 9/2/14, ERCOT comments were posted. - On 9/11/14, PRS again considered NPRR640. - On 10/9/14, PRS considered the 9/11/14 PRS Report and Impact Analysis for NPRR640. |  |  |
| PRS Decision |  | On 8/14/14, PRS unanimously voted to table NPRR640 for one month. All Market Segments were present for the vote. On 9/11/14, PRS unanimously voted to recommend approval of NPRR640 as amended by the 9/2/14 ERCOT comments. All Market Segments were present for the vote. On 10/9/14, PRS unanimously voted to endorse and forward the 9/11/14 PRS Report and the Impact Analysis for NPRR640 to TAC. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 8/14/14, ERCOT requested PRS to table NPRR640, to allow additional time to revise the proposed Loads in SCED language with input from Demand Side Working Group (DSWG) and WMS prior to the next PRS meeting. On 9/11/14, PRS reviewed the 9/2/14 ERCOT comments. On 10/9/14, there was no discussion. |  |  |

| Business Case |  |
| --- | --- |
| Qualitative Benefits | Clarifies and aligns Protocol language with current ERCOT systems and processes. |
| Quantitative Benefits |  |
| Impact to Market Segments |  |
| Credit Implications | No. |
| Other |  |

| Sponsor |  |
| --- | --- |
| Name | Paul Wattles |
| E-mail Address | Paul.Wattles@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6578 |
| Cell Number |  |
| Market Segment | Not applicable. |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Cory Phillips |
| E-Mail Address | cory.phillips@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 090214 | Clarified Load Resource qualifications for participation in SCED. |

Comments

Please note that NPRR633, Remove Requirement for Confirmation of Ancillary Service Test Using Primary and Secondary Voice Circuits, also proposes revisions to Section 8.1.1.2.1.3.

Proposed Protocol Language Revision

- 3.6.1	Load Resource Participation

(1)	A Load Resource may participate by providing:

(a)	Ancillary Service:

(i)	Regulation Up (Reg-Up) Service as a Controllable Load Resource capable of providing Primary Frequency Response;

(ii)	Regulation Down (Reg-Down) Service as a Controllable Load Resource capable of providing Primary Frequency Response;

(iii)	Responsive Reserve (RRS) Service as a Controllable Load Resource qualified for Security-Constrained Economic Dispatch (SCED) Dispatch and capable of providing Primary Frequency Response, or as a Load Resource controlled by high-set under-frequency relay; and

(iv)	Non-Spinning Reserve (Non-Spin) Service as a Controllable Load Resource qualified for SCED Dispatch;

(b)	Energy in the form of Demand response from a Controllable Load Resource in Real-Time via SCED;

(c)	Emergency Response Service (ERS) for hours in which the Load Resource does not have an Ancillary Service Resource Responsibility; and

(d)	Voluntary Load response in Real-Time.

(2)	Except for voluntary Load response and ERS, loads participating in any ERCOT market must be registered as a Load Resource and are subject to qualification testing administered by ERCOT.

(3)	All ERCOT Settlements resulting from Load Resource participation are made only with the Qualified Scheduling Entity (QSE) representing the Load Resource.

(4)	A QSE representing a Load Resource and submitting a bid to buy for participation in SCED, as described in Section 6.4.3.1, Real-Time Market (RTM) Energy Bids, must represent the Load Serving Entity (LSE) serving the Load of the Load Resource.  If the Load Resource is an Aggregate Load Resource (ALR), the QSE must represent the LSE serving the Load of all sites within the ALR.

(5)	The Settlement Point for a Controllable Load Resource with an RTM Energy Bid is its Load Zone Settlement Point.

8.1.1.2.1.3	Non-Spinning Reserve Qualification

(1)	Each Resource providing Non-Spin must be capable of being synchronized and ramped to its Ancillary Service Schedule for Non-Spin within 30 minutes.  Non-Spin may be provided from Generation Resource capacity that can ramp within 30 minutes or Load Resources capable of unloading within 30 minutes.  Non-Spin may only be provided from capacity that is not fulfilling any other energy or capacity commitment.

(2)	A Controllable Load Resource providing Non-Spin must be qualified to participate in SCED and must provide a telemetered output signal, including breaker status.

(3)	Each Generation Resource and Controllable Load Resource providing Non-Spin must meet additional technical requirements specified in this Section.

(4)	QSEs using a Controllable Load Resource to provide Non-Spin must be capable of responding to ERCOT Dispatch Instructions in a similar manner to QSEs using Generation Resource to provide Non-Spin.

(5)	Each QSE shall ensure that each Resource is able to meet the Resource’s obligations to provide the Ancillary Service Resource Responsibility.  Each Generation Resource and Controllable Load Resource providing Non-Spin must meet additional technical requirements specified in this Section.

(6)	For any Resource requesting qualification for Non-Spin, a qualification test for each Resource to provide Non-Spin is conducted during a continuous eight hour period agreed to by the QSE and ERCOT.  ERCOT shall confirm the date and time of the test with the QSE using both the primary and alternate voice circuits to validate the voice circuits. ERCOT shall administer the following test requirements.

(a)	At any time during the window (selected by ERCOT when market and reliability conditions allow and not previously disclosed to the QSE), ERCOT shall notify the QSE by using the messaging system and requesting that the QSE provide an amount of Non-Spin from each  Resource equal to the amount for which the QSE is requesting qualification.  The QSE shall acknowledge the start of the test.

(b)	For Generation Resources: during the test window, ERCOT shall send a message to the QSE representing a Generation Resources to deploy Non-Spin.  ERCOT shall monitor the adjustment of the Generation Resource’s Non-Spin Ancillary Service Schedule within five minutes for Resources On-Line.  ERCOT shall measure the test Resource’s response as described under Section 8.1.1.4.3, Non-Spinning Reserve Service Energy Deployment Criteria.  ERCOT shall evaluate the response of the Generation Resource given the current operating conditions of the system and determine the Resource’s qualification to provide Non-Spin.

(c)	For Controllable Load Resources, ERCOT shall send an instruction to deploy Non-Spin.  ERCOT shall measure the Resource’s response as described under Section 8.1.1.4.3.