| NPRR Number | 664 | NPRR Title | Index Fuel Price |
| --- | --- | --- | --- |
| Date |  | October 30, 2014 |  |
| Submitter’s Information |  |  |  |
| Name |  | Seth Cochran on behalf of Wholesale Market Subcommittee (WMS) |  |
| E-mail Address |  | seth.cochran@dc-energy.com |  |
| Company |  | DC Energy |  |
| Phone Number |  | 512-971-8767 |  |
| Cell Number |  |  |  |
| Market Segment |  | Not applicable |  |

Comments

At its October 29, 2014 meeting, WMS unanimously voted to endorse Nodal Protocol Revision Request (NPRR) 664 as revised by WMS and to recommend the Protocol Revision Subcommittee (PRS) Grant it Urgent status.  All Market Segments were present for the vote.

Proposed Protocol Language Revision

## DEFINITIONS

Index Fuel Price (IFP)

The Index Fuel Price (IFP) for a Resource shall be equal to one of the following:

(a)	Fuel Index Price (FIP);

(b)	Waha Fuel Price; or

(c)	The weighted fuel price of FIP and Waha Fuel Price, as described in Section 7 of the Verifiable Cost Manual.

If the IFP is not designated by the Resource, the IFP will default to FIP.

Waha Fuel Price

The midpoint price expressed in dollars per million British thermal units ($/MMBtu), published in Gas Daily, in the Daily Price Survey, under the heading “Permian Basin Area, Waha.”  The Gas Daily indicates which flow dates the prices are effective.  For Saturdays, Sundays, holidays, and other days for which Gas Daily does not publish an effective price, the effective price shall be the effective price for the Operating Day following the holiday or day without a published price.  If, or at the time of Settlement or calculation of generic costs, the described midpoint price for a particular Operating Day is not available, the effective price for the most recent preceding Operating Day shall be used.

2.2 	ACRONYMS AND ABBREVIATIONS

IFP	Index Fuel Price

3.14.1	Reliability Must Run

(1)	RMR Service is the use by ERCOT, under contracts with Resource Entities, of capacity and energy from Generation Resources that otherwise would not operate and that are necessary to provide voltage support, stability or management of localized transmission constraints under applicable reliability criteria, where market solutions do not exist.  This includes service provided by RMR Units and Must-Run Alternative (MRA) Resources.

(a)	Upon receiving Notice from a Resource Entity as described in Section 3.14.1.1, Notification of Suspension of Operations, ERCOT may enter into RMR Agreements and begin procurement of RMR Service under this Section.

(b)	Before entering into an RMR Agreement, ERCOT shall assess alternatives to the proposed RMR Agreement.  ERCOT shall evaluate and present in a written report posted on the Market Information System (MIS) Secure Area the information in items (i) through (v) below.  ERCOT is not limited in the number of additional scenarios it chooses to evaluate.  The written report shall include an explanation as to why the items below are insufficient, either alone or in combination, to fill the requirement that will be met by the potential RMR Unit.  The report shall be posted in the time frame required under paragraph (3) of Section 3.14.1.2, ERCOT Evaluation.  The list of alternatives ERCOT must consider includes (as reasonable for each type of reliability concern identified):

(i)	Redispatch/reconfiguration through operator instruction;

(ii)	Remedial Action Plans (RAPs);

(iii)	Special Protection Systems (SPSs) initiated on unit trips or Transmission Facilities’ Outages;

(iv)	Load response alternatives once a suitable Load response service is defined and available; and

(v)	Resource alternatives, including capabilities of Distributed Generation (DG), Load Resources, Direct Current Ties (DC Ties), Block Load Transfers (BLTs), etc.

(c)	ERCOT shall minimize the use of RMR Units as much as practicable subject to the other provisions of these Protocols.  ERCOT may Dispatch an RMR Unit at any time for ERCOT System security.

(d)	Each RMR Unit must meet technical requirements specified in Section 8.1.1.1, Ancillary Service and Reserves Qualification and Testing.

(e)	ERCOT may execute RMR Agreements for no less than one month and no more than one year, with one exception.  ERCOT may execute an RMR Agreement for a term longer than 12 months if the Resource Entity must make a significant capital expenditure to meet environmental regulations or to ensure availability to continue operating the RMR Unit so as to make an RMR Agreement in excess of 12 months appropriate, in ERCOT’s opinion.  The term of a multi-year RMR Agreement must take into account the appropriate RMR exit strategy discussed in Section 3.14.1.4, Exit Strategy from an RMR Agreement.  In the event ERCOT chooses to contract for an RMR Unit for longer than one year, ERCOT shall annually re-evaluate the need for the RMR Unit under the criteria set forth in paragraph (b) above.  If ERCOT determines the RMR Unit is no longer needed, ERCOT shall enter into exit negotiations with the contract signatories to attempt to exit the contract early.  However, ERCOT shall not enter into such negotiations until a Market Notice is issued providing the anticipated RMR exit time frame.  The RMR standard Agreement is included in Section 22, Attachment B, Standard Form Reliability Must-Run Agreement.  ERCOT shall post each RMR Agreement in its entirety, including amendments or modifications thereto, within five Business Days of execution on the MIS Secure Area.

(f)	A Generation Resource is eligible for RMR status based on criteria established by ERCOT indicating its operation is necessary to support ERCOT System reliability according to the Operating Guides.  A combined-cycle generation Facility must be treated as a single unit for RMR purposes unless the combustion turbine and the steam turbine can operate separately.  If the steam turbine and combustion turbine can operate separately, and the steam turbine is powered by waste heat from more than one combustion turbine, the combustion turbine accepted for RMR Service and a proportionate part of the steam turbine must be treated as a single unit for RMR purposes.  If the combustion turbine accepted for RMR Service can operate separately from the steam turbine, and only the combustion turbine is accepted as an RMR Unit, the RMR energy price will be reduced by the value of the combustion turbine’s waste heat calculated at the Index Fuel Price (IFP), except when the steam turbine is Off-Line.  ERCOT shall post to the MIS Secure Area the criteria upon which it evaluates whether an RMR Unit meets the test of operational necessity to support ERCOT System reliability within five Business Days of change and shall issue a Market Notice stating the determination is available.  This includes the case where a unit previously identified by ERCOT as potentially needed for RMR Service is no longer needed regardless of whether an RMR Agreement was ever signed.

(g)	A Resource Entity cannot be compelled to enter into an RMR Agreement.  A Resource Entity that owns a Generation Resource that is uneconomic to remain in service can voluntarily petition ERCOT for contracted RMR status by following the process in this subsection.  ERCOT shall determine whether the Generation Resource is necessary for system reliability based on the criteria set forth in this Section.

(h)	ERCOT must contract for the entire capacity of each RMR Unit.

(i)	ERCOT shall post on the MIS Secure Area all information relative to the use of RMR Units including energy deployed monthly.

(j)	The Resource Entity that owns the RMR Unit may not use the RMR Unit for:

(i)	Participating in the bilateral energy market;

(ii)	Self-providing of energy except for plant auxiliary Load obligations under the RMR Agreement; and

(iii)	Providing of Ancillary Service to any Entity.

(k)	ERCOT shall issue a Market Notice on the need for an RMR Unit prior to entering negotiations for the RMR Unit.  Such Market Notice shall include the link to the ERCOT final RMR evaluation, the Resource name and pneumonic, the name of the Resource Entity, the name of the Qualified Scheduling Entity (QSE) for the Resource, the Resource MW rating by Season, and potential duration of the RMR Agreement, including anticipated start and end dates.

(l)	ERCOT shall, through the issuance of Market Notices, provide the same information, contemporaneously, about the need for, or elimination of an RMR Unit to all registered Market Participants, including QSEs and Resource Entities with RMR Units.

3.14.1.16	Reporting Actual Eligible Fuel Costs

(1)	The RMR Unit owner shall provide ERCOT with actual fuel costs on a monthly basis for the RMR Unit in a level of detail sufficient for ERCOT to verify that all fuel costs are actual and appropriate.  The estimated fuel payments may include a fuel adder to better approximate expected actual fuel costs.  The fuel adder shall represent the difference between the forecasted average actual future fuel price paid and the forecasted average of the relevant index price Index Fuel Price (IFP), Fuel Oil Price (FOP) or solid fuel) over the RMR contract period.  The fuel adder must also include the forecasted cost of transporting, delivering and fuel imbalances to the Resource.  QSEs must provide to ERCOT supporting documentation indicating how the fuel adder was determined.  ERCOT shall perform a true-up of the estimated fuel costs using the submitted and verified actual fuel costs for the RMR Unit.  Actual cost data must be submitted on time by the Resource Entity for the RMR Unit and then verified by ERCOT so the actual cost data can be reflected in the True-Up Settlement Statement.  To be considered timely for the final, actual cost data for month ‘x’ must be submitted by the 20th of the month following month ‘x.’  To be considered timely for the true-up, actual cost data for month ‘x’ must be submitted 30 days prior to the publishing date of the True-Up Settlement Statement for the first day in month ‘x.’  Any deviation in filing actual cost data in accordance with this calendar must be requested of ERCOT, by the QSE representing an RMR Unit.  Such request for deviation shall contain the reason for the inability to meet the calendar and an expected date that the cost data will be provided to ERCOT.  At its discretion ERCOT may choose to honor such a request.  ERCOT shall post on the MIS Public Area any such request and response thereto.  In the event that actual cost data is not submitted in accordance with the calendar or approved deviation for the true-up, then the cost for the portion of Eligible Cost that has not been submitted is deemed to be zero.

(2)	Actual fuel costs must be appropriate actual costs attributable to ERCOT’s scheduling and/or deployment of the RMR Unit.  Actual fuel costs may include cost of fuel (including the cost of exceeding swing gas contract limits, additional gas demand costs set by fuel supply, or transportation contracts); demand fees, imbalance penalties, transportation charges, and cash out premiums.

4.4.9.2	Startup Offer and Minimum-Energy Offer

The Startup Offer component represents all costs incurred by a Generation Resource in starting up and reaching its LSL, minus the average energy produced during the time period between breaker close and LSL multiplied by the heat rate proxy “H” multiplied by the appropriate Index Fuel Price (IFP) or Fuel Oil Price (FOP).  The Minimum-Energy Offer component represents a proxy for the costs incurred by a Resource in producing energy at the Resource’s LSL.

4.4.9.2.1	Startup Offer and Minimum-Energy Offer Criteria

(1)	Each Startup Offer and Minimum-Energy Offer must be reported by a QSE and must include the following information:

(a)	The selling QSE;

(b)	The Resource represented by the QSE from which the offer would be supplied;

(c)	The Resource’s hot, intermediate, and cold Startup Offer in dollars;

(d)	The Resource’s Minimum-Energy Offer in dollars per MWh;

(e)	The first and last hour of the Startup and Minimum-Energy Offers

(f)	The expiration time and date of the offer;

(g)	Percentage of the IFP to the extent that the startup and minimum energy will be supplied by gas to determine the offer cap; and

(h)	Percentage of the FOP to the extent that the startup and minimum energy will be supplied by oil to determine the offer cap.

(2)	Valid Startup Offers and Minimum-Energy Offers (which must be part of a Three-Part Supply Offer) must be received before 1000 for the effective DAM and DRUC.

(3)	A QSE may update and submit a Three-Part Supply Offer for a Resource during the Adjustment Period for any hours in which the Resource is not RUC-committed before the offer is updated or submitted.

(4)	The Resource’s Startup Offer must be equal to or less than the Resource Category Generic Startup Cost for that type of Resource listed in Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, unless ERCOT has approved verifiable Resource-specific startup costs for that Resource, under Section 4.4.9.2.4, Verifiable Startup Offer and Minimum-Energy Offer Caps, in which case the Resource’s Startup Offer must be equal to or less than those approved verifiable Resource-specific startup costs.

(5)	The Resource’s Minimum-Energy Offer must be equal to or less than the Resource Category Generic Minimum-Energy Cost for that type of Resource listed in Section 4.4.9.2.3 unless ERCOT has approved verifiable Resource-specific minimum-energy costs for that Resource, under Section 4.4.9.2.4 in which case the Resource’s Minimum-Energy Offer must be equal to or less than those approved verifiable Resource-specific minimum-energy costs.

(6)	Prior to 1000 for the effective DAM, a QSE may submit and update a Three-Part Supply Offer for a Resource for any hours which were Weekly Reliability Unit Commitment (WRUC)-instructed.

4.4.9.2.3	Startup Offer and Minimum-Energy Offer Generic Caps

(1)	The Resource Category Startup Offer Generic Cap, by applicable Resource category, is determined by the following Operations and Maintenance (O&M) costs by Resource category:

| Resource Category | O&M Costs ($) |
| --- | --- |
| Nuclear, coal, lignite and hydro | 7,200 |
| Combined Cycle Generation Resource with a combustion turbine ≥ 90 MW, as determined by the largest combustion turbine in the Combined Cycle Generation Resource and for each combustion turbine in the Combined Cycle Generation Resource | 6,810 |
| Combined Cycle Generation Resource with a combustion turbine < 90 MW, as determined by the largest combustion turbine in the Combined Cycle Generation Resource and for each combustion turbine in the Combined Cycle Generation Resource | 6,810 |
| Gas steam supercritical boiler | 4,800 |
| Gas steam reheat boiler | 3,000 |
| Gas steam non-reheat or boiler w/o air-preheater | 2,310 |
| Simple cycle greater than 90 MW | 5,000 |
| Simple cycle less than or equal to 90 MW | 2,300 |
| Reciprocating Engines | $58 /MW * the average of the seasonal net max sustainable ratings |
| RMR Resource | Not Applicable |
| Wind generation Resources | 0 |
| Any Resources not defined above | 0 |

[NPRR588: Replace paragraph (1) above with the following upon system implementation:] (1) The Resource Category Startup Offer Generic Cap, by applicable Resource category, is determined by the following Operations and Maintenance (O&M) costs by Resource category: Resource Category; O&M Costs ($). Nuclear, coal, lignite and hydro; 7,200. Combined Cycle Generation Resource with a combustion turbine ≥ 90 MW, as determined by the largest combustion turbine in the Combined Cycle Generation Resource and for each combustion turbine in the Combined Cycle Generation Resource; 6,810. Combined Cycle Generation Resource with a combustion turbine < 90 MW, as determined by the largest combustion turbine in the Combined Cycle Generation Resource and for each combustion turbine in the Combined Cycle Generation Resource; 6,810. Gas steam supercritical boiler; 4,800. Gas steam reheat boiler; 3,000. Gas steam non-reheat or boiler w/o air-preheater; 2,310. Simple cycle greater than 90 MW; 5,000. Simple cycle less than or equal to 90 MW; 2,300. Reciprocating Engines; $58 /MW * the average of the seasonal net max sustainable ratings. RMR Resource; Not Applicable. Wind generation Resources; 0. PhotoVoltaic Generation Resources (PVGRs); 0. Any Resources not defined above; 0.

(2)	The Resource Category Minimum-Energy Generic Cap is the cost per MWh of energy for a Resource to produce energy at the Resource’s LSL and is as follows:

(a)	Hydro = $10.00/MWh;

- (b)	Coal and lignite = $18.00/MWh;

- (c)	Combined-cycle greater than 90 MW = 8 MMBtu/MWh * ((Percentage of IFP * IFP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (d)	Combined-cycle less than or equal to 90 MW = 9 MMBtu/MWh * ((Percentage of IFP * IFP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (e)	Gas steam supercritical boiler = 14 MMBtu/MWh * ((Percentage of IFP * IFP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (f)	Gas steam reheat boiler = 14.5 MMBtu/MWh * ((Percentage of IFP * IFP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (g)	Gas steam non-reheat or boiler without air-preheater = 16.0 MMBtu/MWh * ((Percentage of IFP * IFP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (h)	Simple-cycle greater than 90 MW = 15.0 MMBtu/MWh * ((Percentage of IFP * IFP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (i)	Simple-cycle less than or equal to 90 MW = 14.0 MMBtu/MWh * ((Percentage of IFP * IFP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (j)	Reciprocating engines = 16.0 MMBtu/MWh * ((Percentage of IFP * IFP) + (Percentage of FOP * FOP))/100, as specified in the Minimum-Energy Offer;

- (k)	RMR Resource = RMR contract estimated fuel cost using its contract I/O curve at its LSL times IFP;

- (l)	Nuclear = Not Applicable;

- (m)	Wind generation Resources = $0; and

[NPRR588: Insert item (n) below upon system implementation and renumber accordingly:] - (n) PVGRs = $0;

- (n)	Other Resources not defined above = $0.

- (3)	The IFP and FOP used to calculate the Resource Category Minimum-Energy Generic Cap shall be the IFP or FOP for the Operating Day.  In the event the Resource Category Minimum-Energy Generic Cap must be calculated before the IFP or FOP is available for the particular Operating Day, the IFP and FOP for the most recent preceding Operating Day shall be used.  Once the IFP and FOP are available for a particular Operating Day, those values shall be used in the calculations.  If the percentage fuel mix is not specified for Resource categories having the option to specify the fuel mix, then the minimum of IFP or FOP shall be used.

(4)	Items (2)(c) and (2)(d) above are determined by capacity of largest simple-cycle combustion turbine in the train.

4.4.9.3.1	Energy Offer Curve Criteria

(1)	Each Energy Offer Curve must be reported by a QSE and must include the following information:

(a)	The selling QSE;

(b)	The Resource represented by the QSE from which the offer would be supplied;

(c)	A monotonically increasing offer curve for both price (in $/MWh) and quantity (in MW) with no more than ten price/quantity pairs;

(d)	The first and last hour of the Offer;

(e)	The expiration time and date of the offer;

(f)	List of Ancillary Service Offers from the same Resource;

(g)	Inclusive or exclusive designation relative to other DAM offers; and

(h)	Percentage of IFP and percentage of FOP for generation above LSL subject to the sum of the percentages not exceeding 100%.

(2)	An Energy Offer Curve must be within the range of -$250.00 per MWh and the SWCAP in dollars per MWh.  The software systems must be able to provide ERCOT with the ability to enter Resource-specific Energy Offer Curve floors and caps.

(3)	The minimum amount per Resource for each Energy Offer Curve that may be offered is one MW.

4.4.9.3.3	Energy Offer Curve Caps for Make-Whole Calculation Purposes

(1)	The following Energy Offer Curve Caps must be used for the purpose of make-whole Settlements:

- (a)	Nuclear = $15.00/MWh;

- (b)	Coal and Lignite = $18.00/MWh;

- (c)	Combined Cycle greater than 90 MW = 9 MMBtu/MWh * ((Percentage of IFP*IFP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (d)	Combined Cycle less than or equal to 90 MW = 10 MMBtu/MWh * ((Percentage of IFP*IFP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (e)	Gas - Steam Supercritical Boiler = 10.5 MMBtu/MWh * ((Percentage of IFP*IFP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (f)	Gas Steam Reheat Boiler = 11.5 MMBtu/MWh * ((Percentage of IFP*IFP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (g)	Gas Steam Non-reheat or boiler without air-preheater = 14.5 MMBtu/MWh * ((Percentage of IFP*IFP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (h)	Simple Cycle greater than 90 MW = 14 MMBtu/MWh * ((Percentage of IFP*IFP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (i)	Simple Cycle less than or equal to 90 MW = 15 MMBtu/MWh * ((Percentage of IFP*IFP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (j)	Reciprocating Engines = 16 MMBtu/MWh * ((Percentage of IFP*IFP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (k)	Hydro = $10.00/MWh;

- (l)	Other Renewable = $0/MWh; and

- (m)	RMR Resource = RMR contract price Energy Offer Curve.

(2)	Items in paragraphs (1)(c) and (d) above are determined by capacity of largest simple-cycle combustion turbine in the train selected.

(3)	The IFP and FOP used to calculate the Energy Offer Curve Cap for Make-Whole Payment calculation purposes shall be the IFP or FOP for the Operating Day.  In the event the Energy Offer Curve Cap for Make-Whole Payment calculation purposes must be calculated before the IFP or FOP is available for the particular Operating Day, the IFP and FOP for the most recent preceding Operating Day shall be used.  Once the IFP and FOP are available for a particular Operating Day, those values shall be used in the calculations.  If the percentage fuel mix is not specified or if no Energy Offer Curve exists, then the minimum of IFP or FOP shall be used.

4.4.9.4.1	Mitigated Offer Cap

Energy Offer Curves may be subject to mitigation in Real-Time operations under Section 6.5.7.3, Security Constrained Economic Dispatch, using a Mitigated Offer Cap.  The “Mitigated Offer Cap” is:

(a)	For a Resource contracted by ERCOT under paragraph (2) of Section 6.5.1.1, ERCOT Control Area Authority, ERCOT shall increase the O&M cost such that every point on the Mitigated Offer Cap curve (cap vs. output level) is greater than the SWCAP in $/MWh.

- (b)	For a Generation Resource that commences commercial operation after January 1, 2004, ERCOT shall construct an incremental Mitigated Offer Cap curve (Section 6.5.7.3) such that each point on the Mitigated Offer Cap curve (cap vs. output level) is the greater of:

(i)	14.5 MMBtu/MWh times the IFP; or

(ii)	The Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by [((Percentage of IFP * IFP) + (Percentage of FOP * FOP))/100 + fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual], as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (e) below.

- (c)	For all other Generation Resources, each point on the Mitigated Offer Cap curve (cap vs. output level) is the greater of:

(i)	10.5 MMBtu/MWh times the IFP; or

(ii)	The Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by [((Percentage of IFP * IFP) + (Percentage of FOP * FOP))/100 + fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual], as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (e) below.

(d)	Notwithstanding paragraphs (b)(ii) and (c)(ii) above, the Mitigated Offer Cap verifiable variable O&M cost ($/MWh) for Quick Start Generation Resources (QSGRs) shall incorporate the generic or verifiable O&M cost to start the Resource from first fire to LSL including the startup fuel, plus a minimum energy component to account for LSL commitment costs, and consideration of a fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual.

- (e)	The multipliers for paragraphs (b)(ii) and (c)(ii) above are as follows:

(i)	1.10 for Resources running at a ≥ 50% capacity factor for the previous 12 months;

(ii)	1.15 for Resources running at a ≥ 30 and < 50% capacity factor for the previous 12 months;

(iii)	1.20 for Resources running at a ≥ 20 and < 30% capacity factor for the previous 12 months;

(iv)	1.25 for Resources running at a ≥ 10 and < 20% capacity factor for the previous 12 months;

(v)	1.30 for Resources running at a ≥ 5 and < 10% capacity factor for the previous 12 months;

(vi)	1.40 for Resources running at a ≥ 1 and < 5% capacity factor for the previous 12 months; and

(vii)	1.50 for Resources running at a less than 1% capacity factor for the previous 12 months.

- (f)	The previous 12 months’ capacity factor must be updated by ERCOT by the 20th day of each month using the most recent data for use in the next month.  ERCOT shall post to the MIS Secure Area the capacity factor for each Resource before the start of the effective month.

- (g)	The process for developing the Mitigated Offer Cap in paragraphs (a), (b), (c), and (d) above must be described by ERCOT in a procedure approved by the appropriate Technical Advisory Committee (TAC) subcommittee, and posted to the MIS Secure Area within one Business Day after initial approval, and after each approved change.

4.4.9.4.2	Mitigated Offer Floor

Energy Offer Curves may be subject to mitigation in the RTM under Section 6.5.7.3, Security Constrained Economic Dispatch, using a Mitigated Offer Floor.  The “Mitigated Offer Floor” is:

| Resource Category | Mitigated Offer Floor |
| --- | --- |
| Nuclear and Hydro | -$250/MWh |
| Coal and Lignite | -$20/MWh |
| Combined Cycle | 1 MMBtu/MWh * IFP |
| Gas/Oil Steam and Combustion Turbine | 6 MMBtu/MWh * IFP or FOP, as specified in the Energy Offer Curve |
| Qualifying Facility (QF) | -$50/MWh |
| Wind | -$100/MWh |
| Other Renewables | -$50/MWh |

[NPRR588: Replace Section 4.4.9.4.2, Mitigated Offer Floor, above with the following upon system implementation:] 4.4.9.4.2 Mitigated Offer Floor Energy Offer Curves may be subject to mitigation in the RTM under Section 6.5.7.3, Security Constrained Economic Dispatch, using a Mitigated Offer Floor. The “Mitigated Offer Floor” is: Resource Category; Mitigated Offer Floor. Nuclear and Hydro; -$250/MWh. Coal and Lignite; -$20/MWh. Combined Cycle; 1 MMBtu/MWh * IFP. Gas/Oil Steam and Combustion Turbine; 6 MMBtu/MWh * IFP or FOP, as specified in the Energy Offer Curve. Qualifying Facility (QF); -$50/MWh. Wind; -$100/MWh. PhotoVoltaic (PV); -$50/MWh. Other Renewables; -$50/MWh.

5.6.1	Verifiable Costs

(1)	The Qualified Scheduling Entity (QSE) is responsible for submitting verifiable costs unless both the QSE and Resource Entity agree that the Resource Entity will have this responsibility, in which case both the QSE and Resource Entity shall submit an affidavit to ERCOT stating this arrangement.  Notwithstanding the foregoing, QSEs that submit Power Purchase or Tolling Agreements (PPAs) do not have the option of allowing Resource Entities to file verifiable costs.

(2)	Make-Whole Payments for a Resource are based on the Startup Offers and Minimum-Energy Offers for the Resource, limited by caps.  Until ERCOT approves verifiable unit-specific costs for that Resource, the caps are the Resource Category Startup Generic Cap and the Resource Category Minimum-Energy Generic Cap.  When ERCOT approves verifiable unit-specific costs for that Resource the caps are those verifiable unit-specific costs.  A QSE or Resource Entity may file verifiable unit-specific costs for a Resource at any time, but it must file those costs no later than 30 days after five Reliability Unit Commitment (RUC) events for that Resource in a calendar year.  A RUC event begins when a Resource receives a RUC instruction to come or stay On-Line and ends the later of when the Resource shuts down or the end of the Operating Day.  The most recent ERCOT-approved verifiable costs must be used going forward.

(3)	These unit-specific verifiable costs may include and are limited to the following average incremental costs:

(a)	Allocation of maintenance requirements based on number of starts between maintenance events using, at the option of the QSE or Resource Entity, either:

(i)	Manufacturer-recommended maintenance schedule;

(ii)	Historical data for the unit and actual maintenance practices; or

(iii)	Another method approved in advance by ERCOT in writing;

(b)	Startup fuel calculations based on recorded actual measured flows when the data is available or based on averages of historical flows for similar starts (for example, hot, cold, intermediate) when actual data is not available.  Startup fuel will include filing separately the startup fuel required to reach breaker close and fuel after breaker close to Low Sustained Limit (LSL).  Any fuel required to shutdown a Resource will be submitted as the fuel from breaker open to shutdown;

(c)	Operation costs;

(d)	Chemical costs;

(e)	Water costs; and

(f)	Emission credits.

(4)	Standard Operations and Maintenance (O&M) costs pursuant to paragraph (6) below may be used in lieu of the incremental O&M costs set forth in items (3)(a), (c), (d) and (e) above.

(5)	These unit-specific verifiable costs may not include:

(a)	Fixed costs, which are any cost that is incurred regardless of whether the unit is deployed or not; and

(b)	Costs for which the QSE or Resource Entity cannot provide sufficient documentation for ERCOT to verify the costs.

(6)	At their election, QSEs or Resource Entities may receive standard O&M costs for both startup and minimum energy.  This election may be made by submitting an election form to ERCOT.  If a QSE or Resource has received final approval for actual verifiable O&M costs under the verifiable cost process, it may not elect to receive standard O&M costs.

(a)	Until December 31, 2011, standard O&M costs are defined as follows:

| Resource Category Start Year = 2009 | Cold Startup ($/start) | Intermediate Startup ($/start) | Hot Startup ($/start) | Variable O&M ($/MWh) |
| --- | --- | --- | --- | --- |
| Aeroderivative simple cycle commissioned after 1996 | 1,000.00 | 1,000.00 | 1,000.00 | 3.94 |
| Reciprocating Engine | $58/MW * the average of the Seasonal net max sustainable ratings | $58/MW * the average of the Seasonal net max sustainable ratings | $58/MW * the average of the Seasonal net max sustainable ratings | 5.09 |
| Simple cycle ≤ 90 MW | 2,300.00 | 2,300.00 | 2,300.00 | 3.94 |
| Simple cycle ≥ 90 MW | 5,000.00 | 5,000.00 | 5,000.00 | 3.94 |
| Combined cycle: for each Combined-Cycle Configuration, the Startup Cost for that configuration is the sum of the Startup Costs for each unit within that configuration as follows: |  |  |  | 3.19 |
| Combustion turbine < 90 MW | 2,300.00 | 2,300.00 | 2,300.00 |  |
| Combustion turbine ≥ 90 MW | 5,000.00 | 5,000.00 | 5,000.00 |  |
| Steam turbine | 3,000.00 | 2,250.00 | 1,250.00 |  |
| Gas-steam non-reheat boiler | 2,310.00 | 1,732.50 | 866.25 | 7.08 |
| Gas-steam reheat boiler | 3,000.00 | 2,250.00 | 1,125.00 | 7.08 |
| Gas-steam supercritical boiler | 4,800.00 | 3,600.00 | 1,800.00 | 7.08 |
| Nuclear, coal, lignite and hydro | 7,200.00 | 5,400.00 | 2,700.00 | 5.02 |
| Renewable | Not Applicable | Not Applicable | Not Applicable | 5.50 |

(b)	For the period beginning January 1, 2012 and ending December 31, 2012, standard O&M costs shall be reduced by 10% from the levels specified in the table in paragraph (a) above as follows:

| Resource Category | Cold Startup ($/start) | Intermediate Startup ($/start) | Hot Startup ($/start) | Variable O&M ($/MWh) |
| --- | --- | --- | --- | --- |
| Start Year = 2009 |  |  |  |  |
| Aeroderivative simple cycle commissioned after 1996 | 900.00 | 900.00 | 900.00 | 3.55 |
| Reciprocating Engine | $52.20/MW * the average of the Seasonal net max sustainable ratings | $52.20/MW * the average of the Seasonal net max sustainable ratings | $52.20/MW * the average of the Seasonal net max sustainable ratings | 4.58 |
| Simple cycle ≤ 90 MW | 2,070.00 | 2,070.00 | 2,070.00 | 3.55 |
| Simple cycle ≥ 90 MW | 4,500.00 | 4,500.00 | 4,500.00 | 3.55 |
| Combined cycle: for each Combined-Cycle Configuration, the Startup Cost for that configuration is the sum of the Startup Costs for each unit within that configuration as follows: |  |  |  | 2.87 |
| Combustion turbine < 90 MW | 2,070.00 | 2,070.00 | 2,070.00 |  |
| Combustion turbine ≥ 90 MW | 4,500.00 | 4,500.00 | 4,500.00 |  |
| Steam turbine | 2,700.00 | 2,025.00 | 1,125.00 |  |
| Gas-steam non-reheat boiler | 2,079.00 | 1,559.25 | 779.63 | 6.37 |
| Gas-steam reheat boiler | 2,700.00 | 2,025.00 | 1,012.50 | 6.37 |
| Gas-steam supercritical boiler | 4,320.00 | 3,240.00 | 1,620.00 | 6.37 |
| Nuclear, coal, lignite and hydro | 6,480.00 | 4,860.00 | 2,430.00 | 4.52 |
| Renewable | Not Applicable | Not Applicable | Not Applicable | 4.95 |

(c)	Beginning January 1, 2013 and going forward, standard O&M costs shall be reduced by 20% from the levels specified in the table in paragraph (a) above as follows:

| Resource Category | Cold Startup ($/start) | Intermediate Startup ($/start) | Hot Startup ($/start) | Variable O&M ($/MWh) |
| --- | --- | --- | --- | --- |
| Start Year = 2009 |  |  |  |  |
| Aeroderivative simple cycle commissioned after 1996 | 800.00 | 800.00 | 800.00 | 3.15 |
| Reciprocating Engine | $46.40/MW * the average of the Seasonal net max sustainable ratings | $46.40 /MW * the average of the Seasonal net max sustainable ratings | $46.40 /MW * the average of the Seasonal net max sustainable ratings | 4.07 |
| Simple cycle ≤ 90 MW | 1,840.00 | 1,840.00 | 1,840.00 | 3.15 |
| Simple cycle ≥ 90 MW | 4,000.00 | 4,000.00 | 4,000.00 | 3.15 |
| Combined cycle: for each Combined-Cycle Configuration, the Startup Cost for that configuration is the sum of the Startup Costs for each unit within that configuration as follows: |  |  |  | 2.55 |
| Combustion turbine < 90 MW | 1,840.00 | 1,840.00 | 1,840.00 |  |
| Combustion turbine ≥ 90 MW | 4,000.00 | 4,000.00 | 4,000.00 |  |
| Steam turbine | 2,400.00 | 1,800.00 | 1,000.00 |  |
| Gas-steam non-reheat boiler | 1,848.00 | 1,386.00 | 693.00 | 5.66 |
| Gas-steam reheat boiler | 2,400.00 | 1,800.00 | 900.00 | 5.66 |
| Gas-steam supercritical boiler | 3,840.00 | 2,880.00 | 1,440.00 | 5.66 |
| Nuclear, coal, lignite and hydro | 5,760.00 | 4,320.00 | 2,160.00 | 4.02 |
| Renewable | Not Applicable | Not Applicable | Not Applicable | 4.40 |

(d)	If the QSE or Resource Entity chooses to utilize the standard O&M costs for O&M, standard O&M costs will be used by ERCOT going forward until either:

(i)	Verifiable variable O&M costs are filed; or

(ii)	ERCOT notifies the QSE or Resource Entity to update its verifiable costs as set forth in either paragraph (9) or (10) below.  If a Resource is receiving standard O&M costs, it may reelect standard O&M costs when resubmitting verifiable costs.

(7)	When submitting verifiable costs for combined cycle Resources, the QSE or Resource Entity must elect standard O&M costs for all Combined-Cycle Configurations or verifiable costs for all Combined-Cycle Configurations within the combined cycle train.

(8)	QSEs submitting PPAs as Resource-specific verifiable costs documentation are subject to the guidelines detailed below and in the Verifiable Cost Manual.

(a)	Only QSEs offering Three-Part Supply Offers for a specific Resource may submit a PPA as verifiable costs documentation.

(b)	A QSE submitting a PPA as verifiable costs documentation must represent 100% of the Resource’s capacity.

(c)	Only PPAs:

(i) 	Signed prior to July 16, 2008; and

(ii)	Not between Affiliates, subsidiaries or partners will be accepted as verifiable cost documentation.

(d)	Verifiable costs for PPAs shall be capped at the level of the highest comparable Resource (referred to as the reference Resource) specific verifiable costs approved by ERCOT without a PPA.  The ERCOT approved verifiable costs for a PPA shall be equal to the lesser of:

(i)	The cap as described in paragraph (d) above; and

(ii)	The costs from the PPA.

(e)	ERCOT shall use the Resource actual fuel costs submitted by the QSE for startup and operation at minimum-energy level (LSL), and shall use the Resource Category Startup Offer Generic Costs as the cap for the O&M portion of the Startup Costs until ERCOT receives and approves comparable Resource specific verifiable costs.

(f)	PPAs will no longer be accepted as verifiable cost documentation after the primary term of the contract expires.

(g)	ERCOT shall produce a report each April that provides the percentage of RUC Make-Whole Payments for Resources with PPAs during the 12 months of the previous calendar year.  The report shall be based on the final Settlements and include the total number of Resources that used a PPA for their most recent verifiable cost submission that was approved by ERCOT.  ERCOT shall present the results of this study to the appropriate Technical Advisory Committee (TAC) subcommittee.

(h)	Notwithstanding anything to the contrary in this Section 5.6.1, QSEs representing PPAs may, at any time, submit data from a Resource as verifiable costs documentation and such documentation will be accepted for consideration by ERCOT.  A QSE submitting verifiable costs documentation pursuant to this paragraph shall not be required to submit a PPA to ERCOT for consideration for verifiable cost recovery.

(9)	ERCOT shall notify a QSE to update verifiable cost data of a Resource when the Resource has received more than 50 RUC instructions meeting the criteria in Section 5.6.2, RUC Startup Cost Eligibility, in a year, but ERCOT may not request an update more frequently than annually.

(10)	ERCOT shall notify a QSE to update verifiable cost data of a Resource if at least five years have passed since ERCOT previously approved verifiable cost data for that Resource.

(11)	Within 30 days after receiving an update Notice from ERCOT under either paragraph (9) or (10) above, a QSE or Resource Entity must submit verifiable cost data for the Resource.  Despite the provisions in paragraph (2) above, if the QSE or Resource Entity does not submit verifiable cost data within 30 days after receiving an update Notice, then, for all Operating Days until updated verifiable costs are approved, ERCOT shall determine payment using the lower of:

(a)	Resource Category Startup Generic and Resource Category Minimum-Energy Generic Caps; and

(b)	Current ERCOT-approved verifiable startup and minimum-energy costs.

(12)	Resource Entities that represent Reliability Must-Run (RMR) Resources shall submit to ERCOT, Startup and variable O&M Cost estimates to be used by ERCOT as proxies for verifiable Startup Cost and minimum-energy verifiable cost and for Settlement.  The ERCOT-approved verifiable Startup Cost estimate will equal the startup fuel estimate times the sum of the appropriate Index Fuel Price (IFP) or Fuel Oil Price (FOP) and the fuel adder, plus the startup O&M.  The ERCOT-approved minimum-energy verifiable cost estimate will equal the heat rate from the RMR Agreement contract times the sum of the appropriate IFPor FOP and the fuel adder, plus the variable O&M.  The O&M cost estimates shall be revised monthly to be consistent with the latest actual costs for the RMR Unit submitted in accordance with Section 3.14.1.12, Reporting Actual Eligible Cost.  The O&M values will be effective until updated costs have been submitted to ERCOT.

5.7.1.1	RUC Guarantee

(1)	The allowable Startup Costs and minimum-energy costs of a Resource committed by RUC is the RUC Guarantee.  The RUC Guarantee minimum-energy costs are prorated according to the actual generation when the Resource’s average output during a 15-minute Settlement Interval is below the corresponding LSL.

(2)	The SUPR, MEPR and LSL used to calculate the RUC Guarantee for a Combined Cycle Train are the SUPR, MEPR and LSL that correspond to the Combined Cycle Generation Resource, within the Combined Cycle Train, that is RUC-committed for the hour.

(3)	For an Aggregate Generation Resource (AGR), the Startup Cost shall be scaled according to the maximum number of its generators online during a contiguous block of RUC-committed intervals, as indicated by telemetry, compared to the total number of generators registered to the AGR and used in the approved verifiable cost for the AGR.

(4)	The RUC Guarantee is calculated for non-Combined Cycle Trains as follows:

RUCGq,r,d	=	 Σs(SUPRq,r,s * RUCSUFLAGq,r,s) + Σi(MEPRq,r,i * Min ((LSLq,r,i * (¼)), RTMGq,r,i))

(5)	The RUC Guarantee is calculated for Combined Cycle Trains as follows:

RUCGq,r,d	=	(SUPRq,r,s * RUCSUFLAGq,r,s) +

Σt(MAX (0, SUPR - SUPR)) +

Σi(MEPRq r,i * Min ((LSLq,r,i * (¼)), RTMGq,r i))

(a)	If a Combined Cycle Train transitions to a RUC-committed configuration from a QSE-committed or other RUC-committed configuration, the transition is calculated as follows:

MAX (0, SUPRafterCCGR – SUPRbeforeCCGR)

(b)	If a Combined Cycle Train transitions to a QSE-committed configuration from a RUC-committed configuration, the transition is calculated as follows:

MAX (0, SUPRbeforeCCGR – SUPRafterCCGR)

(6)	If a validated Three-Part Supply Offer has been submitted for a Resource for the RUC, then the RUC Guarantee for that Resource is based on the Startup Offer and Minimum-Energy Offer in that validated Three-Part Supply Offer.  If a validated Three-Part Supply Offer has not been submitted for a Resource for the RUC and ERCOT has not yet approved verifiable unit-specific costs for the Resource, then the RUC Guarantee for a Resource is based on the Resource Category Startup Generic Cap and the Resource Category Minimum-Energy Generic Cap.  If a validated Three-Part Supply Offer has not been submitted for a Resource for the RUC and ERCOT has approved verifiable unit-specific costs for the Resource, then the RUC Guarantee for a Resource is based on the most recent ERCOT-approved verifiable unit-specific costs for that Resource.

For a Resource which is not an AGR,

If the QSE submitted a validated Three-Part Supply Offer,

Then, 		SUPRq,r,s	=	SUOq,r,s

MEPRq,r,i	=	MEOq,r,i

Otherwise, 	SUPRq,r,s 	= 	SUCAPq,r,s

MEPRq,r,i 	= 	MECAPq,r,i

If ERCOT has approved verifiable Startup Costs and minimum-energy costs for the Resource,

Then, 		SUCAP q, r, s	=	verifiable Startup Costs q, r, s

MECAPq,r,i	=	verifiable minimum-energy costs q, r, i

Otherwise, 	SUCAP q, r, s 	= 	RCGSC s

MECAP q, r, i	= 	RCGMEC i

For AGRs,

If the QSE submitted a validated Three-Part Supply Offer,

Then, 		SUPR q,r,s	=	Min(SUO q,r,s, SUCAP q,r,s)

MEPR q,r,i	=	MEO q,r,i

Otherwise, 	SUPR q,r,s 	= 	SUCAP q,r,s

MEPR q,r,i 	= 	MECAP q,r,i

If ERCOT has approved verifiable Startup Costs and minimum-energy costs for the Resource,

Then, 		SUCAP q, r, s	=	Max c (AGRRATIO q, p, r ) * verifiable Startup Costs q, r, s

MECAP q,r,i	=	verifiable minimum-energy costs q, r, i

Where, 		AGRRATIO q,p,r	 =	AGRMAXON q, p, r / AGRTOT q, p, r

Otherwise, 	SUCAP q, r, s 	= 	Max c (AGRRATIO q, p, r) * RCGSC s

MECAP q, r, i	= 	RCGMEC i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCGq,r,d | $ | RUC Guarantee—The sum of eligible Startup Costs and minimum-energy costs for Resource r during all RUC-Committed Hours, for the Operating Day. When one or more Combined Cycle Generation Resources are committed by RUC, guaranteed costs are calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| SUPRq,r,s | $/Start | Startup Price per start—The Settlement price for Resource r for the start s. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| SUOq,r,s | $/Start | Startup Offer per start—Represents an offer for all costs incurred by Generation Resource r in starting up and reaching the Resource’s LSL, minus the average energy produced during the time period between breaker close and LSL multiplied by the heat rate proxy multiplied by the appropriate Index Fuel Price (IFP) or Fuel Oil Price (FOP), as described in the Verifiable Cost Manual. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| SUCAPq,r,s | $/Start | Startup Cap—The amount used for Resource r as Startup Costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGSC unless ERCOT has approved verifiable unit-specific Startup Costs for that Resource, in which case the startup cap is the verifiable unit-specific Startup Cost. See Section 5.6.1, Verifiable Costs, for more information on verifiable costs. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| AGRRATIO q, p, r | none | Aggregate Generation Resource Ratio per QSE per Settlement Point per Aggregate Generation Resource—A value which represents the ratio of the maximum number of generators online during an hour, as indicated by telemetry, compared to the total number of generators registered to the AGR and used in the approved verifiable cost for the AGR. The value is only applicable if the Resource is an AGR. |
| AGRMAXON q, p, r | none | Aggregate Generation Resource Maximum Online per QSE per Settlement Point per Aggregate Generation Resource—The maximum number of generators online during an hour, as indicated by telemetry. The value is only applicable if the Resource is an AGR. |
| AGRTOT q, p, r | none | Aggregate Generation Resource Total per QSE per Settlement Point per Aggregate Generation Resource—The total number of generators registered to the AGR and used in the approved verifiable cost for the AGR. The value is only applicable if the Resource is an AGR. |
| RCGSCs | $/Start | Resource Category Generic Startup Cost—The Resource Category Generic Startup Cost cap for the category of the Resource, according to Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. |
| RUCSUFLAGq,r,s | none | RUC Startup Flag—The flag that indicates whether or not the start s for Resource r is eligible for RUC Make-Whole Payment. Its value is one if eligible; otherwise, zero. See Section 5.6.2, RUC Startup Cost Eligibility, and Section 5.6.3, Forced Outage of RUC-Committed Resource, for more information on startup eligibility. For a Combined Cycle Train, the Resource r must be one of the registered Combined Cycle Generation Resources within the Combined Cycle Train. When one or more Combined Cycle Generation Resources are committed by RUC, the RUC Startup Flag is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| MEPRq,r,i | $/MWh | Minimum-Energy Price—The Settlement price for Resource r for minimum energy for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MEOq,r,i | $/MWh | Minimum-Energy Offer—Represents an offer for the costs incurred by Resource r in producing energy at the Resource’s LSL for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MECAPq,r,i | $/MWh | Minimum-Energy Cap—The amount used for Resource r for minimum-energy costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGMEC unless ERCOT has approved verifiable unit-specific minimum energy costs for that Resource, in which case the Minimum-Energy Cap is the verifiable unit-specific minimum energy cost. See Section 5.6.1 for more information on verifiable costs. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RCGMECi | $/MWh | Resource Category Generic Minimum-Energy Cost—The Resource Category Generic Minimum Energy Cost cap for the category of the Resource, according to Section 4.4.9.2.3, for the Operating Day. |
| RTMGq,r,i | MWh | Real-Time Metered Generation—The Resource r’s metered generation for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| LSLq,r,i | MW | Low Sustained Limit—The LSL of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the Current Operating Plan (COP). Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | A RUC-committed Generation Resource. |
| d | none | An Operating Day containing the RUC-commitment. |
| i | none | A 15-minute Settlement Interval within the hour that includes a RUC-commitment. |
| s | none | A start that is eligible to have its costs included in the RUC Guarantee. |
| t | none | A transition that is eligible to have its costs included in the RUC Guarantee. |
| c | none | A contiguous block of RUC–Committed Hours. |
| afterCCGR | none | The Combined Cycle Generation Resource to which a Combined Cycle Train transitions. |
| beforeCCGR | none | The Combined Cycle Generation Resource from which a Combined Cycle Train transitions. |

5.7.3	Payment When ERCOT Decommits a QSE-Committed Resource

(1)	If ERCOT decommits a QSE-committed Resource during the RUC process earlier than its scheduled shutdown within the Operating Day, then no compensation is due to the affected QSE from ERCOT.

(2)	If ERCOT decommits a QSE committed Resource that is not scheduled to shutdown within the Operating Day, then ERCOT shall pay the affected QSE an amount as calculated below for the hours of decommitment.  The number of continuous decommitted hours used in the calculation are the hours beginning with the first decommitted hour until the earlier of:

(a)	The hour ERCOT determines that the Resource may again be at LSL; and

(b)	The end of the last hour of the Operating Day.

(3)	If ERCOT decommits a QSE-committed Resource not scheduled to shutdown within the Operating Day, and the decommitment period spans more than one Operating Day, the RUC Decommitment Payment Amount shall be calculated and paid in the Operating Day in which the RUC decommitment originated.  The number of continuous decommitted hours used in the calculation are the hours beginning with the first decommitted hour until the end of the last hour of the Operating Day in which the RUC decommitment originated.

(4)	The payment for a RUC Cancellation instruction for a Resource is settled for each hour through an adjustment in the RUC Decommitment Payment Amount as shown in paragraph (8) below.

(5)	ERCOT shall produce a report each April that provides the percentage of the RUC Decommitment Payment Amounts that are a result of RUC cancellations during the 12 months of the previous calendar year.  The report shall be based on the Final Settlements.  ERCOT shall present the results of this study to the appropriate Technical Advisory Committee (TAC) subcommittee.

(6)	The SUPR, MEPR and LSL used to calculate payment when ERCOT decommits a QSE-committed Combined Cycle Train is the SUPR, MEPR and LSL that corresponds to the Combined Cycle Generation Resource, within the Combined Cycle Train, that is RUC-decommitted in the first hour of a contiguous decommitted period.

(7)	If the SUPR used to calculate payment when ERCOT decommits a QSE-committed Aggregate Generation Resource (AGR) is based upon approved Verifiable Cost for all of the generators associated with the AGR, ERCOT shall scale the startup payment according to the number of generators of the AGR that started following the decommitment.  ERCOT shall make the adjustment no later than on Final Settlement.

(8)	The payment for a RUC decommitment instruction for a Resource, including RMR Units, is calculated for each hour as follows:

RUCDCAMTq,r,h	=	(-1) * Max (0, (SUPRq,r,s - Σi(Max (0, MEPRq,r,i - RTSPPp,i) * (LSLq,r,i * (¼))))) / NCDCHRq,r,h

Where:

If the QSE submitted a validated Three-Part Supply Offer for the Resource,

Then, 		SUPRq,r,s	=	SUOq,r,s

MEPRq,r,i	=	MEOq,r,i

Otherwise, 	SUPRq,r,s 	= 	SUCAPq,r,s

MEPRq,r,i 	= 	MECAPq,r,i

If ERCOT has approved verifiable Startup Costs and minimum-energy costs for the Resource,

Then, 		SUCAPq,r,s	=	verifiable Startup Costsq,r,s

MECAPq,r,i	=	verifiable minimum-energy costsq,r,i

Otherwise, 	SUCAPq,r,s 	= 	RCGSCs

MECAPq,r,i	= 	RCGMECi

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCDCAMTq,r,h | $ | RUC Decommitment Payment Amount—The payment to the QSE for the Resource that was decommitted by ERCOT but that was not scheduled to shut down in the Operating Day, for each decommited hour of the Operating Day. When one or more Combined Cycle Generation Resources are decommitted by RUC, payment is made to the Combined Cycle Train for all RUC-decommitted Combined Cycle Generation Resources. |
| SUPRq,r,s | $/Start | Startup Price per start—The Settlement price for Resource r for the start s. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| SUOq,r,s | $/Start | Startup Offer per start—Represents an offer for all costs incurred by Generation Resource r in starting up and reaching the Resource’s LSL, minus the average energy produced during the time period between breaker close and LSL multiplied by the heat rate proxy multiplied by the appropriate IFP or FOP, as described in the Verifiable Cost Manual. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| SUCAPq,r,s | $/Start | Startup Cap—The amount used for Resource r as Startup Costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGSC unless ERCOT has approved verifiable unit-specific Startup Costs for that Resource, in which case the Startup Cap is the verifiable unit-specific Startup Cost. See Section 5.6.1, Verifiable Costs, for more information on verifiable costs. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RCGSCs | $/Start | Resource Category Generic Startup Cost—The Resource Category Startup Offer Generic Cap cost for the category of the Resource, according to Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. |
| MEPRq,r,i | $/MWh | Minimum-Energy Price—The Settlement price for Resource r for minimum energy for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MEOq,r,i | $/MWh | Minimum-Energy Offer—Represents an offer for the costs incurred by Resource r in producing energy at the Resource’s LSL for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MECAPq,r,i | $/MWh | Minimum-Energy Cap—The amount used for Resource r for minimum-energy costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGMEC unless ERCOT has approved verifiable unit-specific minimum energy costs for that Resource, in which case the Minimum-Energy Cap is the verifiable unit-specific minimum energy cost. See Section 5.6.1 for more information on verifiable costs. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RCGMECi | $/MWh | Resource Category Generic Minimum-Energy Cost—The Resource Category Minimum-Energy Generic Cap cost for the category of the Resource, according to Section 4.4.9.2.3. |
| LSLq,r,i | MW | Low Sustained Limit—The LSL of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the COP. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTSPPp,i | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource’s Settlement Point for the Settlement Interval i. |
| NCDCHRq,r,h | none | Number of Continuous Decommitted Hours—The number of continuous decommitment hours for Resource r within an Operating Day. When one or more Combined Cycle Generation Resources are decommitted by RUC, the Number of Continuous Decommitted Hours is calculated for the Combined Cycle Train for all RUC-decommitted Combined Cycle Generation Resources. |
| q | none | A QSE. |
| r | none | A RUC-decommitted Generation Resource. |
| h | none | An hour in the RUC decommitment period. |
| p | none | A Resource Node Settlement Point. |
| s | none | A start. |
| i | none | A 15-minute Settlement Interval within the contiguous decommited period. |

6.6.3.7	Real-Time Make-Whole Payment for Exceptional Fuel Cost

(1)	Generation Resources that have approved verifiable costs and received at least one Base Point on their Mitigated Offer Cap during a 15-minute Settlement Interval may be eligible to receive a Real-Time Make-Whole Payment for exceptional fuel costs if the actual price paid for delivered fuel for a specific Resource is greater than the fuel price (e.g. IFP, FOP) used by ERCOT for the Resource plus a fuel adder plus a threshold fuel price, as defined in the Verifiable Cost Manual.  The calculation for Real-Time Make-Whole Payment for exceptional fuel costs shall be determined as follows:

EFCMWAMT q, p, r, i  =  (-1) * EFCPR q, p, r, i  * EFCQTY q, p, r, i

Where:

EFCPR q, p, r, i   =  Max[0, MOCFLAG * (Min(EOCPR q, p, r, i  ,  ADMOCPR q, p, r, i  ) -  RTSPP p, i  )]

MOCFLAG q,p,r,i = 1 or 0

And:,

EFCQTY q, p, r, i  =  Min(AGBP q, r, p,i , RTMG q, r, p, i )

AGBP q, r, p,i  = Σy(BP q, r, p, y * TLMPy / 3600)

| Variable | Unit | Definition |
| --- | --- | --- |
| EFCMWAMT q, p, r, i | $ | Exceptional Fuel Cost Make-Whole Payment Amount per QSE per Settlement Point per Generation Resource—The exceptional fuel cost Make-Whole Payment to QSE q at Resource Node p as a result of Generation Resource r receiving a Base Point at the Mitigated Offer Cap during a 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| EFCPR q, p, r, i | $/MWh | Exceptional Fuel Cost Base Point Average Price per QSE per Settlement Point per Generation Resource—The average of the energy prices corresponding with the minimum of the Base Point Energy Offer Curve price (EOCPR) or the adjusted mitigated offer curve price (ADMOCPR), minus the maximum of the mitigated offer curve price (MOCPR) or the Real-Time Settlement Point Price (RTSPP) for Resource r at Resource Node p represented by QSE q, for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| EFCQTY q, p, r, i | MWh | Exceptional Fuel Cost Energy per QSE per Settlement Point per Generation Resource—The energy produced by Generation Resource r at Resource Node p represented by QSE q in Real-Time during an exceptional fuel cost event, for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| EOCPR q, p, r, i | $/MWh | Energy Offer Curve Price per QSE per Settlement Interval per Generation Resource—The Energy Offer Curve price for Resource r at Resource Node p represented by QSE q corresponding to the average five-minute clock interval Base Points (AVGBP) over the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| BP q, r, p, y | MW | Base Point per QSE per Settlement Point per Generation Resource—The SCED Base Point y of Resource r at Resource Node p represented by QSE q. For a Combined Cycle Train, the Resource r must be one of the registered Combined Cycle Generation Resources within the Combined Cycle Train. |
| ADMOCPR q, p, r, i | $/MWh | Adjusted Mitigated Offer Cap Price per QSE per Settlement Interval per Generation Resource —The adjusted by actual fuel cost Mitigated Offer Cap (pursuant to Section 4.4.9.4.1, Mitigated Offer Cap) price for the Generation Resource r at Resource Node p represented by QSE q in Real-Time, corresponding to the average five-minute clock interval Base Points (AVGBP) over the 15-minute Settlement Interval i. Notwithstanding the above, the multiplier, pursuant to item (e) of Section 4.4.9.4.1 shall be set to 1 for all capacity factors. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. The value of ADMOCPR shall be set initially to zero until the QSE has filed to ERCOT actual fuel costs. |
| RTSPP p, i | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource Node p for the Settlement Interval i. |
| RTMG q, r, p, i | MWh | Real-Time Metered Generation per QSE per Settlement Point per Generation Resource—The metered generation, of Resource r, at Resource Node p represented by QSE q in Real-Time for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| MOCFLAG q,p,r,i | none | Mitigated Offer Cap Flag per QSE per Settlement Point per Settlement Interval per Generation Resource —The Mitigated Offer Cap flag for the Generation Resource r at Resource Node p represented by QSE q in Real-Time, for the corresponding 15-minute Settlement Interval i. For any 15-minute Settlement Interval that has at least one mitigated SCED Base Point, the MOCFLAG shall have a value of 1 for the Settlement Interval; otherwise, the value shall be 0. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| AGBP q, r, p,i | MWh | Aggregated Base Point—The Generation Resource’s aggregated Base Point, of Resource r, at Resource Node p, represented by QSE q, for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| AVGBP q, r, p, i | MW | Average Base Point per QSE per Settlement Point per Generation Resource—The average of the five-minute clock interval Base Points (pursuant to Section 6.6.5.1, Resource Base Point Deviation Charge) over the 15-minute Settlement Interval i for Generation Resource r represented by QSE q at Settlement Point p. |
| TLMP y | second | Duration of SCED interval per intervalThe duration of the SCED interval y. |
| q | none | A QSE. |
| r | none | A Generation Resource. |
| p | none | A Resource Node Settlement Point. |
| i | none | A 15-minute Settlement Interval. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| 3600 | second | The number of seconds in one hour. |

(2)	The total compensation to each QSE for Real-Time Make-Whole Payment for exceptional fuel costs for the 15-minute Settlement Interval is calculated as follows:

EMWAMTQSETOT q, i   =  rp EFCMWAMT q, p, r, i

| Variable | Unit | Definition |
| --- | --- | --- |
| EFCMWAMT q, p, r, i | $ | Exceptional Fuel Cost Make-Whole Payment Amount per QSE per Settlement Point per Generation Resource—The exceptional fuel cost Make-Whole Payment to QSE q at Resource Node p as a result of Generation Resource r receiving a Base Point at the Mitigated Offer Cap during a 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| EMWAMTQSETOT q, i | $ | Exceptional Fuel Cost Make-Whole Payment Amount per QSE —The total payment to QSE q for Real-Time exceptional fuel cost Make-Whole Payment during a 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | A Generation Resource. |
| i | none | A 15-minute Settlement Interval. |

- 6.6.3.8	Real-Time Make-Whole Payment for Exceptional Fuel Cost Charge

ERCOT shall charge each QSE representing Load Serving Entities (LSEs) the total payment for Exceptional Fuel costs as specified in Section 6.6.3.7, Real-Time Make-Whole Payment for Exceptional Fuel Costs, based on LRS. The charge to each QSE for a given 15-minute Settlement Interval is calculated as follows:

LAEFCAMT q, i  	=	(-1) * EMWAMTTOT i * LRS q, i

Where:

EMWAMTTOT i  	=	Σq EMWAMTQSETOT q, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LAEFCAMT q, i | $ | Exceptional Fuel Cost Make-Whole Payment Charge per QSE—The charge to QSE q for Real-Time exceptional fuel cost Make-Whole Payments, for the 15-minute Settlement Interval i. |
| EMWAMTTOT i | $ | Exceptional Fuel Cost Make-Whole Payment Amount —The total payment to all QSEs for Real-Time exceptional fuel cost Make-Whole Payment during a 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| EMWAMTQSETOT q, i | $ | Exceptional Fuel Cost Make-Whole Payment Amount per QSE —The total payment to QSE q for Real-Time exceptional fuel cost Make-Whole Payment during a 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| LRS q, i | none | The Load Ratio Share calculated for QSE q for the 15-minute Settlement Interval i. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. |
| q | none | A QSE. |
| i | none | A 15-minute Settlement Interval. |

6.6.6.2	RMR Payment for Energy

(1)	Payment for energy on the Initial Settlement and settlements executed before true-up and before actual cost data is submitted must be calculated using the estimated input/output curve and startup fuel as specified in the RMR Agreement, the actual energy produced and the IFP.  The payment for energy for all other settlements must be based on actual fuel costs for the RMR Unit.  The payment for energy for each hour is calculated as follows:

RMREAMT q, r	=	(-1) * ((IFP + RMRCEFA q, r) * RMRSUFQ q, r / RMRH q, r) * RMRALLOCFLAG q, r + Σ4i=1(((IFP + RMRCEFA q, r) * RMRHR q, r, i + RMRVCC q, r) * RTMG q, r, i)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMREAMT q, r | $ | Reliability Must-Run Energy Amount per QSE per Resource by hour—The energy payment to QSE q for RMR Unit r, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| IFP | $/MMBtu | Index Fuel Price—The IFP for the Operating Day. |
| RMRSUFQ q, r | MMBtu | Reliability Must-Run Startup Fuel Quantity per QSE per ResourceThe Estimated Start Up Fuel specified in the RMR Agreement for RMR Unit r represented by QSE q. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRH q, r,h | hour | Reliability Must-Run Hours—The number of hours during which RMR Unit r represented by QSE q is instructed On-Line for the Operating Day. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRALLOCFLAG q, r | none | Reliability Must-Run Startup Flag per QSE per Resource by hour—The number that indicates whether or not the startup fuel cost of RMR Unit r represented by QSE q is allocated to the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. The startup fuel cost will be allocated equally to all contiguous intervals for which there is an eligible start. The RMRALLOCFLAG q, r value is 1 if the startup fuel cost is allocated; otherwise, its value is 0. The RMRALLOCFLAG q, r for eligibility is determined in Sections 5.6.2, RUC Startup Cost Eligibility, and 5.6.3, Forced Outage of a RUC-Committed Resource, for start-up payments and commitments in either the Reliability Unit Commitment (RUC) or DAM. |
| RMRHR q, r, i | MMBtu /MWh | Reliability Must-Run Heat Rate per QSE per Resource by Settlement Interval by hour—The multiplier determined based on the input/output curve and the Real-Time generation of RMR Unit r represented by QSE q, for the 15-minute Settlement Interval i in the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRVCC q, r | $/MWh | Reliability Must-Run Variable Cost Component per QSE per Resource—The monthly cost component that is used to adjust the energy cost calculation to reflect the actual fuel costs of RMR Unit r represented by QSE q. The value is initially set to zero. For resettlements, see item (2) below. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTMG q, r, i, | MWh | Real-Time Metered Generation per QSE per Resource by Settlement Interval by hour—The Real-Time energy from RMR Unit r represented by QSE q, for the 15-minute Settlement Interval i in the hour h. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRCEFA q, r | $/MMBtu | Reliability Must-Run Contractual Estimated Fuel Adder—The Estimated Fuel Adder that is contractually agreed upon in Section 22, Attachment B, Standard Form Reliability Must-Run Agreement. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. The fuel adder will be subsequently trued up to reflect actual fuel costs as set forth in item (1) above. |
| q | none | A QSE. |
| r | none | An RMR Unit. |
| i | none | A 15-minute Settlement Interval. |

(2)	If the RMR actual fuel cost is filed in accordance with the timeline in these Protocols, the monthly RMR variable cost component is calculated for the subsequent resettlements as follows:

RMRVCC q, r	=	(RMRMFCOST q, r + ΣhRMREAMT q, r, f, h) /

(ΣiRTMG q, r, i)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMRVCC q, r | $/MWh | Reliability Must-Run Variable Cost Component per QSE per Resource—The monthly cost component that is used to adjust the energy cost calculation to reflect the actual fuel costs of RMR Unit r represented by QSE q. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRMFCOST q, r | $ | Reliability Must-Run Monthly actual Fuel Cost per QSE per Resource—The monthly actual fuel cost of RMR Unit r represented by QSE q, for the month. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTMG q, r, i | MWh | Real-Time Metered Generation per QSE per Resource by Settlement Interval—The Real-Time energy from RMR Unit r represented by QSE q for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMREAMT q, r, f, h | $ | Reliability Must-Run Energy Amount per QSE per Resource by hour—The energy payment to QSE q for RMR Unit r, for the hour h, from the former Settlement Statement f. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | An RMR Unit. |
| h | none | An hour in the month. |
| i | none | A 15-minute Settlement Interval in the month. |
| f | none | Amount from former settlement run. |

(3)	The total of the payments for energy to each QSE for all RMR Units represented by this QSE for a given hour is calculated as follows:

RMREAMTQSETOT q	=	ΣrRMREAMT q, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMREAMTQSETOT q | $ | Reliability Must-Run Energy Amount QSE Total per QSEThe total of the energy payments to QSE q for all RMR Units represented by this QSE for the hour. |
| RMREAMT q, r | $ | Reliability Must-Run Energy Amount per QSE per Resource by hour—The energy payment to QSE q for RMR Unit r, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | An RMR Unit. |

7.9.1.3	Minimum and Maximum Resource Prices

(1)	For purposes of Section 7.9.1, Day-Ahead CRR Payments and Charges, Settlements data published to the MIS Secure Area shall include the association of the Resource Category for each Generation Resource.  The following prices specified in paragraphs (2) and (3) below are used in the CRR hedge value calculation for CRRs settled in the DAM.

(2)	Minimum Resource Prices of source Settlement Points are:

MINRESPR j	 =	Min ( MINRESRPR j, r ) r

Where:

Minimum Resource Prices for Resources located at source Settlement Points (MINRESRPR j, r) are:

(a)	Nuclear = -$20.00/MWh;

(b)	Hydro = -$20.00/MWh;

(c)	Coal and Lignite = $0.00/MWh;

(d)	Combined Cycle greater than 90 MW = Index Fuel Price (IFP) * 5 MMBtu/MWh;

(e)	Combined Cycle less than or equal to 90 MW = IFP * 6 MMBtu/MWh;

(f)	Gas -Steam Supercritical Boiler = IFP * 6.5 MMBtu/MWh;

(g)	Gas Steam Reheat Boiler = IFP * 7.5 MMBtu/MWh;

(h)	Gas Steam Non-Reheat or Boiler without Air-Preheater = IFP * 10.5 MMBtu/MWh;

(i)	Simple Cycle greater than 90 MW = IFP * 10 MMBtu/MWh;

(j)	Simple Cycle less than or equal to 90 MW = IFP * 11 MMBtu/MWh;

(k)	Diesel = IFP * 12 MMBtu/MWh;

(l)	Wind = -$35/MWh;

[NPRR588: Insert paragraph (m) below upon system implementation and renumber accordingly:] (m) PhotoVoltaic (PV) = -$10;

(m)	Reliability Must-Run (RMR) Resource = RMR contract price Energy Offer Curve at Low Sustained Limit (LSL); and

(n)	Other Renewable = -$10.

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| MINRESPR j | $/MWh | Minimum Resource Price for source—The lowest Minimum Resource Price for the Resources located at the source Settlement Point j. |
| MINRESRPR j | $/MWh | Minimum Resource Price for Resource—The Minimum Resource Price for the Resources located at the source Settlement Point j. |
| r | none | A Generation Resource located at the source Settlement Point j. |
| j | none | A source Settlement Point. |

(3)	Maximum Resource Prices of sink Settlement Points are:

MAXRESPR k	 =	Max (MAXRESRPR k, r ) r

Where:

Maximum Resource Prices for Resources located at sink Settlement Points (MAXRESRPR k, r ) are:

(a)	Nuclear = $15.00/MWh;

(b)	Hydro = $10.00/MWh;

(c)	Coal and Lignite = $18.00/MWh;

(d)	Combined Cycle greater than 90 MW = IFP * 9 MMBtu/MWh;

(e)	Combined Cycle less than or equal to 90 MW = IFP * 10 MMBtu/MWh;

(f)	Gas -Steam Supercritical Boiler = IFP * 10.5 MMBtu/MWh;

(g)	Gas Steam Reheat Boiler = IFP * 11.5 MMBtu/MWh;

(h)	Gas Steam Non-Reheat or Boiler without Air-Preheater = IFP * 14.5 MMBtu/MWh;

(i)	Simple Cycle greater than 90 MW = IFP * 14 MMBtu/MWh;

(j)	Simple Cycle less than or equal to 90 MW = IFP * 15 MMBtu/MWh;

(k)	Diesel = IFP * 16 MMBtu/MWh;

(l)	Wind = $0/MWh;

[NPRR588: Insert paragraph (m) below upon system implementation and renumber accordingly:] (m) PV = $0/MWh;

(m)	RMR Resource = RMR contract price Energy Offer Curve at HSL; and

(n)	Other Renewable = $0.

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| MAXRESPR k | $/MWh | Maximum Resource Price for source—The highest Maximum Resource Price for the Resources located at the sink Settlement Point k. |
| MAXRESRPR k | $/MWh | Maximum Resource Price for Resource—The Maximum Resource Price for the Resources located at the sink Settlement Point k. |
| r | none | A Generation Resource located at the sink Settlement Point k. |
| k | none | A sink Settlement Point. |

- 9.5.3	Real-Time Market Settlement Charge Types

(1)	ERCOT shall provide, on each RTM Settlement Statement, the dollar amount for each RTM Settlement charge and payment.  The RTM Settlement “Charge Types” are:

(a)	Section 5.7.1, RUC Make-Whole Payment;

(b)	Section 5.7.2, RUC Clawback Charge;

(c)	Section 5.7.3, Payment When ERCOT Decommits a QSE-Committed Resource;

(d)	Section 5.7.4.1, RUC Capacity-Short Charge;

(e)	Section 5.7.4.2, RUC Make-Whole Uplift Charge;

(f)	Section 5.7.5, RUC Clawback Payment;

(g)	Section 5.7.6, RUC Decommitment Charge;

(h)	Section 6.6.3.1, Real-Time Energy Imbalance Payment or Charge at a Resource Node;

(i)	Section 6.6.3.2, Real-Time Energy Imbalance Payment or Charge at a Load Zone;

(j)	Section 6.6.3.3, Real-Time Energy Imbalance Payment or Charge at a Hub;

(k)	Section 6.6.3.4, Real-Time Energy Payment for DC Tie Import;

(l)	Section 6.6.3.5, Real-Time Payment for a Block Load Transfer Point;

(m)	Section 6.6.3.6, Real-Time Energy Charge for DC Tie Export Represented by the QSE Under the Oklaunion Exemption;

(n)	Section 6.6.3.7, Real-Time Make-Whole Payment for Exceptional Fuel Cost;

(o)	Section 6.6.3.8, Real-Time Make-Whole Payment for Exceptional Fuel Charge;

(p)	Section 6.6.4, Real-Time Congestion Payment or Charge for Self-Schedules;

(q)	Section 6.6.5.1.1.1, Base Point Deviation Charge for Over Generation;

(r)	Section 6.6.5.1.1.2, Base Point Deviation Charge for Under Generation;

(s)	Section 6.6.5.2, IRR Generation Resource Base Point Deviation Charge;

(t)	Section 6.6.5.4, Base Point Deviation Payment;

(u)	Section 6.6.6.1, RMR Standby Payment;

(v)	Section 6.6.6.2, RMR Payment for Energy;

(w)	Section 6.6.6.3, RMR Adjustment Charge;

(x)	Section 6.6.6.4, RMR Charge for Unexcused Misconduct;

(y)	Section 6.6.6.5, RMR Service Charge;

(z)	Paragraph (2) of Section 6.6.7.1, Voltage Support Service Payments;

(aa)	Paragraph (4) of Section 6.6.7.1;

(bb)	Section 6.6.7.2, Voltage Support Charge;

(cc)	Section 6.6.8.1, Black Start Hourly Standby Fee Payment;

(dd)	Section 6.6.8.2, Black Start Capacity Charge;

(ee)	Section 6.6.9.1, Payment for Emergency Power Increase Directed by ERCOT;

(ff)	Section 6.6.9.2, Charge for Emergency Power Increases;

(gg)	Section 6.6.10, Real-Time Revenue Neutrality Allocation;

(hh)	Paragraph (a) of Section 6.7.1, Payments for Ancillary Service Capacity Sold in a Supplemental Ancillary Service Market;

(ii)	Paragraph (b) of Section 6.7.1;

(jj)	Paragraph (c) of Section 6.7.1;

(kk)	Paragraph (d) of Section 6.7.1;

(ll)	Paragraph (a) of Section 6.7.2, Charges for Ancillary Service Capacity Replaced Due to Failure to Provide;

(mm)	Paragraph (b) of Section 6.7.2;

(nn)	Paragraph (c) of Section 6.7.2;

(oo)	Paragraph (d) of Section 6.7.2;

(pp)	Paragraph (2) of Section 6.7.3, Adjustments to Cost Allocations for Ancillary Services Procurement;

(qq)	Paragraph (3) of Section 6.7.3;

(rr)	Paragraph (4) of Section 6.7.3;

(ss)	Paragraph (5) of Section 6.7.3;

(tt)	Paragraph (6) of Section 6.7.4, Real-Time Ancillary Service Imbalance Payment or Charge;

(uu)	Paragraph (7) of Section 6.7.4;

(vv) 	Section 6.7.5, Real Time Ancillary Service Imbalance Revenue Neutrality Allocation;

(ww)	Section 7.9.2.1, Payments and Charges for PTP Obligations Settled in Real-Time;

(xx)	Paragraph (3) of Section 7.9.3.3, Shortfall Charges to CRR Owners; and

(yy)	Section 9.16.1, ERCOT System Administration Fee.

[NPRR626: Replace paragraph (1) above with the following upon system implementation:] (1) ERCOT shall provide, on each RTM Settlement Statement, the dollar amount for each RTM Settlement charge and payment. The RTM Settlement “Charge Types” are: (a) Section 5.7.1, RUC Make-Whole Payment; (b) Section 5.7.2, RUC Clawback Charge; (c) Section 5.7.3, Payment When ERCOT Decommits a QSE-Committed Resource; (d) Section 5.7.4.1, RUC Capacity-Short Charge; (e) Section 5.7.4.2, RUC Make-Whole Uplift Charge; (f) Section 5.7.5, RUC Clawback Payment; (g) Section 5.7.6, RUC Decommitment Charge; (h) Section 6.6.3.1, Real-Time Energy Imbalance Payment or Charge at a Resource Node; (i) Section 6.6.3.2, Real-Time Energy Imbalance Payment or Charge at a Load Zone; (j) Section 6.6.3.3, Real-Time Energy Imbalance Payment or Charge at a Hub; (k) Section 6.6.3.4, Real-Time Energy Payment for DC Tie Import; (l) Section 6.6.3.5, Real-Time Payment for a Block Load Transfer Point; (m) Section 6.6.3.6, Real-Time Energy Charge for DC Tie Export Represented by the QSE Under the Oklaunion Exemption; (n) Section 6.6.3.7, Real-Time Make-Whole Payment for Exceptional Fuel Cost; (o) Section 6.6.3.8, Real-Time Make-Whole Payment for Exceptional Fuel Charge; (p) Section 6.6.4, Real-Time Congestion Payment or Charge for Self-Schedules; (q) Section 6.6.5.1.1.1, Base Point Deviation Charge for Over Generation; (r) Section 6.6.5.1.1.2, Base Point Deviation Charge for Under Generation; (s) Section 6.6.5.2, IRR Generation Resource Base Point Deviation Charge; (t) Section 6.6.5.4, Base Point Deviation Payment; (u) Section 6.6.6.1, RMR Standby Payment; (v) Section 6.6.6.2, RMR Payment for Energy; (w) Section 6.6.6.3, RMR Adjustment Charge; (x) Section 6.6.6.4, RMR Charge for Unexcused Misconduct; (y) Section 6.6.6.5, RMR Service Charge; (z) Paragraph (2) of Section 6.6.7.1, Voltage Support Service Payments; (aa) Paragraph (4) of Section 6.6.7.1; (bb) Section 6.6.7.2, Voltage Support Charge; (cc) Section 6.6.8.1, Black Start Hourly Standby Fee Payment; (dd) Section 6.6.8.2, Black Start Capacity Charge; (ee) Section 6.6.9.1, Payment for Emergency Power Increase Directed by ERCOT; (ff) Section 6.6.9.2, Charge for Emergency Power Increases; (gg) Section 6.6.10, Real-Time Revenue Neutrality Allocation; (hh) Paragraph (a) of Section 6.7.1, Payments for Ancillary Service Capacity Sold in a Supplemental Ancillary Service Market; (ii) Paragraph (b) of Section 6.7.1; (jj) Paragraph (c) of Section 6.7.1; (kk) Paragraph (d) of Section 6.7.1; (ll) Paragraph (a) of Section 6.7.2, Charges for Ancillary Service Capacity Replaced Due to Failure to Provide; (mm) Paragraph (b) of Section 6.7.2; (nn) Paragraph (c) of Section 6.7.2; (oo) Paragraph (d) of Section 6.7.2; (pp) Paragraph (2) of Section 6.7.3, Adjustments to Cost Allocations for Ancillary Services Procurement; (qq) Paragraph (3) of Section 6.7.3; (rr) Paragraph (4) of Section 6.7.3; (ss) Paragraph (5) of Section 6.7.3; (tt) Paragraph (6) of Section 6.7.4, Real-Time Ancillary Service Imbalance Payment or Charge (Real-Time Ancillary Service Imbalance Amount); (uu) Paragraph (6) of Section 6.7.4, (Real-Time Reliability Deployment Ancillary Service Imbalance Amount); (vv) Paragraph (7) of Section 6.7.4; (ww) Paragraph (7) of Section 6.7.4, (Real-Time RUC Ancillary Service Reserve Amount); (xx) Paragraph (7) of Section 6.7.4, (Real-Time Reliability Deployment RUC Ancillary Service Reserve Amount); (yy) Section 6.7.5, Real Time Ancillary Service Imbalance Revenue Neutrality Allocation (Load-Allocated Ancillary Service Imbalance Revenue Neutrality Amount); (zz) Section 6.7.5, (Load-Allocated Reliability Deployment Ancillary Service Imbalance Revenue Neutrality Amount); (aaa) Section 7.9.2.1, Payments and Charges for PTP Obligations Settled in Real-Time; (bbb) Paragraph (3) of Section 7.9.3.3, Shortfall Charges to CRR Owners; and (ccc) Section 9.16.1, ERCOT System Administration Fee.

(2)	In the event that ERCOT is unable to execute the Day-Ahead Market (DAM), ERCOT shall provide, on each RTM Settlement Statement, the dollar amount for the following RTM Congestion Revenue Right (CRR) Settlement charges and payments:

(a)	Section 7.9.2.4, Payments for FGRs in Real-Time; and

(b)	Section 7.9.2.5, Payments and Charges for PTP Obligations with Refund in Real-Time.

9.14.7	Disputes for RUC Make-Whole Payment for Exceptional Fuel Costs

(1)	If the actual price paid for delivered natural gas for a specific Resource during a Reliability Unit Commitment (RUC)-Committed Interval is greater than Index Fuel Price (IFP) * 1.X, as defined in the Verifiable Cost Manual, then the QSE may file a Settlement dispute for that Resource’s RUC Make-Whole Payment.  The IFP shall be the fuel price that was used to calculate the Startup and Minimum-Energy Offer Caps. The maximum amount that may be recovered through this dispute process is the difference between the RUC Guarantee based on the actual price paid and a fuel price of IFP * 1.X.  The QSE must provide documentation (invoices) that identifies intra-day costs of natural gas consumed during the RUC-Committed Interval.  Such documentation is necessary to justify recovery of natural gas costs, which is limited to the actual fuel amount (MMBtus) consumed during RUC-Committed Intervals.  All documentation submitted by the QSE for natural gas costs incurred intra-day must show a nexus from the seller or distributor of natural gas products to the QSE, Resource Entity or Generation Entity as the ultimate buyer.  The QSE must demonstrate that the seller or distributor has procured natural gas fuel intra-day.  Power Purchase or Tolling Agreements (PPAs) filed as documentation of proof of fuel costs will not be accepted unless it meets the criteria in paragraph (4) below.

(2)	If the actual price paid for the delivered fuel oil used to replace oil consumed during a RUC-Committed Interval is greater than Fuel Oil Price (FOP), then the QSE may file a Settlement dispute for the Resource’s RUC Make-Whole Payment.  The FOP shall be the fuel price that was used to calculate the Startup and Minimum-Energy Offer Caps, as applicable. The QSE must provide documentation that identifies purchases of fuel oil by the QSE, Resource Entity or Generation Entity to replace oil consumed for a RUC-Committed Interval.  In addition, the QSE must provide proof that the Resource actually consumed fuel oil during the RUC-Committed Interval.  Proof of actual consumption may be based on the Resource’s technical specifications or flow meters as appropriate.  Documentation of fuel oil purchases must show that these were made no later than seven Business Days after the end of the last consecutive RUC-Committed Interval.  Such documentation is necessary to justify recovery of replacement fuel oil costs which is limited to the actual gallons/barrels of fuel oil consumed during RUC-Committed Intervals.

(3)	If the QSE representing the Generation Resource made a Three-Part Supply Offer into the DAM based on IFP and had to run on fuel oil in a RUC-Committed Hour, the QSE may file a Settlement dispute to recover the difference between the RUC Guarantee based actual price paid for delivered fuel oil minus the offer price.

(4)	A QSE submitting documents for the recovery of fuel costs for RUC deployments other than those specifically discussed in paragraph (1) above must either:

(a)	Request to have such documents approved by the ERCOT Board during an Executive Session at the next regularly scheduled meeting of the ERCOT Board.  If the ERCOT Board approves the inclusion of such documentation as proof of fuel purchases, the QSE must file a Nodal Protocol Revision Request (NPRR) in accordance with Section 21, Revision Request Process, to add this category of documentation to the process for approval of RUC Make-Whole Payments; or

(b)	Have incurred the cost of the fuel with a PPA signed prior to July 16, 2008 that is not between Affiliates, subsidiaries, or partners.