| NPRR Number | 679 | NPRR Title | ONOPTOUT for RUC Given After the Adjustment Period |
| --- | --- | --- | --- |
| Date |  | May 5, 2015 |  |
| Submitter’s Information |  |  |  |
| Name |  | Ino Gonzalez |  |
| E-mail Address |  | igonzalez@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 512-248-3954 |  |
| Cell Number |  | 512-632-7927 |  |
| Market Segment |  | Not applicable. |  |

Comments

ERCOT recommends additional changes to the NPRR in order to reduce implementation costs while preserving the objectives described by Tenaska and approved by the Protocol Revision Subcommittee (PRS).  To this end, ERCOT offers the following language changes and clarifications for consideration:

- These comments propose to allow a Qualified Scheduling Entity (QSE) of a Reliability Unit Commitment  (RUC)-committed Resource to opt out of any RUC either by a timely change to COP or, if the instruction (and any ensuing RUC instructions for the same contiguous block of RUC-Committed Hours) came at any time after ninety minutes before the start of, or within the first hour of, a contiguous block of RUC-Committed Hours, by changing the Resource’s telemetered status to ONOPTOUT and submitting a Settlement and billing dispute pursuant to Section 9.14, Settlement and Billing Dispute Process.

- These comments also propose to clarify that a QSE may opt out of a RUC instruction by setting the COP Resource Status to ONOPOUT for any hour within the contiguous block of RUC-Committed Hours, and not just the first hour in that block.

Revised Cover Page Language

None at this time.

Revised Proposed Protocol Language

- 5.5.2	Reliability Unit Commitment (RUC) Process

(1)	The RUC process recommends commitment of Generation Resources, to match ERCOT’s forecasted Load including Direct Current Tie (DC Tie) Schedules, subject to all transmission constraints and Resource performance characteristics.  The RUC process takes into account Resources already committed in the Current Operating Plans (COPs), Resources already committed in previous RUCs, and Resource capacity already committed to provide Ancillary Service.  The formulation of the RUC objective function must employ penalty factors on violations of security constraints. The objective of the RUC process is to minimize costs based on Three-Part Supply Offers, substituting a proxy Energy Offer Curve for the Energy Offer Curve, over the RUC Study Period.

(2)	The RUC process can recommend Resource decommitment.  ERCOT may only decommit a Resource to resolve transmission constraints that are otherwise unresolvable. Qualifying Facilities (QFs) may be decommitted only after all other types of Resources have been assessed for decommitment.  In addition, the HRUC process provides decision support to ERCOT regarding a Resource decommitment requested by a Qualified Scheduling Entity (QSE).

(3)	ERCOT shall review the RUC-recommended Resource commitments to assess feasibility and shall make any changes that it considers necessary, in its sole discretion.  ERCOT may deselect Resources recommended in DRUC and in all HRUC processes if in ERCOT’s sole discretion there is enough time to commit those Resources in the future HRUC processes, taking into account the Resources’ start-up times, to meet ERCOT System reliability.  After each RUC run, ERCOT shall post the amount of capacity deselected per hour in the RUC Study Period to the MIS Secure Area.  A Generation Resource shown as On-Line and available for Security-Constrained Economic Dispatch (SCED) dispatch for an hour in its COP prior to a DRUC or HRUC process execution, according to Section 5.3, ERCOT Security Sequence Responsibilities, will be considered self-committed for that hour.  For purpose of Settlement, snapshot data will be used as specified in paragraph (2) of Section 5.3.  ERCOT shall issue RUC instructions to each QSE specifying its Resources that have been committed as a result of the RUC process.  ERCOT shall, within one day after making any changes to the RUC-recommended commitments, post to the MIS Secure Area any changes that ERCOT made to the RUC-recommended commitments with an explanation of the changes.

(4)	To determine the projected energy output level of each Resource and to project potential congestion patterns for each hour of the RUC, ERCOT shall calculate proxy Energy Offer Curves based on the Mitigated Offer Caps for the type of Resource as specified in Section 4.4.9.4, Mitigated Offer Cap and Mitigated Offer Floor, for use in the RUC.  Proxy Energy Offer Curves are calculated by multiplying the Mitigated Offer Cap by a constant selected by ERCOT from time to time that is no more than 0.10% and applying the cost for all Generation Resource output between High Sustained Limit (HSL) and Low Sustained Limit (LSL).

(5)	ERCOT shall use the RUC process to evaluate the need to commit Resources for which a QSE has submitted Three-Part Supply Offers and other available Off-Line Resources in addition to Resources that are planned to be On-Line during the RUC Study Period.  All of the above commitment information must be as specified in the QSE’s COP.

(6)	ERCOT shall create Three-Part Supply Offers for all Resources that did not submit a Three-Part Supply Offer, but are specified as available but Off-Line, excluding Resources with a Resource Status of EMR, in a QSE’s COP.  For such Resources, ERCOT shall use in the RUC process 150% of any approved verifiable Startup Cost and verifiable minimum-energy cost or if verifiable costs have not been approved, the applicable Resource Category Generic Startup Offer Cost and the applicable Resource Category Generic Minimum-Energy Offer Cost as described specified in Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, registered with ERCOT.  However for Settlement purposes, ERCOT shall use any approved verifiable Startup Costs and verifiable minimum-energy cost for such Resources, or if verifiable costs have not been approved, the applicable Resource Category Generic Startup Offer Cost and Generic Minimum-Energy Offer Cost.

(7)	The RUC process must treat all Resource capacity providing Ancillary Service as unavailable for the RUC Study Period, unless that treatment leads to infeasibility (i.e., that capacity is needed to resolve some local transmission problem that cannot be resolved by any other means).  If an ERCOT Operator decides that the Ancillary Service capacity allocated to that Resource is undeliverable based on ERCOT System conditions, then, ERCOT shall inform each affected QSE of the amount of its Resource capacity that does not qualify to provide Ancillary Service, and the projected hours for which this is the case.  In that event, the affected QSE may, under Section 6.4.9.1.2, Replacement of Undeliverable Ancillary Service Due to Transmission Constraints, either:

(a) 	Substitute capacity from Resources represented by that QSE;

(b)	Substitute capacity from other QSEs using Ancillary Service Trades; or

(c)	Ask ERCOT to replace the capacity.

(8)	Factors included in the RUC process are:

(a)	ERCOT System-wide hourly Load forecast allocated appropriately over Load buses;

(b)	Transmission constraints – Transfer limits on energy flows through the electricity network;

(i)	Thermal constraints – protect transmission facilities against thermal overload;

(ii)	Generic constraints – protect the transmission system against transient instability, dynamic instability or voltage collapse;

(c)	Planned transmission topology;

(d)	Energy sufficiency constraints;

(e)	Inputs from the COP, as appropriate;

(f)	Inputs from Resource Parameters, as appropriate;

(g)	Each Generation Resource’s Minimum-Energy Offer and Startup Offer, from its Three-Part Supply Offer;

(h)	Any Generation Resource that is Off-Line and available but does not have a Three-Part Supply Offer;

(i)	Forced Outage information; and

(j)	Inputs from the eight-day look ahead planning tool, which may potentially keep a unit On-Line (or start a unit for the next day) so that a unit minimum duration between starts does not limit the availability of the unit (for security reasons).

(9)	The HRUC process and the DRUC process are as follows:

(a)	The HRUC process uses current Resource Status for the initial condition for the first hour of the RUC Study Period.  All HRUC processes use the projected status of transmission breakers and switches starting with current status and updated for each remaining hour in the study as indicated in the COP for Resources and in the Outage Scheduler for transmission elements.

(b)	The DRUC process uses the Day-Ahead forecast of total ERCOT Load including DC Tie Schedules for each hour of the Operating Day.  The HRUC process uses the current hourly forecast of total ERCOT Load including DC Tie Schedules for each hour in the RUC Study Period.

(c)	The DRUC process uses the Day-Ahead weather forecast for each hour of the Operating Day.  The HRUC process uses the weather forecast information for each hour of the balance of the RUC Study Period.

(10)	A QSE that has one or more of its Resources RUC-committed to provide Ancillary Services must increase its Ancillary Service Supply Responsibility by the total amount of RUC-committed Ancillary Service quantities.  The QSE may only use a RUC-committed Resource to meet its Ancillary Service Supply Responsibility during that Resource’s RUC-Committed Interval if the Resource has been committed by the RUC process to provide Ancillary Service.  The QSE shall indicate the exact amount and type of Ancillary Service for which it was committed as the Resource’s Ancillary Service Resource Responsibility and Ancillary Services Schedule for the RUC-Committed Intervals for both telemetry and COP information provided to ERCOT.  Upon deployment of the Ancillary Services, the QSE shall adjust its Ancillary Services Schedule to reflect the amounts requested in the deployment.

(11)	A QSE with a Resource that is not a Reliability Must-Run (RMR) Unit that has been committed in a RUC process or by a RUC Verbal Dispatch Instruction (VDI) may opt out of the RUC instruction (or “buy back” the Resource) by self-committing the Resource as described in this paragraph.  A QSE that opts out of the RUC instruction forfeits RUC Settlement for that Resource.  For purposes of Settlement, all hours in the contiguous block of RUC-Committed Hours that includes a RUC Buy-Back Hour shall be considered RUC Buy-Back Hours.  If a RUC instruction is extended, all contiguous RUC-Committed Hours shall be treated as one block.  Notwithstanding the foregoing, if a contiguous block of RUC-Committed Hours spans more than one Operating Day, each set of contiguous RUC-Committed Hours within each Operating Day shall be treated as a separate block and a QSE must opt out of each block independently.  The QSE may self-commit the Resource in one of the following two ways:

(a)	By setting the COP Resource Status to ONOPTOUT for any hour within a contiguous block of RUC-Committed Hours before the start of the first hour of the contiguous block of RUC-Committed Hours; or

(b)	If ERCOT issues a RUC instruction no earlier than 90 minutes before the start of the first hour in a contiguous block of RUC-Committed Hours or within the first hour of a contiguous block of RUC-Committed Hours (intra-hour), by:

(i) 	Setting the telemetered status of the RUC-committed Resource to ONOPTOUT at the start of the first hour of the contiguous block of RUC-Committed Hours or, for any instructions requiring immediate commitment of the Resource, upon receipt of the commitment, and maintaining that status for the duration of the block of contiguous RUC-Committed Hours; and

(ii) 	Submitting a dispute pursuant to Section 9.14, Settlement and Billing Dispute Process, seeking a correction of the RUC Settlement treatment.

(12)	If a QSE-committed Resource experiences a Forced Outage or Startup Loading Failure in an hour for which another Resource under the control of the same QSE is committed by a RUC instruction, the QSE may opt out of the RUC instruction by self-committing the Resource in accordance with paragraph (11) above, or if the Forced Outage or Startup Loading Failure occurs after the start of the first hour within a contiguous block of RUC-Committed Hours, the QSE may opt out of the RUC instruction by:

(a)	Setting the telemetered status of the RUC-committed Resource to ONOPTOUT immediately after the Forced Outage or Startup Loading Failure and maintaining that status for the duration of the block of contiguous RUC-Committed Hours; and

(b)	Submitting a dispute pursuant to Section 9.14 requesting a correction of the RUC Settlement treatment for the RUC-committed Resource.

(13)	If a QSE self-commits a Resource by either of the methods described in paragraphs (11)(b) or (12) above, the QSE must make reasonable efforts to update its COP to ONOPTOUT for all RUC-Committed Hours for which the COP may be adjusted.

(14)	ERCOT shall, as soon as practicable, post to the MIS Secure Area a report identifying those hours that were considered RUC Buy-Back Hours.  The Resources included in the report shall only include those Resources that were self-committed pursuant to paragraph (11)(a) above.