| NPRR Number | 662 | NPRR Title | Proxy Energy Offer Curves |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Recommended Approval |
| Date of Decision |  | May 28, 2015 |  |  |
| Proposed Effective Date |  | Upon system implementation. |  |  |
| Priority and Rank Assigned |  | Priority – 2015; Rank – 1310 |  |  |
| Nodal Protocol Sections Requiring Revision |  | 6.5.7.3, Security Constrained Economic Dispatch |  |  |
| Other Binding Documents Requiring Revision or Related Revision Requests |  | None. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) addresses proxy Energy Offer Curves generated by ERCOT for a Resource that does not submit an Energy Offer Curve or submits an incomplete Energy Offer Curve. For a Resource without an Energy Offer Curve, a monotonically increasing proxy Energy Offer Curve will be created such that the last point on the curve is extended. For a Resource with an incomplete Energy Offer Curve (typically cogeneration facilities that update their HSL, but not their offer curve), a proxy Energy Offer Curve will be created to extend the offer price from last point on the submitted Energy Offer Curve up to the Resource’s HSL. |  |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) The Protocols currently create an offer at the System-Wide Offer Cap (SWCAP) for certain energy without an associated Energy Offer Curve. This has the effect of withholding energy from Security-Constrained Economic Dispatch (SCED) until prices reach the cap, which can cause irregular pricing outcomes and may signal scarcity when scarcity conditions do not exist. At times, the energy subject to this treatment has totaled several hundred MWs. The Independent Market Monitor (IMM) has repeatedly identified creating “proxy offers” at the cap as an issue that needs to be addressed. In the 2012 State of the Market Report, the IMM stated that “Whatever … future market design changes may entail, we recommend stakeholder consideration of the following … Modify the Protocols related to proxy offer curve provisions such that all unoffered capacity is not automnatically priced at the system-wide offer cap.” (2012 SOM Report at xxix-xxx). In the recently released 2013 State of the Market Report, the IMM likewise noted “We continue to recommend modifying the Protocols related to proxy offer curve provisions such that all unofered capacity is not automatically priced at the system-wide offer cap . . . During 2013, the average amount of capacity priced in this manner exceeded 180 MW.” (2013 SOM Report at xxvii). This issue was also discussed in conjunction with NPRR568, Real-Time Reserve Price Adder Based on Operating Reserve Demand Curve, when the Reliability Unit Commitment (RUC) offer floor was reduced to $1,000, and Texas Industrial Energy Consumers (TIEC) committed to submit this NPRR. (Note that the RUC Offer Floor was raised again to $1,500 through the NPRR626, Reliability Deployment Price Adder (formerly “ORDC Price Reversal Mitigation Enhancements”), compromise discussed by the Technical Advisory Committee (TAC) on August 28, 2014.) This concern is heightened with the upcoming SWCAP increase to $9,000. In general, there are very few market generation offers above approximately $1,500, which provides a better proxy for an HSL offer. This is consistent with the recent revisions to the RUC offer floor, which were set at $1,500 as a result of NPRR626 in order to keep RUC energy out of SCED until most market offers are exhausted. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR662 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 10/2/14, NPRR662 was posted. - On 10/7/14, ERCOT comments were posted. - On 11/6/14, PRS considered NPRR662. - On 12/4/14, WMS comments were posted. - On 1/6/15, TIEC comments were posted. - On 2/27/15, a second set of TIEC comments were posted. - On 3/5/15, a second set of WMS comments were posted. - On 3/12/15, PRS again considered NPRR662. - On 4/15/15, an Impact Analysis was posted. - On 4/16/15, PRS considered the 3/12/15 PRS Report and Impact Analysis for NPRR662. - On 5/14/15, PRS considered the 4/16/15 PRS Report and Impact Analysis for NPRR662. - On 5/28/15, TAC considered NPRR662. |  |  |
| PRS Decision |  | On 11/6/14, PRS unanimously voted to table NPRR662 and refer the issue to WMS. All Market Segments were present for the vote. On 3/12/15, PRS voted to recommend approval of NPRR662 as amended by the 2/27/15 TIEC comments. There were three abstentions from the Investor Owned Utility (IOU) (2) and Independent Power Marketer (IPM) Market Segments. All Market Segments were present for the vote. On 4/16/15, PRS voted to table NPRR662 for one month. There were five abstentions from the Consumer, Independent Generator, and Municipal (3) Market Segments. All Market Segments were present for the vote. On 5/14/15, PRS voted to endorse and forward to TAC the 4/16/15 PRS Report and Impact Analysis for NPRR662 with a recommended priority of 2015 and rank of 1310. There was one opposing vote from the IOU Market Segment and three abstentions from Independent Generator (2) and IOU Market Segments. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 11/6/14, participants raised questions on the scope of the NPRR’s proposed language and briefly discussed potential solutions to its stated issue using the current market processes. On 3/12/15, concerns were raised regarding the purpose of NPRR662; that it was not targeting an existing problem; and that it could potentially change the market design. On 4/16/15, participants discussed the Impact Analysis, and whether tabling NPRR662 for one additional month might allow time to develop a lower cost alternative approach. On 5/14/15, participants discussed the costs and business case for NPRR662; its intended versus potential impacts; and the efficacy of training to address incomplete Energy Offer Curves. |  |  |
| TAC Decision |  | On 5/28/15, TAC voted via roll call vote to recommend approval of NPRR662 as recommended by PRS in the 5/14/15 PRS Report and as revised by TAC. There were four opposing votes from the IOU (2) and IPM (2) Market Segments and six abstentions from the IOU (2), IPM (2), and Municipal (2) Market Segments. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 5/28/15, participants discussed the costs and business case for NPRR662; revisions to the business case; and the efficacy of training to address incomplete Energy Offer Curves. |  |  |
| ERCOT Opinion |  | ERCOT supports Market Participants’ decision on NPRR662 as it aligns proxy Energy Offer Curve creation with recommendations made by the IMM. |  |  |

| Business Case |  |
| --- | --- |
| Qualitative Benefits | - Responsive to issues raised by the IMM - Facilitates more efficient and appropriate scarcity pricing - Makes additional energy available to the market sooner based on an automatically generated Energy Offer Curve |
| Quantitative Benefits | - Creates more appropriate pricing outcomes and will smooth price spikes when proxy energy is dispatched through SCED |
| Impact to Market Segments | - Positive impact to Consumer Market Segment in the form of lower energy prices - Positive impact to Generator Market Segement in the form of more appropriate price signals and more efficient proxy offers for energy without an offer |
| Credit Implications | No. |
| Other | None. |

| Sponsor |  |
| --- | --- |
| Name | Katie Coleman |
| E-mail Address | katie.coleman@tklaw.com |
| Company | Thompson & Knight LLP, on behalf of TIEC |
| Phone Number | 512-404-6705 |
| Cell Number | 512-773-0394 |
| Market Segment | Consumer |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Cory Phillips |
| E-Mail Address | cory.phillips@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 100714 | Proposed a correction in the revised language for paragraph (6) of Section 6.5.7.3. |
| WMS 121414 | Requested PRS continue to table NPRR662 for further review. |
| TIEC 010615 | Proposed changes regarding the extension of Energy Offer Curves and requested NPRR662 remain tabled until the ERCOT Board approved NPRR665, As-Built Clarification of Reliability Deployment Price Adder. |
| TIEC 022715 | Proposed revisions which superceded the 1/6/15 TIEC comments to align with the ERCOT Board-approved changes in NPRR665. |
| WMS 030515 | Endorsed NPRR662 as amended by the 2/27/15 TIEC comments. |

Comments

Please note that the baseline Protocol language in Section 6.5.7.3 has been updated due to the incorporation of NPRR665 into the March 1, 2015 Protocols.

Proposed Protocol Language Revision

6.5.7.3	Security Constrained Economic Dispatch

(1)	The SCED process is designed to simultaneously manage energy, the system power balance and network congestion through Resource Base Points and calculation of LMPs every five minutes.  The SCED process uses a two-step methodology that applies mitigation prospectively to resolve Non-Competitive Constraints for the current Operating Hour.  The SCED process evaluates Energy Offer Curves, Output Schedules and Real-Time Market (RTM) Energy Bids to determine Resource Dispatch Instructions by maximizing bid-based revenues minus offer-based costs, subject to power balance and network constraints.  The SCED process uses the Resource Status provided by SCADA telemetry under Section 6.5.5.2, Operational Data Requirements, and validated by the Real-Time Sequence, instead of the Resource Status provided by the COP.  An RTM Energy Bid represents the bid for energy distributed across all nodes in the Load Zone in which the Controllable Load Resource is located.

(2)	The SCED solution must monitor cumulative deployment of Regulation Services and ensure that Regulation Services deployment is minimized over time.

(3)	In the generation-to-be-dispatched determined by LFC, ERCOT shall subtract the sum of the telemetered net real power consumption from all Controllable Load Resources available to SCED.

[NPRR626: Replace paragraph (3) above with the following upon system implementation:] (3) In the Generation To Be Dispatched (GTBD) determined by LFC, ERCOT shall subtract the sum of the telemetered net real power consumption from all Controllable Load Resources available to SCED.

(4)	For use as SCED inputs, ERCOT shall use the available capacity of all committed Generation Resources by creating proxy Energy Offer Curves for certain Resources as follows:

(a)	Non-WGRs and Dynamically Scheduled Resources (DSRs) without Energy Offer Curves

ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below for:

(i)	Each non-WGR for which its QSE has submitted an Output Schedule instead of an Energy Offer Curve; and

(ii)	Each DSR that has not submitted Incremental and Decremental Energy Offer Curves.

| MW | Price (per MWh) |
| --- | --- |
| HSL | SWCAP |
| Output Schedule MW plus 1 MW | SWCAP minus $0.01 |
| Output Schedule MW | -$249.99 |
| LSL | -$250.00 |

[NPRR588: Replace paragraph (4)(a) above with the following upon system implementation:] (a) Non-IRRs and Dynamically Scheduled Resources (DSRs) without Energy Offer Curves ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below for: (i) Each non-IRR for which its QSE has submitted an Output Schedule instead of an Energy Offer Curve; and (ii) Each DSR that has not submitted Incremental and Decremental Energy Offer Curves. MW; Price (per MWh). HSL; SWCAP. Output Schedule MW plus 1 MW; SWCAP minus $0.01. Output Schedule MW; -$249.99. LSL; -$250.00.

(b)	DSRs with Energy Offer Curves

For each DSR that has submitted incremental and decremental Energy Offer Curves, ERCOT shall create a monotonically increasing proxy Energy Offer Curve.  That curve must consist of the incremental Energy Offer Curve that reflects the available capacity above the Resource’s Output Schedule to its HSL and the decremental Energy Offer Curve that reflects the available capacity below the Resource’s Output Schedule to the LSL.  The curve must be created as described below:

| MW | Price (per MWh) |
| --- | --- |
| Output Schedule MW plus 1 MW to HSL | Incremental Energy Offer Curve |
| LSL to Output Schedule MW | Decremental Energy Offer Curve |

(c)	Non-WGRs without full-range Energy Offer Curves

For each non-WGR for which its QSE has submitted an Energy Offer Curve that does not cover the full range of the Resource’s available capacity, ERCOT shall create a proxy Energy Offer Curve that extends the submitted Energy Offer Curve to use the entire available capacity of the Resource above the highest point on the Energy Offer Curve to the Resource’s HSL and the offer floor from the lowest point on the Energy Offer Curve to its LSL, using these points:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in submitted Energy Offer Curve) | Price associated with highest MW in submitted Energy Offer Curve |
| Energy Offer Curve | Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

(d)	WGRs

(i)	For each WGR that has not submitted an Energy Offer Curve, ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL | $1,500 |
| HSL minus 1 MW | -$249.99 |
| LSL | -$250.00 |

(ii)	For each WGR for which its QSE has submitted an Energy Offer Curve that does not cover the full range of the WGR’s available capacity, ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in submitted Energy Offer Curve) | Price associated with the highest MW in submitted Energy Offer Curve |
| Energy Offer Curve | Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

[NPRR588: Replace paragraphs (4)(c) and (4)(d) above with the following upon system implementation:] (c) Non-IRRs without full-range Energy Offer Curves For each non-IRR for which its QSE has submitted an Energy Offer Curve that does not cover the full range of the Resource’s available capacity, ERCOT shall create a proxy Energy Offer Curve that extends the submitted Energy Offer Curve to use the entire available capacity of the Resource above the highest point on the Energy Offer Curve to the Resource’s HSL and the offer floor from the lowest point on the Energy Offer Curve to its LSL, using these points: MW; Price (per MWh). HSL (if more than highest MW in submitted Energy Offer Curve); Price associated with the highest MW in submitted Energy Offer Curve. Energy Offer Curve; Energy Offer Curve. 1 MW below lowest MW in Energy Offer Curve (if more than LSL); -$249.99. LSL (if less than lowest MW in Energy Offer Curve); -$250.00. (d) IRRs (i) For each IRR that has not submitted an Energy Offer Curve, ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL; $1,500. HSL minus 1 MW; -$249.99. LSL; -$250.00. (ii) For each IRR for which its QSE has submitted an Energy Offer Curve, ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL (if more than highest MW in submitted Energy Offer Curve); Price associated with the highest MW in submitted Energy Offer Curve. Energy Offer Curve; Energy Offer Curve. 1 MW below lowest MW in Energy Offer Curve (if more than LSL); -$249.99. LSL (if less than lowest MW in Energy Offer Curve); -$250.00.

[NPRR665: Insert paragraph (4)(e) below upon system implementation of NPRR626:] (e) RUC-committed Resources (i) For each RUC-committed Resource that has not submitted an Energy Offer Curve, ERCOT shall create a proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL; RUC price floor, as detailed in paragraph (1) of Section 6.4.4.1, Energy Offer Curve for RUC-Committed Resources. Zero; Value of the RUC price floor, as detailed in paragraph (1) of Section 6.4.4.1. (ii) For each RUC-committed Resource that has submitted an Energy Offer Curve, ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL (if more than highest MW in Energy Offer Curve); Higher of RUC price floor or price associated with the highest MW in QSE submitted Energy Offer Curve. Energy Offer Curve; Greater of the RUC price floor, as detailed in paragraph (1) of Section 6.4.4.1, or the QSE submitted Energy Offer Curve. Zero; Greater of the value of the RUC price floor, as detailed in paragraph (1) of Section 6.4.4.1, or the first price point of the QSE submitted Energy Offer Curve.

(5)	The Entity with decision making authority, as more fully described in Section 3.19.1, Constraint Competitiveness Test Definitions, over how a Resource or Split Generation Resource is offered or scheduled, shall be responsible for all offers associated with each Resource, including offers represented by a proxy Energy Offer Curve.

(6)	For a Controllable Load Resource whose QSE has submitted an RTM Energy Bid that does not cover the full range of the Resource’s available Demand response capability, consistent with the Controllable Load Resource’s telemetered quantities, ERCOT shall create a proxy energy bid as described below:

| MW | Price (per MWh) |
| --- | --- |
| LPC to MPC minus maximum MW of RTM Energy Bid | Price associated with the lowest MW in submitted RTM Energy Bid curve |
| MPC minus maximum MW of RTM Energy Bid to MPC | RTM Energy Bid curve |
| MPC | Right-most point (lowest price) on RTM Energy Bid curve |

(7)	ERCOT shall ensure that any RTM Energy Bid is monotonically non-increasing.  The QSE representing the Controllable Load Resource shall be responsible for all RTM Energy Bids, including bids updated by ERCOT as described above.

(8)	A Controllable Load Resource with a telemetered status of OUTL is not considered as dispatchable capacity by SCED.  A QSE may use this function to inform ERCOT of instances when the Controllable Load Resource is unable to follow SCED Dispatch Instructions.  Under all telemetered statuses including OUTL, the remaining telemetry quantities submitted by the QSE shall represent the operating conditions of the Controllable Load Resource that can be verified by ERCOT.  A QSE representing a Controllable Load Resource with a telemetered status of OUTL is still obligated to provide any applicable Ancillary Service Resource Responsibilities previously awarded to that Controllable Load Resource.

(9)	Energy Offer Curves that were constructed in whole or in part with proxy Energy Offer Curves shall be so marked in all ERCOT postings or references to the energy offer.

(10)	The two-step SCED methodology referenced in paragraph (1) above is:

(a)	The first step is to execute the SCED process to determine Reference LMPs.  In this step, ERCOT executes SCED using the full Network Operations Model while only observing limits of Competitive Constraints.  Energy Offer Curves for all On-Line Generation Resources and RTM Energy Bids from available Controllable Load Resources, whether submitted by QSEs or created by ERCOT under this Section, are used in the SCED to determine “Reference LMPs.”

(b)	The second step is to execute the SCED process to produce Base Points, Shadow Prices, and LMPs, subject to security constraints (including Competitive and Non-Competitive Constraints) and other Resource constraints.  The second step must:

(i)	Use Energy Offer Curves for all On-Line Generation Resources, whether submitted by QSEs or created by ERCOT.  Each Energy Offer Curve must be bounded at the lesser of the Reference LMP (from Step 1) at the Resource Node or the appropriate Mitigated Offer Floor.  In addition, each Energy Offer Curve subject to mitigation under the criteria described in Section 3.19.4, Security-Constrained Economic Dispatch Constraint Competitiveness Test, must be capped at the greater of the Reference LMP (from Step 1) at the Resource Node plus a variable not to exceed 0.01 multiplied by the value of the Resource’s Mitigated Offer Cap curve at the LSL or the appropriate Mitigated Offer Cap;

(ii)	Use RTM Energy Bid curves for all available Controllable Load Resources, whether submitted by QSEs or created by ERCOT.  There is no mitigation of RTM Energy Bids; and

(iii)	Observe all Competitive and Non-Competitive Constraints.

(c)	ERCOT shall archive information and provide monthly summaries of security violations and any binding transmission constraints identified in Step 2 of the SCED process.  The summary must describe the limiting element (or identified operator-entered constraint with operator’s comments describing the reason and the Resource-specific impacts for any manual overrides).  ERCOT shall provide the summary to Market Participants on the MIS Secure Area and to the Independent Market Monitor (IMM).

(11)	For each SCED process, in addition to the binding Base Points and LMPs, ERCOT shall calculate a non-binding projection of the Base Points and Resource Node LMPs, Real-Time On-Line Reserve Price Adders, Real-Time Off-Line Reserve Price Adders, Hub LMPs and Load Zone LMPs at a frequency of every five minutes for at least 15 minutes into the future based on the same inputs to the SCED process as described in this Section, except that the Resource’s HDL and LDL and the total generation requirement will be as estimated at future intervals.  The Resource’s HDL and LDL will be calculated for each interval of the projection based on the ramp rate capability over the study period.  ERCOT shall estimate the projected total generation requirement by calculating a Load forecast for the study period.  ERCOT shall post the projected non-binding Base Points for each Resource for each interval study period on the MIS Certified Area and the projected non-binding LMPs for Resource Nodes, Real-Time On-Line Reserve Price Adders, Real-Time Off-Line Reserve Price Adders, Hub LMPs and Load Zone LMPs on the MIS Public Area pursuant to Section 6.3.2, Activities for Real-Time Operations.

[NPRR626: Replace paragraph (11) above with the following upon system implementation:] (11) For each SCED process, in addition to the binding Base Points and LMPs, ERCOT shall calculate a non-binding projection of the Base Points and Resource Node LMPs, Real-Time Reliability Deployment Price Adders, Real-Time On-Line Reserve Price Adders, Real-Time Off-Line Reserve Price Adders, Hub LMPs and Load Zone LMPs at a frequency of every five minutes for at least 15 minutes into the future based on the same inputs to the SCED process as described in this Section, except that the Resource’s HDL and LDL and the total generation requirement will be as estimated at future intervals. The Resource’s HDL and LDL will be calculated for each interval of the projection based on the ramp rate capability over the study period. ERCOT shall estimate the projected total generation requirement by calculating a Load forecast for the study period. In lieu of the steps described in Section 6.5.7.3.1, Determination of Real-Time On-Line Reliability Deployment Price Adder, the non-binding projection of Real-Time Reliability Deployment Price Adders shall be estimated based on GTBD, reliability deployments MWs, and aggregated offers. The Energy Offer Curve from SCED Step 2, the virtual offers for Load Resources deployed and the power balance penalty curve will be compared against the updated GTBD to get an estimate of the System Lambda from paragraph (2)(h) of Section 6.5.7.3.1. ERCOT shall post the projected non-binding Base Points for each Resource for each interval study period on the MIS Certified Area and the projected non-binding LMPs for Resource Nodes, Real-Time Reliability Deployment Price Adders, Real-Time On-Line Reserve Price Adders, Real-Time Off-Line Reserve Price Adders, Hub LMPs and Load Zone LMPs on the MIS Public Area pursuant to Section 6.3.2, Activities for Real-Time Operations.

(12)	For each SCED process, ERCOT shall calculate a Real-Time On-Line Reserve Price Adder and a Real-Time Off-Line Reserve Price Adder based on the On-Line and Off-Line available reserves in the ERCOT System and the Operating Reserve Demand Curve (ORDC).  The Real-Time Off-Line available reserves shall be administratively set to zero when the SCED snapshot of the Physical Responsive Capability (PRC) is equal to or below the PRC MW at which Energy Emergency Alert (EEA) Level 1 is initiated.  The Real-Time On-Line Reserve Price Adder shall be averaged over the 15-minute Settlement Interval and added to the Real-Time LMPs to determine the Real-Time Settlement Point Prices.  The price after the addition of the Real-Time On-Line Reserve Price Adder to LMPs approximates the pricing outcome of Real-Time energy and Ancillary Service co-optimization since the Real-Time On-Line Reserve Price Adder captures the value of the opportunity cost of reserves based on the defined ORDC.  An Ancillary Service imbalance Settlement shall be performed pursuant to Section 6.7.4, Real-Time Ancillary Service Imbalance Payment or Charge, to make Resources indifferent to the utilization of their capacity for energy or Ancillary Service reserves.

[NPRR626: Replace paragraph (12) above with the following upon system implementation:] (12) For each SCED process, ERCOT shall calculate a Real-Time On-Line Reserve Price Adder and a Real-Time Off-Line Reserve Price Adder based on the On-Line and Off-Line available reserves in the ERCOT System and the Operating Reserve Demand Curve (ORDC). The Real-Time Off-Line available reserves shall be administratively set to zero when the SCED snapshot of the Physical Responsive Capability (PRC) is equal to or below the PRC MW at which Energy Emergency Alert (EEA) Level 1 is initiated. In addition, for each SCED process, ERCOT shall calculate a Real-Time On-Line Reliability Deployment Price Adder. The sum of the Real-Time Reliability Deployment Price Adder and the Real-Time On-Line Reserve Price Adder shall be averaged over the 15-minute Settlement Interval and added to the Real-Time LMPs to determine the Real-Time Settlement Point Prices. The price after the addition of the sum of the Real-Time On-Line Reliability Deployment Price Adder and the Real-Time On-Line Reserve Price Adder to LMPs approximates the pricing outcome of the impact to energy prices from reliability deployments and the Real-Time energy and Ancillary Service co-optimization since the Real-Time On-Line Reserve Price Adder captures the value of the opportunity cost of reserves based on the defined ORDC. An Ancillary Service imbalance Settlement shall be performed pursuant to Section 6.7.4, Real-Time Ancillary Service Imbalance Payment or Charge, to make Resources indifferent to the utilization of their capacity for energy or Ancillary Service reserves.

(13)	ERCOT shall determine the methodology for implementing the ORDC to calculate the Real-Time On-Line Reserve Price Adder and Real-Time Off-Line Reserve Price Adder.  Following review by TAC, the ERCOT Board shall review the recommendation and approve a final methodology.  Within two Business Days following approval by the ERCOT Board, ERCOT shall post the methodology on the MIS Public Area.

(14)	At the end of each season, ERCOT shall determine the ORDC for the same season in the upcoming year, based on historic data using the ERCOT Board-approved methodology for implementing the ORDC.  Annually, ERCOT shall verify that the ORDC is adequately representative of the loss of Load probability for varying levels of reserves.  Twenty days after the end of the Season, ERCOT shall post the ORDC for the same season of the upcoming year on the MIS Public Area.

(15)	ERCOT may override one or more of a Controllable Load Resource’s parameters in SCED if ERCOT determines that the Controllable Load Resource’s participation is having an adverse impact on the reliability of the ERCOT System.