| VCMRR Number | 007 | VCMRR Title | Alignment with NPRR699, Energy Offer Curve Caps for Make-Whole Calculations for Resource Type Other |
| --- | --- | --- | --- |
| Date Posted |  | April 29, 2015 |  |
| Requested Resolution |  | Normal |  |
| Verifiable Cost Manual Sections Requiring Revision |  | Appendix 10, Procedure for Evaluating Costs and Caps for Energy Storage Resources |  |
| Other Binding Documents Requiring Revision or Related Revision Requests |  | Nodal Protocol Revision Request (NPRR) 699, Energy Offer Curve Caps for Make-Whole Calculations for Resource Type Other |  |
| Revision Description |  | NPRR699 revises paragraph (1)(l) in Protocol Section 4.4.9.2.3, Energy Offer Curve Validation, to specify the cap for Make-Whole Payments for all Resources categorized as Other. This Verifiable Cost Manual Revision Request (VCMRR) removes from the manual references to the cap for Energy Storage Resources as this would be inconsistent with the provision added to Protocols by NPRR699. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |

| Business Case |  |
| --- | --- |
| Qualitative Benefits | - Aligns the Verifiable Cost Manual language with the changes proposed in NPRR699. |
| Quantitative Benefits |  |
| Impact to Market Segments |  |
| Other |  |

| Sponsor |  |
| --- | --- |
| Name | Paul Vinson |
| E-mail Address | ptvinson@cpsenergy.com |
| Company | CPS Energy on behalf of the Resource Cost Working Group (RCWG) |
| Phone Number | 210-353-2101 |
| Cell Number |  |
| Market Segment | Not applicable. |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Kelly Landry |
| E-Mail Address | klandry@ercot.com |
| Phone Number | 512-248-4630 |

Proposed Manual Language Revision

# Appendix 10: Procedure for Evaluating Costs and Caps for Energy Storage Resources

Description

Energy Storage Resources are Resources that are capable of storing energy using Wholesale Storage Load (WSL) and releasing that energy at a later time to generate electric energy. These Resources will need to register under the Resource Category, “Other.” Energy Storage Resources that have registered with ERCOT will receive the generic offer caps as described in this appendix, with the exception of standard O&M costs. ERCOT will utilize the costs and caps as described below. ERCOT will calculate these caps on a monthly basis.

Standard O&M Costs

For Energy Storage Resources that choose to submit actual verifiable costs, these Resources may elect to utilize the standard costs for O&M as listed in the table below.

| Resource Type | Cold Startup ($/Start) | Intermediate Startup ($/Start) | Hot Startup ($/Start) | Variable O&M ($/MWh) |
| --- | --- | --- | --- | --- |
| Natural gas driven Compressed Air Energy Storage Resource | 5,000 | 5,000 | 5,000 | 3.15 |
| Non-natural gas driven Compressed Air Energy Storage Resource | 5,000 | 5,000 | 5,000 | 3.15 |
| All other Energy Storage Resources | 0 | 0 | 0 | 0 |

Startup Offer Generic Cap for Energy Storage Resources:

Energy Storage Resources that have registered with ERCOT will receive the following O&M Costs:

| Resource Type | O&M Costs ($) |
| --- | --- |
| Natural gas driven Compressed Air Energy Storage Resource | 5,000 |
| Non-natural gas driven Compressed Air Energy Storage Resource | 5,000 |
| All other Energy Storage Resources | 0 |

Minimum-Energy Generic Cap for Energy Storage Resources:

The Minimum-Energy Generic Cap for Energy Storage Resources represents a threshold limit. These Resources, however, may submit offers that exceed this limit. For Resources that receive RUC instructions without a Minimum Energy Offer, ERCOT will calculate the Minimum Energy Price per paragraph (3) of Section 5.7.1.4, Revenue Less Cost During QSE Clawback Intervals, of the Nodal Protocols, utilizing the data from the formula shown below.

Minimum-Energy Generic Cap = a1 * (Previous month first 15-day DAM average of corresponding WSL Resource Node Settlement Point Price) + b * FIP + c

Where:

For Natural Gas Driven Compressed Air Energy Storage Resources:

a1 = 1.2

b = 6 MMBtu/MWh

c = $15/MWh

For Non-Natural Gas Driven Compressed Air Energy Storage Resources:

a1 = 1.45

b = 0 MMBtu/MWh

c = $35/MWh

For All Other Energy Storage Resources:

a1 = 1.25

b = 0 MMBtu/MWh

c = $35/MWh

Mitigated Offer Cap (MOC) for Energy Storage Resources:

Standard Mitigated Offer Cap for Resources that have submitted VC:

- MOC = [IHR * FIP+ O&M] * multiplier

Mitigated Offer Cap for Energy Storage Resources:

- MOC = [a2 * (Previous month first 15-day DAM average of corresponding WSL Resource Node Settlement Point Price) + b * FIP + c] * multiplier

In order to manually implement the Mitigated Offer Cap for Energy Storage Resources, ERCOT will utilize the standard MOC format as follows:

Let O&M = a2 * (Previous month first 15-day DAM average of corresponding WSL Resource Node Settlement Point Price) + c

Let IHR = b

Where:

For Natural Gas Driven Compressed Air Energy Storage Resources:

a2 = 1.5

b = 6 MMBtu/MWh

c = $15/MWh

For Non-Natural Gas Driven Compressed Air Energy Storage Resources:

a2 = 1.75

b = 0 MMBtu/MWh

c = $35/MWh

For All Other Energy Storage Resources:

a2 = 1.75

b = 0 MMBtu/MWh

c = $35/MWh

Sample Calculation for a Natural Gas Driven Compressed Air Energy Storage Resource

- a1 = 1.2

- a2 = 1.5

- b = 6 MMBtu/MWh

- c = $15/MWh

- Fuel Index Price (FIP) = $5/MMBtu

- Previous month first 15-day DAM average of corresponding WSL Resource Node Settlement Point Price = $30/MWh

- Capacity Factor = 35%

- Multiplier = 1.15

Determine Startup Offer Generic Cap:

Startup Offer Generic Cap = $5,000

Determine Minimum Energy Generic Cap:

Minimum-Energy Generic Cap = a1 * (Previous month first 15-day DAM average of corresponding WSL Resource Node Settlement Point Price) + b * FIP + c

Minimum Energy Generic Cap = 1.2 * ($30/MWh) + 6 MMBtu/MWh * $5/MMBtu + $15/MWh

= $36/MWh + $30/MWh + $15/MWh

= $81/ MWh

Determine Mitigated Offer Cap:

MOC = [IHR * FIP+ O&M] * multiplier

IHR= 6MMBtu/MWh

O&M = a2 * (Previous month first 15-day DAM average of corresponding WSL Resource Node Settlement Point Price) +c

= 1.5 * $30/MWh + $15/MWh

= $45/MWh + $15/MWh

= $60/MWh

MOC = [6MMBtu/MWh * $5/MMBtu+ $60/MWh] * 1.15

= [$30/MWh + $60/MWh ] * 1.15

= $90/MWh * 1.15

= $103.50/MWh

The Minimum Energy Generic Cap is only used when Energy Storage Resources receive RUC instructions without a Minimum Energy Offer.