| VCMRR Number | 008 | VCMRR Title | Alignment with NPRR700, Utilizing Actual Fuel Cost in Startup Offer Caps |
| --- | --- | --- | --- |
| Date of Decision |  | July 8, 2015 |  |
| Action |  | Recommended Approval |  |
| Timeline |  | Normal. |  |
| Proposed Effective Date |  | To be determined. |  |
| Priority and Rank Assigned |  | To be determined. |  |
| Verifiable Cost Manual Sections Requiring Revision |  | Section 3, Verifiable Startup Costs Appendix 5, Specification of Relevant Equations |  |
| Related Documents Requiring Revision/Revision Requests |  | Nodal Protocol Revision Request (NPRR) 700, Utilizing Actual Fuel Costs in Startup Offer Caps |  |
| Revision Description |  | NPRR700 revises Protocols to incorporate actual fuel costs into the Day-Ahead Market (DAM) and Reliability Unit Commitment (RUC) procurement process and DAM Make-Whole Payment calculations. This Verifiable Cost Manual Revision Request (VCMRR) aligns the Verifiable Cost Manual with the provisions proposed by NPRR700. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | This VCMRR aligns the Verifiable Cost Manual language with the changes proposed in NPRR700, and allows Resources to submit Startup Offers that more accurately represent their actual costs. |  |
| WMS Decision |  | On 6/3/15, WMS unanimously voted to table VCMRR008. The Consumer Market Segment was not present for the vote. On 7/8/15, WMS voted to recommend approval of VCMRR008 as amended by the 6/26/15 ERCOT comments. There was one abstention from the Consumer Market Segment. All Market Segments were present for the vote. |  |
| Summary of WMS Discussion |  | On 6/3/15, ERCOT noted that comments are pending to VCMRR008 to make clarifications and corrections consistent with its original intent. On 7/8/15, participants reviewed the 6/26/15 ERCOT comments. |  |

| Sponsor |  |
| --- | --- |
| Name | Paul Vinson |
| E-mail Address | ptvinson@cpsenergy.com |
| Company | CPS Energy on behalf of the Resource Cost Working Group (RCWG) |
| Phone Number | 210-353-2101 |
| Cell Number |  |
| Market Segment | Not applicable. |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Kelly Landry |
| E-Mail Address | klandry@ercot.com |
| Phone Number | 512-248-4630 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 062615 | Proposed additional revisions in order to align with NPRR700 as recommended for approval by the Protocol Revision Subcommittee (PRS) at its June 11, 2015 meeting. |

Market Rules Notes

Please note that VCMRR009, Calculation of the Minimum Requirements Fee, also proposes revisions to Section 3.

Proposed Manual Language Revision

# Section 3:  Verifiable Startup Costs

This section of ERCOT’s Verifiable Cost Manual describes policies and procedures that relate to the submission of Verifiable Startup Costs.

### Verifiable Startup Costs Policies

- The components of startup costs are fuel consumption rates and incremental O&M costs.  These are to be derived by applying financial and/or engineering analysis to manufacturer data, operational data, or the results of recent tests on a Resource.

- Startup fuel consumption rates (MMBtu/start) must be submitted for all startup types (cold, intermediate, and hot) for a Filing Entity to be considered as having submitted Verifiable Startup Costs.  However, if a Resource does not have a distinct start type which is analogous to intermediate startup (or if there are no costs specific to an intermediate start), the Filing Entity must assign a value to the intermediate startup costs equal to the hot startup costs.

- Submitted startup fuel consumption rates and O&M costs will be reviewed by ERCOT and, upon approval, will be used prospectively to calculate Verifiable Startup Costs.

- Fuel consumed from breaker open to Shutdown may be included as a Verifiable Startup Cost.

- For additional information on how to calculate the Verifiable Startup Costs, please refer to Appendix 5, Specification of Relevant Equations.

### Submitting Startup Costs

Verifiable Startup Costs for a Resource represent a proxy for all of the costs incurred in order to bring a Resource online and make it available to produce power.  Only the costs incurred from startup through LSL and from breaker open to Shutdown are permissible startup costs.

The nature of the data that a Filing Entity may submit is set forth below and also partially within Section 5 of the Protocols, which defines Verifiable Startup Costs as:

1.	Adjusted fuel consumption rate per start (MMBtu/start) multiplied by a resource category generic fuel price (FIP, FOP, or $1.50 per MMBtu, as applicable); and

[VCMRR005: Replace paragraph 1 above with the following upon system implementation of NPRR664:] - Adjusted fuel consumption rate per start (MMBtu/start) multiplied by a resource category generic fuel price (FIPRr, FOP, or $1.50 per MMBtu, as applicable); and

- Unit-specific verifiable operation and maintenance expenses.

#### Startup Fuel Consumption

Fuel consumed during a startup is defined as the fuel consumed from first fire through LSL plus the fuel consumed from breaker open through Shutdown (Including auxiliary boiler fuel and auxiliary-equipment fuel or electrical power requirements), excluding normal plant heating.

It is expected that the amount of fuel consumed will be different for each of the three start types.  If available, historical data must be used to determine the typical amount of fuel consumed per start for each start type.  This typical per-start fuel consumption is to be determined in accordance with the following rules:

- When possible, startup fuel consumption rates are to be based on the amount of fuel a Resource has historically consumed per start.

- For a Filing Entity that does not submit seasonal data, submitted historical usage data should, for each start type, include fuel consumption rates for the lesser of the last 10 starts or every start within the past three (3) years.  For a Filing Entity that submits seasonal data, submitted historical usage data should, for each start type, include fuel consumption rates for the lesser of the last three (3) starts or every start within the past three years for each season. For each start type, the Filing Entity shall submit the historical fuel consumption data, an average amount of historical fuel consumption, and the fuel consumption rate that the Resource believes represents the current startup fuel consumption rate.

- If a Resource does not have the historical fuel consumption rates for each start described above, the Filing Entity must submit the aforementioned data that it does possess and may also include per-start fuel consumption rates based on manufacturer suggested values or tests which are ERCOT approved.

- If a Filing Entity submits historical startup fuel consumption data on a per hour basis (MMBtu/hour), it must also provide proof of the average number of hours it requires to reach LSL for each startup type.

- In its sole discretion, however, ERCOT may choose not to accept the Filing Entity’s submitted per-start fuel consumption rates if ERCOT determines that they do not represent a Resource’s “true” startup fuel consumption or that they have not been proven in sufficient detail.

- Historical fuel consumption rates must be based on documented metered reads when available.

Additional Rules for Submitting Fuel Costs

- Filing Entities may elect to submit an actual fuel adder ($/MMBtu) for each Resource for verification by ERCOT. For any Filing Entity that submits actual verifiable costs, the fuel adder will default to $0.50/MMBtu until the Filing Entity establishes an actual fuel adder in those verifiable costs and the Filing Entity must continue to provide actual fuel costs as prescribed in paragraph (2) below.  The fuel adder is included in the value of X (VOX) as described in Appendix 6, Calculation and Application of Proxy Heat Rate and the Value of X.

- Any Filing Entity that submits an actual fuel adder must provide documentation that establishes the historical costs for fuel, including transportation, spot fuel, and any additional verifiable cost associated with fuel contracts that can be easily differentiated from the standard commodity cost of fuel and clearly attributable to the Resource for the period.  The fuel adder for a rolling 12-month period is the difference between the Filing Entity’s average fuel price paid (including all fees) during the period and the fuel price utilized by ERCOT for the corresponding Resource.  The Filing Entity shall provide rolling 12-month supporting data to verify total fuel price for all purchased volumes to support the actual Resource fuel consumption.  Data to support these costs should include, but are not limited to, accounting ledger entries, invoices, and copies of fuel contracts.  In addition, the actual costs used to calculate the fuel adder may include, but are not limited to, the following categories: transportation, deliveries, storage, injection, withdrawal, imbalance, and minimum requirements fees.  Other costs not described herein may be included and approved by ERCOT.

Note:   Review and approval of fuel costs follows the same timeline as verifiable costs; however, ERCOT may require additional time to verify the fuel costs based on the complexity of the submission.  In such case, ERCOT will notify the Filing Entity within 15 Business Days of submission if additional time is needed.  For clarification on the submission timeline for the fuel adder, please see the table below.  The fuel adder will be implemented the first day of the month after fuel costs have been approved.

| Submission Months | Submission Period | Approval Period |
| --- | --- | --- |
| September- February | April | May-June |
| March – August | October | November-December |

Fuel Type Percentages

For each start type, the Filing Entity must provide documentation establishing the respective ratios of gas, oil, and solid fuel consumed during the startup through LSL sequence. Historical and/or manufacturer suggested ratios are to be submitted as percentages and in accordance with the manner of submitting startup fuel consumption data, detailed above.  For each start type, the Filing Entity must calculate and submit:

- total fuel consumption per-start (MMBtu/start); and

- the ratio of each type of fuel consumed to the total amount of fuel consumed per startup.

#### Filing Entities with approved fuel consumption ratios for the associated Resource are to submit updated data to ERCOT if they subsequently use a different fuel type during startup and if they also anticipate doing so for any substantial period of time, whether due to fuel market conditions or otherwise.

#### Non-Fuel Startup Costs

Verifiable Non-Fuel Startup Costs represent a proxy for all non-fuel costs that a Resource incurs during the startup through LSL and from breaker open to shutdown sequence.   The costs that ERCOT considers in calculating this proxy include incremental operation and maintenance costs (Verifiable O&M) that can reasonably be said to result from the Resource starting up.  Verifiable O&M Costs include and incremental emission costs applicable to net generation between BC and LSL. For more information see Section 2: General Rules of Verifiable Costs.

To be included as a Verifiable O&M Cost of Startup, O&M costs must be submitted in accordance with Section 9:  Operating and Maintenance Cost Guidelines.  ERCOT will not approve submitted O&M startup costs if the amounts or the methods used to calculate them do not coincide with other O&M costs a Filing Entity has submitted, unless there is a reasonable, documented reason for doing so.  For example, startup operating costs might be different because there are greater chemical, water or emissions costs during the startup sequence.  Also, it might be reasonable to multiply an hours-based maintenance cost by the amount of time it takes to complete a startup to LSL sequence.  Additionally, if maintenance costs are allocated on a per-start basis, it might be reasonable for the maintenance component of verifiable startup costs to differ per start type.

#### Start Type Descriptions

The following is a general description of startup costs per start type:

Hot Startup Cost

Hot startup cost is the expected cost to start a Resource, which is in the "hot" condition. Hot conditions vary unit by unit, but in general, a steam unit is hot through an overnight shutdown.

Intermediate Startup Cost

Intermediate startup cost is the expected cost to start a Resource that has recently been online and for which neither hot nor cold conditions are applicable.

Cold Startup Cost

Cold startup cost is the expected cost to start a Resource which is in the "cold" condition.  Cold conditions vary unit by unit, but in general, a steam unit is cold after a two or three-day shutdown.

#### Table 1:  Startup Verifiable Cost Data Requirements per Resource per start type

| Input Data | Description |
| --- | --- |
| RAFCRS (MMBtu/Start) | Verified Total Fuel consumption rate per start |
| GASPERSU (%) | Gas fuel consumed per start as a percentage of total per start fuel consumption |
| OILPERSU (%) | Oil fuel consumed per start as a percentage of total per start fuel consumption |
| SFPERSU (%) | Solid fuel consumed per start as a percentage of total per start fuel consumption |
| O&M ($/start) | Verifiable O&M expenses per start |

# Appendix 5:  Specification of Relevant Equations

Equation 1:  Verifiable Startup Offer Cap  ($/Start)

Verifiable Startup Offer Cap ($/Start) = DAFCRS (MMBtu/Start) * [(GASPERSU*FIP + OILPERSU*FOP)/100] + VOMS

Where:	DAFCRS = Total Fuel * (1+VOX)

Total Fuel = [FuelStartup-BC + FuelBC-LSL + FuelBO-Shutdown]

The bill determinants utilized above are defined as:

DAFCRS = the adjusted verified fuel consumption for the start type (MMBtu/Start)

GASPERSU = Percentage of natural gas used for a start

FIP = Fuel Index Price ($/MMBtu)

OILPERSU = Percentage of oil used for a start

FOP = Fuel Oil Price ($/MMBtu)

VOMS = the verified O&M cost for a hot start ($/Start)

VOX= Value of X

FuelStartup-BC= Fuel quantity required to bring Resource from Startup to Breaker Close (MMBtu)

FuelBC-LSL= Fuel quantity required to bring Resource from Breaker Close to Minimum Energy at LSL (MMBtu)

FuelBO-Shutdown= Fuel quantity required to take Resource from Breaker Open to Shutdown (MMBtu)

Note 1: GASPERSU and OILPERSU are decimal percentages in the Settlements equations and will be multiplied by 100 during the Integration process.

Note 2: ERCOT will use the solid fuel price and percentages to create Startup offers when no offer is submitted by the QSE for solid fuel Resources.

Note 3: This equation does not include any adjustments made to the final calculation of the Startup Offer cap, as described in Protocol Section 4.4.9.2.1, Startup Offer and Minimum-Energy Offer Criteria.

[VCMRR005: Replace Equation 1 above with the following upon system implementation of NPRR664:] Equation 1: Verifiable Startup Offer Cap ($/Start) Verifiable Startup Offer Cap ($/Start) = DAFCRS (MMBtu/Start) * [(GASPERSU*FIPRr+ OILPERSU*FOP)/100] + VOMS Where: DAFCRS = Total Fuel * (1+VOX) Total Fuel = [FuelStartup-BC + FuelBC-LSL + FuelBO-Shutdown] The bill determinants utilized above are defined as: DAFCRS = the adjusted verified fuel consumption for the start type (MMBtu/Start) GASPERSU = Percentage of natural gas used for a start FIPRr = Fuel Index Price for Resource ($/MMBtu) OILPERSU = Percentage of oil used for a start FOP = Fuel Oil Price ($/MMBtu) VOMS = the verified O&M cost for a hot start ($/Start) VOX= Value of X FuelStartup-BC= Fuel quantity required to bring Resource from Startup to Breaker Close (MMBtu) FuelBC-LSL= Fuel quantity required to bring Resource from Breaker Close to Minimum Energy at LSL (MMBtu) FuelBO-Shutdown= Fuel quantity required to take Resource from Breaker Open to Shutdown (MMBtu) Note 1: GASPERSU and OILPERSU are decimal percentages in the Settlements equations and will be multiplied by 100 during the Integration process. Note 2: ERCOT will use the solid fuel price and percentages to create Startup offers when no offer is submitted by the QSE for solid fuel Resources. Note 3: This equation does not include any adjustments made to the final calculation of the Startup Offer cap, as described in Protocol Section 4.4.9.2.1, Startup Offer and Minimum-Energy Offer Criteria.

Equation 2:  Verifiable Minimum-Energy Offer Cap ($/MWh)

Verifiable Minimum-Energy Offer Cap ($/MWh) = AHR*[(GASPERME*FIP + OILPERME*FOP)/100] + VOMLSL

Where:	AHR(1)= Fuel Rate (MMBtu/Hour) divided by LSL (MW)

GASPERME = Percentage of natural gas used at LSL

FIP = Fuel Index Price ($/MMBtu)

OILPERME = Percentage of oil used at LSL

FOP = Fuel Oil Price ($/MMBtu)

VOMLSL = the verified O&M cost at Minimum-Energy ($/MWh)

(1)  Adjusted by VOX

And:		AHR= (verified fuel consumption/LSL)*(1+VOX)

Note 1: GASPERME and OILPERME are decimal percentages in the Settlements equations and will be multiplied by 100 during the Integration process.

Note 2:  ERCOT will use the solid fuel price and percentages to create Startup offers when no offer is submitted by the QSE for solid fuel Resources.

Note 3: This equation does not include any adjustments made to the final calculation of the Minimum-Energy Offer cap, as described in Protocol Section 4.4.9.2.1, Startup Offer and Minimum-Energy Offer Criteria.

[VCMRR005: Replace Equation 2 above with the following upon system implementation of NPRR664:] Equation 2: Verifiable Minimum-Energy Offer Cap ($/MWh) Verifiable Minimum-Energy Offer Cap ($/MWh) = AHR*[(GASPERME*FIPRr + OILPERME*FOP)/100] + VOMLSL Where: AHR(1)= Fuel Rate (MMBtu/Hour) divided by LSL (MW) GASPERME = Percentage of natural gas used at LSL FIPRr = Fuel Index Price for Resource ($/MMBtu) OILPERME = Percentage of oil used at LSL FOP = Fuel Oil Price ($/MMBtu) VOMLSL = the verified O&M cost at Minimum-Energy ($/MWh) (1) Adjusted by VOX And: AHR= (verified fuel consumption/LSL)*(1+VOX) Note 1: GASPERME and OILPERME are decimal percentages in the Settlements equations and will be multiplied by 100 during the Integration process. Note 2: ERCOT will use the solid fuel price and percentages to create Startup offers when no offer is submitted by the QSE for solid fuel Resources. Note 3: This equation does not include any adjustments made to the final calculation of the Minimum-Energy Offer cap, as described in Protocol Section 4.4.9.2.1, Startup Offer and Minimum-Energy Offer Criteria.

Equation 3:  Calculation of Composite Unit Parameters using Alternate Unit Specifications

Composite Unit Parameter = [Alt_Unit_Par*Alt_Unit_HSL +  Non_Alt_Unit_Par* Non_Alt_Unit_HSL] / [Alt_Unit_HSL + Non_Alt_Unit_HSL]

Where:		Alt_Unit_Par = Relevant parameter of Alternate Unit

Alt_Unit_HSL = High Sustained Limit of Alternate Unit

Non_Alt_Unit_Par = Relevant parameter of non-Alternate Unit

Non_Alt_Unit_HSL = High Sustained Limit of non-Alternate Unit

This calculation would be executed for all relevant parameters of the alternate and non-alternate units.  This would include for example Startup Cost data, Minimum-Energy Cost data and heat rate data.

Equation 4:  Equation for Calculation of Verifiable Startup Emission Costs

Verifiable Startup Emission Cost ($/Start) = RAFCRS * ∑Emission Rate i * Emission Cost Index i

Where            RAFCRS = Quantity of approved startup fuel consumed by Resource (including fuel used to shutdown Resource (MMBtu/Start)

Emission Rate i = Quantity of emission i emitted by resource (lbs/MMBtu)

Emission Cost Index i = Published cost index of emission i ($/lb)

i = Index for each emittent approved for inclusion in Startup Cost

Equation 5:  Equation for Calculation of Verifiable Minimum-Energy Emission Costs

Verifiable Minimum-Energy Emission Costs ($/MWh) =

[AHR] * ∑Emission Rate i * Emission Cost Index i

Where		AHR = Average heat rate at Minimum Energy (MMBtu/Hr)

Emission Rate i = Quantity of emission i emitted by resource (lbs/MMBtu)

Emission Cost Index i = Published cost index of emission i

i = Index of each emittent approved for inclusion in Minimum-Energy Cost

Equation 6:  Verifiable Startup Costs (VERISU)  ($/Start)

- For RUC Settlements, the Verifiable Startup Costs are calculated as follows:

VERISU = AFCRS + VOMS

Where		AFCRS = [Total Fuel - PHR * AVGEN + Total Fuel*VOX] * [FIP*GASPERSU(%) + FOP*OILPERSU(%) + SFP*SFPERSU(%)]

Total Fuel = [FuelStartup-BC + FuelBC-LSL + FuelBO-Shutdown]

VOMS = IO&MStart-LSL +IO&MBO-Shutdown + Verifiable Startup Emission Costs

- For DAM Make-Whole Payments, the Verifiable Startup Costs are calculated as follows:

- VERISU = DAFCRS + VOMS

- Where	DAFCRS = [Total Fuel + Total Fuel*VOX] * [FIP*GASPERSU(%) + FOP*OILPERSU(%) + SFP*SFPERSU(%)]

- Total Fuel = [FuelStartup-BC + FuelBC-LSL + FuelBO-Shutdown]

- VOMS = IO&MStart-LSL +IO&MBO-Shutdown + Verifiable Startup Emission Costs

WhereThe bill determinants utilized above are defined as:

VERISU = Verifiable Startup Costs ($/Start)

AFCRS= Verifiable Startup Fuel Costs adjusted by VOX and PHR($/Start)

DAFCRS(1) = the adjusted verified fuel consumption rate for a hot  the start type (MMBtu/Start)

VOMS = Verifiable Operations and Maintenance Costs ($/Start)

FuelStartup-BC= Fuel Quantity required to bring Resource from Startup to Breaker Close (MMBtu)

FuelBC-LSL= Fuel Quantity required to bring Resource from Breaker Close to Minimum Energy at LSL (MMBtu)

FuelBO-Shutdown= Fuel Quantity required to take Resource from Breaker Open to Shutdown (MMBtu)

PHR = Proxy Heat Rate (MMBtu/MWh)

AVGEN= Average Generation between Breaker Close and LSL (MWh)

VOX= Value of X

FIP = Fuel Price Index for gas ($/MMBtu)

FOP = Fuel Price Index for oil ($/MMBtu)

SFP = Fuel Price Index for solid fuel = $1.50/MMBtu

GASPERSU = Percent of gas used during startup

OILPERSU = Percent of oil used during startup

SFPERSU = Percent of solid fuel used during startup

IO&MStart-LSL = Incremental O&M costs incurred to bring Resource from Start to LSL ($/Start)

IO&MBO-Shutdown = Incremental O&M costs incurred to take Resource from Breaker Open to Shutdown ($/Start)

Verifiable Startup Emission Costs = The allowable costs of acquiring emission credits required to start up Resource and defined in Equation 4 above .

[VCMRR005: Replace Equation 6 above with the following upon system implementation of NPRR664:] Equation 6: Verifiable Startup Costs (VERISU) ($/Start) - For RUC Settlements, the Verifiable Startup Costs are calculated as follows: VERISU = AFCRS + VOMS Where AFCRS = [Total Fuel - PHR * AVGEN + Total Fuel*VOX] * [FIPRr*GASPERSU(%) + FOP*OILPERSU(%) + SFP*SFPERSU(%)] Total Fuel = [FuelStartup-BC + FuelBC-LSL + FuelBO-Shutdown] VOMS = IO&MStart-LSL +IO&MBO-Shutdown + Verifiable Startup Emission Costs - For DAM Make-Whole Payments, the Verifiable Startup Costs are calculated as follows: - VERISU =DAFCRS + VOMS Where DAFCRS = [Total Fuel + Total Fuel*VOX] * [FIP*GASPERSU(%) + FOP*OILPERSU(%) + SFP*SFPERSU(%)] Total Fuel = [FuelStartup-BC + FuelBC-LSL + FuelBO-Shutdown] VOMS = IO&MStart-LSL +IO&MBO-Shutdown + Verifiable Startup Emission Costs WhereThe bill determinants utilized above are defined as: VERISU = Verifiable Startup Costs ($/Start) AFCRS= Verifiable Startup Fuel Costs adjusted by VOX and PHR ($/Start) DAFCRS= the adjusted verified fuel consumption rate for the start type (MMBtu/Start) VOMS = Verifiable Operations and Maintenance Costs ($/Start) FuelStartup-BC= Fuel Quantity required to bring Resource from Startup to Breaker Close (MMBtu) FuelBC-LSL= Fuel Quantity required to bring Resource from Breaker Close to Minimum Energy at LSL (MMBtu) FuelBO-Shutdown = Fuel Quantity required to take Resource from Breaker Open to Shutdown (MMBtu) PHR = Proxy Heat Rate (MMBtu/MWh) AVGEN= Average Generation between Breaker Close and LSL (MWh) VOX= Value of X FIPRr= Fuel Index Price for Resource ($/MMBtu) FOP = Fuel Price Index for oil ($/MMBtu) SFP = Fuel Price Index for solid fuel = $1.50/MMBtu GASPERSU = Percent of gas used during startup OILPERSU = Percent of oil used during startup SFPERSU = Percent of solid fuel used during startup IO&MStart-LSL = Incremental O&M costs incurred to bring Resource from Start to LSL ($/Start) IO&MBO-Shutdown = Incremental O&M costs incurred to take Resource from Breaker Open to Shutdown ($/Start) Verifiable Startup Emission Costs = The allowable costs of acquiring emission credits required to start up Resource and defined in Equation 4 above .

Equation 7:  The Equation for calculating Verifiable Minimum Energy Costs ($/MWh)

VERIME = FCLSL + VOMLSL

Where 		VERIME = Verifiable Minimum Energy Costs

FCLSL = Verifiable Fuel Costs at Minimum Energy

VOMLSL = Verifiable variable O&M costs at Minimum Energy

FCLSL = [(AHR)] * [FIP*GASPERME(%) + FOP*OILPERME(%) + SFP*SFPERME(%)]

Where		AHR = Adjusted average heat rate at Minimum Energy (MMBtu/Hr)

FIP = Fuel Price Index for gas ($/MMBtu)

FOP = Fuel Price Index for oil ($/MMBtu)

SFP = Fuel Price Index for solid fuel = $1.50/MMBtu

GASPERME = Percent of gas used at minimum energy

OILPERME = Percent of oil used at minimum energy

SFPERME = Percent of solid fuel used at minimum energy

VOMLSL = IO&MLSL + Verifiable Emission Costs at Minimum Energy

Where		IO&MLSL = Incremental O&M costs at minimum energy

Verifiable Emission Costs at Minimum Energy = The allowable costs of acquiring emission credits required to operate Resource at minimum energy and defined in Equation 5 above.

[VCMRR005: Replace Equation 7 above with the following upon system implementation of NPRR664:] Equation 7: The Equation for calculating Verifiable Minimum Energy Costs ($/MWh) VERIME = FCLSL + VOMLSL Where VERIME = Verifiable Minimum Energy Costs FCLSL = Verifiable Fuel Costs at Minimum Energy VOMLSL = Verifiable variable O&M costs at Minimum Energy FCLSL = [(AHR)] * [FIPRr*GASPERME(%) + FOP*OILPERME(%) + SFP*SFPERME(%)] Where AHR = Adjusted average heat rate at Minimum Energy (MMBtu/Hr) FIPRr = Fuel Index Price for Resource ($/MMBtu) FOP = Fuel Price Index for oil ($/MMBtu) SFP = Fuel Price Index for solid fuel = $1.50/MMBtu GASPERME = Percent of gas used at minimum energy OILPERME = Percent of oil used at minimum energy SFPERME = Percent of solid fuel used at minimum energy VOMLSL = IO&MLSL + Verifiable Emission Costs at Minimum Energy Where IO&MLSL = Incremental O&M costs at minimum energy Verifiable Emission Costs at Minimum Energy = The allowable costs of acquiring emission credits required to operate Resource at minimum energy and defined in Equation 5 above.