| NPRR Number | 714 | NPRR Title | Real-Time Make-Whole Payment for Exceptional Fuel Cost |
| --- | --- | --- | --- |
| Date of Decision |  | September 10, 2015 |  |
| Action |  | Recommended Approval. |  |
| Timeline |  | Normal. |  |
| Proposed Effective Date |  | November 1, 2015 (manual implementation). |  |
| Priority and Rank Assigned |  | Priority – 2015; Rank – 1370 (partial automation). |  |
| Nodal Protocol Sections Requiring Revision |  | 6.6.3.7, Real-Time Make-Whole Payment for Exceptional Fuel Cost 6.6.3.8, Real-Time Make-Whole Charge for Exceptional Fuel Cost Charge 9.5.3, Real-Time Market Settlement Charge Types |  |
| Related Documents Requiring Revision/Revision Requests |  | Verifiable Cost Manual Revision Request (VCMRR) 010, Alignment with NPRR714, Real-Time Make-Whole Payment for Exceptional Fuel Cost |  |
| Revision Description |  | Nodal Protocol Revision Request (NPRR) 664, Fuel Index Price for Resource Definition and Real-Time Make-Whole Payments for Exceptional Fuel Cost Events, included a Real-Time Make-Whole Payment provision to address exceptional fuel cost events that are not resolved by a change to the Fuel Index Price for Resource (FIPRr). ERCOT implemented a manual process to satisfy the requirements of these sections until May 1, 2015, at which time Sections 6.6.3.7 and 6.6.3.8 were grey boxed pending an automated solution. This NPRR restores the manual process for Real-Time Make-Whole Payments and Charges to address exceptional fuel cost events; and includes revisions that clarify and improve the Settlement calculations for the manual process consistent with the intent of NPRR664. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | The change will allow Resources to file for a Real-Time Make-Whole Payment when mitigated to below its actual cost due to spikes in gas prices. |  |
| Credit Work Group Review |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR714 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |
| PRS Decision |  | On 7/16/15, PRS voted to recommend approval of NPRR714 as submitted. There was one abstention from the Municipal Market Segment. All Market Segments were present for the vote. On 8/13/15, PRS voted to table NPRR714. There was one abstention from the Independent Power Marketer (IPM) Market Segment. All Market Segments were present for the vote. On 9/10/15, PRS voted to endorse and forward to TAC the 8/13/15 PRS Report and Impact Analysis (Option 2) for NPRR714 with an effective date of November 1, 2015 (manual) and a recommended priority of 2015 and a rank of 1370 (partial automation). There was one opposing vote from the Independent Power Marketer (IPM) Market Segment and one abstention from the Independent Retail Electric Provider (IREP) Market Segment. All Market Segments were present for the vote. |  |
| Summary of PRS Discussion |  | On 7/16/15, participants discussed the merits of NPRR714; its relation to NPRR664; and the clarification edits to the Settlement calculations included in the original submission. On 8/13/15, participants discussed possible methodologies for implementation of NPRR714 and noted pending discussion of this issue at the next regularly scheduled TAC meeting. On 9/10/15, participants discussed the need for manual implementation of NPRR714 until the parital automation can be completed; and that the rank and priority for partial automation should ensure that it is achieved prior to the winter of 2016. |  |

| Sponsor |  |
| --- | --- |
| Name | Robert Helton |
| E-mail Address | Robert.helton@gdfsuzna.com |
| Company | GDF SUEZ Energy NA |
| Phone Number | 713-636-1814 |
| Cell Number | 832-435-7815 |
| Market Segment | Independent Generator |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Kelly Landry |
| E-Mail Address | klandry@ercot.com |
| Phone Number | 512-248-4630 |

Market Rules Notes

Please note that the following NPRRs also propose revisions to Section 9.5.3:

- NPRR649, Lost Opportunity Payments for HDL Manual Overrides

- NPRR667, Ancillary Service Redesign

- NPRR689, Settlement of Ancillary Service Assignment in Real-Time Operations

Proposed Protocol Language Revision

- 6.6.3.7	Real-Time Make-Whole Payment for Exceptional Fuel Cost

(1)	Natural Gas or Fuel Oil Generation Resources that have approved verifiable costs and received at least one Base Point on their Mitigated Offer Cap during a 15-minute Settlement Interval may be eligible to receive a Real-Time Make-Whole Payment for exceptional fuel cost.  ERCOT shall deem a Generation Resource eligible for this payment if the QSE for the Resource submits the actual price paid for delivered fuel for the specific Resource and it is greater than the sum of the fuel price (e.g. Fuel Index Price (FIP) or Fuel Oil Price (FOP)) used by ERCOT for the Resource, the fuel adder, and a threshold fuel price, as defined in the Verifiable Cost Manual.  For eligible Resources, calculation for Real-Time Make-Whole Payment for exceptional fuel costs shall be determined as follows:

EFCMWAMT q, p, r, i  =  (-1) * EFCPR q, p, r, i  * EFCQTY q, p, r, i

Where:

EFCPR q, p, r, i   =  Max[0, (Min(EFAIEC q, p, r, i  ,  ADMOCPR q, p, r, i  ) -  RTSPP p, i  ) - EBPWAPR q, r, p,i]

And:

EFCQTY q, p, r, i  =  Min(AVGBP q, r, p,i *¼, RTMG q, r, p, i )

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| EFCMWAMT q, p, r, i | $ | Exceptional Fuel Cost Make-Whole Payment Amount per QSE per Settlement Point per Generation Resource—The exceptional fuel cost Make-Whole Payment to QSE q at Resource Node p as a result of Generation Resource r receiving a Base Point at the Mitigated Offer Cap during a 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| EFCPR q, p, r, i | $/MWh | Exceptional Fuel Cost Base Point Average Price per QSE per Settlement Point per Generation Resource—The price used to compensate an eligible Generation Resource r at Resource Node p represented by QSE q, for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| EFCQTY q, p, r, i | MWh | Exceptional Fuel Cost Energy per QSE per Settlement Point per Generation Resource—The quantity used to compensate an eligible Generation Resource r at Resource Node p represented by QSE q in Real-Time during an exceptional fuel cost event, for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| EFAIEC q, p, r, i | $/MWh | Exceptional Fuel Cost Average Incremental Energy Cost per QSE per Settlement Interval per Generation Resource— The average incremental energy cost calculated according to the Energy Offer Curve (not subject to the cost cap) for the corresponding output levels between the Average Base Point (AVGBP) and the Low Sustained Limit (LSL). The calculation of EFAIEC shall follow the methodology as described in Section 4.6.5, Calculation of “Average Incremental Energy Cost” (AIEC), for Resource r at Resource Node p represented by QSE q. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| ADMOCPR q, p, r, i | $/MWh | Adjusted Mitigated Offer Cap Price per QSE per Settlement Interval per Generation Resource—The mitigated offer curve price corresponding to the Average Base Point (AVGBP) adjusted by the verified actual fuel cost for the Generation Resource r at Resource Node p represented by QSE q over the 15-minute Settlement Interval i. Notwithstanding the above, the multiplier, pursuant to item (e) of Section 4.4.9.4.1, Mitigated Offer Cap, shall be set to 1 for all capacity factors. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTSPP p, i | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource Node p for the Settlement Interval i. |
| RTMG q, r, p, i | MWh | Real-Time Metered Generation per QSE per Settlement Point per Generation Resource— The metered generation, of Resource r, at Resource Node p represented by QSE q in Real-Time for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| AVGBP q, r, p, i | MW | Average Base Point per QSE per Settlement Point per Generation Resource—The average of the five-minute clock interval Base Points (pursuant to Section 6.6.5.1, Resource Base Point Deviation Charge) over the 15-minute Settlement Interval i for Generation Resource r represented by QSE q at Settlement Point p. |
| EBPWAPR q, r, p,i | $/MWh | Emergency Base Point Weighted Average Price per QSE per Settlement Point per Resource—The weighted average of the energy prices corresponding with the Emergency Base Points on the Energy Offer Curve for Resource r at Resource Node p represented by QSE q, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. The EBPWAPR used in EFCPR above shall be the price calculated for Resources that receive manual overrides and qualify for additional payments per paragraph (4) of Section 6.6.9, Emergency Operations Settlement. |
| LSL q,r,i | MW | Low Sustained Limit Generation per QSE per Settlement Point per Resource—The LSL of Generation Resource r represented by QSE q at Resource Node p for the hour that includes the 15-minute Settlement Interval. Where for a combined cycle resource r is a Combined Cycle Generation Resource. |
| q | none | A QSE. |
| r | none | A Generation Resource. |
| p | none | A Resource Node Settlement Point. |
| i | none | A 15-minute Settlement Interval. |

(2)	The total compensation to each QSE for Real-Time Make-Whole Payment for exceptional fuel costs for the 15-minute Settlement Interval is calculated as follows:

EFCMWAMTQSETOT q, i   =   EFCMWAMT q, p, r, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| EFCMWAMT q, p, r, i | $ | Exceptional Fuel Cost Make-Whole Payment Amount per QSE per Settlement Point per Generation Resource—The exceptional fuel cost Make-Whole Payment to QSE q at Resource Node p as a result of Generation Resource r receiving a Base Point at the Mitigated Offer Cap during a 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| EFCMWAMTQSETOT q, i | $ | Exceptional Fuel Cost Make-Whole Payment Amount per QSE—The total payment to QSE q for Real-Time exceptional fuel cost Make-Whole Payment during a 15-minute Settlement Interval i. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | A Generation Resource. |
| i | none | A 15-minute Settlement Interval. |

[NPRR664: Replace Section 6.6.3.7 above with the following upon system implementation:] - 6.6.3.7 Real-Time Make-Whole Payment for Exceptional Fuel Cost (1) Natural Gas or Fuel Oil Generation Resources that have approved verifiable costs and received at least one Base Point on their Mitigated Offer Cap during a 15-minute Settlement Interval may be eligible to receive a Real-Time Make-Whole Payment for exceptional fuel cost. ERCOT shall deem a Generation Resource eligible for this payment if the QSE for the Resource submits the actual price paid for delivered fuel for the specific Resource and it is greater than the sum of the fuel price (e.g. Fuel Index Price for Resource (FIPRr), Fuel Oil Price (FOP)) used by ERCOT for the Resource, the fuel adder, and a threshold fuel price, as defined in the Verifiable Cost Manual. For eligible Resources, calculation for Real-Time Make-Whole Payment for exceptional fuel costs shall be determined as follows: EFCMWAMT q, p, r, i = (-1) * EFCPR q, p, r, i * EFCQTY q, p, r, i Where: EFCPR q, p, r, i = Max[0, (Min(EFAIEC q, p, r, i , ADMOCPR q, p, r, i ) - RTSPP p, i ) - EBPWAPR q, r, p] And: EFCQTY q, p, r, i = Min(AVGBP q, r, p,i * ¼, RTMG q, r, p, i ) The above variables are defined as follows: Variable; Unit; Definition. EFCMWAMT q, p, r, i; $; Exceptional Fuel Cost Make-Whole Payment Amount per QSE per Settlement Point per Generation Resource—The exceptional fuel cost Make-Whole Payment to QSE q at Resource Node p as a result of Generation Resource r receiving a Base Point at the Mitigated Offer Cap during a 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. EFCPR q, p, r, i; $/MWh; Exceptional Fuel Cost Base Point Average Price per QSE per Settlement Point per Generation Resource—The price used to compensate an eligible Generation Resource r at Resource Node p represented by QSE q, for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. EFCQTY q, p, r, i; MWh; Exceptional Fuel Cost Energy per QSE per Settlement Point per Generation Resource—The quantity used to compensate an eligible Generation Resource r at Resource Node p represented by QSE q in Real-Time during an exceptional fuel cost event, for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. EFAIEC q, p, r, i; $/MWh; Exceptional Fuel Cost Average Incremental Energy Cost per QSE per Settlement Interval per Generation Resource— The average incremental energy cost calculated according to the Energy Offer Curve (not subject to the cost cap) for the corresponding output levels between the Average Base Point (AVGBP) and the Low Sustained Limit (LSL). The calculation of EFAIEC shall follow the methodology as described in Section 4.6.5, Calculation of “Average Incremental Energy Cost” (AIEC), for Resource r at Resource Node p represented by QSE q. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train.. ADMOCPR q, p, r, i; $/MWh; Adjusted Mitigated Offer Cap Price per QSE per Settlement Interval per Generation Resource—The mitigated offer curve price corresponding to the Average Base Point (AVGBP) adjusted by the verified actual fuel cost for the Generation Resource r at Resource Node p represented by QSE q over the 15-minute Settlement Interval i. Notwithstanding the above, the multiplier, pursuant to item (e) of Section 4.4.9.4.1, Mitigated Offer Cap, shall be set to 1 for all capacity factors. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train.. RTSPP p, i; $/MWh; Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource Node p for the Settlement Interval i.. RTMG q, r, p, i; MWh; Real-Time Metered Generation per QSE per Settlement Point per Generation Resource—The metered generation, of Resource r, at Resource Node p represented by QSE q in Real-Time for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. AVGBP q, r, p, i; MW; Average Base Point per QSE per Settlement Point per Generation Resource—The average of the five-minute clock interval Base Points (pursuant to Section 6.6.5.1, Resource Base Point Deviation Charge) over the 15-minute Settlement Interval i for Generation Resource r represented by QSE q at Settlement Point p.. EBPWAPR q, r, p,i; $/MWh; Emergency Base Point Weighted Average Price per QSE per Settlement Point per Resource—The weighted average of the energy prices corresponding with the Emergency Base Points on the Energy Offer Curve for Resource r at Resource Node p represented by QSE q, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. The EBPWAPR used in EFCPR above shall be the price calculated for Resources that receive manual overrides and qualify for additional payments per paragraph (4) of Section 6.6.9, Emergency Operations Settlement.. LSL q,r,i; MW; Low Sustained Limit Generation per QSE per Settlement Point per Resource—The LSL of Generation Resource r represented by QSE q at Resource Node p for the hour that includes the 15-minute Settlement Interval. Where for a combined cycle resource r is a Combined Cycle Generation Resource.. q; none; A QSE.. r; none; A Generation Resource.. p; none; A Resource Node Settlement Point.. i; none; A 15-minute Settlement Interval.. (2) The total compensation to each QSE for Real-Time Make-Whole Payment for exceptional fuel costs for the 15-minute Settlement Interval is calculated as follows: EFCMWAMTQSETOT q, i = EFCMWAMT q, p, r, i The above variables are defined as follows: Variable; Unit; Definition. EFCMWAMT q, p, r, i; $; Exceptional Fuel Cost Make-Whole Payment Amount per QSE per Settlement Point per Generation Resource—The exceptional fuel cost Make-Whole Payment to QSE q at Resource Node p as a result of Generation Resource r receiving a Base Point at the Mitigated Offer Cap during a 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. EFCMWAMTQSETOT q, i; $; Exceptional Fuel Cost Make-Whole Payment Amount per QSE—The total payment to QSE q for Real-Time exceptional fuel cost Make-Whole Payment during a 15-minute Settlement Interval i.. q; none; A QSE.. p; none; A Resource Node Settlement Point.. r; none; A Generation Resource.. i; none; A 15-minute Settlement Interval..

- 6.6.3.8	Real-Time Make-Whole Charge for Exceptional Fuel Cost

(1)	ERCOT shall charge each QSE representing Load Serving Entities (LSEs) the total payment for exceptional fuel costs as specified in Section 6.6.3.7, Real-Time Make-Whole Payment for Exceptional Fuel Cost, based on LRS. The charge to each QSE for a given 15-minute Settlement Interval is calculated as follows:

LAEFCAMT q, i  	=	(-1) * EFCMWAMTTOT i * LRS q, i

Where:

EFCMWAMTTOT i  	=	 EFCMWAMTQSETOT q, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LAEFCAMT q, i | $ | Load- Allocated Exceptional Fuel Cost Amount per QSE—The charge to QSE q for Real-Time exceptional fuel cost Make-Whole Payments, for the 15-minute Settlement Interval i. |
| EFCMWAMTTOT i | $ | Exceptional Fuel Cost Make-Whole Payment Amount —The total payment to all QSEs for Real-Time exceptional fuel cost Make-Whole Payment during a 15-minute Settlement Interval i. |
| EFCMWAMTQSETOT q, i | $ | Exceptional Fuel Cost Make-Whole Payment Amount per QSE —The total payment to QSE q for Real-Time exceptional fuel cost Make-Whole Payment during a 15-minute Settlement Interval i. |
| LRS q, i | none | The Load Ratio Share calculated for QSE q for the 15-minute Settlement Interval i. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. |
| q | none | A QSE. |
| i | none | A 15-minute Settlement Interval. |

The above variables are defined as follows:

9.5.3	Real-Time Market Settlement Charge Types

(1)	ERCOT shall provide, on each RTM Settlement Statement, the dollar amount for each RTM Settlement charge and payment.  The RTM Settlement “Charge Types” are:

(a)	Section 5.7.1, RUC Make-Whole Payment;

(b)	Section 5.7.2, RUC Clawback Charge;

(c)	Section 5.7.3, Payment When ERCOT Decommits a QSE-Committed Resource;

(d)	Section 5.7.4.1, RUC Capacity-Short Charge;

(e)	Section 5.7.4.2, RUC Make-Whole Uplift Charge;

(f)	Section 5.7.5, RUC Clawback Payment;

(g)	Section 5.7.6, RUC Decommitment Charge;

(h)	Section 6.6.3.1, Real-Time Energy Imbalance Payment or Charge at a Resource Node;

(i)	Section 6.6.3.2, Real-Time Energy Imbalance Payment or Charge at a Load Zone;

(j)	Section 6.6.3.3, Real-Time Energy Imbalance Payment or Charge at a Hub;

(k)	Section 6.6.3.4, Real-Time Energy Payment for DC Tie Import;

(l)	Section 6.6.3.5, Real-Time Payment for a Block Load Transfer Point;

(m)	Section 6.6.3.6, Real-Time Energy Charge for DC Tie Export Represented by the QSE Under the Oklaunion Exemption;

(n)	Section 6.6.3.7, Real-Time Make-Whole Payment for Exceptional Fuel Cost;

(o)	Section 6.6.3.8, Real-Time Make-Whole Charge for Exceptional Fuel Cost;

(p)	Section 6.6.4, Real-Time Congestion Payment or Charge for Self-Schedules;

(q)	Section 6.6.5.1.1.1, Base Point Deviation Charge for Over Generation;

(r)	Section 6.6.5.1.1.2, Base Point Deviation Charge for Under Generation;

(s)	Section 6.6.5.2, IRR Generation Resource Base Point Deviation Charge;

(t)	Section 6.6.5.4, Base Point Deviation Payment;

(u)	Section 6.6.6.1, RMR Standby Payment;

(v)	Section 6.6.6.2, RMR Payment for Energy;

(w)	Section 6.6.6.3, RMR Adjustment Charge;

(x)	Section 6.6.6.4, RMR Charge for Unexcused Misconduct;

(y)	Section 6.6.6.5, RMR Service Charge;

(z)	Paragraph (2) of Section 6.6.7.1, Voltage Support Service Payments;

(aa)	Paragraph (4) of Section 6.6.7.1;

(bb)	Section 6.6.7.2, Voltage Support Charge;

(cc)	Section 6.6.8.1, Black Start Hourly Standby Fee Payment;

(dd)	Section 6.6.8.2, Black Start Capacity Charge;

(ee)	Section 6.6.9.1, Payment for Emergency Power Increase Directed by ERCOT;

(ff)	Section 6.6.9.2, Charge for Emergency Power Increases;

(gg)	Section 6.6.10, Real-Time Revenue Neutrality Allocation;

(hh)	Paragraph (a) of Section 6.7.1, Payments for Ancillary Service Capacity Sold in a Supplemental Ancillary Service Market;

(ii)	Paragraph (b) of Section 6.7.1;

(jj)	Paragraph (c) of Section 6.7.1;

(kk)	Paragraph (d) of Section 6.7.1;

(ll)	Paragraph (a) of Section 6.7.2, Charges for Ancillary Service Capacity Replaced Due to Failure to Provide;

(mm)	Paragraph (b) of Section 6.7.2;

(nn)	Paragraph (c) of Section 6.7.2;

(oo)	Paragraph (d) of Section 6.7.2;

(pp)	Paragraph (2) of Section 6.7.3, Adjustments to Cost Allocations for Ancillary Services Procurement;

(qq)	Paragraph (3) of Section 6.7.3;

(rr)	Paragraph (4) of Section 6.7.3;

(ss)	Paragraph (5) of Section 6.7.3;

(tt)	Paragraph (7) of Section 6.7.4, Real-Time Ancillary Service Imbalance Payment or Charge (Real-Time Ancillary Service Imbalance Amount);

(uu)	Paragraph (7) of Section 6.7.4, (Real-Time Reliability Deployment Ancillary Service Imbalance Amount);

(vv) 	Paragraph (8) of Section 6.7.4, (Real-Time RUC Ancillary Service Reserve Amount);

(ww) 	Paragraph (8) of Section 6.7.4, (Real-Time Reliability Deployment RUC Ancillary Service Reserve Amount);

(xx) 	Section 6.7.5, Real Time Ancillary Service Imbalance Revenue Neutrality Allocation (Load-Allocated Ancillary Service Imbalance Revenue Neutrality Amount);

(yy) 	Section 6.7.5, (Load-Allocated Reliability Deployment Ancillary Service Imbalance Revenue Neutrality Amount);

(zz)	Section 7.9.2.1, Payments and Charges for PTP Obligations Settled in Real-Time;

(aaa)	Paragraph (3) of Section 7.9.3.3, Shortfall Charges to CRR Owners; and

(bbb)	Section 9.16.1, ERCOT System Administration Fee.

(2)	In the event that ERCOT is unable to execute the Day-Ahead Market (DAM), ERCOT shall provide, on each RTM Settlement Statement, the dollar amount for the following RTM Congestion Revenue Right (CRR) Settlement charges and payments:

(a)	Section 7.9.2.4, Payments for FGRs in Real-Time; and

(b)	Section 7.9.2.5, Payments and Charges for PTP Obligations with Refund in Real-Time.