| NPRR Number | 727 | NPRR Title | Removal of Language Related to NPRR327, State Estimator Data Redaction Methodology |
| --- | --- | --- | --- |
| Date |  | October 8, 2015 |  |
| Submitter’s Information |  |  |  |
| Name |  | Chad Seely / Nathan Bigbee |  |
| E-mail Address |  | Chad.Seely@ercot.com / Nathan.Bigbee@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 512.225.7035 / 512.225.7093 |  |
| Cell Number |  |  |  |
| Market Segment |  | Not applicable. |  |

Comments

At the September 24, 2015 open meeting of the Public Utility Commission of Texas (PUCT), the Commissioners determined, without taking formal action, that the redacted State Estimator report potentially required by PUC Substantive Rule 25.505(f)(3)(E)(ii) is not cost-effective and is therefore not required to be developed under the provisions of that rule.  Based on this conclusion, the Commissioners instructed ERCOT to remove the grey-boxed language associated with NPRR327, State Estimator Data Redaction Methodology, which had previously been approved by the ERCOT Board of Directors in anticipation of a need to develop the redacted reports.  Given the Commission’s guidance on these issues, ERCOT supports the approval of NPRR727 to remove the grey-boxed language associated with NPRR327.  ERCOT has also terminated all efforts to develop the redacted State Estimator report.

ERCOT has determined that the Commission’s decision also requires deletion of existing paragraph (7) of Section 6.5.7.1.13, Data Inputs and Outputs for the Real-Time Sequence and SCED, which, like the grey-boxed version of paragraph (7), requires posting of the redacted State Estimator report.  Existing paragraph (7) was introduced as part of NPRR209, Data Posting Changes to Comply with PUC Substantive Rule 25.505, following the PUC’s 2010 rulemaking that created the requirement to post the redacted State Estimator report (See PUC Project 38470, Confidentiality of Electric Generation Information in the Electric Reliability Council of Texas Region).   NPRR327 simply replaced paragraph (7) with new language providing more detail on the redaction methodology, subject to system implementation.  ERCOT thus proposes to delete both the existing and grey-boxed versions of the report-posting requirement.

Revised Cover Page Language

None at this time.

Revised Proposed Protocol Language

- 6.5.7.1.13	Data Inputs and Outputs for the Real-Time Sequence and SCED

(1)	Inputs:  The following information must be provided as inputs to the Real-Time Sequence and SCED.  ERCOT may require additional information as required, including:

(a)	Real-Time data from TSPs including status indication for each point if that data element is stale for more than 20 seconds;

(i)	Transmission Electrical Bus voltages;

(ii)	MW and MVAr pairs for all transmission lines, transformers, and reactors;

(iii)	Actual breaker and switch status for all modeled devices; and

(iv)	Tap position for auto-transformers;

(b)	State Estimator results (MW and MVAr pairs and calculated MVA) for all modeled Transmission Elements;

(c)	Transmission Element ratings from TSPs;

(i)	Data from the Network Operations Model:

(A)	Transmission lines – Normal, Emergency, and 15-Minute Ratings (MVA); and

(B)	Transformers and Auto-transformers – Normal, Emergency, and 15-Minute Ratings (MVA) and tap position limits;

(ii)	Data from QSEs:

(A)	Generator Step-Up (GSU) transformers tap position;

(B)	Resource HSL (from telemetry); and

(C)	Resource LSL (from telemetry); and

(d)	Real-Time weather, from WGRs, and where available from TSPs or other sources.  ERCOT may elect to obtain other sources of weather data and may utilize such information to calculate the dynamic limit of any Transmission Element.

(2)	ERCOT shall validate the inputs of the Resource Limit Calculator as follows:

(a)	The calculated SURAMP and SDRAMP are each greater than or equal to zero; and

(b)	Other provision specified under Section 3.18, Resource Limits in Providing Ancillary Service.

(3)	Outputs for ERCOT Operator information and possible action include:

(a)	Operator notification of any change in status of any breaker or switch;

(b)	Lists of all breakers and switches not in their normal position;

(c)	Operator notification of all Transmission Element overloads detected from telemetered or State-Estimated data;

(d)	Operator notification of all Transmission Element security violations; and

(e)	Operator summary displays:

(i)	Transmission system status changes;

(ii)	Overloads;

(iii)	System security violations; and

(iv)	Base Points.

(4)	Every hour, ERCOT shall post on the MIS Secure Area the following information:

(a)	Status of all breakers and switches used in the NSA except breakers and switches connecting Resources to the ERCOT Transmission Grid;

(b)	All binding transmission constraints and the contingency or overloaded element pairs that caused such constraint; and

(c)	Shift Factors by Resource Node.

(5)	Sixty days after the applicable Operating Day, ERCOT shall post on the MIS Secure Area, the following information:

(a)	Hourly transmission line flows and voltages from the State Estimator, excluding transmission line flows and voltages for Private Use Networks; and

(b)	Hourly transformer flows, voltages and tap positions from the State Estimator, excluding transformer flows, voltages, and tap positions for Private Use Networks.

(6)	Notwithstanding paragraph (5) above, ERCOT, in its sole discretion, shall release relevant State Estimator data less than 60 days after the Operating Day if it determines the release is necessary to provide complete and timely explanation and analysis of unexpected market operations and results or system events including, but not limited to, pricing anomalies, recurring transmission congestion, and system disturbances.  ERCOT’s release of data under this paragraph shall be limited to intervals associated with the unexpected market or system event as determined by ERCOT.  The data release shall be made available simultaneously to all Market Participants.

(7)	Every hour, ERCOT shall post on the MIS Public Area, the sum of ERCOT generation, and flow on the DC Ties, all from the State Estimator.

(8)	After every SCED run, ERCOT shall post to the MIS Public Area the sum of the HDL and the sum of the LDL for all Generation Resources On-Line and Dispatched by SCED.

(9)	Sixty days after the applicable Operating Day, ERCOT shall provide the summary LDL and HDL report from paragraph (8) above and include, for any Generation Resource, instances of manual overrides of HDL or LDL, including the name of the Generation Resource and the type of override.

(10)	After every SCED run, ERCOT shall post to the MIS Certified Area, for any QSE, instances of a manual override of the HDL or LDL for a Generation Resource, including the original and overridden HDL or LDL.