ERCOT Load Profiling Guide

Section 8:  Load Profile Models

Draft

8	LOAD PROFILE MODELS	8-1

8.1	Routine and Non-Routine Load Profile Model Evaluations	8-1

8.1.1	Routine Evaluation of Load Profile Model Performance	8-1

8.1.2	Non-Routine Evaluation of Model Performance	8-2

8.2	Evaluation of Load Profile Models Using Current Load Research Data	8-2

8.2.1	Sources of Load Research Data	8-2

8.2.2	Procedures	8-2

8.2.3	Using Comparable Weather Zone Data	8-3

8.2.4	Factors Considered in Comparisons	8-3

8.3	Evaluating Load Profile Models without Current Load Research Data	8-5

8.3.1	 Applications	8-5

8.3.2	 Load Profile Model Comparisons	8-5

8.4	Routine Load Profile Model Evaluations	8-7

8.4.1	 Routine Evaluation of Weather Zones	8-8

8.5	Non-Routine Load Profile Model Evaluations	8-9

8.6	Assessing the Type of Load Profile Model Change Needed	8-9

8.6.1	 Possible Changes	8-9

8.6.2	 Qualitative Criteria	8-9

8.7	Criteria for Requiring a Load Profile Model Change	8-12

8.8	Procedures for Requesting a Change to Load Profile Models	8-13

8.8.1	 Request for Load Profile Model Changes	8-13

8.8.2	 General Information Required with a Request	8-14

8.8.3	 Requesting Load Profile Model Adjustment Factors	8-14

8.8.4	 Requesting Change to Engineering Estimates	8-14

8.8.5	 Requesting Re-Estimation of Models	8-15

8.9	Approval Process for Load Profile Model Changes	8-15

8.9.1	 Timeline Prior to Implementing a Load Profile Change	8-15

8.9.2	 Adjusted Static Models	8-15

8.9.3	 Engineering Estimates	8-16

# LOAD PROFILE MODELS

(1)	Protocol Section 18.2.5, Adjustments and Changes to Load Profile Development, defines  evaluation of Load Profiling Methodology  including changes to methodology, adjustments to existing profiles, and development of new profiles.  This Section addresses changes to models within approved methodologies.    ar

(2)	The Microsoft Excel© representation of the ERCOT Load Profile Models can be found in Appendix E, Load Profile Model Spreadsheets.

(3)	There shall be no retroactive application of any approved modifications to Load Profile Models.

(4)	This Section discusses changes to Load Profile Models not addressed in the following the Load Profiling Guide (LPG) sections:

(a)	Section 7, Request for Changes to Load Profiling Methodology;

(b)	Section 12, Request for Load Profile Segment Changes, Additions, or Removals; and

(c)	Section 13, Changes to Weather Zone Definitions.

8.1	Load Profile Model Evaluations

8.1.1	Upon request from the appropriate TAC Subcommittee, ERCOT shall perform evaluations of Load Profile Model performance for all Load Profile Models and Load Profile Types,using industry standard practices as described in The Association of Edison Illuminating Companies (AEIC) Load Research Manual..  Any changes resulting from these evaluations shall be recommended to the Profiling Working Group (PWG). Ea

(	F(2)	For each adjusted static Load Profile Type, consider whether any current data are available that would indicate substantial changes in end-use saturation between current populations and those used to fit the models.

)	(a)	Possible sources of data on operating schedules and equipment saturations include:

gi by the Energy Information Administration;i)	Regional or state data on operating practices published by the Census Bureau;

(i	Saturation or other studies by Market Participants, if available.

(b)have changed substantially in the region since the Load Profile Mod)	Review the magnitude of Load migrated into and out of each Load Profiling segment since the time the Load research data were collected.

(6)	If Unaccounted for Energy (UFE) is calculated by Weather Zone or other geographic subdivision, examine systematic patterns in UFE by day-type and hour for each such zone or region.

1	Ea

Assessment of Weather Zone definition shall focus on the adequacy of the current set of weather stations and weighting.  ERCOT uses National Oceanic and Atmospheric Administration (NOAA) first or second order weather stations as the source  for weather data for each Weather Zone, wher available.  Weather Zone evaluation willingindustry standard practices as described in The Association of Edison Illuminating Companies (AEIC) Load Research Manual. Assessment steps of the evaluation of each Weat

(b)	Calculate weather station segment Load Profiles.  Apply each Load Profile Segment model to weather data from each weather station, using the model coefficients for the Weather Zone that includes that weather station;

e(d)	For each weather station and adjusted static segment, calculate the total annual energy for Electric Service Identifiers (ESI IDs) in ZIP codes assigned to the station;

)	(f)	Sum the results of item (e) above over all weather stations within each Weather Zone;

an(h)	For each Weather Zone and segment, compare the summed Load Profile from item (f) above with the current Load Profile Model from item (a) above, using the methods described in Appendix C, Measuring Differences Between Load Profiles.

r 8.2	Evaluatingon of Load Profile Models Using Current Load Research Data

s

(1)	Load research data may be  collected by ERCOT developed Linterval data and from any available Transmission and/or Distribution Service Provider’s (TDSP) Load research samples.

(2)	In certain circumstances, Load research data from other sources may also be considered by ERCOT as representative of a particular subgroup.  For such data to be used, the party submitting the data for use in an evaluation shall provide information on the source of the data.  Submission requirements are the same as those described in Section 12.6, Information Required with Request for Change.

8.2.2	Procedures

The overall procedure for comparing existing Load Profile Models against current Load research data shall use industry standard practices as described in The Association of Edison Illuminating Companies (AEIC) Load Research Manual.

referenced in the Association

Bf

8.3	Evaluating Load Profile Models without Current Load Research Data

8.3.1		Applications

(1)	In many situations, current Load research data are not available as a basis for assessing the performance of Load Profile Models.  Situations where techniques are required that do not depend on Load research data include:

(a)	Assessing model performance for geographic areas where Load research data are no longer collected;

(b)	Assessing model performance for geographic areas where Load research data have never been collected, or have not contributed to current models; and

(c)	Assessing Engineering Estimates.

(2)	These techniques may also be used as another way of assessing model performance even for geographic areas where current Load research data are available.

8.3.2		Load Profile Model Comparisons

8.3.2.1	Comparisons for Adjusted Static Models

(1)	For premises with a Advanced Meter installed, Adjusted Static Models may be assessed based on techniques described in The Association of Edison Illuminating Companies (AEIC) Load Research Manual.populati

8.3.2.2	Examination of Monthly Patterns

(1)	For premises with no Advanced Meter installed, monthly consumption data are available to ERCOT for evaluation.

8.3.2.3	Comparisons for Engineering Estimates

(1)	Engineering Estimates are used in the ERCOT market for Non-Metered Loads, such as lighting.  Engineering Estimates are typically based on an assumed fixed operating schedule together with the assumption that the Load is approximately the same whenever the equipment is operating.  If more current information is available for the ESI IDs in a Load Profile Segment using an engineering Load Profile, this information may be compared with the assumptions of the estimate.

(2)	Monthly consumption data may also be compared with the Load Profile monthly patterns using the methods described above for Adjusted Static Models.

6

6

6

6

6

6

.

.

.

.

.

.

8.4	Criteria for Requiring a Load Profile Model Change

(1)		Upon request from the appropriate TAC Subcommittee, ERCOT will evaluate Load Profile Models using industry standard practices as described in The Association of Edison Illuminating Companies (AEIC) Load Research Manual.

8.5	Procedures for Requesting a Change to Load Profile Models

(1)	This Section describes the procedures for requesting changes to Load Profile Models.  Procedures for requesting changes to Load Profile Segments are described in Section 12, Request for Load Profile Segment Changes, Additions, or Removals.  Procedures for requesting changes to Weather Zones are described in Section 13, Changes to Weather Zone Definitions.

8.5.1		Request for Load Profile Model Changes

(1)	The following Entities may submit requests for Load Profile Model changes:

(a)	Any Market Participant;

(b)	Any Entity that is an ERCOT Member;

(c)	Public Utility Commission of Texas (PUCT) Staff;

(d)	ERCOT Staff; and

(e)	Any other Entity who resides (or represent residents) in Texas or operates in the Texas electricity market.

(2)	Requests for Load Profile Model changes shall be submitted to the Profiling Working Group (PWG) and are subject to approval as outlined in Section 8.6.

8.5.2		General Information Required with a Request

(1)	Requests for Load Profile Model Changes shall include the following:

(a)	Identifying the party making the request, with contact information;

(b)	Identifying the Load Profile Segment(s) and Weather Zone(s) affected; and

(c)	Describe why the evaluation is needed.

8.6	Approval Process for Load Profile Model Changes

(1)	For any proposed Load Profile Model changes, PWG shall recommend the proposed changes to COPS with subsequent approval by TAC.  RMS may be asked to review the proposed changes as well.

(2)	Load Profile Model changes shall be accompanied by an implementation plan to mitigate the impact of transitioning between old and new Load Profile Models.  The implementation plan shall be approved by COPS and TAC and may be reviewed by RMS.