| NPRR Number | 741 | NPRR Title | Clarifications to TPE and EAL Credit Exposure Calculations |
| --- | --- | --- | --- |
| Date |  | January XX, 2016 |  |
| Submitter’s Information |  |  |  |
| Name |  | XXX on behalf of the Market Credit Working Group (MCWG) |  |
| E-mail Address |  |  |  |
| Company |  |  |  |
| Phone Number |  |  |  |
| Cell Number |  |  |  |
| Market Segment |  | Not applicable |  |

Comments

On January 14, 2016, MCWG reviewed NPRR741.  MCWG was in consensus to endorse NPRR741 as revised by MCWG.

Please note that baseline Protocol language in Section 16.11.4.5 has been updated to reflect the January 1, 2016 incorporation of NPRR728 into the Protocols.

Proposed Protocol Language Revision

16.11.4.1	Determination of Total Potential Exposure for a Counter-Party

(1)	A Counter-Party’s TPE is the sum of its “Total Potential Exposure Any” (TPEA) and TPES:

(a)	TPEA is the positive net exposure of the Counter-Party that may be satisfied by any forms of Financial Security defined under paragraphs (a) through (d) of Section 16.11.3, Alternative Means of Satisfying ERCOT Creditworthiness Requirements.  TPEA will include all exposure not included in TPES.

(b)	TPES is the positive net exposure of the Counter-Party that may be satisfied only by forms of Financial Security defined under paragraphs (b) through (d) of Section 16.11.3.  The FCE that reflects the future mark-to-market value for CRRs registered in the name of the Counter-Party is included in TPES.

(2)	For all Counter-Parties:

TPEA 	= 	Max [0, MCE, Max [0, (EAL q + EAL a)]] + PUL

TPES	=	Max [0, FCE a] + IA

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| EAL q | $ | Estimated Aggregate Liability for all QSEs—EAL for all QSEs represented by the Counter-Party. |
| EAL a | $ | Estimated Aggregate Liability for all CRR Account Holders—EAL for all CRR Account Holders represented by the Counter-Party. |
| PUL | $ | Potential Uplift—Potential uplift to the Counter-Party, to the extent and in the proportion that the Counter-Party represents Entities to which an uplift of a short payment will be made pursuant to Section 9.19, Partial Payments by Invoice Recipients. It is calculated as the sum of: (a) Amounts expected to be uplifted within one year of the date of the calculation; and (b) 25%, or such other percentage based on available statistics regarding payment default under bankruptcy reorganization plans, of any short payment amounts being repaid to ERCOT under a bankruptcy reorganization plan that are due more than one year from the date of the calculation. |
| FCE a | $ | Future Credit Exposure for all CRR Account Holders—FCE for all CRR Account Holders represented by the Counter-Party. |
| MCE | $ | Minimum Current Exposure—For each Counter-Party, ERCOT shall determine a Minimum Current Exposure (MCE) as follows: MCE = Max[0, { [[[L i, od, p * T2 - G i, od, p * (1-NUCADJ ) * T3] * RTSPP i, od, p *SAF] + [RTQQNET i, od, p * T5]]/n}, { [G i, od, p * NUCADJ * T1 * RTSPP i, od, p * SAF]/n}, { ( (DARTNET i, od, p)) * SAF * T4/n}] RTQQNET i, od, p = Σc Max[Σc (RTQQES i, od, p, c - RTQQEP i, od, p, c), BTCF * Σc (RTQQES i, od, p, c – RTQQEP i, od, p, c)] * RTSPP i, od, p * SAF DARTNET i, od, p = DAM EOO Cleared i, od, p * DART i, od, p + DAM TPO Cleared i, od, p * DART i, od, p + DAM PTP Cleared i, od, p * DARTPTP i, od, p – DAM EOB Cleared i, od, p * DART i, od, p - Where: - G i, od, p = Total Metered Generation at all Resource Nodes for the Counter-Party for interval i for Operating Day od at Settlement Point p L i, od, p = Total Adjusted Metered Load (AML) at all Load Zones for the Counter-Party for interval i for Operating Day od at Settlement Point p SAF = Seasonal Adjustment Factor—Used to provide for the potential for Seasonal price increases based on historical trends. ERCOT shall initially set this factor equal to 100%. This factor will not go below 100%. ERCOT will provide Notice to Market Participants of any change at least 14 days prior to effective date along with the analysis supporting the change. NUCADJ = Net Unit Contingent Adjustment—To allow for situations where a generator may unintentionally or intentionally meet its requirement from the Real-Time Market (RTM). RTQQNET i, od, p = Net QSE-to-QSE Energy Trades for the Counter-Party for interval i for Operating Day od at Settlement Point p RTQQES i, od, p, c = QSE Energy Trades for which the Counter-Party is the seller for interval i for Operating Day od at Settlement Point p with Counter-Party c RTQQEP i, od, p, c = QSE Energy Trades for which the Counter-Party is the buyer for interval i for Operating Day od at Settlement Point p with Counter-Party c BTCF = Bilateral Trades Credit Factor RTSPP i, od, p = Real-Time Settlement Point Price for interval i for Operating Day od at Settlement Point p DARTNET i, od, p = Net DAM activities for the Counter-Party for interval i for Operating Day od at Settlement Point p DART i, od, p = Day Ahead - Real-Time Spread for interval i for Operating Day d at Settlement Point p DAM EOB Cleared i, od, p = DAM Energy Only Bids Cleared for interval i for Operating Day od at Settlement Point p DAM EOO Cleared i, od, p = DAM Energy Only Offers Cleared for interval i for Operating Day od at Settlement Point p DAM TPO Cleared i, od, p = DAM Three-Part Offers Cleared for interval i for Operating Day od at Settlement Point p DAM PTP Cleared i, od, p = DAM Point-to-Point (PTP) Obligations Cleared for interval i for Operating Day od at Settlement Point p DARTPTP i, od, p = Day Ahead - Real-Time Spread for value of PTP Obligation for interval i for Operating Day od at Settlement Point p c = Bilateral Counter-Party od = Operating Day e = Most recent n Operating Days for which RTM Initial Settlement Statements are available i = Settlement Interval n = Days used for averaging p = A Settlement Point |
| q | None. | QSEs represented by the Counter-Party. |
| a | None. | CRR Account Holders represented by the Counter-Party. |
| IA | $ | Independent Amount—The amount required to be posted as defined in Section 16.16.1, Counter-Party Criteria. |

The above parameters are defined as follows.

| Parameter | Unit | Current Value* |
| --- | --- | --- |
| NUCADJ | Percentage | Minimum value of 20%. |
| T1 | Days | 2 |
| T2 | Days | 5 |
| T3 | Days | 5 |
| T4 | Days | 1 |
| T5 | Days | For a Counter-Party that represents Load this value is equal to 5, otherwise this value is equal to 2. |
| BTCF | Percentage | 80% |
| n | Days | 14 |
| * The current value for the parameters referenced in this table above will be recommended by TAC and approved by the ERCOT Board. ERCOT shall update parameter values on the first day of the month following ERCOT Board approval unless otherwise directed by the ERCOT Board. ERCOT shall provide a Market Notice prior to implementation of a revised parameter value. |  |  |

[NPRR620: Replace paragraph (2) above with the following upon system implementation:] (2) For all Counter-Parties: TPEA = Max [0, MCE, Max [0, ( (1-TOA) * EAL q + TOA * EAL t +EAL a)]] + PUL TPES = Max [0, FCE a] + IA The above variables are defined as follows: Variable; Unit; Description. EAL q; $; Estimated Aggregate Liability for all QSEs that represents Load or generation—EAL for all QSEs represented by the Counter-Party if at least one QSE represented by the Counter-Party represents either Load or generation.. EAL t; $; Estimated Aggregate Liability for all QSEs—EAL for all QSEs represented by the Counter-Party if none of the QSEs represented by the Counter-Party represent either Load or Generation.. EAL a; $; Estimated Aggregate Liability for all CRR Account Holders—EAL for all CRR Account Holders represented by the Counter-Party.. PUL; $; Potential Uplift—Potential uplift to the Counter-Party, to the extent and in the proportion that the Counter-Party represents Entities to which an uplift of a short payment will be made pursuant to Section 9.19, Partial Payments by Invoice Recipients. It is calculated as the sum of: (a) Amounts expected to be uplifted within one year of the date of the calculation; and (b) 25%, or such other percentage based on available statistics regarding payment default under bankruptcy reorganization plans, of any short payment amounts being repaid to ERCOT under a bankruptcy reorganization plan that are due more than one year from the date of the calculation.. FCE a; $; Future Credit Exposure for all CRR Account Holders—FCE for alltCRR Account Holders represented by the Counter-Party.. MCE; $; Minimum Current Exposure—For each Counter-Party, ERCOT shall determine a Minimum Current Exposure (MCE) as follows: MCE = Max[0, { [[[L i, od, p * T2 - G i, od, p * (1-NUCADJ) * T3] * RTSPP i, od, p *SAF] + [RTQQNET i, od, p * T5]]/n}, { [G i, od, p * NUCADJ * T1 * RTSPP i, od, p*SAF]/n}, { ( (DARTNET i, od, p)) * SAF * T4/n}kp, IMCE] RTQQNET i, od, p = Σc [Max[(RTQQES i, od, p, c - RTQQEP i, od, p, c), BTCF * (RTQQES i, od, p, c – RTQQEP i, od, p, c)] * RTSPP i, od, p *SAF] DARTNET i, od, p = DAM EOO Cleared i, od, p * DART i, od, p + DAM TPO Cleared i, od, p * DART i, od, p + DAM PTP Cleared i, od, p * DARTPTP i, od, p – DAM EOB Cleared i, od, p * DART i, od, p - Where: - G i, od, p = Total Metered Generation at all Resource Nodes for the Counter-Party for interval i for Operating Day od at Settlement Point p L i, od, p = Total Adjusted Metered Load (AML) at all Load Zones for the Counter-Party for interval i for Operating Day od at Settlement Point p SAF = Seasonal Adjustment Factor—Used to provide for the potential for Seasonal price increases based on historical trends. ERCOT shall initially set this factor equal to 100%. This factor will not go below 100%. ERCOT will provide Notice to Market Participants of any change at least 14 days prior to effective date along with the analysis supporting the change. NUCADJ = Net Unit Contingent Adjustment—To allow for situations where a generator may unintentionally or intentionally meet its requirement from the Real-Time Market (RTM). RTQQNET i, od, p = Net QSE-to-QSE Energy Trades for the Counter-Party for interval i for Operating Day od at Settlement Point p RTQQES i, od, p, c = QSE Energy Trades for which the Counter-Party is the seller for interval i for Operating Day od at Settlement Point p with Counter-Party c RTQQEP i, od, p, c = QSE Energy Trades for which the Counter-Party is the buyer for interval i for Operating Day od at Settlement Point p with Counter-Party c BTCF = Bilateral Trades Credit Factor RTSPP i, od, p = Real-Time Settlement Point Price for interval i for Operating Day od at Settlement Point p DARTNET i, od, p = Net DAM activities for the Counter-Party for interval i for Operating Day od at Settlement Point p DART i, od, p = Day-Ahead Real-Time Spread for interval i for Operating Day od at Settlement Point p DAM EOB Cleared i, od, p = DAM Energy Only Bids Cleared for interval i for Operating Day od at Settlement Point p DAM EOO Cleared i, od, p = DAM Energy Only Offers Cleared for interval i for Operating Day od at Settlement Point p DAM TPO Cleared i, od, p = DAM Three-Part Offers Cleared for interval i for Operating Day od at Settlement Point p DAM PTP Cleared i, od, p = DAM Point-to-Point (PTP) Obligations Cleared for interval i for Operating Day od at Settlement Point p DARTPTP i, od, p = Day-Ahead - Real-Time Spread for value of PTP Obligation for interval i for Operating Day od at Settlement Point p nm = Notional Multiplier cif = Cap Interval Factor - Represents the historic largest percentage of SWCAP intervals during a calendar day. c = Bilateral Counter-Party od = Operating Day e = Most recent n Operating Days for which RTM Initial Settlement Statements are available i = Settlement Interval n = Days used for averaging p = A Settlement Point. IMCE; $; Initial Minimum Current Exposure IMCE = TOA * (EFFCAP * nm * cif%) * SAF Where: EFFCAP = Effective Cap. The greater of Value of Lost Load (VOLL), as described in the Methodology for Implementing Operating Reserve Demand Curve (ORDC) to Calculate Real-Time Reserve Price Adder, or the System-Wide Offer Cap, as determined in accordance with PUCT Substantive Rules.. TOA; None; Trade-Only Activity — Counter-Party that does not represent either a Load or a generation QSE. Set to “0” if Counter-Party represents a QSE that has an association with a Load Serving Entity (LSE) or a Resource Entity, or if Counter-Party does not represent any QSE; otherwise set to 1.. q; None.; QSEs represented by Counter-Party.. a; None.; CRR Account Holders represented by Counter-Party.. IA; $; Independent Amount—The amount required to be posted as defined in Section 16.16.1, Counter-Party Criteria.. The above parameters are defined as follows. Parameter; Unit; Current Value*. nm; None; 50. cif; Percentage; 9%. NUCADJ; Percentage; Minimum value of 20%.. T1; Days; 2. T2; Days; 5. T3; Days; 5. T4; Days; 1. T5; Days; For a Counter-Party that represents Load this value is equal to 5, otherwise this value is equal to 2.. BTCF; Percentage; 80%. n; Days; 14. * The current value for the parameters referenced in this table above will be recommended by TAC and approved by the ERCOT Board. ERCOT shall update parameter values on the first day of the month following ERCOT Board approval unless otherwise directed by the ERCOT Board. ERCOT shall provide a Market Notice prior to implementation of a revised parameter value..

(3)	If ERCOT, in its sole discretion, determines that the TPEA or the TPES for a Counter-Party calculated under paragraphs (1) or (2) above does not adequately match the financial risk created by that Counter-Party’s activities under these Protocols, then ERCOT may set a different TPEA or TPES for that Counter-Party.  ERCOT shall, to the extent practical, give to the Counter-Party the information used to determine that different TPEA or TPES.  ERCOT shall provide written or electronic Notice to the Counter-Party of the basis for ERCOT’s assessment of the Counter-Party’s financial risk and the resulting creditworthiness requirements.

(4)	ERCOT shall monitor and calculate each Counter-Party’s TPEA and TPES daily.

16.11.4.2	Determination of Counter-Party Initial Estimated Liability

(1)	For each Counter-Party, ERCOT shall determine an IEL for purposes of Section 16.11.3, Alternative Means of Satisfying ERCOT Creditworthiness Requirements.

[NPRR620: Replace paragraph (1) above with the following upon system implementation:] (1) For each Counter-Party, except those Counter-Parties that represent neither Load nor generation or those Counter-Parties that are only CRR Account Holders, ERCOT shall determine an IEL for purposes of Section 16.11.3, Alternative Means of Satisfying ERCOT Creditworthiness Requirements.

(2)	For a Counter-Party that has all its QSEs representing only Load-Serving Entities (LSEs), ERCOT shall calculate the IEL using the following formula:

IEL = DEL * Max [0.2, RTEFL] * RTAEP * (M1 + M2)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| IEL | $ | Initial Estimated LiabilityThe Counter-Party’s Initial Estimated Liability. |
| DEL | MWh | Daily Estimated LoadThe Counter-Party’s estimated average daily Load as determined by ERCOT based on information provided by the Counter-Party. |
| RTEFL | none | Real-Time Energy Factor for LoadThe ratio of the Counter-Party’s estimated energy purchases in the RTM as determined by ERCOT based on information provided by the Counter-Party, to the Counter-Party’s Daily Estimated Load. |
| RTAEP | $/MWh | Real-Time Average Energy PriceAverage Settlement Point Price for the “ERCOT 345” as defined in Section 3.5.2.5, ERCOT Hub Average 345 kV Hub (ERCOT 345), based upon the previous seven days’ average Real-Time Settlement Point Prices. |

(3)	For a Counter-Party that has all its QSEs representing only Resources, ERCOT shall calculate the IEL using the following formula:

IEL	=	DEG * Max [0.2, RTEFG] * RTAEP * (M1 + M2)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| IEL | $ | Initial Estimated LiabilityThe Counter-Party’s Initial Estimated Liability. |
| DEG | MWh | Daily Estimated GenerationThe Counter-Party’s estimated average daily generation as determined by ERCOT based on information provided by the Counter-Party. |
| RTEFG | none | Real-Time Energy Factor for GenerationThe ratio of the Counter-Party’s estimated energy sales in the RTM as determined by ERCOT based on information provided by the Counter-Party, to the Counter-Party’s Daily Estimated Generation. |
| RTAEP | $/MWh | Real-Time Average Energy PriceAverage Settlement Point Price for the “ERCOT 345” as defined in Section 3.5.2.5 based upon the previous seven days average Real-Time Settlement Point Prices. |

(4)	For a Counter-Party that has QSEs representing both LSE and Resources, ERCOT shall calculate the Counter-Party’s IEL using the following formula:

IEL	=	DEL * Max [0.1, RTEFL] * RTAEP * (M1 + M2) + DEG * Max [0.1, RTEFG] * RTAEP * (M1 + M2)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| IEL | $ | Initial Estimated LiabilityThe Counter-Party’s Initial Estimated Liability. |
| DEL | MWh | Daily Estimated LoadThe Counter-Party’s estimated average daily Load as determined by ERCOT based on information provided by the Counter-Party. |
| DEG | MWh | Daily Estimated GenerationThe Counter-Party’s estimated average daily generation as determined by ERCOT based on information provided by the Counter-Party. |
| RTEFL | none | Real-Time Energy Factor for LoadThe ratio of the Counter-Party’s estimated energy purchases in the RTM as determined by ERCOT based on information provided by the Counter-Party, to the Counter-Party’s Daily Estimated Load. |
| RTAEP | $/MWh | Real-Time Average Energy PriceAverage Settlement Point Price for the “ERCOT 345” as defined in Section 3.5.2.5 based upon the previous seven days’ average Real-Time Settlement Point Prices. |
| RTEFG | none | Real-Time Energy Factor for Generation—The ratio of the Counter-Party’s estimated energy sales in the RTM as determined by ERCOT, based on information provided by the Counter-Party, to the Counter-Party’s Daily Estimated Generation. |

(5)	For a Counter-Party that is only a CRR Account Holder and is not a QSE, the IEL is zero.

16.11.4.3	Determination of Counter-Party Estimated Aggregate Liability

After a Counter-Party commences activity in ERCOT markets, ERCOT shall monitor and calculate the Counter-Party’s EAL based on the formulas below.

EAL q 	= 	Max [IEL during the first 40-day period only beginning on the date that the Counter-Party commences activity in ERCOT markets, Max {RTLE during the previous 40-day period}, RTLF] + DALE

+ Max [RTLCNS, Max {URTA during the previous 40 day period}] + OUT q + ILE q

EAL a =	OUT a

ERCOT may adjust the number of days used in determining the highest RTLE and/or URTA, and/or to exclude specific Operating Days to calculate RTLE, URTA, OUT, or DALE.

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| EAL q | $ | Estimated Aggregate Liability for all the QSEs represented by the Counter-Party. |
| EAL a | $ | Estimated Aggregate Liability for all the CRR Account Holders represented by the Counter-Party. |
| IEL | $ | Initial Estimated Liability for all the QSEs represented by the Counter-Party as defined in Section 16.11.4.2, Determination of Counter-Party Initial Estimated Liability. |
| q |  | QSEs represented by Counter-Party. |
| a |  | CRR Account Holders represented by Counter-Party. |
| RTLE | $ | Real-Time Liability Extrapolated—M1 multiplied by the sum of the net amount due to or from ERCOT by the Counter-Party for Operating Days for which RTM Initial Statements were generated in the 14 most recent calendar days, divided by the number of Operating Days for which Real-Time Initial Settlement Statements were generated for the Counter-Party in the 14 most recent calendar days. |
| URTA | $ | - Unbilled Real-Time Amount—M2 multiplied by the sum of the net amount due to or from ERCOT by the Counter-Party for which RTM Initial Statements were generated in the 14 most recent calendar days, divided by the number of Operating Days for which Real-Time Initial Settlement Statements were generated for the Counter-Party in the 14 most recent calendar days. |
| RTL | $ | Real-Time Liability—The estimated or settled amounts due to or from ERCOT due to activities in the RTM for an Operating Day, as defined in Section 16.11.4.3.2, Real-Time Liability Estimate. |
| RTLCNS | $ | Real-Time Liability Completed and Not Settled—For each Operating Day that is completed but not settled, ERCOT shall calculate RTL adjusted up by rtlcu% if there is a net amount due to ERCOT or adjusted down by rtlcd% if there is a net amount due to the QSE. RTLCNS = Sum of Max RTL(rtlcu% * RTL, rtlcd% * RTL) for all completed and not settled Operating Ddys Where: rtlcu = Real-Time Liability Markup rtlcd = Real-Time Libaility Markdown |
| RTLF | $ | Real-Time Liability Forward—rtlf% of the sum of estimated RTL from the most recent seven Operating Days. RTLF = rtlfp% of the Sum of Max RTL(rtlcu% * RTL, rtlcd% * RTL) for the most recent seven Operating Days Where: rtlfp = Real-Time Liability Forward |
| OUTq | $ | Outstanding Unpaid Transactions for all QSEs represented by the Counter-Party—Outstanding unpaid transactions of the Counter-Party, which include (a) outstanding Invoices to the Counter-Party; (b) estimated unbilled items to the Counter-Party, to the extent not adequately accommodated in the RTLE calculation (including resettlements and other known liabilities); and (c) estimated CRR Auction revenue available for distribution for Operating Days in the previous two months, to the extent not invoiced to the Counter-Party. Invoices will not be considered outstanding for purposes of this calculation the Business Day after that Invoice payment is received. OUT q = OIA q + UDAA q + UFA q + UTA q + CARD Where: OIA q = Outstanding Invoice Amounts for all the QSEs represented by the Counter-Party – Sum of any outstanding Real-Time and Day-Ahead unpaid invoices issued to the Counter-Party, including but not limited to CRR Auction Revenue Distribution (CARD) Invoices, CRR Balancing Account Invoices, Default Uplift Invoices and other miscellaneous Invoices. Also included are the amounts or portions of Invoices due to the Counter-Party that have been short-paid as a result of a default or non-payment of Invoices due to ERCOT by another Counter-Party. UDAA q = Unbilled Day-Ahead Amounts for all QSEs represented by the Counter-Party – Sum of DAL for all the QSEs represented by the Counter-Party for all Operating Days for which a DAM Statement is not generated. UFA q = Unbilled Final Amounts for all QSEs represented by the Counter-Party –Unbilled final extrapolated days (ufd) multiplied by the sum of the net amount due to or from ERCOT for all QSEs represented by the Counter-Party for Operating Days for which RTM Final Statements were generated in the 21 most recent calendar days, divided by the number of Operating Days for which RTM Final Settlement Statements were generated for the Counter-Party in the 21 most recent calendar days. UTA q = Unbilled True-Up Amounts for all QSEs represented by the Counter-Party – Unbilled true-up extrapolated days (utd) multiplied by the sum of the net amount due to or from ERCOT by the Counter-Party for all the QSEs represented by the Counter-Party for Operating Days for which RTM True-up Statements were generated in the 21 most recent calendar days, divided by the number of Operating Days for which RTM True-up Settlement Statements were generated for the Counter-Party in the 21 most recent calendar days. CARD = CRR Auction Revenue Distribution for all QSEs represented by the Counter-Party –Estimate of Counter-Party’s unpaid allocation of CRR Auction revenues that have already been collected but have not been paid out to all QSEs represented by the Counter-Party. CRR Auction revenues that have been earned but not billed are distributed based on the following Load Ratio Shares (LRSs): (a) Zonal LRS applied to revenues from CRRs cleared and have source and sink points located within a 2003 ERCOT Congestion Management Zone (CMZ), and (b) ERCOT-wide LRS applied to all other CRR Auction revenues. The LRS will be based on the latest completed operating month for which LRS are available. |
| DAL | $ | Day-Ahead Liability—The estimated or settled amounts due to or from ERCOT due to activities in the DAM for an Operating Day, as defined in Section 16.11.4.3.1, Day-Ahead Liability Estimate. |
| OUT a | $ | Outstanding Unpaid Transactions for all CRR Account Holders represented by the Counter-Party—Outstanding, unpaid transactions of all CRR Account Holders represented by the Counter-Party, which include outstanding Invoices to the Counter-Party. OUT a = OIA a + UDAA a Where: OIA a = Outstanding Invoice Amounts for all CRR Account Holders represented by the Counter-Party – Sum of any outstanding Real-Time and Day-Ahead unpaid Invoices issued to the Counter-Party including but not limited to CARD Invoices, CRR Balancing Account Invoices, Default Uplift Invoices and other miscellaneous Invoices. Also included are the amounts or portions of Invoices due to the Counter-Party that have been short-paid as a result of a Default or non-payment of Invoices due to ERCOT by another Counter-Party. UDAA a = Unbilled Day-Ahead Amounts for all CRR Account Holders represented by the Counter-Party – Sum of DAL for all CRR Account Holders represented by the Counter-Party for all Operating Days for which DAM Statement is not generated. |
| ILE q | $ | Incremental Load Exposure –In the event of a Mass Transition necessitated by the default of a Counter-Party representing a QSE associated with an LSE, ERCOT may adjust the TPE of the Counter-Parties representing QSEs that are qualified as Providers of Last Resort (POLRs) to reflect the estimated Incremental Load Exposure (ILE) resulting from the Mass Transition. The adjustment will be based on the POLR’s pro rata share of the defaulting Counter-Party’s RTLE, based on the total estimated Electric Service Identifiers (ESI IDs) to be transitioned. ERCOT will communicate any such adjustment to the Authorized Representative of each Counter-Party who is a POLR within 24 hours of the initiation of a Mass Transition. The ILE adjustment will remain in place no more than the number of days necessary to effect a Mass Transition for the defaulting Counter-Party, after which time the incremental exposure will be fully reflected in the Counter-Party’s unadjusted TPE. |
| DALE | $ | - Average Daily Day-Ahead Liability Extrapolated—M1 multiplied by the sum of the net amount due to or from ERCOT by the Counter-Party for Operating Days for which DAM Settlement Statements were generated in the seven most recent calendar days, divided by the number of Operating Days for which DAM Settlement Statements were generated for the Counter-Party in the seven most recent calendar days. |
| M1 |  | M1 = M1a + M1b—Multiplier for DALE and RTLE. Provides for forward risk during a Counter-Party termination upon default based upon the sum of the time period required for any termination upon default (M1a) and the time period required for a Mass Transition only (M1b). The M1a component is applicable to all Counter-Parties. The M1b component is applicable only to Counter-Parties representing any QSE associated with a LSE. M1a = Time period required for any termination upon default. M1b = Weighted average transition days = Min(B, (2 + Max(1, (u+1)/2))*(1-DF)), rounded up to whole days Where: u = (ESIn/r) Unscaled number of days to transition. B = Benchmark value. Used to establish a maximum M1 value. ESIn = Number of Electric Service Identifiers (ESI IDs) associated with an individual Counter-Party. This value will be updated no less often than annually by ERCOT and updated values communicated to individual Counter-Parties. Counter-Parties entering the market will provide an estimated number of ESI IDs for use during their first six months of market activity. Subsequent to this time, the value for that Counter-Party shall be updated by ERCOT concurrently with other Counter-Parties with QSEs representing an LSE. r = Assumed ESI ID daily transition rate. DF = Discount Factor applied to M1b if the Counter-Party is eligible for unsecured credit under Section 16.11.2, or meets other creditworthiness standards that may be developed and approved by TAC and the ERCOT Board. Each Counter-Party’s eligibility for unsecured credit will be evaluated at least annually. |
| M2 |  | Multiplier for URTA. . |

The above parameters are defined as follows:

| Parameter | Unit | Current Value* |
| --- | --- | --- |
| Rtlcu | Percentage | 110% |
| Rtlcd | Percentage | 90% |
| Rtlfp | Percentage | 150% |
| ufd | Days | 55 |
| utd | Days | 180 |
| M1a | Days | 12 |
| B | Days | 8 |
| r | none | 100,000 per day |
| DF | Percentage | 0 |
| M2 | Days | 9 |
| * The current value for the parameters referenced in this table above will be recommended by TAC and approved by the ERCOT Board. ERCOT shall update parameter values on the first day of the month following ERCOT Board approval unless otherwise directed by the ERCOT Board. ERCOT shall provide a Market Notice prior to implementation of a revised parameter value. |  |  |

[NPRR620: Replace Section 16.11.4.3 above with the following upon system implementation:] 16.11.4.3 Determination of Counter-Party Estimated Aggregate Liability After a Counter-Party commences activity in ERCOT markets, ERCOT shall monitor and calculate the Counter-Party’s EAL based on the formulas below. EAL q = Max [IEL during the first 40-day period only beginning on the date that the Counter-Party commences activity in ERCOT markets, Max {RTLE during the previous 40-day period}, RTLF] + DALE + Max [RTLCNS, Max {URTA during the previous 40 day period}] + OUT q + ILE q EAL t = Max [Max {RTLE during the previous 20-day period}, RTLF ] + DALE + Max [RTLCNS, Max {URTA during the previous 20-day period}]+ OUT t EAL a = OUT a ERCOT may adjust the number of days used in determining the highest RTLE and/or URTA, and/or to exclude specific Operating Days to calculate RTLE, URTA, OUT, or DALE. The above variables are defined as follows: Variable; Unit; Description. EAL q; $; Estimated Aggregate Liability for all the QSEs represented by Counter-Party if at least one QSE represented by the Counter-Party represents either Load or Generation.. EAL t; $; Estimated Aggregate Liability for all the QSEs represented by a Counter-Party if none of the QSEs represented by the Counter-Party represent either Load or generation.. EAL a; $; - Estimated Aggregate Liability for all the CRR Account Holders represented by the Counter-Party.. IEL; $; Initial Estimated Liability for all the QSEs represented by the Counter-Party if at least one QSE represented by the Counter-Party represents either Load or generation as defined in paragraphs (1), (2), (3) and (4) of Section 16.11.2, Determination of Counter-Party Initial Estimated Liability.. q; QSEs represented by Counter-Party.. t; QSEs represented by a Counter-Party if none of the QSEs represented by the Counter-Party represent either Load or generation. a; CRR Account Holders represented by Counter-Party.. RTLE; $; - Real-Time Liability Extrapolated for a QSE that represents either Load or generation—M1 multiplied by the sum of the net amount due to or from ERCOT by the Counter-Party for Operating Days for which RTM Initial Statements were generated in the 14 most recent calendar days, divided by the number of Operating Days for which Real-Time Initial Settlement Statements were generated for the Counter-Party in the 14 most recent calendar days.. URTA; $; - Unbilled Real-Time Amount—M2 multiplied by the sum of the net amount due to or from ERCOT by the Counter-Party for Operating Days for which RTM Initial Statements were generated in the 14 most recent calendar days divided by the number of Ooperating Days for which Real-Time Initial Settlement Statements were generated for the Counter-Party in the 14 most recent calendar days.. RTL; $; Real-Time Liability—The estimated or settled amounts due to or from ERCOT due to activities in the RTM for an Operating Day, as defined in Section 16.11.4.3.2, Real-Time Liability Estimate.. RTLCNS; $; Real-Time Liability Completed and Not Settled—For each Operating Day that is completed but not settled, ERCOT shall calculate RTL adjusted up by rtlcu% if there is a net amount due to ERCOT or adjusted down by rtlcd% if there is a net amount due to the QSE. RTLCNS = Sum of Max RTL(rtlcu% * RTL, rtlcd% * RTL) for all completed and not settled Operating Days Where: rtlcu = Real-Time Liability Markup rtlcd = Real-Time Libaility Markdown. RTLF; $; Real- Time Liability Forward—rtlfp% of the sum of estimated RTL from the most recent seven Operating Days. RTLF = rtlf% of the Sum of Max RTL(rtlcu% * RTL, rtlcd% * RTL) for the most recent seven Operating Days Where: rtlfp = Real-Time Liability Forward. OUTq; $; Outstanding Unpaid Transactions—Outstanding unpaid transactions for all QSEs represented by the Counter-Party, which include (a) outstanding Invoices to the Counter-Party; (b) estimated unbilled items to the Counter-Party, to the extent not adequately accommodated in the RTLE calculation (including resettlements and other known liabilities); and (c) estimated CRR Auction revenue available for distribution for Operating Days in the previous two months, to the extent not invoiced to the Counter-Party. Invoices will not be considered outstanding for purposes of this calculation the Business Day after that Invoice payment is received. OUT q = OIA q + UDAA q + UFA q + UTA q + CARDq Where: OIA q = Outstanding Invoice Amounts for all the QSEs represented by the Counter-Party – Sum of any outstanding Real-Time and Day-Ahead unpaid invoices issued to the Counter-Party, including but not limited to CRR Auction Revenue Distribution (CARD) Invoices, CRR Balancing Account Invoices, Default Uplift Invoices and other miscellaneous Invoices. Also included are the amounts or portions of Invoices due to the Counter-Party that have been short-paid as a result of a default or non-payment of Invoices due to ERCOT by another Counter-Party. UDAA q = Unbilled Day-Ahead Amounts for all the QSEs represented by the Counter-Party – Sum of DAL for all the QSEs represented by the Counter-Party for all Operating Days for which a DAM Statement is not generated. UFA q = Unbilled Final Amounts for all the QSEs represented by the Counter-Party – Unbilled final extrapolated days (ufd) multiplied by the sum of the net amount due to or from ERCOT for all QSEs represented by the Counter-Party for Operating Days for which RTM Final Statements were generated in the 21 most recent calendar days, divided by the number of Operating Days for which RTM Final Settlement Statements were generated for the Counter-Party in the 21 most recent calendar days. UTA q = Unbilled True-Up Amounts for all the QSEs represented by the Counter-Party –– Unbilled true-up extrapolated days (utd) multiplied by the sum of the net amount due to or from ERCOT by the Counter-Party for all the QSEs represented by the Counter-Party for Operating Days for which RTM True-up Statements were generated in the 21 most recent calendar days, divided by the number of Operating Days for which RTM True-up Settlement Statements were generated for the Counter-Party in the 21 most recent calendar days. CARD = CRR Auction Revenue Distribution for all the QSEs represented by the Counter-Party –Estimate of the Counter-Party’s unpaid allocation of CRR Auction revenues that have already been collected but have not been paid out to all QSEs represented by the Counter-Party. CRR Auction revenues that have been earned but not billed are distributed based on the following Load Ratio Shares (LRS): (a) Zonal LRS applied to revenues from CRRs cleared and have source and sink points located within a 2003 ERCOT Congestion Management Zone (CMZ), and (b) ERCOT-wide LRS applied to all other CRR Auction revenues. The LRS will be based on the latest completed operating month for which LRS are available.. DAL; $; Day-Ahead Liability—The estimated or settled amounts due to or from ERCOT due to activities in the DAM for an Operating Day, as defined in Section 16.11.4.3.1, Day-Ahead Liability Estimate.. OUTt; $; Outstanding Unpaid Transactions—Outstanding unpaid transactions for all QSEs represented by the Counter-Party if none of the QSEs represented by the Counter-Party represent either Load or generation, which include (a) outstanding Invoices to the Counter-Party; (b) estimated unbilled items to the Counter-Party, to the extent not adequately accommodated in the RTLE calculation (including resettlements and other known liabilities). OUT t = OIA t + UDAA t + UFA t + UTA t Where: OIA t = Outstanding Invoice Amounts for all the QSEs represented by the Counter-Party if none of the QSEs represented by the Counter-Party represent either Load or generation – Sum of any outstanding Real-Time and Day-Ahead unpaid Invoices issued to the Counter-Party, including but not limited to CRR Balancing Account Invoices, Default Uplift Invoices and other miscellaneous Invoices. Also included are the amounts or portions of invoices due to the Counter-Party that have been short-paid as a result of a Default or non-payment of invoices due to ERCOT by another Counter-Party. UDAA t = Unbilled Day-Ahead Amounts for all the QSEs represented by the Counter-Party if none of the QSEs represented by the Counter-Party represent either Load or generation – Sum of DAL for all the QSEs represented by the Counter-Party for all Operating Days for which DAM Statement is not generated. UFA t = Unbilled Final Amounts for all the QSEs represented by the Counter-Party if none of the QSEs represented by the Counter-Party represent either Load or generation – ufd multiplied by the sum of the net amount due to or from ERCOT for all QSEs represented by the Counter-Party for Operating Days for which RTM Final Statements were generated in the 21 most recent calendar days, divided by the number of Operating Days for which RTM Final Settlement Statements were generated for the Counter-Party in the 21 most recent calendar days. UTA t = Unbilled True-Up Amounts for all the QSEs represented by the Counter-Party if none of the QSEs represented by the Counter-Party represent either Load or generation – utd multiplied by the sum of the net amount due to or from ERCOT by the Counter-Party for all the QSEs represented by the Counter-Party for Operating Days for which RTM True-up Statements were generated in the 21 most recent calendar days, divided by the number of Operating Days for which RTM True-up Settlement Statements were generated for the Counter-Party in the 21 most recent calendar days.. OUT a; $; Outstanding Unpaid Transactions for all CRR Account Holders represented bythe Counter-Party—Outstanding, unpaid transactions of all the CRR Account Holders represented by the Counter-Party, which include (a) outstanding Invoices to the Counter-Party. Invoices will not be considered outstanding for purposes of this calculation the Business Day after that Invoice payment is received. OUT a = OIA a + UDAA a Where: OIA a = Outstanding Invoice Amounts for all the CRR Account Holders represented by the Counter-Party – Sum of any outstanding Real-Time and Day-Ahead unpaid Invoices issued to the Counter-Party including but not limited to CRR Balancing Account Invoices, Default Uplift Invoices and other miscellaneous Invoices. Also included are the amounts or portions of Invoices due to the Counter-Party that have been short-paid as a result of a default or non-payment of Invoices due to ERCOT by another Counter-Party. UDAA a = Unbilled Day-Ahead Amounts for all the CRR Account Holders represented by the Counter-Party – Sum of DAL of all the CRR Account Holders represented by the Counter-Party for all Operating Days for which DAM Statement is not generated.. ILE q; $; Incremental Load Exposure –In the event of a Mass Transition necessitated by the default of a Counter-Party representing a QSE associated with an LSE, ERCOT may adjust the TPE of the Counter-Parties representing QSEs that are qualified as Providers of Last Resort (POLRs) to reflect the estimated Incremental Load Exposure (ILE) resulting from the Mass Transition. The adjustment will be based on the POLR’s pro rata share of the defaulting Counter-Party’s RTLE, based on the total estimated Electric Service Identifiers (ESI IDs) to be transitioned. ERCOT will communicate any such adjustment to the Authorized Representative of each Counter-Party who is a POLR within 24 hours of the initiation of a Mass Transition. The ILE adjustment will remain in place no more than the number of days necessary to effect a Mass Transition for the defaulting Counter-Party, after which time the incremental exposure will be fully reflected in the Counter-Party’s unadjusted TPE.. DALE; $; Average Daily Day-Ahead Liability Extrapolated—M1 multiplied by the sum of the net amount due to or from ERCOT by the Counter-Party for all Operating Days for which the DAM Settlement Statements were generated in the seven most recent calendar days, divided by the number of Operating Days for which DAM Settlement Statements were generated for the Counter-Party in the seven most recent calendar days.. M1; M1 = M1a + M1b—Multiplier for DALE and RTLE. Provides for forward risk during a Counter-Party termination upon default based upon the sum of the time period required for any termination upon default (M1a) and the time period required for a Mass Transition only (M1b). The M1a component is applicable to all Counter-Parties. The M1b component is applicable only to Counter-Parties representing any QSE associated with a LSE. M1a= Time period required for any termination upon default. M1b = Weighted average transition days = Min(B, (2 + Max(1, (u+1)/2))*(1-DF)), rounded up to whole days Where: u = (ESIn/r) Unscaled number of days to transition. B = Benchmark value. Used to establish a maximum M1 value. ESIn = Number of Electric Service Identifiers (ESI IDs) associated with an individual Counter-Party. This value will be updated no less often than annually by ERCOT and updated values communicated to individual Counter-Parties. Counter-Parties entering the market will provide an estimated number of ESI IDs for use during their first six months of market activity. Subsequent to this time, the value for that Counter-Party shall be updated by ERCOT concurrently with other Counter-Parties with QSEs representing an LSE. r = Assumed ESI ID daily transition rate. DF = Discount Factor applied to M1b if the Counter-Party is eligible for unsecured credit under Section 16.11.2, or meets other creditworthiness standards that may be developed and approved by TAC and the ERCOT Board.. M2; Multiplier for URTA.. The above parameters are defined as follows: Parameter; Unit; Current Value*. Rtlcu; Percentage; 110%. Rtlcd; Percentage; 90%. Rtlfp; Percentage; 150%. ufd; Days; 55. utd; Days; 180. M1a; Days; 12. B; Days; 8. r; none; 100,000 per day. DF; Percentage; 0. M2; Days; 9. * The current value for the parameters referenced in this table above will be recommended by TAC and approved by the ERCOT Board. ERCOT shall update parameter values on the first day of the month following ERCOT Board approval unless otherwise directed by the ERCOT Board. ERCOT shall provide a Market Notice prior to implementation of a revised parameter value..

16.11.4.3.1	Day-Ahead Liability Estimate

(1)	ERCOT shall estimate Day-Ahead Liability (DAL) for an Operating Day as the sum of estimates for the following DAM Settlement charges and payments:

(a)	Section 4.6.2.1, Day-Ahead Energy Payment;

(b)	Section 4.6.2.2, Day-Ahead Energy Charge;

(c)	Section 4.6.3, Settlement for PTP Obligations Bought in DAM;

(d)	Section 4.6.4.1.1, Regulation Up Service Payment;

(e)	Section 4.6.4.1.2, Regulation Down Service Payment;

(f)	Section 4.6.4.1.3, Responsive Reserve Service Payment;

(g)	Section 4.6.4.1.4, Non-Spinning Reserve Service Payment;

(h)	Section 4.6.4.2.1, Regulation Up Service Charge;

(i)	Section 4.6.4.2.2, Regulation Down Service Charge;

(j)	Section 4.6.4.2.3, Responsive Reserve Service Charge;

(k)	Section 4.6.4.2.4, Non-Spinning Reserve Service Charge;

(l)	Section 7.9.1.1, Payments and Charges for PTP Obligations Settled in DAM;

(m)	Section 7.9.1.2, Payments for PTP Options Settled in DAM;

(n)	Section 7.9.1.5, Payments and Charges for PTP Obligations with Refund Settled in DAM; and

(o)	Section 7.9.1.6, Payments for PTP Options with Refund Settled in DAM.

16.11.4.3.2	Real-Time Liability Estimate

(1)	ERCOT shall estimate RTL for an Operating Day as the sum of estimates for the following RTM Settlement charges and payments:

(a)	Section 6.6.3.1, Real-Time Energy Imbalance Payment or Charge at a Resource Node, using Real-Time Metered Generation (RTMG) as generation estimate;

(b)	Section 6.6.3.2, Real-Time Energy Imbalance Payment or Charge at a Load Zone, using 14 day or seven day old LRS for Load estimate;

(c)	Section 6.6.3.3, Real-Time Energy Imbalance Payment or Charge at a Hub;

(d)	Section 6.6.3.4, Real-Time Energy Payment for DC Tie Import;

(e)	Section 6.6.3.6, Real-Time Energy Charge for DC Tie Export Represented by the QSE Under the Oklaunion Exemption;

- (f)	Section 6.6.4, Real-Time Congestion Payment or Charge for Self-Schedules; and

- (g)	Section 7.9.2.1, Payments and Charges for PTP Obligations Settled in Real-Time.

- 16.11.4.5	Determination of the Counter-Party Future Credit Exposure

(1)	ERCOT shall monitor and calculate the Counter-Party’s FCE for all the CRR Account Holders represented by the Counter-Party as CRR Owner of record at ERCOT.

FCE a  =  FCEOBL a + FCEOPT a

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| FCE a | $ | Future Credit Exposure for all CRRs held by all CRR Account Holders represented by the Counter-Party. |
| FCEOBL a | $ | Future Credit Exposure for PTP Obligations for all PTP Obligations held by all CRR Account Holders represented by the Counter-Party as CRR Owner of record at ERCOT, for all Operating Days in the current operating month, Prompt Month, and all Forward Months. |
| FCEOPT a | $ | Future Credit Exposure for PTP Options for all PTP Options held by all CRR Account Holders represented by the Counter-Party as CRR Owner of record at ERCOT, for all Operating Days remaining in the current operating month and Prompt Month. |
| a | none | All CRR Account Holders represented by the Counter-Party. |

(2)	The Counter-Party’s FCE for PTP Obligations (FCEOBL) held by all CRR Account Holders represented by the Counter-Party as CRR Owner of record at ERCOT are calculated as follows:

FCEOBL a 	=	∑m [(∑h ∑j, k NAOBLMW m, h, (j, k)) * (-Min(0, PWA ci100, m, PWACP m))]

PWACP m 	=	∑h ∑j, k [NAOBLMW m, h, (j, k) * EACP m, h, (j, k)] / ∑h ∑j, k [NAOBLMW m, h, (j, k)]

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| FCEOBL a | $ | Future Credit Exposure for PTP Obligations for all PTP Obligations held by all CRR Account Holders represented by the Counter-Party as CRR Owner of record at ERCOT for all Operating Days in the current operating month, Prompt Month, and all Forward Months. |
| NAOBLMW m, h, (j, k) | MW | Net Awarded PTP ObligationsNet awarded PTP Obligations with the source j and sink k for the hour h and month m owned by all CRR Account Holders represented by the Counter-Party as CRR Owner of record at ERCOT for all Operating Days in the current operating month, Prompt Month, and Forward Months. |
| PWA ci100, m | $/MW per hour | Portfolio Weighted AdderThe portfolio weighted adder calculated as the 100th percentile of a volume weighted average rolling consecutive DAM settled price for all CRR Account Holders represented by the Counter-Party as CRR Owner of record at ERCOT based on volumes owned for the month m, over a period that represents a month for each product type (18 days for 5*16, 8 days for 2*16, 28 days for 7*8). The look-back period for DAM settled prices shall be the lesser of January 1, 2011 to the current time, and the current time minus three years. If historical Day-Ahead Settlement Point Prices (DASPPs) are not available for a Settlement Point for one or more Operating Days, ERCOT will designate a proxy Settlement Point for this purpose, and the DASPPs of the proxy Settlement Point of corresponding Operating Days are used. |
| PWACP m | $/MW per hour | Portfolio Weighted Auction Clearing PriceThe portfolio weighted auction clearing price calculated as the volume weighted auction clearing price for all CRR Account Holders represented by the Counter-Party as CRR Owner of record at ERCOT based on the most recent auction clearing price for the month m and volumes owned for the month m. |
| EACP m, h, (j, k) | $/MW per hour | Effective Auction Clearing Price—The auction clearing price with the source j and sink k for the hour h, and month m. For each CRR PTP Obligation, this value is equal to the auction clearing price of an awarded CRR selected as follows: (a) Awarded CRRs with the source j and sink k containing hour h and operating month m are selected from the set of unexpired awarded PTP Obligations. If no awarded CRRs are found the EACP value is zero. (b) If (a) results in more than one awarded CRR, awarded CRRs with the most recent award date are selected. (c) If (b) results in more than one awarded CRR, then the awarded CRR with the lowest auction clearing price is selected. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| m | none | An operating month. |
| h | none | An Operating Hour. |
| a | none | All CRR Account Holders represented by the Counter-Party. |
| ci100 | none | 100th percentile confidence interval. |

(3)	The FCE for PTP Options (FCEOPT) held by all the CRR Account Holders represented by the Counter-Party as CRR Owner of record at ERCOT are calculated as follows:

FCEOPT a 	=	- ∑m ∑h ∑j, k [(NAOPTMW m, h, (j, k)) * Max(0, A ci99, ctou, (j, k))]

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| FCEOPT a | $ | Future Credit Exposure for PTP OptionsFCE for all PTP Options held by all CRR Account Holders represented by the Counter-Party as CRR Owner of record at ERCOT for all Operating Days remaining in the current operating month and Prompt Month. |
| A ci99, ctou, (j, k) | $/MW per hour | Path Specific DAM Based AdderPath specific DAM based adder calculated as 99th percentile of the average rolling consecutive DAM settled price for the CRR source j and sink k over a period that represents a month for each CRR Time Of Use (TOU) ctou product type (18 days for 5*16, 8 days for 2*16, 28 days for 7*8). The look-back period for DAM settled prices shall be the lesser of January 1, 2011 to the current time, and the current time minus three years. If historical DASPPs are not available for a Settlement Point for one or more Operating Days, ERCOT will designate a proxy Settlement Point for this purpose, and the DASPPs of the proxy Settlement Point of corresponding Operating Days are used. |
| NAOPTMW m, h, (j, k) | MW | Net Awarded PTP OptionsNet awarded PTP Options with the source j and sink k for the hour h and month m owned by all CRR Account Holders represented by the Counter-Party as CRR Owner of record at ERCOT for remaining Operating Days in the current operating month, and Prompt Month. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| m | none | An operating month. |
| ctou | none | CRR Time Of Use block. |
| h | none | An Operating Hour. |
| a | none | All CRR Account Holders represented by the Counter-Party. |
| ci99 | none | 99th percentile confidence interval. |