ERCOT Load Profiling Guide

Section 17:  Load Profiling Metering Requirements

December 1, 2010

17	LOAD PROFILING METERING REQUIREMENTS	17-1

17.1	Introduction	17-1

17.2	Interval Data Recorder Requirement	17-1

17.3	Demand Meter Changes	17-2

17.4	Metering for Supplemental Load Profiling	17-3

# LOAD PROFILING METERING REQUIREMENTS

17.1	Introduction

(1)	This Section defines the requirements for metering with regard to Load Profiling and Interval Data Recorder (IDR) Meter activities.  Meter reading data in this context encompasses monthly consumption, Demand and interval meter data.  The Transmission and/or Distribution Service Providers (TDSPs) are the only Entities authorized to provide Settlement meter data to ERCOT in accordance with Protocol Section 10, Metering.

(2)	Each Electric Service Identifier (ESI ID) in ERCOT shall be assigned a Load Profile ID.  Meter reading data is necessary to perform this assignment because the information used for assignment of the Load Profile ID is energy and/or Demand data.  Only meter reading data provided to ERCOT shall be used to assign the Load Profile ID.  Other primary uses of meter reading data are:

(a)	To allocate daily Load for Settlement and aggregation process;

(b)	To allow validation for Load Profile ID assignments;

(c)	To ensure Load Profile Models are appropriately specified; and

(d)	To allow for Load Profile Model development.

(3)	Meter reading data shall be supplied to ERCOT in accordance with its intended purpose to meet the needs of ERCOT billing and Settlement activities.

(4)	This Section addresses the following topics:

(a)	IDR Meter requirement;

(b)	Demand meter changes; and

(c)

Supplemental Load Profiling.

(5)	Details for metering activities may be found in Protocol Section 10, Metering.

17.2	Interval Data Recorder Meter Requirement

(1)	Interval Data Recorder (IDR) Meters shall be installed or removed in accordance with Protocol Section 18.6, Installation and Use of Interval Data Recorder Meters and Retail Market Guide Section 7.13, Interval Data Recorder (IDR) Meter Optional Removal and Installation Process.

(2)	Costs associated with mandatory installation of IDR Meters by Transmission and/or Distribution Service Providers (TDSPs) shall be the responsibility of the TDSP and be in accordance with approved TDSP tariffs.

17.3	Demand Meter Changes

(1)	Section 9.2.1, Load Profile ID Changes Initiated by Transmission and/or Distribution Service Providers, presents the procedure for changing Load Profile ID assignment. The following provides a brief discussion regarding the circumstances, which may involve a meter change.

(2)	When a Transmission and/or Distribution Service Providers (TDSP) determines that a Demand meter should be changed based on the TDSPtariff, the TDSP shall notify the Competitive Retailer (CR) prior to making the meter change.  Upon CR notification, the TDSP shall not change the Demand meter.

(3)	If the Demand meter is no longer needed by TDSP tariff, the TDSP shall reassign the ESI ID to the appropriate Load Profile ID in accordance with Section 9.2, Processes to Change Load Profile ID Assignments.

(4)	Conversely, the ESI ID’s Load growth may warrant the measurement of Demand.  TDSPs shall enforce appropriate thresholds and TDSP tariffs requiring the installation of a Demand meter.

(5)	Once it has been determined that a Demand meter change is warranted, the TDSP shall make appropriate changes in accordance with Protocol Section 18.4, Assignment of Load Profile ID.  The TDSP shall notify the CR of the completed changes as well.

17.4	Metering for Supplemental Load Profiling

(1)	If a Competitive Retailer (CR) wants supplemental Load Profiling, the CR shall follow procedures in Section 16, Supplemental Load Profiling.