| LPGRR Number | XXX | LPGRR Title |  |
| --- | --- | --- | --- |
| Date Posted |  |  |  |
| Requested Resolution |  | Normal or Urgent, and justification for Urgent status |  |
| Load Profiling Guide Sections Requiring Revision |  | 1.1, Purpose of the Load Profiling Guide 1.2, Document Purpose 1, LOAD PROFILING GUIDE REVISION PROCESS 2.1, Introduction 2.2, Submission of Load Profiling Guide Revision Request 2.3, Profiling Working Group 2.4, Load Profiling Guide Revision Procedure 2.4.1, Review and Posting of Load Profiling Guide Revision Requests 2.4.2, Withdrawal of a Load Profiling Guide Revision Request 2.4.3, Profiling Working Group Review and Action 2.4.4, Comments to the Profiling Working Group Recommendation Report 2.4.5, Load Profiling Guide Revision Request Impact Analysis 2.4.6, Profiling Working Group Review of Impact Analysis 2.4.7, Commercial Operations Subcommittee Vote 2.4.8, ERCOT Impact Analysis Based on Commercial Operations Subcommittee Report 2.4.9, Protocol Revision Subcommittee Review of Project Prioritization 2.4.10, Technical Advisory Committee Vote 2.4.11, ERCOT Board Vote 2.5, Appeal of Action 2.6, Urgent Requests 2.7, Revision Implementation 3, RESERVED 4, THE PROFILING WORKING GROUP 4.1, Purpose of the Profiling Working Group 4.2, Profiling Working Group Responsibilities 4.3, Profiling Working Group Reporting Structure 4.4, Profiling Working Group Membership 4.5, Profiling Working Group Contact Information 5, GUIDELINES FOR LOAD PROFILE DEVELOPMENT (delete) 5.1, Background (delete) 5.2, Guidelines (delete) 6.1, Introduction 6.2, Review of Load Profiling Methodology 6.3, Considerations for Load Profiling Methodology Evaluation 6.3.1, Load Profile Model Performance 6.3.2, Methodology Performance 6.3.3, Alternative Methodology Impacts to Load Profiling Issues 6.3.4, Practical Implementation of Load Profiling Methodology 6.4, Possible Results of the Evaluation of Methodologies 6.4.1, No Changes to Load Profiling Methodologies 6.4.2, Modify Existing Load Profiling Methodology 6.4.3, Implement Alternative Load Profiling Methodology 7, REQUEST FOR CHANGES TO LOAD PROFILING METHODOLOGY(delete) 7.1, Current Methodologies (delete) 7.2, Request for Load Profiling Methodology Changes (delete) 7.3, Timeline for Processing a Load Profiling Methodology Change Request (delete) 7.4, Information Required with Request for Change (delete) 7.5, Evaluation of the Request (delete) 7.6, Approval of the Request (delete) 7.7, Costs for Load Profiling Methodology Changes (delete) 8, Load Profile Models 8.1, ROUTINE AND NON-ROUTINE LOAD PROFILE MODEL EVALUATIONS 8.1.1, Routine Evaluation of Load Profile Model Performance (delete) 8.1.2, Non-Routine Evaluation of Model Performance (delete) 8.1.2 Evaluation of Weather Zones (new) 8.2, EVALUATION OF LOAD PROFILE MODELS USING CURRENT LOAD RESEARCH DATA 8.2.1, Sources of Load Research Data 8.2.2, Procedures 8.2.3, Using Comparable Weather Zone Data (delete) 8.2.4, Factors Considered in Comparisons (delete) 8.2.4.1, Load-Weighted Average Price (delete) 8.2.4.2, On-Peak/Off-Peak Ratio (delete) 8.2.4.3, Load Factor (delete) 8.2.4.4, Summary Statistics on Differences Between Series (delete) 8.2.4.5, Deadweight Loss (delete) 8.3, EVALUATING LOAD PROFILE MODELS WITHOUT CURRENT LOAD RESEARCH DATA 8.3.1, Applications 8.3.2, Load Profile Model Comparisons (delete) 8.3.2.1, Comparisons for Adjusted Static Models 8.3.2.2, Examination of Monthly Patterns (delete) 8.3.2.3, Comparisons for Engineering Estimates 8.4, ROUTINE LOAD PROFILE MODEL EVALUATIONS (delete) 8.4.1, Routine Evaluation of Weather Zones (delete) 8.5, NON-ROUTINE LOAD PROFILE MODEL EVALUATIONS (delete) 8.6, ASSESSING THE TYPE OF LOAD PROFILE MODEL CHANGE NEEDED (delete) 8.6.1, Possible Changes (delete) 8.6.2, Qualitative Criteria (delete) 8.6.2.1, Substantial Bias (delete) 8.6.2.2, No Substantial Bias Indicated by Evaluation (delete) 8.6.2.3, Substantial Bias Indicated by Analysis of Current Load Research Data (delete) 8.6.2.3.1, Modest Scope (delete) 8.6.2.3.2, Extensive Scope (delete) 8.6.2.3.3, Adjustment Factors (delete) 8.6.2.4, Substantial Bias Indicated without Current Load Research Data (delete) 8.6.2.4.1, Similar Bias across Several Load Profile Segments within a Weather Zone (delete) 8.6.2.4.2, Bias Not Resolved by Modifying Weather Zones (delete) 8.7, CRITERIA FOR REQUIRING A LOAD PROFILE MODEL CHANGE (delete) 8.8, PROCEDURES FOR REQUESTING A CHANGE TO LOAD PROFILE MODELS (delete) 8.8.1, Request for Load Profile Model Changes (delete) 8.8.2, General Information Required with a Request (delete) 8.8.3, Requesting Load Profile Model Adjustment Factors (delete) 8.8.4, Requesting Change to Engineering Estimates (delete) 8.8.5, Requesting Re-Estimation of Models (delete) 8.9, APPROVAL PROCESS FOR LOAD PROFILE MODEL CHANGES 8.9.1, Timeline Prior to Implementing a Load Profile Change (delete) 8.9.2, Adjusted Static Models (delete) 8.9.2.1, Development of Adjustment Factors (delete) 8.9.2.2, Model Re-Estimation (delete) 8.9.3, Engineering Estimates (delete) 9.1, ASSIGNMENT OF LOAD PROFILE IDS 9.1.1, Profile Decision Tree Revision and Approval Process 9.1.2, Assignment of Load Profile IDs for New Service Delivery Points 9.1.3, Assignment of Load Profile IDs for New Electric Service Identifiers Resulting from a Mass Transition 9.1.4, Assignment of BUSOGFLT Profile Type 9.1.5, Assignment of Load Profile IDs for Distributed Generation 9.1.6, kVA Metered Loads 9.1.7, Load Profile ID Assignment for Non-ERCOT Electric Service Identifiers 9.1.8, Load Profile ID Assignment for Non-Opt In Entities 9.2, PROCESSES TO CHANGE LOAD PROFILE ID ASSIGNMENTS 9.2.1, Load Profile ID Changes Initiated By Transmission and/or Distribution Service Providers 9.2.1.1, Load Profile ID Change Related to a Transmission and/or Distribution Service Provider Tariff Change 9.2.1.2, Recognized Error in Current Assignment 9.2.1.3, Load Profile ID Changes Resulting from Meter Type Changes 9.2.1.3.1, Non-Interval Data Recorder to Interval Data Recorder and Interval Data Recorder to Non-Interval Data Recorder 9.2.1.3.2, Non-Time Of Use to Time Of Use (delete) 9.2.1.3.3, Time Of Use to Non-Time Of Use (delete) 9.2.1.3.4, Business Demand to Business Non-Demand 9.2.1.3.5, Non-Demand to Demand 9.2.1.4, CR Requested Change from a Default Load Profile ID 10, KVA TO KW CONVERSION 11, VALIDATION OF LOAD PROFILE ID 11.1, INITIAL ASSIGNMENT OF LOAD PROFILE IDS FOR OPT-IN ENTITIES 11.1.1, Validation of Initial Opt-In Entity Assignments 11.2, ANNUAL VALIDATION OF LOAD PROFILE TYPE 11.2.1, Annual Validation of Load Profile Type Assignment for RES and BUS Load Factor Electric Service Identifiers 11.3, ADDITIONAL VALIDATIONS 11.3.1, Validation of BUSOGFLT Profile Type 11.3.2, Validation of NMFLAT and NMLIGHT Profile Types 11.3.3, Validation of Profile Segments for Distributed Generation 11.3.4, Comparison of Profile Type to Premise Type 11.3.5, Validation of Service Address Zone Improvement Plan Code 11.3.6, Validation of Weather Zone Code 11.3.7, Comparison of Meter Data Type Code to Profile Type Code 11.3.8, Comparison of Weather Sensitivity Code to Meter Data Type Code 12.1, TYPES OF REQUESTS 12.1.1, Creation of a New Load Profile Segment 12.1.2, Redefinition of an Existing Load Profile Segment 12.1.3, Removal of Existing Load Profiles Segments 12.2, REQUEST FOR LOAD PROFILE SEGMENT CHANGES 12.4, PROCESS TIMING FOR REQUESTING CHANGES 12.5, GROUPS OF ELECTRIC SERVICE IDENTIFIERS ELIGIBLE TO BECOME LOAD PROFILE SEGMENTS 12.5.1, Universal Load Profile Segment Applicability 12.5.2, List-Based Load Profile Segments (delete) 12.6, INFORMATION REQUIRED WITH REQUEST FOR CHANGE 12.6.1, Unambiguous Group Identification 12.6.1.1, Identification Based on Data Currently in ERCOT’s Systems 12.6.1.3, List Based Load Profile Segments (delete) 12.6.2, Difference from Current Load Profile Segments 12.6.2.1, Supporting Data Requires 12.6.2.1.1, Loard Research Data 12.6.2.1.2, Other Kinds of Supporting Data (delete) 12.6.2.2, Basis for Assessment of Differences Based on Loard Research Data 12.6.2.3, Accounting for Weather Zones Effects in Load Profile Comparisons 12.6.2.4, Separate Comparisons for Each Weather Zone 12.6.2.5, Comparison for the Proposed Segment as a Whole 12.6.2.6, Summary Statistics on Differences of Series (delete) 12.6.2.7, Load Weighted Average Price (delete) 12.6.2.8, On Peak/Off Peak Ratio (delete) 12.6.2.9, Load Factor (delete) 12.6.2.10, Deadweight Loss (delete) 12.6.3, Size 12.6.4, Homogeneity 12.6.4.1, Load Research Demonstrating Homogeneity 12.6.4.2, Other supporting Evidence of Homogeneity (delete) 12.6.5, Quality Assurance Methodology for Electric Service Identifier Identification (delete) 12.7, COSTS FOR PROFILE SEGMENT CHANGES (delete) 12.8, EVALUATION OF THE REQUEST 12.9, RESOLUTION OF THE REQUEST 12.9.1, ERCOT Staff Initial Recommendation 12.9.2, Submitter and ERCOT Revisions 12.9.3, Presentation to Profiling Working Group (delete) 12.9.4, Profiling Working Group Disposition of Request (delete) 13, CHANGES TO WEATHER ZONE DEFINITIONS 13.1.1, Timeline for Processing a Request 13.1.2, Uniformity 13.2.1, Types of Weather Zone Boundary Changes 13.2.2, Eligible Areas for Weather Zones 13.2.2.1, Size 13.2.2.2, Weather Stations 13.2.3, Supporting Data 13.2.3.1, Calculated Load Profiles 13.2.3.2, Additional Supporting Data for Shifts across Weather Modeling Regions 13.2.3.3, Basis for Assessing Differences (delete) 13.3. Changes to Weather Modeling Regions 13.3.1, Supporting Data Required 13.4.1, Requests for Changes 13.5.1, Periodic Assessment 13.5.2.1, Changes in National Oceanic and Atmospheric Administration Weather Station (delete) 14, LOAD PROFILE ID DISPUTE PROCEDURE 14.1, Filing of a Load Profile ID Dispute 14.2, General Load Profile ID Dispute Resolution Guidelines 14.2.1, Disputes Involving ERCOT 14.2.2, Disputes Involving Transmission and/or Distribution Service Providers 14.2.3, Alternative Dispute Resolution 14.3, Resolutions of Disputes 15, LOAD RESEARCH SAMPLES (delete) 15.1, TRANSMISSION AND/OR DISTRIBUTION SERVICE PROVIDER SAMPLES (delete) 15.1.1, Maintenance of Existing Samples (delete) 15.1.2, Notification to ERCOT (delete) 15.1.3, Availability of Data (delete) 15.1.4, Creation of New Transmission and/or Distribution Service Provider Samples (delete) 15.2, ERCOT SAMPLES (delete) 15.2, ERCOT SAMPLES (delete) 15.2.1, Maintenance (delete) 15.2.2, Availability of Data (delete) 15.2.3, Criteria of Standards (delete) 15.2.4, Creation of New Samples (delete) 15.2.5, Guidelines for Installing and Refreshing Load Research Samples (delete) 16, SUPPLEMENTAL LOAD PROFILING (delete) 16.1, Load Profiling for Time Of Use Schedules (delete) 16.1.1, Establishing New Time Of Use Schedules (delete) 16.1.2, Chunking Load Profiling Methodology Description (delete) 16.1.3, Evaluation of the Chunking Load Profiles (delete) 16.1.3.1, Assessment of Chunking as a General Method Based on Load Profiles from Other Areas (delete) 16.1.3.2, Assessment of the Chunking Method Using ERCOT Load Research Data (delete) 16.1.3.3, Assessment of Chunking Method Based on Characteristics of the Time Of Use and Non Time Of Use Populations (delete) 16.1.4, Revisions to Time Of Use Load Profile Methods if Changes Are Needed (delete) 16.2, Other Supplemental Load Profiles (delete) 17.1, Introduction 17.2, Interval Data Recorder Requirement 17.3, Demand Meter Changes 17.4, Load Research Samples 17.5, Metering for Supplemental Load Profiling 18, ACCESS TO LOAD PROFILING MATERIALS 19.1, Definitions 19.2, Acronyms Appendix B Appendix C Appendix D, Profile Decision Tree Appendix E, Profile Model Spreadsheets |  |
| Related Documents Requiring Revision/Related Revision Requests |  | Include title of document to be revised (i.e. Operating Guide, Telemetry Standards, etc.) or related Revision Request number and title. |  |
| Revision Description |  | Describe the basic function of the Revision Request. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | Describe qualitative benefits (Examples: satisfies regulatory requirements, data transparency enhancement, etc.), quantitative benefits (benefit calculations), impacts to market segments and other information relating to the impacts or benefits of the LPGRR. |  |

| Sponsor |  |
| --- | --- |
| Name |  |
| E-mail Address |  |
| Company |  |
| Phone Number |  |
| Cell Number |  |
| Market Segment |  |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Lindsay Butterfield |
| E-Mail Address | Lindsay.Butterfield@ercot.com |
| Phone Number | 512-248-6521 |

Proposed Guide Language Revision

# 1       INTRODUCTION

1.1	Purpose of the Load Profiling Guide

Load Profiling within the ERCOT market is the practice of estimating 15-minute interval Load for Customers who do not have devices that measure interval consumption.  Load Profiling enables the participation of these Customers in the ERCOT market.  This practice shall be conducted in a way that attempts to minimize the Load Profile’s contribution to Unaccounted for Energy (UFE) by the Load Profile overall Settlement Intervals and that no unfair advantage is given to any Market Participant.

- 1.2	Document Purpose

(1)	The purpose of the Electric Reliability Council of Texas (ERCOT) Load Profiling Guide (LPG) is to explicate the language and intent in the Protocols that affect Load Profiling.  It is not a substitute for the ERCOT Protocols or the Public Utility Commission of Texas (PUCT) Substantive Rules.  Each Market Participant shall comply with the Protocols and the PUCT Substantive Rules.  In the event of a conflict of Protocols or PUCT Substantive Rules, the Protocols and PUCT Substantive Rules take precedence over the LPG.

(2)	This LPG may be updated monthly.  The most recent version of this LPG is posted on the ERCOT website.

# 2       LOAD PROFILING GUIDE REVISION PROCESS

- 2.1	Introduction

(1)	A request to make additions, edits, deletions, revisions, or clarifications to this Load Profiling Guide (LPG), including any attachments and exhibits to this LPG, is called a Load Profiling Guide Revision Request (LPGRR).  Except as specifically provided in other sections of this LPG, this Section 2, Load Profiling Guide Revision Process, shall be followed for all LPGRRs.  ERCOT Members, Market Participants, Public Utility Commission of Texas (PUCT) Staff, ERCOT, and any other Entities are required to utilize the process described herein prior to requesting, through the PUCT or other Governmental Authority, that ERCOT make a change to this LPG, except for good cause shown to the PUCT or other Governmental Authority.

(2)	The “next regularly scheduled meeting” of the Profiling Working Group (PWG), Commercial Operations Subcommittee (COPS), Technical Advisory Committee (TAC), or the ERCOT Board shall mean the next regularly scheduled meeting for which required Notice can be timely given regarding the item(s) to be addressed, as specified in the appropriate ERCOT Board or committee procedures.

(3)	Throughout the LPG, references are made to the ERCOT Protocols.  ERCOT Protocols supersede the LPG and any LPGRRs must be compliant with the ERCOT Protocols.  The ERCOT Protocols are subject to the revision process outlined in Protocol Section 21, Process for Nodal Protocol Revision.

(4)	ERCOT may make non-substantive corrections at any time during the processing of a particular LPGRR.  Under certain circumstances, however, the LPG can also be revised by ERCOT rather than using the LPGRR process outlined in this Section.

(a)	This type of revision is referred to as an “Administrative LPGRR” or “Administrative Changes” and shall consist of non-substantive corrections, such as typos (excluding grammatical changes), internal references (including table of contents), improper use of acronyms, references to ERCOT Protocols, PUCT Substantive Rules, the Public Utility Regulatory Act (PURA), North American Electric Reliability Corporation (NERC) regulations, Federal Energy Regulatory Commission (FERC) rules, etc., and revisions for the purpose of maintaining consistency between Section 2, Load Profiling Guide Revision Process, and Protocol Section 21, Revision Request Process.

(b)	ERCOT shall post such Administrative LPGRRs on the ERCOT website and distribute the LPGRRs to the PWG at least ten Business Days before implementation.  If no Entity submits comments to the Administrative LPGRR in accordance with paragraph (1) of Section 2.4.3, Profiling Working Group Review and Action, ERCOT shall implement it according to paragraph (4) of Section 2.7, Revision Implementation.  If any ERCOT Member, Market Participant, PUCT Staff, or ERCOT submits comments to the Administrative LPGRR, then it shall be processed in accordance with the LPGRR process outlined in this Section 2.

- 2.2	Submission of Load Profiling Guide Revision Request

The following Entities may submit a Load Profiling Guide Revision Request (LPGRR):

(a)	Any Market Participant;

(b)	Any ERCOT Member;

(c)	Public Utility Commission of Texas (PUCT) Staff;

(d)	ERCOT; and

(e)	Any other Entity who resides (or represents residents) in Texas or operates in the Texas electricity market.

- 2.3	Profiling Working Group

(1)	The Profiling Working Group (PWG) shall review and recommend action on formally submitted Load Profiling Guide Revision Requests (LPGRRs) provided that:

(a)	PWG meetings are open to ERCOT, ERCOT Members, Market Participants, and the Public Utility Commission of Texas (PUCT) Staff; and

(b)	Each Market Segment is allowed to participate.

(2)	Where additional expertise is needed, the PWG may request that the Commercial Operations Subcommittee (COPS) refer an LPGRR to existing Technical Advisory Committee (TAC) subcommittees, working groups or task forces for review and comment on the LPGRR.  Suggested modifications or alternative modifications if a consensus recommendation is not achieved by a non-voting working group or task force, to the LPGRR should be submitted by the chair or the chair’s designee on behalf of the commenting TAC subcommittee, working group or task force as comments on the LPGRR for consideration by the PWG.  However, the PWG shall retain ultimate responsibility for the processing of all LPGRRs.

(3)	The PWG shall ensure that the Load Profiling Guide (LPG) is compliant with the ERCOT Protocols.  As such, the PWG shall monitor all changes to the ERCOT Protocols and initiate any LPGRRs necessary to bring the LPG in conformance with the ERCOT Protocols.  The PWG shall also initiate a Nodal Protocol Revision Request (NPRR) if such a change is necessary to accommodate a proposed LPGRR prior to proceeding with that LPGRR.

(4)	ERCOT shall consult with the PWG chair to coordinate and establish the meeting schedule for the PWG.  The PWG shall meet at least once per month, unless no LPGRRs were submitted during the prior 24 days, and shall ensure that reasonable advance notice of each meeting, including the meeting agenda, is posted on the ERCOT website.

- 2.4	Load Profiling Guide Revision Procedure

- 2.4.1	Review and Posting of Load Profiling Guide Revision Requests

(1)	Load Profiling Guide Revision Requests (LPGRRs) shall be submitted electronically to ERCOT by completing the designated form provided on the ERCOT website.  ERCOT shall provide an electronic return receipt response to the submitter upon receipt of the LPGRR.

(2)	The LPGRR shall include the following information:

(a)	Description of requested revision and reason for suggested change;

(b)	Impacts and benefits of the suggested change on ERCOT market structure, ERCOT operations, and Market Participants to the extent that the submitter may know this information;

(c)	Impact Analysis (applicable only for an LPGRR submitted by ERCOT);

(d)	List of affected Load Profiling Guide (LPG) sections and subsections;

(e)	General administrative information (organization, contact name, etc.); and

(f)	Suggested language for requested revision.

(3)	ERCOT shall evaluate the LPGRR for completeness and shall notify the submitter within five Business Days of receipt if the LPGRR is incomplete, then ERCOT shall include the reasons for such status.  ERCOT may provide information to the submitter that will correct the LPGRR and render it complete.  An incomplete LPGRR shall not receive further consideration until it is completed.  In order to pursue the LPGRR, a submitter must submit a completed version of the LPGRR.

(4)	If a submitted LPGRR is complete or once an LPGRR is completed, ERCOT shall post the LPGRR on the ERCOT website and distribute to the Profiling Working Group (PWG) within three Business Days.

- 2.4.2	Withdrawal of a Load Profiling Guide Revision Request

(1)	A submitter may withdraw or request to withdraw an LPGRR by submitting a completed Request for Withdrawal form provided on the ERCOT website.  ERCOT shall post the submitter’s Request for Withdrawal on the ERCOT website within three Business Days of submittal.

(2)	The submitter of an LPGRR may withdraw the LPGRR at any time before the PWG recommends approval of the LPGRR.  If the PWG has recommended approval of the LPGRR, the Request for Withdrawal must be approved by the Commercial Operations Subcommittee (COPS) if the LPGRR has not yet been recommended for approval by COPS.

(3)	If COPS has recommended approval of the LPGRR, the Request for Withdrawal must be approved by the Technical Advisory Committee (TAC) if the LPGRR has not yet been approved by TAC.

(4)	If TAC has recommended approval of an LPGRR that requires an ERCOT project for implementation, the Request for Withdrawal must be approved by the ERCOT Board if the LPGRR has not yet been approved by the ERCOT Board.

(5)	Once an LPGRR that requires an ERCOT project for implementation is approved by the ERCOT Board or an LPGRR that does not require an ERCOT project for implementation is approved by TAC, such LPGRR cannot be withdrawn.

- 2.4.3	Profiling Working Group Review and Action

(1)	Any ERCOT Member, Market Participant, Public Utility Commission of Texas (PUCT) Staff or ERCOT may comment on the LPGRR.

(2)	To receive consideration, comments must be delivered electronically to ERCOT in the designated format provided on the ERCOT website within 14 days from the posting date of the LPGRR.  Comments submitted after the 14 day comment period may be considered at the discretion of the PWG after these comments have been posted.  Comments submitted in accordance with the instructions on the ERCOT website, regardless of date of submission, shall be posted on the ERCOT website and distributed electronically to the PWG within three Business Days of submittal.

(3)	The PWG shall consider the LPGRR at its next regularly scheduled meeting after the end of the 14 day comment period.  At such meeting, the PWG may take action on the LPGRR.  In considering action on an LPGRR, the PWG may:

(a)	Recommend approval of the LPGRR as submitted or as modified;

(b)	Recommend rejection of the LPGRR;

(c)	If no consensus can be reached on the LPGRR, present options for COPS consideration;

(d)	Defer decision on the LPGRR; or

(e)	Recommend that COPS refer the LPGRR to a subcommittee, working group, or task force as provided in Section 2.3, Profiling Working Group.

(4)	Within three Business Days after the PWG takes action, ERCOT shall issue a PWG Report reflecting the PWG action and post it on the ERCOT website.  The PWG Report shall contain the following items:

(a)	Identification of submitter;

(b)	LPG language recommended by the PWG, if applicable;

(c)	Identification of authorship of comments, if applicable;

(d)	Proposed effective date of the LPGRR;

(e)	Recommended priority and rank for any LPGRRs requiring an ERCOT project for implementation; and

(f)	PWG action.

- 2.4.4	Comments to the Profiling Working Group Recommendation Report

(1)	Any ERCOT Member, Market Participant, PUCT Staff, or ERCOT may comment on the PWG Report.  Within three Business Days of receipt of comments related to the PWG Report, ERCOT shall post such comments on the ERCOT website.  Comments submitted in accordance with the instructions on the ERCOT website, regardless of date of submission, shall be posted on the ERCOT website within three Business Days of submittal..

(2)	The comments on the PWG Report will be considered at the next regularly scheduled PWG or COPS meeting where the LPGRR is being considered.

- 2.4.5	Load Profiling Guide Revision Request Impact Analysis

(1)	ERCOT shall submit to the PWG an initial Impact Analysis based on the original language in the LPGRR with any ERCOT-sponsored LPGRR.  The initial Impact Analysis will provide the PWG with guidance as to what ERCOT computer systems, operations, or business functions could be affected by the LPGRR as submitted.

(2)	If PWG recommends approval of an LPGRR, ERCOT shall prepare an Impact Analysis based on the proposed language in the PWG Report.  If ERCOT has already prepared an Impact Analysis, ERCOT shall update the existing Impact Analysis, if necessary, to accommodate the language recommended for approval in the PWG Report.

(3)	The Impact Analysis shall assess the impact of the LPGRR on ERCOT computer systems, operations, or business functions and shall contain the following information:

(a)	An estimate of any cost and budgetary impacts to ERCOT for both implementation and ongoing operations;

(b)	The estimated amount of time required to implement the LPGRR;

(c)	The identification of alternatives to the LPGRR that may result in more efficient implementation; and

(d)	The identification of any manual workarounds that may be used as an interim solution and estimated costs of the workaround.

(4)	Unless a longer review period is warranted due to the complexity of the proposed PWG Report, ERCOT shall issue an Impact Analysis for an LPGRR for which PWG has recommended approval of prior to the next regularly scheduled PWG meeting.  ERCOT shall post the results of the completed Impact Analysis on the ERCOT website.  If a longer review period is required by ERCOT to complete an Impact Analysis, ERCOT shall submit comments with a schedule for completion of the Impact Analysis to the PWG.

- 2.4.6	Profiling Working Group Review of Impact Analysis

(1)	After ERCOT posts the results of the Impact Analysis, the PWG shall review the Impact Analysis at its next regularly scheduled meeting.  The PWG may revise its PWG Report after considering the information included in the Impact Analysis or additional comments received on the PWG Report.

(2)	After consideration of the Impact Analysis and the PWG Report, ERCOT shall issue a revised PWG Report and post it on the ERCOT website within three Business Days of the PWG consideration of the Impact Analysis and PWG Report.  If the PWG revises the proposed LPGRR, ERCOT shall update the Impact Analysis, if necessary and issue the updated Impact Analysis to COPS.  If a longer review period is required for ERCOT to update the Impact Analysis, ERCOT shall submit comments with a schedule for completion of the Impact Analysis to COPS.

(3)	If the LPGRR requires an ERCOT project for implementation, at the same meeting the PWG shall assign a recommended priority and rank for the associated project.

- 2.4.7	Commercial Operations Subcommittee Vote

(1)	COPS shall consider any LPGRRs that the PWG has submitted to COPS for consideration for which both a PWG Report and an Impact Analysis (as updated if modified by the PWG under Section 2.4.6, Profiling Working Group Review of Impact Analysis) have been posted on the ERCOT website.  The following information must be included for each LPGRR considered by COPS:

(a)	The PWG Report and Impact Analysis; and

(b)	Any comments received in timely manner in response to the PWG Report.

(2)	The quorum and voting requirements for COPS action are set forth in the Technical Advisory Committee Procedures.  In considering action on a PWG Report, COPS shall:

(a)	Recommend approval of the LPGRR as recommended in the PWG Report or as modified by COPS;

(b)	Reject the LPGRR;

(c)	Defer decision on the LPGRR;

(d)	Remand the LPGRR to the PWG with instructions; or

(e)	Refer the LPGRR to another COPS working group or task force or another TAC subcommittee with instructions.

(3)	If a motion is made to recommend approval of an LPGRR and that motion fails, the LPGRR shall be deemed rejected by COPS unless at the same meeting COPS later votes to recommend approval of, defer,  remand, or refer the LPGRR.  If a motion to recommend approval of an LPGRR fails via e-mail vote according to the Technical Advisory Committee Procedures, the LPGRR shall be deemed rejected by COPS unless at the next regularly scheduled COPS meeting or in a subsequent e-mail vote prior to such meeting, COPS votes to recommend approval of, defer, remand, or refer the LPGRR.  The rejected LPGRR shall be subject to appeal pursuant to Section 2.5, Appeal of Action.

(4)	Within three Business Days after COPS takes action on the LPGRR, ERCOT shall issue a COPS Report reflecting the COPS action, and post the report on the ERCOT website.  The COPS Report shall contain the following items:

(a)	Identification of the submitter of the LPGRR;

(b)	Modified LPG language proposed by COPS, if applicable;

(c)	Identification of the authorship of comments, if applicable;

(d)	Proposed effective date(s) of the LPGRR;

(e)	Recommended priority and rank for any LPGRR requiring a an ERCOT project for implementation;

(f)	PWG action; and

(g)	COPS action.

- 2.4.8	ERCOT Impact Analysis Based on Commercial Operations Subcommittee Report

ERCOT shall review the COPS Report and, if necessary, update the Impact Analysis as soon as practicable.  ERCOT shall issue the updated Impact Analysis, if applicable, to TAC and post it on the ERCOT website.  If a longer review period is required for ERCOT to update the Impact Analysis, ERCOT shall submit comments with a schedule for completion of the Impact Analysis to TAC.

- 2.4.9	Protocol Revision Subcommittee Review of Project Prioritization

At the next regularly scheduled Protocol Revision Subcommittee (PRS) meeting after COPS recommends approval of an LPGRR that requires an ERCOT project for implementation, the PRS shall assign a recommended priority and rank for the associated project.

- 2.4.10	Technical Advisory Committee Vote

(1)	TAC shall consider any LPGRR that COPS has submitted to TAC for consideration for which both a COPS Report and an Impact Analysis (as updated if modified by COPS under Section 2.4.8, ERCOT Impact Analysis Based on Commercial Operations Subcommittee Report) have been posted on the ERCOT website.  The following information must be included for each LPGRR considered by TAC:

(a)	The COPS Report and Impact Analysis;

(b)	The recommended priority and rank, if an ERCOT project is required; and

(c)	Any comments timely received in response to the COPS Report.

(2)	The quorum and voting requirements for TAC action are set forth in the Technical Advisory Committee Procedures.  In considering action on a COPS Report, TAC shall:

(a)	Approve the LPGRR as recommended in the COPS Report or as modified by TAC, if the LPGRR does not require an ERCOT project for implementation;

(b)	Recommend approval of the LPGRR as recommended in the COPS Report or as modified by TAC, including modification of the recommended priority and rank if the LPGRR requires an ERCOT project for implementation;

(c)	Reject the LPGRR;

(d)	Defer decision on the LPGRR;

(e)	Remand the LPGRR to COPS with instructions; or

(f)	Refer the LPGRR to another TAC subcommittee or a TAC working group or task force with instructions.

(3)	If a motion is made to approve or recommend approval of an LPGRR and that motion fails, the LPGRR shall be deemed rejected by TAC unless at the same meeting TAC later votes to approve, recommend approval of, defer, remand or refer the LPGRR.  If a motion to approve or recommend approval of an LPGRR fails via e-mail vote according to the Technical Advisory Committee Procedures, the LPGRR shall be deemed rejected by TAC unless at the next regularly scheduled TAC meeting or in a subsequent e-mail vote prior to the such meeting, TAC votes to approve, recommend approval of, defer, remand, or refer the LPGRR.  The rejected LPGRR shall be subject to appeal pursuant to Section 2.5, Appeal of Action.

(4)	Within three Business Days after TAC takes action on an LPGRR, ERCOT shall issue a TAC Report reflecting the TAC action and post it on the ERCOT website.  The TAC Report shall contain the following items:

(a)	Identification of the submitter of the LPGRR;

(b)	Modified LPG language proposed by TAC, if applicable;

(c)	Identification of the authorship of comments, if applicable;

(d)	Proposed effective date(s) of the LPGRR;

(e)	Priority and rank for any LPGRR requiring an ERCOT project for implementation;

(f)	COPS action;

(g)	TAC action; and

(h)	ERCOT’s position for any LPGRR requiring an ERCOT project for implementation.

(5)	If TAC recommends approval of an LPGRR requiring an ERCOT project for implementation, ERCOT shall forward the TAC Report, to the ERCOT Board for consideration pursuant to Section 2.4.11, ERCOT Board Vote.

(6)	The TAC chair shall report the results of all votes by TAC related to LPGRRs to the ERCOT Board at its next regularly scheduled meeting.

- 2.4.11	ERCOT Board Vote

(1)	For any LPGRR requiring an ERCOT project for implementation, upon issuance of a TAC Report and Impact Analysis to the ERCOT Board, the ERCOT Board shall review the TAC Report and the Impact Analysis at the following month’s regularly scheduled meeting.  For Urgent LPGRRs, the ERCOT Board shall review the TAC Report and Impact Analysis at the next regularly scheduled meeting, unless a special meeting is required due to the urgency of the LPGRR.

(2)	The quorum and voting requirements for ERCOT Board action are set forth in the ERCOT Bylaws.  In considering action on a TAC Report, the ERCOT Board shall:

(a)	Approve the LPGRR as recommended in the TAC Report or as modified by the ERCOT Board;

(b)	Reject the LPGRR;

(c)	Defer decision on the LPGRR; or

(d)	Remand the LPGRR to TAC with instructions.

(3)	If a motion is made to approve an LPGRR and that motion fails, the LPGRR shall be deemed rejected by the ERCOT Board unless at the same meeting the ERCOT Board later votes to approve, defer or remand the LPGRR.  The rejected LPGRR shall be subject to appeal pursuant to Section 2.5, Appeal of Action.

(4)	Within three Business Days after the ERCOT Board takes action on an LPGRR, ERCOT shall issue a Board Report reflecting the ERCOT Board action and post it on the ERCOT website.

- 2.5	Appeal of Action

(1)	Any ERCOT Member, Market Participant, Public Utility Commission of Texas (PUCT) Staff, or ERCOT may appeal a Profiling Working Group (PWG) action to recommend rejection of, defer, or recommend referral of a Load Profiling Guide Revision Request (LPGRR) directly to the Commercial Operations Subcommittee (COPS).  Such appeal to COPS must be submitted electronically to ERCOT by completing the designated form provided on the ERCOT website within seven days after the date of the relevant PWG appealable event.  ERCOT shall reject appeals made after that time.  ERCOT shall post appeals on the ERCOT website within three Business Days of receiving the appeal.  Appeals shall be heard at the next regularly scheduled COPS meeting that is at least seven days after the date of the requested appeal.  An appeal of an LPGRR to COPS suspends consideration of the LPGRR until the appeal has been decided by COPS.

(2)	Any ERCOT Member, Market Participant, PUCT Staff, or ERCOT may appeal a COPS action to reject, defer, remand or refer an LPGRR directly to the Technical Advisory Committee (TAC).  Such appeal to TAC must be submitted electronically to ERCOT by completing the designated form provided on the ERCOT website within seven days after the date of the relevant COPS appealable event.  ERCOT shall reject appeals made after that time.  ERCOT shall post appeals on the ERCOT website within three Business Days of receiving the appeal.  Appeals shall be heard at the next regularly scheduled TAC meeting that is at least seven days after the date of the requested appeal.  An appeal of an LPGRR to TAC suspends consideration of the LPGRR until the appeal has been decided by TAC.

(3)	Any ERCOT Member, Market Participant, PUCT Staff, or ERCOT may appeal a TAC action to approve, reject, defer, remand or refer an LPGRR directly to the ERCOT Board.  Appeals to the ERCOT Board shall be processed in accordance with the ERCOT Board Policies and Procedures.  An appeal of an LPGRR to the ERCOT Board suspends consideration of the LPGRR until the appeal has been decided by the ERCOT Board.

(4)	Any ERCOT Member, Market Participant, or PUCT Staff, may appeal any decision of the ERCOT Board regarding the LPGRR to the PUCT or other Governmental Authority.  Such appeal to the PUCT or other Governmental Authority must be made within any deadline prescribed by the PUCT or other Governmental Authority, but in any event no later than 35 days of the date of the relevant ERCOT Board appealable event.  Notice of any appeal to the PUCT or other Governmental Authority must be provided, at the time of the appeal to ERCOT’s General Counsel.  If the PUCT or other Governmental Authority rules on the LPGRR, ERCOT shall post the ruling on the ERCOT website.

- 2.6	Urgent Requests

(1)	The party submitting a Load Profiling Guide Revision Request (LPGRR) may request that the LPGRR be considered on an urgent timeline (“Urgent”) only when the submitter can reasonably show that an existing Load Profiling Guide (LPG) provision is impairing or could imminently impair wholesale or retail market operations, or is causing or could imminently cause a discrepancy between a Settlement formula and a provision of the ERCOT Protocols.

(2)	The Commercial Operations Subcommittee (COPS) may designate the LPGRR for Urgent consideration if a submitter requests Urgent status or upon valid motion in a regularly scheduled meeting of COPS.  Criteria for designating an LPGRR as Urgent are that the LPGRR requires immediate attention due to:

(a)	Serious concerns about ERCOT System reliability or market operations under the unmodified language; or

(b)	The crucial nature of Settlement activity conducted pursuant to any Settlement formula.

(3)	ERCOT shall prepare an Impact Analysis for Urgent LPGRRs as soon as practicable.

(4)	COPS or the Profiling Working Group (PWG) shall consider the Urgent LPGRR and Impact Analysis if available at the next regularly scheduled PWG or COPS meeting, or at a special meeting called by the PWG or COPS chair to consider the Urgent LPGRR.

(5)	If the submitter desires to further expedite processing of the LPGRR, a request for voting via e-mail may be submitted to the COPS chair.  The COPS chair may grant the request for voting via e-mail.  Such voting shall be conducted pursuant to the Technical Advisory Committee Procedures.  If COPS recommends approval of the Urgent LPGRR, ERCOT shall issue a COPS Report to reflecting the COPS action and post it on the ERCOT website within three Business Days after COPS takes action.  The Technical Advisory Committee (TAC) chair may request action from TAC to accelerate or alter the procedures described herein, as needed, to address the urgency of the situation.

(6)	Any LPGRRs that take effect pursuant to an Urgent request shall be subject to an Impact Analysis pursuant to Section 2.4.8, ERCOT Impact Analysis Based on Commercial Operations Subcommittee Report, and TAC consideration pursuant to Section 2.4.10, Technical Advisory Committee Vote.

- 2.7	Revision Implementation

(1)	For Load Profiling Guide Revision Requests (LPGRRs) that do not require an ERCOT project for implementation, upon Technical Advisory Committee (TAC) approval, ERCOT shall implement LPGRRs on the first day of the month following TAC approval, unless otherwise provided in the TAC Report for the approved LPGRR.

(2)	For LPGRRs that require an ERCOT project for implementation, upon ERCOT Board, approval ERCOT shall implement LPGRRs on the first day of the month following the ERCOT Board approval, unless otherwise provided in the Board Report for the approved LPGRR.

(3)	For LPGRRs for which an effective date other than the first day of the month following, TAC or ERCOT Board approval, as applicable, is provided, the ERCOT Impact Analysis shall provide an estimated implementation date and ERCOT shall provide notice as soon as practicable, but no later than ten days prior to actual implementation, unless a different notice period is required in the TAC or Board Report, as applicable, for the approved LPGRR.

(4)	ERCOT shall implement an Administrative LPGRR on the first day of the month following the end of the ten Business Day posting requirement outlined in Section 2.1, Introduction.

# 3	[ RESERVED]

# 4       THE PROFILING WORKING GROUP

The Profiling Working Group (PWG) is a standing informal, open working group that provides technical support to the Commercial Operations Subcommittee (COPS) on Load Profiling issues.

- 4.1	Purpose of the Profiling Working Group

The Profiling Working Group (PWG) is a forum in which Market Participants  may participate to facilitate changes in the market rules pertaining to Load Profiling issues as reflected in the Protocols and the Load Profiling Guide (LPG).  The PWG shall be involved in all policy issues and some operational aspects of Load Profiling in the ERCOT market.

- 4.2	Profiling Working Group Responsibilities

The PWG has several responsibilities and duties, which include the following:

(a)	Maintains and upholds Protocol Section 18, Load Profiling;

(b)	Reviews all requests for changes to Load Profiles, Load Profiling Methodologies, and implementation of the Load Profiling process;

(c)	Reviews and makes recommendations to the Commercial Operations Subcommittee (COPS) regarding the Load Profiling Guide (LPG) change control, Load Profile Models, and Load Profile Methodologies;

(d)	Reviews and makes recommendations to Appendix D, Profile Decision Tree;

(e)	Participates in defining Weather Zones and Load Profile types;

(f)	Evaluates the validation and assignment processes for Load Profile IDs;

(g)	Evaluates the impact of the Interval Data Recorder (IDR) requirement for possible revision prior to retail metering;

(h)	Periodically reviews the selected profiling technique for Time-Of-Use (TOU);

(i)	Coordinates with ERCOT in developing Load Profiles for particular Customer segments that may require special Load Profiling techniques (e.g., supplemental Load Profiles);

(j)	Develops and maintains the LPG;

(k)	Reviews and makes recommendations to the ERCOT Load Profiling Department on Load Research Sample Design;

(l)	Performs a liaison function between Market Participants and the ERCOT Load Profiling Department and facilitates market acceptance of Load Profiling processes; and

(m)	Provides a forum for Market Participants to be involved with ERCOT Load Profiling.

- 4.3	Profiling Working Group Reporting Structure

(1)	At the time of the development of the Load Profiling Guide (LPG), the Profiling Working Group (PWG) reported to the Commercial Operations Subcommittee (COPS), which is a standing subcommittee of Technical Advisory Committee (TAC).  The PWG chair and the PWG vice-chair are elected annually by the PWG on a calendar year basis.  The chair leads the PWG meeting, establishes the PWG meeting dates and frequency, and represents the PWG at COPS and other ERCOT forums, as necessary.  The vice-chair’s primary responsibilities are to perform the chair’s duties in the absence of the chair.  The PWG shall continue to meet at least quarterly to review profiling processes and profiling issues.

(2)	To obtain current reporting structure information, please refer to the following website: http://www.ercot.com/committees/index.html.

- 4.4	Profiling Working Group Membership

The Profiling Working Group (PWG) membership is open to all Market Participants and any other interested parties (e.g., consultants, Non-Opt-In Entities (NOIEs), future Market Participants, and Public Utility Commission of Texas (PUCT) Staff).  All Market Participants are invited to attend all PWG meetings.

- 4.5	Profiling Working Group Contact Information

(1)	To begin receiving electronic mail related to the Profiling Working Group (PWG), subscribe to the PWG electronic mailing list at http://lists.ercot.com/.

(2)	To discontinue receiving electronic mail related to the PWG, unsubscribe from the PWG electronic mailing list at http://lists.ercot.com/.

(3)	The ERCOT Load Profiling Department may also assist with contact information.

# LOAD PROFILING METHODOLOGY

- 6.1	Introduction

(1)	This Section 6, Load Profiling Methodology, of the Load Profiling Guide (LPG) describes  a request for an evaluation of the Load Profiling Methodologies or a change to a Load Profiling Methodology as specified in Protocol Section 18.2.5, Adjustments and Changes to Load Profile Development.

(

(2)	Any changes to the Load Profiling Methodology shall be submitted as a Load Profiling Revision Request (LPGRR) as described in Section 2.4, Load Profiling Guide Revision Procedure.

(3)	There shall be no retroactive application of any approved modifications to Load Profiling Methodology.

- 6.2	Current Methodologies

The following methodologies are used to establish Load Profiles:

| Type of Load | Load Profiling Methodology |
| --- | --- |
| Non-Price-Responsive |  |
| Non-interval metered | Adjusted Static Models |
| Non-interval metered with Distributed Generation (DG) | Adjusted Static Models and Engineering Estimates |
| Non-metered | Engineering Estimates |
| Interval Data Recorder (IDR) (Estimation) | Proxy day |
| Price-Responsive |  |
| Other price-responsive | To be determined |

- 6.3	Request for Load Profiling Methodology Review and Changes

ny Market Participant, the Profiling Working Group (PWG), or the appropriate TAC Subcommittee may submit a request for a review and change to the Load Profiling Methodology according to the procedures outlined in the Load Profiling Guide (LPG).  When special circumstances warrant, a more immediate review may be necessary.  The findings of all Load Profiling Methodology reviews shall be presented to the Profiling Working Group (PWG) for consideration.

- 6.4	Timeline for Processing a Load Profiling Methodology Review and Change Request

(1)	This Section 6.4, Timeline for Processing a Load Profiling Methodology Review and Change Request, modifies the normal Load Profiling Guide Revision Request (LPGRR) change request timeline.  Within two Business Days of receiving the request, ERCOT shall reply to the submitter indicating that the request has been received and inform the submitter of the dates of the next Profiling Working Group (PWG) meetings.  The submitter shall then schedule a time to present the request, in person, to the PWG and ERCOT at a regularly scheduled PWG meeting.

(2)	The submitter or a designated representative shall present the methodology change request, in person, to the PWG at a scheduled PWG meeting.  During the submitter’s presentation, ERCOT and the PWG may ask for clarification of the request.  The PWG and ERCOT shall then determine what data and supporting documentation are needed from the submitter to evaluate the request.  All data, supporting files, and documentation shall be provided in electronic form.

(3)	After the request has been presented to the PWG, ERCOT shall post the methodology request to the ERCOT website and respond to the request within 60 days of the posted date of the request.  This period does not include the time to analyze and render the complete assessment of the request. The response shall indicate:

(a)	Whether the request is complete;

(b)	What additional data is required to evaluate the request, if applicable;

(c)	How the request shall be assessed;

(d)	An estimate of the time by which a decision on the request is expected to be ready; and

(e)	An estimate of the implementation date of the requested change, if approved.

(4)	During ERCOT’s evaluation of the request, ERCOT may request supplemental information determined to be important to fully evaluate the methodology change.

(5)	Due to the significance of a change to Load Profiling Methodologies, according to Protocol Section 18.2.5, Adjustments and Changes to Load Profile Development, a change shall only be implemented after Technical Advisory Committee (TAC) approval and with at least 150 days’ notice to all Market Participants.  An exception may be made to the criteria defined in this section, if special circumstances indicate a need to implement a change more immediately to address critical market issues.

- 6.5	Information Required with Request for Change

(1)	The submitter shall describe the reason why a change to methodology is necessary, why the proposed methodology is superior to the current methodology, and how the benefits of the change outweigh the costs to implement the proposed methodology.

(2)	The submitter shall identify the following:

(a)	The Entity submitting the request;

(b)	Contact information;

(c)	The current methodology to be modified;

(d)	The proposed methodology or modification(s) proposed to the current methodology; and

(e)	The affected Load Profile Segment(s) and Weather Zone(s).

(3)	The submitter shall include pertinent supporting data with the initial request to ERCOT.  Examples include the following:

(a)	Analysis of data available in ERCOT systems (e.g., Load research data, weather data from weather stations used by ERCOT Load Profiling, and monthly consumption data).  The submitter shall document data sources in detail and show analysis of any factors listed above to be considered in the evaluation.

(b)	Analysis of Load research data not available to ERCOT.  The submitter shall document data sources in detail, describe how the data was collected, document any data Validation, Editing, and Estimating (VEE) that has been performed, and describe the analysis.

(c)	Analysis of other data or other supporting evidence.  The submitter shall document data sources and present the associated analysis.

(4)	The submitter shall also provide evidence that:

(a)	The current profiles have substantial bias;

(b)	The proposed alternative mitigates the problem(s);

(c)	The change in methodology is warranted due to the severity of the problem(s) with the current profiles; and/or

(d)	The proposed alternative methodology corrects the problem(s) with the current profiles efficiently and cost-effectively.

- 6.6	Evaluation of the Request

ERCOT shall assess the request based on the data and analysis submitted with the request as well as possible additional analysis by ERCOT.  Factors considered in assessing any request shall include:

(a)	The quality of the supporting data provided;

(b)	The magnitude of differences indicated;

(c)	The size of the affected population; and

(d)	The effect on the rest of the market if the change is accepted.

- 6.7	Considerations for Load Profiling Methodology Evaluation

The evaluation shall consider the following factors, which is neither an exclusive nor an exhaustive list:

(a)	Load Profile Model performance;

(b)	Methodology performance;

(c)	Alternative methodology impacts to Load Profiling issues; and

(d)	Practical implementation of Load Profiling Methodology.

- 6.7.1		Load Profile Model Performance

Model performance serves as a basis for evaluating Load Profiling Methodology.  The result of Load Profile Model performance evaluations shall help determine if a methodology modification is necessary.  Load Profile Model performance shall be evaluated according to Section 8, Load Profile Models.

- 6.7.2		Methodology Performance

The performance of alternative Load Profiling Methodologies shall be assessed according to the evaluation criteria presented in Section 8, Load Profile Models.

- 6.7.3		Alternative Methodology Impacts to Load Profiling Issues

The effect of the proposed alternative methodology on any Load Profiling issues requiring resolution shall be considered when evaluating the methodology.  Alternative Load Profiling Methodologies may mitigate, intensify or have no effect on these issues.  These effects shall be assessed for probability and manageability. Some effects of the alternative methodology may include the following:

(a)	Unusual events that affect the ERCOT System;

(b)	Dramatic changes in a relatively short period of time;

(c)	Sensitivity of the methodology to random error;

(d)	Changes to data quality; and

(e)	Impacts to the cost.

- 6.7.4		Practical Implementation of Load Profiling Methodology

The practical implementation of a Load Profiling Methodology is a key-determining factor.  The time and the resources needed to implement the change may make the proposed methodology prohibitive.  Additional issues that may be considered are:

(a)	Alternative changes (e.g., changes to models), which may provide the Market Participants the desired result; and

(b)	The complexity of implementation and operational production (e.g., system functionality) for ERCOT and Market Participants.

- 6.8	Possible Results of the Evaluation of Methodologies

The following are possible resolutions of requests to review and change Load Profiling Methodologies:

(a)	No changes to Load Profiling Methodologies;

(b)	Modify existing Load Profiling Methodology; and

(c)	Implement alternative Load Profiling Methodology.

- 6.8.1		No Changes to Load Profiling Methodologies

The evaluation of the methodology may conclude that no changes are needed.  Another outcome of the evaluation may indicate that adjustments to model coefficients are needed for specified segments and/or Weather Zones.  Either case shall be resolved by not altering the current Load Profiling Methodology.

- 6.8.2		Modify Existing Load Profiling Methodology

During  the evaluation, significant biases may be exposed which require major changes such as re-estimating models, changing Weather Zones, or changing segments.  In such cases, modifying the existing Load Profiling Methodology may be employed as a practical resolution.  The Profiling Working Group (PWG) shall determine “significant biases” with market experience.

- 6.8.3		Implement Alternative Load Profiling Methodology

If the evaluation indicates that substantial biases exist, and that these biases are unlikely to be mitigated or are likely to be increased by reasonable modifications to the existing methodology, a more comprehensive change to an alternative Load Profiling Methodology shall be considered.  The likely effects on these biases and other processing issues shall be determining factors in the decision to adopt a new methodology.

- 6.9	Approval of the Request

The Technical Advisory Committee (TAC) approval is required to implement any change to a Load Profiling Methodology in accordance with Protocol Section 18.2, Methodology. The request shall follow the approval sequence described in Section 12, Request for Load Profile Segment Changes, Additions, or Removals.

- 6.10	Costs for Load Profiling Methodology Changes

(1)	The party requesting the methodology change shall pay all costs associated with developing the supporting data and documentation submitted to ERCOT for evaluation.

(2)	In the event the methodology change is approved, costs for implementing the changes in ERCOT data systems shall be the responsibility of ERCOT.  Responsibility for re-assigning Load Profiles remains with the Transmission and/or Distribution Service Provider (TDSP).

to establish Load Profiles:

8       		LOAD PROFILE MODELS

(1)	Protocol Section 18.2.5, Adjustments and Changes to Load Profile Development, defines  evaluation of Load Profiling Methodology  including changes to methodology, adjustments to existing profiles, and development of new profiles.  This Section addresses changes to models within approved methodologies.    ar

(2)	The Microsoft Excel© representation of the ERCOT Load Profile Models can be found in Appendix E, Load Profile Model Spreadsheets.

(3)	There shall be no retroactive application of any approved modifications to Load Profile Models.

(4)	This Section discusses changes to Load Profile Models not addressed in the following the Load Profiling Guide (LPG) sections:

(a)	Section 6, Load Profiling Methodology;

(b)	Section 12, Request for Load Profile Segment Changes, Additions, or Removals; and

(c)	Section 13, Changes to Weather Zone Definitions.

8.1	Load Profile Model Evaluations

(1) 	Upon request from the appropriate TAC Subcommittee, ERCOT shall perform evaluations of Load Profile Model performance for all Load Profile Models and Load Profile Types,using industry standard practices as described in The Association of Edison Illuminating Companies (AEIC) Load Research Manual..  Any changes resulting from these evaluations shall be recommended to the Profiling Working Group (PWG). Ea

8.1.2		Evaluation of Weather Zones

(1) 	Assessment of Weather Zone definition shall focus on the adequacy of the current set of weather stations and weighting.  ERCOT uses National Oceanic and Atmospheric Administration (NOAA) first or second order weather stations as the source  for weather data for each Weather Zone, wher available.  Weather Zone evaluation willingindustry standard practices as described in The Association of Edison Illuminating Companies (AEIC) Load Research Manual. Assessment steps of the evaluation of each Weat8.2	Evaluatingon of Load Profile Models Using Current Load Research Data

8.2.1	Sources  d Research Data

(1)	Load research data may be  collected by ERCOT developed Linterval data and from any available Transmission and/or Distribution Service Provider’s (TDSP) Load research samples.

(2)	In certain circumstances, Load research data from other sources may also be considered by ERCOT as representative of a particular subgroup.  For such data to be used, the party submitting the data for use in an evaluation shall provide information on the source of the data.  Submission requirements are the same as those described in Section 12.6, Information Required with Request for Change.

8.2.2	Procedures

(1)	The overall procedure for comparing existing Load Profile Models against current Load research data shall use industry standard practices as described in The Association of Edison Illuminating Companies (AEIC) Load Research Manual.

Bf

8.3	Evaluating Load Profile Models without Current Load Research Data

8.3.1		Applications

(1)	In some situations, current Load research data may not be available as a basis for assessing the performance of Load Profile Models.  Situations where techniques are required that do not depend on Load research data include:

(a)	Assessing model performance for geographic areas where Load research data are no longer collected;

(b)	Assessing model performance for geographic areas where Load research data have never been collected, or have not contributed to current models; and

(c)	Assessing Engineering Estimates.

(2)	These techniques may also be used as another way of assessing model performance even for geographic areas where current Load research data are available.

8.3.2		Load Profile Model Comparisons

8.3.2.1	Comparisons for Adjusted Static Models

(1)	For premises with an Advanced Meter installed, Adjusted Static Models may be assessed based on techniques described in The Association of Edison Illuminating Companies (AEIC) Load Research Manual.populati

8.3.2.3	Comparisons for Engineering Estimates

(1)	Engineering Estimates are used in the ERCOT market for Non-Metered Loads, such as lighting.  Engineering Estimates are typically based on an assumed fixed operating schedule together with the assumption that the Load is approximately the same whenever the equipment is operating.  If more current information is available for the ESI IDs in a Load Profile Segment using an engineering Load Profile, this information may be compared with the assumptions of the estimate.

(2)	Monthly consumption data may also be compared with the Load Profile monthly patterns using the methods described above for Adjusted Static Models.

8.4	Requesting Load Profile Model Evaluations or Changes

(1)	This Section describes the procedures for requesting changes to Load Profile Models.  Procedures for requesting changes to Load Profile Segments are described in Section 12, Request for Load Profile Segment Changes, Additions, or Removals.  Procedures for requesting changes to Weather Zones are described in Section 13, Changes to Weather Zone Definitions.  Upon request from the appropriate TAC Subcommittee, ERCOT will evaluate Load Profile Models using industry standard practices as described in The Association of Edison Illuminating Companies (AEIC) Load Research Manual.

(2)	The following Entities may submit requests for Load Profile Model changes:

(a)	Any Market Participant;

(b)	Any Entity that is an ERCOT Member;

(c)	Public Utility Commission of Texas (PUCT) Staff;

(d)	ERCOT Staff; and

(e)	Any other Entity who resides (or represent residents) in Texas or operates in the Texas electricity market.

(3)	Requests for Load Profile Model changes shall be submitted to the Profiling Working Group (PWG) and are subject to approval as outlined in Section 8.5.

(4)	Requests for Load Profile Model Changes shall include the following:

(a)	Identifying the party making the request, with contact information;

(b)	Identifying the Load Profile Segment(s) and Weather Zone(s) affected; and

(c)	Describe why the evaluation is needed.

8.5	Approval Process for Load Profile Model Changes

(1)	For any proposed Load Profile Model changes, the Profiling Working Group (PWG) shall propose changes to the appropriate Technical Advisory Committee (TAC) subcommittee.

(2)	Load Profile Model changes shall be accompanied by an implementation plan to mitigate the impact of transitioning between old and new Load Profile Models.  The implementation plan shall be approved by TAC and the appropriate TAC subcommittee.

# 9      LOAD PROFILE IDS

- 9.1	Assignment of Load Profile IDs

Transmission and/or Distribution Service Providers (TDSPs) are responsible for initially assigning the Load Profile IDs of all Electric Service Identifiers (ESI IDs), as well as any changes in assignment. ERCOT is responsible for calculating the Load Profile Segment for the Load Profile ID as defined by the Annual Validation process in Section 11.2, Annual Validation of Load Profile Type.  The Profile Decision Tree is a dynamic Microsoft Office Excel© file (see Appendix D) that contains the directions to use when assigning Load Profile IDs to ESI IDs.

- 9.1.1		Profile Decision Tree Revision and Approval Process

(1)	All revisions to the Profile Decision Tree shall be submitted through the Load Profiling Guide Revision Request (LPGRR) process described in Section 2, Load Profiling Guide Revision Process.  ERCOT may use an administrative LPGRR to revise the contents of the following Profile Decision Tree tabs:

(a)	FAQ – frequently asked questions related to the assignment of Load Profile IDs;

(b)	Use of Components – information about how each component of the Load Profile ID is used by ERCOT in the Settlement process;

(c)	ZipToZone – a table that maps Zone Improvement Plan (ZIP) Codes to Weather Zones;

(d)	Valid Profile IDs – a list of all Load Profile IDs that can be assigned to ESI IDs that are within the ERCOT region;

(e)	Non-ERCOT Profile IDs – a list of Load Profile IDs that can be assigned to ESI IDs that are within Texas, but outside of the ERCOT region; and

(f)	NOIEs – directions for Non-Opt-In Entities (NOIEs) to use in determining Load Profile ID assignments.

- 9.1.2		Assignment of Load Profile IDs for New Service Delivery Points

TDSPs shall create and submit ESI IDs as new Service Delivery Points (SDPs) are established.  It is the responsibility of the TDSP to make the Load Profile ID assignment for each new ESI ID.  To assign the Load Profile Type for new ESI IDs, the TDSP shall assign the default Load Profile Segment designated in Appendix D, Profile Decision Tree, on the “Segment Assignment” worksheet.

- 9.1.3	Assignment of Load Profile IDs for New Electric Service Identifiers Resulting from a Mass Transition

When a Mass Transition involves moving SDPs from one TDSP to another, the gaining TDSP creates and submits ESI IDs for all gained SDPs.  To assign the Load Profile ID for new ESI IDs, the gaining TDSP shall obtain the current Load Profile ID assignment from either the losing TDSP or ERCOT.  For detailed information on the Mass Customer Transition Process, please refer to Retail Market Guide.

- 9.1.4		Assignment of BUSOGFLT Profile Type

(1)	Competitive Retailers (CRs) seeking to have the Oil & Gas Flat (OGFLT) Profile Segment assigned to one of their Business (BUS) ESI IDs shall follow the instructions on the Oil & Gas tab of Appendix D, Profile Decision Tree.

(2)	ERCOT shall review all assignments of the BUSOGFLT Profile Type on an annual basis, per Section 11.3.1, Validation of BUSOGFLT Profile Type.

- 9.1.5		Assignment of Load Profile IDs for Distributed Generation

(1)	CRs seeking to have the profile segments for Photovoltaic, wind or other Distributed Generation (DG) assigned to one of their Residential (RES) or Business (BUS) ESI IDs shall follow the instructions on the “DG” tab of Appendix D, Profile Decision Tree.

(2)	ERCOT shall review all assignments of the Load Profile Segments for Photovoltaic, wind, and other DG on an annual basis, per Section 11.3.3, Validation of Profile Segments for Distributed Generation.

- 9.1.6		kVA Metered Loads

Any TDSP that routinely measures kVA Demand instead of kW Demand shall coordinate with the PWG to determine the Power Factor that shall be used to estimate their kW Demand, in accordance with Section 10, kVA to kW Conversion.  Approved Power Factors are listed in Appendix D, Profile Decision Tree.

- 9.1.7	Load Profile ID Assignment for Non-ERCOT Electric Service Identifiers

(1)	TDSPs are required to assign ESI IDs for all SDPs within Texas, not just those within the ERCOT Region.  Therefore, a Load Profile ID shall also be submitted to ERCOT by the respective TDSP, even though the non-ERCOT information shall not be used in ERCOT Settlements. To ensure that the non-ERCOT Load Profile IDs are not confused with the ERCOT Load Profile IDs, it is necessary to give them names that are different than those for ESI IDs within ERCOT.

(2)	A list of valid Load Profile IDs to be assigned to ESI IDs within Texas, but outside of the ERCOT Region (non-ERCOT ESI IDs), is included in Appendix D, Profile Decision Tree, under the “Non-ERCOT Profile IDs” worksheet.  TDSPs shall submit for approval to ERCOT additional names or changes for their non-ERCOT Load Profile IDs.  The Load Profile ID may be no more than 30 characters in length.  A comprehensive listing of non-ERCOT Load Profile IDs shall be maintained in the Profile Decision Tree.

- 9.1.8	Load Profile ID Assignment for Non-Opt In Entities

NOIEs are required to submit Load Profile IDs for the ESI IDs that represent the NOIE metering points, as defined in Protocol Section 10, Metering.  The Profile Decision Tree contains details on Load Profile ID assignments for NOIEs.  The Load Profile ID shall be based on default values for four of the five fields in the Load Profile ID.  The only component determined by the NOIE is the Weather Zone code.  This is assigned based on the ZIP code at the metering point.

- 9.2	Processes to Change Load Profile ID Assignments

(1)	ERCOT, a Transmission and/or Distribution Service Provider (TDSP), or a Competitive Retailer (CR) may request a change in the Load Profile ID assignment of an ESI ID.  ERCOT may initiate a change as a result of the ERCOT Load Profile ID validation process.  A TDSP shall initiate a change, when necessary, due to a change in the TDSP tariff to which the ESI ID is assigned, a meter type change, or an error with the Load Profile ID assignment.  A CR may submit a change request to the TDSP when the CR believes there is an error in the existing Load Profile ID or when the CR believes adequate data has become available to replace a default Load Profile ID assigned to a new ESI ID.  A Customer may request a Load Profile ID change by contacting their CR.  Load Profile ID assignments shall always be based on the criteria defined in the appropriate Profile Decision Tree.  Regardless of which Entity initiates a change in the Load Profile ID assignment for an ESI ID, the TDSP is responsible for formally updating ERCOT’s systems using the appropriate Texas Standard Electronic Transaction (TX SET).

(2)	All communication among Market Participants and between Market Participants and ERCOT regarding Load Profile ID changes shall be implemented per the appropriate TX SET transaction, except for alternative communication processes that are specified within the Load Profiling Guide (LPG).

(3)	For any change made to a Load Profile ID, it is the responsibility of the TDSP to make sure the effective date of change is concurrent with a specific meter read date and that the meter read information reaches ERCOT prior to the Load Profile ID change.  For Load Profile ID changes that result from Annual Validation, a TDSP tariff change, a meter type change, or a CR request to change a default Load Profile ID when adequate data becomes available, the TDSP shall submit the change after said meter read has been sent to ERCOT.   For any Load Profile ID assignments that are found to be in error by dispute, the effective date of change shall be retroactive to the meter read date when no profile segment assignment error existed; however, the effective date of the change shall not go any farther back than what would affect the True-Up Settlement.

- 9.2.1	Load Profile ID Changes Initiated By Transmission and/or Distribution Service Providers

- 9.2.1.1	Load Profile ID Change Related to a Transmission and/or Distribution Service Provider Tariff Change

When a Premise changes between residential and business TDSP tariffs, or when a meter type change is made for a TDSP tariff billing requirement, the TDSP is required to submit a Load Profile ID change effective on the meter read date of the TDSP tariff change.

- 9.2.1.2	Recognized Error in Current Assignment

Should the TDSP become aware of an error in the assignment of a Load Profile ID, the TDSP shall notify the CR of the error as soon as practical via the appropriate Texas SET transaction.If a dispute is created, refer to Section 14.2, General Load Profile ID Dispute Resolution Guidelines.

- 9.2.1.3	Load Profile ID Changes Resulting from Meter Type Changes

The following subsections outline the procedures for implementing Load Profile ID changes when a meter type change occurs.

The TDSP shall submit the Load Profile ID change to ERCOT using the appropriate TX SET transaction with the effective date of the meter change once the meter installation is complete.

- 9.2.1.3.1		Business Demand to Business Non-Demand

(1)	When Demand data is no longer required by the TDSP tariffs, and the CR has no need for Demand data then the TDSP shall change the assignment of the ESI ID to BUSNODEM.  If a Demand meter is present and used for billing purposes, then the TDSP shall send Demand data to ERCOT via TX SET transactions.

(2)	When a TDSP determines that an ESI ID assignment should be changed to BUSNODEM based on the TDSP metering tariff rules, the TDSP shall notify the CR at least 30 days prior to making the Load Profile ID change. If the CR requires Demand data to support Customer billing for the ESI ID in question, then the CR shall notify the TDSP of its requirement for Demand data. Upon CR notification, the TDSP shall not change the Demand meter and the TDSP shall continue collecting Demand data.  The ESI ID shall retain its Load factor Load Profile ID assignment.

(3)	If it is determined that Demand data is no longer required by either the CR or the TDSP, the TDSP has the option of:

(a)	Replacing the Demand meter with a non-Demand meter; or

(b)	Leaving the Demand meter in place but discontinue sending any Demand data for that ESI ID to ERCOT.

(4)	Regardless of which Demand meter change option the TDSP pursues, the effective date of the Load Profile ID change shall coincide with the last meter read date where Demand data is sent to ERCOT.

(5)	If a TDSP elects to leave a Demand meter in service on an ESI ID that no longer requires a Demand meter, the Load Profile ID shall be changed to the BUSNODEM profile.  The TDSP shall submit the appropriate TX SET transaction to change the Load Profile ID to ERCOT before the next regularly scheduled meter read date with an effective date of the last meter read.

(6)	If the TDSP elects to replace the meter, then the TDSP shall submit the appropriate TX SET transaction to ERCOT to change the Load Profile ID with an effective date of the meter change date.

- 9.2.1.3.2		Non-Demand to Demand

The CR shall notify the TDSP when it requires a specific ESI ID to have a Demand meter.  Under normal Demand meter installations, the TDSP has until the second regularly scheduled meter read date after receipt of the CR’s request to install the requested meter type at the Premise and submit the Load Profile ID change to ERCOT.

- 9.2.1.4	CR Requested Change from a Default Load Profile ID

After a new ESI ID has sufficient usage history, a CR may request a change from a default Load Profile ID using the ERCOT retail transaction issue resolution system.  The requested Load Profile ID shall follow the guidelines for calculations contained in Appendix D, Profile Decision Tree.  In the case of a Business ESI ID, the 12 months used in the calculations shall be the first 12 months of usage for the ESI ID.  In the case of a residential ESI ID, the first consecutive seven months from October through April is all that is needed for the calculation of Winter Ratio.  Once the Winter Ratio is known then the CR may request a change from the default Load Profile ID.  After ERCOT has validated the CR’s calculated Load Profile ID change request, ERCOT will then submit the request to the appropriate TDSP.  The TDSP will verify that the change is consistent with their tariff and send the appropriate TX SET transaction to complete the request.

# kVA TO kW CONVERSION

(1)	The majority of Transmission and/or Distribution Service Providers (TDSPs) meter kW Demand. However, some TDSPs only meter kVA Demand. To assign a Load Profile ID to an Electric Service Identifier (ESI ID), the kVA shall be converted to a kW value for the Load factor calculation when necessary.  This Section 10, kVA to kW Conversion, of the Load Profiling Guide (LPG) addresses how kVA shall be converted to kW for Load Profile ID assignments.

(2)	This Section of the LPG applies to any Market Participants such as:

(a)	A TDSP that currently meters kVA;

(b)	A TDSP that changes from kW to kVA metering; or

(c)	A Non-Opt-In Entity (NOIE) that currently meters kVA and decides to opt-in.

(3)	Appendix D, Profile Decision Tree, defines how kVA is to be converted to kW (kW is equivalent to the product of kVA and Power Factor).  The Power Factor(s) for this conversion shall be determined by a case study performed by the TDSP.  Without a case study, a default Power Factor of 1.0 shall be imposed.  A default Power Factor of 1.0 means kVA shall be considered equivalent to kW.

11     VALIDATION OF LOAD PROFILE ID

(1)	A Load Profile ID is comprised of five components:

(a)	Load Profile Type;

(b)	Weather Zone;

(c)	Meter Data Type;

(d)	Weather sensitivity; and

(e)	Time Of Use Schedule (TOUS).

(2)	ERCOT shall validate the first two components, the Load Profile Type and Weather Zone, at the following times:

(a)	As part of the initial assignment of Load Profile IDs for Opt-In Entities;

(b)	When Load Profile Segment definitions change; and

(c)	At least one time per year during the Annual Validation process.

(3)	At the start of the validation process, the Transmission and/or Distribution Service Provider (TDSP) shall be asked by ERCOT to confirm contact information for the primary and backup TDSP contact persons.  ERCOT shall provide the TDSP contact information on an ERCOT contact person.

(4)	Electronic mail is the primary means of communication among ERCOT, the Profiling Working Group (PWG), and Market Participants regarding the validation process detailed in this section.  Other methods of communication shall be accommodated if all affected parties mutually agree to alternative methods.

- 11.1	Initial Assignment of Load Profile IDs for Opt-In Entities

(1)	When a Non-Opt-In Entity (NOIE) chooses to participate in the Retail market, it thereby becomes an Opt-In Entity and the initial assignment of Load Profile IDs to Electric Service Identifiers (ESI IDs) shall be subject to all requirements detailed in this Section11.1, Initial Assignment of Load Profile IDs for Opt-In Entities.

(2)	Once the Opt-In Entity has given notice to ERCOT of its intent to participate in the Retail market, the Opt-In Entity  shall submit all assigned ESI IDs, their Load Profile Group, and their historical usage to ERCOT.  Additionally, the Profile Type shall be submitted for all ESI IDs assigned to the non-metered group.

(a)	The Opt-In Entity shall provide to ERCOT, in a comma-delimited format at 	least 120 days prior to the effective start date, the ESI ID’s monthly usage and 	Demand values for a period of time established in coordination with ERCOT.

(b)	Load Profile ID assignments shall be based on the criteria defined in Appendix D, 	Profile Decision Tree.

(i)	ERCOT will calculate the Load Profile Segment using the historical usage 		provided by the Opt-In Entity.

(ii)	ERCOT shall provide the Opt-In Entity a file containing all of the ESI IDs 		and their Load Profile Type.

(iii)	The Opt-In Entity shall use the provided information to assign the Load 			Profile ID via the appropriate Texas Standard Electronic Transactions (TX 		SET) and shall coordinate with ERCOT to schedule submittal of the 			transaction(s).			.

- 11.1.1	Validation of Initial Opt-In Entity Assignments

(1)	The Opt-In Entity shall notify ERCOT via email when the transactions to create the Opt-In ESI IDs have been submitted and accepted in the ERCOT System.  After receiving notification, ERCOT shall perform three additional reviews to ensure all ESI IDs are set up in accordance with the appropriate Profile Decision Tree:

(a)	ERCOT will compare each ESI ID and Load Profile ID assignment in the ERCOT database with the previously approved initial Load Profile Type;

(b)	ERCOT will validate that Weather Zone assignment is consistent with the appropriate Profile Decision Tree; and

(c)	ERCOT shall validate Load Profile Group assignment for Residential and Business ESI IDs by using the Premise Type field in ERCOT’s registration database.  The Residential Load Profile Group must match the Residential Premise Type in the registration database.  The Business Load Profile Group must match either the Small Non-Residential or Large Non-Residential Premise Type in the registration database.

(2)	Any discrepancies will be reported to the Opt-In Entity via email.  The Opt-In Entity shall submit corrections to ERCOT via appropriate TX SET transaction(s) or provide details as to why the data elements have changed.

(3)	The initial Load Profile ID assignment validation is complete after all discrepancies are resolved.

- 11.2	Annual Validation of Load Profile Type

(1)	Transmission and/or Distribution Service Providers (TDSPs) and ERCOT shall work together to complete the Annual Validation process. When a date is listed in this Section 11.2 and a year is not specified, the date shall apply to the current year in which the Annual Validation is performed.

- 11.2.1	Annual Validation of Load Profile Type Assignment for RES and BUS Load Factor Electric Service Identifiers

(1)	For the purposes of Annual Validation, ERCOT is responsible for determining the Load 	Profile Type assignment for all Residential and Business Load factor Electric Service 	Identifiers (ESI IDs).

(2)	The following timeline shall be adhered to, unless otherwise approved by an appropriate 	Technical Advisory Committee (TAC) subcommittee.  ERCOT shall utilize the historical 	usage and Demand data in its systems to derive usage time period values for each active 	and de-energized ESI ID for the time period specified in Appendix D, Profile Decision 	Tree.

(a)	Residential Load Profile Group Timeline

(i)	ERCOT shall determine the Load Profile Segment for the Load Profile ID for each active and de-energized ESI ID based on the current Profile Decision Tree in Appendix D.  ERCOT shall provide the TDSPs with a list of Residential ESI IDs containing the current Load Profile Type and the recommended Load Profile Type for those ESI IDs where ERCOT recommends a change in Load Profile Type assignment.  An electronic copy of each list shall be delivered to each TDSP no later than June 30.

(ii)	For each ESI ID contained in the lists, the TDSPs shall review the recommended Load Profile Segment assignment and determine whether the recommended change is consistent with the TDSP tariffs, the applicable Retail Electric Provider (REP) billing requirements, and whether the ESI ID is active or de-energized.  The TDSP shall then send finalized lists of ESI IDs back to ERCOT no later than July 10.  The finalized lists shall indicate all revisions determined to be necessary by the TDSP.

(iii)	ERCOT shall send notification to Competitive Retailers (CRs) and the Profiling Working Group (PWG) by July 15 announcing these lists are available to the CR of record.  Upon request, ERCOT shall make available to the current CR of record the list of those ESI IDs that are expected to have a Load Profile ID change as a result of Annual Validation.

(iv)	The TDSPs shall coordinate with ERCOT to submit the necessary Texas Standard Electronic Transaction (TX SET) transactions to update Load Profile ID assignments for the population of the Residential Load Profile Group to be effective on the most current meter read date on or after August 15.

(v)	TDSPs are responsible for verifying that TX SET transactions related to Annual Validation have been successfully accepted into ERCOT’s systems by monitoring the appropriate response transactions.  The TDSPs and ERCOT shall work together to have TX SET transactions successfully completed for the Residential Load Profile Group by September 30.

(vi)	Within the first two Business Days of the TDSP successfully submitting all of its Residential transactions, ERCOT shall compare the finalized lists of recommended changes with the current Load Profile ID in the ERCOT system.  ERCOT and the TDSPs shall work closely and expeditiously to resolve any discrepancies.  The TDSP and ERCOT shall be in contact until at least 99.0% of the finalized list of changes is resolved to their mutual satisfaction.

(vii)	ERCOT and the TDSPs shall provide regular updates on the progress of Annual Validation as needed, or at a minimum during the regularly scheduled PWG meetings.

(b)	Business Load Profile Group Timeline

(i)	ERCOT shall determine the Load Profile Type for the Load Profile ID for each active and de-energized ESI ID based on the current Profile Decision Tree (Appendix D).  ERCOT shall provide the TDSPs with a list of Business Load factor ESI IDs containing the current Load Profile Type and the recommended Load Profile Segment for those ESI IDs where ERCOT recommends a change in Load Profile Segment assignment.  An electronic copy of each list shall be delivered to each TDSP no later than August 15.

(ii)	For each ESI ID in the lists, the TDSPs shall review the recommended Load Profile Segment assignment and determine whether the recommended change is consistent with the TDSP tariffs, the applicable Retail Electric Provider (REP) billing requirements, and whether the ESI ID is active or de-energized.  The TDSP shall then send finalized lists of ESI IDs back to ERCOT no later than August 25.  The finalized lists shall indicate all revisions determined to be necessary by the TDSP.

(iii)	ERCOT shall send Notification to CRs and the PWG by September 1 announcing these lists are available to the CR of record.  Upon request, ERCOT shall make available to the current CR of record the list of those ESI IDs that are expected to have a Load Profile ID change as a result of Annual Validation.

(iv)	The TDSPs shall coordinate with ERCOT to submit the necessary TX SET transactions to update Load Profile ID assignments for the population of Business Load factor group to be effective on the most current meter read date on or after October 1.

(v)	TDSPs are responsible for verifying that TX SET transactions related to Annual Validation have been successfully accepted into ERCOT’s systems by monitoring the appropriate response transactions.  The TDSPs and ERCOT shall work together to have TX SET transactions successfully completed for the Business Load factor group by November 30.

(vi)	Within the first two Business Days of the TDSP successfully submitting all of its Business Load factor transactions, ERCOT shall compare the finalized lists of recommended changes with the current Load Profile Segment in the ERCOT system.  ERCOT and the TDSPs shall work closely and expeditiously to resolve any discrepancies.  The TDSP and ERCOT shall be in contact until at least 99.0% of the finalized list of changes is resolved to their mutual satisfaction.

(vii)	ERCOT and the TDSPs shall provide regular updates on the progress of Annual Validation as needed, or at a minimum during the regularly scheduled PWG meetings.

- 11.3		Additional Validations

(1)	During the Annual Validation process, ERCOT shall perform additional validations to 	identify potentially incorrect Load Profile ID or Premise Type assignments.  If ERCOT 	and the Transmission and/or Distribution Service Provider (TDSP) confirm a Load 	Profile ID or Premise Type change is necessary, the TDSP shall update the Load Profile 	ID in the ERCOT system using the appropriate Texas Standard Electronic 	Transaction 	(TX SET) transaction.

- 11.3.1	Validation of BUSOGFLT Profile Type

(1)	ERCOT shall verify that only eligible ESI IDs are assigned the Business Oil and Gas Flat 	(BUSOGFLT) Profile Type.  Should an ESI ID be found to have been assigned the 	BUSOGFLT Profile Type erroneously, ERCOT shall work with the TDSP to have the 	Profile Type assignment corrected, and ERCOT shall notify the Competitive Retailer 	(CR) of record.

- 11.3.2	Validation of NMFLAT and NMLIGHT Profile Types

(1)	ERCOT shall review all ESI IDs and their usage which are classified with either a Non-	Metered Flat (NMFLAT) or Non-Metered Light (NMLIGHT) Profile Type and calculate 	the Average Daily Use (ADU) for each ESI ID.  ESI IDs with excessive fluctuation over 	the 12-month period being reviewed shall be reported to the TDSP.

- 11.3.3	Validation of Profile Segments for Distributed Generation

(1)	ERCOT shall verify that only eligible ESI IDs are assigned Load Profile Segments for 	Distributed Generation (DG).  For ESI IDs found to have been assigned a profile segment 	for DG erroneously, ERCOT shall work with the TDSP to have the profile segment 	assignment corrected.

- 11.3.4	Comparison of Profile Type to Premise Type

(1)	ERCOT shall review and identify all ESI IDs with conflicting Profile and Premise Type 	combinations.  Any discrepancies shall be reported to the TDSP.

- 11.3.5	Validation of Service Address Zone Improvement Plan Code

(1)	ERCOT shall validate that the service address Zone Improvement Plan (ZIP) code for 	each ESI ID is located within the ERCOT region, and shall perform consistency checks 	for Congestion Zone, TDSP service area, and substation.  ERCOT shall provide lists to 	the TDSP of any ESI IDs which have been identified as having a suspect ZIP code or 	substation assignment.

- 11.3.6	Validation of Weather Zone Code

(1)	ERCOT shall compare the current ESI ID Weather Zone component of the Load Profile 	ID to the Weather Zone assignment based on the current Profile Decision Tree in 	Appendix D, Profile Decision Tree, utilizing the service address ZIP code in ERCOT’s 	system.  Any discrepancies shall be reported to the TDSP.

- 11.3.7	Comparison of Meter Data Type Code to Profile Type Code

(1)	ERCOT shall compare the Meter Data Type code component of the Profile ID to the 	Load Profile Group code for all ESI IDs.  Any discrepancies shall be reported to the 	TDSP.

- 11.3.8	Comparison of Weather Sensitivity Code to Meter Data Type Code

(1)	ERCOT shall verify that all ESI IDs with a Meter Data Type of Non-Interval Data 	Recorder (NIDR) are assigned a Weather Sensitivity code of Non-Weather Sensitivity 	(NWS).  ERCOT shall also verify that only ESI IDs having a Meter Data Type of IDR 	which were identified by ERCOT during the most recent weather sensitivity analysis as 	being weather sensitive are assigned a weather sensitivity code of WS.  Any 	discrepancies shall be reported to the TDSP.  The annual procedures for reviewing of the 	weather sensitivity code are located in Protocol Section 11.4.3.1, Weather 	Responsiveness Determination.

- 12 	REQUEST FOR LOAD PROFILE SEGMENT CHANGES, ADDITIONS, OR REMOVALS

(1)	This Section 12, Request for Load Profile Segment Changes, Additions, or Removals, of the Load Profiling Guide (LPG) addresses changes, additions, and deletions to Load Profile Segments, with the exception of Load Profile Segment modifications addressed in Section 16, Supplemental Load Profiling.

2)	The steps and tests identified to introduce new Load Profiles or changes to Load Profiles are intended to fulfill the criteria established in Protocol Section , Guidelines for Development of Load Profiles.

(3)	Any change to Load Profile ID assignments resulting from an approved modification to the definitions of Load Profile Segments shall not be retroactively applied.

- 12.1		Types of Requests

he  types of requests are:

(a)	Creation of a new Load Profile Segment from one or more existing Load Profile Segments;

(b)	Redefinition of existing Load Profile Segments; and

(c)	Removal of existing Load Profile Segments.

- 12.1.1	Creation of a New Load Profile Segment

(	When a new Load Profile Segment is created, there may be an impact to the Electric Service Identifiers (ESI IDs) assigned to one or more existing Load Profile Segments.  The new segment will be applied to ESI IDs that meet the criteria for the new Segment while being removed from the existing Segment.

(2)

- 12.1.2		Redefinition of an Existing Load Profile Segment

Redefinition of existing Load Profile Segment  definitions may require that some ESI IDs be moved from one segment to another.

- 12.1.3	Removal of Existing Load Profiles Segments

(1)	A request to remove an existing Load Profile Segment shall include supporting documentation  that the Load Profile Segment proposed for removal does not satisfy the standards for a separate Load Profile.  Specifically, the group represented by the Load Profile may be:

(a)	Too small to justify a separate Load Profile Segment, as described in Section 12.5, Groups of Electric Service Identifiers Eligible to Become Load Profile Segments; and/or

(b)	Sufficiently similar to one or more existing Load Profiles, according to the measures defined in Section 12.5.

2)	Removal of an existing Load Profile Segment  requires changing assignments  of  existing  ESI IDs currently in the proposed removed segment.

- 12.2	Request for Load Profile Segment Changes

Any Market Participant, ERCOT, or the Profiling Working Group (PWG) may submit a request for a change to Load Profile Segments.

- 12.4	Process Timing for Requesting Changes

(1)	Requests for changes may be submitted to ERCOT at any time.  Within two Business Days of receiving the request, ERCOT shall reply to the submitter indicating that the request has been received.

(2)	As required by Protocol Section 18.2.8, Adjustments and Changes to Load Profile Development, ERCOT shall respond to the request within 60 days.  This period does not include the time required to analyze and render the final decision of the request. The response shall indicate:

(a)	Whether the request is complete;

(b)	The date by which a recommendation on the request is expected to be ready and available to the requestor;

(c)	The date by which the recommendation is expected to be presented to the Profiling Working Group (PWG); and

(d)	The best guess time the requested change is expected to be implemented (ready for Settlement), if approved.

(3)	A requestor may, at their discretion, submit a Load Profile Segment change request with supporting information and documentation, which includes all the criteria listed in Section 12.6, Information Required with Request for Change, except for providing Load research sample data of sufficient quality to support the request.  In this case, the requestor shall indicate that the request is for conditional approval.

(4)	Upon completion of the review outlined in Section 12.7, Evaluation of the Request, ERCOT shall make a recommendation to the PWG regarding conditional approval.  If the recommendation is to grant conditional approval, then ERCOT shall specify the requirements for additional Load research sampling  for final approval.

(5)	According to Protocol Section 18.2.8, ERCOT shall provide appropriate notice to all Market Participants prior to implementation of any change. Load Profile ID changes to each Electric Service Identifier (ESI ID) shall be made in accordance with Section 9.2, Processes to Change Load Profile ID Assignments.

- 12.5	Groups of Electric Service Identifiers Eligible to Become Load Profile Segments

(1)	For a group of Electric Service Identifiers (ESI IDs) to be a distinct Load Profile Segment, the group shall satisfy the following requirements:

(a)	The group is based on readily identifiable parameters, which are not subject to frequent change;

(b)	The group is reasonably homogeneous as defined in Section 12.6.4, Homogeneity;

(c)	The group is sufficiently different from other existing Load Profiles as defined in Section 12.6.2, Difference from Current Load Profiles; and

(d)	The group is of sufficient size to justify its own profile segment as defined in Section 12.6.3, Size.

- 12.5.1	Universal Load Profile Segment Applicability

(1)	 One primary goal is to make all Load Profile Segment definitions universally applicable.  Universally applicable means:

(a)	The Load Profile may be applicable to all CRs;

(b)	The Load Profile Segment shall be applied to any ESI ID in the ERCOT System that meets the eligibility criteria;

(c)	The Load Profile Segment shall be public; and

(

(2)	There are limited exceptions as described in Section 16, Supplemental Load Profiling.

12.6	Information Required with Request for Change

All requests shall include the following:

(a)	Unambiguous group identification;

(b)	Difference from current Load Profile Segments;

(c)	Size;

(d)	Homogeneity;and

(e)	 Validation methodology.

- 12.6.1	Unambiguous Group Identification

The definition of the group shall be provided in the request for the new Load Profile Segment.  The request shall unambiguously define specific criteria for an ESI ID to be included in the new Load Profile Segment.  In a request to change an existing Load Profile Segment, the group to be re-assigned shall be identified.

- 12.6.1.1	Identification Based on Data Currently in ERCOT’s Systems

(1)	The most direct way a group may satisfy the requirement of being unambiguously identified occurs when the group may be identified based solely on information currently available in the ERCOT data systems or readily derived from such data.

(2)	Examples of information available in or derived from the ERCOT data systems include, but are not limited to:

(a)	Monthly or annual kWh consumption;

(b)	Metered monthly or annual peak Demand for Demand-metered Customers;

(c)	Monthly or annual Load factor;

(d)	Ratio of seasonal consumption values; and

(e)	Zone Improvement Plan (ZIP) code.

- 12.6.1.2	Identification Based on Other Means

Segments based on other criteria may be requested.  ERCOT, in coordination with the Profiling Working Group (PWG), shall evaluate such requests in terms of the feasibility and reliability of the proposed identification method.  If the method requires data not currently in ERCOT’s systems, the request shall describe how these data shall be made available to ERCOT on an ongoing basis.  If the identification method is judged to be impractical or unreliable, the request may be denied.

12.6.2	Difference from Current Load Profile Segments

1)	A requested new Load Profile Segment shall be shown in the supporting documentation to be different from existing Load Profiles, and

(2)	that the group re-assigned from one segment to another is more similar to the proposed new assignment(s) than to the old one, in ways that improve the accuracy of Settlement.

(2)	If the ESI IDs in the requested Load Profile Segment are different from the Load Profile Segment that they are currently assigned and more similar to another existing Load Profile Segment, then the resolution of the request may be to reassign these ESI IDs to the most similar existing Load Profile Segment.

(3)	Requests to create new Load Profile Segments or to change the definition of existing segments require supporting documentation to provide a basis for assessing differences between the affected group and existing Load Profile Segments.  All differences between Load Profiles that are important for evaluating a change shall be  documented in the request.

- 12.6.2.1	Supporting Data Required

I  Types of supporting data that may be submitted and the associated documentation are described in the following subsections.

- 12.6.2.1.1	Load Research Data

1)	 The  best documentation would be a statistically valid Load research sample from the proposed new or re-assigned segment population.

2)	The Load research sample design, data collection and statistical calculations shall be in compliance with industry accepted standards as defined in the Association of Edison Illuminating Companies (AEIC) Load Research Manual. a

(3)	Examples of less  rigorous, but supportive documentation would be other types of Load research data, such as:

(a)	Data from ad-hoc or convenience samples; and

(b)	Data from a similar population. .

4)	When less  rigorous data is submitted, the submitter should also submit evidence to support the applicability of the data to the proposed Load Profile Segment population, including but not limited to:

(a)	Documentation of operating schedules for the proposed group and comparison with typical schedules for Premises in the currently assigned Load Profile;

(b)	End-use saturation data, comparing the proportions of Premises with particular types of electric end uses for the proposed group and currently assigned Load Profiles.  Such data shall be relevant to the proposed population in ERCOT;

(c)	Monthly billing data comparing consumption patterns, particularly related to heating and cooling.   These comparisons shall be made separately by Weather Zone or account for variations by Weather Zone.

- 12.6.2.1		Basis for Assessment of Differences Based on Load Research Data

(1)	In assessing differences between the initial profile segment and the requested profile segment,  ERCOT shall use industry accepted standards as described in the Association of Edison Illuminating Companies (AEIC) Load Research Manual.  The requester may submit analysis  but is not required to do so.

(

(

)

- 12.6.2.2	Accounting for Weather Zone Effects in Load Profile Comparisons

(1)	Comparisons between profiles  shall take into account Weather Zone effects.   These effects may be accounted for in the comparisons in one of two ways:

(a)	The comparison between the proposed segment and the existing Load Profile is made separately for each Weather Zone; and

(b)	A single Load Profile representing the proposed segment as a whole is compared with a single composite Load Profile for the existing segment.

(2)	These methods are not required for Load Profiles that are the same across all Weather Zones.

- 12.6.2.3		Separate Comparisons for Each Weather Zone

1)	If Load research data for individual sample Customers are provided for the proposed segment, a separate profile may be constructed for each Weather Zone.

(2)	The limitation of separate comparisons by Weather Zone is that some or all of the separate Weather Zone profiles may have large statistical errors due to low sample sizes.  The magnitude of these errors should be considered in assessing the comparisons.

- 12.6.2.4	Comparison for the Proposed Segment as a Whole

(1)	If a single Load Profile is estimated for the proposed segment as a whole across several Weather Zones, this Load Profile may be compared with a composite of existing Load Profiles.  The composite existing Profile will be constructed using the standards described in the Association of Edison Illuminating Companies (AEIC) Load Research Manual.

- 12.6.3	Size

(1)	Supporting documentation shall show that the population represented by the proposed

segment(s) is of sufficient size to justify a separate segment.  Size shall be provided in terms of both number of Customers and total energy consumption.

(2)	If the proposed segment is identified based on information available in the ERCOT data systems and also available to the requesting party, documentation of the total ESI ID count and annual energy use is sufficient.  ERCOT shall verify this information using the ERCOT data systems.

(3)	If the requesting party has information on only a portion of the population represented by the segment, the request shall include estimates of the ESI ID counts and energy use, and documentation of the basis for the estimates.

- 12.6.4	Homogeneity

For a new Load Profile Segment, the request shall provide evidence that the requested group is homogeneous with respect to Load shape characteristics.  For a change to definitions of existing segments, the request shall provide evidence that the re-defined segments are homogeneous in support of the change in definition. .

- 12.6.4.1	Load Research Demonstrating Homogeneity

1)	Evidence of homogeneity may be provided by a statistically valid Load research sample from the population of the requested segment(s).

(4)

2)	Alternatively, data from case studies or samples of convenience may be provided.  This may include but is not limited to:

(a)	Survey data or other evidence of appliance or equipment present in the Premises;

(b)	Data on operating schedules; and

(c)	Variances of parameters of monthly billing data, such as size, ratio of seasonal consumption values, or Load factors.

12.7	Evaluation of the Load Profile Segment Request

(1)	ERCOT shall evaluate the request based on Load research standards described in the Association of Edison Illuminating Companies (AEIC) Load Research Manual.

(

- 12.8	Resolution of the Load Profile Segment Request

- 12.8.1	ERCOT Staff Initial Recommendation

(1)		ERCOT shall provide a written report detailing their evaluation of the Load Profile Segment change request to the submitter on or before the date specified in Section 12.4, Process Timing for Requesting Changes.  ERCOT shall request that the presentation detailing their evaluation of the Load Profile Segment change request be placed on the Profiling Working Group (PWG) agenda at the next available opportunity.  If ERCOT is unable to meet the specified deadline, they shall notify the submitter prior to the date and specify a revised date by which the report shall be available.

- 12.8.2	Profiling Working Group Disposition of Request

(1)	Upon receipt of the written report, the submitter shall have up to 30 days to make comments and recommendations to ERCOT and PWG.  Upon receiving the submitter’s comments, ERCOT shall reconsider and, if appropriate, revise their recommendation and provide a revised written report to the submitter and present the report at the next PWG meeting.

(2)	At any time during the process of resolving the request, the submitter may withdraw the request.  If the submitter withdraws the request, they retain the right to amend and/or resubmit the request at a later date.

(3)	The PWG recommendation regarding the disposition of the request(s) shall be presented to the Commercial Operations Subcommittee (COPS) and then, if approved, be forwarded to the Technical Advisory Committee (TAC) for further disposition.

13	   CHANGES TO WEATHER ZONE CHARACTERISTICS

(1)	Changes to Weather Zones and any combination thereof that may be requested including changes in:

(a)	Weather Zone boundary definitions;

(b)	Boundaries of weather modeling regions; and

(c)	Weather stations used and

(d)	Weighting factors assigned to weather stations used within a Weather Zone.

(2)	Any change to Load Profile ID assignments resulting from an approved modification to the definitions of Weather Zones shall only be applied on a prospective basis.

(3)	Sufficient documentation shall be provided to demonstrate that the Weather Zone modification improves the accuracy of Settlement.

- 13.1.1	Timeline for Processing a Request

(1)	Timing and responses of  requests, and change implementation shall be as defined 	for Segmentation in Section 12.4, Process Timing for Requesting Changes.

- 13.1.2	Uniformity

(1)	Weather Zone characteristics shall be applied to all Electric Service Identifiers (ESI IDs) 	located within the geographic boundaries of the Weather Zone.  Zone Improvement Plan 	(ZIP) codes are mapped to Weather Zones and are defined by the ZIP-to-Zone mapping 	in the Appendix D, Profile Decision Tree.

- 13.2.1	Types of Weather Zone Boundary Changes

(1)	Changes to Weather Zone boundaries and any combination thereof, may occur due to the following conditions:

(a)	Subdivision:  An existing Weather Zone is divided into two or more zones;

(b)	Boundary shifting:  Existing Weather Zone boundaries are moved so that areas are shifted between Weather Zones; or

(c)	Boundary collapsing:  Existing Weather Zone boundaries are moved so that one Weather Zone is created from two or more existing Weather Zones.

(2)	When creating a new Weather Zone, the other zones affected by the boundary change shall satisfy the Weather Zone criteria in Section 13.2.2, Eligible Areas for Weather Zones.

- 13.2.2	Eligible Areas for Weather Zones

(1)	Each Weather Zone that results from a requested Weather Zone boundary change shall be 	a geographically contiguous area defined by identifiable physical, Transmission and/or 	Distribution Service Provider (TDSP) territory, or ZIP code boundaries.

- 13.2.2.1	Size

(1)	The requested Weather Zone changes shall be shown in supporting documentation to be 	of sufficient size, both in number of Customers and in total energy consumption, to 	justify the changes.  While no explicit size threshold is set, the size of each proposed new 	or changed Weather Zone shall be considered in evaluating a Weather Zone change request.

- 13.2.2.2	Weather Stations

(1)	Only weather data from National Oceanic and Atmospheric Administration (NOAA) first or second order weather stations shall be used in model calculations.  Each proposed new or changed weather zone shall have at least two NOAA first or second order weather stations to represent it No weather station is permitted to have more than 50% weighting factor within a Weather Zone.

13.2.3	Supporting Data

(1)	A requested new Weather Zone shall be	 shown to be different from the current Weather Zone assignment			 that results in important differences (or lack of important differences) in Load 	Profiles which materially affect the accuracy of	 ERCOT Settlement. Only those Load Profile Models dependent on weather variables shall be used in the assessment of a Weather Zone change.

- 13.2.3.1	Calculated Load Profiles

(1)	Load Profile calculations should be provided on both a current Weather Zone and a proposed Weather Zone.  The results of the change(s) should be significant enough to justify the proposed Weather Zone.

(a)	For a subdivision or a shift within a weather-modeling region, Load Profiles shall be calculated using the existing zone constants.

(i)

.

(b)	For a shift across modeling regions, the following calculated Load Profiles shall be provided for each shifted area:

(i)	Load Profiles calculated using the zone constants of the currently assigned zone:

(A)	Using the weighted average for the current Weather Zone;

(B)	Using the weighted average weather of the current zone after the shifted area is removed, with the proposed weights;

(C)	Using the weather of the shifted area only.

(ii)	Load Profiles calculated using the zone constants of the receiving zone, to which the shifted area is proposed to be moved:

(A)	Using the weighted average for the receiving Weather Zone;

(B)	Using the weighted average weather of the receiving zone after the shifted area is added, with the proposed weights;

(C)	Using the weather of the shifted area only.

(2)	The Load Profile using the weather for the shifted area and its current assigned zone constants shall be different from the other two Load Profiles calculated with the current zone constants for the area.  The Load Profile developed by the weather of the shifted area and the zone constants of the receiving zone shall be similar to those of the other Load Profiles calculated with the zone constants of the current zone.  Differences of an area from its current zone and similarity of an area to a proposed receiving zone shall be assessed using the measures described in Appendix C, Measuring Differences Between Load Profiles.

- 13.2.3.2	Additional Supporting Data for Shifts across Weather Modeling Regions

(1)	For a shift across weather modeling regions, evidence shall be provided that demonstrates the weather response of the affected area is likely to be more similar to the proposed new region than to the currently assigned region.  The types of evidence that may be offered for this purpose are the same as those types described in Section 12, Request for Profile Segment Changes, Additions, or Removals, and may include but is not limited to:

(a)	Load research data from the affected area, from the current Weather Zone excluding the affected area, and from the proposed receiving Weather Zone; and

(b)

(d)	Monthly consumption patterns from each area.

(2)	Based on the supporting data, the request shall indicate whether the zone constant(s) should be re-estimated.

13.3		Changes to Weather Modeling Regions

(1)	A weather modeling region boundary shall be changed if shifting an area across weather 	modeling regions changes a Weather Zone boundary.  In some cases ERCOT and the 	Profiling Working Group (PWG) may recommend retaining current zone constants even 	though they shall be applied to a region different from the one for which the estimation 	was conducted.

- 13.3.1	Supporting Data Required

(1)	Any requested change to weather modeling regions shall be treated as a special case of a request for a change in Load Profile Segment definitions.  Supporting data required for such a request is described in Section 12.6, Information Required with Request for Change.

(2)	The requested Weather Zone shall be different from the current Weather Zone in ways that improve Load Profiles.  A change in Weather Zone requiring new coefficients for the new zone shall be considered as a special case of a request for a new Weather Zone segment.  Procedures for submitting and assessing requests are the same as the rules for requesting a change in Segmentation, as described in Section 12, Requests for Load Profile Segment Changes, Additions, or Removals. The assessment shall include the effect on the rest of the Weather Zone(s) of changing this area’s coefficients.

- 13.4.1	Requests for Changes

(1)	A request may be made to change the weighting factors assigned to weather stations within a Weather Zone.  Such a change would include adding a weather station that was not previously used, or deleting a station currently used.  Changing weather stations may require re-estimation of zone constants for weather modeling regions and model-based Load Profile Types.

(2)	A request for such a change shall be accompanied by evidence that the proposed new set of stations and weighting factors are more representative of the population in each affected Weather Zone than the current ones.  An example of such evidence would be analysis of the distribution of population and Weather Zone patterns similar to that conducted for the initial development of the weather modeling procedures.  However, given the broad implications of changing the weather stations, the evidence shall also prove to be a substantial benefit to current specifications.

(1)	ERCOT shall present any proposed Weather Zone definition changes to the Profiling 	Working Group (PWG) for evaluation and implementation according to the procedures 	contained herein.

- 13.5.1.1	Changes in National Oceanic and Atmospheric Administration Weather Station

(1)			.  In the event National Oceanic and Atmospheric Administration (NOAA) discontinues or downgrades an existing weather station, 	ERCOT shall assess and propose reasonable adjustments.

- 14       LOAD PROFILE ID DISPUTE PROCEDURE

ERCOT and Market Participants shall adhere to this procedure for disputing Load Profile ID assignments.

- 14.1	Filing of a Load Profile ID Dispute

ERCOT and any Market Participant, other than a retail Customer, may file disputes related to Load Profile ID assignments.  Retail Customers with disputes, related to Load Profile ID assignments, shall first request resolution from their Competitive Retailers (CRs).  The CR shall address the Customer’s issue, and if necessary, request changes or corrections from ERCOT related to the retail Customer’s request.  A retail Customer who is not satisfied with the CR’s response may appeal to the Public Utility Commission of Texas (PUCT) or the appropriate regulatory authority.  ERCOT does not resolve such disputes.

- 14.2	General Load Profile ID Dispute Resolution Guidelines

Transmission and/or Distribution Service Providers (TDSPs) and ERCOT share responsibility for the assignment of Load Profile IDs.  Competitive Retailers (CRs) may request a Load Profile ID assignment change as a dispute of an existing Load Profile ID assignment.  Requested changes to remove an Electric Service Identifier (ESI ID) from a default Load Profile ID should only be made after adequate monthly data becomes available.

- 14.2.1	Disputes Involving ERCOT

(1)	Disputes involving ERCOT should be submitted using the MarkeTrak system for any of the following cases:

(a)	Requests to remove an ESI ID from a default Load Profile ID - such requests should only be made after adequate monthly data becomes available;

(b)	Disputes regarding ERCOT calculations made as a part of Annual Validation; and

(c)	Disputes regarding ERCOT calculations relating to the weather sensitivity code.

(2)	ERCOT is responsible for all disputes defined in this Section all Profile Decision Tree versions, and all Annual Validation years.

- 14.2.2	Disputes Involving Transmission and/or Distribution Service Providers

All disputes related to Load Profile ID assignments other than those described in the preceding section must be addressed with each TDSP in accordance with their individual processes.

- 14.2.3	Alternative Dispute Resolution

If attempts to clarify or resolve the issue using one of the processes listed above are unsuccessful, parties should refer to Protocol Section 20, Alternative Dispute Resolution Procedure.

- 14.3		Resolutions of Disputes

When the resolution of a dispute requires a change in a Load Profile ID assignment, the change shall be implemented by the Transmission and/or Distribution Service Provider (TDSP) issuing the appropriate Texas Standard Electronic Transaction (TX SET).

# 18	ACCESS TO LOAD PROFILING MATERIALS

(1)	The following Load Profiling related documents and materials may be found on the ERCOT website:

(a)	Backcasted (Actual) Load Profiles – Extract files Load Profiles for individual  trade days.  The trade day occurring one day prior to the current date will be the most current backcast available;

(b)	Forecasted Load Profiles – Files include current day and three days forward of forecasted Load Profiles;

(c)	Historical Backcasted Load Profiles – Multiple years of Load Profile history for each Load Profile Type and Weather Zone combination;

(d)	Methodology-Description of Materials Used in Preparation of Load Profiles – Documents that provide an evaluation of the utility data used to generate the ERCOT Load Profile Models.

(2)	Profiling Working Group (PWG) information and meetings may be found on the ERCOT website.

- 19	Definitions and Acronyms

- 19.1		Definitions

The defined terms in this Section are limited to those used specifically in the Load Profiling Guide (LPG).  Any additional defined terms used in the LPG can be found in Protocol Section 2, Definitions and Acronyms.

- Links to definitions:

- A, B, C, D, E, F, G, H, I, J, K, L, M, N, O, P, Q, R, S, T, U, V, W, X, Y, Z;

- List of Acronyms

- A	[Back to Top]

- Adjusted Static Models

Load Profiles that are generated from statistical models that are based on static historical Load data, and adjusted for conditions of the day (e.g., weather, Season, etc.)

- Annual Validation

The formal process performed every year whereby ERCOT re-determines the first component of each Load Profile ID—the Load Profile Type—for Residential and Business Load Factor Electric Service Identifiers (ESI IDs).  ERCOT then works with the Transmission and/or Distribution Service Providers (TDSPs) to have them update ERCOT’s databases with the resulting Load Profile ID changes via Texas Standard Electronic Transactions (TX SETs).

- B	[Back to Top]

- Business (BUS)

Load Profile Group designation for non-residential Electric Service Identifiers (ESI IDs) whose service is metered.  This encompasses rate classes for business ESI IDs, in addition to other classes.

- C	[Back to Top]

- D	[Back to Top]

- E	[Back to Top]

- Electric Service Identifier (ESI ID)

See Protocol Section 2.1, Definitions.

- Active ESI ID

ESI ID is presently receiving service (energized) and a Retail Electric Provider (REP) is currently assigned to it in ERCOT’s system.

- De-Energized ESI ID

ESI ID does not have a REP assigned in ERCOT’s system, but has not been retired.  An 814_16, Move-In Request, is necessary to change to active status.

- Inactive ESI ID

ESI ID is retired and will never again receive service.

- Engineering Estimated

Estimated Loads based on engineering studies applied to unmetered Loads to allocate energy across specified periods of time.

- F	[Back to Top]

- G	[Back to Top]

- H	[Back to Top]

- I	[Back to Top]

- Interval Data Recorder (IDR) Requirement

The kW level at which the installation of interval data recorders are required for settlement purposes as set forth in Protocol Section 18.6.1, Interval Data Recorder Meter Mandatory Installation Requirements.

- J	[Back to Top]

- K	[Back to Top]

- L	[Back to Top]

- Lagged Dynamic Profiling Methodology

The use of an active set of Load research sample sites to build an aggregated Load Profile for the sample group from actual metered usage processed after the target day.

- Load Profile Class

A classification of a group of Customers having similar energy usage patterns and that are assigned the same Load Profile.  Load Profile Class is comprised of a Load Profile Group and a Load Profile Segment.  An example of a Load Profile Class:  Residential Low Winter Ratio (RESLOWR).  Load Profile Type and Load Profile Class are used interchangeably.

- Load Profile Group

A high-level classification of a set of Customers who have similar characteristics.  The Load Profile Groups are: Non-Metered, Residential, and Business.  Together, the Load Profile Group and the Load Profile Segment form the Load Profile Type.

- Load Profile ID

The Load Profile designation string that contains, the Load Profile Type Code,  the Weather Zone Code, the Meter Data Type Code, the Weather Sensitivity Code, and the Time Of Use Schedule (TOUS) Code.   All Load Profile IDs are listed in Appendix D, Profile Decision Tree.

- Load Profile Models

Processes that use analytical modeling techniques to create Load Profiles.

- Load Profile Segment

A sub-classification of a Load Profile Group.  High Winter Ratio (HIWR) is an example.  Together, the Load Profile Group and the Load Profile Segment form the Load Profile Type.

Load Profile Type (see Load Profile Class)

- M	[Back to Top]

- Meter Data Type

The component of the Load Profile ID that identifies the type of meter data—either interval (IDR) or non-interval (NIDR)—that is to be submitted to ERCOT by the Transmission and/or Distribution Service Provider (TDSP) and used for settlement.

- N	[Back to Top]

- O	[Back to Top]

- Opt-In Entity

A Municipally Owned Utility (MOU) or Electric Cooperative (EC) opting-in to Customer Choice.

- P	[Back to Top]

- Power Factor

The ratio of real power (kW) to the apparent power (kVA) for any given Load and time.

- Profile Decision Tree

The document that contains the directions for determining the Load Profile ID to be assigned to an Electric Service Identifier (ESI ID).

- Profile Type (see Load Profile Class)

- Q	[Back to Top]

- R	[Back to Top]

- Residential (RES)

Load Profile Group designation for Electric Service Identifiers (ESI IDs) served within a residential rate class.

- Segmentation

The process of dividing a population into a number of sub-sets, according to certain parameters, for the purpose of creating Load Profiles for sub-sets of the population.

- T	[Back to Top]

- Target Profile

The Target Profile is the best available estimated Load shape for a particular proposed subgroup.

- U	[Back to Top]

- V	[Back to Top]

- W	[Back to Top]

- Weather Sensitivity Code

- Winter Ratio

The proportion of usage in winter months to usage in the fall base and spring base months and is used to differentiate residential Electric Service Identifiers (ESI IDs).

- X	[Back to Top]

- Y	[Back to Top]

- Z	[Back to Top]

- 19.2		Acronyms

The defined terms in this Section are limited to those used specifically in the Load Profiling Guide (LPG).  Any additional defined terms used in the LPG can be found in Protocol Section 2, Definitions and Acronyms.

COPS	Commercial Operations Subcommittee

DR	Demand Response

HIWR	High Winter Ratio

LPG	Load Profiling Guide

LPGRR	Load Profiling Guide Revision Request

MAD	   Mean Absolute Deviation

MAPE	Mean Absolute Percent Error

NIDR	Non-Interval Data Recorder

NOAA	National Oceanic and Atmospheric Administration

NODEM	Non-Demand

NOTOU 	Non-Time Of Use

NWS	Non-Weather Sensitive

PWG	Profiling Working Group

LOWR	Low Winter Ratio

RMSE	Root Mean Square Error

- Appendix B

Start

Process

Validation of Initial Assignment of Load Profile Type Code

ERCOT

determines

error

severity

Any invalid LP

types?

End

Process

Audit

Needed?

Yes

no

no

yes

This step performed only when

incorrect usage history provided

to ERCOT

Submit correct

sampled ESI ID’s

usage history

ERCOT draws

random sample of

ESI IDs for

validation

TDSP submits

historical usage

data to ERCOT

ERCOT performs

validation on

sample of ESI IDs

TDSP provides

reasoning for

discrepencies

ERCOT provides

error listing to

TDSPs

Submit corrected LP

assignments for all

ESI IDs in LP Group

(EDI)

TDSP corrects

problems

TDSPs submit

changes via EDI

ERCOT performs

audit

- Appendix C

- Measuring Differences Between Load Profiles

- This Appendix describes and illustrates measures that may be used for assessing the differences between Load Profiles.

- Differences between Load Profiles are a consideration in many decisions regarding Load Profiling Methods and models, such as:

- Evaluating Load Profile Model performance or methodologies (Section 8, Load Profile Models, and Section 7, Requests for Changes to Load Profiling Methodology);

- Evaluating requests for changes to Load Profile Segments or Weather Zones (Section 12, Request for Load Profile Segment Changes, Additions, or Removals, and Section 13, Changes to Weather Zone Definitions); and

- Designing Load research samples (Section 15, Load Research Samples).

- Target and Default Load Profiles

  - In most cases when Load Profile differences are measured, the question of concern is whether an existing or proposed method or model is adequate in a particular context, or an alternative is needed.  The alternative might be a finer Segmentation or Weather Zone, use of more recent or more local data in estimating models, or an alternative Load Profiling Methodology.  In all these cases, the analysis compares a “target” Load Profile against a “default” Load Profile.

  - The default Load Profile is the one generated by the existing method or model, or the one that is used if the alternative is not accepted.  The Target Profile is the best available estimated Load shape for a particular proposed subgroup.  The default Load Profile is good enough to represent the target if the two are not significantly different.  Some examples of default and Target Profiles in addressing particular questions are given in the following Table C-1, Examples of Target and Default Profiles.

Table C-1
Examples of Target and Default Profiles

- Load Shape Parameters

Load shapes may be compared in terms of several different parameters that characterize the Load shape.  Some of these parameters are a series of numbers that jointly characterize the shape.  Others are single parameters that represent a key characteristic.

- General Notation

The following general notation is used in this Appendix, and elsewhere in the Load Profiling Guide (LPG).  General quantities are defined.  Suffixes and subscripts are used to signify specific quantities.  Explicit definitions are given in the formulas that follow.

General Quantities

e 	= 	Elasticity of electricity Demand with respect to the commodity price

- E 	= 	Energy

f 	= 	Fraction

DWL 	= 	Deadweight loss

L 	= 	Load

LF	= 	Load factor

N 	= 	Number of intervals in a period for which the quantity is calculated

r 	=	Ratio

ut 	=	Market commodity price at interval t

T 	= 	Total across some calendar dimension (clock-hour, day-type, month, Season, year, on- or off-peak period)

U	= 	Load-weighted average price

- Suffixes

- D	= 	Daily

H 	= 	Clock-hourly

M 	= 	Monthly

S 	= 	Seasonal

Y 	= 	Yearly

- Subscripts

d 	=	Day

m 	= 	Month

h 	=	Clock-hour

p	= 	Day-type

ON 	= 	On-peak

OFF 	= 	Off-peak

s 	= 	Segment

t	= 	Interval

z	=	Weather Zone

- Series Parameters

(1)	Series parameters include:

(a)	Unitized Load shape, or interval fractions;

(b)	Monthly fractions;

(c)	Daily fractions; and

(d)	Clock-hour fractions by day-type.

- Unitized Load Shapes

  - A Load Profile defines the fraction of total energy use over a period that occurs in each time interval within the period.  For most comparisons of Load shapes for purposes of Load Profiling, the Load shapes shall first be unitized.  That is, the interval total or average Loads for the group are translated into interval fractions.

  - For most comparisons, the time period of interest is a year and the fractions are calculated as fractions of total annual energy consumption.  In some cases, shorter time periods such as a Season or month may be used.

  - The unitized Load ft for time interval t is calculated from the interval Loads Lt as

ft = (Load at interval t)/(sum of Loads over all intervals in the period).

= Lt/ T

where

t Lt

and the summation is over all intervals in the period.

  - Each of the other Load shape characteristics described below may be calculated using Loads Lt as indicated in the formulas, or using the unitized Loads ft in place of Lt.  The same result shall be obtained either way.

- Profile Totals

If interval data are finer than hourly, the hourly Loads may be averaged for each hour of the period to get the hourly Demand, equal to the total energy in each hour.  For quarter-hour data, each hour’s four quarters are averaged to give the hourly value.

Hourly totals

TH are the sum over days in the period for each of the 24 hours in a day.  Hourly totals may be calculated across a year, a month, or a day-type.

- Daily totals

- TD are the sum over hours in each day of the hourly values.

Day-type totals

TP are the sum over all hours in the day-type of the hourly values.

- Monthly totals

- TM are the sum over days in the month of the daily totals.

Seasonal totals

TS are the sum over all the months in a Season of the monthly totals.

Yearly totals

TY are the sum over all the months in the year of the monthly totals.

On-peak totals

TON are the sum over all on-peak hours in the period of the hourly values.

Off-peak totals

TOFF are the sum over all off-peak hours in the period of the hourly values.

- Profile Fractions

A year, Season, or month of Load data may be condensed into the fraction of total consumption occurring in each month, day, clock-hour, or day-type.

- Monthly Fractions

- For each month m = 1 to 12, the monthly fraction fMm is calculated as

fMm	=	(sum of interval Loads in month m)/ (sum of all interval Loads in the year)

=	(tMm Lt)/ (t Lt ).

=	TMm/ TY.

where

t		= 	interval of time

Mm	= 	indicates month m

Lt	= 	Load at interval t

TMm and TY, respectively, are Load totals over month m and over the year.

- Daily Fractions

- Daily fractions may be calculated similarly to monthly fractions.  Daily fractions may be calculated as fractions of the year or separately for each month.  The daily fraction for day d is calculated as

fDd	=	(sum of interval Loads in day d)/ (sum of all interval Loads in the period)

=	(tDd Lt)/ (t Lt ).

=	TDd/T

where

t		= interval of time

Dd	= day d

Lt	= Load at interval t

TDd and T, respectively are Load total sums over day d and over the entire period (e.g., year or month) for which the daily fractions are calculated.

- This calculation gives one daily fraction for every day of the month or year.

- Clock-Hour Fractions by Day-type

- When hourly data are used, the unitized Load gives one hourly fraction for each hour of the month or year.  This information may be condensed to give the average clock-hour fraction for a period.  Each of the 24 hourly totals THph for the period is divided by the total for the period, TM, TP or TY.  For clock hour h, the clock-hour fraction for day-type p is calculated as

fHph = THph/TPp

where

THph is the interval Load totals for day-type p and clock-hour h, and TPp is the total over all hours.

- Clock-hour fractions for a month or year are calculated analogously.

- Single Parameters

- Single parameters include:

- Load factor;

- On-/off-peak ratio;

- Seasonal consumption ratio;

- Weekday fraction; and

- Load-weighted average price.

- Load Factor

The Load factor for a period is the ratio of the average to the peak Load for the period:

LF = (average Load)/(maximum Load)

= (T/N) /max(Lt)

where

- Lt = Load at interval t

T = sum of interval Loads over all intervals in the period

N = the number of intervals in the period.

- Average Load Factor

- Average Load factor is an average of monthly Load factors.  This average is computed as

AvgLF=m=112AHUsemm=112MaxKWm

where

- AHUsem = average hourly use in billing month m

= KWhm/(billing days x 24)

MaxKWm = maximum metered kW Demand in billing month m

- This definition and notation are consistent with Appendix D, Profile Decision Tree.

- In terms of the notation used elsewhere in this Appendix, if the interval t is hourly,

- AHUsem = Tm/Nm

and

MaxkWm = max(Lt)m.

AvgLF=m=112TmNmm=112maxt∈MmLtThus,

- On-peak/Off-peak Ratio

The on-peak/off-peak ratio is the ratio of total consumption during on-peak periods to the consumption during off-peak periods.  The specific definition depends on the definitions of on-peak and off-peak periods.  The ratio is calculated as

ron/off	=	(on-peak consumption)/(off-peak consumption)

=	TON/TOFF

where

TON = sum of interval Loads over all intervals in on-peak periods

TOFF = sum of interval Loads over all intervals in off-peak periods.

- Seasonal Consumption Ratios

Seasonal consumption ratios are the ratio of total consumption in one Season to consumption in another Season.  Most common is the ratio of summer to winter consumption.  The ratio of summer to annual or winter to annual consumption may also be used.  When using the seasonal consumption ratio calculation, the months of the Seasons being used shall be defined.  The ratios are calculated as

rS1/S2	=	(total for Season 1)/(total for Season 2)

=	TS1/TS2

where

TS1 and TS2 denote totals of interval Loads over the two Seasons being compared.

- Load-Weighted Average or Annual Price

- The Load-weighted average price U for a period is calculated as

- U = Σt Lt ut /Σt Lt

where

Lt = Load at interval t

ut = market price for commodity at time interval t

the summation is over all intervals t in the period.

- Equivalently, the Load-weighted average price may be calculated from the unitized Loads as

- U = t ft ut.

where

ft is the unitized Load defined above.

- Load-weighted average price is most often considered on an annual basis.  When the period is annual the Load-weighted average price is also called the Load-weighted annual price.

- Unlike the other parameters defined, Load-weighted average price is not only a characteristic of a Load shape, but depends also on the price series ut.  When Load-weighted average price is used, the price series shall be specified.

- Hourly and Quarter-Hourly Load Data

- Although ERCOT uses quarter-hourly data for Settlement, hourly Load data may be used for supporting analysis to assess Load Profiling Methods.  When hourly data are used, the intervals t are hourly.  In this case, the Load Lt in Demand for each interval t is equal to the energy for the interval, and the sum of the Loads T is the total profile energy for the period.

- For calculations expressed as the ratio of Loads, the ratio may be calculated in the same way whether the Load data are hourly or quarter-hourly.  The same result will be obtained either way.  This rule applies to unitized Loads, Load factor, on-/off-peak ratio, and seasonal consumption ratios.

- Measuring Differences Between Two Load Profiles

Direct Comparison of Simple Parameters

- Simple parameters are those that represent a Load shape in terms of a single number.  These parameters include the following:

- Load factor;

- On-/off-peak ratios;

- Seasonal consumption ratios; or

- Load-weighted average price.

  - Each simple parameter may be the basis for measuring differences between Load Profiles.  The value of the parameter for the Target Profile is subtracted from the value for the default profile.  For example, the difference in Load factors is expressed as:

LF = LFDefault – LFTarget

  - This difference may be thought of as the magnitude of the error if the default profile is used to represent the target.

- Comparison of Two Series

- A Load Profile over a year may be characterized by:

- Its unitized Load shape;

- The separately unitized Load shapes for each month;

- The 12 monthly fractions;

- The 24 clock-hour fractions for the year; or

- The set of 24 clock-hour fractions for each month or day-type.

  - For any of these series, two Load Profiles may be compared in terms of various summary measures of the difference between their two series.  Each of these summary measures is called a measure of “error” or “deviation.”  In the context of comparing two Load Profiles, this error is a measure of how far the default Load profile is from the target.

- Mean Deviation

The simplest measure of difference is the average difference in values or deviation between corresponding elements of the series.  The Mean deviation is calculated by:

- Taking the difference between the default and target for each element in the series; or

- Taking the Mean of these differences over all terms in the series.

- Mean Absolute Deviation (MAD)

MAD is calculated by:

- Taking the difference between the default and target for each element in the series;

- Taking the absolute value of each difference; or

- Taking the mean of these absolute differences over all terms in the series.

Thus,

- MAD = (1/J) j |XTARGETj - XDEFAULTj|

where

J is the number of elements in the series (e.g., 12 for monthly fractions, 365 for daily fractions of the year, 24 for clock-hour fractions of a period).

- Mean Absolute Percent Error (MAPE)

The MAPE is calculated by:

- Taking the difference between the default and target for each element in the series;

- Taking the absolute value of each difference expressing this absolute value as a percent of the target value; or

- Taking the Mean of these absolute percent errors over all terms in the series.

Thus,

- MAPE = (1/J) j |XTARGETj - XDEFAULTj|/XTARGETj

where

J is the number of elements in the series (e.g., 12 for monthly fractions, 365 for daily fractions of the year, 24 for clock-hour fractions of a period).

- Root Mean Square Error (RMSE)

The RMSE is calculated by:

- Taking the difference between the default and target for each element in the series;

- Squaring each difference;

- Taking the Mean of the squared difference over all terms in the series; or

- Taking the square root of the Mean squared difference.

Thus,

where

J is the number of elements in the series (e.g., 12 for monthly fractions, 365 for daily fractions of the year, 24 for clock-hour fractions of a period).

- Measuring Differences for a Group of Load Profiles

Some decisions require comparisons across a group of Load Profiles that are jointly affected by a possible change.  Key examples of such decisions are to either:

- Subdivide an existing segment into smaller segments;

- Subdivide an existing Weather Zone into smaller Weather Zones;

- Change segment definitions in a way that shall affect multiple segments; or

- Change the boundaries of Weather Zones in ways that shall affect multiple Weather Zones.

- Deadweight Loss Reduction

One measure that reflects the combined effects of a change on several Load Profiles is deadweight loss.  A reduction in deadweight loss is the gain in economic efficiency due to providing Customers with Load Profiles that are closer to their true Load shapes.  This reduction is a value to society, measured in dollars per year.  Given that ERCOT’s costs are ultimately paid by consumers through their electricity rates, this societal value is theoretically the maximum that it would be worth to implement a change paid for by ERCOT.  Changes that would cost more than this to implement shall cost more than the economic value of the benefit.

- Deadweight Loss Reduction from Finer Subdivision

- When performing an analysis where a single group is divided into smaller groups, the Load Profiles for the smaller segments or zones are considered the Target Load Profiles. The profile for the single group is considered the default Load Profile.  The difference between the default and each of the Target Profiles may be measured by any of the difference measures described in this Appendix titled Measuring Differences between Two Load Profiles, as described in Section 12.6.2, Difference from Current Load Profile Segments.

- The combined effect of subdividing may be expressed as the deadweight loss reduction.  The calculation formula is

DWL = ½ e k Ek U0 ((Uk – U0)/U0)2

- Where

e	=	Elasticity of electricity Demand with respect to the commodity price

Ek	=	Total annual consumption for subgroup k of group 0 (kWh)

Uk	=	Load-weighted annual price for subgroup k of group 0 ($/kWh)

U0	=	Load-weighted annual price for the group 0 ($/kWh).

- For purposes of this calculation, elasticity estimates from secondary sources may be considered, and scenario analysis using a range of values may be used.  A value of 0.2 has been used in some studies.  Note that the annual consumption Ek is the total energy use of all Customers represented by the subgroup profile k.  If the profile is scaled so that the profile hourly value is an estimate of total Load for all Customer represented by the profile, the annual consumption Ek is the sum of the profile hourly values over all hours in the year.  If the profile is scaled in some other way, the group annual consumption Ek may be very different from the sum of the profile hourly values.

- To apply this formula to subdivisions of a Weather Zone, the combined group 0 is the entire Weather Zone.  Each subgroup k is a subdivision proposed as a new Weather Zone.  The formula is evaluated separately for each Load Profile Segment, and summed over segments to provide the total benefit of the Weather Zone subdivision.

- To apply this formula to subdivisions of a segment, the combined group 0 is the entire current segment.  Each subgroup k is a subdivision proposed as a new segment.  The formula is evaluated separately for each Weather Zone, and summed over Weather Zones to provide the total benefit of the segment subdivision.

- In either case, the subgroup profiles k defined by profile segment and Weather Zone are applied to the interval prices ut to produce distinct Load-weighted average annual prices Uk.  The Load-weighted average annual price for the existing segment or Weather Zone is U0.

- Deadweight Loss Reduction if Only One Subdivision Changes

If only one proposed new segment shall have a new Load Profile, while the remainder shall continue to have the existing Load Profile, the deadweight loss formula reduces to

DWL = ½ e  Ek* U0 ((Uk* – U0)/U0)2

where the subscript k* indicates the proposed new segment.  All other terms in the summation for the full formula are zero.

- Deadweight Loss Reduction by Creating a New Segment from Multiple Segments

- When performing an analysis where several groups are being combined to form a single group, the Load Profile for the single group is considered the Target Load Profile.  The Load Profiles for the groups contributing to the formation of the single group are considered the default Load Profiles. Examples where such analyses shall apply include:

- Changing a Weather Zone boundary so that a portion is removed from one zone and added to another

- Changing a set of Load Profile Segment definitions so that part of one segment is shifted from one to another

- Choosing between two alternative schemes for defining Weather Zones or Load Profile Segments.

  - Any of these choices may be assessed as a difference among possible subdivisions using the formula titled “Deadweight Loss from Finer Subdivision.”  If a single group may be divided into subgroups the deadweight loss reduction from each possible subdivision is calculated using the formula.  The method with the greatest deadweight loss reduction is the preferred method.

  - Thus, to request a change of the definitions of existing segments, a combined segment that includes all segments affected by the change is considered.  The deadweight loss reduction from “subdividing” the combined segment is then calculated.  Likewise, the deadweight loss reduction from subdividing the combined segment into the proposed new set of definitions is calculated.  The reduction from the current segmentation is subtracted from the reduction from the proposed segmentation.  Theoretically, the proposed segmentation is justified in terms of societal value if this difference is greater than the total cost of implementing the change.

  - A change that involves moving a part of a group into another group, or a combination of such moves, may be assessed using this same approach.  The combination of all affected subgroups is considered as the overall group.  The original and alternative groupings are then regarded as two possible subdivisions of this overall group.  The deadweight loss reduction compared to having the full combined group is evaluated for each of these “subdivisions.”  The preferred subdivision is the one with the greater deadweight loss reduction from the combined group.  The benefit of going from the original subdivision to the new one is the increase in the deadweight loss reduction.

  - For example, if the group A is to be moved from being included with group B to being included with group C, the combined group is the combination of A, B, and C.  Under one “subdivision” the subgroups are A+B and C.  Under the other, the subgroups are B and A+C.  The change is theoretically worth making if the deadweight loss reduction for (B, A+C) versus (A+B+C) is greater than that for (A+B, C) versus (A+B+C), by an amount greater than the cost of making the change.

- Deadweight Loss Reduction from Revising the Load Profile Model Based on Current Load Research Data

(1)	The Load Profile based on current Load research may be regarded as the best available estimate of the Load Profile for a particular segment.  Continuing to use the current model is then viewed as an approximation to this best available or Target Profile.  One way to measure the severity of the error in this approximation is in terms of the deadweight loss.  Revising the current Load Profile to bring it closer to the target would reduce societal deadweight loss by at most this amount.

Table C-2

Hypothetical Loads and Prices for Illustration

(2)	The deadweight loss due to using the current Load Profile Model (default) rather than the target is calculated as

- DWL = ½ e E UDEFAULT ((UTARGET – UDEFAULT)/UDEFAULT)2

- Where

e 			= Elasticity of electricity Demand with respect to the commodity price

E 			= 	Total energy consumption for the profile segment (MWh)

UTARGET 	= 	Load-weighted average annual commodity price using the Target Profile for the segment ($/MWh)

UDEFAULT = Load-weighted average annual commodity price using the default profile for the segment ($/kWh).

- Illustration of Measures of Differences

- To illustrate some of the measures of differences, following Table C-2, Hypothetical Loads and Prices for Illustration, shows hourly Loads and hourly market prices for a hypothetical period of two days.  Loads are shown for two sub segments that together make up an existing segment within a Weather Zone.  The highlighted hours are on-peak hours, which are defined as hour ending 0800 through 1900.

- Following Table C-3, Single Parameter Characteristics and Differences, shows some of the single-parameter characteristics statistics for each of the three Load Profiles.  Also shown are the differences between each subsegment and the existing combined segment.  These differences are the errors if the subsegments are considered as the targets and the combined is the default that estimates them if the finer Segmentation is not adopted.

Table C-3

Single Parameter Characteristics and Differences

- Following Table C-4, Daily and Clock-Hour Totals and Fractions, shows the calculations of daily and clock-hour totals and fractions.

Table C-4
Daily and Clock-Hour Totals and Fractions

- Following Table C-5, Unitized Loads and Difference Measures, shows the unitized Load for the two-day period, and illustrates some of the difference measures based on this series.

Table C-5
Unitized Loads and Difference Measures

First, the unitized Loads are shown for each Load Profile.  The differences or errors between each segment and the existing Load Profile are then shown for the unitized Loads.  Also shown are the absolute difference, squared error, and absolute percent errors.  These are combined at the bottom to give the mean deviation, MAD, MAPE, and RMSE.

- Following Table C-6, Calculation of Deadweight Loss Reduction for Finer Segmentation, illustrates a calculation of the reduction in deadweight loss achieved by changing from the single existing Load Profile to separate Load Profiles for each subsegment.

Table C-6
Calculation of Deadweight Loss Reduction for Finer Segmentation