| PGRR Number | 046 | PGRR Title | Addition of Geomagnetically Induced Current (GIC) Model Building Requirements |
| --- | --- | --- | --- |
| Date of Decision |  | March 23, 2016 |  |
| Action |  | Tabled |  |
| Timeline |  | Normal |  |
| Proposed Effective Date |  | To be determined |  |
| Priority and Rank Assigned |  | To be determined |  |
| Planning Guide Sections Requiring Revision |  | 2.2, Acronyms and Abbreviations 6.11, Process for Developing Geomagnetically-Induced Currents (GIC) System Models (new) |  |
| Related Documents Requiring Revision/Revision Requests |  | RRGRR009, Adding Voltage Limit Sets, Relay Loadability, MLSE, and GMD Data Planning Geomagnetic Disturbance Task Force (PGDTF) Procedure Manual |  |
| Revision Description |  | This Planning Guide Revision Request (PGRR) aligns the Planning Guides with North American Electric Reliability Corporation (NERC) Reliability Standard TPL-007-1, Transmission System Planned Performance for Geomagnetic Disturbance Events, by specifying a process for developing geomagnetically-induced current system models. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | To satisfy NERC Reliability Standard TPL-007-1 regulatory requirements. |  |
| PLWG Decision |  | On 3/23/16, the Planning Working Group (PLWG) was in consensus to table PGRR046 for one month. |  |
| Summary of PLWG Discussion |  | On 3/23/16, participants discussed associated RRGRR009 requirements and decided tabling for one month would allow time to review comments from Market Participants at the April PLWG meeting. |  |

| Sponsor |  |
| --- | --- |
| Name | James Teixeira |
| E-mail Address | jteixeira@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6582 |
| Cell Number | 512-656-6734 |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Lindsay Butterfield |
| E-Mail Address | Lindsay.Butterfield@ercot.com |
| Phone Number | 512-658-6521 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None |  |

Market Rules Notes

None

Proposed Guide Language Revision

## ACRONYMS AND ABBREVIATIONS

GIC	Geomagnetically-Induced Currents

- 6.11	Process for Developing Geomagnetically-Induced Currents (GIC) System Models

(1)	To simulate geomagnetic disturbances, it is necessary to establish and maintain GIC system models and conduct geomagnetic disturbance vulnerability assessments to determine whether the ERCOT System can withstand performance requirements of the benchmark geomagnetic disturbance event described in NERC Reliability Standards.  These models, known as GIC base cases, shall contain appropriate system data, and shall represent projected system conditions that provide a starting point for the required year(s).

(a)	Transmission Service Providers (TSPs) and ERCOT shall develop the GIC base cases.  These base cases are derived from the steady-state base cases developed by Steady-State Working Group (SSWG) for the near-term transmission planning horizon to ensure consistency between the System topology in the SSWG base cases and GIC system models.

(b)	Each TSP, or its Designated Agent, shall provide its respective transmission network GIC model data in accordance with the Planning Geomagnetic Disturbance Task Force (PGDTF) Procedure Manual.

(c)	Each Resource Entity, or its Designated Agent, shall provide its respective Resource Entity-owned generating units, plants, and Generation Step Ups (GSUs) connected to the ERCOT System GIC model data in accordance with the PGDTF Procedure Manual.

(d)	ERCOT shall aggregate the GIC system model data supplied by each TSP and Resource Entity and shall compile and maintain the GIC system models.

(e)	Other requirements for the GIC system model maintenance and use for conducting assessments can be found in the PGDTF Procedure Manual.

(f)	GIC Data is considered Protected Information pursuant to Protocol Section 1.3, Confidentiality.