| SCR Number | XXX | SCR Title | Generation Curtailment Constraint Above the Resource Node |
| --- | --- | --- | --- |
| Date Posted |  | March XX, 2016 |  |
| Requested Resolution |  | Normal |  |
| Supporting Protocol or Guide Section(s) (If applicable) |  | Protocol Section 6.5.7.1.13, Data Inputs and Outputs for the Real-Time Sequence and SCED |  |
| Other Document Reference/Source |  | None |  |
| System Change Description |  | This System Change Request (SCR) reduces the need for manual High Dispatch Limit (HDL) overrides by entering a constaint above the Resource Node for each node in the Network Operations Model to be automatically activated and passed to Security-Constrained Economic Dispatch (SCED), Day-Ahead Market (DAM) and Reliability Unit Commitment (RUC). |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | Given the number of Resource Nodes currently in the Network Operations Model, close to 600 ,automation is needed to ensure timely and accurate behaviors in SCED. |  |

| Sponsor |  |
| --- | --- |
| Name |  |
| E-mail Address |  |
| Company |  |
| Phone Number |  |
| Cell Number |  |
| Market Segment |  |

| Market Rules Staff Contact |  |
| --- | --- |
| Name |  |
| E-Mail Address |  |
| Phone Number |  |

Business Case for Proposed System Change

Issue:

The BOD and TAC have requested ERCOT to work with stakeholders on determining alternative solution for an NPRR649 which got appealed at BOD.  NPRR 649 proposed a settlement solution to pay a resource for “lost opportunity” due to an HDL override. HDL overrides are used in Operations to pre-posture for outages, ensure reliability against unsolved contingencies, reduce post-contingency flows which are above 125% and resolve unit control system issues. This system change request would allow the Operator to impose a flow limit by using a generic transmission constraint in place of overriding the resource parameters.

Resolution:

Create and maintain generation curtailment constraints corresponding to each RN.  Develop displays to help operator decide which generation curtailment constraint to activate to get the HDL override that the operator desires. Develop logic to get the generation curtailment constraints in SCED/DAM/RUC. Update reporting to include additional attribute.

Systems changes include:

| CIM Model |  |
| --- | --- |
|  | Define Power Plant Generation Curtailment Constaints (GCC's) (close to 600) Under existing Branch Group Class & With a new class to distinguish these powerplant GCC's |
| EMS |  |
|  | Add new class in existing NETMOM INTRFC record to Power Plant GCC's attribute and change CIM Importer to consume the attribute from CIM Model |
|  | RTCA/TCM Change to hold close to 600 Manual Constraints (of Power Plant GCC's) and perform pre processing for shift factor calculations once they are activated. Develop New TCM display to sort/filter based on station name. Develop the GCC's Day Ahead Limits Interface (Display and database changes) based on the data from flat file and transfer to MMS with current DA Limit Data Transfer. |
|  | EMS Interface Changes to transfer the Power Plant GCC's attribute to MMS as part of TCM Constraint Table |
| MMS |  |
|  | MMS CIM Importer to consume the attribute from CIM Model |
|  | SCED Changes to consume the attribute and display in Input/Output Display of Constraints and publish to MIDB Tables |
|  | DAM Changes to consume the attribute and display in Input/Output Display of Constraints and publish to MIDB Tables |
|  | RUC Changes to consume the attribute and display in Input/Output Display of Constraints and publish to MIDB Tables |
| Reports |  |
|  | Existing SCED Activated Constraints Report change to add new class that distinguishes these Power Plant GCC's from GTC's |
|  | Generic Transmission Limits Report (MIS, Grid, Transmission) either update existing posting or develop new report for DAM |
| Operations (DAM, RUC, RT) | Develop Operator training for use of new displays and attributes |