| SCR Number | 789 | SCR Title | Update NMMS Topology Processor to PSSE 34 Capability |
| --- | --- | --- | --- |
| Date Posted |  | March 8, 2016 |  |
| Requested Resolution |  | Normal |  |
| Supporting Protocol or Guide Section(s) |  | Protocol Section 3.10, Network Operations Modeling and Telemetry Planning Guide Section 6.1, Steady-State Model Development |  |
| Other Document Reference/Source |  | None |  |
| System Change Description |  | This System Change Request (SCR) updates the Network Model Management System (NMMS) Topology Processor to output Power System Simulator for Engineering (PSS/E) 34 RAW files. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | The NMMS Topology Processor is used to satisfy Protocol Section 3.10 requirements that, “ERCOT shall use consistent information within and between the various models used by ERCOT Operations, ERCOT Planning, and other workgroups in a manner that yields consistent reults.” Power Technologies International (PTI) PSS/E is the software tool used by most Transmission Planning entities in ERCOT to conduct transmission planning, steady state, and dynamic studies. Version 34 is the latest version of PSS/E. The RAW file format created by PTI is the industry standard for transferring bus/branch models typically used for transmission planning between software supplied by various vendors such as Powerworld, Uplan, Congestion Revenue Rights (CRRs), etc... This SCR will update the capability of the NMMS Topology Processor to add PSS/E 34 capability that is desired to be used in the annual planning and Steady State Working Group (SSWG) Base Cases to the RAW file created by the NMMS Topology Processor. This data includes an additional rating required by North American Electric Reliability Corporation (NERC) Standard TPL001-4, Tranmissions System Planning Performance Requirements, transmission line names, Distributed Generation (DG) included on load feeders, and node-breaker modeling. |  |

| Sponsor |  |
| --- | --- |
| Name | James Teixeira |
| E-mail Address | jteixeira@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6582 |
| Cell Number | 512-656-6734 |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Cory Phillips |
| E-Mail Address | cphillips@ercot.com |
| Phone Number | 512-248-6464 |

Business Case for Proposed System Change

Issue:

In order to keep pace with Power System Simulator for Engineering (PSS/E) capability, a change to the Network Model Management System (NMMS) Topology Processor is needed so that data can be stored and extracted from the NMMS Network Operations Model (NOM) database using the NMMS Topology Processor for use in planning and Congestion Revenue Right (CRR) models.  System Change Request (SCR) 760, Recommended Changes Needed for Information Model Manager and Topology Processor for Planning Models, included a mandate to “implement an evaluation process where SSWG recommends to ROS and ROS approves which new PSS/E data models should be evaluated for implementation into the [NMMS].”  The SSWG described the evaluation process in the SSWG Procedure Manual, Section 5.3, as directed by the 3/22/11 Board Report for SCR760.

Resolution:

The SSWG completed its review and the following items where chosen for inclusion in the NMMS Topology Processor:

Item 1: Increased Customizable Ratings Sets

The number of ratings available in PSS/E 34 has increased from three to twelve. Due to this limitation, only three of the four static ratings (Normal, Emergency, 15 minute, and Conductor/Transformer) in the NOM database can be extracted into the planning models.  Due to North American Electric Reliability Corporation (NERC) Standard TPL001-4, Tranmissions System Planning Performance Requirements, a new rating, Relay Loadability Rating, needs to be extracted.  The increase in the number of ratings available will allow all four current NOM ratings and the new rating to be extracted to the planning models.  Other ratings could also be extracted from the NOM database such as the dynamic Emergency rating.

Item 2: Branch/Transformer Names

Currently branches (AC Lines and Series Compensators in the NOM) in the planning cases do not have a place holder for a name.  In the planning cases, branches could be mapped to NOM names much more easily than they can currently be.  All AC lines, two-winding transformers, and three-winding transformers can have names up to 40 characters long. Names can be blank, however, all non-blank names must be unique within the case for future name-based identification.

The CRR model uses branch names in a mapping document that has LNNAME mapped to From Bus,To Bus and Circuit.  CRR would have to upgrade their system to accept a version 34 formatted RAW file before this would benefit them.

TSPs may desire to use their own name from the IMM Local Name attribute rather than a concatenation of the Line + ACLineSegment for line names and Power Transformer Name for transformers.  The Local Name field is in all objects inheriting from ConductingEquipment.  Would need validation rule to make this Local Name unique.

Item 3: Modeling Distributed Generation on Load Feeders

Distributed Generation (DG) (for example, rooftop solar) can be modeled as part of the load data in separate fields.  This would make it much easier to identify where DG is modeled in the case.

Item 4: Node-Breaker Modeling

Version 34 introduces fully-integrated Node-Breaker modeling into the power flow and contingency analysis engines. This type of modeling already exists in the NOM and should be relatively easy to pull it out for use in the planning cases.  Substation names and contingencies that only open one end of a line could be created making the planning cases more consistent with the Operations cases.

Item 5: New Contingency Types to Support Node-Breaker Modeling

Several new contingency definitions have been added to the CON file specification to leverage Node-Breaker substation models. Breakers can be opened, closed and isolated. Node voltages and breaker flows can also be reported using new MON file additions.