| PGRR Number | 042 | PGRR Title | Regional Transmission Plan Model Reserve Requirement and Load-Generation Imbalance Methodology |
| --- | --- | --- | --- |

| Date | April 19, 2016 |
| --- | --- |

| Submitter’s Information |  |
| --- | --- |
| Name | Mark Walker and Randy Jones |
| E-mail Address | mark.walker@nrg.com and rajones@calpine.com |
| Company | NRG Texas Power LLC and Calpine Corporation |
| Phone Number | 512-691-6261 / 713-830-8846 |
| Cell Number | 512-585-0450 / 832-385-3322 |
| Market Segment | Independent Generator |

Comments

NRG Texas Power LLC and Calpine Corporation (NRG and Calpine) submit these comments on Planning Guide Revision Request (PGRR) 042 building on the comments submitted by ERCOT dated April 8, 2016 as the baseline.

NRG and Calpine have been working with ERCOT and other stakeholders on this PGRR for over a year.  NRG and Calpine continue to believe that significant policy changes are needed in the ERCOT transmission planning process to ensure that planning decisions are built upon sound foundations of reasonable assumptions and methodologies.  At a time when transmission costs have skyrocketed - tripling since 2002 - it is more imperative than ever to infuse needed discipline and focus to ensure the ratepayers’ dollars are prudently invested.  Additionally, the ERCOT Region’s energy-only wholesale market construct requires effective checks and balances to ensure that competitive resource solutions are allowed to present themselves to solve system needs and are not obscured by excessive and premature transmission investment driven by unduly extreme assumptions and methodologies.

Load Forecasts

The current approach in ERCOT transmission planning starts from the “base case” model forming the initial basis for ERCOT’s studies utilizes the Steady State Working Group (“SSWG”) Load forecasts. These are bus-by-bus forecasts that each Transmission Service Provider (“TSP”) provides to the SSWG, and which the SSWG sums and uses to produce a Weather Zone level non-coincident peak Load forecast, i.e. the expected peak demand for each individual Weather Zone, which may or may not coincide with the time of the entire ERCOT system’s “coincident peak” demand.

Neither the SSWG nor ERCOT’s transmission planners are permitted by current rules to engage in rigorous review of such forecasts. The SSWG has never adopted any standardized criteria for the TSP to utilize in preparing such forecasts, leading to great disparity between various TSPs methodologies. Not only does the SSWG not critique or evaluate the TSP forecasts, no Market Participants other than TSPs may even participate in SSWG meetings and reviews. Nor may ERCOT’s transmission planners adjust SSWG forecasts with that TSP’s concurrence. As such, reliance on SSWG forecasts do not benefit from a truly independent review by ERCOT for the most critical component of a power flow model—the initial peak demand forecast. Rather, ERCOT must accept the Load forecast supplied by TSPs, who may be inherently biased towards finding a need for transmission projects for TSPs are guaranteed cost recovery and investment rate of return (or the equivalent). This inherent, structural conflict of interest should not be permitted on this key component of transmission planning – particularly when there is a more reliable alternative.

In 2014, ERCOT implemented dramatic changes to how it forecasts power demand, using much more sophisticated techniques and tools, such as its neural network, that have proven very accurate.  In ERCOT’s own words, the new forecasts show a “more realistic view of the future electric demand we need to be prepared to serve.”  [ERCOT press release Feb. 28, 2014.]  Indeed, the products of the new forecast methodology have set an impressive standard for accuracy that is remarkable, particularly in predicting the peak demand of the system, which is what matters in transmission planning.  Recent ERCOT reports confirm this accuracy:

ERCOT 2015 Demand and Energy Report, January 15, 2016

ERCOT’s enhanced, sophisticated forecast derives from an econometric model involving customer premise counts, weather data, and economic forecasts that ERCOT obtains from outside vendors.  ERCOT compiles the information and applies it against its existing consumption data to derive a Weather Zone level non-coincident peak forecast spanning all customer classes.  This forecast undergoes significant review, both internally at ERCOT and through stakeholder review, including by TSP representatives who actively participate in the stakeholder process. Also, unlike the SSWG forecast, ERCOT’s transmission planners may adjust the 90/10 forecast if it fails to include known revisions (such as industrial Load development) or to accurately reflect expected future conditions.  In addition, ERCOT has enhanced its staffing by adding a “competitive intelligence” specialist who actively monitors potential Load additions, and that this individual has had high levels of success including estimates “almost exactly on the button” regarding Load additions in the Houston area.

NRG and Calpine believe that the ERCOT 90/10 forecast, fortified by enhanced ERCOT staff expertise, is now the appropriate basis for all transmission planning in ERCOT.  The time has come to take the obvious prudent step in the advancement of the ERCOT transmission planning process to take full advantage of the prudent investment by ERCOT to achieve high levels of accuracy in peak demand forecasting – and to eliminate the use of widely inconsistent, unchecked SSWG-based forecasts and instead use ERCOT’s independent forecast as the basis for all transmission planning.   This advancement will achieve sounder decision-making by ERCOT and would eliminate a clear conflict of interest in the current process.  In addition, this modification will bring much needed consistency between ERCOT’s transmission planning process and ERCOT’s critical role in forecasting and reporting on ERCOT resource adequacy.

Notwithstanding ERCOT’s addition of market intelligence capabilities, NRG and Calpine concede that parties, such as TSPs, who have “on the ground” connection with activities in their service areas should be given the opportunity to provide input to ERCOT regarding specific, documented, and measurable planned Load additions (such as large industrial developments) that may not be apparent to ERCOT in its forecasting process.  ERCOT may then evaluate whether and how to make reasoned adjustments to the Load forecasts;  however, ERCOT must take care to consider whether such additions may have already been taken into account in ERCOT’s methodology, such as customer premise additions and economic trend assumptions, to ensure such identified Load adjustments are not “double counting” the same activity.

Reasoned Approach to Solve Imbalanced Cases

ERCOT’s April 8, 2016, comments remove virtually all guidance and limitations, previously advanced in this PGRR, on how ERCOT addresses circumstances where a planning case cannot “solve” due to inadequate generation in the planning model, resulting from Load forecasts that lean towards higher (even extreme) Load levels and limitations in the Planning Guide on what resources may be counted as being available in future years.  NRG and Calpine disagree.  It is vitally important, given the significant ratepayer impact of transmission planning decisions and market impacts of those decisions, that clear standards be in place to ensure rigorous and consistent standards and discipline are applied - to each and every project – to ensure that the ERCOT planning process only moves forward with projects that are truly necessary to sustain a reliable grid or can meet economic criteria.  This will help ensure that participants in the competitive market are given an opportunity to respond to system needs before premature, regulated solutions are employed.

ERCOT’s April 8 comments instead propose an approach that allows transmission planners to suggest any method and study assumptions they choose – and then “give proper notice and an opportunity to comment . . . regarding study assumptions . . .” in the Regional Planning Group stakeholder process.  This is an untenable approach.  First, given the high impact of these planning decisions it is imperative that there be clear and consistent standards for review of all transmission projects.  The piecemeal standards that would result from ERCOT’s proposal are by their nature unreliable, would lead to inconsistent results, and are prone to biases and hijacking by parties in the best positions to influence outcomes for their own benefit.  Second, as a practical matter the only parties in the RPG process that can effectively analyze and offer meaningful comments on such one-off planning assumptions are the TSPs themselves, which either benefit from certain potential biases in the analysis and/or who are the direct or indirect beneficiaries in each particular project review (the ability to build rate-based facilities with guaranteed cost recovery and rate of return). Consumers and competitive market participants simply do not have access to the same level of system data as TSPs and do not have the sophisticated analytical resources needed to engage in the kind of rigorous auditing analyses of the studies and assumptions that would be needed to result in a balanced vetting of proposed piecemeal assumptions.  Finally, the RPG does not have a stakeholder voting structure and its participants are highly weighted in favor of TSP perspectives - thus consumer and competitive market views are unlikely to gain a fair measure of input.

Prior iterations of PGRR042 have set forth a number of measures designed to create reasonable “guardrails” for allowable techniques that ERCOT can use to modify planning cases that cannot solve due to inadequate forecasted generation.  Rather than recite those discussions here, NRG and Calpine suggest the reader look to those prior documents that can be found at: http://www.ercot.com/mktrules/issues/PGRR042

In these comments, NRG and Calpine suggest reinstating, with some modifications, the limitations suggested in prior comments, which ERCOT has suggested removing in its April 8 comments.  In summary, NRG and Calpine support allowing ERCOT specific techniques to achieve a balance case, in order of priority:

  - Increase the dispatch level of wind generation up to the seasonal peak average.

  - Increase the dispatch level of (utility scale) solar to 100%.

  - Increase the import from DC Ties to their full seasonal net maximum.

  - Add mothballed generation that have not announced their return to service during the study period.

  - Add any or all proposed generation resources that have signed an interconnection agreement but have not met other requirements.

  - Scaling as last resort – using multiple study regions (but in no case less than four) scale down Load in Weather Zones (outside study area) -

    - only to the point still necessary to solve the model,

    - but never reduce Load in a zone below its average percentage of peak Load during the top ten hourly peak Load conditions of the study area Weather Zone.

With these objectives as guidance, NRG and Calpine provide the following suggested changes to the 7/2/15 ERCOT comments for PGRR042 [building on ERCOT comments dated April 8, 2016.  The ERCOT Nodal market is designed to function based on Market Participant response to price signals - and rate-based utility solutions to support reliable delivery.  If market signals are distorted by overly aggressive and premature transmission investment, more efficient market solutions are not allowed to fully form, and unnecessary rate-based transmission solutions will add to the already high cost burden on ratepayers instead of private investment.  Without needed reform, the impact of ERCOT’s transmission planning will continue to add to the unduly extreme growth in transmission costs charges to consumers.

INCLUDEPICTURE  "cid:image004.png@01D167FC.A4BCEF40" \* MERGEFORMATINET

Revised Cover Page Language

| Planning Guide Sections Requiring Revision | 2.1, Definitions 3.1.1, Overview of Major Transmission Planning Activities 3.1.1.2, Regional Transmission Plan 3.1.2.1, All Projects 3.1.3, Project Evaluation 3.1.3.1, Definitions of Reliability-Driven and Economic-Driven Projects 3.1.4.1, Development of Regional Transmission Plan 3.1.4.1.1, Regional Transmission Plan Cases (new) 3.1.4.2, Use of Regional Transmission Plan |
| --- | --- |

Revised Proposed Guide Language

- 2.1	DEFINITIONS

- ERCOT 90/10 Forecast

- The non-coincident peak Load forecast developed independently by ERCOT delineating, by Weather Zone, the peak demand forecast having a 10% probability of being exceeded by the actual peak demand; and that such forecast is derived from the same underlying independent ERCOT Load forecast used by ERCOT for resource adequacy planning and reporting.

- 3.1.1	Overview of Major Transmission Planning Activities

(1)	The process of planning a reliable and efficient transmission system for the ERCOT Region is composed of several types of activities and studies.

(2)	Consideration of the Year 6 case in the Regional Transmission Plan is required.

- 3.1.1.2	Load Forecast and Regional Transmission Plan

(1)	ERCOT shall create ERCOT 90/10 Forecasts to establish Regional Transmission Plan base cases for use in Regional Transmission Plan activities and RPG Project Review.  On an annual basis, ERCOT shall prepare and present for RPG comment the planned release of the ERCOT 90/10 Forecast regarding future planning years.  ERCOT shall consider comments from stakeholders and may make modifications to the ERCOT 90/10 Forecast for specific, documented, and measurable future Load additions.  ERCOT must ensure that any such adjustments to forecasted Load are not duplicative of assumptions of Load growth already a part of the ERCOT 90/10 Forecast, and must minimize any Load modifications accordingly.

(2)	The Regional Transmission Plan is developed annually by ERCOT, in coordination with the RPG and Transmission Service Providers (TSPs).  The Regional Transmission Plan addresses regional and ERCOT-wide reliability and economic transmission needs and the planned improvements to meet those needs for the upcoming six years starting with the ERCOT 90/10 Forecast base cases.  These planned improvements include projects previously approved by the ERCOT Board, projects previously reviewed by the RPG, new projects that will be refined at the appropriate time by TSPs in order to complete RPG review, and the local projects currently planned by TSPs.  Combined, these projects represent ERCOT’s plan which addresses the reliability and efficiency of the ERCOT System in order to meet North American Electric Reliability Corporation (NERC) Reliability Standards, the Protocols, Operating Guides and this Planning Guide.  Projects that are included in the Regional Transmission Plan are not considered to have been endorsed by ERCOT until they have undergone the appropriate level of RPG Project Review as outlined in Protocol Section 3.11.4, Regional Planning Group Project Review Process, if required.  The process used by ERCOT to develop the Regional Transmission Plan is outlined in Section 3.1.4, Regional Transmission Plan Development Process.

(3)	ERCOT shall post the Regional Transmission Plan to the Market Information System (MIS) Secure Area by December 31 of each year.

(4)	ERCOT shall include in the Regional Transmission Plan report a list of Transmission Facilities that are loaded above 95% of their applicable Ratings for the following conditions:

(a)	Normal system conditions; or

(b)	Following the contingency loss of a single generating unit, transmission circuit, transformer, or common tower outage.

- 3.1.2.1	All Projects

(1)	The submittal of each transmission project (60 kV and above) for RPG Project Review should include the following elements:

(a)	The proposed project description including expected cost, feasible alternative(s) considered, transmission topology and Transmission Facility modeling parameter data, and all study cases used to generate results supporting the need for the project in electronic format (powerflow data should be in PTI PSS/E RAWD format).  Also, the submission should include accurate maps and one-line diagrams showing locations of the proposed project and feasible alternatives (AutoCad-compatible format preferred);

(b)	Identification of the ERCOT 90/10 Forecast base cases or Regional Transmission Plan powerflow cases used as a basis for the study and any associated changes that describe and allow accurate modeling of the proposed project;

(c)	Description and data for all changes made to the ERCOT 90/10 Forecast base cases or Regional Transmission Plan cases used to identify the need for the project, such as Generation Resource unavailability and area peak Load forecast;

(d)	A description of the reliability and/or economic problem that is being solved;

(e)	Desired/needed in-service date for the project, and feasible in-service date, if different; and

(f)	The phone number and email address of the single point of contact who can respond to ERCOT and RPG participant questions or requests for additional information necessary for stakeholder review.

- 3.1.3	Project Evaluation

(1)	ERCOT and the RPG shall evaluate proposed transmission projects using a variety of tools and techniques as needed to ensure that the system is able to meet applicable reliability criteria in a cost-effective manner.  For most proposed projects, several alternatives will be identified to meet the reliability criteria or other performance improvement objectives that the proposed project is designed to meet.  The project alternative with the expected lowest cost over the life of the project is generally recommended, subject to consideration of the expected long-term system needs in the area, and consideration of the relative operational impacts of the alternatives.

(2)	In some cases, one alternative may be to dispatch the system in such a way that all reliability requirements are met, even without the proposed transmission project or any transmission alternative, resulting in a less efficient dispatch than what would be required to meet the reliability requirements if the proposed project was in place.  Consideration of the merits of this alternative relative to the proposed transmission project is more complex.  To facilitate the discussion and consideration of these alternatives, ERCOT has adopted certain definitions and practices, described in paragraph (4) of Protocol Section 3.11.2, Planning Criteria, and Sections 3.1.3.1, Definitions of Reliability-Driven and Economic-Driven Projects, and 3.1.3.2, Reliability-Driven Project Evaluation below.

(3)	In its independent review of reliability-driven projects classified as Tier 1 or 2 pursuant to Protocol Section 3.11.4, Regional Planning Group Project Review Process, ERCOT shall utilize the following procedures to satisfy Load/generation imbalances for projects intended to solve the identified reliability criteria violation of a transmission circuit that crosses at least one Weather Zone boundary:

(a)	ERCOT shall not decrease the Load from the forecasted level in any of the Weather Zones in which a transmission circuit with an identified reliability criteria violation is located.

(b)	ERCOT may utilize the following in the order listed below to satisfy Load/generation imbalances:

(i)	Increase the Dispatch level of each Wind-powered Generation Resource (WGR) up to the Seasonal Peak Average Wind Capacity as a Percent of Installed Capacity as defined in Protocol Section 3.2.6.2.2, Total Capacity Estimate.

(ii)	Increase the Dispatch level of each PhotoVoltaic Generation Resource (PVGR) up to the Solar Unit Capacity as defined in Protocol Section 3.2.6.2.2, Total Capacity Estimate.

(iii)	Increase the output from the Direct Current Ties (DC Ties) to their full Seasonal net max sustainable ratings for DC Tie Resources importing into the ERCOT Region as defined in Protocol Section 3.2.6.2.2, Total Capacity Estimate.

(iv)	Add any or all Mothballed Generation Resources that have not yet announced their return to service during the study period.

(v)	Add any or all proposed Generation Resources that are outside of any study Weather Zone and have signed Standard Generation Interconnection Agreements (SGIAs) but have not yet met the other requirements of Section 6.9, Addition of Proposed Generation Resources to the Planning Models.

(vi)	Reduce Load in the study case outside of the Weather Zones in which the identified reliability criteria violation of a transmission circuit is located such that the total Load in the case is equal to ERCOT 90/10 Forecast system wide coincident peak Load forecast plus self-serve Load.  The Load scaling in any single Weather Zone shall never reduce the Load in the scaled Weather Zone below its average percentage of peak Load during the top ten hourly peak Load conditions for the past three years of the study Weather Zone.

(4) 	As part of its independent review of any project classified as Tier 1 pursuant to Protocol Section 3.11.4, ERCOT shall perform a sensitivity analysis to evaluate the effect on a recommended transmission project of proposed Generation Resources that have signed Standard Generation Interconnection Agreements (SGIAs) but were not included in the study cases because they did not meet all of the requirements for inclusion in the cases pursuant to Planning Guide Section 6.9, Addition of Proposed Generation Resources to the Planning Models.

- 3.1.3.1	Definitions of Reliability-Driven and Economic-Driven Projects

(1)	Proposed transmission projects are categorized for evaluation purposes into two types:

(a)	Reliability-driven projects; and

(b)	Economic-driven projects.

(2)	The differentiation between these two types of projects is based on whether a simultaneously-feasible, security-constrained generating unit commitment dispatch is expected to be available for all hours of the planning horizon that can resolve the system reliability issue that the proposed project is intended to resolve.  If it is not possible to simulate a dispatch of the Generation Resources such that all reliability criteria are met without the project, and the addition of the project allows the reliability criteria to be met, then the project is classified as a reliability-driven project.  If it is possible to simulate a dispatch of the Generation Resources in such a way that all reliability criteria are met without the project, but the project may allow the reliability criteria to be met at a lower total cost, then the project is classified as an economic-driven project.  When performing a simulation of the generating unit commitment and dispatch, only contingencies and limits that would be considered in the operations horizon shall be simulated.

- 3.1.4.1	Development of Regional Transmission Plan

(1)	The planning process begins with computer modeling studies of the generation and Transmission Facilities and substation Loads under normal conditions in the ERCOT System.  Contingency conditions along with changes in Load and generation that might be expected to occur in operation of the ERCOT Transmission Grid are also modeled.  To maintain adequate service and minimize interruptions during Outages, model simulations are used to identify adverse results based upon the planning criteria and to examine the effectiveness of various problem-solving alternatives.

(2)	The effectiveness of each alternative will be evaluated under a variety of possible operating environments because Loads and operating conditions cannot be predicted with certainty.  As a result, repeated simulations under different conditions are often required.  In addition, options considered for future installation may affect other alternatives so that several different combinations must be evaluated, thereby multiplying the number of simulations required.

(3)	Once feasible alternatives have been identified, the process is continued with a comparison of those alternatives.  To determine the most favorable, the short-range and long-range benefits of each alternative must be considered including operating flexibility and compatibility with future plans.

- 3.1.4.1.1	Regional Transmission Plan Cases

(1)	The starting base cases for the Regional Transmission Plan development are created by removing all Tier 1, 2 and 3 projects that have not received RPG acceptance or, if applicable, ERCOT endorsement  RPG Pfrom the most recent SSWGERCO/10 Forecast  summer base cases.

(2)	ERCOT shall set all non-seasonal Mothballed Generation Resources to out of service in the Regional Transmission Plan reliability base cases. ERCOT shall add proposed Generation Resources that have met the criteria for inclusion in Section 6.9, Addition of Proposed Generation Resources to the Regional Transmission Plan base cases.

(3)	ERCOT shall update the Regional Transmission Plan reliability and economic base cases to reflect any updates to the amount of Switchable Generation Resource capacity available to the ERCOT Region.

(4)	In the Regional Transmission Plan reliability base cases, ERCOT shall set the output from the DC Ties at the Seasonal net max sustainable ratings for DC Tie Resources as defined in Protocol Section 3.2.6.2.2, Total Capacity Estimate.

(5)	In the Regional Transmission Plan reliability base cases, ERCOT shall dispatch hydro Generation Resources up to the Hydro Unit Capacity as defined in Protocol Section 3.2.6.2.2, Total Capacity Estimate.

(6)	In the Regional Transmission Plan economic base cases, 8,760-hour profiles shall be used for hydro Generation Resources, WGRs, PVGRs and DC Ties.  ERCOT profiles shall be used for WGRs and PVGRs.  Average historical output for the past three years shall be used to create the hydro Generation Resource and DC Tie profiles.

(7)	The Load utilized in the Regional Transmission Plan reliability base cases shall be organized and evaluated by Weather Zone.  ERCOT shall use each Weather Zone’s 90th percentile peak Load forecast, plus self-serve Load, for the study year.  ERCOT may adjust this Load forecast to reflect specific, publicly known Load additions or subtractions that ERCOT reasonably anticipates.

(8)	If the total generation capacity in a Regional Transmission Plan reliability base case is less than the peak Load in the case plus losses plus an operating reserve equal to the two largest units in the case, ERCOT shall group one or more Weather Zones into no fewer than four study regions and create a separate base case for each study region for the season and year being studied.

(a)	ERCOT shall not change Load or total generation capacity inside a study region.  ERCOT may redispatch dispatchable Generation Resources inside a study region as necessary.

(b)	ERCOT shall use the following procedures in the order listed below to balance the case:

(i)	ERCOT may increase the Dispatch level of each WGR and PVGR outside the study region to a level that does not exceed the following maximums.

(A)	For a WGR, the maximum Dispatch level is the Seasonal Peak Average Wind Capacity as a Percent of Installed Capacity as defined in Protocol Section 3.2.6.2.2, Total Capacity Estimate.

(B)	For a PVGR, the maximum Dispatch level is the Solar Unit Capacity as defined in Protocol Section 3.2.6.2.2, Total Capacity Estimate.

(ii)	ERCOT shall reduce Load outside of the study region such that the total Load in the case is approximately equal to ERCOT’s 90th percentile system-wide coincident peak Load forecast plus self-serve Load.

(iii)	If the adjustments in paragraphs (7)(b)(i) and (ii) above are still not enough to balance the case, Load outside the study region may be reduced to a level sufficient to balance the case, provided that such Load reductions never reduce the Load in a scaled Weather Zone below its average percentage of peak Load during the top ten hourly peak Load conditions for the past three years of the study Weather Zone.

- 3.1.4.2	Use of Regional Transmission Plan

(1)	If a project submitted for RPG review is included in the Regional Transmission Plan, and no changes are identified which would affect the need for the proposed project through the 21-day comment period described in Section 3.1.5, Regional Planning Group Comment Process, then the Regional Transmission Plan may serve as the ERCOT Independent Review of the proposed project, if required.

(2)	Tier 1, 2, and 3 projects that are included in the Regional Transmission Plan should be submitted for RPG Project Review at an appropriate lead time.  Generally, this lead time should be sufficient to allow the review to be completed before the TSP reaches the decision point at which it must initiate the engineering and procurement in order to meet the required in-service date, but not farther in advance than is necessary.  In general, these lead times will be three to four months for Tier 3 projects and six to seven months for Tier 1 and 2 projects.

(3)	Tier 1, 2 and 3 projects that are included in the Regional Transmission Plan but do not reach this decision point before the development of the next year’s Regional Transmission Plan begins will be removed from the case used to develop the Regional Transmission Plan and will be re-evaluated as a part of the development of this subsequent Regional Transmission Plan.

The current methodology of constructing the Load forecasts is regularly defended as being the “conservative” approach necessary to ensure that any combination of peak loadings can be covered for reliability purposes.  NRG and Calpine assert that sensitivity cases can be added following the primary case using ERCOT’s independent 90/10 Load forecast to adequately vet ERCOT independent base cases, discussed below.

See Platts Megawatt Daily April 18, p. 5 quoting Calvin Opheim, ERCOT Manager of Load Forecasting.

http://interchange.puc.state.tx.us/WebApp/Interchange/Documents/45382_52_881975.PDF