| PGRR Number | 046 | PGRR Title | Addition of Geomagnetically Induced Current (GIC) Model Building Requirements |
| --- | --- | --- | --- |

| Date | April 19, 2016 |
| --- | --- |

| Submitter’s Information |  |
| --- | --- |
| Name | Larisa Loyferman |
| E-mail Address | larisa.loyferman@centerpointenergy.com |
| Company | CenterPoint Energy |
| Phone Number | 713-207-2499 |
| Cell Number |  |
| Market Segment | Investor Owned Utility (IOU) |

Comments

CenterPoint Energy is offering additional edits to the revised proposed Planning Guide language in an effort to put in place Geomatically Induced Current (GIC) models review process by the Planning Geomagnetic Disturbance (PDGTF) Task Force.

Revised Cover Page Language

None

Revised Proposed Guide Language

## ACRONYMS AND ABBREVIATIONS

GIC	Geomagnetically-Induced Current

GMD                           Geomagnetic Disturbance

- 6.11	Process for Developing Geomagnetically-Induced Current (GIC) System Models

(1)	To adequately simulate Geomagnetic Disturbance (GMD) events, it is necessary to establish and maintain GIC system models and conduct geomagnetic disturbance vulnerability assessments to determine whether the ERCOT System can withstand performance requirements of the benchmark geomagnetic disturbance event described in NERC Reliability Standards.  These models, known as GIC base cases, shall contain appropriate system data, and shall represent projected system conditions that provide a starting point for the required year(s).

(a)	Transmission Service Providers (TSPs) and ERCOT shall develop the GIC base cases.  These base cases are derived from the steady-state base cases developed by Steady-State Working Group (SSWG) for the near-term transmission planning horizon to ensure consistency between the System topology in the SSWG base cases and GIC system models.

(b)	Each TSP, or its Designated Agent, shall provide its respective transmission network GIC model data in accordance with the Planning Geomagnetic Disturbance Task Force (PGDTF) Procedure Manual.

(c)	Each Resource Entity, or its Designated Agent, shall provide its respective Resource Entity-owned generating units, plants, transmission lines, shunt devices and Generation Step Ups (GSUs) connected to the ERCOT System in accordance with the PGDTF Procedure Manual and the Resource Registration Glossary.

(d)	ERCOT shall aggregate the GIC system model data supplied by each TSP and Resource Entity and shall compile and maintain the GIC system models. Upon completion of compiling the data for the GIC base cases, ERCOT shall post these cases on the ERCOT Market Information System (MIS) website for PGDTF review and comments as described in the PGDTF Procedure Manual.

(e)	Other requirements for the GIC system model maintenance and use for conducting assessments can be found in the PGDTF Procedure Manual.

(f)	GIC Data is considered Protected Information pursuant to Protocol Section 1.3, Confidentiality.