| PGRR Number | 046 | PGRR Title | Addition of Geomagnetically Induced Current (GIC) Model Building Requirements |
| --- | --- | --- | --- |
| Date of Decision |  | July 7, 2016 |  |
| Action |  | Recommended Approval |  |
| Timeline |  | Normal |  |
| Proposed Effective Date |  | Upon system implementation of Registration Glossary Revision Request (RRGRR) 009, Adding Voltage Limit Sets, Relay Loadability, MLSE, and GMD Data |  |
| Priority and Rank Assigned |  | Not applicable |  |
| Planning Guide Sections Requiring Revision |  | 2.2, Acronyms and Abbreviations 6.11, Process for Developing Geomagnetically-Induced Current (GIC) System Models (new) |  |
| Related Documents Requiring Revision/Revision Requests |  | RRGRR009 GIC System Model Procedure Manual |  |
| Revision Description |  | This Planning Guide Revision Request (PGRR) aligns the Planning Guides with North American Electric Reliability Corporation (NERC) Reliability Standard TPL-007-1, Transmission System Planned Performance for Geomagnetic Disturbance Events, by specifying a process for developing Geomagnetically-Induced Current (GIC) system models. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | To satisfy NERC Reliability Standard TPL-007-1 regulatory requirements. |  |
| PLWG Decision |  | On 3/23/16, the Planning Working Group (PLWG) was in consensus to table PGRR046 for one month. On 4/20/16, PLWG was in consensus to table PGRR046 for one month. On 5/18/16, PLWG was in consensus to recommend approval of PGRR046 as amended by the 4/19/16 Wind Coalition comments as revised by PLWG. On 6/29/16, PLWG was in consensus to endorse and forward to ROS the 5/18/16 PLWG Report as revised by PLWG and the Revised Impact Analysis for PGRR046. |  |
| Summary of PLWG Discussion |  | On 3/23/16, participants discussed associated RRGRR009 requirements and decided tabling for one month would allow time to review comments from Market Participants at the April PLWG meeting. On 4/20/16, participants discussed the 4/14/16 Oncor, 4/19/16 CenterPoint, and 4/19/16 Wind Coalition comments and determined more time was needed to address concerns in the proposed language changes. On 5/18/16, participants reviewed the 4/19/16 Wind Coalition comments; discussed approval timelines; and recommended that PGRR046 be considered coincident to RRGRR009 and the GIC System Model Procedure Manual. On 6/29/16, participants determined additional edits were needed to clarify that GIC base cases would be posted to the Market Information System (MIS) Certified Transmission Service Provider Information page in alignment with language in the associated GIC System Model Procedure Manual. |  |
| ROS Decision |  | On 7/7/16, ROS voted to recommend approval of PGRR046 as recommended by PLWG in the 6/29/16 PLWG Report. There was one opposing vote from the Municipal Market Segment. All Market Segments were present for the vote. |  |
| Summary of ROS Discussion |  | On 7/7/16, there was no discussion. |  |

| Sponsor |  |
| --- | --- |
| Name | James Teixeira |
| E-mail Address | jteixeira@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6582 |
| Cell Number | 512-656-6734 |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Lindsay Butterfield |
| E-Mail Address | Lindsay.Butterfield@ercot.com |
| Phone Number | 512-658-6521 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| Oncor 041416 | Proposed language changes to clarify GIC system models and Geomagnetic Disturbance (GMD). |
| CNP 041916 | Proposed language changes where GIC base cases shall be posted on the MIS for PGDTF review. |
| Wind Coalition 041916 | Proposed language changes that would limit the collection of data for geomagnetic disturbance modeling only for Transmission Elements and removes the reference to the PGDTF Procedure Manual. |

Market Rules Notes

None

Proposed Guide Language Revision

## ACRONYMS AND ABBREVIATIONS

GIC	Geomagnetically-Induced Current

GMD                           Geomagnetic Disturbance

- 6.11	Process for Developing Geomagnetically-Induced Current (GIC) System Models

(1)	To adequately simulate Geomagnetic Disturbance (GMD) events, it is necessary to establish and maintain GIC system models and conduct geomagnetic disturbance vulnerability assessments to determine whether the ERCOT System can withstand performance requirements of the benchmark geomagnetic disturbance event described in NERC Reliability Standards.  These models, known as GIC base cases, shall contain appropriate system data, and shall represent projected system conditions that provide a starting point for the required year(s).

(a)	Transmission Service Providers (TSPs) and ERCOT shall develop the GIC base cases.  These base cases are derived from the steady-state base cases developed by Steady-State Working Group (SSWG) for the near-term transmission planning horizon to ensure consistency between the System topology in the SSWG base cases and GIC system models.

(b)	Each TSP, or its Designated Agent, shall provide its respective transmission network GIC model data in accordance with the GIC System Model Procedure Manual.

(c)	Each Resource Entity, or its Designated Agent, shall provide its respective Resource Entity-owned generating units, plants, transmission lines, shunt devices and Generation Step Ups (GSUs) connected to the ERCOT System in accordance with the GIC System Model Procedure Manual and the Resource Registration Glossary.

(d)	ERCOT shall aggregate the GIC system model data supplied by each TSP and Resource Entity and shall compile and maintain the GIC system models. Upon completion of compiling the data for the GIC base cases, ERCOT shall post these cases on the ERCOT Market Information System (MIS) Certified Transmission Service Provider Information page for review and comments as described in the GIC System Model Procedure Manual.

(e)	Guidelines and formats for the GIC system model data and model maintenance can be found in the GIC System Model Procedure Manual.

(f)	GIC Data is considered Protected Information pursuant to Protocol Section 1.3, Confidentiality.