| LPGRR Number | 057 | LPGRR Title | Alignment with NPRR781, Updates to Nodal Protocol Section 18, Load Profiling, to Align with Current Market Processes |
| --- | --- | --- | --- |
| Date of Decision |  | August 31, 2016 |  |
| Action |  | Recommended Approval |  |
| Timeline |  | Normal |  |
| Proposed Effective Date |  | November 1, 2016 |  |
| Priority and Rank Assigned |  | Not applicable |  |
| Load Profiling Guide Sections Requiring Revision |  | 1, Introduction 1.1, Purpose of the Load Profiling Guide (delete) 1.2, Document Purpose (delete) 4, The Profiling Working Group 4.2, Profiling Working Group Responsibilities 4.3, Profiling Working Group Reporting Structure 4.4, Profiling Working Group Membership (delete) 4.5, Profiling Working Group Contact Information 5, Guidelines for Load Profile Development (delete) 5.1, Background (delete) 5.2, Guidelines (delete) 6, Load Profiling Methodology 6.1, Introduction 6.1, Current Methodologies (new) 6.2, Review of Load Profiling Methodology 6.3, Considerations for Load Profiling Methodology Evaluation (delete) 6.3.1, Load Profile Model Performance (delete) 6.3.2, Methodology Performance (delete) 6.3.3, Alternative Methodology Impacts to Load Profiling Issues (delete) 6.3.4, Practical Implementation of Load Profiling Methodology (delete) 6.4, Possible Results of the Evaluation of Methodologies (delete) 6.4.1, No Changes to Load Profiling Methodologies (delete) 6.4.2, Modify Existing Load Profiling Methodology (delete) 6.4.3, Implement Alternative Load Profiling Methodology (delete) 7, Request for Changes to Load Profiling Methodology 7.1, Current Methodologies (delete) 7.2, Request for Load Profiling Methodology Changes (delete) 7.3, Timeline for Processing a Load Profiling Methodology Change Request (delete) 7.4, Information Required with Request for Change (delete) 7.5, Evaluation of the Request (delete) 7.6, Approval of the Request (delete) 7.7, Costs for Load Profiling Methodology Changes (delete) 8, Load Profile Models 8.1, Routine and Non-Routine Load Profile Model Evaluations 8.1.1, Routine Evaluation of Load Profile Model Performance (delete) 8.1.2, Non-Routine Evaluation of Model Performance (delete) 8.1.2 Evaluation of Weather Zones (new) 8.2, Evaluation of Load Profile Models Using Current Load Research Data 8.2.1, Sources of Load Research Data 8.2.2, Procedures 8.2.3, Using Comparable Weather Zone Data (delete) 8.2.4, Factors Considered in Comparisons (delete) 8.2.4.1, Load-Weighted Average Price (delete) 8.2.4.2, On-Peak/Off-Peak Ratio (delete) 8.2.4.3, Load Factor (delete) 8.2.4.4, Summary Statistics on Differences Between Series (delete) 8.2.4.5, Deadweight Loss (delete) 8.3, Evaluating Load Profile Models without Current Load Research Data 8.3.1, Applications 8.3.2, Load Profile Model Comparisons 8.3.2.1, Comparisons for Adjusted Static Models 8.3.2.2, Examination of Monthly Patterns (delete) 8.3.2.3, Comparisons for Engineering Estimates 8.4, Routine Load profile Model Evaluations (delete) 8.4.1, Routine Evaluation of Weather Zones (delete) 8.5, Non-Routine Load Profile Model Evaluations (delete) 8.6, Assessing the Type of Load Profile Model Change Needed (delete) 8.6.1, Possible Changes (delete) 8.6.2, Qualitative Criteria (delete) 8.6.2.1, Substantial Bias (delete) 8.6.2.2, No Substantial Bias Indicated by Evaluation (delete) 8.6.2.3, Substantial Bias Indicated by Analysis of Current Load Research Data (delete) 8.6.2.3.1, Modest Scope (delete) 8.6.2.3.2, Extensive Scope (delete) 8.6.2.3.3, Adjustment Factors (delete) 8.6.2.4, Substantial Bias Indicated without Current Load Research Data (delete) 8.6.2.4.1, Similar Bias across Several Load Profile Segments within a Weather Zone (delete) 8.6.2.4.2, Bias Not Resolved by Modifying Weather Zones (delete) 8.7, Criteria for Requiring a Load Profile Model Change 8.8, Procedures for Requesting a Change to Load Profile Models (delete) 8.8.1, Request for Load Profile Model Changes (delete) 8.8.2, General Information Required with a Request (delete) 8.8.3, Requesting Load Profile Model Adjustment Factors (delete) 8.8.4, Requesting Change to Engineering Estimates (delete) 8.8.5, Requesting Re-Estimation of Models (delete) 8.9, Approval Process for Load Profile Model Changes 8.9.1, Timeline Prior to Implementing a Load Profile Change (delete) 8.9.2, Adjusted Static Models (delete) 8.9.2.1, Development of Adjustment Factors (delete) 8.9.2.2, Model Re-Estimation (delete) 8.9.3, Engineering Estimates (delete) 9, Load Profile IDs 9.1.1, Profile Decision Tree Revision and Approval Process 9.1.4, Assignment of BUSOGFLT Profile Type 9.2.1, Load Profile ID Changes Initiated By Transmission and/or Distribution Service Providers 9.2.1.1, Load Profile ID Change Related to a Transmission and/or Distribution Service Provider Tariff Change 9.2.1.2, Recognized Error in Current Assignment 9.2.1.3, Load Profile ID Changes Resulting from Meter Type Changes 9.2.1.3.1, Non-Interval Data Recorder to Interval Data Recorder and Interval Data Recorder to Non-Interval Data Recorder 9.2.1.3.2, Non-Time Of Use to Time Of Use (delete) 9.2.1.3.3, Time Of Use to Non-Time Of Use (delete) 10, kVA to KW Conversion 11, Validation of Load Profile ID 11.1, Initial Assignment of Load Profile IDs for Opt-In Entities 11.1.1, Validation of Initial Opt-In Entity Assignments 11.2, Annual Validation of Load Profile Type 11.2.1, Annual Validation of Load Profile Type Assignment for RES and BUS Load Factor Electric Service Identifiers 11.3, Additional Validations 12, Request for Load Profile Segment Changes, Additions, or Removals 12.1, Types of Requests 12.1.1, Creation of a New Load Profile Segment 12.1.2, Redefinition of an Existing Load Profile Segment 12.1.3, Removal of Existing Load Profiles Segments 12.2, Request for Load Profile Segment Changes 12.4, Process Timing for Requesting Changes (delete) 12.5, Groups of Electric Service Identifiers Eligible to Become Load Profile Segments 12.5.1, Universal Load Profile Segment Applicability 12.5.2, List-Based Load Profile Segments (delete) 12.6, Information Required with Request for Change 12.6.1, Unambiguous Group Identification 12.6.1.1, Identification Based on Data Currently in ERCOT’s Systems 12.6.1.2, Identification Based on Other Means 12.6.1.3, List Based Load Profile Segments (delete) 12.6.2, Difference from Current Load Profile Segments 12.6.2.1, Supporting Data Requires 12.6.2.1.1, Load Research Data 12.6.2.1.2, Other Kinds of Supporting Data (delete) 12.6.2.2, Basis for Assessment of Differences Based on Load Research Data 12.6.2.3, Accounting for Weather Zones Effects in Load Profile Comparisons 12.6.2.4, Separate Comparisons for Each Weather Zone 12.6.2.5, Comparison for the Proposed Segment as a Whole 12.6.2.6, Summary Statistics on Differences of Series (delete) 12.6.2.7, Load Weighted Average Price (delete) 12.6.2.8, On Peak/Off Peak Ratio (delete) 12.6.2.9, Load Factor (delete) 12.6.2.10, Deadweight Loss (delete) 12.6.3, Size 12.6.4, Homogeneity 12.6.4.1, Load Research Demonstrating Homogeneity 12.6.4.2, Other supporting Evidence of Homogeneity (delete) 12.6.5, Quality Assurance Methodology for Electric Service Identifier Identification (delete) 12.7, Costs for Profile Segment Changes 12.8, Evaluation of the Request 12.9, Resolution of the Request (delete) 12.9.1, ERCOT Staff Initial Recommendation (delete) 12.9.2, Submitter and ERCOT Revisions (delete) 12.9.3, Presentation to Profiling Working Group (delete) 12.9.4, Profiling Working Group Disposition of Request (delete) 13, Changes to Weather Zone Definitions 13.1.1, Timeline for Processing a Request (delete) 13.1.2, Uniformity 13.2.1, Types of Weather Zone Boundary Changes 13.2.2.2, Weather Stations 13.2.3, Supporting Data (delete) 13.2.3.1, Calculated Load Profiles (delete) 13.2.3.2, Additional Supporting Data for Shifts across Weather Modeling Regions (delete) 13.2.3.3, Basis for Assessing Differences (delete) 13.3. Changes to Weather Modeling Regions (delete) 13.3.1, Supporting Data Required (delete) 13.4, Changing Weather Stations (delete) 13.4.1, Requests for Changes (delete) 13.5.1, Periodic Assessment (delete) 13.5.2.1, Changes in National Oceanic and Atmospheric Administration Weather Station 15, Load Research Samples 15.1, Transmission and/or Distribution Service Provider Samples (delete) 15.1.1, Maintenance of Existing Samples (delete) 15.1.2, Notification to ERCOT (delete) 15.1.2.1, Types of Changes Given Advance Notice (delete) 15.1.2.2, General Reporting Procedures (delete) 15.1.2.3, Required Information (delete) 15.1.2.4, Additional Requested Information (delete) 15.1.3, Availability of Data (delete) 15.1.4, Creation of New Transmission and/or Distribution Service Provider Samples (delete) 15.2, ERCOT Samples (delete) 15.2.1, Maintenance (delete) 15.2.2, Availability of Data (delete) 15.2.3, Criteria of Standards (delete) 15.2.4, Creation of New Samples (delete) 15.2.5, Guidelines for Installing and Refreshing Load Research Samples (delete) 16, Supplemental Load Profiling (delete) 16.1, Load Profiling for Time Of Use Schedules (delete) 16.1.1, Establishing New Time Of Use Schedules (delete) 16.1.2, Chunking Load Profiling Methodology Description (delete) 16.1.3, Evaluation of the Chunking Load Profiles (delete) 16.1.3.1, Assessment of Chunking as a General Method Based on Load Profiles from Other Areas (delete) 16.1.3.2, Assessment of the Chunking Method Using ERCOT Load Research Data (delete) 16.1.3.3, Assessment of Chunking Method Based on Characteristics of the Time Of Use and Non Time Of Use Populations (delete) 16.1.4, Revisions to Time Of Use Load Profile Methods if Changes Are Needed (delete) 16.2, Other Supplemental Load Profiles (delete) 17, Load Profile Metering (delete) 17.1, Introduction (delete) 17.2, Interval Data Recorder Requirement (delete) 17.3, Demand Meter Changes (delete) 17.4, Load Research Samples (delete) 17.5, Metering for Supplemental Load Profiling (delete) 18, Access to Load Profiling Materials 19.1, Definitions 19.2, Acronyms Appendix C (delete) Appendix D, Profile Decision Tree |  |
| Related Documents Requiring Revision/Revision Requests |  | Nodal Protocol Revision Request (NPRR) 781, Updates to Nodal Protocol Section 18, Load Profiling, to Align with Current Market Processes |  |
| Revision Description |  | With the proliferation of Advanced Metering Systems (AMS) in the ERCOT market, this Load Profiling Guide Revision Request (LPGRR) provides a comprehensive review of the Load Profiling Guide (LPG) and makes the following changes: - Updates language to clarify purpose and definitions; - Updates current processes and methodologies; and - Removes outdated processes and practices. Much of the removed language was originally created when the retail market deregulated and outlined the theories and approaches utilized for the initial development of Load Profile assignments for non-interval Electronic Service Identifiers (ESI IDs). As the market has matured, particularly with the deployment of AMS, such detailed explanations of creating Load Profiling Methodologies and processes are no longer utilized. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | Load Profiles still serve a vital role in the ERCOT market as a means for estimating and allocating 15-minute Load for ESI IDs during the Settlement process when interval consumption data is not available. The LPG outlines the processes and details the responsibilities as described in the ERCOT Protocols. Revision Overview - Updating and streamlining: - Definitions & Acronyms - Responsibilities of the Profiling Working Group (PWG) - Methodologies used to establish/review Load Profile changes - Changes to Load Profile Models - Changes to accessing Load Profiling materials - Evaluation and changes to Weather Zones - Creating Load Profile IDs - Changes to Annual Validation - Requests for Load Profile Segment changes - kVA to kW conversion - Standardizing Distributed Generation (DG) reporting within Appendix D, Load Profile Decision Tree (new report template/format & codifying instruction) - Language regarding the following areas has been removed due to duplicity with the current Association of Edison Illuminating Companies (AEIC) Load Research Manual: - Guidelines for the development of Load Profiles - Procedures for comparing Load Profile Models to Load research data - Load Research Samples – no longer used for the maintenance of load samples - Supplemental Load Profiling - Appendix C – Measures for Assessing Differences between Load Profiles - Language regarding the following areas has been removed due to being no longer applicable in the current ERCOT market structure: - Time-of-Use (TOU) metering - List-based Load Profile Segment requests - Metering specifically used for Load Profiling purposes |  |
| PWG Decision |  | On 6/29/16, the Profiling Working Group (PWG) was in consensus to recommend approval of LPGRR057 as revised by PWG. On 7/27/16, PWG was in consensus to table LPGRR057. On 8/31/16, PWG was in consensus to endorse and forward to COPS the 7/27/16 PWG Report as revised by PWG and the Impact Analysis for LPGRR057. |  |
| Summary of PWG Discussion |  | On 6/29/16, participants reviewed Appendix D and made revisions in purple to clarify DG language in the spreadsheet. On 7/27/16, participants reviewed Appendix D and determined that more discussion was needed to come to a consensus on the DG reporting language. On 8/31/16, participants discussed Load Profiling conflicts relating to energy storage, and the group was in consensus to remove language in Appendix D relating to energy storage. |  |

| Sponsor |  |
| --- | --- |
| Name | Jim Lee on behalf of the Profiling Working Group (PWG) |
| E-mail Address | jclee@aep.com |
| Company | AEP |
| Phone Number | 512-391-2972 |
| Cell Number | N/A |
| Market Segment | N/A |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Lindsay Butterfield |
| E-Mail Address | Lindsay.Butterfield@ercot.com |
| Phone Number | 512-248-6521 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None |  |

Market Rules Notes

Please note the color code below regarding edits to Appendix D, Profile Decision Tree:

- PWG= red

- PWG 062916= purple

- PWG 083116= orange

Proposed Guide Language Revision

# 1       INTRODUCTION

(1)	Load Profiling within the ERCOT market is the practice of estimating 15-minute interval Load for Customers where interval consumption is not available.

The purpose of the Electric Reliability Council of Texas (ERCOT) Load Profiling Guide (LPG) is to explicate the language and intent in the Protocols that affect Load Profiling.  It is not a substitute for the ERCOT Protocols or the Public Utility Commission of Texas (PUCT) Substantive Rules.  In the event of a conflict of Protocols or PUCT Substantive 	Rules, the Protocols and PUCT Substantive Rules take precedence over the LPG.

4       THE PROFILING WORKING GROUP

(1)	The Profiling Working Group (PWG) is a standing working group that provides technical support to the Commercial Operations Subcommittee (COPS) on Load Profiling issues.  The PWG establishes high-level principles to be utilized in the development of Load Profiles. These principles are specified in Protocol Section 18.2.1, Guidelines for Development of Load Profiles.  The PWG membership is open to all Market Participants and any other interested parties.

- 4.2	Profiling Working Group Responsibilities

(1)	The PWG has several responsibilities and duties, which include the following:

(a)	Maintains and upholds Protocol Section 18, Load Profiling;

(b)	Reviews all requests for changes to Load Profiles, Load Profiling Methodologies, and implementation of the Load Profiling process;

(c)	Reviews and makes recommendations to the Commercial Operations Subcommittee (COPS) regarding the Load Profiling Guide (LPG) changes, including Appendix D, Profile Decision Tree, and Load Profiling Methodologies;

(d)	Participates in defining Weather Zones and Load Profile Types;

(e)	Evaluates the validation and assignment processes for Load Profile IDs;

(

(f)	Coordinates with ERCOT in developing Load Profiles for particular Customer segments that may require special Load Profiling techniques (e.g., supplemental Load Profiles);

(g)	Develops and maintains the LPG; and

(h)	Provides a forum for Market Participants to be involved with ERCOT Load Profiling.

- 4.3	Profiling Working Group Reporting Structure

(1)	The Profiling Working Group (PWG) reports to the Commercial Operations Subcommittee (COPS), which is a standing subcommittee of Technical Advisory Committee (TAC).  The PWG  leadership is elected annually by the PWG on a calendar year basis.  PWG leadership establishes the PWG meeting dates and represents the PWG at COPS and other ERCOT forums, as necessary.  The PWG shall meet as needed to review Load Profiling processes and issues.

- 4.4	Profiling Working Group Contact Information

(1)	To begin or discontinue receiving electronic mail related to the Profiling Working Group (PWG), subscribe or unsubscribe  through the ERCOT ListServ.

5       [RESERVED]

- 6       LOAD PROFILING METHODOLOGY

evaluation of th Load Profiling Mpecified id

2	(1)	Any changes to the Load Profiling Methodology shall be submitted as a Load Profiling Guide Revision Request (LPGRR) as described in Section 2.4, Load Profiling Guide Revision Procedure.

()	There shall be no retroactive application of any approved modifications to Load Profiling Methodology.

.1urrent Methodologies

(1)	The following methodologies are used to establish Load Profiles:

| Type of Load | Load Profiling Methodology |
| --- | --- |
| Non-Price-Responsive |  |
| Non-interval metered | Adjusted Static Models |
| Non-interval metered with Distributed Generation (DG) | Adjusted Static Models and Engineering Estimates |
| Non-metered | Engineering Estimates |
| Interval Data Recorder (IDR) (Estimation) | Proxy day |
| Price-Responsive |  |
| Other price-responsive | To be determined |

6.3.1		Load Profile Model Performance

6.3.4		Practical Implementation of Load Profiling Methodology

6.4.1		No Changes to Load Profiling Methodologies

6.4.3		Implement Alternative Load Profiling Methodology

- 7       [RESERVED]

- The following methodologies are used to establish Load Profiles:

8       	LOAD PROFILE MODELS

(1)	Protocol Section 18.2.5, Adjustments and Changes to Load Profile Development, defines  evaluation of Load Profiling Methodology  including changes to the Load Profiling Methodology, adjustments to existing Load Profiles, and development of new Load Profiles.  This Section addresses changes to models within approved Load Profiling Methodologies.

(2)	The Microsoft Excel© representation of the ERCOT Load Profile Models can be found in Appendix E, Load Profile Model Spreadsheets.

(3)	There shall be no retroactive application of any approved modifications to Load Profile Models.

(4)	This Section discusses changes to Load Profile Models not addressed in the following the Load Profiling Guide (LPG) sections:

(a)	Section 6, Load Profiling Methodology;

(b)	Section 12, Request for Load Profile Segment Changes, Additions, or Removals; and

(c)	Section 13, Changes to Weather Zone Definitions.

8.1	Load Profile Model Evaluations

(1) 	Upon request from the appropriate Technical Advisory Committee (TAC) subcommittee, ERCOT shall perform evaluations of Load Profile Model performance for all Load Profile Models and Load Profile Types using industry standard practices as described in the Association of Edison Illuminating Companies (AEIC) Load Research Manual.  Any changes resulting from these evaluations shall be recommended to the Profiling Working Group (PWG).

8.1.2		Evaluation of Weather Zones

(1) 	Assessment of Weather Zone definition shall focus on the adequacy of the current set of weather stations and weighting.  ERCOT uses National Oceanic and Atmospheric Administration (NOAA) first or second order weather stations as the source for weather data for each Weather Zone. Weather Zone evaluation will be conducted using industry standard practices as described in the Association of Edison Illuminating Companies (AEIC) Load Research Manual.

8.2	Evaluating Load Profile Models Using Load Research Data

8.2.1	Sources of Load Research Data

(1)	Load research data may be  collected by ERCOT developed Lnced Metering System (AMS) interval data and from any available Transmission and/or Distribution Service Provider’s (TDSP’s) Load research samples.

(2)	In certain circumstances, Load research data from other sources may also be considered by ERCOT as representative of a particular subgroup.  For such data to be used, the party submitting the data for use in an evaluation shall provide information on the source of the data.  Submission requirements are the same as those described in Section 12.6, Information Required with Request for Change.

8.2.2	Procedures

(1)	The overall procedure for comparing existing Load Profile Models against current Load research data shall use industry standard practices as described in the Association of Edison Illuminating Companies (AEIC) Load Research Manual.

Bf

8.3	Evaluating Load Profile Models without Current Load Research Data

8.3.1		Applications

(1)	In some situations, current Load research data may not be available as a basis for assessing the performance of Load Profile Models.  Situations where techniques are required that do not depend on Load research data include:

(a)	Assessing Load Profile Model performance for geographic areas where Load research data are no longer collected;

(b)	Assessing Load Profile Model performance for geographic areas where Load research data have never been collected, or have not contributed to current Load Profile Models; and

(c)	Assessing Engineering Estimates.

(2)	These techniques may also be used as another way of assessing Load Profile Model performance even for geographic areas where current Load research data are available.

8.3.2		Load Profile Model Comparisons

8.3.2.1	Comparisons for Adjusted Static Models

(1)	For premises with an Advanced Meter installed, Adjusted Static Models may be assessed based on techniques described in the Association of Edison Illuminating Companies (AEIC) Load Research Manual.populati

8.3.2.3	Comparisons for Engineering Estimates

(1)	Engineering Estimates are used in the ERCOT market for Non-Metered Loads, such as lighting.  Engineering Estimates are typically based on an assumed fixed operating schedule together with the assumption that the Load is approximately the same whenever the equipment is operating.  If more current information is available for the ESI IDs in a Load Profile Segment using an engineering Load Profile, this information may be compared with the assumptions of the Engineering Estimate.

(2)	Monthly consumption data may also be compared with the Load Profile monthly patterns using the methods described above for Adjusted Static Models.

- 8.4	Requesting Load Profile Model Evaluations or Changes

(1)	This Section describes the procedures for requesting changes to Load Profile Models.  Procedures for requesting changes to Load Profile Segments are described in Section 12, Request for Load Profile Segment Changes, Additions, or Removals.  Procedures for requesting changes to Weather Zones are described in Section 13, Changes to Weather Zone Definitions.  Upon request from the appropriate Technical Advisory Committee (TAC) subcommittee, ERCOT will evaluate Load Profile Models using industry standard practices as described in the Association of Edison Illuminating Companies (AEIC) Load Research Manual.

(2)	The following Entities may submit requests for Load Profile Model changes:

(a)	Any Market Participant;

(b)	Any Entity that is an ERCOT Member;

(c)	Public Utility Commission of Texas (PUCT) Staff;

(d)	ERCOT Staff; and

(e)	Any other Entity who resides (or represent residents) in Texas or operates in the Texas electricity market.

(3)	Requests for Load Profile Model changes shall be submitted to the Profiling Working Group (PWG) and are subject to approval as outlined in Section 8.5, Approval Process for Load Profile Model Changes.

(4)	Requests for Load Profile Model changes shall include the following:

(a)	Identifying the party making the request, with contact information;

(b)	Identifying the Load Profile Segment(s) and Weather Zone(s) affected; and

(c)	Describe why the evaluation is needed.

8.5	Approval Process for Load Profile Model Changes

(1)	For any proposed Load Profile Model changes, the Profiling Working Group (PWG) shall propose changes to the appropriate Technical Advisory Committee (TAC) subcommittee.

(2)	Load Profile Model changes shall be accompanied by an implementation plan to mitigate the impact of transitioning between old and new Load Profile Models.  The implementation plan shall be approved by TAC and the appropriate TAC subcommittee.

# 9      LOAD PROFILE IDS

- 9.1.1		Profile Decision Tree Revision and Approval Process

(1)	All revisions to Appendix D, Profile Decision Tree, shall be submitted through the Load Profiling Guide Revision Request (LPGRR) process described in Section 2, Load Profiling Guide Revision Process.  ERCOT may use an administrative LPGRR to revise the contents of the following Profile Decision Tree tabs:

(a)	FAQ – frequently asked questions related to the assignment of Load Profile IDs;

(b)	Use of Components – information about how each component of the Load Profile ID is used by ERCOT in the Settlement process;

(c)	ZipToZone – a table that maps Zone Improvement Plan (ZIP) Codes to Weather Zones;

(d)	Valid Profile IDs – a list of all Load Profile IDs that can be assigned to ESI IDs that are within the ERCOT region;

(e)	Non-ERCOT Profile IDs – a list of Load Profile IDs that can be assigned to ESI IDs that are within Texas, but outside of the ERCOT Region; and

(f)	NOIEs – directions for Non-Opt-In Entities (NOIEs) to use in determining Load Profile ID assignments.

- 9.1.4		Assignment of BUSOGFLT Profile Type

(1)	Competitive Retailers (CRs) seeking to have the Oil & Gas Flat (OGFLT) Load Profile Segment assigned to one of their Business (BUS) ESI IDs shall follow the instructions on the Oil & Gas tab of Appendix D, Profile Decision Tree.

(2)	ERCOT shall review all assignments of the BUSOGFLT Profile Type on an annual basis, per Section 11.3.1, Validation of BUSOGFLT Profile Type.

- 9.2.1	Load Profile ID Changes Initiated By Transmission and/or Distribution Service Providers

- 9.2.1.2	Recognized Error in Current Assignment

(1)	Should the TDSP become aware of an error in the assignment of a Load Profile ID, the TDSP shall notify the CR of the error as soon as practical via the appropriate Texas SET transaction.  If a dispute is created, refer to Section 14.2, General Load Profile ID Dispute Resolution Guidelines.

- 9.2.1.3	Load Profile ID Changes Resulting from Meter Type Changes

(1)	The following subsections outline the procedures for implementing Load Profile ID changes when a meter type change occurs.

The TDSP shall  submit the Load Profile ID change to ERCOT using the appropriate TX SET transaction with the effective date of the meter change once the meter installation is complete.

10		kVA TO kW CONVERSION

(1)	The majority of Transmission and/or Distribution Service Providers (TDSPs) meter kW Demand. However, some TDSPs only meter kVA Demand. To assign a Load Profile ID to an Electric Service Identifier (ESI ID), the kVA shall be converted to a kW value for the Load factor calculation when necessary.  This Section 10, kVA to kW Conversion, addresses how kVA shall be converted to kW for Load Profile ID assignments.

(2)	This section applies to any Market Participants such as:

(a)	A TDSP that currently meters kVA;

(b)	A TDSP that changes from kW to kVA metering; or

(c)	A Non-Opt-In Entity (NOIE) that currently meters kVA and decides to opt-in.

(3)	Appendix D, Profile Decision Tree, defines how kVA is to be converted to kW (kW is equivalent to the product of kVA and Power Factor).  The Power Factor(s) for this conversion shall be determined by a case study performed by the TDSP.  Without a case study, a default Power Factor of 1.0 shall be imposed.  A default Power Factor of 1.0 means kVA shall be considered equivalent to kW.

11     VALIDATION OF LOAD PROFILE ID

(1)	A Load Profile ID is comprised of five components:

(a)	Load Profile Type;

(b)	Weather Zone;

(c)	Meter Data Type;

(d)	Weather sensitivity; and

(e)	Time Of Use Schedule (TOUS).

(2)	ERCOT shall validate the first two components, the Load Profile Type and Weather Zone, at the following times:

(a)	As part of the initial assignment of Load Profile IDs for Opt-In Entities;

(b)	When Load Profile Segment definitions change; and

(c)	At least one time per year during the Annual Validation process.

(3)	At the start of the validation process, the Transmission and/or Distribution Service Provider (TDSP) shall be asked by ERCOT to confirm contact information for the primary and backup TDSP contact persons.  ERCOT shall provide the TDSP contact information on an ERCOT contact person.

(4)	Electronic mail is the primary means of communication among ERCOT, the Profiling Working Group (PWG), and Market Participants regarding the validation process detailed in this section.  Other methods of communication shall be accommodated if all affected parties mutually agree to alternative methods.

- 11.1	Initial Assignment of Load Profile IDs for Opt-In Entities

(1)	When a Non-Opt-In Entity (NOIE) chooses to participate in the retail market, it thereby becomes an Opt-In Entity and the initial assignment of Load Profile IDs to Electric Service Identifiers (ESI IDs) shall be subject to all requirements detailed in this Section 11.1, Initial Assignment of Load Profile IDs for Opt-In Entities.

(2)	Once the Opt-In Entity has given notice to ERCOT of its intent to participate in the retail market, the Opt-In Entity  shall submit all assigned ESI IDs, their Load Profile Group, and their historical usage to ERCOT.  Additionally, the Profile Type shall be submitted for all ESI IDs assigned to the non-metered group.

(a)	The Opt-In Entity shall provide to ERCOT, in a comma-delimited format at least 120 days prior to the effective start date, the ESI ID’s monthly usage and Demand values for a period of time established in coordination with ERCOT.

(b)	Load Profile ID assignments shall be based on the criteria defined in Appendix D, 	Profile Decision Tree.

(i)	ERCOT will calculate the Load Profile Segment using the historical usage 		provided by the Opt-In Entity.

(ii)	ERCOT shall provide the Opt-In Entity a file containing all of the ESI IDs 		and their Load Profile Type.

(iii)	The Opt-In Entity shall use the provided information to assign the Load Profile ID via the appropriate Texas Standard Electronic Transactions (TX SET)and shall coordinate with ERCOT to schedule submittal of the  ransaction(s).

- 11.1.1	Validation of Initial Opt-In Entity Assignments

(1)	The Opt-In Entity shall notify ERCOT via email when the transactions to create the Opt-In ESI IDs have been submitted and accepted in the ERCOT System.  After receiving notification, ERCOT shall perform three additional reviews to ensure all ESI IDs are set up in accordance with the appropriate Profile Decision Tree:

(a)	ERCOT will compare each ESI ID and Load Profile ID assignment in the ERCOT database with the previously approved initial Load Profile Type;

(b)	ERCOT will validate that Weather Zone assignment is consistent with the appropriate Profile Decision Tree; and

(c)	ERCOT shall validate Load Profile Group assignment for Residential and Business ESI IDs by using the Premise Type field in ERCOT’s registration database.  The Residential Load Profile Group must match the Residential Premise Type in the registration database.  The Business Load Profile Group must match either the Small Non-Residential or Large Non-Residential Premise Type in the registration database.

(2)	Any discrepancies will be reported to the Opt-In Entity via email.  The Opt-In Entity shall submit corrections to ERCOT via appropriate TX SET transaction(s) or provide details as to why the data elements have changed.

(3)	The initial Load Profile ID assignment validation is complete after all discrepancies are resolved.

- 11.2	Annual Validation of Load Profile Type

(1)	Transmission and/or Distribution Service Providers (TDSPs) and ERCOT shall work together to complete the Annual Validation process. When a date is listed in this Section 11.2, Annual Valiation of Load Profile Type, and a year is not specified, the date shall apply to the current year in which the Annual Validation is performed.

- 11.2.1	Annual Validation of Load Profile Type Assignment for RES and BUS Load Factor Electric Service Identifiers

(1)	For the purposes of Annual Validation, ERCOT is responsible for determining the Load Profile Type assignment for all Residential and Business Load factor Electric Service Identifiers (ESI IDs).

(2)	The following timeline shall be adhered to, unless otherwise approved by an appropriate Technical Advisory Committee (TAC) subcommittee.  ERCOT shall utilize the historical usage and Demand data in its systems to derive usage time period values for each active and de-energized ESI ID for the time period specified in Appendix D, Profile Decision Tree.

(a)	Residential Load Profile Group Timeline

(i)	ERCOT shall determine the Load Profile Segment for the Load Profile ID for each active and de-energized ESI ID based on the current Profile Decision Tree in Appendix D.  ERCOT shall provide the TDSPs with a list of Residential ESI IDs containing the current Load Profile Type and the recommended Load Profile Type for those ESI IDs where ERCOT recommends a change in Load Profile Type assignment.  An electronic copy of each list shall be delivered to each TDSP no later than June 30.

(ii)	For each ESI ID contained in the lists, the TDSPs shall review the recommended Load Profile Segment assignment and determine whether the recommended change is consistent with the TDSP tariffs, the applicable Retail Electric Provider (REP) billing requirements, and whether the ESI ID is active or de-energized.  The TDSP shall then send finalized lists of ESI IDs back to ERCOT no later than July 10.  The finalized lists shall indicate all revisions determined to be necessary by the TDSP.

(iii)	ERCOT shall send notification to Competitive Retailers (CRs) and the Profiling Working Group (PWG) by July 15 announcing these lists are available to the CR of record.  Upon request, ERCOT shall make available to the current CR of record the list of those ESI IDs that are expected to have a Load Profile ID change as a result of Annual Validation.

(iv)	The TDSPs shall coordinate with ERCOT to submit the necessary Texas Standard Electronic Transaction (TX SET) transactions to update Load Profile ID assignments for the population of the Residential Load Profile Group to be effective on the most current meter read date on or after August 15.

(v)	TDSPs are responsible for verifying that TX SET transactions related to Annual Validation have been successfully accepted into ERCOT’s systems by monitoring the appropriate response transactions.  The TDSPs and ERCOT shall work together to have TX SET transactions successfully completed for the Residential Load Profile Group by September 30.

(vi)	Within the first two Business Days of the TDSP successfully submitting all of its Residential transactions, ERCOT shall compare the finalized lists of recommended changes with the current Load Profile ID in the ERCOT system.  ERCOT and the TDSPs shall work closely and expeditiously to resolve any discrepancies.  The TDSP and ERCOT shall be in contact until at least 99.0% of the finalized list of changes is resolved to their mutual satisfaction.

(vii)	ERCOT and the TDSPs shall provide regular updates on the progress of Annual Validation as needed, or at a minimum during the regularly scheduled PWG meetings.

(b)	Business Load Profile Group Timeline

(i)	ERCOT shall determine the Load Profile Type for the Load Profile ID for each active and de-energized ESI ID based on the current Profile Decision Tree (Appendix D).  ERCOT shall provide the TDSPs with a list of Business Load factor ESI IDs containing the current Load Profile Type and the recommended Load Profile Segment for those ESI IDs where ERCOT recommends a change in Load Profile Segment assignment.  An electronic copy of each list shall be delivered to each TDSP no later than August 15.

(ii)	For each ESI ID in the lists, the TDSPs shall review the recommended Load Profile Segment assignment and determine whether the recommended change is consistent with the TDSP tariffs, the applicable Retail Electric Provider (REP) billing requirements, and whether the ESI ID is active or de-energized.  The TDSP shall then send finalized lists of ESI IDs back to ERCOT no later than August 25.  The finalized lists shall indicate all revisions determined to be necessary by the TDSP.

(iii)	ERCOT shall send Notification to CRs and the PWG by September 1 announcing these lists are available to the CR of record.  Upon request, ERCOT shall make available to the current CR of record the list of those ESI IDs that are expected to have a Load Profile ID change as a result of Annual Validation.

(iv)	The TDSPs shall coordinate with ERCOT to submit the necessary TX SET transactions to update Load Profile ID assignments for the population of Business Load factor group to be effective on the most current meter read date on or after October 1.

(v)	TDSPs are responsible for verifying that TX SET transactions related to Annual Validation have been successfully accepted into ERCOT’s systems by monitoring the appropriate response transactions.  The TDSPs and ERCOT shall work together to have TX SET transactions successfully completed for the Business Load factor group by November 30.

(vi)	Within the first two Business Days of the TDSP successfully submitting all of its Business Load factor transactions, ERCOT shall compare the finalized lists of recommended changes with the current Load Profile Segment in the ERCOT system.  ERCOT and the TDSPs shall work closely and expeditiously to resolve any discrepancies.  The TDSP and ERCOT shall be in contact until at least 99.0% of the finalized list of changes is resolved to their mutual satisfaction.

(vii)	ERCOT and the TDSPs shall provide regular updates on the progress of Annual Validation as needed, or at a minimum during the regularly scheduled PWG meetings.

- 11.3		Additional Validations

(1)	During the Annual Validation process, ERCOT shall perform additional validations to identify potentially incorrect Load Profile ID or Premise Type assignments.  If ERCOT and the Transmission and/or Distribution Service Provider (TDSP) confirm a Load Profile ID or Premise Type change is necessary, the TDSP shall update the Load Profile ID in the ERCOT system using the appropriate Texas Standard Electronic Transaction (TX SET) transaction.

- 12 	REQUEST FOR LOAD PROFILE SEGMENT CHANGES, ADDITIONS, OR REMOVALS

(1)	This Section 12, Request for Load Profile Segment Changes, Additions, or Removals, of the Load Profiling Guide (LPG) addresses changes, additions, and deletions to Load Profile Segments, with the exception of Load Profile Segment modifications addressed in Section 16, Supplemental Load Profiling.

(2)	The steps and tests identified to introduce new Load Profiles or changes to Load Profiles are intended to fulfill the criteria established in Protocol Section 18.2.1, Guidelines for Development of Load Profiles.

(3)	Any change to Load Profile ID assignments resulting from an approved modification to the definitions of Load Profile Segments shall not be retroactively applied.

- 12.1	Types of Requests

(1)	The  types of requests are:

(a)	Creation of a new Load Profile Segment from one or more existing Load Profile Segments;

(b)	Redefinition of existing Load Profile Segments; and

(c)	Removal of existing Load Profile Segments.

- 12.1.1	Creation of a New Load Profile Segment

(1)	When a new Load Profile Segment is created, there may be an impact to the Electric Service Identifiers (ESI IDs) assigned to one or more existing Load Profile Segments.  The new Load Profile Segment will be applied to ESI IDs that meet the criteria for the new Load Profile Segment while being removed from the existing Load Profile Segment.

- 12.1.2	Redefinition of an Existing Load Profile Segment

(1)	Redefinition of existing Load Profile Segment definitions may require that some ESI IDs be moved from one Load Profile Segment to another.

- 12.1.3	Removal of Existing Load Profiles Segments

(1)	A request to remove an existing Load Profile Segment shall include supporting documentation  that the Load Profile Segment proposed for removal does not satisfy the standards for a separate Load Profile.  Specifically, the group represented by the Load Profile may be:

(a)	Too small to justify a separate Load Profile Segment, as described in Section 12.5, Groups of Electric Service Identifiers Eligible to Become Load Profile Segments; and/or

(b)	Sufficiently similar to one or more existing Load Profiles, according to the measures defined in Section 12.5.

(2)	Removal of an existing Load Profile Segment  requires changing assignments  of  existing  ESI IDs currently in the proposed removed Load Profile Segment.

- 12.2	Request for Load Profile Segment Changes

(1)	Any request for a change to Load Profile Segments shall be submitted in accordance with Section 2.4, Load Profiling Guide Revision Procedure.

)

)

)

12.5	Groups of Electric Service Identifiers Eligible to Become Load Profile Segments

(1)	For a group of Electric Service Identifiers (ESI IDs) to be a distinct Load Profile Segment, the group shall satisfy the following requirements:

(a)	The group is based on readily identifiable parameters, which are not subject to frequent change;

(b)	The group is reasonably homogeneous as defined in Section 12.6.4, Homogeneity;

(c)	The group is sufficiently different from other existing Load Profiles as defined in Section 12.6.2, Difference from Current Load Profiles; and

(d)	The group is of sufficient size to justify its own Load Profile Segment as defined in Section 12.6.3, Size.

- 12.5.1	Universal Load Profile Segment Applicability

(1)	 One primary goal is to make all Load Profile Segment definitions universally applicable.  Universally applicable means:

(a)	The Load Profile may be applicable to all CRs;

(b)	The Load Profile Segment shall be applied to any ESI ID in the ERCOT System that meets the eligibility criteria; and

(c)	The Load Profile Segment shall be public.

(2)	There are limited exceptions as described in Section 16, Supplemental Load Profiling.

12.6	Information Required with Request for Change

(1)	All requests shall include the following:

(a)	Unambiguous group identification;

(b)	Difference from current Load Profile Segments;

(c)	Size;

(d)	Homogeneity; and

(e)	Validation methodology.

- 12.6.1	Unambiguous Group Identification

(1)	The definition of the group shall be provided in the request for the new Load Profile Segment.  The request shall unambiguously define specific criteria for an Electronic Service Identifier (ESI ID) to be included in the new Load Profile Segment.  In a request to change an existing Load Profile Segment, the group to be re-assigned shall be identified.

- 12.6.1.1	Identification Based on Data Currently in ERCOT’s Systems

(1)	The most direct way a group may satisfy the requirement of being unambiguously identified occurs when the group may be identified based solely on information currently available in the ERCOT data systems or readily derived from such data.

(2)	Examples of information available in or derived from the ERCOT data systems include, but are not limited to:

(a)	Monthly or annual kWh consumption;

(b)	Metered monthly or annual peak Demand for Demand-metered Customers;

(c)	Monthly or annual Load factor;

(d)	Ratio of seasonal consumption values; and

(e)	Zone Improvement Plan (ZIP) code.

- 12.6.1.2	Identification Based on Other Means

(1)	Load Profile Segments based on other criteria may be requested.  ERCOT, in coordination with the Profiling Working Group (PWG), shall evaluate such requests in terms of the feasibility and reliability of the proposed identification method.  If the method requires data not currently in ERCOT’s systems, the request shall describe how these data shall be made available to ERCOT on an ongoing basis.  If the identification method is judged to be impractical or unreliable, the request may be denied.

M

12.6.2	Difference from Current Load Profile Segments

(1)	A requested new Load Profile Segment shall be shown in the supporting documentation to be different from existing Load Profiles, and 	that the group re-assigned from one Load Profile Segment to another is more similar to the proposed new assignment(s) than to the old one, in ways that improve the accuracy of Settlement.

(2)	If the ESI IDs in the requested Load Profile Segment are different from the Load Profile Segment that they are currently assigned and more similar to another existing Load Profile Segment, then the resolution of the request may be to reassign these ESI IDs to the most similar existing Load Profile Segment.

(3)	Requests to create new Load Profile Segments or to change the definition of existing Load Profile Segments require supporting documentation to provide a basis for assessing differences between the affected group and existing Load Profile Segments.  All differences between Load Profiles that are important for evaluating a change shall be  documented in the request.

- 12.6.2.1	Supporting Data Required

(1)	Types of supporting data that may be submitted and the associated documentation are described in the following subsections.

- 12.6.2.1.1	Load Research Data

(1)	The  best documentation would be a statistically valid Load research sample from the proposed new or re-assigned Load Profile Segment population.

(2)	The Load research sample design, data collection and statistical calculations shall be in compliance with industry accepted standards as defined in the Association of Edison Illuminating Companies (AEIC) Load Research Manual.

(3)	Examples of less  rigorous, but supportive documentation would be other types of Load research data, such as:

(a)	Data from ad-hoc or convenience samples; and

(b)	Data from a similar population.

(4)	When less  rigorous data is submitted, the submitter should also submit evidence to support the applicability of the data to the proposed Load Profile Segment population, including but not limited to:

(a)	Documentation of operating schedules for the proposed group and comparison with typical schedules for Premises in the currently assigned Load Profile;

(b)	End-use saturation data, comparing the proportions of Premises with particular types of electric end uses for the proposed group and currently assigned Load Profiles.  Such data shall be relevant to the proposed population in ERCOT;

(c)	Monthly billing data comparing consumption patterns, particularly related to heating and cooling.   These comparisons shall be made separately by Weather Zone or account for variations by Weather Zone.

- 12.6.2.2	Basis for Assessment of Differences Based on Load Research Data

(1)	In assessing differences between the initial profile segment and the requested profile segment,  ERCOT shall use industry accepted standards as described in the AEIC Load Research Manual.  The requester may submit analysis but is not required to do so.

- 12.6.2.3	Accounting for Weather Zone Effects in Load Profile Comparisons

(1)	Comparisons between Load Profiles shall take into account Weather Zone effects.   These effects may be accounted for in the comparisons in one of two ways:

(a)	The comparison between the proposed Load Profile Segment and the existing Load Profile is made separately for each Weather Zone; and

(b)	A single Load Profile representing the proposed Load Profile Segment as a whole is compared with a single composite Load Profile for the existing Load Profile Segment.

(2)	These methods are not required for Load Profiles that are the same across all Weather Zones.

- 12.6.2.4		Separate Comparisons for Each Weather Zone

(1)	If Load research data for individual sample Customers are provided for the proposed segment, a separate Load Profile may be constructed for each Weather Zone.

(2)	The limitation of separate comparisons by Weather Zone is that some or all of the separate Weather Zone Load Profiles may have large statistical errors due to low sample sizes.  The magnitude of these errors should be considered in assessing the comparisons.

- 12.6.2.5	Comparison for the Proposed Segment as a Whole

(1)	If a single Load Profile is estimated for the proposed Load Profile Segment as a whole across several Weather Zones, this Load Profile may be compared with a composite of existing Load Profiles.  The composite existing Load Profile will be constructed using the standards described in the AEIC Load Research Manual.

- 12.6.3	Size

(1)	Supporting documentation shall show that the population represented by the proposed

segment(s) is of sufficient size to justify a separate segment.  Size shall be provided in terms of both number of Customers and total energy consumption.

(2)	If the proposed Load Profile Segment is identified based on information available in the ERCOT data systems and also available to the requesting party, documentation of the total ESI ID count and annual energy use is sufficient.  ERCOT shall verify this information using the ERCOT data systems.

(3)	If the requesting party has information on only a portion of the population represented by the segment, the request shall include estimates of the ESI ID counts and energy use, and documentation of the basis for the estimates.

- 12.6.4	Homogeneity

(1)	For a new Load Profile Segment, the request shall provide evidence that the requested group is homogeneous with respect to Load shape characteristics.  For a change to definitions of existing Load Profile Segments, the request shall provide evidence that the re-defined Load Profile Segments are homogeneous in support of the change in definition.

- 12.6.4.1	Load Research Demonstrating Homogeneity

(1)	Evidence of homogeneity may be provided by a statistically valid Load research sample from the population of the requested Load Profile Segment(s).

- (2)	Alternatively, ddata from case studies or,samples of convenience,oay be provided.  This .ay include but is not limited to:1

(a)	Survey data or other evidence of appliance or equipment present in the Premises;

(b)	Data on operating schedules; and

(c)	Variances of parameters of monthly billing data, such as size, ratio of Seasonal consumption values, or Load factors.

12.7	Costs for Profile Segment Changes

(1)	The party requesting the segment change shall bear all costs associated with developing the supporting data and documentation that is submitted to ERCOT for evaluation of the proposed Load Profile Segment changes. In addition, the requestor shall bear all costs, except for ERCOT’s analytical costs, for additional Load research required in conjunction with a request for conditional approval of a Load Profile Segment change.

(2)	In the event the change is approved, costs for implementing the changes in ERCOT data systems shall be the responsibility of ERCOT.  Responsibility for re-assigning Load Profiles remains with the Transmission and/or Distribution Service Provider (TDSP).

(3)	If a Load Profile Segment change request receives final approval under the provisions of the Load Profiling Guide (LPG), and results in the adoption of a new Load Profile Segment available to all Competitive Retailers (CRs), the provisions of Protocol Section 9.18, Profile Development Cost Recovery Fee for a Non-ERCOT Sponsored Load Profile Segment, shall be followed to provide for compensating the requestor by CRs seeking to assign Customers to the Load Profile Segment.  Once a Load Profile Segment change request receives final approval, any subsequent costs required for ongoing support of the Load Profile Segment shall be considered part of the usual operation and maintenance expense for Load Profile Segments available for use by all CRs.

- 12.8	Evaluation of the Load Profile Segment Request

(1)	ERCOT shall evaluate the request based on Load research standards described in the Association of Edison Illuminating Companies (AEIC) Load Research Manual.

1RC

.

4	p

13	   CHANGES TO WEATHER ZONE CHARACTERISTICS

(1)	Changes to Weather Zones and any combination thereof that may be requested including changes in:

(a)	Weather Zone boundary definitions;

(b)	Boundaries of weather modeling regions;

(c)	Weather stations used; and

(d)	Weighting factors assigned to weather stations used within a Weather Zone.

(2)	Any change to Load Profile ID assignments resulting from an approved modification to the definitions of Weather Zones shall only be applied on a prospective basis.

(3)	Sufficient documentation shall be provided to demonstrate that the Weather Zone modification improves the accuracy of Settlement.

- 13.1.2	Uniformity

(1)	Weather Zone characteristics shall be applied to all Electric Service Identifiers (ESI IDs) located within the geographic boundaries of the Weather Zone.  Zone Improvement Plan (ZIP) codes are mapped to Weather Zones and are defined by the ZIP-to-Zone mapping in the Appendix D, Profile Decision Tree.

- 13.2.1	Types of Weather Zone Boundary Changes

(1)	Changes to Weather Zone boundaries and any combination thereof, may occur due to the following conditions:

(a)	Subdivision:  An existing Weather Zone is divided into two or more zones;

(b)	Boundary shifting:  Existing Weather Zone boundaries are moved so that areas are shifted between Weather Zones; or

(c)	Boundary collapsing:  Existing Weather Zone boundaries are moved so that one Weather Zone is created from two or more existing Weather Zones.

(2)	When creating a new Weather Zone, the other zones affected by the boundary change shall satisfy the Weather Zone criteria in Section 13.2.2, Eligible Areas for Weather Zones.

- 13.2.2.2	Weather Stations

- (1)	Only weather data from National Oceanic and Atmospheric Administration (NOAA) first or second order weather stations shall be used in model calculations.  Each proposed new or changed Weather Zone shall have at least two NOAA first or second order weather stations to represent it.  No weather station is permitted to have more than 50% weighting factor within a Weather Zone.

- d

- 13.5.1.1	Changes in National Oceanic and Atmospheric Administration Weather Station

(1)	In the event National Oceanic and Atmospheric Administration (NOAA) discontinues or downgrades an existing weather station, ERCOT shall assess and propose reasonable adjustments.

# 15	LOAD RESEARCH SAMPLES [RESERVED]

- 16	[RESERVED]

- 17	[RESERVED]

- 18	ACCESS TO LOAD PROFILING MATERIALS

(1)	The following Load Profiling related documents and materials may be found on the ERCOT website:

(a)	Backcasted (Actual) Load Profiles – Extract files Load Profiles for individual  trade days.  The trade day occurring one day prior to the current date will be the most current backcast available;

(b)	Forecasted Load Profiles – Files include current day and three days forward of forecasted Load Profiles;

(c)	Historical Backcasted Load Profiles – Multiple years of Load Profile history for each Load Profile Type and Weather Zone combination;

(d)	Methodology – Description of Materials Used in Preparation of Load Profiles – Documents that provide an evaluation of the utility data used to generate the ERCOT Load Profile Models.

(2)	Profiling Working Group (PWG) information and meetings may be found on the ERCOT website.

- 19	Definitions and Acronyms

- 19.1		Definitions

see Load Profile Cl

- Meter Data Type

The component of the Load Profile ID that identifies the type of meter data—either interval (IDR) or non-interval (NIDR)—that is to be submitted to ERCOT by the Transmission and/or Distribution Service Provider (TDSP) and used for settlement.

The document that contains the directions for determining the Load Profile ID to be assigned to an Electric Service Identifier (ESI ID).

- 19.2		Acronyms

ERCOT Load Profiling Guide

Appendix C

TBD

- Appendix C

Appendix intentionally left blank

AvgLF=m=112AHUsemm=112MaxKWm

AvgLF=m=112TmNmm=112maxt∈MmLt

ERCOT Load Profiling Guide

Appendix D:  Profile Decision Tree

TBD

- Appendix D

Profile Decision Tree

See electronic Microsoft Office Excel© file on the ERCOT Website posted with the Load Profiling Guide