| NPRR Number | 791 | NPRR Title | Clarifications to IEL, MCE and Aggregate Amount Owed by Breaching Party |
| --- | --- | --- | --- |
| Date of Decision |  | August 11, 2016 |  |
| Action |  | Tabled |  |
| Timeline |  | Normal |  |
| Proposed Effective Date |  | To be determined |  |
| Priority and Rank Assigned |  | To be determined |  |
| Nodal Protocol Sections Requiring Revision |  | 16.11.4.1, Determination of Total Potential Exposure for a Counter-Party 16.11.4.2, Determination of Counter-Party Initial Estimated Liability 16.11.6.1.3, Aggregate Amount Owed by Breaching Market Participant Immediately Due |  |
| Related Documents Requiring Revision/ Related Revision Requests |  | Counter-Party Credit Application |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) revises the Initial Estimated Liability (IEL) description to clarify that for generation the IEL is based on estimated Qualified Scheduling Entity (QSE)-to-QSE energy sales. A parallel change has been made to the Counter-Party Credit Application. This NPRR also restores the IEL for traders, inadvertently omitted from NPRR741, Clarifications to TPE and EAL Credit Exposure Calculations, and originally defined in NPRR620, Collateral Requirements for Counter-Parties with No Load or Generation. Additionally, this NPRR corrects subscripts in the Minimum Current Exposure (MCE) formula that were inadvertently overwritten by NPRR743, Revision to MCE to Have a Floor For Load Exposure; modifies the RTQQNET and DARTNET in the gray boxed language for NPRR741 to match the current baseline Protocols; and simplifies the MCE formula. This NPRR also removes an obsolete reference to “Deferred Invoice Exposure”, a concept removed by NPRR728, Removal of Language and Definitions Related to NPRR484, Revisions to Congestion Revenue Rights Credit Calculations and Payments, and NPRR554, Clarification of Future Credit Exposure Calculation. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | This NPRR provides additional clarity by providing further explanation of credit calculation components and removing discrepancies in the Protocol language. |  |
| Credit Work Group Review |  | To be determined |  |
| PRS Decision |  | On 8/11/16, PRS unanimously voted to table NPRR791 and refer the issue to the Credit Work Group (Credit WG) and WMS. All Market Segments were present for the vote. |  |
| Summary of PRS Discussion |  | On 8/11/16, participants requested further review of NPRR791 by the Credit WG and Market Credit Working Group (MCWG). |  |

| Sponsor |  |
| --- | --- |
| Name | Vanessa Spells |
| E-mail Address | vanessa.spells@ercot.com |
| Company | ERCOT |
| Phone Number | 512 225 7014 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Kelly Landry |
| E-Mail Address | KLandry@ercot.com |
| Phone Number | 512 248 4630 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None |  |

Market Rules Notes

None

Proposed Protocol Language Revision

16.11.4.1	Determination of Total Potential Exposure for a Counter-Party

(1)	A Counter-Party’s TPE is the sum of its “Total Potential Exposure Any” (TPEA) and TPES:

(a)	TPEA is the positive net exposure of the Counter-Party that may be satisfied by any forms of Financial Security defined under paragraphs (a) through (d) of Section 16.11.3, Alternative Means of Satisfying ERCOT Creditworthiness Requirements.  TPEA will include all exposure not included in TPES.

(b)	TPES is the positive net exposure of the Counter-Party that may be satisfied only by forms of Financial Security defined under paragraphs (b) through (d) of Section 16.11.3.  The Future Credit Exposure (FCE) that reflects the future mark-to-market value for CRRs registered in the name of the Counter-Party is included in TPES.

(2)	For all Counter-Parties:

TPEA 	= 	Max [0, MCE, Max [0, (EAL q + EAL a)]] + PUL

TPES	=	Max [0, FCE a] + IA

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| EAL q | $ | Estimated Aggregate Liability for all QSEs—EAL for all QSEs represented by the Counter-Party. |
| EAL a | $ | Estimated Aggregate Liability for all CRR Account Holders—EAL for all CRR Account Holders represented by the Counter-Party. |
| PUL | $ | Potential Uplift—Potential uplift to the Counter-Party, to the extent and in the proportion that the Counter-Party represents Entities to which an uplift of a short payment will be made pursuant to Section 9.19, Partial Payments by Invoice Recipients. It is calculated as the sum of: (a) Amounts expected to be uplifted within one year of the date of the calculation; and (b) 25%, or such other percentage based on available statistics regarding payment default under bankruptcy reorganization plans, of any short payment amounts being repaid to ERCOT under a bankruptcy reorganization plan that are due more than one year from the date of the calculation. |
| FCE a | $ | Future Credit Exposure for all CRR Account Holders—FCE for all CRR Account Holders represented by the Counter-Party. |
| MCE | $ | Minimum Current Exposure—For each Counter-Party, ERCOT shall determine a Minimum Current Exposure (MCE) as follows: MCE = Max[0, SAF*Max[{[[[L i, od, p * T2 - G i, od, p * (1-NUCADJ) * T3] * RTSPP i, od, p ] + [RTQQNET i, od, p * T5]]/n}, {[G i, od, p * NUCADJ * T1 * RTSPP i, od, p ]/n}, {abs((DARTNET i, od, p)) * T4/n}]] RTQQNET i, od, p = Max[Σc(RTQQES i, od, p, c - RTQQEP i, od, p, c), BTCF * Σc(RTQQES i, od, p, c – RTQQEP i, od, p, c)] * RTSPP i,od, p DARTNET i, od, p = DAM EOO Cleared i, od, p * DART i, od, p + DAM TPO Cleared i, od, p * DART i, od, p + DAM PTP Cleared i, od, p * DARTPTP i, od, p – DAM EOB Cleared i, od, p * DART i, od, p - Where: - G i, od, p = Total Metered Generation at all Resource Nodes for the Counter-Party for interval i for Operating Day od at Settlement Point p L i, od, p = Total Adjusted Metered Load (AML) at all Load Zones for the Counter-Party for interval i for Operating Day od at Settlement Point p SAF = Seasonal Adjustment Factor—Used to provide for the potential for Seasonal price increases based on historical trends. ERCOT shall initially set this factor equal to 100%. This factor will not go below 100%. ERCOT will provide Notice to Market Participants of any change at least 14 days prior to effective date along with the analysis supporting the change. NUCADJ = Net Unit Contingent Adjustment—To allow for situations where a generator may unintentionally or intentionally meet its requirement from the Real-Time Market (RTM). RTQQNET i, od, p = Net QSE-to-QSE Energy Trades for the Counter-Party for interval i for Operating Day od at Settlement Point p RTQQES i, od, p, c = QSE Energy Trades for which the Counter-Party is the seller for interval i for Operating Day od at Settlement Point p with Counter-Party c RTQQEP i, od, p, c = QSE Energy Trades for which the Counter-Party is the buyer for interval i for Operating Day od at Settlement Point p with Counter-Party c BTCF = Bilateral Trades Credit Factor RTSPP i, od, p = Real-Time Settlement Point Price for interval i for Operating Day od at Settlement Point p DARTNET i, od, p = Net DAM activities for the Counter-Party for interval i for Operating Day od at Settlement Point p DART i, od, p = Day-Ahead - Real-Time Spread for interval i for Operating Day od at Settlement Point p DAM EOB Cleared i, od, p = DAM Energy Only Bids Cleared for interval i for Operating Day od at Settlement Point p DAM EOO Cleared i, od, p = DAM Energy Only Offers Cleared for interval i for Operating Day od at Settlement Point p DAM TPO Cleared i, od, p = DAM Three-Part Offers Cleared for interval i for Operating Day od at Settlement Point p DAM PTP Cleared i, od, p = DAM Point-to-Point (PTP) Obligations Cleared for interval i for Operating Day od at Settlement Point p DARTPTP i, od, p = Day-Ahead - Real-Time Spread for value of PTP Obligation for interval i for Operating Day od at Settlement Point p c = Bilateral Counter-Party e = Most recent n Operating Days for which RTM Initial Settlement Statements are available i = Settlement Interval n = Days used for averaging od = Operating Day p = A Settlement Point |
| q | None. | QSEs represented by the Counter-Party. |
| a | None. | CRR Account Holders represented by the Counter-Party. |
| IA | $ | Independent Amount—The amount required to be posted as defined in Section 16.16.1, Counter-Party Criteria. |

The above parameters are defined as follows.

| Parameter | Unit | Current Value* |
| --- | --- | --- |
| NUCADJ | Percentage | Minimum value of 20%. |
| T1 | Days | 2 |
| T2 | Days | 5 |
| T3 | Days | 5 |
| T4 | Days | 1 |
| T5 | Days | For a Counter-Party that represents Load this value is equal to 5, otherwise this value is equal to 2. |
| BTCF | Percentage | 80% |
| n | Days | 14 |
| * The current value for the parameters referenced in this table above will be recommended by TAC and approved by the ERCOT Board. ERCOT shall update parameter values on the first day of the month following ERCOT Board approval unless otherwise directed by the ERCOT Board. ERCOT shall provide a Market Notice prior to implementation of a revised parameter value. |  |  |

[NPRR620, NPRR741, and NPRR743: Replace applicable portions of paragraph (2) above with the following upon system implementation:] (2) For all Counter-Parties: TPEA = Max [0, MCE, Max [0, ((1-TOA) * EAL q + TOA * EAL t + EAL a)]] + PUL TPES = Max [0, FCE a] + IA The above variables are defined as follows: Variable; Unit; Description. EAL q; $; Estimated Aggregate Liability for all QSEs that represents Load or generation—EAL for all QSEs represented by the Counter-Party if at least one QSE represented by the Counter-Party represents either Load or generation.. EAL t; $; Estimated Aggregate Liability for all QSEs —EAL for all QSEs represented by the Counter-Party if none of the QSEs represented by the Counter-Party represent either Load or generation.. EAL a; $; Estimated Aggregate Liability for all CRR Account Holders—EAL for all CRR Account Holders represented by the Counter-Party.. PUL; $; Potential Uplift—Potential uplift to the Counter-Party, to the extent and in the proportion that the Counter-Party represents Entities to which an uplift of a short payment will be made pursuant to Section 9.19, Partial Payments by Invoice Recipients. It is calculated as the sum of: (a) Amounts expected to be uplifted within one year of the date of the calculation; and (b) 25%, or such other percentage based on available statistics regarding payment default under bankruptcy reorganization plans, of any short payment amounts being repaid to ERCOT under a bankruptcy reorganization plan that are due more than one year from the date of the calculation.. FCE a; $; Future Credit Exposure for all CRR Account Holders—FCE for all CRR Account Holders represented by the Counter-Party.. MCE; $; Minimum Current Exposure—For each Counter-Party, ERCOT shall determine a Minimum Current Exposure (MCE) as follows: MCE = SAF*Max[{[L i,od, p * RTSPP i, od, p ]/n}, { [[[L i, od, p * T2 - G i, od, p * (1-NUCADJ ) * T3] * RTSPP i, od, p ] + [RTQQNET i, od, p * T5]]/n}, { [G i, od, p * NUCADJ * T1 * RTSPPi, od, p]/n}, { DARTNET i, od, p *T4/n}, IMCE] RTQQNET i, od, p =ΣcMax[Σc (RTQQES i, od, p, c - RTQQEP i, od, p, c), BTCF * Σc (RTQQES i, od, p, c – RTQQEP i, od, p, c)] * RTSPP i, od, p DARTNET i, od, p = DAM EOO Cleared i, od, p * DART i, od, p + DAM TPO Cleared i, od, p * DART i, od, p + DAM PTP Cleared i, od, p * DARTPTP i, od, p – DAM EOB Cleared i, od, p * DART i, od, p - Where: - G i, od, p = Total Metered Generation at all Resource Nodes for the Counter-Party for interval i for Operating Day od at Settlement Point p L i, od, p = Total Adjusted Metered Load (AML) at all Load Zones for the Counter-Party for interval i for Operating Day od at Settlement Point p SAF = Seasonal Adjustment Factor—Used to provide for the potential for Seasonal price increases based on historical trends. ERCOT shall initially set this factor equal to 100%. This factor will not go below 100%. ERCOT will provide Notice to Market Participants of any change at least 14 days prior to effective date along with the analysis supporting the change. NUCADJ = Net Unit Contingent Adjustment—To allow for situations where a generator may unintentionally or intentionally meet its requirement from the Real-Time Market (RTM). RTQQNET i, od, p = Net QSE-to-QSE Energy Trades for the Counter-Party for interval i for Operating Day od at Settlement Point p RTQQES i, od, p, c = QSE Energy Trades for which the Counter-Party is the seller for interval i for Operating Day od at Settlement Point p with Counter-Party c RTQQEP i, od, p, c = QSE Energy Trades for which the Counter-Party is the buyer for interval i for Operating Day od at Settlement Point p with Counter-Party c BTCF = Bilateral Trades Credit Factor RTSPP i, od, p = Real-Time Settlement Point Price for interval i for Operating Day od at Settlement Point p DARTNET i, od, p = Net DAM activities for the Counter-Party for interval i for Operating Day od at Settlement Point p DART i, od, p = Day-Ahead - Real-Time Spread for interval i for Operating Day od at Settlement Point p DAM EOB Cleared i, od, p = DAM Energy Only Bids Cleared for interval i for Operating Day od at Settlement Point p DAM EOO Cleared i, od, p = DAM Energy Only Offers Cleared for interval i for Operating Day od at Settlement Point p DAM TPO Cleared i, od, p = DAM Three-Part Offers Cleared for interval i for Operating Day od at Settlement Point p DAM PTP Cleared i, od, p = DAM Point-to-Point (PTP) Obligations Cleared for interval i for Operating Day od at Settlement Point p DARTPTP i, od, p = Day-Ahead - Real-Time Spread for value of PTP Obligation for interval i for Operating Day od at Settlement Point p c = Bilateral Counter-Party cif = Cap Interval Factor - Represents the historic largest percentage of System-Wide Offer Cap (SWCAP) intervals during a calendar day d = Operating Day e = Most recent n Operating Days for which RTM Initial Settlement Statements are available i = Settlement Interval n = Days used for averaging nm = Notional Multiplier od = Operating Day p = A Settlement Point. IMCE; $; Initial Minimum Current Exposure IMCE = TOA * (EFFCAP * nm * cif%) Where: EFFCAP = Effective Cap. The greater of Value of Lost Load (VOLL), as described in the Methodology for Implementing Operating Reserve Demand Curve (ORDC) to Calculate Real-Time Reserve Price Adder, or the SWCAP, as determined in accordance with Public Utility Commission of Texas (PUCT) Substantive Rules.. TOA; None; Trade-Only Activity—Counter-Party that does not represent either a Load or a generation QSE. Set to “0” if Counter-Party represents a QSE that has an association with a Load Serving Entity (LSE) or a Resource Entity, or if Counter-Party does not represent any QSE; otherwise set to 1.. q; None.; QSEs represented by Counter-Party.. a; None.; CRR Account Holders represented by Counter-Party.. IA; $; Independent Amount—The amount required to be posted as defined in Section 16.16.1, Counter-Party Criteria.. The above parameters are defined as follows. Parameter; Unit; Current Value*. nm; None; 50. cif; Percentage; 9%. NUCADJ; Percentage; Minimum value of 20%.. T1; Days; 2. T2; Days; 5. T3; Days; 5. T4; Days; 1. T5; Days; For a Counter-Party that represents Load this value is equal to 5, otherwise this value is equal to 2.. BTCF; Percentage; 80%. n; Days; 14. * The current value for the parameters referenced in this table above will be recommended by TAC and approved by the ERCOT Board. ERCOT shall update parameter values on the first day of the month following ERCOT Board approval unless otherwise directed by the ERCOT Board. ERCOT shall provide a Market Notice prior to implementation of a revised parameter value..

(3)	If ERCOT, in its sole discretion, determines that the TPEA or the TPES for a Counter-Party calculated under paragraphs (1) or (2) above does not adequately match the financial risk created by that Counter-Party’s activities under these Protocols, then ERCOT may set a different TPEA or TPES for that Counter-Party.  ERCOT shall, to the extent practical, give to the Counter-Party the information used to determine that different TPEA or TPES.  ERCOT shall provide written or electronic Notice to the Counter-Party of the basis for ERCOT’s assessment of the Counter-Party’s financial risk and the resulting creditworthiness requirements.

(4)	ERCOT shall monitor and calculate each Counter-Party’s TPEA and TPES daily.

16.11.4.2	Determination of Counter-Party Initial Estimated Liability

(1)	For each Counter-Party, ERCOT shall determine an IEL for purposes of Section 16.11.3, Alternative Means of Satisfying ERCOT Creditworthiness Requirements.

[NPRR620: Replace paragraph (1) above with the following upon system implementation:] (1) For each Counter-Party, except those Counter-Parties that are only CRR Account Holders, ERCOT shall determine an IEL for purposes of Section 16.11.3, Alternative Means of Satisfying ERCOT Creditworthiness Requirements.

(2)	For a Counter-Party that has all its QSEs representing only Load-Serving Entities (LSEs), ERCOT shall calculate the IEL using the following formula:

IEL = DEL * Max [0.2, RTEFL] * RTAEP * (M1 + M2)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| IEL | $ | Initial Estimated LiabilityThe Counter-Party’s Initial Estimated Liability. |
| DEL | MWh | Daily Estimated LoadThe Counter-Party’s estimated average daily Load as determined by ERCOT based on information provided by the Counter-Party. |
| RTEFL | none | Real-Time Energy Factor for LoadThe ratio of the Counter-Party’s estimated energy purchases in the RTM as determined by ERCOT based on information provided by the Counter-Party, to the Counter-Party’s Daily Estimated Load. |
| RTAEP | $/MWh | Real-Time Average Energy PriceAverage Settlement Point Price for the “ERCOT 345” as defined in Section 3.5.2.5, ERCOT Hub Average 345 kV Hub (ERCOT 345), based upon the previous seven days’ average Real-Time Settlement Point Prices. |

(3)	For a Counter-Party that has all its QSEs representing only Resources, ERCOT shall calculate the IEL using the following formula:

IEL	=	DEG * Max [0.2, RTEFG] * RTAEP * (M1 + M2)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| IEL | $ | Initial Estimated LiabilityThe Counter-Party’s Initial Estimated Liability. |
| DEG | MWh | Daily Estimated GenerationThe Counter-Party’s estimated average daily generation as determined by ERCOT based on information provided by the Counter-Party. |
| RTEFG | none | Real-Time Energy Factor for GenerationThe ratio of the Counter-Party’s QSE to QSE estimated energy sales as determined by ERCOT based on information provided by the Counter-Party, to the Counter-Party’s Daily Estimated Generation. |
| RTAEP | $/MWh | Real-Time Average Energy PriceAverage Settlement Point Price for the “ERCOT 345” as defined in Section 3.5.2.5 based upon the previous seven days average Real-Time Settlement Point Prices. |

(4)	For a Counter-Party that has QSEs representing both LSE and Resources, ERCOT shall calculate the Counter-Party’s IEL using the following formula:

IEL	=	DEL * Max [0.1, RTEFL] * RTAEP * (M1 + M2) + DEG * Max [0.1, RTEFG] * RTAEP * (M1 + M2)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| IEL | $ | Initial Estimated LiabilityThe Counter-Party’s Initial Estimated Liability. |
| DEL | MWh | Daily Estimated LoadThe Counter-Party’s estimated average daily Load as determined by ERCOT based on information provided by the Counter-Party. |
| DEG | MWh | Daily Estimated GenerationThe Counter-Party’s estimated average daily generation as determined by ERCOT based on information provided by the Counter-Party. |
| RTEFL | none | Real-Time Energy Factor for LoadThe ratio of the Counter-Party’s estimated energy purchases in the RTM as determined by ERCOT based on information provided by the Counter-Party, to the Counter-Party’s Daily Estimated Load. |
| RTAEP | $/MWh | Real-Time Average Energy PriceAverage Settlement Point Price for the “ERCOT 345” as defined in Section 3.5.2.5 based upon the previous seven days’ average Real-Time Settlement Point Prices. |
| RTEFG | none | Real-Time Energy Factor for Generation—The ratio of the Counter-Party’s QSE to QSE estimated energy sales as determined by ERCOT based on information provided by the Counter-Party, to the Counter-Party’s Daily Estimated Generation. |

(5)	For a Counter-Party that has all its QSEs representing neither Load nor generation, and that is not representing a CRR Account Holder, the IEL is equal to IMCE as defined in paragraph (2) of Section 16.11.4.1, Determination of Total Potential Exposure for a Counter-Party.

(6)	For a Counter-Party that is only a CRR Account Holder and is not a QSE, the IEL is zero.

16.11.6.1.3	Aggregate Amount Owed by Breaching Market Participant Immediately Due

(1)	ERCOT shall aggregate all amounts due it by the Market Participant under any agreement with ERCOT and these Protocols into a single amount to the fullest extent allowed by law.  The entire unpaid net balance owed to ERCOT by the Market Participant, at ERCOT’s option, and its sole discretion, is immediately due and payable without further notice and demand for payment.  Any such notice and demand for payment are expressly waived by the Market Participant.