- Section 2:

NPRR747 – Revision of Voltage Control Requirements

This Nodal Protocol Revision Request (NPRR) adds new definitions related to Voltage Profiles, defines various Entities’ responsibilities related to Voltage Support, and also clarifies that the interconnecting Transmission Service Provider (TSP) (or its designated agent) may modify a Generation Resource’s Voltage Set Point.

Revised Subsection:	2.1 [effective upon system implementation of Nodal Operating Guide Revision Request (NOGRR) 150, Alignment with NPRR747, Revision of Voltage Control Requirements]

- Section 3:

NPRR747 – Revision of Voltage Control Requirements

See Section 2 above.

Revised Subsections:	3.15, 3.15.3 [effective upon system implementation of NOGRR150]

- Section 5:

NPRR767 – Commitment by RUC for Long Lead Time Resource

This Nodal Protocol Revision Request (NPRR) changes the eligibility check for the startup portion of the Reliability Unit Commitment (RUC) Make-Whole Payment.  Resources with lead times longer than six hours may submit a Settlement dispute to have their Resource-specific startup times considered when determining eligibility for Startup Costs to be included in the RUC Make-Whole Payment calculation.

Revised Subsection:  5.6.2

- Section 6:

NPRR662 – Proxy Energy Offer Curves

This Nodal Protocol Revision Request (NPRR) addresses proxy Energy Offer Curves generated by ERCOT for a Resource that does not submit an Energy Offer Curve or submits an incomplete Energy Offer Curve.  For a Resource without an Energy Offer Curve, a monotonically increasing proxy Energy Offer Curve will be created such that the last point on the curve is extended.  For a Resource with an incomplete Energy Offer Curve (typically cogeneration facilities that update their High Sustained Limit (HSL), but not their offer curve), a proxy Energy  Offer Curve will be created to extend the offer price from last point on the submitted Energy Offer Curve up to the Resource’s HSL.

Revised Subsection:	6.5.7.3 [unboxed due to system implementation]

NPRR696 – Price Correction Process Following a SCED Failure

This Nodal Protocol Revision Request (NPRR) establishes a price correction policy which uses the last good price for the intervals covering the first 15 minutes following a Security-Constrained Economic Dispatch (SCED) failure.

Revised Subsections:  6.3, 6.5.9.2

NPRR747 – Revision of Voltage Control Requirements

See Section 2 above.

Revised Subsection:	6.5.7.7 [effective upon system implementation of NOGRR150]

NPRR770 – Addition of Outaged Resource Capacity to the Ancillary Services Capacity Monitor

This Nodal Protocol Revision Request (NPRR) adds visibility and situational awareness to the market by posting the aggregate quantity of Resources that are telemetered as OUT, OUTL or EMR to the Ancillary Service Capacity Monitor.

Revised Subsection:  6.5.7.5 [effective upon system implementation]

- Section 8:

NPRR738 – ERS Performance Calculations During TDSP Outages

This Nodal Protocol Revision Request (NPRR) excludes intervals from performance calculations for which an Emergency Response Service (ERS) Generator is unable to meet its obligations due to Transmission and/or Distribution Service Provider (TDSP) Outages.

Revised Subsections:  8.1.3.1.3.1, 8.1.3.1.3.2, 8.1.3.1.4, 8.1.3.2, 8.1.3.3.1, 8.1.3.3.2, 8.1.3.3.3, 8.1.3.3.4 [effective upon system implementation]

- Section 13:

NPRR774 – Remove Duplicate Language to Calculate Seasonal Transmission Loss Factors

This Nodal Protocol Revision Request (NPRR) removes duplicate language regarding calculation of Seasonal Transmission Loss Factors (TLFs).

Revised Subsections:	13.2.4, 13.4.1

- Section 15:

NPRR771 – Clarifications to New ESI ID Creation Process

This Nodal Protocol Revision Request (NPRR) clarifies that the Transmission and/or Distribution Service Providers (TDSPs) must ensure that an Electric Service Identifier (ESI ID) has been created in the ERCOT systems prior to initiating electric service at a Premise. As a result, related transactional, billing and out-of-sync issues may be avoided.

Revised Subsection:	15.4.1.4

- Administrative Changes:

- Non-substantive administrative changes were made such as spelling corrections, formatting, and correcting Section numbering and references.

- Revised Subsections:  3.2.3, 13.1, 13.1.2, 13.2.3, 13.2.5, 13.3.2