ERCOT Load Profiling Guide

Section 12:  Request for Load Profile Segment Changes, Additions, or Removals

November 1, 2016

12	REQUEST FOR LOAD PROFILE SEGMENT CHANGES, ADDITIONS, OR REMOVALS	12-1

12.1	 Types of Requests	12-1

12.1.1	Creation of a New Load Profile Segment	12-1

12.1.2	 Redefinition of an Existing Load Profile Segment	12-1

12.1.3	Removal of Existing Load Profiles Segments	12-1

12.2	Request for Load Profile Segment Changes	12-2

12.3	Procedure for Submitting a Request	12-2

12.4	Groups of Electric Service Identifiers Eligible to Become Load Profile Segments	12-2

12.4.1	Universal Load Profile Segment Applicability	12-3

12.5	Information Required with Request for Change	12-3

12.5.1	Unambiguous Group Identification	12-3

12.5.2	Difference from Current Load Profile Segments	12-4

12.5.3	Size	12-6

12.5.4	Homogeneity	12-7

12.5.5	Quality Assurance Methodology for Electric Service Identifier Identification	12-8

12.6	Costs for Profile Segment Changes	12-8

12.7	Evaluation of the Load Profile Segment Request	12-9

# REQUEST FOR LOAD PROFILE SEGMENT CHANGES, ADDITIONS, OR REMOVALS

(1)	This Section 12, Request for Load Profile Segment Changes, Additions, or Removals, of the Load Profiling Guide (LPG) addresses changes, additions, and deletions to Load Profile Segments, with the exception of Load Profile Segment modifications addressed in Section 16, Supplemental Load Profiling.

(2)	The steps and tests identified to introduce new Load Profiles or changes to Load Profiles are intended to fulfill the criteria established in Protocol Section 18.2.1, Guidelines for Development of Load Profiles.

(3)	Any change to Load Profile ID assignments resulting from an approved modification to the definitions of Load Profile Segments shall not be retroactively applied.

12.1		Types of Requests

(1)	The types of requests are:

(a)	Creation of a new Load Profile Segment from one or more existing Load Profile Segments;

(b)	Redefinition of existing Load Profile Segments; and

(c)	Removal of existing Load Profile Segments.

12.1.1	Creation of a New Load Profile Segment

(1)	When a new Load Profile Segment is created, there may be an impact to the Electric Service Identifiers (ESI IDs) assigned to one or more existing Load Profile Segments.  The new Load Profile Segment will be applied to ESI IDs that meet the criteria for the new Load Profile Segment while being removed from the existing Load Profile Segment.

12.1.2		Redefinition of an Existing Load Profile Segment

(1)	Redefinition of existing Load Profile Segment definitions may require that some ESI IDs be moved from one Load Profile Segment to another.

12.1.3	Removal of Existing Load Profiles Segments

(1)	A request to remove an existing Load Profile Segment shall include supporting documentation that the Load Profile Segment proposed for removal does not satisfy the standards for a separate Load Profile.  Specifically, the group represented by the Load Profile may be:

(a)	Too small to justify a separate Load Profile Segment, as described in Section 12.4, Groups of Electric Service Identifiers Eligible to Become Load Profile Segments; and/or

(b)	Sufficiently similar to one or more existing Load Profiles, according to the measures defined in Section 12.4.

(2)	Removal of an existing Load Profile Segment requires changing assignments of existing ESI IDs currently in the proposed removed Load Profile Segment.

## 12.2	Request for Load Profile Segment Changes

(1)	Any request for a change to Load Profile Segments shall be submitted in accordance with Section 2.4, Load Profiling Guide Revision Procedure.

12.3	Procedure for Submitting a Request

(1)	ERCOT shall post a Load Profile Segment change request form to the ERCOT website.  A completed application form shall accompany all requests for a Load Profile Segment change.  Data sets, supporting files, and documentation shall be provided in electronic form.

(2)	If the originator of the Load Profile Segment change request is a Market Participant other than ERCOT, they shall indicate on the submitted form that they are requesting either a conditional or full approval of the change.  Subsequent to submitting the form, the originator may amend the request from being conditional to full or vice versa by notifying ERCOT and the Profiling Working Group (PWG).

12.4	Groups of Electric Service Identifiers Eligible to Become Load Profile Segments

(1)	For a group of Electric Service Identifiers (ESI IDs) to be a distinct Load Profile Segment, the group shall satisfy the following requirements:

(a)	The group is based on readily identifiable parameters, which are not subject to frequent change;

(b)	The group is reasonably homogeneous as defined in Section 12.5.4, Homogeneity;

(c)	The group is sufficiently different from other existing Load Profiles as defined in Section 12.5.2, Difference from Current Load Profiles; and

(d)	The group is of sufficient size to justify its own Load Profile Segment as defined in Section 12.5.3, Size.

12.4.1	Universal Load Profile Segment Applicability

(1)	One primary goal is to make all Load Profile Segment definitions universally applicable.  Universally applicable means:

(a)	The Load Profile may be applicable to all Competitive Retailers (CRs);

(b)	The Load Profile Segment shall be applied to any ESI ID in the ERCOT System that meets the eligibility criteria; and

(c)	The Load Profile Segment shall be public.

(2)	There are limited exceptions.

12.5	Information Required with Request for Change

(1)	All requests shall include the following:

(a)	Unambiguous group identification;

(b)	Difference from current Load Profile Segments;

(c)	Size;

(d)	Homogeneity; and

(e)	Validation methodology.

12.5.1	Unambiguous Group Identification

(1)	The definition of the group shall be provided in the request for the new Load Profile Segment.  The request shall unambiguously define specific criteria for an Electronic Service Identifier (ESI ID) to be included in the new Load Profile Segment.  In a request to change an existing Load Profile Segment, the group to be re-assigned shall be identified.

12.5.1.1	Identification Based on Data Currently in ERCOT’s Systems

(1)	The most direct way a group may satisfy the requirement of being unambiguously identified occurs when the group may be identified based solely on information currently available in the ERCOT data systems or readily derived from such data.

(2)	Examples of information available in or derived from the ERCOT data systems include, but are not limited to:

(a)	Monthly or annual kWh consumption;

(b)	Metered monthly or annual peak Demand for Demand-metered Customers;

(c)	Monthly or annual Load factor;

(d)	Ratio of seasonal consumption values; and

(e)	Zone Improvement Plan (ZIP) code.

12.5.1.2	Identification Based on Other Means

(1)	Load Profile Segments based on other criteria may be requested.  ERCOT, in coordination with the Profiling Working Group (PWG), shall evaluate such requests in terms of the feasibility and reliability of the proposed identification method.  If the method requires data not currently in ERCOT’s systems, the request shall describe how these data shall be made available to ERCOT on an ongoing basis.  If the identification method is judged to be impractical or unreliable, the request may be denied.

12.5.2	Difference from Current Load Profile Segments

(1)	A requested new Load Profile Segment shall be shown in the supporting documentation to be different from existing Load Profiles, and that the group re-assigned from one Load Profile Segment to another is more similar to the proposed new assignment(s) than to the old one, in ways that improve the accuracy of Settlement.

(2)	If the ESI IDs in the requested Load Profile Segment are different from the Load Profile Segment that they are currently assigned and more similar to another existing Load Profile Segment, then the resolution of the request may be to reassign these ESI IDs to the most similar existing Load Profile Segment.

(3)	Requests to create new Load Profile Segments or to change the definition of existing Load Profile Segments require supporting documentation to provide a basis for assessing differences between the affected group and existing Load Profile Segments.  All differences between Load Profiles that are important for evaluating a change shall be documented in the request.

12.5.2.1	Supporting Data Required

(1)	Types of supporting data that may be submitted and the associated documentation are described in the following subsections.

12.5.2.1.1	Load Research Data

(1)	The best documentation would be a statistically valid Load research sample from the proposed new or re-assigned Load Profile Segment population.

(2)	The Load research sample design, data collection and statistical calculations shall be in compliance with industry accepted standards as defined in the Association of Edison Illuminating Companies (AEIC) Load Research Manual.

(3)	Examples of less rigorous, but supportive documentation would be other types of Load research data, such as:

(a)	Data from ad-hoc or convenience samples; and

(b)	Data from a similar population.

(4)	When less rigorous data is submitted, the submitter should also submit evidence to support the applicability of the data to the proposed Load Profile Segment population, including but not limited to:

(a)	Documentation of operating schedules for the proposed group and comparison with typical schedules for Premises in the currently assigned Load Profile;

(b)	End-use saturation data, comparing the proportions of Premises with particular types of electric end uses for the proposed group and currently assigned Load Profiles.  Such data shall be relevant to the proposed population in ERCOT;

(c)	Monthly billing data comparing consumption patterns, particularly related to heating and cooling.  These comparisons shall be made separately by Weather Zone or account for variations by Weather Zone.

12.5.2.2		Basis for Assessment of Differences Based on Load Research Data

(1)	In assessing differences between the initial profile segment and the requested profile segment, ERCOT shall use industry accepted standards as described in the AEIC Load Research Manual.  The requester may submit analysis but is not required to do so.

12.5.2.3	Accounting for Weather Zone Effects in Load Profile Comparisons

(1)	Comparisons between Load Profiles shall take into account Weather Zone effects.  These effects may be accounted for in the comparisons in one of two ways:

(a)	The comparison between the proposed Load Profile Segment and the existing Load Profile is made separately for each Weather Zone; and

(b)	A single Load Profile representing the proposed Load Profile Segment as a whole is compared with a single composite Load Profile for the existing Load Profile Segment.

(2)	These methods are not required for Load Profiles that are the same across all Weather Zones.

12.5.2.4		Separate Comparisons for Each Weather Zone

(1)	If Load research data for individual sample Customers are provided for the proposed segment, a separate Load Profile may be constructed for each Weather Zone.

(2)	The limitation of separate comparisons by Weather Zone is that some or all of the separate Weather Zone Load Profiles may have large statistical errors due to low sample sizes.  The magnitude of these errors should be considered in assessing the comparisons.

12.5.2.5	Comparison for the Proposed Segment as a Whole

(1)	If a single Load Profile is estimated for the proposed Load Profile Segment as a whole across several Weather Zones, this Load Profile may be compared with a composite of existing Load Profiles.  The composite existing Load Profile will be constructed using the standards described in the AEIC Load Research Manual.

12.5.3	Size

(1)	Supporting documentation shall show that the population represented by the proposed segment(s) is of sufficient size to justify a separate segment.  Size shall be provided in terms of both number of Customers and total energy consumption.

(2)	If the proposed Load  Profile Segment is identified based on information available in the ERCOT data systems and also available to the requesting party, documentation of the total ESI ID count and annual energy use is sufficient.  ERCOT shall verify this information using the ERCOT data systems.

(3)	If the requesting party has information on only a portion of the population represented by the segment, the request shall include estimates of the ESI ID counts and energy use, and documentation of the basis for the estimates.

12.5.4	Homogeneity

(1)	For a new Load Profile Segment, the request shall provide evidence that the requested group is homogeneous with respect to Load shape characteristics.  For a change to definitions of existing Load Profile Segments, the request shall provide evidence that the re-defined Load Profile Segments are homogeneous in support of the change in definition.

12.5.4.1	Load Research Demonstrating Homogeneity

(1)	Evidence of homogeneity may be provided by a statistically valid Load research sample from the population of the requested Load Profile Segment(s).

(2)	Alternatively, data from case studies or samples of convenience may be provided.  This may include but is not limited to:

(a)	Survey data or other evidence of appliance or equipment present in the Premises;

(b)	Data on operating schedules; and

(c)	Variances of parameters of monthly billing data, such as size, ratio of Seasonal consumption values, or Load factors.

12.5.4.2	Other Supporting Evidence of Homogeneity

(1)	Less direct evidence of Load shape homogeneity may be submitted.  Examples of such evidence include:

(a)	Survey data or other evidence of appliance or equipment present in the Premises;

(b)	Data on operating schedules; and

(c)	Variances of parameters of monthly billing data, such as size, ratio of seasonal consumption values, or Load factors.

12.5.5	Quality Assurance Methodology for Electric Service Identifier Identification

(1)	If the procedure for identifying ESI IDs applicable to the new Load Profile Segment relies on data that is not currently in ERCOT’s systems, the requestor shall submit the description of a quality assurance procedure, to be managed by ERCOT, to assure that ESI IDs are assigned correctly to the Load Profile Segment and that they are removed from the Load Profile Segment when appropriate.

(2)	The described quality assurance procedure shall be accurate, workable, and reasonable in terms of cost and timeliness. An ideal quality assurance procedure would be one that enables ERCOT to have direct access to a data source of well established reliability, and is maintained by a disinterested third party.  If the validity of the data source has not been well established, a quality control sample, as described below, may be used for quality assurance purposes.

(3)	At a minimum, the quality assurance procedure shall meet a classification accuracy of  5% at 95% confidence such as could be obtained with a random sample for quality control purposes.  If random sampling is identified as the quality assurance methodology, the sampling shall be managed and administered by ERCOT.

(4)	Adequacy of the quality assurance methodology shall be a primary consideration in deciding whether to approve or disapprove the Load Profile Segment change request.

12.6	Costs for Profile Segment Changes

(1)	The party requesting the segment change shall bear all costs associated with developing the supporting data and documentation that is submitted to ERCOT for evaluation of the proposed Load Profile Segment changes. In addition, the requestor shall bear all costs, except for ERCOT’s analytical costs, for additional Load research required in conjunction with a request for conditional approval of a Load Profile Segment change.

(2)	In the event the change is approved, costs for implementing the changes in ERCOT data systems shall be the responsibility of ERCOT.  Responsibility for re-assigning Load Profiles remains with the Transmission and/or Distribution Service Provider (TDSP).

(3)	If a Load Profile Segment change request receives final approval under the provisions of the Load Profiling Guide (LPG), and results in the adoption of a new Load Profile Segment available to all Competitive Retailers (CRs), the provisions of Protocol Section 9.18, Profile Development Cost Recovery Fee for a Non-ERCOT Sponsored Load Profile Segment, shall be followed to provide for compensating the requestor by CRs seeking to assign Customers to the Load Profile Segment.  Once a Load Profile Segment change request receives final approval, any subsequent costs required for ongoing support of the Load Profile Segment shall be considered part of the usual operation and maintenance expense for Load Profile Segments available for use by all CRs.

12.7	Evaluation of the Load Profile Segment Request

(1)	ERCOT shall evaluate the request based on Load research standards described in the Association of Edison Illuminating Companies (AEIC) Load Research Manual.