| PGRR Number | 052 | PGRR Title | Stability Assessment for Interconnecting Generation |
| --- | --- | --- | --- |

| Date | January 11, 2017 |
| --- | --- |

| Submitter’s Information |  |
| --- | --- |
| Name | Jeff Billo |
| E-mail Address | Jeff.Billo@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6334 |
| Cell Number |  |
| Market Segment | N/A |

Comments

ERCOT submits these comments to further define the rights and obligations of both ERCOT and the Interconnecting Entity when, at any point after an All-Inclusive Generation Resource has been included in the quarterly stability assessment but before its Initial Synchronization, either (1) the design of the All-Inclusive Generation Resource changes in a way that could materially impact ERCOT System stability, or (2) the All-Inclusive Generation Resource no longer satisfies the prerequisites for inclusion in the quarterly stability assessment as described in paragraph (4) of Section 5.9, Quarterly Stability Assessment.  The comment seeks to clarify that ERCOT may delay Initial Synchronization until ERCOT has had an opportunity to re-evaluate the project in the next available quarterly stability assessment using the appropriately updated information.  However, the comment also seeks to clarify that ERCOT will not delay the Initial Synchronization of a project if ERCOT determines that a change is not material to System stability.

Additionally, ERCOT notes that it intends to interpret the quarterly stability assessment requirement in paragraph (2) of Section 5.9 to apply only to those projects that are required to meet the prerequisites for inclusion in the assessment at some point on or after the effective date of the Planning Guide Revision Request (PGRR).

With these clarifications, ERCOT requests that the Proposed Effective Date of PGRR052 be revised to “upon system implementation, but no earlier than August 1, 2017”.

| Revised Cover Page Language |  |
| --- | --- |
| Proposed Effective Date | Upon system implementation, but no earlier than August 1, 2017 |

Revised Proposed Guide Language

5.9	Quarterly Stability Assessment

(1)	ERCOT shall conduct a stability assessment every three months to assess the impact of planned new All-Inclusive Generation Resources connecting to the ERCOT System.  The assessment shall derive the conditions to be studied with consideration given to the results of the Full Interconnection Study (FIS) stability studies for the All-Inclusive Generation Resources with planned initial synchronization in the period under study.  ERCOT may study conditions other than those identified in the FIS stability studies.

(2)	All-Inclusive Generation Resources that are not included in the assessment as described in this Section as result of the Interconnecting Entity (IE) failing to meet the prerequisites by the deadlines as listed in the table below will not be eligible for initial synchronization during that three month period.  The timeline for the quarterly stability assessment shall be in accordance with the following table:

| All-Inclusive Generation Resource Initial Synchronization Date | Last Day for an IE to meet prerequisites as listed in paragraph (4) below | Completion of Quarterly Stability Assessment |
| --- | --- | --- |
| Upcoming January, February, March | Prior August 1 | End of October |
| Upcoming April, May, June | Prior November 1 | End of January |
| Upcoming July, August, September | Prior February 1 | End of April |
| Upcoming October, November, December | Prior May 1 | End of July |

- (3)	If the last day for an IE to meet prerequisites or if completion of the quarterly stability assessment as shown in the above table falls on a weekend or holiday, the deadline will extend to the next Business Day.

- (4)	Prerequisites to be satisfied prior to the planned new All-Inclusive Generation Resource being included in the quarterly stability assessment:

- (a) 	The All-Inclusive Generation Resource has met the requirements of Section 6.9, Addition of Proposed All-Inclusive Generation Resources to the Planning Models.

- (b)	The IE has provided all All-Inclusive Generation Resource data in accordance with the Resource Registration Glossary, Planning Model column, including but not limited to steady state, system protection and stability models.

- (c) 	The following elements must be complete:

- (i)	FIS studies;

- (ii)	Reactive Power Study; and

- (iii)	System improvements or mitigation plans that were identified in these studies as required prior to synchronizing a new All-Inclusive Generation Resource in order to meet the operational standards established in the Protocols, Planning Guide, Nodal Operating Guides, and Other Binding Documents.

- (d)	The data used in the studies identified in (c) above is consistent with All-Inclusive Generation Resource data submitted by the IE as required by Section 6.9.

- (5)	At any time following the inclusion of an All-Inclusive Generation Resource in a stability assessment, but before the Initial Synchronization of the All-Inclusive Generation Resource, if ERCOT determines, in its sole discretion, that the All-Inclusive Generation Resource no longer meets the prerequisites described in paragraph (4), or that an IE has made a change to the design of the All-Inclusive Generation Resource that could have a material impact on ERCOT System stability, then ERCOT may refuse to allow Initial Synchronization of the All-Inclusive Generation Resource, provided that ERCOT shall include the All-Inclusive Generation Resource in the next quarterly stability assessment period that commences after identification of the material change or after the All-Inclusive Generation Resource meets the prerequisites specified in paragraph (4), as applicable.  If ERCOT determines, in its sole discretion, that the change to the design of the All-Inclusive Generation Resource would not have a material impact on ERCOT System stability, then ERCOT may not refuse to allow Initial Synchronization of the All-Inclusive Generation Resource due to this change.

- (6)	ERCOT shall post to the Market Information System (MIS) Secure Area a report summarizing the results of the quarterly stability assessment within ten Business Days of completion.