| NPRR Number | 818 | NPRR Title | Allow Curtailment of DC Tie Load Prior to Declaring Emergency Condition |
| --- | --- | --- | --- |
| Date Posted |  | February 1, 2017 |  |
| Requested Resolution |  | Urgent. Urgent status is requested in order to minimize inefficient wholesale market operations and consequent on-going harm to Market Participants as soon as possible. The Protocol change allows ERCOT to curtail Direct Current Tie (DC Tie) Schedules prior to reaching an Emergency Condition thereby reducing instances of reaching Emergency Conditions. Currently the Comision Federal de Electricidad (CFE) DC Ties are under-utilized and the uncertainty created by current procedures of denying Electronic Tags (e-Tags) after the close of the Day-Ahead Market (DAM) further exacerbates the inefficiencies and harm to the market. |  |
| Nodal Protocol Sections Requiring Revision |  | 6.5.9.3.3, Watch 6.5.9.3.4, Emergency Notice |  |
| Related Documents Requiring Revision/Related Revision Requests |  | DC Tie Operating Procedure |  |
| Revision Description |  | This NPRR allows ERCOT to curtail DC Tie Load during a Watch prior to declaring an Emergency Condition. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: Prevents unnecessary harm to Market Participants (please select all that apply) |  |
| Business Case |  | This NPRR minimizes inefficient wholesale market operations and consequent harm to Market Participants. Power Operations Bulletin (POB) #755 issued on 9/28/16 with an implementation date of 9/30/16 drastically changed ERCOT's operation of the DC Ties with particular impact on exports over the CFE DC Ties to Mexico. Prior to POB #755, ERCOT would accept DC Tie Schedule e-Tags up to the physical limit of the corresponding ties and (rarely) only curtail e-Tags close to the Operating Hour if necessary under Emergency Conditions. Post POB #755, ERCOT denies e-Tags based on Resources showing to be On-Line in their Current Operating Plan (COP) submitted just prior to Day-Ahead Reliability Unit Commitment (DRUC) process. Thus, Market Participants wanting to export power have to procure power in the DAM without knowing whether the e-Tag will be approved or denied. In turn, they are subjected to a loss and contractual difficulties with their counterparties if the e-Tag is denied. There are many instances where the DAM price was relatively high whereas the Real-Time Market (RTM) price was depressed when e-Tags were denied and DC Tie capacity was under-utilized - an avoidable inefficient outcome for the market and loss for the Market Participant. The reason for such ERCOT action is ERCOT's preference of taking action in advance to avoid the chance of an Emergency Condition and subsequent North American Electric Reliability Corporation (NERC) scrutiny of why ERCOT did not act in advance to avoid the Emergency Condition. By allowing ERCOT to curtail e-Tags in a Watch prior to an Emergency Condition, ERCOT can revert back to it's pre-POB #755 operating procedure of approving e-Tags up to the physical limits of the DC Ties. E-Tag approval can again be done prior to DAM - allowing Market Participants to correspondingly procure energy in DAM and have the added certainty in its contractual obligations. |  |

| Sponsor |  |
| --- | --- |
| Name | Shams Siddiqi |
| E-mail Address | shams@crescentpower.net |
| Company | Rainbow Energy Marketing Corporation |
| Phone Number | 512-619-3532 |
| Cell Number | 512-619-3532 |
| Market Segment | Independent Power Marketer (IPM) |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Cory Phillips |
| E-Mail Address | cory.phillips@ercot.com |
| Phone Number | 512-248-6464 |

Proposed Protocol Language Revision

- 6.5.9.3.3	Watch

(1)	A Watch is the third of four levels of communication issued by ERCOT in anticipation of a possible Emergency Condition.

(2)	ERCOT shall issue a Watch when ERCOT determines that:

(a)	Conditions have developed such that additional Ancillary Services are needed in the current Operating Period;

(b)	There are insufficient Ancillary Services or Energy Offers in the DAM;

(c)	Market-based congestion management techniques embedded in SCED as specified in these Protocols will not be adequate to resolve transmission security violations;

(d)	Forced Outages or other abnormal operating conditions have occurred, or may occur that require operations with active violations of security criteria as defined in the Operating Guides unless a CMP exists;

(e)	ERCOT varies from timing requirements or omits one or more Day-Ahead or Adjustment Period and Real-Time procedures;

(f)	ERCOT varies from timing requirements or omits one or more scheduling procedures in the Real-Time process; or

(g)	The SCED process fails to reach a solution, whether or not ERCOT is using one of the measures specified in paragraph (3) of Section 6.5.9.2, Failure of the SCED Process.

(3)	With the issuance of a Watch pursuant to paragraph (2)(a) above, ERCOT may exercise its authority to immediately procure the following services from existing offers:

(a)	Regulation Services;

(b)	RRS services; and

(c)	Non-Spin services.

(4)	If ERCOT issues a Watch because insufficient Ancillary Service Offers were received in the DAM or Supplemental Ancillary Service Market (SASM), and if the Watch does not result in sufficient offers and the DAM or SASM is executed with insufficient offers, then ERCOT may acquire the insufficient amount of Ancillary Services as follows:

(a)	The SASM process shall be conducted in accordance with Section 6.4.9.2.2, SASM Clearing Process.  If the SASM process is not sufficient, then;

(b)	The HRUC process shall be conducted to commit planned Off-Line Resources qualified to provide the Ancillary Service(s) that are insufficient in accordance with Section 5.2.2.2, RUC Process Timeline After an Aborted Day-Ahead Market.  If the HRUC process is not sufficient, then;

(c)	If the insufficiency arose due to insufficient Ancillary Service Offers received in the DAM or ERCOT needs to increase the Ancillary Service requirements after DAM clearing, ERCOT may assign the insufficient amounts of Ancillary Service(s) to QSEs with planned On-Line Resources qualified to provide the insufficient Ancillary Service(s), even if there are no existing Ancillary Service Offers for those QSEs’ Resources.  ERCOT shall prorate the required Ancillary Service capacity among QSEs representing On-Line capacity not already reserved for Ancillary Services in the COP in a way that maximizes the distribution of the assignment.

(d)	A QSE may request cancellation of the assignment of Ancillary Services to its On-Line Resources if there are equipment or Resource control issues which limit the ability of the Resources to provide the Ancillary Services.  If ERCOT accepts the cancellation, ERCOT may require QSEs to submit supporting information describing the Resource control issues.

(5)	If ERCOT issues a Watch because market-based congestion management techniques embedded in SCED as specified in these Protocols will not be adequate to resolve transmission security violations, ERCOT may curtail DC Tie Load to return the ERCOT System to a reliable condition.

(6)	ERCOT shall post the Watch message electronically to the MIS Public Area and shall provide verbal notice to all TSPs and QSEs via the Hotline.  Corrective actions identified by ERCOT must be communicated through Dispatch Instructions to all TSPs, DSPs and QSEs required to implement the corrective action.  Each QSE shall immediately notify the Market Participants that it represents of the Watch.  To minimize the effects on the ERCOT System, each TSP or DSP shall identify and prepare to implement actions, including restoration of transmission lines as appropriate and preparing for Load shedding.  ERCOT may instruct TSPs or DSPs to reconfigure ERCOT System elements as necessary to improve the reliability of the ERCOT System.  On notice of a Watch, each QSE, TSP, and DSP shall prepare for an Emergency Condition in case conditions worsen.  ERCOT may require information from QSEs representing Resources regarding the Resources’ fuel capabilities.  Requests for this type of information shall be for a time period of no more than seven days from the date of the request.  The specific information that may be requested shall be defined in the Operating Guides.  QSEs representing Resources shall provide the requested information in a timely manner, as defined by ERCOT at the time of the request.

- 6.5.9.3.4	Emergency Notice

(1)	Emergency Notice is the fourth of four levels of communication issued by ERCOT when operating in an Emergency Condition.

(2)	ERCOT shall issue an Emergency Notice for one or both of the following reasons:

(a)	ERCOT cannot maintain minimum reliability standards (for reasons including fuel shortages) during the Operating Period using every Resource practicably obtainable from the market; or

(b)	Immediate action cannot be taken to avoid or relieve a Transmission Element operating above its Emergency Rating.

(3)	The actions ERCOT takes during an Emergency Condition depend on the nature and severity of the situation.

(4)	ERCOT is considered to be in an Emergency Condition whenever ERCOT Transmission Grid status is such that a violation of security criteria, as defined in the Operating Guides, presents the threat of uncontrolled separation or cascading Outages and/or large-scale service disruption to Load (other than Load being served from a radial transmission line) and/or overload of a Transmission Element, and no timely solution is obtainable through SCED or CMPs.

(5)	If the Emergency Condition is the result of a transmission problem, ERCOT shall act immediately to return the ERCOT System to a reliable condition, including instructing Resources to change output and instructing TSPs or DSPs to drop Load.

(6)	If the Emergency Condition is the result of an Ancillary Service insufficiency, then ERCOT shall follow the EEA procedures.