| NPRR Number | 830 | NPRR Title | Revision of 4-Coincident Peak Methodology |
| --- | --- | --- | --- |
| Date Posted |  | April 26, 2017 |  |
| Requested Resolution |  | Normal |  |
| Nodal Protocol Sections Requiring Revision |  | 9.17.1, Billing Determinant Data Elements |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) revises the basis of ERCOT’s calculation of the 4-Coincident Peak (4-CP) to a manner consistent with the North American Electric Reliability Corporation (NERC) net energy for Load methodology. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | Current Protocols require that the 4-CP calculation shall use the most recent aggregate data produced by the ERCOT Settlement system. Per paragraph (9) of Section 4.4.4, DC Tie Schedules, ERCOT shall use the Direct Current Tie (DC Tie) Electronic Tag (e-Tag) MW amounts for Settlement. Consequently, scheduled e-Tag MW volumes are used in the 4-CP calculation. As seen in July 2016, variances between scheduled and actual DC Tie flows may be significant enough to change the peak interval. The proposed methodology avoids this issue by using actual metered net DC Tie flows. Back-tested for 2015 and 2016, the revised methodology resulted in an average change of -0.1% with respect to the existing methodology in both years. |  |

| Sponsor |  |
| --- | --- |
| Name | Randy Roberts |
| E-mail Address | rroberts@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6800 |
| Cell Number | NA |
| Market Segment | NA |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Lindsay Butterfield |
| E-Mail Address | Lindsay.Butterfield@ercot.com |
| Phone Number | 512-248-6521 |

Proposed Protocol Language Revision

- 9.17.1	Billing Determinant Data Elements

(1)	ERCOT shall calculate and provide to Market Participants on the Market Information System (MIS) Public Area the following data elements annually to be used by TSPs and DSPs as billing determinants for transmission access service.  This data must be provided by December 1 of each year.  This calculation must be made under the requirements of P.U.C. Subst. R. 25.192, Transmission Service Rates.  ERCOT shall use the most recent aggregate data produced by the ERCOT Settlement system to perform these calculations.

(a)	The 4-Coincident Peak (4-CP) for each DSP and External Load Serving Entity (ELSE), as applicable;

(b)	The ERCOT average 4-CP;

(c)	The average 4-CP for each DSP and ELSE, as applicable, coincident to the ERCOT average 4-CP.

(2)	Average 4-CP is defined as the average Settlement Interval coincidental MW peak occurring during the months of June, July, August, and September.

(3)	Settlement Interval coincidental MW peak is defined as the highest monthly 15-minute MW peak for the entire ERCOT Transmission Grid as calculated per the followingformula:  The sum of all net energy produced by All-Inclusive Generation Resources + Block Load Transfers (BLTs) from ERCOT to another Control Area that have been registered for Settlement purposes + actual Direct Current Tie (DC Tie) imports - BLTs to ERCOT from another Control Area that are not reflected in a Non-Opt-In Entity’s (NOIE’s) Load - actual DC Tie exports - Wholesale Storage Load (WSL).