| PGRR Number | 057 | PGRR Title | Responsibilities for Performing Geomagnetic Disturbance (GMD) Vulnerability Assessments |
| --- | --- | --- | --- |
| Date of Decision |  | June 13, 2017 |  |
| Action |  | Approved |  |
| Timeline |  | Normal |  |
| Effective Date |  | Upon system implementation |  |
| Priority and Rank Assigned |  | Priority – 2017; Rank – 150 |  |
| Guide Sections Requiring Revision |  | 3.1.1.5, Geomagnetic Disturbance (GMD) Vulnerability Assessment (new) 3.1.8, Planning Geomagnetic Disturbance (GMD) Activities (new) 6.11, Process for Developing Geomagnetically-Induced Current (GIC) System Models |  |
| Related Documents Requiring Revision/Revision Requests |  | None |  |
| Revision Description |  | This Planning Guide Revision Request (PGRR) aligns the Planning Guides with North American Electric Reliability Corporation (NERC) Reliability Standard TPL-007-1, Transmission System Planned Performance for Geomagnetic Disturbance Events, by identifying responsibilities for performing studies needed to complete Geomagnetic Disturbance (GMD) vulnerability assessments. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | This PGRR satisfies NERC Reliability Standard TPL-007-1. |  |
| PLWG Decision |  | On 2/22/17, the Planning Working Group (PLWG) was in consensus to recommend approval of PGRR057 as amended by the 2/16/17 ERCOT comments and as revised by PLWG. On 3/22/17, PLWG was in consensus to endorse and forward to ROS the 2/22/17 PLWG Report and the Impact Analysis with a recommended priority of 2017 and a rank of 150. |  |
| Summary of PLWG Discussion |  | On 2/22/17, participants discussed the applicability of various elements of PGRR057 in satisfying NERC Reliability Standard TPL-007-01 and reviewed the 2/16/17 ERCOT comments. On 3/22/17, ERCOT Staff, in response to participant questions, noted the costs to implement PGRR057 will be assigned to the Regulatory category of the Project Priority List (PPL). |  |
| ROS Decision |  | On 4/6/17, ROS voted unanimously to recommend approval of PGRR057 as recommended by PLWG in the 3/22/17 PLWG Report. All Market Segments were present for the vote. |  |
| Summary of ROS Discussion |  | On 4/6/17, participants discussed the components of studies and corrective action plans required by NERC Reliability Standard TPL-007-1. |  |
| TAC Decision |  | On 5/25/17, TAC voted unanimously to recommend approval of PGRR057 as recommended by ROS in the 4/6/17 ROS Report. All Market Segments were present for the vote. |  |
| Summary of TAC Discussion |  | On 5/25/17, participants reviewed the Revised Impact Analysis and Business Case for PGRR057. |  |
| ERCOT Opinion |  | ERCOT supports approval of PGRR057. |  |
| Board Decision |  | On 6/13/17, the ERCOT Board approved PGRR057 as recommended by TAC in the 5/25/17 TAC Report. |  |

| Sponsor |  |
| --- | --- |
| Name | Ben Richardson |
| E-mail Address | ben.richardson@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-4505 |
| Cell Number |  |
| Market Segment | Not Applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Brittney Albracht |
| E-Mail Address | Brittney.Albracht@ercot.com |
| Phone Number | 512-225-7027 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 021617 | Addressed informal comments received by the Planning Geomagnetic Disturbance Task Force (PGDTF) and discussed at the 2/16/17 PGDTF meeting. |
| PRS 041417 | Endorsed the ROS recommended priority of 2017 and rank of 150. |

Market Rules Notes

Please note that baseline Planning Guide language in the following section has been updated due to the incorporation of the following PGRR into the Planning Guide:

- PGRR046, Addition of Geomagnetically Induced Current (GIC) Model Building Requirements (incorporated 5/8/17)

  - Section 6.11

Proposed Guide Language Revision

- 3.1.1.5 	Geomagnetic Disturbance (GMD) Vulnerability Assessment

(1)	The purpose of the Geomagnetic Disturbance (GMD) vulnerability assessment is to provide a coordinated assessment and corrective action plan(s) for the ERCOT System to meet ERCOT and NERC GMD reliability performance criteria for a GMD event.  The most recent Geomagnetically-Induced Current (GIC) system models developed and maintained by ERCOT in conjunction with the TSPs and Resource Entities as described in Section 6.11, Process for Developing Geomagnetically-Induced Current (GIC) System Models, shall be used as the basis for the ERCOT GMD vulnerability assessment.  Projects that are included in the corrective action plan(s) are not considered to have been endorsed by ERCOT until they have undergone the appropriate level of RPG Project Review as outlined in Protocol Section 3.11.4, Regional Planning Group Project Review Process, if required.  The process used by ERCOT to develop the GMD vulnerability assessment is outlined in Section 3.1.8, Planning Geomagnetic Disturbance (GMD) Activities.

- 3.1.8 	Planning Geomagnetic Disturbance (GMD) Activities

(1)	As required by the applicable NERC Reliability Standard, ERCOT shall employ the GIC system models described in Section 6.11, Process for Developing Geomagnetically-Induced Current (GIC) System Models, to perform simulations to identify maximum effective GIC flow in the high side wye-grounded transformers for the worst case geoelectric field orientation for the benchmark GMD event.  ERCOT shall provide the preliminary GIC flow results to the TSPs and Resource Entities for comment before finalizing the results.  Upon consideration of the comments, ERCOT shall make the maximum effective GIC flows in the high side wye-grounded transformers available to TSPs and Resource Entities by posting this data on the ERCOT MIS Secure Area.

(a)	Upon written request from the TSP or Resource Entity who owns a high side wye-grounded transformer within the ERCOT planning area that is included in the ERCOT GIC system models, ERCOT shall perform simulations to make effective GIC time series available no later than 90 calendar days after ERCOT’s receipt of such written requests.

(2)	Each TSP and Resource Entity that owns a high side wye-grounded transformer(s) within the ERCOT planning area shall perform transformer thermal impact assessments as required in the applicable NERC Reliability Standard and shall provide to ERCOT any suggested actions to mitigate the impact of GICs on its transformers.

(3)	ERCOT and the TSPs shall develop for approval by the Technical Advisory Committee (TAC) criteria for acceptable steady-state voltage performance during the benchmark GMD event.

(4)	ERCOT shall perform the ERCOT GMD vulnerability assessment as required in the applicable NERC Reliability Standard.  ERCOT shall provide preliminary GMD vulnerability assessment results to the TSPs and Resource Entities for comment before finalizing the results.  Upon request, ERCOT shall make available to the TSPs the GIC system models and other model information used for the GMD vulnerability assessment, including suggested actions described in paragraph (2) of Section 3.1.8.

(5)	ERCOT and the TSPs shall develop and document corrective action plan(s) as required in the applicable NERC Reliability Standard.

(6)	ERCOT shall post the GMD vulnerability assessment report and corrective action plan(s) on the ERCOT MIS Secure Area.

- 6.11	Process for Developing Geomagnetically-Induced Current (GIC) System Models

(1)	To adequately simulate Geomagnetic Disturbance (GMD) events, it is necessary to establish and maintain GIC system models and conduct geomagnetic disturbance vulnerability assessments to determine whether the ERCOT System can withstand performance requirements of the benchmark geomagnetic disturbance event described in North American Electric Reliability Corporation (NERC) Reliability Standards.  These models, known as GIC base cases, shall contain appropriate system data, and shall represent projected system conditions that provide a starting point for the required year(s).

(a)	Transmission Service Providers (TSPs) and ERCOT shall develop the GIC base cases.  These base cases are derived from the steady-state base cases developed by Steady-State Working Group (SSWG) for the near-term transmission planning horizon to ensure consistency between the system topology in the SSWG base cases and GIC system models.

(b)	Each TSP, or its Designated Agent, shall provide its respective transmission network GIC model data in accordance with the GIC System Model Procedure Manual.

(c)	Each Resource Entity, or its Designated Agent, shall provide its respective Resource Entity-owned generating units, plants, transmission lines, shunt devices and Generation Step Ups (GSUs) connected to the ERCOT System in accordance with the GIC System Model Procedure Manual and the Resource Registration Glossary.

(d)	ERCOT shall aggregate the GIC system model data supplied by each TSP and Resource Entity and shall compile and maintain the GIC system models.  Upon completion of compiling the data for the GIC base cases, ERCOT shall post these cases on the ERCOT Market Information System (MIS) Certified Transmission Service Provider Information page for review and comments as described in the GIC System Model Procedure Manual.

(e)	Guidelines and formats for the GIC system model data and model maintenance can be found in the GIC System Model Procedure Manual.

(f)	GIC data is considered Protected Information pursuant to Protocol Section 1.3, Confidentiality.

(2)       Each TSP and Resource Entity shall provide to ERCOT a list of GMD event contingencies as described in the applicable NERC Reliability Standard for use in the GMD vulnerability assessment as outlined in Section 3.1.8.