Procedure for Identifying Resource Nodes

Effective Date: TBD

Introduction:

This procedure is the guiding document for ERCOT and Market Participants with Generation Resources, to identify Resource Nodes and manage the lifecycle of the Resource Node.

Revisions to this document shall be approved by the Wholesale Market Subcommittee (WMS).  Upon approval of revisions, ERCOT shall post the revised procedure to the ERCOT website within three Business Days.

Procedure to Incorporate a Resource Node into the Network Operations Model:

- At the designated time period as determined by Protocol Section 3.10, Network Operations Modeling and Telemetry, and associated ERCOT business processes, a Resource Entity must submit a Resource Asset Registration Form (RARF) that includes a detailed electrical one-line drawing of the generation facility. The ERCOT Business process indicates that the RARF will be presented to the Network Modeling Group (NMG) within ERCOT.

- The Network Modeling Group will utilize the “Principles for Resource Node Definition” located in Appendix A to determine the Resource Node parameters.

- The NMG will provide documentation back to the RE clearly indicating the Resource Node parameters.

- The Resource Entity will have five (5) Business Days to accept or reject the suggested Resource Node parameters.

- If there are any disagreements with the Resource Node parameters, ERCOT and the Resource Entity shall work together to reach consensus.

- If consensus cannot be reached by ERCOT and the Resource Entity, the Wholesale Market Subcommittee (WMS) or appropriate WMS working group shall help guide the decision.

- Upon a consensus between ERCOT and the Resource Entity, the Resource Node parameters will be implemented in the Network Operations Model.

- The normal timeline for this procedure shall not exceed 20 Business Days after the submission date of a validated RARF submittal that includes a detailed electrical one-line drawing.

- In the event that consensus between ERCOT and the Resource Entity cannot be reached within 20 Business Days, no Resource Node parameters will be entered into the Network Operations Model. This may have an effect on Congestion Revenue Right (CRR) Network Models and associated CRR activities regarding the Generation Resource in question. There must be a consensus between ERCOT and the Resource Entity before Resource Node parameters will be implemented into the Network Operations Model.

- Once effective in the Network Operations Model, the Resource Node name cannot be changed.

- Once incorporated into the Network Operations Model, the Resource Node will be used in all associated ERCOT models, applications, and processes.

- The Resource Node parameters, associated electrical one-line drawings, and other relevant data shall be posted on the Market Information System (MIS) Secure Area and will be available to Market Participants with Digital Certificates.

Procedure to Retire a Resource Node in the Network Operations Model:

- Resource Nodes cannot be retired until all outstanding CRRs on that point have been settled or converted to a nearby Resource Node; TSPs cannot submit Network Operations Model Change Requests (NOMCRs) to delete a Resource Node.

- ERCOT’s CRR team will submit a NOMCR with the appropriate effective date once it has been notified that equipment tied to a Resource Node is requested to be removed from the Network Operations Model. The effective date is determined based on the last active CRR date.

- ERCOT’s Day-Ahead Market (DAM) team will prepare and post a Market Notice concerning the retirement of the Resource Node.

# Appendix A

PRINCIPLES FOR RESOURCE NODE DEFINITION

- Network Operations Model

  - Annual/Monthly CRR Auctions use network model as close as possible to the Network Operations Model.

- Market Management System (MMS) and Energy Management System (EMS) use the same Network Operations Model for both commercial and operational purposes.

- Breakers between the Connectivity Nodes and the Resource Node are closed as default so that Resource Nodes and associated Connectivity Nodes appear energized.

- Transmission Element Outages, as defined in the Protocols, are submitted into Outage Scheduler and posted before DAM submission, i.e. de-energized Resource Nodes (Settlement Points) are known in advance of DAM submission.

- Connectivity Nodes

  - Connectivity Node represents the Electrical Bus where physical generator is connected.

  - Generator power output is injected at the Connectivity Node.

  - More than one Resource can be connected to the same Connectivity Node.

- Resource Nodes

  - Resource Node Definition

  - Resource Node represents the Electrical Bus or the logical construct that defines the location of a Settlement Point.

  - Resource Nodes include Generation Resource Nodes, Combined Cycle Plant (CCP) Logical Resource Nodes, Combined Cycle Unit (CCU) Resource Nodes and Private Use Network (PUN) Resource Nodes.

  - Generation Resource Node represents the Settlement Point for ERCOT and PUN Generation Resources. The Three-Part Supply Offers, DAM Energy Only offers, Ancillary Service Offers and DAM energy bids as well as Point-to-Point (PTP) bids can be submitted and settled at a Generation Resource Node.

  - CCP Logical Resource Node represents the Settlement Point for Three-Part Supply Offers for CCP configurations. Only Three-Part Supply Offers, and Ancillary Service Offers for CCP configurations can be submitted and settled at a CCP Logical Resource Node.

  - CCU Resource Node represents the Settlement Point for the CCU. Only DAM Energy Only offers, DAM energy bids and PTP bids can be submitted and settled at a CCU Resource Node.

  - PUN Resource Node represents the Settlement Point at the PUN interconnection to ERCOT. Only DAM Energy Only offers, DAM energy bids and PTP bids can be submitted and settled at a PUN Resource Node.

  - Multiple Generation Resources can be mapped to the same Resource Node, i.e. offers from different Generation Resources can be settled at the same Settlement Point.

  - Generation Resource can be mapped only to one Resource Node, i.e. offers from the Generation Resources can be settled only at one Settlement Price.

  - The Resource Nodes for “single” Resources and for Resources that are a component of a Combined Cycle Plant shall be identified prior to the identification of the PUN Resource Nodes. Once those Resource Nodes have been located, the PUN Resource Nodes shall be located for PUN Resources that are not co-located with an existing Resource Node.

  - Resource Nodes shall not be located at the DC-ties. (The DC Ties are Load Zones.)

  - Resource Nodes shall not be located at the Block Load Transfer (BLT) buses.

  - Do not identify or locate Resource Nodes for Non-Modeled Generators.

  - Resource Node Location

    - First Fork Rule: Locate Resource Node at the first Electrical Bus with alternate paths starting from Generator Connectivity Node. Parallel network paths do not count as alternate paths.

      - Exception: There is an exception to this rule for placing Generation Resource Nodes and CCU Resource Nodes that are mapped to Generation Resources within a PUN. If the Generation Resource(s) is within a PUN that has only one interconnection to the ERCOT Transmission Grid, locate the Resource Node at the Electrical Bus that is the interconnection point of the PUN to the ERCOT Transmission Grid.

      - EPS Meter location check: As the network connectivity path is traversed in searching for the first Electrical Bus with alternate paths (First Fork Rule), if an Electrical Bus is encountered with a mapped EPS Meter first, then place the Resource Node at this Electrical Bus.

    - ERCOT-Polled Settlement (EPS) Meter Rule: Locate Resource Node, subject to First Fork Rule, as close as possible to EPS Meter location, i.e. where energy is effectively metered. If the EPS Meter location changes, then a new Resource Node will need to be established and the old Resource Node retired in accordance with the procedure in this document.

    - Ownership Rule: Locate Resource Node at the Electrical Bus that essentially is the ERCOT interconnection point (if practical). Subsequent ownership changes shall not change the Resource Node location.

    - De-Energization Rule: Locate Resource Node at Electrical Bus that is less often de-energized, if alternate choices exist. Settlement Point Prices (SPPs) for de-energized Resource Nodes are calculated using heuristic rules.

    - Generic Transmission Constraint (GTC) Rule: A GTC cannot include Transmission Elements between a Resource Node and any Generation Resources mapped to it.

    - Transmission Constraint Rule: Initial placement of the Resource should not be such that Transmission Elements between Resource Node and associated Connectivity Nodes could be constrained. The parameters of the Network Model are evaluated at that point in time when the determination of the Resource Node placement is being made such that there is no congestion between the location of the Resource Node and the connectivity node that the Generation Resource is physically connected to in the Network Model. Ongoing monitoring to ensure that there is no congestion between the Resource Node and the connectivity node of the Generation Resource requires the Resource Entity and TDSP to monitor and coordinate changes that may impact this. See Articles 5, 6 and 7 of the Standard Generation Interconnection Agreement (SGIA).

    - Publicity Rule: Market Participants need to know where the Resource Nodes are located.

    - In the event of a subsequent NOMCR that changes the topology, ERCOT shall review the impact to the Resource Node location.

      - In cases where a NOMCR that is to be effective in the future requires the placement of a new Resource Node, there may be instances where the Common Information Model (CIM) may show both the current and the future topology with the new Resource Node. This is done to handle situations where the energization date/time of the future network changes are different than the date/time of the migration of the changes in the network model into the ERCOT production systems. In such cases:

        - The location of the new Resource Node will be based on the future topology only

        - The transition of the mapping between Generation Resource and the new Resource Node (if applicable) will be performed by ERCOT support staff

If all rules cannot be simultaneously satisfied, then the rules are listed in order of priority; ERCOT will use discretion in choosing the appropriate location, assuming that such a location does not allow the Resource Entity to control its Resource Node price.

- Combined Cycle Plant (CCP) Modeling

  - CCP Logical Resource Node

- Each CCP Configuration for a train represents a CCP Logical Generation Resource.

- Each CCP Logical Generation Resource is mapped to a CCP Logical Resource Node. All CCP Logical Generation Resources, i.e. all CCP Configurations for a train are mapped to the same CCP Logical Resource Node.

- Each CCP train has its own CCP Logical Resource Node, i.e. CCP Logical Generation Resources for different CCP trains are mapped to different CCP Logical Resource Nodes.

- Each CCP Logical Resource Node is a Settlement Point.

- CCP Logical Resource Nodes are used only for Resource specific Three-Part Supply Offer’s and Ancillary Service Offers for CCP configurations.

  - CCU Resource Node

- Combined Cycle Unit (CCU) Resource Nodes are mapped to a CCP Logical Resource Node.

- A CCU Resource Node is the Electrical Bus determined by above rules (First Fork and others as described in 3.2 above) starting from the Connectivity Node of the physical CCP train Resources.

- A CCU Resource Node is a Settlement Point.

- Only DAM Energy Only offers, DAM energy bids and PTP bids can be submitted at CCU Resource Nodes.

  - CCP/CCU Resource Node Processing

- PTP cleared quantities are injected at Electrical Buses of CCU Resource Nodes.

- DAM SPP for CCU Resource Node is used as Settlement Price for PTP bids that sink or source at CCU Resource Node.

- In DAM, energy for CCP Logical Resource is distributed to Connectivity Nodes of physical CCP Resources proportionally to the Resource capacities that are online in selected CCP configuration.

- In DAM, Shift Factor for CCP Logical Resource Node is calculated as capacity weighted average of Shift Factors for CCU Resource Nodes using the Resource capacities that are online in the selected CCP configuration as weights. Note that the assumption here is that there is no congestion between the connectivity node of the Combined Cycle Unit (CCU) and the Resource Node.

- DAM SPP for CCP Logical Resource Node is equal to weighted average of DAM SPPs at CCU Resource Nodes using the Resource capacities that are online in selected CCP configuration as weights.

- DAM SPP for CCP Logical Resource Node is used as Settlement Price for CCP Three-Part Supply Offer’s.

- In RTM, Shift Factor for CCP Logical Resource Node is calculated as weighted average of Shift Factors for CCU Resource Nodes using the telemetered outputs of CCU Resources that are online in current CCP configuration as weights. Note that the assumption here is that there is no congestion between the connectivity node of the Combined Cycle Unit (CCU) and the Resource Node.

- RTM LMP for CCP Logical Resource Node is calculated as weighted average of LMPs at CCU Resource Nodes using telemetered outputs of CCU Resources that are online in current CCP configuration as weights. Note that the assumption here is that there is no congestion between the connectivity node of the Combined Cycle Unit (CCU) and the Resource Node.

- RTM SPP for the CCP Logical Resource Node is the Base Point or time weighted average of RTM LMPs at Logical Resource Node.

- Private Use Network (PUN) Modeling

  - PUN Resource Node

- The placement of a PUN Resource Node is optional. At a PUN, after all the Generation Resource Nodes, CCP Logical Resource Nodes and CCU Resource Nodes are placed (if applicable), if none of the Generation Resource Nodes or CCU Resource Nodes are placed where the EPS Meter is effectively located, then this is the location of the PUN Resource Node.

- PUN Resource Node represents the Electrical Bus where an EPS Meter is effectively located that is measuring the flow at a point of interconnection with ERCOT.

- PUN Resource Node is a Settlement Point.

- PUN Resource Node cannot have mapped PUN Generation Resources.

- There can be several PUN Resource Nodes for one PUN.

- Only PTP and DAM Energy Bids and Energy Only Offers can be submitted at PUN Resource Node.

- For DAM Energy Only Offers, power is injected at the Electrical Bus of the PUN Resource Node.

- Cleared quantities are settled at PUN Resource Node Settlement Prices.

  - Resource Nodes for PUN Generation Resource

  - Connectivity Node for PUN Generation Resource represents the Electrical Bus where physical Resource is connected.

  - Generator power outputs are injected at Connectivity Nodes.

  - Resource Node for PUN Generation Resource represents the Electrical Bus where Settlement Point for PUN Generation Resource is located.

  - Resource Node for PUN Generation Resource is defined using First Fork Rule and others as described in 3.2 above.

  - A Resource Node for a PUN Generation Resource is a Settlement Point.

  - PUN energy offers represent net in respect to PUN self-served load.

  - Three-Part Supply Offer and Ancillary Service Offers can be submitted for PUN Generation Resource for the excess capacity and energy not used to serve the PUN self-serve Load.

  - DAM Resource-specific Offers for PUN Generation Resources are settled at SPPs at Resource Nodes for PUN Generation Resources.

  - Constraints within in a PUN can be monitored but will not be enforced by DAM, RUC and SCED.

  - CCP Modeling within a PUN

  - CCP trains within a PUN are treated in the same way as any CCP within ERCOT.

- Settlement Points

- Settlement Point is a Resource Node, Load Zone or Hub.

- Resource Nodes include Generation Resource Nodes, CCP Logical Resource Nodes, CCU Resource Nodes and PUN Resource Nodes.

- Generation Resource Nodes within ERCOT as well as within PUN are Settlement Points.

- DAM Clearing and Settlements

- PTP bids can be submitted using any Settlement Point (except CCP Logical Resource Nodes) as a source and sink.

- Annual/Monthly CRRs acquired at de-energized Settlement Points will not be considered by SFT function.

- DAM Energy Only Offers can be submitted at any Settlement Point (except CCP Logical Resource Nodes).

- DAM Resource-specific energy offers that are submitted are mapped to a Generation Resource Node or a CCP Logical Resource Node only.

- DAM Energy Bids can be submitted at Load Zones, Hubs, Generation Resource Nodes, CCU Resource Nodes and PUN Resource Nodes, i.e. at any Settlement Point except CCP Logical Resource Nodes.

- DAM/SASM Ancillary Service Offers are Generation/Load Resource-specific, not Settlement Point-specific.

- DAM scheduling determines hourly quantities for PTP, energy and Ancillary Service Offers and bids.

- DAM pricing determines hourly LMPs for all Settlement Points.

- DAM Settlements is based on DAM quantities and DAM SPPs.

- RTM Clearing and Settlements

  - SCED dispatch determines Base Points for Generation Resources.

  - SCED pricing determines LMPs for all Generation Resource Nodes, CCP Logical Resource Nodes, CCU Resource Nodes, PUN Resource Nodes and all EPS Meter locations.

  - RTM determines 15-minute SPPs for each Settlement Point and each EPS Meter location. These prices are the Base Point weighted and time weighted average of the Real-Time LMPs.

  - RTM Settlements uses 15-minute RTM SPPs (prices at Settlement Points) and Settlement Prices (prices at EPS Meter locations).

  - RTM Energy Settlement for the measured output from the Generation Resources uses the prices at the EPS Meter locations as specified in Protocol Section 6.6.3, Real-Time Energy Charges and Payments.

- Summary of Allowed Activities

|  | ACTIVITIES |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Settlement Points | Three-Part Supply Offer | Ancillary Service Offer | DAM Energy Only Offers | DAM Energy Bid | PTP bids | QSE to QSE Transaction |
| Resource Node | Yes | Yes | Yes | Yes | Yes | Yes |
| CCU Resource Node | No | No | Yes | Yes | Yes | Yes |
| PUN Resource Node | No | No | Yes | Yes | Yes | Yes |
| CCP Logical Resource Node | Yes | Yes | No | No | No | No |

Note that Resource Specific offers (Three-Part Supply Offer and Ancillary Service Offers) are made for the Resource and the submittal does NOT specify a Resource Node.