# ERCOT Concept Paper for

# Real-Time Co-Optimization of Energy and Ancillary Services

DRAFT version 0.1

June 28, 2017

# Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 06/28/2014 | 0.1 | Initial Working Draft | ERCOT Team |

Table of Contents

1.	Executive Summary	4

2.	RT energy and AS Co-optimization	5

2.1.	Ancillary Service Product Set	5

2.2.	Setup of AS Demand Curves (ASDC) under Real-Time Co-optimization	6

2.3.	Co-ordination of the Power Balance Penalty Curve, Maximum value of ORDC, and Value Of Lost Load (VOLL)	11

2.4.	Settlements	13

2.4.1.	Are There any Make-Whole Payments to Resources?	14

2.4.2.	Is There Any Uplift Required?	14

2.5.	RUC/SASM Changes (2017 SAWG):	14

2.6.	Locational Reserves	15

2.7.	AS Deliverability	15

2.8.	High Level Description of the Clearing Process and Outputs	15

2.8.1.	Pricing Run Changes (modifications to NPRR 626)	17

2.9.	Telemetry Changes for Generation and Controllable Load Resources	18

2.10.	Telemetry Changes for Load Resources with UFR	18

2.11.	AS Deployment Process	19

2.12.	Discussion Items:	21

3.	Appendix 1: High Level Mathematical Formulation of energy and AS Co-Optimization for Option 2	26

# Executive Summary

ERCOT stakeholders have been tasked with studying the implications of adding Real-Time Co-optimization (RTC) to the wholesale market.

Since nodal go-live, the procurement of ancillary services (AS, or operating reserves) has been co-optimized with the procurement of energy in the day-ahead market (DAM). Capacity from resources selected in the DAM to provide Responsive Reserves, Regulating Reserves or Offline Non-Spinning Reserves is set aside and unavailable for economic energy dispatch in the real-time market (RTM) under normal operating conditions (Non-Spinning Reserves provided from capacity of online resources is available for economic dispatch in the RTM, subject to an energy offer floor of $75/MWh).

A market design change, which was implemented on June 1, 2014, represents an approximation of a Real-Time Co-optimization based on the Operating Reserve Demand Curve (ORDC). The existing ORDC approximates the shortage pricing of operating reserves that would be accomplished with RTC by applying a price “adder” outside of the execution of the Security Constrained Economic Dispatch (SCED) engine.  However, ORDC does not distinguish between the type and quality of online operating reserves, and does not achieve the efficient coordination of the provision of energy and operating reserves by Resources.

While RTC is a common feature in other wholesale electricity markets, the provision of energy and reserves is not co-optimized in the current ERCOT real-time market.

Real-Time Co-optimization is the process of simultaneously procuring energy and Ancillary Services (AS) from available Resources, at the lowest production cost to meet the Real-Time system demand for energy and AS. This results in the optimal allocation of all Resources’ capacity between energy and AS. However, the current Real-Time (RT) market is unable to consider capacity reserved for AS (e.g., capacity above the Resource’s High Ancillary Services Limit, or HASL) even if the energy offer or bid for that capacity would be economical.  Using Real-Time Co-optimization, the Real-Time Market clearing process would consider all available capacity to serve system demand for energy and procure AS capacity at the lowest possible cost1.  Market clearing is designed to normally occur every 5 minutes. The process under consideration would be similar to the clearing process for energy and AS in the DAM.  Accordingly, Real-Time Co-optimization can be thought of as “running the DAM every five minutes in Real-Time.”   The objectives of Real-Time Co-optimization are to enable appropriate scarcity pricing through optimal use of a Resource’s capacity for energy and AS, and to enable market participants to adjust their energy and AS portfolios in Real-Time.

ERCOT Staff has developed this concept paper as a starting point for stakeholder consideration of these Real-Time Market improvements.

# RT energy and AS Co-optimization

Co-optimization is the process of simultaneously procuring energy and Ancillary Services (AS) at the lowest production cost  while meeting system demand for energy and AS. The result is optimal allocation of all Resources’ capacity between energy and AS.

In other words, the energy and AS Co-optimization clearing process ensures that, while maintaining the lowest cost for procuring the required MWs, the pricing outcomes for energy and AS — Locational Marginal Prices, or LMPs, and AS Market Clearing Prices for AS Capacity, or MCPCs), are such that, the Resources are provided the best possible total revenue outcome from the energy and AS awards.

The objectives of Real-Time Co-optimization are to enable appropriate scarcity pricing through optimal use of Resource’s capacity for energy and AS, and to enable market participants to adjust their energy and AS portfolios in Real-Time.

## Ancillary Service Product Set

This concept document presents two options.

Option 1:

This option, keeps the current set of AS products — namely, Regulation Up (Reg-Up), Fast Responding Regulation Service-Up (FRRS-Up), Regulation Down (Reg-Down), Fast Responding Regulation Service-Down (FRRS-Down), Responsive Reserve Service (RRS) and Non-Spin.

The AS product set for Option 1 (no change from current AS product set) :

- Reg-Up,

- Reg-Down,

- RRS,

- Non-Spin (can be provided by qualified On-Line and Off-Line Resources)

Option 2:

This option proposes a change from the current set of AS products. The change is to replace the Non-Spin AS product with two products – Spinning Operating Reserve (SOR) and Non-Spinning Operating Reserve (NSOR) with different prices (MCPC). The other products (Regulation, Responsive Reserve) remain the same.

The AS product set for Option 2:

- Reg-Up,

- Reg-Down,

- RRS,

- Spinning Operating Reserve (SOR): provided by On-Line Resources that can convert capacity to energy in 30 minutes – same requirement as current Non-Spin

- Non-Spin Operating Reserves (NSOR): provided by Off-Line Resources that can convert capacity to energy in 30 minutes – same requirement as current Non-Spin

Spinning Operating Reserve (SOR): This is the available On-Line headroom minus the Regulation Up and RRS awards/responsibility. The qualification criteria proposed are the same as what is currently in place for Resources to be eligible to receive the On-Line Reserve Price Adder. The MCPC for SOR is the sum of the shadow prices of the SOR and NSOR reserve capacity procurement constraint. This capacity can be converted to energy in 30 minutes (same requirements as current Non-Spin)

Should On-Line capacity that cannot be converted to energy in 30 minutes (some duct burner capacity and Resources with low ramp rates) be eligible for SOR? ERCOT Operations input required.

Non-Spinning Operating Reserve (NSOR): This is the available Off-Line capacity that can be converted to energy in 30 minutes. The qualification criteria proposed are the same as what is currently in place for Resources to be eligible to receive the Off-Line Reserve Price Adder. The MCPC for NSOR is the shadow price of only the NSOR reserve capacity procurement constraint.

The high level mathematical formulations for Option 2 is presented in the appendix. The mathematical formulation for Option 1 can be derived from these equations.

The discussion in this concept paper does not contemplate changing the current AS offer structure where the offered AS capacity MW has slots for offer prices for each of the AS products. This AS offer structure allows the market clearing engine (DAM or Real-Time Co-Optimization) to determine the optimal allocation of the offered MW among the various AS products.

## Setup of AS Demand Curves (ASDC) under Real-Time Co-optimization

Under RT Co-optimization of energy and AS, the AS requirements for each type of AS (e.g. Reg-Up, Reg-Down, RRS, Non-Spin, SOR, NSOR) to be procured are modeled as a demand curve. The AS demand curves for AS serve the same purpose as the Power Balance Penalty Curve for energy, i.e. the AS demand curve will set the price (MCPC) under the respective AS shortage conditions.

Two approaches are currently being discussed. One approach is to develop a process to disaggregate the ORDC into the individual ASDCs. In this approach, any change to the ORDC (changing minimum contingency level, shifting of mean or standard deviation) will get reflected in the individual ASDC. Note that with this approach, aggregating the individual ASDCs will reproduce the relevant ORDC.

The second approach is not to use the ORDC as reference to develop a process to setup the individual ASDCs for Reg-Up, Reg-Down, and RRS. The ASDCs for Non-Spin (Option 1) or SOR and NSOR (Option 2) could be based on some portion of the ORDC.

Three examples of these approaches are presented.

Example 1: Disaggregation of the ORDC into Reg-Up, RRS, Non-Spin Demand Curves (Option 1)

The Operating Reserve Demand Curve (ORDC), which is based on statistical distributions (mean and standard deviation) of Online Reserves will be used to setup the AS demand curves for each AS type.

For Non-Spin, the demand curve continues on until the price on the ORDC is zero (0 $/MW) — which is currently around 7,000 MW of total reserve.

AS Plan MW Requirements (for Reg-Up and RRS) are used to disaggregate the ORDC as shown in the figure below.

Figure 1: Example 1: Disaggregation of the ORDC into Reg-Up, RRS, Non-Spin Demand Curves (Option1)

MW Reserves

Minimum Contingency X=2000 MW

$/MW

$/MW

MW Reserves

Reg-Up Demand Curve

RRS Demand Curve

NSpin Demand Curve

ORDC – Spinning Reserves Reserves

ORDC – Combined Spinning & Non-Spinning Reserves

Minimum Contingency X=2000 MW

Example 2: Disaggregation of the ORDC into Reg-Up, RRS, SOR and NSOR Demand Curves (Option 2)

The approach used to setup the ASDC is similar to Example 1. The difference is with the AS product set (Option 2) and the manner by which the ORDC is disaggregated into the individual ASDCs.

The Operating Reserve Demand Curve (ORDC), which is based on statistical distributions (mean and standard deviation) of Online Reserves will be used to setup the AS demand curves for each AS type.

For SOR and NSOR, the demand curves continues on until the price on the ORDC is zero (0 $/MW) — which is currently around 7,000 MW of total reserve.

AS Plan MW Requirements are used to disaggregate the ORDC as shown in the figure below.

Figure 2: Example 2: Disaggregation of the ORDC into Reg-Up, RRS, SOR and NSOR Demand Curves (Option2)

MW Reserves

$/MW

$/MW

MW Reserves

Reg-Up Demand Curve

RRS Demand Curve

SOR Demand Curve

ORDC – Spinning Reserves Reserves

ORDC – Combined Spinning & Non-Spinning Reserves

NSOR Demand Curve

$/MW

Reg-Up Demand Curve

MW Reserves

$/MW

RRS Demand Curve

MW Reserves

Minimum Contingency X=2000 MW

Example 3: ASDC for Reg-Up, RRS, SOR and NSOR Demand Curves (Option 2) – Not based on ORDC

In this example, Reg-Up, Reg-Down, RRS are not derived from the ORDC. SOR and NSOR are based on the ORDDC with minimum contingency removed.

The Regulation Up and Responsive Reserve Demand Curve are rectangles as shown below and in aggregate may exceed the minimum contingency reserve of the ORDC.

Figure 3: AS Demand Curves where only SOR and NSOR are derived from the ORDC (Option 2)

Reg-Up Demand Curve

$/MW

$/MW

RRS Demand Curve

MW

MW

NSOR Demand Curve from ORDC Combined Spinning & Non-Spinning Reserves with Minimum Contingency (X) removed

$/MW

$/MW

MW

SOR Demand Curve from ORDC Spinning Reserves with Minimum Contingency (X) removed

MW

The sum of the maximum prices of the two demand curves for SOR and NSOR is equal to the original ORDC Spinning Reserve Demand curve with Minimum Contingency (X) removed. e.g. 50% of each of the curves as is currently done for the ORDC price

## Co-ordination of the Power Balance Penalty Curve, Maximum value of ORDC, and Value Of Lost Load (VOLL)

In scarcity conditions, the AS demand curves sets the AS MCPC, similar to how the Power Balance Penalty Curve sets LMPs in the energy market under scarcity conditions.

The design of a Real-Time energy and AS Co-optimization is such that the market clearing will ensure that serving the inelastic system demand for energy (GTBD) is given priority over reserving capacity for AS. This means that the relationship between the prices for energy (LMP) and AS (MCPC) are such that the awards (Base Points) to serve energy will be prioritized over awards for AS.

This will be achieved by setting the maximum value ($/MW/h) on the AS demand curves (one curve for each AS type) and coordinating these values with the maximum value of the Power Balance Penalty Curve for energy. AS demand curves will be based on the ORDC.

This co-ordination ensures that in the worst case scenario, the minimum excess revenue a supplier can receive from energy sales (LMP minus EOC) is greater than the maximum excess revenue the supplier could receive from the sale of AS (MCPC minus AS Offer).

Excess revenue per MWh of energy award, over and above the submitted EOC from the sale of energy, is (in $/MWh)

LMP – EOC

Excess revenue per MW of AS award, over and above the submitted AS Offer from the sale of AS, is (in $/MW/h)

MCPC – AS Offer

Thus, in a worst case scenario, a supplier’s minimum excess revenue per MWh of energy sales occurs when the Power Balance Penalty curve sets the energy price (LMP) and the supplier has submitted its EOC at System Wide Offer Cap or SWOC, as follows:

Minimum Excess Revenue per MWh of energy award =

Maximum value of Power Balance Penalty Curve minus SWOC =

VOLL+1 – SWOC

Similarly, in a worst case scenario, a supplier’s maximum excess revenue per MW/h of AS sales occurs when the AS demand curve sets the AS price (MCPC) and the supplier has submitted its AS Offer at 0 $/MW/h, as follows:

Maximum Excess Revenue per MW/h of AS award =

Maximum value of ORDC minus AS Offer of 0$/MW/h =

Maximum value of ORDC

Therefore, the design parameters should be such that the maximum excess revenue per MW/h of AS award is less than minimum excess revenue per MWh of energy award, as follows:

Maximum Value of ORDC < VOLL+1-SWOC

The table below shows five different examples of these values (VOLL, SWOC and maximum value on the PBPC), with Examples 1-4 meeting the coordination criteria, and Example 5 not being properly coordinated.

|  | Example 1 | Example 2 | Example 3 | Example 4 | Example 5 |
| --- | --- | --- | --- | --- | --- |
| VOLL | $9,000 | $18,000 | $9,000 | $9,000 | $18,000 |
| SWOC | $9,000 | $9,000 | $2,000 | $4,500 | $9,000 |
| PBPC | $9,001 | $18,001 | $9,001 | $9,001 | $18,001 |
| Max ASDC | $0 | $9,000 | $7,000 | $4,500 | $18,000 |
| Comments | Coordinated Unworkable because Max ASDC is $0/MWh and unable to effectively establish reserve shortage pricing. | Coordinated Energy prices at $18,000/MWh can occur due to ramp rate exhaustion or when marginal energy offers are at SWOC. More likely to achieve maximum shortage pricing in the range of $9,000 to $10,000 if marginal energy offers range from $0 to $1,000. | Coordinated More likely than other examples to achieve energy prices close to VOLL during acute reserve shortage conditions. SWOC of $2,000 is within the range of observed submitted maximum energy offers. | Coordinated If marginal energy offers are $2,000/MWh or less, energy prices at VOLL will occur only on rare occasions of ramp rate exhaustion. More likely to achieve maximum shortage prices for energy in the range of $4,500 to $6,500 than VOLL. | Not Coordinated Unworkable because Ancillary Services are given higher priority than ensuring that supply is equal to demand. |

As shown in the table above, Example 1 includes a VOLL of $9,000/MWh and a SWOC of $9,000.  Each of these is consistent with existing Commission rules and/or prior decisions.  However, to meet the RTC coordination criteria, the maximum ASDC value for Example 1 is $0/MWh, which is unworkable because such a value is unable to effectively establish reserve shortage pricing.  Example 2 addresses the flaw in Example 1 by raising the VOLL to $18,000/MWh, which allows the maximum ASDC value to rise to $9,000.  However, this change presents the possibility of energy prices of $18,000/MWh.  Example 3 and 4 maintain a VOLL of $9,000/MWh, but with the SWOC changed to $2,000 and $4,500/MWh, respectively.  If marginal energy offers are $2,000/MWh or less, Example 3 is more likely than Example 4 to achieve energy prices close to VOLL during acute reserve shortage conditions.  Finally, Example 5 is a case that is not properly coordinated, and therefore unworkable because Ancillary Services are given higher priority than ensuring that supply is equal to demand.

These examples show that to achieve a coordinated and workable result with the implementation of RTC, decisions related to the appropriate VOLL and/or SWOC are required by the Commission (i.e., maintaining the current SWOC = VOLL = $9,000/MWh and proper coordination is unworkable, as shown in Example 1).  Example 2 would involve a value of VOLL that is different than the value previously adopted by the Commission in Project No. 40000 in 2013.  Example 3 and 4 (or another coordinated variation) would maintain a VOLL of $9,000, but would require a change to the offer caps in P.U.C. Subst. R. 25.505 (at least as applicable to the real-time market, and depending on whether the day-ahead market is modified to incorporate ASDCs with the implementation of RTC).

## Settlements

In general, the current approach described in the ERCOT Nodal Protocols for AS imbalance settlements for ORDC will be employed for the settlement of AS in Real-Time. The changes from the current approach will be that each AS type will have its own AS imbalance settlement. Each AS type will have a 15 minute MCPC that is computed as a time-weighted average of the individual RT Market (i.e., 5 minute) AS MCPCs.

There will be checks on comparing the 5 minute average of the integrated HSL with the sum of the energy Base Points and AS Awards to ensure that Resources are providing sufficient capacity between consecutive SCED executions.

The 15 minute Settlement Interval AS MCPC is calculated as:

RTASMCPC =  (WF  y * RTASSP y)

The AS imbalance settlement for a given AS type is calculated as:

ASImbalanceAmount ($) = 0.25*(DAASAwd – RTASAvgAwd)*RTASMCPC

RTASAvgAwd = (WF  y * RTASAwd y)

WF y = TLMP y / TLMP y

| Variable | Unit | Description |
| --- | --- | --- |
| RTASMCPC | $/MW per hour | Real-Time AS Market Clearing Price for Capacity The Real-Time AS MCPC for the 15-minute Settlement Interval. |
| RTASSP y | $/MW per hour | Real-Time AS Shadow Price The Real-Time AS constraint Shadow Price for the SCED interval y. |
| DAASAwd | MW per hour | Day-Ahead AS MW Award |
| RTASAvgAwd | MW | Real-Time Time Weighted Average AS MW Award The Real-Time Time Weighted Average MW AS Award for the 15-minute Settlement Interval. |
| RTASAwd | MW | Real-Time AS MW Award The Real-Time AS MW Award for the SCED interval y. |
| WF y | none | Weighting Factor per intervalThe time weight used for the portion of the SCED interval y within the Settlement Interval in calculating a given Settlement Interval AS MCPC. |
| TLMPy | second | Duration of SCED interval per intervalThe duration of the portion of the SCED interval y within the Settlement Interval. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |

### Are There any Make-Whole Payments to Resources?

There are no Make-Whole Payments to Resources that can be directly attributed to the conceptual market design changes presented here. Make-Whole provisions to Resources that exist in the current market design may still apply (e.g. Emergency Base Point Settlements).

### Is There Any Uplift Required?

No changes from current market.

Just as occurs in the current Real-Time AS Imbalance Settlement process, as implemented by NPRR 568, etc., any occurrence of uplift to load on a Load-Ratio-Share basis arises only when the Real-Time AS Imbalance Settlement process results in a net payment to Resources.

## RUC/SASM Changes (2017 SAWG):

Implementation of RTC would also include modifications to the RUC engine, such that the ability to coordinate the provision of energy and AS among resources in the RTM would be recognized in RUC

- Compared to the current market, offers the potential to reduce RUC activity, particularly in broader import-constrained areas.

- SASM activity would be reduced due to the coordinated access to the ERCOT portfolio of resources for the provision of energy and AS in RT

With RT co-optimization, Ancillary Services procurements are reassigned every 5 minutes (nominally).

- Thus, if for whatever reason a QSE’s Day-Ahead AS responsibility cannot be provided, and, there is sufficient capacity available, then RT co-optimization will economically procure the required AS in a seamless fashion

- Current thinking is that the SASM process will NOT be needed under RT-co-optimization

- The RUC process that ensures sufficient capacity to meet load forecast and Ancillary Service requirements will be used to meet the scenario where a QSE reports to ERCOT that it cannot meet its Day-Ahead AS responsibility for future hours

- RUC engine (DRUC,HRUC,WRUC) will be modified to include co-optimization.

  - RUC process will commit sufficient capacity to meet load forecast and AS for the RUC study horizon

  - Not protect HASL – allows for improved congestion management

  - Discussion needed on which Resources can provide AS in the RUC study

## Locational Reserves

Implementation of RTC with locational reserve requirement will need further stakeholder discussions. Is there a reliability requirement and other conceptual discussions. What are the locational reliability requirements for reserves? To what extent should mitigation of AS offer curves be considered if there is a concern about market power with the introduction of locational reserve requirements?

## AS Deliverability

Though not directly related to the implementation of RTC, the deliverability of AS is being brought up as an item for discussion. Further discussions involving ERCOT operations and stakeholders are required.

## High Level Description of the Clearing Process and Outputs

Under Co-optimization, the RT market clearing procures the following, nominally once every 5 minutes (for the current 5 minutes):

Please note that the descriptions in the sections below are applicable for both the options unless explicitly stated otherwise.

- Energy, in the form of Base Points, to meet the system demand for energy as represented by Generation To be Dispatched (GTBD); and

- Procure capacity sufficient to meet the AS demand.  Each type of AS will have its own ‘bid-to-buy’ demand curve.

Put another way, every 5 minutes the market clearing would result in the following:

- Base Points for energy to all Resources to meet the system demand for energy (GTBD). These Base points are valid for the next 5 minutes or until the next clearing of the RT market.  This is the same as in the current RT Market.

- LMPs at all required locations that are binding for the next 5 minutes or until the next clearing of the RT market. This is the same as in the current RT Market

- Reg-Up and Reg-Down awards and associated Reg-Up and Reg-Down MCPCs. This capacity is procured as Regulation Reserve Service and will be utilized by the ERCOT Load Frequency Control (LFC) engine to send regulation signals every LFC cycle – 4 seconds for the next 5 minutes or until next clearing of the RT market

- RRS awards and RRS MCPC. This procured capacity must be available to be converted into energy, if required, in the next 10 minutes (like a call option). This responsibility to deliver exists for the next 5 minutes or until the next clearing of the RT market

- Option 1: Non-Spin awards and Non-Spin MCPC. This procured capacity (Online and/or Offline) must be available to be converted into energy, if required, within the next 30 minutes following an instruction and must also be able to sustain the energy deployment for a 1 hour period (like a call option). This responsibility to deliver exists for the next 5 minutes or until the next clearing of the Real-Time market. On-Line Non-Spin capacity must be available to SCED for dispatch.

Resources with Non-Spin awards in RT are not eligible for Make-Whole payments. This treatment is the same as in the current design.

In order to award an Offline Resource Non-spin, the following factors are considered

- Is it qualified for Non-Spin?

- Has the resource satisfied its minimum down time?

- Can the Resource startup in 30 minutes and reach LSL (check the warmth state and the associated cold, intermediate and hot startup times)?

- Does it have adequate ramping capability to meet the performance requirements associated with its award?

Option 2:

Spinning Operating Reserve (SOR): This capacity is On-Line and available to SCED for dispatch. This capacity is capable of being converted into energy, if required, through successive SCED dispatch or manual instruction over the next 30 minutes and must also be able to sustain the energy deployment for a 1 hour period (like a call option). This responsibility to deliver exists for the next 5 minutes or until the next clearing of the Real-Time market.

Non-Spinning Operating Reserve (NSOR): This capacity is Off-Line and can be converted to energy, if required, within the next 30 minutes following an instruction and must also be able to sustain the energy deployment for a 1 hour period (like a call option). This responsibility to deliver exists for the next 5 minutes or until the next clearing of the Real-Time market.

Resources with Non-Spinning Operating Reserve (NSOR) awards in RT are not eligible for Make-Whole payments.

In order to award an Off-Line Resource NSOR, the following factors are considered

- Is it qualified for NSOR?

- Has the resource satisfied its minimum down time?

- Can the Resource startup in 30 minutes and reach LSL (check the warmth state and the associated cold, intermediate and hot startup times)?

- Does it have adequate ramping capability to meet the performance requirements associated with its award?

- Appropriate constraints are enforced to limit energy and AS awards based on the Resources’ telemetered ramp rates. This is to ensure that the awards for energy and AS are ramp feasible.

- Stakeholder discussions will be required to develop methodology to share the ramp rates between energy and AS (similar to the ramp sharing between current Security Constrained Economic Dispatch or SCED and LFC).

  - When to use Emergency Ramp Rates?

  - Should Reg-Up awards be limited by Reg_down ramp rate? This is to prevent Resources from being harmed by sequence of increasing Base Points over a period of time and then a reduction where due to ramp down limitations, they become price takers and simultaneously there is a Reg-Up shortage pricing.

- AS MW awards and associated MCPCs are published simultaneously with energy Base Points and associated LMPs — i.e. after every RT market clearing (nominally every 5 minutes). QSEs will not be required to immediately incorporate these AS awards into their control systems; i.e., there is no hand shaking between ERCOT and the QSEs representing Resources with regard to receipt and acknowledgement of AS awards. Rather, this is intrinsically done via the AS offer that can be updated at any time by the QSE or a Real-Time mechanism to inform ERCOT systems that Resource is unwilling or unable to provide AS.

- Market clearing can be re-initiated by ERCOT, prior to the normal 5-minutes, as with the current design.

- Resources in Start Up or Shut Down mode are not considered as available for energy dispatch nor are they considered available to provide AS.

- The key performance metric for the Resource is its ability to follow the energy Base Point. Like the current RT Market, the results of the proposed RT Market with energy and AS Co-optimization are effective immediately; i.e., the Base Points, LMPs, AS awards and AS MCPCs are binding upon RT Market clearing.

### Pricing Run Changes (modifications to NPRR 626)

A pricing run based on NPRR 626 (Reliability Deployment Price Adder) will be required to mitigate price reversals associated with the deployment of Load Resources and/or out-of-market reliability deployments that may otherwise reduce the total Real-Time price in times of scarcity. The objective of the pricing run will remain the same — that is, to determine the positive change to System Lambda when Load Resources or out-of-market reliability deployments occur. This positive change to System Lambda is added to all original energy prices (LMPs), and under RT Co-optimization the same positive change to System Lambda would be added to all the original MCPCs.

This pricing run will not require the modification of dispatch limits (High Dispatch Limit or HDL, and Low Dispatch Limit or LDL) for “in-market” Resources because in RT Co-optimization, the concept of High Ancillary Service Limit or HASL does not exist. Rather, the Resource’s entire capacity (between its Low Sustained Limit or LSL and High Sustained Limit or HSL) is considered available for allocation between energy and AS.

In addition, automatic UFR trip of Load Resources may need to be considered in the NPRR 626 pricing run if Load Resources providing RRS are not charged an AS imbalance payment for RRS under UFR trip.

## Telemetry Changes for Generation and Controllable Load Resources

Currently, the QSE representing the Generation Resources or Controllable Load Resource sends every 2 seconds, for each Resource, their respective AS Responsibility (Reg-Up, Reg-Down, RRS, Non-Spin) and corresponding applicable AS Schedule (RRS and Non-Spin).

Under RT Co-Optimization, the telemetry for AS responsibility and schedule are not required. The Resource specific AS Offers can be updated at any point in time (or a Real-Time mechanism for the QSE to inform ERCOT the inability of a specific Resource in providing specific AS type) and will represent the willingness of the QSE representing the Resource to sell capacity for AS in the upcoming next RT Market.

The proposed RT Market considers the following Resource specific data to optimally allocate the Resource’s capacity:

- Telemetered HSL, LSL (for Controllable Load Resource or CLR it is the Maximum Power Consumption or MPC and Low Power Consumption or LPC) , and ramp-rates, every 2 seconds

- Energy Offer Curve or EOC that is locked down by the end of the adjustment period (for CLR, this is the Real-Time Market Energy Bid)

- AS Offer that can be modified at any point in time (or a Real-Time mechanism for the QSE to inform ERCOT the inability of a specific Resource in providing specific AS type)

Further discussion via the stakeholder process will be needed to determine the proper timeline for updating AS Offers and EOCs and the eligibility criteria. There may be a need for additional telemetry to be used in conjunction with the AS offer to provide QSEs more flexibility.

## Telemetry Changes for Load Resources with UFR

One key issue involved in integrating Load Resources armed with high-set Under Frequency Relays (UFRs) into RT Co-optimization will be the practicality and necessity of arming/disarming the UFRs based on RRS awards. If determined to be impractical or infeasible, then these Load Resources would be ineligible to actively participate in the RT Market; instead their AS responsibilities originating from DAM, Trades, etc. would not be re-cleared in the RT Market —and they would effectively become price takers in the RT Market. This requires further discussion by ERCOT staff and stakeholders.

Update based on SAWG discussions:

Load Resource (LR) with UFR carrying RRS Responsibility (blocky AS):

- Upon deployment will be exempt from AS imbalance charge

- In RT, LR with UFR carrying RRS responsibility will be considered to be price takers (i.e. RRS offer from these LR = 0$/MW). i.e. no reduction of RRS demand curve by the amount of RRS responsibility from Load Resources.

- However, when deployed, LR will be considered to be still providing RRS till the 3 hour limit after recall.

- Need to develop process to account and differentiate scenarios where RRS from LR is deployed versus failure to provide, etc. This has impact on the AS imbalance settlement

- NPRR 626 enhancement to consider LR MW under UFR trip

Note 1: The above exemptions from AS Imbalance charges is a deviation from current practice with ORDC price adders

Note 2: Need to allow/account for non-frequency responsive capacity (NFRC) telemetered by Resource

Accounting for Resource providing "truly" Offline Non-Spin (Option 1) and NSOR (Option 2):

Should these Resources be exempt from AS Imbalance charges upon deployment? Note - We do not do that currently with the ORDC price adder settlements.

## AS Deployment Process

- Regulation Up and Regulation Down:

No material change from current process. LFC will deploy Regulation using the latest Regulation awards from the Real-Time Co-optimization engine.

Change is that LFC, will get as input, after every RT market clearing (usually every 5 minutes), a new set of QSE level Reg-Up and Reg-Down Responsibilities based on the Resource specific Reg-Up and Reg-Down awards.

- Responsive Reserve Service or RRS:

- Self-Deployment of UFR-type Load Resources due to frequency deviation from a forced outage (Unit Trips):

No change

- ERCOT release of RRS capacity to SCED:

Major change

Under RT Co-optimization, the current process of ERCOT reserving RRS capacity and releasing it under scarcity conditions would be discontinued and replaced with a “HASL-free” operation in which RRS is procured until the RRS supply and demand curves intersect — i.e., “RRS supply”  = “RRS demand”. Under scarcity conditions, energy to be served is given priority and smaller amounts of RRS would be procured — effectively similar to the release of RRS to SCED in current market. This results in scarcity pricing through the ORDC.

The current ability for the operator to manually enter an offset to GTBD will be retained to allow ERCOT the ability to react to persistent frequency deviations.

- ERCOT deployment of  RRS from Load Resource with High Set Under Frequency Relay:

Even under a RT Co-optimization, similar mechanism as the current market design would be employed to deploy RRS from Load Resources with High Set Under Frequency Relay.  ERCOT can, depending on the analysis of current and forecasted conditions, instruct deployment of RRS from Load Resources with RRS responsibility (at that time). The pricing run will mitigate price reversals due to this type of ERCOT deployment.

- On-Line Non-Spin (Option 1) or Spinning Operating Reserve (SOR – Option 2):

  - On-Line Non-Spin (including Quick Start Generation Resource with telemetered status of On-Line) or SOR:

Minor change. Offer floors on Non-Spin capacity will be removed(?). Currently the online Non-Spin capacity is always available to SCED. Under RT Co-optimization, the “HASL-free” operation described above would allow this capacity to be converted to energy, if required.

  - Off-Line Non-Spin (truly Off-Line) (Option 1) or Non-Spinning Operating Reserve (NSOR) that is truly Off-Line (Option 2)

Even under a RT Co-optimization, similar mechanism as the current market design would be employed to deploy truly Off-Line Non-Spin or truly Off-Line NSOR.  ERCOT can, depending on the analysis of current and forecasted conditions, instruct Offline Resources with Non-Spin or NSOR responsibility (at that time) to come Online. These Resources have the responsibility to deliver for 1 hour after they come online in or are recalled whichever comes first.

Further ERCOT staff and stakeholder discussions will be required to revisit the current procedures for deploying truly Offline Non-Spin or NSOR to mitigate local congestion when no market solution is available.

Under RT Co-optimization, consistent with current practice, Resources with Non-Spin or NSOR Responsibility that are Offline and deployed for Non-Spin or NSOR are not made whole to their Energy Offer Curve, nor are they made whole on their Startup Costs and Minimum Energy Costs.

## Discussion Items:

- AS products: Two options currently under discussion (Option 1, Option 2).

Status: Still under discussion. Option 2 seems to have more interest

Should On-Line capacity that cannot be converted to energy in 30 minutes (some duct burner capacity and Resources with low ramp rates) be eligible for SOR? ERCOT Operations input required.

- AS demand curves: Three examples currently under discussion.

Status: Still under discussion.

- Co-ordination of PBPC, VOLL, SWOC and max. value on ASDC

Status: Consensus at SAWG on the need for co-ordination. The values are still under discussion

- Locational Reserves: Is there a reliability requirement and other conceptual discussions. Examine the implementation of local ASDC as part of real time co-optimization in other markets to understand the need.  What are the requirements? To what extent should mitigation of AS offer curves be considered?

Status: not discussed yet

- RUC/SASM changes

Status: ERCOT presented its opinion and there seemed to be general consensus that RUC process would need to be modified to consider RTC and that SASM would not be needed

- Do we need to model Real-Time AS demand curves in the DAM?

Status: This issue is being revisited in 2017 SAWG discussions.

Update from 2015 SAWG discussions: AS Demand Curves will be used in DAM. The AS demand curves will be the same in DAM and Real-Time

- If we model AS demand curves in the DAM, as the DAM is voluntary, are there any issues? Resources do not have to participate since it is not mandatory to submit Resource specific Offers for energy and AS.

Status: This issue is being revisited in 2017 SAWG discussions.

Update from 2015 SAWG discussions: Consensus that this is not an issue

- If we model AS demand curves in the DAM, then what is the process of achieving required levels of AS procurement in the DAM when the DAM process does not procure enough? Is it Reliability Unit Commitment or RUC?

Status: This issue is being revisited in 2017 SAWG discussions.

Update from 2015 SAWG discussions: Prefer to go into Real-Time with AS insufficiency or need to develop procedure to deal with AS insufficiency in DAM if it is unacceptable to go into RT with AS insufficiency

- If AS demand curves are not modeled in the DAM, then AS procurement is effectively given higher priority than energy — that is, in DAM, all energy demand has a price and hence can be curtailed,  whereas the AS penalty is extremely high. This ensures that the AS Plan MW Requirements are procured. However, in RT, AS demand curves are modeled. The amount of Non-Spin procured depends on the intersection of the Non-Spin demand curve and the offer stack for Non-Spin. Hence, the procured amounts of Non-Spin can exceed or be less than the AS Plan MW Requirement for Non-Spin. If there is more Non-Spin procurement than the AS Plan, then loads are charged this amount on a Load-Ratio-Share. This is the same as the current process of AS Imbalance Settlements.

Status: This issue is being revisited in 2017 SAWG discussions.

Update from 2015 SAWG: Consensus is to model AS demand curves in DAM. For AS demand curves with a “long tail” – e.g. Non-Spin (Option 1) or SOR/NSOR (Option 2, Option 3), ERCOT may procure more AS than required for reliability if it is economical. Please note that a QSE attempting to self-arrange 100 % of AS obligation for Non-Spin (Option 1) or SOR/NSOR (Option 2, Option 3) may still get charged if the procured amounts exceed the amounts required for reliability.

- AS Participation rules in RTC: Must every Resource qualified to provide AS participate? Under what conditions can a Resource opt-out of AS participation in RTC?

Status: This issue is being revisited in 2017 SAWG discussions.

From 2015 SAWG discussions on AS Offer Submission rule changes to increase substitutability/liquidity and also promote price cascading of AS MCPC from higher quality AS to lower quality AS.

  - Rule 1: In the Day-Ahead Market, If MW capacity offered for Reg-Up, then the same MW capacity offer is also considered for Non-Spin (Option 1) or SOR (Option 2) at the same price as Reg-Up if the QSE did not enter an offer price for Non-Spin (Option 1) or SOR (Option 2)

  - Rule 2: In the Day-Ahead Market, If MW capacity offered for RRS, then the same MW capacity offer is also considered for Non-Spin (Option 1) or SOR (Option 2) at the same price as RRS if the QSE did not enter an offer price for Non-Spin (Option 1) or SOR (Option 2)

  - Rule 3: In Real-Time, all SCED dispatchable capacity (LSL to HSL) for On-Line Resources considered to be offering Non-Spin (Option 1) or SOR (Option 2) at zero $/MW, this includes QSGR telemetering a status of ON, i.e. in Real-Time, On-Line Resources have mandatory participation in Non-Spin (Option 1) or SOR (Option 2).

  - Rule 4: Develop exception rules where Resources can inform ERCOT of their inability to provide MW capacity for a particular AS. Is withdrawal of AS Offers or reduction in offered AS MW capacity an acceptable solution? If so, delete this rule.

- AS participation in RTC – timeline for informing ERCOT of changes to AS offers

Status: This issue is being revisited in 2017 SAWG discussions.

From SAWG discussions on need to develop timeline for updating AS Offers and Energy Offers. There may be a need for additional telemetry to be used in conjunction with the AS offer to provide QSEs more flexibility. Alternative would be to use telemetry for specific AS types from a Resource to indicate inability or unwillingness to provide AS.

- Load Resource Deployment for RRS: Need to develop process to account and differentiate scenarios where RRS from LR is deployed versus failure to provide, etc. This has impact on the AS imbalance settlement

Status: not discussed yet

- Load Resource Deployment for RRS: AS imbalance charges

Status: Update from 2015 SAWG discussions:

Load Resource (LR) with UFR carrying RRS Responsibility (blocky AS):

- Upon deployment will be exempt from AS imbalance charge – this is a change from current practice with ORDC price adders

- In RT, LR with UFR carrying RRS responsibility will be considered to be price takers (i.e. RRS offer from these LR = 0$/MW). i.e. no reduction of RRS demand curve by the amount of RRS responsibility from Load Resources. Discussion required on changes to NPRR 626 pricing run for including RRS MW deployment from Load Resource due to UFR Trip.

- However, when deployed, LR will be considered to be still providing RRS till the 3 hour limit after recall.

Note 1: The above exemptions from AS Imbalance charges is a deviation from current practice

In addition, automatic UFR trip of Load Resources may need to be considered in the NPRR 626 pricing run if Load Resources providing RRS are not charged an AS imbalance payment for RRS under UFR trip.

- Should Offer floors on Non-Spin (NSOR) be removed?

Status: not discussed yet

- For Non-Spin, should the statistical distribution of Online Reserves be used or Online+Offline Reserves or some combination? Note that the price on the Non-Spin demand curve will be lower if the statistical distribution of the Online+Offline Reserves is used.

Status: Update from 2015 SAWG discussions: Use the statistical distribution of Online+Offline Reserves to build the Non-Spin (Option 1) or NSOR (Option 2, Option 3) Demand Curve. For SOR (Option 2, Option 3) use the statistical distribution of Online Reserves to build the SOR Demand Curve

- How should the demand curve for Reg-Down be determined? Is it the same as the Reg-Up demand curve?

Status: Update from 2015 SAWG discussions: Reg-Down Demand Curve will have the same shape as the Reg-Up Demand Curve. Reg-Down Demand Curve width (MW required for Reg-Down)) will be based on ERCOT operational requirements. The price points on the Reg-Down Demand Curve will be the same as the Reg-Up Demand Curve till the Reg-Down MW requirement.

- Accounting for Resource providing "truly" Offline Non-Spin (Option 1) and NSOR (Option 2):

Should these Resources be exempt from AS Imbalance charges upon deployment?

Note - We do not do that currently with the ORDC price adder settlements.

Status: not discussed yet

- What other metrics apart from following Base Points, GREDP are required?

Status: not discussed yet

- Stakeholder discussions will be required to develop methodology to share the ramp rates between energy and AS (similar to the ramp sharing between current Security Constrained Economic Dispatch or SCED and LFC).

Status: not discussed yet

- When to use Emergency Ramp Rates? Only upon ERCOT notification?

Status: not discussed yet

- Should Reg-Up awards be limited by Reg_down ramp rate? This is to prevent Resources from being harmed by sequence of increasing Base Points over a period of time and then a reduction where due to ramp down limitations, they become price takers and simultaneously there is a Reg-Up shortage pricing.

Status: not discussed yet

- No Make Whole payments if Resources financially harmed with respect to Day-Ahead positions due to RT energy+AS co-optimization.

Status: SAWG Discussion on 2/25/2015: SAWG consensus to NOT have make-whole payments for this type of scenario.

Under Real-Time co-optimization when a Resource buys out of its Day ahead AS responsibility, the QSE representing the Resource could get financially harmed by the Real-Time imbalance settlement for energy and AS if it had made sales for AS from that Resource in the Day-Ahead Market. There are specific conditions under which this can occur:

1. System conditions are such that there is shortage/scarcity in one AS but surplus of capacity for energy. i.e. scarcity pricing for AS is not reflected in the energy prices (LMPs)

2. Resource is ramp constrained (how much it can move in 5 minutes) and the awards are limited by this. For example, if a Resource is generating energy at HSL and the next RT co-optimization results in a very high price for RRS, then the Resource MW award for RRS is how much it can be ramped down in 5 minutes.

3. QSE has made a sale of AS on this Resource in the Day-Ahead Market

Note: if a Resource has not made a Day-Ahead Sale for AS, then this scenario does not apply.

- Should ERCOT move to a 5 minute settlement?

Status: Raised at SAWG in 2015

ERCOT Staff commented that currently the ORDC price adders for capacity is done on a 15 minute basis and Real-Time Co-optimization does not have to change the settlement interval as the AS Imbalance settlement can continue to use a 15 minute settlement interval for AS capacity.

- What are the AS deliverability concerns and how can it be mitigated?

Status: not discussed yet

# Appendix 1: High Level Mathematical Formulation of energy and AS Co-Optimization for Option 2

The simplified formulation of the optimization problem (objective, constraints, pricing analysis) is presented below where energy and the various AS products are co-optimized in both Day-Ahead and Real-Time Markets.

Simplifications:

- Transmission constraints in both Day-Ahead and Real-Time, PTPs, block offers and bids in Day-Ahead are not considered

- Constraints on how much AS can be awarded to a single resource based on a % of HSL or ramp capability are not considered

- CiEnergyBid,CiEnergyOffer,CiASOffer are the submitted bids ($/MWh), energy offers ($/MWh), AS offers ($/MW) respectively. For simplicity, these bids and offers are considered to be constant for the entire MW bid or offered.

- In the Day-Ahead Market, Resources must submit offers to sell energy and AS (all the types) in order to be awarded.

- The equations below are describing Option 2. If the reader is interested in the equations for the Option 1 approach in the concept paper, then in the equations below:

  - Consider SOR offers (spinning operating reserve) as Non-Spin (both On-Line and Off-Line)

  - Replace the demand curve for SOR with Non-Spin demand curve.

  - Remove terms with NSOR (non-spinning operating reserve) – Off-line non-spin is already included in (v) a. above.

Day-Ahead Market Objective Function:

Obj≡Minimize -i=1NebCiEnergyBid×MWiEnergyBidAward-i=1NregUpDemandCiRegUpDemand×MWiRegUpDemandAward-i=1NregDnDemandCiRegDnDemand×MWiRegDnDemandAward-i=1NRRSDemandCiRRSDemand×MWiRRSDemandAward-i=1SORDemandCiSORDemand×MWiSORDemandAward-i=1NSORDemandCiNSORDemand×MWiNSORDemandAward+i=1NgCiEnergyOffer×MWiEnergyOfferAward+i=1Ng+NclCiRegUpOffer×MWiRegUpAward+i=1Nfg+NfclCiFRRSUpOffer×MWiFRRSUpAward+i=1Ng+NclCiRegDnOffer×MWiRegDnAward+i=1NfclCiFRRSDnOffer×MWiFRRSDnAward+i=1Ng+Ncl+NblCiRRSOffer×MWiRRSAward+i=1Ng+Ncl+NblCiSOROffer×MWiSORAward+i=1NgCiNSOROffer×MWiNSORAward

Real-Time Market Objective Function:

Obj≡Minimize i=1NPBCiPowerBalance×MWiPowerBalanceMW-i=1NebCiEnergyBid×MWiEnergyBidAward-i=1NregUpDemandCiRegUpDemand×MWiRegUpDemandAward-i=1NregDnDemandCiRegDnDemand×MWiRegDnDemandAward-i=1NRRSDemandCiRRSDemand×MWiRRSDemandAward-i=1NSORDemandCiSORDemand×MWiSORDemandAward-i=1NSORDemandCiNSORDemand×MWiNSORDemandAward+i=1NgCiEnergyOffer×MWiEnergyOfferAward+i=1Ng+NclCiRegUpOffer×MWiRegUpAward+i=1Nfg+NfclCiFRRSUpOffer×MWiFRRSUpAward+i=1Ng+NclCiRegDnOffer×MWiRegDnAward+i=1NfclCiFRRSDnOffer×MWiFRRSDnAward+i=1Ng+Ncl+NblCiRRSOffer×MWiRRSAward+i=1Ng+Ncl+NblCiSOROffer =0 for Real-Time×MWiSORAward+i=1NgCiNSOROffer×MWiNSORAward

Subject to:

Ignoring transmission constraints and focusing on power balance, AS procurement and the main Resource limit constraints, the set of constraints are given below:

System wide constraints:

- Power Balance: (Shadow price =  λ)

Day-Ahead:

i=1NgMWiEnergyOfferAward-i=1NebMWiEnergyBidAward=0

Real-Time:

i=1NgMWiEnergyOfferAward+i=1NPBMWiPowerBalanceMW-GTBD=0

- Reg-Up Procurement (including FRRS-Up): (Shadow price =  α)

i=1Ng+NclMWiRegUpAward+i=1Nfg+NfclMWiFRRSUpAward-i=1NregUpDemandMWiRegUpDemandAward=0

- FRRS-Up maximum procurement limit: (Shadow price =  ωFRRSUp)

MaxFRRSUp-i=1Nfg+NfclMWiFRRSUpAward≥0

- Reg-Down Procurement (including FRRS-Down): (Shadow price =  β)

i=1Ng+NclMWiRegDnAward+i=1NfclMWiFRRSDnAward-i=1NregDnDemandMWiRegDnDemandAward=0

- FRRS-Down maximum procurement limit: (Shadow price =  ωFRRSDn)

MaxFRRSDn-i=1NfclMWiFRRSDnAward ≥0

- RRS Procurement: (Shadow price =  γ)

i=1Ng+Ncl+NblMWiRRSAward-i=1NRRSDemandMWiRRSDemandAward=0

- RRS maximum procurement from “blocky” Load Resource: (Shadow price =  ωRRSbl)

MaxRRSbl- i=1NblMWiRRSAward≥0

- SOR Procurement: (Shadow price =  δ)

i=1Ng+Ncl+NblMWiSORAward-i=1NSORDemandMWiSORDemandAward=0

- NSOR Procurement: (Shadow price =  ϕ)

i=1Ng+Ncl+NblMWiSORAward+i=1NgMWiNSORAward- i=1NSORDemandMWiNSORDemandAward=0

Individual Energy Bid constraints:

- Energy Bid MW constraint for every energy bid i=1,2,3…Neb: (Shadow price =  θieb respectively)

EnergyBidMWi-MWiEnergyBidAward ≥0

Individual Resource constraints:

Each Resource will have its own set of constraints to ensure awards are within bounds of its own upper (HSL/MPC) and low (LSL/LPC) limits.

- LSL Constraint for every modeled Generation Resource i=1,2,3…Ng: (Shadow price =  θiLSL)

MWiEnergyOfferAward-MWiRegDnAward-LSLi ≥0

- HSL Constraint for every modeled Generation Resource i=1,2,3…Ng: (Shadow price = θiHSL)

On-Line:

HSLi-MWiEnergyOfferAward-MWiRegUpAward-MWiRRSAward-MWiSORAward≥0

Off-Line:

HSLi-MWiNSORAward≥0

- AS Offer MW constraint for every modeled Generation Resource i=1,2,3…Ng: (Shadow price =  θig-ASup,θig-ASdn respectively)

On-Line:

ASOfferMWUp-i-MWiRegUpAward-MWiRRSAward-MWiSORAward ≥0

ASOfferMWDn-i-MWiRegDnAward ≥0

Off-Line:

ASOfferMWUp-i-MWiRSNSORAward ≥0

- MPC & LPC Constraint for every modeled “Blocky” Load Resource i=1,2,3…Nbl: (Shadow price =  θibl)

Note that a “blocky” Load Resource is awarded only one AS product.

MPCi-LPCi-MWiRRSAward ≥0

or

MPCi-LPCi-MWiSORAward ≥0

- AS Offer MW constraint for every modeled “Blocky” Load Resource i=1,2,3…Nbl: (Shadow price =  θibl-AS)

ASOfferMWUp-i-MWiRRSAward ≥0

or

ASOfferMWUp-i-MWiSORAward ≥0

- MPC & LPC Constraint for every modeled Controllable Load Resource i=1,2,3…Ncl: (Shadow price =  θicl)

MPCi-LPCi-MWiRegDnAward-MWiRegUpAward-MWiRRSAward-MWiSORAward≥0

- AS Offer MW constraint for every Controllable Load Resource i=1,2,3…Ncl: (Shadow price =  θicl-ASup,θicl-ASdn respectively)

ASOfferMWUp-i-MWiRegUpAward-MWiRRSAward-MWiSORAward ≥0

ASOfferMWDn-i-MWiRegDnAward ≥0

- HSL & LSL Constraint for every “Quick/Fast” Resource qualified for FRRS-Up and FFR1 and partly modeled as Generation Resource i=1,2,3…Nfg: (Shadow price =  θifg)

HSLi-LSLi-MWiFRRSUpAward ≥0

- AS Offer MW constraint for every “Quick/Fast” Resource qualified for FRRS-Up and FFR1 and partly modeled as Generation Resource i=1,2,3…Nfg: (Shadow price =  θifg-AS)

ASOfferMWUp-i-MWiFRRSUpAward ≥0

- MPC & LPC Constraint for every “Quick/Fast” Resource qualified for FRRS-Up, FRRS-Down and RRS and partly modeled as Controllable Load Resource i=1,2,3…Nfcl: (Shadow price =  θifcl)

MPCi-LPCi-MWiFRRSDnAward-MWiFRRSUpAward-MWiRRSAward ≥0

- AS Offer MW constraint for every “Quick/Fast” Resource qualified for FRRS-Up and FRRS-Down and partly modeled as Controllable Load Resource i=1,2,3…Nfcl: (Shadow price =  θifcl-ASup,θifcl-ASdn respectively)

ASOfferMWUp-i-MWiFRRSUpAward ≥0

ASOfferMWDn-i-MWiFRRSDnAward ≥0

Lagrangian Function:

The objective and constraints are combined to form the Lagrange function:

L=Objective- iShadowpricei×Constrainti

At optimal solution ∇L=0 (optimality condition)

i.e. the partial derivative of L with respect to each award

MWiEnergyBidAward,MWiEnergyOfferAward,MWiRegUpAward, etc and the shadow prices λ,α,θieb,θiLSL,θiHSL, etc. will equate to zero at the optimal solution.

Taking the partial derivative of  L with respect to each award MWiEnergyBidAward,MWiEnergyOfferAward,MWiRegUpAward, etc.

and rearranging the terms by

∆MWiEnergyBidAward, ∆MWiEnergyOfferAward,∆MWiRegUpAward, etc. we get:

- Day Ahead: For each energy bid i=1,2,3,…Neb, the following equation holds true

CiEnergyBid= λ-θieb

If the energy bid i is marginal to the power balance constraint, then θieb=0 and the energy bid i sets the shadow price for the power balance constraint (System Lambda λ)

Real-Time: If the Power Balance Penalty Curve is marginal to energy, the following equation holds true and the Power Balance Penalty Curve sets the shadow price for the power balance constraint (System Lambda λ):

CiPowerBalance= λ

- For each energy offer from modeled Generation Resource i=1,2,3,…Ng, the following equation holds true

CiEnergyOffer=λ-θiHSL+θiLSL

If the energy offer i is marginal to the power balance constraint, then, θiHSL=0,θiLSL=0  and the energy offer i sets the shadow price for the power balance constraint (System Lambda λ)

- For each Reg-Up offer from modeled Generation Resource i=1,2,3,…Ng, the following equation holds true

CiRegUpOffer= α-θiHSL-θig-ASup

If the Reg-Up offer i is marginal to the Reg-Up Procurement constraint, then in most cases,  θiHSL=0,θig-ASup=0  and the Reg-Up Offer i sets the shadow price for the Reg-Up Procurement constraint ( this is the Reg-Up MCPC α )

- For each Reg-Down offer from modeled Generation Resource i=1,2,3,…Ng, the following equation holds true

CiRegDnOffer= β-θiLSL-θig-ASdn

If the Reg-Down offer i is marginal to the Reg-Down Procurement constraint, then, in most cases,  θiHSL=0,θig-ASup=0  and the Reg-Down Offer i sets the shadow price for the Reg-Down Procurement constraint ( this is the Reg-Down MCPC β )

- For each RRS offer from modeled Generation Resource i=1,2,3,…Ng, the following equation holds true

CiRRSOffer= γ-θiHSL-θig-ASup

If the RRS offer i is marginal to the RRS Procurement constraint, then,  θiHSL=0,θig-ASup=0  and the RRS Offer i sets the shadow price for the RRS Procurement constraint (this is the RRS MCPC γ )

- For each SOR offer from modeled Generation Resource i=1,2,3,…Ng, the following equation holds true

CiSOROffer= δ+ϕ-θiHSL-θig-ASup

If the SOR offer i is marginal to the SOR Procurement constraint, then,  θiHSL=0,θig-ASup=0  .

Please note that the MCPC for SOR is the sum of the shadow price for the SOR and NSOR (δ+ϕ) procurement constraints.

- For each NSOR offer from modeled offline Generation Resource i=1,2,3,…Ng, the following equation holds true

CiNSOROffer= ϕ-θiHSL-θig-ASup

If the NSOR offer i is marginal to the SNSOR Procurement constraint, then,  θiHSL=0,θig-ASup=0.

Note that the MCPC for NSOR  is the shadow price for the NSOR  procurement constraint

- For each RRS offer from modeled “blocky” Load Resource i=1,2,3,…Nbl, the following equation holds true

CiRRSOffer= γ-ωRRSbl-θibl-θibl-AS

If the RRS offer i is marginal to the RRS Procurement constraint, then, ωRRSbl=0, θibl=0,θibl-AS=0  and the RRS Offer i sets the shadow price for the RRS Procurement constraint (this is the RRS MCPC γ )

Note that if the RRS offer MW is submitted as a block, then, this offer cannot set the MCPC

If the RRS maximum procurement constraint from “blocky” Load Resource is binding then ωRRSbl≠0. In this case the MCPC for RRS is still the shadow price of the RRS procurement constraint (γ) and the RRS awards to “blocky” Load Resources will be paid this price γ

- For each SOR offer from modeled “blocky” Load Resource i=1,2,3,…Nbl, the following equation holds true

CiSOROffer= δ+ϕ-θibl-θibl-AS

If the SOR offer i is marginal to the SOR Procurement constraint, then,  θibl=0,θibl-AS=0  and the SOR Offer i sets the shadow price for the SOR Procurement constraint (this is the SOR MCPC δ )

Note that if the SOR offer MW is submitted as a block, then, this offer cannot set the MCPC

- For each Reg-Up offer from modeled Controllable Load Resource i=1,2,3,…Ncl, the following equation holds true

CiRegUpOffer= α-θicl-θicl-ASup

- For each Reg-Down offer from modeled Controllable Load Resource i=1,2,3,…Ncl, the following equation holds true

CiRegDnOffer= β-θicl-θicl-ASdn

- For each RRS offer from modeled Controllable Load Resource i=1,2,3,…Ncl, the following equation holds true

CiRRSOffer= γ-θicl-θicl-ASup

- For each SOR offer from modeled Controllable Load Resource i=1,2,3,…Ncl, the following equation holds true

CiSOROffer= δ+ϕ-θicl-θicl-ASup

- For each FRRS-Up offer from “Quick/Fast” Resource partly modeled as Generation Resource i=1,2,3,…Nfg, the following equation holds true

CiFRRSUpOffer= α-ωFRRSUp-θifg-θifg-AS

If the FRRS-Up offer i is marginal to the Reg-Up Procurement constraint, then, ωFRRSUP=0, θifg=0,θifg-AS=0  and the FRRS-Up Offer i sets the shadow price for the Reg-Up Procurement constraint (this is the Reg-Up MCPC α )

If the FRRS-Up maximum procurement constraint from “Quick/Fast” Resource is binding then ωFRRSUP≠0. In this case the MCPC for Reg-Up is still the shadow price of the Reg-Up procurement constraint (α) and the FRRS-Up awards to “Quick/Fast” Resources will be paid this price α

- For each FRRS-Up offer from “Quick/Fast” Resource partly modeled as Load Resource i=1,2,3,…Nfcl, the following equation holds true

CiFRRSUpOffer= α-ωFRRSUp-θifcl-θifcl-AS

If the FRRS-Up offer i is marginal to the Reg-Up Procurement constraint, then, ωFRRSUP=0, θifcl=0,θifcl-AS=0  and the FRRS-Up Offer i sets the shadow price for the Reg-Up Procurement constraint (this is the Reg-Up MCPC α )

If the FRRS-Up maximum procurement constraint from “Quick/Fast” Resource is binding then ωFRRSUP≠0. In this case the MCPC for Reg-Up is still the shadow price of the Reg-Up procurement constraint (α) and the FRRS-Up awards to “Quick/Fast” Resources will be paid this price α

- For each FRRS-Down offer from “Quick/Fast” Resource partly modeled as Load Resource i=1,2,3,…Nfcl, the following equation holds true

CiFRRSDnOffer= β-ωFRRSDn-θifcl-θifcl-ASdn

If the FRRS-Down offer i is marginal to the Reg-Down Procurement constraint, then, ωFRRSDN=0, θifcl=0,θifcl-ASdn=0  and the FRRS-Down Offer i sets the shadow price for the Reg-Down Procurement constraint (this is the Reg-Up MCPC β )

If the FRRS-Down maximum procurement constraint from “Quick/Fast” Resource is binding then ωFRRSDN≠0. In this case the MCPC for Reg-Down is still the shadow price of the Reg-Down procurement constraint (β) and the FRRS-Up awards to “Quick/Fast” Resources will be paid this price β

Note: If there is scarcity in any of the AS (Reg-Up, Reg-Down, RRS, SOR and NSOR), then the demand curves prices at the last cleared MW AS demand on the respective demand curves will set the Shadow Prices of the applicable procurement constraints.

MCPC formula (from shadow prices)

| AS Product | MCPC | Comments |
| --- | --- | --- |
| Reg-Up | α | Shadow price of the Reg-Up procurement (including FRRS-Up) constraint |
| FRRS-Up | α | FRRS-Up is valued the same as Reg-Up |
| Reg-Down | β | Shadow price of the Reg-Down procurement (including FRRS-Down) constraint |
| FRRS-Down | β | FRRS-Down is valued the same as Reg-Down |
| RRS | γ | Shadow Price of the RRS procurement constraint |
| SOR | δ+ϕ | Sum of the Shadow Prices of the SOR and NSOR procurement constraints |
| NSOR | ϕ | Shadow Price of the NSOR procurement constraint |

The term Production Costs refers to the use of submitted Energy Offer Curves, AS Offers, and Energy Bid Curves (if applicable) and any associated mitigation of the Energy Offer Curves due to transmission congestion.

The term Production Costs refers to the use of submitted Energy Offer Curves, AS Offers, and Energy Bid Curves (if applicable) and any associated mitigation of the Energy Offer Curves due to transmission congestion.