| NPRR Number | 821 | NPRR Title | Elimination of the CRR Deration Process |
| --- | --- | --- | --- |
| Date of Decision |  | July 20, 2017 |  |
| Action |  | Recommended Approval |  |
| Timeline |  | Normal |  |
| Proposed Effective Date |  | Upon system implementation but no earlier than July 1, 2019 |  |
| Priority and Rank Assigned |  | Priority – 2019; Rank – 2500 |  |
| Nodal Protocol Sections Requiring Revision |  | 4.5.1, DAM Clearing Process 7.9.1.1, Payments and Charges for PTP Obligations Settled in DAM 7.9.1.2, Payments for PTP Options Settled in DAM 7.9.1.3, Minimum and Maximum Resource Prices (delete) 7.9.1.5, Payments and Charges for TPT Obligations with Refund Settled in DAM 7.9.1.6, Payments for PTP Options with Refund Settled in DAM |  |
| Related Documents Requiring Revision/ Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) eliminates the Congestion Revenue Right (CRR) deration process. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | CRRs that are positively valued and source or sink at a Resource Node are subject to the CRR deration process. The process raises market equitability issues as CRR activity does not cause the underlying topology conditions that lead to deration. This NPRR addresses this issue by eliminating the CRR deration process. |  |
| Credit Work Group Review |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR821 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |
| PRS Decision |  | On 4/13/17, PRS voted unanimously to table NPRR821 and refer the issue to WMS. All Market Segments were present for the vote. On 6/15/17, PRS voted to recommend approval of NPRR821 as amended by the 5/31/17 ERCOT comments. There were five opposing votes from the Consumer, Independent Generator, Independent Retail Electric Provider (IREP) (2), and Municipal Market Segments; and two abstentions from the Independent Power Marketer (IPM) and Investor Owned Utility (IOU) Market Segments. All Market Segments were present for the vote. On 7/20/17, PRS voted via roll call to endorse and forward to TAC the 6/15/17 PRS Report and Impact Analysis for NPRR821 with a recommended rank of 2019 and priority of 2500, and a proposed effective date of upon system implementation but no earlier than July 1, 2019. There were six opposing votes from the Consumer (Occidental), Independent Generator (Luminant), IREP (2) (Direct Energy, Source Power and Gas), IOU (AEPSC), and Municipal (Austin Energy) Market Segments; and three abstentions from the IPM (Citigroup), IREP (Reliant Energy Retail Services), and IOU (CenterPoint) Market Segments. All Market Segments were present for the vote. |  |
| Summary of PRS Discussion |  | On 4/13/17, some participants voiced support for NPRR821, and expressed concern that the current process is not well structured. Others expressed support for the 4/12/17 Crescent Power comments to NPRR821, adding that the current design of the CRR deration process was intentional and balanced. The Independent Market Monitor (IMM) stated that it is comfortable with the removal of the CRR deration process, and that there are sufficient tools in place to identify manipulation. Participants requested further discussion at WMS. On 6/15/17, participants debated risks to Load, should NPRR821 be approved and implemented. Some participants expressed concern that NPRR821 unfairly changes allocation such that Loads will bear 100 percent of the risk associated with deration. Other participants countered that shortfall is borne by CRR Account Holders when the Balancing Account is exhausted, and that the risk of shortfall is not exclusive to Load. Participants reviewed the 6/8/17 WMS comments to NPRR821, which recommended an implementation date of no earlier than July 1, 2019, and determined to first review the Impact Analysis, per established stakeholder process, before proposing an effective date for NPRR821. On 7/20/17, participants discussed the Impact Analysis. |  |

| Sponsor |  |
| --- | --- |
| Name | Sadao Millberg |
| E-mail Address | milberg@dc-energy.com |
| Company | DC Energy Texas |
| Phone Number | 703-760-4358 |
| Cell Number |  |
| Market Segment | IPM |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Brittney Albracht |
| E-Mail Address | Brittney.Albracht@ercot.com |
| Phone Number | 512-225-7027 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| Crescent Power 041217 | Expressed concern for oversold CRRs, market manipulation, and uplift from congestion rent shortfall. |
| WMS 050417 | Requested PRS continue to table NPRR821 for review by Congestion Management Working Group (CMWG). |
| ERCOT 053117 | Proposed removing the target payment calculation from Settlement equations, removing a reference to the Day-Ahead Simultaneous Feasibility Test, and an effective date no earlier than January 1, 2020, which is the day after the last CRR already sold expires. |
| Luminant 060517 | Expressed opposition to eliminating the CRR deration process and enumerated three ways in which the deration process supports the ERCOT market. |
| WMS 060817 | Endorsed NPRR821 as amended by the 5/31/17 ERCOT comments with a recommended implementation date no earlier than July 1, 2019. |

Market Rules Notes

Administrative changes to the language were made and authored as “ERCOT Market Rules.”

Please note that the baseline language in Section 4.5.1 has been updated to reflect the incorporation of the following NPRRs:

- NPRR827, Disallow PTP Obligation Bid Award where Clearing Price exceeds Bid Price by $0.25/MW per hour (incorporated 6/29/17)

- NPRR831, Inclusion of Private Use Networks in Load Zone Price Calculations (incorporated 7/1/17)

Please note the following NPRR also proposes revisions to Section 4.5.1:

- NPRR833, Modify PTP Obligation Clearing Change

Proposed Protocol Language Revision

4.5.1	DAM Clearing Process

(1)	At 1000 in the Day-Ahead, ERCOT shall start the Day-Ahead Market (DAM) clearing process.  If the processing of DAM bids and offers after 0900 is significantly delayed or impacted by a failure of ERCOT software or systems that directly impacts the DAM, ERCOT shall post a Notice as soon as practicable on the Market Information System (MIS) Public Area, in accordance with paragraph (1) of Section 4.1.2, Day-Ahead Process and Timing Deviations, extending the start time of the execution of the DAM clearing process by an amount of time at least as long as the duration of the processing delay plus ten minutes.  In no event shall the extension exceed more than one hour from when the processing delay is resolved.

(2)	The purpose of the DAM is to economically and simultaneously clear offers and bids described in Section 4.4, Inputs into DAM and Other Trades.

(3)	The DAM uses a multi-hour mixed integer programming algorithm to maximize bid-based revenues minus the offer-based costs over the Operating Day, subject to security and other constraints, and ERCOT Ancillary Service procurement requirements.

- The bid-based revenues include revenues from DAM Energy Bids and Point-to-Point (PTP) Obligation bids.

(b)	The offer-based costs include costs from the Startup Offer, Minimum Energy Offer, and Energy Offer Curve of any Resource that submitted a Three-Part Supply Offer, DAM Energy-Only Offers and Ancillary Service Offers.

(c)	Security constraints specified to prevent DAM solutions that would overload the elements of the ERCOT Transmission Grid include the following:

(i)	Transmission constraints – transfer limits on energy flows through the ERCOT Transmission Grid, e.g., thermal or stability limits.  These limits must be satisfied by the intact network and for certain specified contingencies.

These constraints may represent:

(A)	Thermal constraints – protect Transmission Facilities against thermal overload.

(B)	Generic constraints – protect the ERCOT Transmission Grid against transient instability, dynamic stability or voltage collapse.

(C)	Power flow constraints – the energy balance at required Electrical Buses in the ERCOT Transmission Grid must be maintained.

(ii)	Resource constraints – the physical and security limits on Resources that submit Three-Part Supply Offers:

(A)	Resource output constraints – the Low Sustained Limit (LSL) and High Sustained Limit (HSL) of each Resource; and

(B)	Resource operational constraints – includes minimum run time, minimum down time, and configuration constraints.

(iii)	Other constraints –

(A)	Linked offers – the DAM may not select any one part of that Resource capacity to provide more than one Ancillary Service or to provide both energy and an Ancillary Service in the same Operating Hour.  The DAM may, however, select part of that Resource capacity to provide one Ancillary Service and another part of that capacity to provide a different Ancillary Service or energy in the same Operating Hour, provided that a Generation Resource may not offer, and the DAM may not select, linked Energy and Off-Line Non-Spinning Reserve (Non-Spin) Ancillary Service Offers in the same Operating Hour.

(B)	The sum of the awarded Ancillary Service capacities for each Resource must be within the Resource limits specified in the Current Operating Plan (COP) and Section 3.18, Resource Limits in Providing Ancillary Service, and the Resource Parameters as described in Section 3.7, Resource Parameters.

(C)	Block Ancillary Service Offers for a Load Resource – blocks will not be cleared unless the entire quantity block can be awarded.  Because block Ancillary Service Offers cannot set the Market Clearing Price for Capacity (MCPC), a block Ancillary Service Offer may clear below the Ancillary Service Offer price for that block.

(D)	Block DAM Energy Bids, DAM Energy-Only Offers, and PTP Obligation bids – blocks will not be cleared unless the entire time and/or quantity block can be awarded.  Because quantity block bids and offers cannot set the Settlement Point Price, a quantity block bid or offer may clear in a manner inconsistent with the bid or offer price for that block.

(E)	Combined Cycle Generation Resources – The DAM may commit a Combined Cycle Generation Resource in a time period that includes the last hour of the Operating Day only if that Combined Cycle Generation Resource can transition to a shutdown condition in the DAM Operating Day.

(d)	Ancillary Service needs for each Ancillary Service include the needs specified in the Ancillary Service Plan that are not part of the Self-Arranged Ancillary Service Quantity and that must be met from available DAM Ancillary Service Offers while co-optimizing with DAM Energy Offers.  ERCOT may not buy more of one Ancillary Service in place of the quantity of a different service.  See Section 4.5.2, Ancillary Service Insufficiency, for what happens if insufficient Ancillary Service Offers are received in the DAM.

(4)	ERCOT shall determine the appropriate Load distribution factors to allocate offers, bids, and source and sink of CRRs at a Load Zone across the Electrical Buses that are modeled with Load in that Load Zone.  The Load distribution factors are based on historical State Estimator hourly distribution using a proxy day methodology representing anticipated weather conditions.  If ERCOT decides, in its sole discretion, to change the Load distribution factors for reasons such as anticipated weather events or holidays, ERCOT shall select a State Estimator hourly distribution from a proxy day reasonably reflecting the anticipated Load in the Operating Day.  ERCOT may also modify the Load distribution factors to account for predicted differences in network topology between the proxy day and Operating Day.  ERCOT shall develop a methodology, subject to Technical Advisory Committee (TAC) approval, to describe the modification of the proxy day bus-load distribution for this purpose.

[NPRR831: Replace paragraph (4) above with the following upon system implementation:] (4) ERCOT shall determine the appropriate Load distribution factors to allocate offers, bids, and source and sink of CRRs at a Load Zone across the Electrical Buses that are modeled with Load in that Load Zone. The non-Private Use Network Load distribution factors are based on historical State Estimator hourly distribution using a proxy day methodology representing anticipated weather conditions. The Private Use Network Load distribution factors are based on an estimated Load value considering historical net consumption at all Private Use Networks. If ERCOT decides, in its sole discretion, to change the Load distribution factors for reasons such as anticipated weather events or holidays, ERCOT shall select a State Estimator hourly distribution from a proxy day reasonably reflecting the anticipated Load in the Operating Day. ERCOT may also modify the Load distribution factors to account for predicted differences in network topology between the proxy day and Operating Day. ERCOT shall develop a methodology, subject to Technical Advisory Committee (TAC) approval, to describe the modification of the proxy day bus-load distribution for this purpose.

(5)	ERCOT shall allocate offers, bids, and source and sink of CRRs at a Hub using the distribution factors specified in the definition of that Hub in Section 3.5.2, Hub Definitions.

(6)	A Resource that has a Three-Part Supply Offer cleared in the DAM may be eligible for Make-Whole Payment of the Startup Offer and Minimum Energy Offer submitted by the Qualified Scheduling Entity (QSE) representing the Resource under Section 4.6, DAM Settlement.

(7)	The DAM Settlement is based on hourly MW awards and on Day-Ahead hourly Settlement Point Prices.  All PTP Options settled in the DAM are settled based on the Day-Ahead Settlement Point Prices (DASPPs).  ERCOT shall assign a Locational Marginal Price (LMP) to de-energized Electrical Buses for use in the calculation of the DASPPs by using heuristic rules applied in the following order:

(a)	Use an appropriate LMP predetermined by ERCOT as applicable to a specific Electrical Bus; or if not so specified

(b)	Use the following rules in order:

(i)	Use average LMP for Electrical Buses within the same station having the same voltage level as the de-energized Electrical Bus, if any exist.

(ii)	Use average LMP for all Electrical Buses within the same station, if any exist.

(iii)	Use System Lambda.

(8)	The Day-Ahead MCPC for each hour for each Ancillary Service is the Shadow Price for that Ancillary Service for the hour as determined by the DAM algorithm.

(9)	If the Day-Ahead MCPC cannot be calculated by ERCOT, the Day-Ahead MCPC for the particular Ancillary Service is equal to the Day-Ahead MCPC for that Ancillary Service in the same Settlement Interval of the preceding Operating Day.

(10)	If the DASPPs cannot be calculated by ERCOT, all CRRs shall be settled based on Real-Time prices.  Settlements for all CRRs shall be reflected on the Real-Time Settlement Statement.

(11)	Constraints can exist between the generator’s Resource Connectivity Node and the Resource Node, in which case the awarded quantity of energy may be inconsistent with the clearing price when the constraint between the Resource Connectivity Node and the Resource Node is binding.

(12)	PTP Obligation bids shall not be settled where the DAM clearing price for the PTP Obligation is greater than the PTP Obligation bid price plus $0.25/MW per hour.

7.9.1.1	Payments and Charges for PTP Obligations Settled in DAM

(1)	ERCOT shall pay or charge the owner of each Point-to-Point (PTP) Obligation based on the difference in the Day-Ahead Settlement Point Price between the sink Settlement Point and the source Settlement Point.

(2)	The payment or charge to each CRR Owner for a given Operating Hour of PTP Obligations with each pair of source and sink Settlement Points settled in the Day-Ahead Market (DAM) is calculated as follows:

DAOBLAMT o, (j, k)	=	(-1) * DAOBLPR (j, k) * DAOBL o, (j, k)

Where:

The price based on the difference of the Settlement Point Prices:

DAOBLPR (j, k)	=	DASPP k - DASPP j

Σc

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOBLAMT o, (j, k) | $ | Day-Ahead Obligation Amount per CRR Owner per source and sink pair—The payment or charge to CRR Owner o for the PTP Obligations with the source j and the sink k settled in the DAM, for the hour. |
| DAOBLPR (j, k) | $/MW per hour | Day-Ahead Obligation Price per source and sink pair—The DAM price of a PTP Obligation with the source j and the sink k, for the hour. |
| DASPP j | $/MWh | Day-Ahead Settlement Point Price at source—The DAM Settlement Point Price at the source Settlement Point j, for the hour. |
| DASPP k | $/MWh | Day-Ahead Settlement Point Price at sink—The DAM Settlement Point Price at the sink Settlement Point k, for the hour. |
| DAOBL o, (j, k) | MW | Day-Ahead Obligation per CRR Owner per source and sink pair—The number of CRR Owner o’s PTP Obligations with the source j and the sink k settled in the DAM for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

(3)	The net total payment or charge to each CRR Owner for the Operating Hour of all its PTP Obligations settled in the DAM is calculated as follows:

DAOBLAMTOTOT o	=	DAOBLCROTOT o + DAOBLCHOTOT o

Where:

DAOBLCROTOT o	=	ΣjΣkMin (0, DAOBLAMT o, (j, k))

DAOBLCHOTOT o	=	ΣjΣkMax (0, DAOBLAMT o, (j, k))

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOBLAMTOTOT o | $ | Day-Ahead Obligation Amount Owner Total per CRR Owner—The net total payment or charge to CRR Owner o for all its PTP Obligations settled in the DAM, for the hour. |
| DAOBLCROTOT o | $ | Day-Ahead Obligation Credit Owner Total per CRR Owner—The total payment to CRR Owner o for its PTP Obligations settled in the DAM, for the hour. |
| DAOBLCHOTOT o | $ | Day-Ahead Obligation Charge Owner Total per CRR Owner—The total charge to CRR Owner o for its PTP Obligations settled in the DAM, for the hour. |
| DAOBLAMT o, (j, k) | $ | Day-Ahead Obligation Amount per CRR Owner per pair of source and sink—The payment or charge to CRR Owner o for its PTP Obligations with the source j and the sink k settled in the DAM, for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

7.9.1.2	Payments for PTP Options Settled in DAM

(1)	ERCOT shall pay the owner of a PTP Option the difference in the Day-Ahead Settlement Point Price between the sink Settlement Point and the source Settlement Point, if positive.

(2)	The payment to each CRR Owner for a given Operating Hour of PTP Options with each pair of source and sink Settlement Points settled in the DAM is calculated as follows:

DAOPTAMT o, (j, k)	=	(-1) * DAOPTPR (j, k) * OPT o, (j, k)

Where:

The price based on the difference of the Settlement Point Prices:

DAOPTPR o, (j, k)	=	Max (0, DASPP k – DASPP j)

Σc

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOPTAMT o, (j, k) | $ | Day-Ahead Option Amount per CRR Owner per source and sink pairThe payment to CRR Owner o for the PTP Options with the source j and the sink k settled in the DAM, for the hour. |
| DAOPTPR (j, k) | $/MW per hour | Day-Ahead Option Price per source and sink pairThe DAM price of a PTP Option with the source j and the sink k, for the hour. |
| DASPP j | $/MWh | Day-Ahead Settlement Point Price at sourceThe DAM Settlement Point Price at the source Settlement Point j, for the hour. |
| DASPP k | $/MWh | Day-Ahead Settlement Point Price at sinkThe DAM Settlement Point Price at the sink Settlement Point k, for the hour. |
| OPT o, (j, k) | MW | Option per CRR Owner per source and sink pairThe number of CRR Owner o’s PTP Options with the source j and the sink k settled in the DAM for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

(3)	The total payment to each CRR Owner for the Operating Hour of all its PTP Options settled in the DAM is calculated as follows:

DAOPTAMTOTOT o	=	ΣjΣkDAOPTAMT o, (j, k)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOPTAMTOTOT o | $ | Day-Ahead Option Amount Owner Total per CRR Owner—The total payment to CRR Owner o for all its PTP Options settled in the DAM, for the hour. |
| DAOPTAMT o, (j, k) | $ | Day-Ahead Option Amount per CRR Owner per pair of source and sink—The payment to CRR Owner o for its PTP Options with the source j and the sink k settled in the DAM, for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

(4)	For informational purposes, the following calculation of PTP Option value shall be posted on the Market Information System (MIS) Public Area:

DAOPTPRINFO (j, k)	=	Σc(DASP c * Max (0, (DAWASF j, c – DAWASF k, c)))

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOPTPRINFO (j, k) | $/MW per hour | Day-Ahead Option Informational Price per pair of source and sink—The informational DAM price of the PTP Options with the source Settlement Point j and the sink Settlement Point k, for the hour. |
| DAWASF j, c |  | Day-Ahead Weighted Average Shift Factor at source per constraint—The Day-Ahead Shift Factor for the source Settlement Point and for the constrained directional network element for constraint c, in the hour. |
| DAWASF k, c | none | Day-Ahead Weighted Average Shift Factor at sink per constraint—The Day-Ahead Shift Factor for the sink Settlement Point and for the constrained directional network element for constraint c, in the hour. |
| DASP c | $/MW per hour | Day-Ahead Shadow Price per constraint—The DAM Shadow Price for the constraint c for the hour. |
| c | none | A constraint associated with a directional network element for the hour. |

(k)	Diesel = FIP * 12 MMBtu/MWh;

(l)	Wind = -$35/MWh;

(3)	Maximum Resource Prices of sink Settlement Points are:

(k)	Diesel = FIP * 16 MMBtu/MWh;

(l)	Wind = $0/MWh;

7.9.1.4	Payments and Charges for PTP Obligations with Refund Settled in DAM

(1)	ERCOT shall pay the owner of a PTP Obligation with Refund the difference in the Day-Ahead Settlement Point Prices between the sink Settlement Point and the source Settlement Point, subject to a charge for refund, when the price difference is positive, as described in the item (1)(e)(i) of Section 7.4.2.2, PCRR Allocations and Nominations.

(2)	The payment or charge to each CRR Owner for a given Operating Hour of PTP Obligations with Refund with each pair of source and sink Settlement Points settled in the DAM is calculated as follows:

DAOBLRAMT o, (j, k)	=	(-1) * DAOBLPR (j, k) * Min (DAOBLR o, (j, k), OBLRACT o, (j, k))

Where:

DAOBLPR (j, k)		=	DASPP k - DASPP j

OBLRACT o, (j, k)	=	Σr(OBLROF o, r * RESACT r * OBLRF o, r, (j, k))

If (a valid OS r, y exists for all SCED intervals within the hour)

RESACT r =    Σy(OS r, y * TLMP y) / (ΣyTLMP y)

Otherwise

RESACT r	=	TGFTH r

Σc

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOBLRAMT o, (j, k) | $ | Day-Ahead Obligation with Refund Amount per CRR Owner per pair of source and sinkThe payment to CRR Owner o for the PTP Obligation with Refund with the source j and the sink k, settled in the DAM, for the hour. |
| DAOBLPR (j, k) | $/MW per hour | Day-Ahead Obligation PriceThe DAM price of a PTP Obligation with the source j and the sink k, for the hour. |
| DASPP j | $/MWh | Day-Ahead Settlement Point Price at sourceThe DAM Settlement Point Price at the source Settlement Point j for the hour. |
| DASPP k | $/MWh | Day-Ahead Settlement Point Price at sinkThe DAM Settlement Point Price at the sink Settlement Point k for the hour. |
| DAOBLR o, (j, k) | MW | Day-Ahead Obligation with Refund per CRR Owner per pair of source and sink The number of CRR Owner o’s PTP Obligations with Refund with the source j and the sink k settled in DAM for the hour. |
| OBLRACT o, (j, k) | MW | Obligation with Refund Actual usage per CRR Owner per pair of source and sink—CRR Owner o’s actual usage for the PTP Obligations with Refund with the source j and the sink k, for the hour. |
| RESACT r | MW | Resource Actual per Resource per hour—The time-weighted average of the Output Schedule of Resource r (if a valid Output Schedule exists) or the telemetered output of Resource r, for the hour. |
| OBLROF o, r | none | Obligation with Refund Ownership Factor per CRR Owner per Resource—The factor showing the percentage usage of Resource r for CRR Owner o’s PTP Obligations with Refund. Its value is 1, if only one CRR Owner has acquired Pre-Assigned Congestion Revenue Right (PCRRs) under the refund provision using this Resource r. |
| OS r, y | MW | Output Schedule per Resource per Security-Constrained Economic Dispatch (SCED) interval—The Output Schedule submitted to ERCOT for Resource r for the SCED interval y. |
| TGFTH r | MWh | Telemetered Generation for the Hour per Resource per hour—The telemetered generation of Generation Resource r, for the hour. |
| OBLRF o, r, (j, k) | none | Obligation with Refund Factor per CRR Owner per Resource associated with pair of source and sink—The ratio of CRR Owner o’s Resource r’s capacity allocated to the PTP Obligations with Refund with the source j and sink k to the same CRR Owner’s total capacity for the Resource r nominated for all the PCRRs under the refund provision with the same source j. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the hour. |
| o | none | A CRR Owner. |
| y | none | A SCED interval in the hour. |
| r | none | A Resource. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

(3)	The net total payment or charge to each CRR Owner for the Operating Hour of all its PTP Obligations with Refund settled in the DAM is calculated as follows:

DAOBLRAMTOTOT o	=	DAOBLRCROTOT o + DAOBLRCHOTOT o

Where:

DAOBLRCROTOT o	=	ΣjΣkMin (0, DAOBLRAMT o, (j, k))

DAOBLRCHOTOT o	=	ΣjΣkMax (0, DAOBLRAMT o, (j, k))

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOBLRAMTOTOT o | $ | Day-Ahead Obligation with Refund Amount Owner Total per CRR Owner—The net total payment or charge to CRR Owner o for all its PTP Obligations with Refund settled in the DAM, for the hour. |
| DAOBLRCROTOT o | $ | Day-Ahead Obligation with Refund Credit Owner Total per CRR Owner—The total payment to CRR Owner o for its PTP Obligations with Refund settled in the DAM, for the hour. |
| DAOBLRCHOTOT o | $ | Day-Ahead Obligation with Refund Charge Owner Total per CRR Owner—The total charge to CRR Owner o for its PTP Obligations with Refund settled in the DAM, for the hour. |
| DAOBLRAMT o, (j, k) | $ | Day-Ahead Obligation with Refund Amount per CRR Owner per pair of source and sink—The payment or charge to CRR Owner o for the PTP Obligations with Refund with the source j and the sink k settled in the DAM, for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

7.9.1.5	Payments for PTP Options with Refund Settled in DAM

(1)	ERCOT shall pay the owner of a PTP Option with Refund the difference in the DAM Settlement Point Prices between the sink Settlement Point and the source Settlement Point, if positive, subject to a charge for refund, as described in item (1)(e)(i) of Section 7.4.2.2, PCRR Allocations and Nominations.

(2)	The payment to each CRR Owner for a given Operating Hour of its PTP Options with Refund with each pair of source and sink Settlement Points settled in the DAM is calculated as follows:

DAOPTRAMT o, (j, k)	=	(-1) * DAOPTPR (j, k) * Min (OPTR o, (j, k), OPTRACT o, (j, k))

Where:

DAOPTPR (j, k)	=	Max (0, DASPP k – DASPP j)

OPTRACT o, (j, k)	=	Σr(OPTROF o, r * RESACT r * OPTRF o, r, (j, k))

If (a valid OS r, y exists for all SCED intervals within the hour)

RESACT r	=	Σy(OS r, y * TLMP y) / (ΣyTLMP y)

Otherwise

RESACT r	=	TGFTH r

Σc

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOPTRAMT o, (j, k) | $ | Day-Ahead Option with Refund Amount per CRR Owner per pair of source and sinkThe payment to CRR Owner o for its PTP Options with Refund with the source j and the sink k, settled in the DAM, for the hour. |
| DAOPTPR (j, k) | $/MW per hour | Day-Ahead Option Price per pair of source and sinkThe DAM price of the PTP Option with the source j and the sink k, for the hour. |
| DASPP j | $/MWh | Day-Ahead Settlement Point Price at sourceThe DAM Settlement Point Price at the source Settlement Point j, for the hour. |
| DASPP k | $/MWh | Day-Ahead Settlement Pont Price at sinkThe DAM Settlement Point Price at the sink Settlement Point k, for the hour. |
| OPTR o, (j, k) | MW | Option with Refund per CRR Owner per pair of source and sinkThe number of CRR Owner o’s PTP Options with Refund with the source j and the sink k, settled in DAM, for the hour. |
| OPTRACT o, (j, k) | MW | Option with Refund Actual usage per CRR Owner per pair of source and sink—CRR Owner o’s actual usage for the PTP Options with Refund with the source j and the sink k, for the hour. |
| RESACT r | MW | Resource Actual per Resource per hour—The time-weighted average of the Output Schedule of Resource r (if a valid Operating Schedule exists) or the telemetered output of Resource r, for the hour. |
| OPTROF o, r | none | Option with Refund Ownership Factor per CRR Owner per Resource—The factor showing the percentage usage of Resource r for CRR Owner o’s PTP Options with Refund. Its value is 1, if only one CRR Owner has acquired PCRRs under the refund provision using this Resource r. |
| OS r, y | MW | Output Schedule per Resource per SCED interval—The Output Schedule submitted to ERCOT for Resource r for the SCED interval y. |
| TGFTH r | MWh | Telemetered Generation for the Hour per Resource per hour—The telemetered generation of Generation Resource r, for the hour. |
| OPTRF o, r, (j, k) | none | Option with Refund Factor per CRR Owner per Resource associated with pair of source and sink—The ratio of CRR Owner o’s Resource r’s capacity allocated to the PTP Options with Refund with the source j and sink k to the same CRR Owner’s total capacity for the Resource r nominated PCRRs under the refund provision with the same source j. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the hour. |
| o | none | A CRR Owner. |
| y | none | A SCED interval in the hour. |
| r | none | A Resource. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

(3)	The total payment to each Non-Opt-In-Entity (NOIE) CRR Owner for the Operating Hour of all its PTP Options with Refund settled in the DAM is calculated as follows:

DAOPTRAMTOTOT o	=	ΣjΣkDAOPTRAMT o, (j, k)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOPTRAMTOTOT o | $ | Day-Ahead Option with Refund Amount Owner Total per CRR Owner—The total payment to NOIE CRR Owner o for all its PTP Options with Refund settled in the DAM, for the hour. |
| DAOPTRAMT o, (j, k) | $ | Day-Ahead Option with Refund Amount per CRR Owner per pair of source and sink—The payment to NOIE CRR Owner o for the PTP Options with Refund with the source j and the sink k settled in the DAM, for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |