| NPRR Number |  | NPRR Title | Addition of the Coal Fuel Index Price |
| --- | --- | --- | --- |
| Date Posted |  |  |  |
| Requested Resolution |  | Normal |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 2.2, Acronyms and Abbreviations 3.14.1.17, Reporting Actual Eligible Fuel Costs 4.4.9.2.1, Startup Offer and Minimum-Energy Offer Criteria 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps 4.4.9.3.1, Energy Offer Curve Criteria 4.4.9.3.3, Energy Offer Curve Caps for Make-Whole Calculation Purposes 4.4.9.4.1, Mitigated Offer Cap 5.7.1.1, RUC Guarantee 6.6.6.2 RMR Payment for Energy |  |
| Related Documents Requiring Revision/Related Revision Requests |  | VCMRRXXX |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) introduces a Coal Fuel Index Price (CFIP) applicable to coal-fired and lignite-fired Resources. |  |
| Reason for Revision |  | Addresses current operational issues. Meets Strategic goals (tied to the ERCOT Strategic Plan or directed by the ERCOT Board). Market efficiencies or enhancements Administrative Regulatory requirements Other: (explain) (please select all that apply) |  |
| Business Case |  | With the implementation of NPRR617, Energy Offer Flexibility, in June 2016, the cost of coal-fired and lignite-fired generation Resources is being determined based on a combination of natural gas (or oil) and solid fuel prices. The solid fuel price for these types of Resources is currently set in the Verifiable Cost Manual at $1.50/MMBtu. However, the average cost of coal delivered (commodity plus transportation) is currently close to $2.60/MMBtu. TAC approved VCMRR014, Revise Fuel Adder Factor for Coal and Lignite Resources, in May 2016 which established a fuel adder of $1.10/MMBtu ($2.60/MMBtu - $1.50/MMBtu) for coal-fired and lignite-fired Resources, with a sunset date of 6/1/18. This was intended to provide an interim, low-impact, solution until a comprehensive solution could be developed and implemented. This NPRR provides a comprehensive solution for an index price applicable only to coal-fired and lignite-fired Resources. This NPRR is the result of a collaborative effort between the Resource Cost Working Group and ERCOT with input from members of WMS. |  |

| Sponsor |  |
| --- | --- |
| Name | Ino González;Magie Annab |
| E-mail Address | igonzalez@ercot.com; Magie.Annab@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-3954 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Kelly Landry |
| E-Mail Address | kelly.landry@ercot.com |
| Phone Number | 512-248-4630 |

Proposed Protocol Language Revision

2.1	DEFINITIONS

Coal Fuel Index Price (CFIP)

A price per MMBtu ($/MMBtu) that reflects the price of  Powder River Basin (PRB) 8400 Btu/lb coal delivered to ERCOT, as derived from published data selected by ERCOT that are generally accepted in the industry and regularly published. The Coal Fuel Index Price for the current day shall be based on the most recent price data received by ERCOT from the publisher for PRB 8400 coal (i.e. prompt quarter or monthly settled price) and the cost of rail transportation from the Powder River Basin to ERCOT. ERCOT shall issue a Market Notice disclosing the name of the ERCOT-selected publication(s) or source(s) used to determine the Coal Fuel Index Price.  In the event that the ERCOT-selected index (or indices) becomes unavailable, or ERCOT determines that the index (or indices) has become unsuitable for the intended purpose, ERCOT may select a substitute index (indices).  Otherwise, ERCOT may use a proxy price “T”, in terms of $/MMBtu, as approved by the Technical Advisory Committee (TAC) until such time that a suitable index is obtained.  ERCOT shall issue a Market Notice disclosing its intent to use a substitute index source and the name of the substitute index source at least 60 days prior to the beginning of its use, or as soon as practicable. Any index or indices utilized hereunder that are in units of [$/per ton] shall be converted to [$/MMBtu] as follows: [$/MMBtu] = [$/ton] * [1 lb/8,400 Btu] * [ton/2,000lb] * [1,000,000 Btu/MMBtu].

Solid Fuel Price (SFP)

A price per MMBtu ($/MMBtu) that reflects the price of  fuel applicable to Resources utilizing solid fuel, excluding coal-fired and lignite-fired Resources. The value for the Solid Fuel Price is $1.50/MMBtu.

  - ACRONYMS AND ABBREVIATIONS

CFIP			Coal Fuel Index Price

SFP			Solid Fuel Price

- 3.14.1.17	Reporting Actual Eligible Fuel Costs

(1)	The RMR Unit owner shall provide ERCOT with actual fuel costs on a monthly basis for the RMR Unit in a level of detail sufficient for ERCOT to verify that all fuel costs are actual and appropriate.  The estimated fuel payments may include a fuel adder to better approximate expected actual fuel costs.  The fuel adder shall represent the difference between the forecasted average actual future fuel price paid and the forecasted average of the relevant index price over the RMR contract period.  The fuel adder must also include the forecasted cost of transporting, delivering and fuel imbalances to the Resource.  QSEs must provide to ERCOT supporting documentation indicating how the fuel adder was determined.  ERCOT shall perform a true-up of the estimated fuel costs using the submitted and verified actual fuel costs for the RMR Unit.  Actual cost data must be submitted on time by the Resource Entity for the RMR Unit and then verified by ERCOT so the actual cost data can be reflected in the True-Up Settlement Statement.  To be considered timely for the final, actual cost data for month ‘x’ must be submitted by the 16th of the month following month ‘x.’  To be considered timely for the true-up, actual cost data for month ‘x’ must be submitted 30 days prior to the publishing date of the True-Up Settlement Statement for the first day in month ‘x.’  Any deviation in filing actual cost data in accordance with this calendar must be requested of ERCOT, by the QSE representing an RMR Unit.  Such request for deviation shall contain the reason for the inability to meet the calendar and an expected date that the cost data will be provided to ERCOT.  At its discretion ERCOT may choose to honor such a request.  ERCOT shall post on the MIS Public Area any such request and response thereto.  In the event that actual cost data is not submitted in accordance with the timeline  or is not an approved deviation for the true-up, then the cost for the portion of Eligible Cost that has not been submitted is deemed to be zero.

[NPRR664 and NPRR659: Replace applicable portions of paragraph (1) above with the following upon system implementation:] (1) The RMR Unit owner shall provide ERCOT with actual fuel costs on a monthly basis for the RMR Unit in a level of detail sufficient for ERCOT to verify that all fuel costs are actual and appropriate. The estimated fuel payments may include a fuel adder to better approximate expected actual fuel costs. The fuel adder shall represent the difference between the forecasted average actual future fuel price paid and the forecasted average of the relevant index price over the RMR contract period. The fuel adder must also include the forecasted cost of transporting, delivering and fuel imbalances to the Resource. QSEs must provide to ERCOT supporting documentation indicating how the fuel adder was determined. ERCOT shall perform a true-up of the estimated fuel costs using the submitted and verified actual fuel costs for the RMR Unit. Actual cost data must be submitted on time by the Resource Entity for the RMR Unit and then verified by ERCOT so the actual cost data can be reflected in the True-Up Settlement Statement. To be considered timely for the final, actual cost data for month ‘x’ must be submitted by the 16th of the month following month ‘x.’ To be considered timely for the true-up, actual cost data for month ‘x’ must be submitted 30 days prior to the publishing date of the True-Up Settlement Statement for the first day in month ‘x.’ Any deviation in filing actual cost data in accordance with this calendar must be requested of ERCOT, by the QSE representing an RMR Unit. Such request for deviation shall contain the reason for the inability to meet the calendar and an expected date that the cost data will be provided to ERCOT. At its discretion ERCOT may choose to honor such a request. No later than two Business Days following its decision, ERCOT shall issue a Market Notice of any such request and its response thereto. In the event that actual cost data is not submitted in accordance with the timeline or is not an approved deviation for the true-up, then the cost for the portion of Eligible Cost that has not been submitted is deemed to be zero.

(2)	Actual fuel costs must be appropriate actual costs attributable to ERCOT’s scheduling and/or deployment of the RMR Unit.  Actual fuel costs may include cost of fuel (including the cost of exceeding swing gas contract limits, additional gas demand costs set by fuel supply, or transportation contracts); demand fees, imbalance penalties, transportation charges, and cash out premiums. In addition, actual fuel costs may include costs incurred to:

(a)	Keep the boiler(s) warmed, if approved in advance by ERCOT; and

(b)	Test the RMR Unit prior to or during the term of the RMR Agreement, if approved in advance by ERCOT.

- 4.4.9.2.1	Startup Offer and Minimum-Energy Offer Criteria

(1)	Each Startup Offer and Minimum-Energy Offer must be reported by a QSE and must include the following information:

(a)	The selling QSE;

(b)	The Resource represented by the QSE from which the offer would be supplied;

(c)	The Resource’s hot, intermediate, and cold Startup Offer in dollars;

(d)	The Resource’s Minimum-Energy Offer in dollars per MWh;

(e)	The first and last hour of the Startup and Minimum-Energy Offers

(f)	The expiration time and date of the offer;

(g)	Percentage of the Fuel Index Price (FIP) to the extent that the startup will be supplied by gas to determine the offer cap;

(h)	Percentage of FIP to the extent that the minimum energy will be supplied by gas to determine the offer cap;

[NPRR664: Replace paragraphs (g) and (h) above with the following upon system implementation:] (g) Percentage of the Fuel Index Price for Resource (FIPRr) to the extent that the startup will be supplied by gas to determine the offer cap; (h) Percentage of the FIPRr to the extent that the minimum energy will be supplied by gas to determine the offer cap;

(i)	Percentage of the Fuel Oil Price (FOP) to the extent that the startup will be supplied by oil to determine the offer cap;

(j)	Percentage of FOP to the extent that the minimum energy will be supplied by oil to determine the offer cap;

(k)	Percentage of the Coal Fuel Index Price (CFIP) to the extent that the startup will be supplied by coal or lignite to determine the offer cap; and

(l)	Percentage of CFIP to the extent that the minimum energy will be supplied by coal or lignite to determine the offer cap.

(2)	Valid Startup Offers and Minimum-Energy Offers (which must be part of a Three-Part Supply Offer) must be received before 1000 for the effective DAM and DRUC.

(3)	A QSE may update and submit a Startup Offer and/or Minimum-Energy Offer for a Resource during the Adjustment Period for any hours in which the Resource is not DAM- or RUC-committed before the offer is updated or submitted.

(4)	The Resource’s Startup Offer must not be greater than 200% of the Resource Category Generic Startup Cost for that type of Resource listed in Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, unless ERCOT has approved verifiable Resource-specific startup costs for that Resource, under Section 4.4.9.2.4, Verifiable Startup Offer and Minimum-Energy Offer Caps, in which case the Resource’s Startup Offer must not be greater than 200% of those approved verifiable Resource-specific Startup Costs.

(5)	The Resource’s Minimum-Energy Offer must not be greater than 200% of the Resource Category Generic Minimum-Energy Cost for that type of Resource listed in Section 4.4.9.2.3 unless ERCOT has approved verifiable Resource-specific minimum-energy costs for that Resource, under Section 4.4.9.2.4 in which case the Resource’s Minimum-Energy Offer must not be greater than 200% of those approved verifiable Resource-specific minimum-energy costs.

(6)	Prior to 1000 for the effective DAM, a QSE may submit and update a Three-Part Supply Offer for a Resource for any hours which were Weekly Reliability Unit Commitment (WRUC)-instructed.

- 4.4.9.2.3	Startup Offer and Minimum-Energy Offer Generic Caps

(1)	The Resource Category Startup Offer Generic Cap, by applicable Resource category, is determined by the following Operations and Maintenance (O&M) costs by Resource category:

| Resource Category | O&M Costs ($) |
| --- | --- |
| Nuclear, coal, lignite and hydro | 7,200 |
| Combined Cycle Generation Resource with a combustion turbine ≥ 90 MW, as determined by the largest combustion turbine in the Combined Cycle Generation Resource and for each combustion turbine in the Combined Cycle Generation Resource | 6,810 |
| Combined Cycle Generation Resource with a combustion turbine < 90 MW, as determined by the largest combustion turbine in the Combined Cycle Generation Resource and for each combustion turbine in the Combined Cycle Generation Resource | 6,810 |
| Gas steam supercritical boiler | 4,800 |
| Gas steam reheat boiler | 3,000 |
| Gas steam non-reheat or boiler w/o air-preheater | 2,310 |
| Simple cycle greater than 90 MW | 5,000 |
| Simple cycle less than or equal to 90 MW | 2,300 |
| Reciprocating Engines | $58 /MW * the average of the seasonal net max sustainable ratings |
| RMR Resource | Not Applicable |
| Wind generation Resources | 0 |
| PhotoVoltaic Generation Resources (PVGRs) | 0 |
| Any Resources not defined above | 0, or as determined by the Verifiable Cost Manual |

(2)	The Resource Category Minimum-Energy Generic Cap is the cost per MWh of energy for a Resource to produce energy at the Resource’s LSL and is as follows:

(a)	Hydro = $10.00/MWh;

(b)	Coal and lignite = 10.5 MMBtu/MWh * ((Percentage of CFIP * CFIP) + (Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

(c)	Combined-cycle greater than 90 MW = 8 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

(d)	Combined-cycle less than or equal to 90 MW = 9 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

(e)	Gas steam supercritical boiler = 14 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

(f)	Gas steam reheat boiler = 14.5 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

(g)	Gas steam non-reheat or boiler without air-preheater = 16.0 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

(h)	Simple-cycle greater than 90 MW = 15.0 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

(i)	Simple-cycle less than or equal to 90 MW = 14.0 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

(j)	Reciprocating engines = 16.0 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Minimum-Energy Offer;

(k)	RMR Resource = RMR contract estimated fuel cost using its contract I/O curve at its LSL times FIP;

[NPRR664: Replace paragraphs (c)-(k) above with the following upon system implementation:] (b) Coal and lignite = 10.5 MMBtu/MWh * ((Percentage of CFIP * CFIP) + (Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer; (c) Combined-cycle greater than 90 MW = 8 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer; (d) Combined-cycle less than or equal to 90 MW = 9 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer; (e) Gas steam supercritical boiler = 14 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer; (f) Gas steam reheat boiler = 14.5 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer; (g) Gas steam non-reheat or boiler without air-preheater = 16.0 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer; (h) Simple-cycle greater than 90 MW = 15.0 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer; (i) Simple-cycle less than or equal to 90 MW = 14.0 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer; (j) Reciprocating engines = 16.0 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in the Minimum-Energy Offer; (k) RMR Resource = RMR contract estimated fuel cost using its contract I/O curve at its LSL times FIPRr;

(l)	Nuclear = Not Applicable;

(m)	Wind generation Resources = $0;

(n)	PVGRs = $0; and

(o)	Other Resources not defined above = $0, or as determined by the Verifiable Cost Manual.

(3)	The FIP, CFIP and FOP used to calculate the Resource Category Minimum-Energy Generic Cap shall be the FIP, CFIP or FOP for the Operating Day.  In the event the Resource Category Minimum-Energy Generic Cap must be calculated before the FIP, CFIP or FOP is available for the particular Operating Day, the FIP, CFIP and FOP for the most recent preceding Operating Day shall be used.  Once the FIP, CFIP and FOP are available for a particular Operating Day, those values shall be used in the calculations.  If the percentage fuel mix is not specified for Resources having the option to specify the fuel mix, with the exception of coal and lignite Resources, then the minimum of FIP or FOP shall be used.  If the percentage fuel mix is not specified for coal and lignite Resources having the option to specify the fuel mix, then the minimum of FIP, CFIP or FOP shall be used.

[NPRR664: Replace paragraph (3) above with the following upon system implementation:] - (3) The FIPRr, CFIP and FOP used to calculate the Resource Category Minimum-Energy Generic Cap shall be the FIPRr, CFIP or FOP for the Operating Day. In the event the Resource Category Minimum-Energy Generic Cap must be calculated before the FIPRr, CFIP or FOP is available for the particular Operating Day, the FIPRr, CFIP and FOP for the most recent preceding Operating Day shall be used. Once the FIPRr, CFIP and FOP are available for a particular Operating Day, those values shall be used in the calculations. If the percentage fuel mix is not specified for Resources having the option to specify the fuel mix, with the exception of coal and lignite Resources, then the minimum of FIPRr or FOP shall be used. If the percentage fuel mix is not specified for coal and lignite Resources having the option to specify the fuel mix, then the minimum of FIPRr, CFIP or FOP shall be used.

(4)	Items (2)(c) and (2)(d) above are determined by capacity of largest simple-cycle combustion turbine in the train.

- 4.4.9.3.1	Energy Offer Curve Criteria

(1)	Each Energy Offer Curve must be reported by a QSE and must include the following information:

(a)	The selling QSE;

(b)	The Resource represented by the QSE from which the offer would be supplied;

(c)	A monotonically increasing offer curve for both price (in $/MWh) and quantity (in MW) with no more than ten price/quantity pairs;

(d)	The first and last hour of the Offer;

(e)	The expiration time and date of the offer;

(f)	List of Ancillary Service Offers from the same Resource;

(g)	Inclusive or exclusive designation relative to other DAM offers; and

(h)	Percentage of FIP, percentage of CFIP and percentage of FOP for generation above LSL subject to the sum of the percentages not exceeding 100%.

[NPRR664: Replace paragraph (h) above with the following upon system implementation:] (h) Percentage of FIPRr, percentage of CFIP and percentage of FOP for generation above LSL subject to the sum of the percentages not exceeding 100%.

(2)	An Energy Offer Curve must be within the range of -$250.00 per MWh and the SWCAP in dollars per MWh.  The software systems must be able to provide ERCOT with the ability to enter Resource-specific Energy Offer Curve floors and caps.

(3)	The minimum amount per Resource for each Energy Offer Curve that may be offered is one MW.

4.4.9.3.3	Energy Offer Curve Caps for Make-Whole Calculation Purposes

(1)	The following Energy Offer Curve Caps must be used for the purpose of make-whole Settlements:

- (a)	Nuclear = $15.00/MWh;

- (b)	Coal and Lignite = 10.5 MMBtu/MWh * ((Percentage of CFIP * CFIP) + (Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (c)	Combined Cycle greater than 90 MW = 9 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (d)	Combined Cycle less than or equal to 90 MW = 10 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (e)	Gas - Steam Supercritical Boiler = 10.5 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (f)	Gas Steam Reheat Boiler = 11.5 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (g)	Gas Steam Non-reheat or boiler without air-preheater = 14.5 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (h)	Simple Cycle greater than 90 MW = 14 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (i)	Simple Cycle less than or equal to 90 MW = 15 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (j)	Reciprocating Engines = 16 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

[NPRR664: Replace paragraphs (b)-(j) above with the following upon system implementation:] - (b) Coal and Lignite = 10.5 MMBtu/MWh * ((Percentage of CFIP * CFIP) + (Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve; - (c) Combined Cycle greater than 90 MW = 9 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve; - (d) Combined Cycle less than or equal to 90 MW = 10 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve; - (e) Gas - Steam Supercritical Boiler = 10.5 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve; - (f) Gas Steam Reheat Boiler = 11.5 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve; - (g) Gas Steam Non-reheat or boiler without air-preheater = 14.5 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve; - (h) Simple Cycle greater than 90 MW = 14 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve; - (i) Simple Cycle less than or equal to 90 MW = 15 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve; - (j) Reciprocating Engines = 16 MMBtu/MWh * ((Percentage of FIPRr * FIPRr) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve;

- (k)	Hydro = $10.00/MWh;

- (l)	Other = SWCAP;

- (m)	RMR Resource = RMR contract price Energy Offer Curve;

(n)	Wind Generation Resources = $0.00/MWh; and

(o) 	PhotoVoltaic Generation Resource (PVGR) = $0.00/MWh.

(2)	ERCOT shall produce an annual report each April that provides the amount of DAM and RUC Make-Whole Payments during the previous calendar year for Resources categorized as Other, per item (1)(l) above, as a percentage of the total amount of DAM and RUC Make-Whole Payments made during the previous calendar year.  The report shall be based on final Settlements and include the total number of Resources classified as Other.  ERCOT shall present this report annually to the appropriate Technical Advisory Committee (TAC) subcommittee.  If there are no Make-Whole Payments for Resources categorized as Other for a given calendar year, then ERCOT will not be required to produce the annual report.

(3)	Items in paragraphs (1)(c) and (d) above are determined by capacity of largest simple-cycle combustion turbine in the train selected.

(4)	The FIP, CFIP and FOP used to calculate the Energy Offer Curve Cap for Make-Whole Payment calculation purposes shall be the FIP, CFIP or FOP for the Operating Day.  In the event the Energy Offer Curve Cap for Make-Whole Payment calculation purposes must be calculated before the FIP, CFIP or FOP is available for the particular Operating Day, the FIP, CFIP and FOP for the most recent preceding Operating Day shall be used.  Once the FIP, CFIP and FOP are available for a particular Operating Day, those values shall be used in the calculations.  If the percentage fuel mix is not specified or if no Energy Offer Curve exists for the Resource, with the exception of coal and ligniteResources, then the minimum of FIP or FOP shall be used.  If the percentage fuel mix is not specified or if no Energy Offer Curve exists for a coal or lignite Resource, then the minimum of FIP, FOP or CFIP shall be used.

[NPRR664: Replace paragraph (4) above with the following upon system implementation:] (4) The FIPRr, CFIP and FOP used to calculate the Energy Offer Curve Cap for Make-Whole Payment calculation purposes shall be the FIPRr, CFIP or FOP for the Operating Day. In the event the Energy Offer Curve Cap for Make-Whole Payment calculation purposes must be calculated before the FIPRr, CFIP or FOP is available for the particular Operating Day, the FIPRr, CFIP and FOP for the most recent preceding Operating Day shall be used. Once the FIPRr, CFIP and FOP are available for a particular Operating Day, those values shall be used in the calculations. If the percentage fuel mix is not specified or if no Energy Offer Curve exists, for the Resource, with the exception of coal and lignite Resources, then the minimum of FIPRr or FOP shall be used. If the percentage fuel mix is not specified or if no Energy Offer Curve exists for a coal or lignite Resource, then the minimum of FIPRr, FOP or CFIP shall be used.

- 4.4.9.4.1	Mitigated Offer Cap

(1)	Energy Offer Curves may be subject to mitigation in Real-Time operations under Section 6.5.7.3, Security Constrained Economic Dispatch, using a Mitigated Offer Cap.  The Mitigated Offer Cap is:

(a)	For a Resource contracted by ERCOT under paragraph (2) of Section 6.5.1.1, ERCOT Control Area Authority, ERCOT shall increase the O&M cost such that every point on the Mitigated Offer Cap curve (cap vs. output level) is greater than the SWCAP in $/MWh.

(b)	For a Generation Resource with a Commercial Operations Date after January 1, 2004, ERCOT shall construct an incremental Mitigated Offer Cap curve (Section 6.5.7.3) such that each point on the Mitigated Offer Cap curve (cap vs. output level) is the greater of:

(i)	14.5 MMBtu/MWh times the FIP; or

(ii)	The Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by [((Percentage of FIP * FIP) + (Percentage of CFIP * CFIP) + (Percentage of FOP * FOP))/100 + fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual], as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (e) below; or

[NPRR664: Replace paragraphs (i) and (ii) above with the following upon system implementation:] (i) 14.5 MMBtu/MWh times the FIPRr; or (ii) The Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by [((Percentage of FIPRr * FIPRr) + (Percentage of CFIP * CFIP) + (Percentage of FOP * FOP))/100 + fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual], as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (e) below; or

(iii)	The amount determined by Verifiable Cost Manual Appendix 10, Procedures for Evaluating Costs and Caps for Energy Storage Resources, for energy storage resources.

(c)	For all other Generation Resources, each point on the Mitigated Offer Cap curve (cap vs. output level) is the greater of:

(i)	10.5 MMBtu/MWh times the FIP; or

(ii)	The Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by [((Percentage of FIP * FIP) + (Percentage of CFIP * CFIP) + (Percentage of FOP * FOP))/100 + fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual], as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (e) below.

[NPRR664: Replace paragraph (c) above with the following upon system implementation:] (c) For all other Generation Resources, each point on the Mitigated Offer Cap curve (cap vs. output level) is the greater of: (i) 10.5 MMBtu/MWh times the FIPRr; or (ii) The Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by [((Percentage of FIPRr * FIPRr) + (Percentage of CFIP * CFIP) + (Percentage of FOP * FOP))/100 + fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual], as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (e) below.

(d)	Notwithstanding paragraphs (b)(ii), (b)(iii), and (c)(ii) above, the Mitigated Offer Cap verifiable variable O&M cost ($/MWh) for Quick Start Generation Resources (QSGRs) shall incorporate the generic or verifiable O&M cost to start the Resource from first fire to LSL including the startup fuel, plus a minimum energy component to account for LSL commitment costs, and consideration of a fuel adder that compensates for the transportation and purchasing of spot fuel as described in the Verifiable Cost Manual.

(e)	The multipliers for paragraphs (b)(ii) and (c)(ii) above are as follows:

(i)	1.10 for Resources running at a ≥ 50% capacity factor for the previous 12 months;

(ii)	1.15 for Resources running at a ≥ 30 and < 50% capacity factor for the previous 12 months;

(iii)	1.20 for Resources running at a ≥ 20 and < 30% capacity factor for the previous 12 months;

(iv)	1.25 for Resources running at a ≥ 10 and < 20% capacity factor for the previous 12 months;

(v)	1.30 for Resources running at a ≥ 5 and < 10% capacity factor for the previous 12 months;

(vi)	1.40 for Resources running at a ≥ 1 and < 5% capacity factor for the previous 12 months; and

(vii)	1.50 for Resources running at a less than 1% capacity factor for the previous 12 months.

(f)	The previous 12 months’ capacity factor must be updated by ERCOT by the 20th day of each month using the most recent data for use in the next month.  ERCOT shall post to the MIS Secure Area the capacity factor for each Resource before the start of the effective month.

(g)	The process for developing the Mitigated Offer Cap in paragraphs (a), (b), (c), and (d) above must be described by ERCOT in a procedure approved by the appropriate TAC subcommittee, and posted to the MIS Secure Area within one Business Day after initial approval, and after each approved change.

- 5.7.1.1	RUC Guarantee

(1)	The allowable Startup Costs and minimum-energy costs of a Resource committed by RUC is the RUC Guarantee.  The RUC Guarantee minimum-energy costs are prorated according to the actual generation when the Resource’s average output during a 15-minute Settlement Interval is below the corresponding LSL.

(2)	The SUPR, MEPR and LSL used to calculate the RUC Guarantee for a Combined Cycle Train are the SUPR, MEPR and LSL that correspond to the Combined Cycle Generation Resource, within the Combined Cycle Train, that is RUC-committed for the hour.

(3)	For an Aggregate Generation Resource (AGR), the Startup Cost shall be scaled according to the maximum number of its generators online during a contiguous block of RUC-committed intervals, as indicated by telemetry, compared to the total number of generators registered to the AGR and used in the approved verifiable cost for the AGR.

(4)	The RUC Guarantee is calculated for non-Combined Cycle Trains as follows:

RUCGq,r,d	=	 (SUPRq,r,s * RUCSUFLAGq,r,s) + (MEPRq,r,i * Min ((LSLq,r,i * (¼)), RTMGq,r,i))

(5)	The RUC Guarantee is calculated for Combined Cycle Trains as follows:

RUCGq,r,d	=	(SUPRq,r,s * RUCSUFLAGq,r,s) +

(MAX (0, SUPR - SUPR)) +

(MEPRq r,i * Min ((LSLq,r,i * (¼)), RTMGq,r i))

(a)	If a Combined Cycle Train transitions to a RUC-committed configuration from a QSE-committed or other RUC-committed configuration, the transition is calculated as follows:

MAX (0, SUPRafterCCGR – SUPRbeforeCCGR)

(b)	If a Combined Cycle Train transitions to a QSE-committed configuration from a RUC-committed configuration, the transition is calculated as follows:

MAX (0, SUPRbeforeCCGR – SUPRafterCCGR)

(6)	If a validated Three-Part Supply Offer has been submitted for a Resource for the RUC, then the RUC Guarantee for that Resource is based on the minimum of the Startup Offer in that validated Three-Part Supply Offer and Startup Cap and the lesser of the Minimum-Energy Offer in that validated Three-Part Supply Offer and the Minimum-Energy Offer Cap.  If a validated Three-Part Supply Offer has not been submitted for a Resource for the RUC and ERCOT has not yet approved verifiable unit-specific costs for the Resource, then the RUC Guarantee for a Resource is based on the Resource Category Startup Generic Cap and the Resource Category Minimum-Energy Generic Cap.  If a validated Three-Part Supply Offer has not been submitted for a Resource for the RUC and ERCOT has approved verifiable unit-specific costs for the Resource, then the RUC Guarantee for a Resource is based on the most recent ERCOT-approved verifiable unit-specific costs for that Resource.

For a Resource which is not an AGR,

If the QSE submitted a validated Three-Part Supply Offer,

Then, 		SUPRq,r,s	=	Min (SUOq,r,s, SUCAP q,r,s)

MEPRq,r,i	=	Min (MEOq,r,i, MECAP q,r,i)

Otherwise, 	SUPRq,r,s 	= 	SUCAPq,r,s

MEPRq,r,i 	= 	MECAPq,r,i

If ERCOT has approved verifiable Startup Costs and minimum-energy costs for the Resource,

Then, 		SUCAP q, r, s	=	verifiable Startup Costs q, r, s

MECAPq,r,i	=	verifiable minimum-energy costs q, r, i

Otherwise, 	SUCAP q, r, s 	= 	RCGSC s

MECAP q, r, i	= 	RCGMEC i

For AGRs,

If the QSE submitted a validated Three-Part Supply Offer,

Then, 		SUPR q,r,s	=	Min (SUO q,r,s, SUCAP q,r,s)

MEPR q,r,i	=	Min (MEO q,r,i, MECAP q,r,i)

Otherwise, 	SUPR q,r,s 	= 	SUCAP q,r,s

MEPR q,r,i 	= 	MECAP q,r,i

If ERCOT has approved verifiable Startup Costs and minimum-energy costs for the Resource,

Then, 		SUCAP q, r, s	=	Max c (AGRRATIO q, p, r ) * verifiable Startup Costs q, r, s

MECAP q,r,i	=	verifiable minimum-energy costs q, r, i

Where, 		AGRRATIO q,p,r	 =	AGRMAXON q, p, r / AGRTOT q, p, r

Otherwise, 	SUCAP q, r, s 	= 	Max c (AGRRATIO q, p, r) * RCGSC s

MECAP q, r, i	= 	RCGMEC i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCGq,r,d | $ | RUC Guarantee—The sum of eligible Startup Costs and minimum-energy costs for Resource r during all RUC-Committed Hours, for the Operating Day. When one or more Combined Cycle Generation Resources are committed by RUC, guaranteed costs are calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| SUPRq,r,s | $/Start | Startup Price per start—The Settlement price for Resource r for the start s. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| SUOq,r,s | $/Start | Startup Offer per start—Represents an offer for all costs incurred by Generation Resource r in starting up and reaching the Resource’s LSL. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| SUCAPq,r,s | $/Start | Startup Cap—The amount used for AGR r or Resource r as Startup Costs. The cap is the Resource Category Startup Offer Generic Cap (RCGSC) unless ERCOT has approved verifiable unit-specific Startup Costs for that Resource, in which case the startup cap is the scaled verifiable unit-specific Startup Cost for the AGR or the verifiable unit-specific Startup Cost for non-AGRs. The verifiable unit-specific Startup Cost will be determined as described in Section 5.6.1, Verifiable Costs, minus the average energy produced during the time period between breaker close and LSL multiplied by the heat rate proxy “H” multiplied by the appropriate Fuel Index Price (FIP), Fuel Oil Price (FOP), Coal Fuel Index Price (CFIP) or Solid Fuel Price (SFP), for AGR and non-AGR Resources. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| [NPRR664: Replace variable “SUCAPq,r,s” above with the following upon system implementation:] SUCAPq,r,s; $/Start; Startup Cap—The amount used for AGR r or Resource r as Startup Costs. The cap is the Resource Category Startup Offer Generic Cap (RCGSC) unless ERCOT has approved verifiable unit-specific Startup Costs for that Resource, in which case the startup cap is the scaled verifiable unit-specific Startup Cost for the AGR or the verifiable unit-specific Startup Cost for non-AGRs. The verifiable unit-specific Startup Cost will be determined as described in Section 5.6.1, Verifiable Costs, minus the average energy produced during the time period between breaker close and LSL multiplied by the heat rate proxy “H” multiplied by the appropriate Fuel Index Price for Resource (FIPRr), Fuel Oil Price (FOP), Coal Fuel Index Price (CFIP) or Solid Fuel Price (SFP), for AGR and non-AGR Resources. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train.. |
| AGRRATIO q, p, r | none | Aggregate Generation Resource Ratio per QSE per Settlement Point per Aggregate Generation Resource—A value which represents the ratio of the maximum number of generators online during an hour, as indicated by telemetry, compared to the total number of generators registered to the AGR and used in the approved verifiable cost for the AGR. The value is only applicable if the Resource is an AGR. |
| AGRMAXON q, p, r | none | Aggregate Generation Resource Maximum Online per QSE per Settlement Point per Aggregate Generation Resource—The maximum number of generators online during an hour, as indicated by telemetry. The value is only applicable if the Resource is an AGR. |
| AGRTOT q, p, r | none | Aggregate Generation Resource Total per QSE per Settlement Point per Aggregate Generation Resource—The total number of generators registered to the AGR and used in the approved verifiable cost for the AGR. The value is only applicable if the Resource is an AGR. |
| RCGSCs | $/Start | Resource Category Generic Startup Cost—The Resource Category Generic Startup Cost cap for the category of the Resource, according to Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. |
| RUCSUFLAGq,r,s | none | RUC Startup Flag—The flag that indicates whether or not the start s for Resource r is eligible for RUC Make-Whole Payment. Its value is one if eligible; otherwise, zero. See Section 5.6.2, RUC Startup Cost Eligibility, and Section 5.6.3, Forced Outage of RUC-Committed Resource, for more information on startup eligibility. For a Combined Cycle Train, the Resource r must be one of the registered Combined Cycle Generation Resources within the Combined Cycle Train. When one or more Combined Cycle Generation Resources are committed by RUC, the RUC Startup Flag is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| MEPRq,r,i | $/MWh | Minimum-Energy Price—The Settlement price for Resource r for minimum energy for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MEOq,r,i | $/MWh | Minimum-Energy Offer—Represents an offer for the costs incurred by Resource r in producing energy at the Resource’s LSL for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MECAPq,r,i | $/MWh | Minimum-Energy Cap—The amount used for Resource r for minimum-energy costs. The minimum cost is the Resource Category Minimum-Energy Generic Cap (RCGMEC) unless ERCOT has approved verifiable unit-specific minimum energy costs for that Resource, in which case the Minimum-Energy Cap is the verifiable unit-specific minimum energy cost. See Section 5.6.1 for more information on verifiable costs. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RCGMECi | $/MWh | Resource Category Generic Minimum-Energy Cost—The Resource Category Generic Minimum Energy Cost cap for the category of the Resource, according to Section 4.4.9.2.3, for the Operating Day. |
| RTMGq,r,i | MWh | Real-Time Metered Generation—The Resource r’s metered generation for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| LSLq,r,i | MW | Low Sustained Limit—The LSL of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the Current Operating Plan (COP). Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | A RUC-committed Generation Resource. |
| d | none | An Operating Day containing the RUC-commitment. |
| i | none | A 15-minute Settlement Interval within the hour that includes a RUC-commitment. |
| s | none | A start that is eligible to have its costs included in the RUC Guarantee. |
| t | none | A transition that is eligible to have its costs included in the RUC Guarantee. |
| c | none | A contiguous block of RUC–Committed Hours. |
| afterCCGR | none | The Combined Cycle Generation Resource to which a Combined Cycle Train transitions. |
| beforeCCGR | none | The Combined Cycle Generation Resource from which a Combined Cycle Train transitions. |

6.6.6.2	RMR Payment for Energy

(1)	Payment for energy on the Initial Settlement and settlements executed before true-up and before actual cost data is submitted must be calculated using the estimated input/output curve and startup fuel as specified in the RMR Agreement, the actual energy produced and the FIP, CFIP, FOP or SFP.  The payment for energy for all other settlements must be based on actual fuel costs for the RMR Unit.  The payment for energy for each hour is calculated as follows:

RMREAMT q, r  =	(-1) * (((RMRSUPR q, r + RMRCEFA q, r) * RMRSUFQ q, r / RMRH q, r) * RMRALLOCFLAG q, r + Σ4i=1(((RMREPR q, r + RMRCEFA q, r) * RMRHR q, r, i + RMRVCC q, r) * RTMG q, r, i))

Where:

RMRSUPR q, r = 	CLPERSU r, s * CFIP + GASPERSU r, s * FIP + OILPERSU r, s * FOP+ SFPERSU r, s * SFP

RMREPR q, r = 		CLPEROL r * CFIP + GASPEROL r * FIP + OILPEROL r * FOP + SFPEROL r *SFP

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMREAMT q, r | $ | Reliability Must-Run Energy Amount per QSE per Resource by hour—The energy payment to QSE q for RMR Unit r, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FIP | $/MMBtu | Fuel Index Price— As defined in Section 2, Definitions and Acronyms. |
| CFIP | $/MMBtu | Coal Fuel Index Price – As defined in Section 2. |
| FOP | $/MMBtu | Fuel Oil Price- As defined in Section 2. |
| SFP | $/MMBtu | Solid Fuel Price- As defined in Section 2. |
| RMRSUFQ q, r, s | MMBtu | Reliability Must-Run Startup Fuel Quantity per QSE per ResourceThe Estimated Start Up Fuel specified in the RMR Agreement for RMR Unit r represented by QSE q. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRH q, r,h | hour | Reliability Must-Run Hours—The number of hours during which RMR Unit r represented by QSE q is instructed On-Line for the Operating Day. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRALLOCFLAG q, r | none | Reliability Must-Run Startup Flag per QSE per Resource by hour—The number that indicates whether or not the startup fuel cost of RMR Unit r represented by QSE q is allocated to the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. The startup fuel cost will be allocated equally to all contiguous intervals for which there is an eligible start. The RMRALLOCFLAG q, r value is 1 if the startup fuel cost is allocated; otherwise, its value is 0. The RMRALLOCFLAG q, r for eligibility is determined in Sections 5.6.2, RUC Startup Cost Eligibility, and 5.6.3, Forced Outage of a RUC-Committed Resource, for start-up payments and commitments in either the RUC or DAM. |
| RMRHR q, r, i | MMBtu /MWh | Reliability Must-Run Heat Rate per QSE per Resource by Settlement Interval by hour—The incremental heat rate based on the input/output curve and the Real-Time generation of RMR Unit r represented by QSE q, for the 15-minute Settlement Interval i in the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRVCC q, r | $/MWh | Reliability Must-Run Variable Cost Component per QSE per Resource—The monthly cost component that is used to adjust the energy cost calculation to reflect the actual fuel costs of RMR Unit r represented by QSE q. The value is initially set to zero. For resettlements, see item (2) below. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTMG q, r, i, | MWh | Real-Time Metered Generation per QSE per Resource by Settlement Interval by hour—The Real-Time energy from RMR Unit r represented by QSE q, for the 15-minute Settlement Interval i in the hour h. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRCEFA q, r | $/MMBtu | Reliability Must-Run Contractual Estimated Fuel Adder—The Estimated Fuel Adder that is contractually agreed upon in Section 22, Attachment B, Standard Form Reliability Must-Run Agreement. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. The fuel adder will be subsequently trued up to reflect actual fuel costs as set forth in item (1) above. |
| GASPERSU r, s | None | Percent Gas to Operate per Start- The percentage of natural gas used by Resource r from startup to LSL, for the start type s, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. See the Verifiable Cost manual for additional information. |
| OILPERSU r, s | None | Percent Oil to Operate per Start- The percentage of fuel oil used by Resource r from startup to LSL, for the start type s, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. See the Verifiable Cost manual for additional information. |
| SFPERSU r, s | None | Percent Solid Fuel to Operate per Start- The percentage of solid fuel used by Resource r from startup to LSL, for the start type s, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. |
| CLPERSU r, s | None | Percent Coal/Lignite to Operate per Start - The percentage of coal and/or lignite used by Resource r from startup to LSL, for the start type s, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. |
| GASPEROL r | None | Percent Gas to Operate above LSL- The percentage of natural gas used by Resource r to operate above LSL, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. See the Verifiable Cost manual for additional information. |
| OILPEROL r | None | Percent Oil to Operate above LSL - The percentage of fuel oil used by Resource r to operate above LSL, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. See the Verifiable Cost manual for additional information. |
| SFPEROL r | None | Percent Solid Fuel to Operate above LSL - The percentage of solid fuel used by Resource r to operate above LSL, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. |
| CLPEROL r | None | Percent Coal/Lignite to Operate above LSL - The percentage of coal and/or lignite used by Resource r to operate above LSL, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. |
| RMRSUPR q, r | $/MMBtu | Reliability Must-Run Startup Price – The calculated index fuel price for Resource r from startup to LSL. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RMREPR q, r | $/MMBtu | Reliability Must-Run Energy Price – The calculated index fuel price for Resource r to operate above LSL. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | An RMR Unit. |
| i | none | A 15-minute Settlement Interval. |
| s | none | The Resource start type. |

[NPRR664: Replace paragraph (1) above with the following upon system implementation:] (1) Payment for energy on the Initial Settlement and settlements executed before true-up and before actual cost data is submitted must be calculated using the estimated input/output curve and startup fuel as specified in the RMR Agreement, the actual energy produced and the FIPRr. The payment for energy for all other settlements must be based on actual fuel costs for the RMR Unit. The payment for energy for each hour is calculated as follows: RMREAMT q, r = (-1) * (((RMRSUPR q, r + RMRCEFA q, r) * RMRSUFQ q, r / RMRH q, r) * RMRALLOCFLAG q, r + Σ4i=1(((RMREPR q, r + RMRCEFA q, r) * RMRHR q, r, i + RMRVCC q, r) * RTMG q, r, i)) Where: RMRSUPR q, r = CLPERSU r, s * CFIP + GASPERSU r, s * FIPR r + OILPERSU r, s * FOP+ SFPERSU r, s * SFP RMREPR q, r = CLPEROL r * CFIP + GASPEROL r * FIPR r + OILPEROL r * FOP + SFPEROL r *SFP The above variables are defined as follows: Variable; Unit; Definition. RMREAMT q, r; $; Reliability Must-Run Energy Amount per QSE per Resource by hour—The energy payment to QSE q for RMR Unit r, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. FIPRr; $/MMBtu; Fuel Index Price for Resource— As defined in Section 2, Definitions and Acronyms.. CFIP; $/MMBtu; Coal Fuel Index Price – As defined in Section 2.. FOP; $/MMBtu; Fuel Oil Price- As defined in Section 2.. SFP; $/MMBtu; Solid Fuel Price- As defined in Section 2.. RMRSUFQ q, r; MMBtu; Reliability Must-Run Startup Fuel Quantity per QSE per ResourceThe Estimated Start Up Fuel specified in the RMR Agreement for RMR Unit r represented by QSE q. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. RMRH q, r,h; hour; Reliability Must-Run Hours—The number of hours during which RMR Unit r represented by QSE q is instructed On-Line for the Operating Day. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. RMRALLOCFLAG q, r; none; Reliability Must-Run Startup Flag per QSE per Resource by hour—The number that indicates whether or not the startup fuel cost of RMR Unit r represented by QSE q is allocated to the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. The startup fuel cost will be allocated equally to all contiguous intervals for which there is an eligible start. The RMRALLOCFLAG q, r value is 1 if the startup fuel cost is allocated; otherwise, its value is 0. The RMRALLOCFLAG q, r for eligibility is determined in Sections 5.6.2, RUC Startup Cost Eligibility, and 5.6.3, Forced Outage of a RUC-Committed Resource, for start-up payments and commitments in either the Reliability Unit Commitment (RUC) or DAM.. RMRHR q, r, i; MMBtu /MWh; Reliability Must-Run Heat Rate per QSE per Resource by Settlement Interval by hour—The incremental heat rate based on the input/output curve and the Real-Time generation of RMR Unit r represented by QSE q, for the 15-minute Settlement Interval i in the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. RMRVCC q, r; $/MWh; Reliability Must-Run Variable Cost Component per QSE per Resource—The monthly cost component that is used to adjust the energy cost calculation to reflect the actual fuel costs of RMR Unit r represented by QSE q. The value is initially set to zero. For resettlements, see item (2) below. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. RTMG q, r, i,; MWh; Real-Time Metered Generation per QSE per Resource by Settlement Interval by hour—The Real-Time energy from RMR Unit r represented by QSE q, for the 15-minute Settlement Interval i in the hour h. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. RMRCEFA q, r; $/MMBtu; Reliability Must-Run Contractual Estimated Fuel Adder—The Estimated Fuel Adder that is contractually agreed upon in Section 22, Attachment B, Standard Form Reliability Must-Run Agreement. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. The fuel adder will be subsequently trued up to reflect actual fuel costs as set forth in item (1) above.. GASPERSU r, s; None; Percent Gas to Operate per Start- The percentage of natural gas used by Resource r from startup to LSL, for the start type s, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. See the Verifiable Cost manual for additional information.. OILPERSU r, s; None; Percent Oil to Operate per Start- The percentage of fuel oil used by Resource r from startup to LSL, for the start type s, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. See the Verifiable Cost manual for additional information.. SFPERSU r, s; None; Percent Solid Fuel to Operate per Start- The percentage of solid fuel used by Resource r from startup to LSL, for the start type s, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. See the Verifiable Cost manual for additional information.. CLPERSU r, s; None; Percent Coal/Lignite to Operate per Start - The percentage of coal and/or lignite used by Resource r from startup to LSL, for the start type s, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. See the Verifiable Cost manual for additional information.. GASPEROL r; None; Percent Gas to Operate above LSL- The percentage of natural gas used by Resource r to operate above LSL, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. See the Verifiable Cost manual for additional information.. OILPEROL r; None; Percent Oil to Operate above LSL - The percentage of fuel oil used by Resource r to operate above LSL, as submitted and approved in the Verifiable Cost process or in the RMR Agreement. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. See the Verifiable Cost manual for additional information.. SFPEROL r; None; Percent Solid Fuel to Operate above LSL - The percentage of solid fuel used by Resource r to operate above LSL, as submitted and approved in the Verifiable Cost process or in the RMR Agreement.. CLPEROL r; None; Percent Coal/Lignite to Operate above LSL - The percentage of coal and/or lignite used by Resource r to operate above LSL, as submitted and approved in the Verifiable Cost process or in the RMR Agreement.. RMRSUPR q, r; $/MMBtu; Reliability Must-Run Startup Price – The calculated index fuel price for Resource r from startup to LSL. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train.. RMREPR q, r; $/MMBtu; Reliability Must-Run Energy Price – The calculated index fuel price for Resource r to operate above LSL. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train.. q; none; A QSE.. r; none; An RMR Unit.. i; none; A 15-minute Settlement Interval.. s; none; The Resource start type..

(2)	If the RMR actual fuel cost is filed in accordance with the timeline in these Protocols, the monthly RMR variable cost component is calculated for the subsequent resettlements as follows:

RMRVCC q, r   =	(RMRMFCOST q, r + ΣhRMREAMT q, r, f, h) /

(ΣiRTMG q, r, i)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMRVCC q, r | $/MWh | Reliability Must-Run Variable Cost Component per QSE per Resource—The monthly cost component that is used to adjust the energy cost calculation to reflect the actual fuel costs of RMR Unit r represented by QSE q. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRMFCOST q, r | $ | Reliability Must-Run Monthly actual Fuel Cost per QSE per Resource—The monthly actual fuel cost of RMR Unit r represented by QSE q, for the month. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTMG q, r, i | MWh | Real-Time Metered Generation per QSE per Resource by Settlement Interval—The Real-Time energy from RMR Unit r represented by QSE q for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMREAMT q, r, f, h | $ | Reliability Must-Run Energy Amount per QSE per Resource by hour—The energy payment to QSE q for RMR Unit r, for the hour h, from the former Settlement Statement f. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | An RMR Unit. |
| h | none | An hour in the month. |
| i | none | A 15-minute Settlement Interval in the month. |
| f | none | Amount from former settlement run. |

(3)	The total of the payments for energy to each QSE for all RMR Units represented by this QSE for a given hour is calculated as follows:

RMREAMTQSETOT q	=	ΣrRMREAMT q, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMREAMTQSETOT q | $ | Reliability Must-Run Energy Amount QSE Total per QSEThe total of the energy payments to QSE q for all RMR Units represented by this QSE for the hour. |
| RMREAMT q, r | $ | Reliability Must-Run Energy Amount per QSE per Resource by hour—The energy payment to QSE q for RMR Unit r, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | An RMR Unit. |